| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0191 |
| --- | --- |

| Title: | AHG9: On decoding process for symmetric motion vector difference reference indices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to clearly specify the invoke order of decoding process for symmetric motion vector difference reference indices, specified in 8.3.5, to avoid the parsing dependency problem with the flag sym_mvd_flag[][] signaled in coding unit syntax. It is asserted that the decoding process for symmetric motion vector difference reference indices shall be invoked at the beginning of the decoding process for each B slice, after decoding of the slice header as well as the invocation of the decoding process for reference picture list construction for the slice, but prior to the parsing and decoding of any coding unit.

# Problem statement

In the current VVC spec., the flag sym_mvd_flag[][] is signaled in coding unit syntax, only when the values of the variables RefIdxSymL0 and RefIdxSymL1 are greater than -1. The values of the variables RefIdxSymL0 and RefIdxSymL1 are derived by Decoding process for symmetric motion vector difference reference indices, specified in 8.3.5. Thus, the 8.3.5 decoding process should be invoked before parsing of any coding unit syntax, specified in 7.3.10.5. Otherwise, parsing error will result from the parsing dependency on the decoding process for symmetric motion vector difference reference indices. The problem is that it is not clearly specified when the decoding process for symmetric motion vector difference reference indices in 8.3.5 is invoked, in the current VVC spec. In order to avoid any parsing dependency problem, it is proposed to specify the invoke order related to decoding process for symmetric motion vector difference reference indices, as described in the following section. Also, the 8.3.5 decoding process shall be invoked after decoding of the slice header as well as the invocation of the decoding process for reference picture list construction for the slice as specified in clause 8.3.2, because reference picture lists shall be constructed before invoking 8.3.5. Finally, the decoding process in 8.3.5 shall be invoked only when B-slices are decoded, because the variables RefIdxSymL0 and RefIdxSymL1 are used only for B-slices.

# Proposal

The following modifications are proposed in Section 8.3 Slice decoding process:

Proposed Spec Text

    - Decoding process for a coded picture

The decoding processes specified in this clause apply to each coded picture, referred to as the current picture and denoted by the variable CurrPic, in BitstreamToDecode.

Depending on the value of sps_chroma_format_idc, the number of sample arrays of the current picture is as follows:

–	If sps_chroma_format_idc is equal to 0, the current picture consists of 1 sample array SL.

–	Otherwise (sps_chroma_format_idc is not equal to 0), the current picture consists of 3 sample arrays SL, SCb, SCr.

The decoding process for the current picture takes as inputs the syntax elements and upper-case variables from clause 7. When interpreting the semantics of each syntax element in each NAL unit, and in the remaining parts of clause 8, the term "the bitstream" (or part thereof, e.g., a CVS of the bitstream) refers to BitstreamToDecode (or part thereof).

The decoding process operates as follows for the current picture CurrPic:

- The decoding of NAL units is specified in clause 8.2.

- The processes in clause 8.3 specify the following decoding processes using syntax elements in the slice header layer and above:

–	Variables and functions relating to picture order count are derived as specified in clause 8.3.1. This needs to be invoked only for the first slice of a picture.

–	At the beginning of the decoding process for each slice of a picture, the decoding process for reference picture lists construction specified in clause 8.3.2 is invoked for derivation of reference picture list 0 (RefPicList[ 0 ]) and reference picture list 1 (RefPicList[ 1 ]).

–	The decoding process for reference picture marking in clause 8.3.3 is invoked, wherein reference pictures may be marked as "unused for reference" or "used for long-term reference". This needs to be invoked only for the first slice of a picture.

–	When the current picture is an IDR picture with sps_idr_rpl_present_flag equal to 1 or pps_rpl_info_in_ph_flag equal to 1, a CRA picture with NoOutputBeforeRecoveryFlag equal to 1, or a GDR picture with NoOutputBeforeRecoveryFlag equal to 1, the decoding process for generating unavailable reference pictures specified in subclause 8.3.4 is invoked, which needs to be invoked only for the first slice of a picture.

  - At the beginning of the decoding process for each B slice, the decoding process for symmetric motion vector difference reference indices specified in clause 8.3.5 is invoked for derivation of the variables RefIdxSymL0 and RefIdxSymL1.

### Decoding process for symmetric motion vector difference reference indices

This process is invoked at the beginning of the decoding process for each B slice, after decoding of the slice header as well as the invocation of the decoding process for reference picture list construction for the slice as specified in clause 8.3.2, but prior to the parsing and decoding of any coding unit.

Output of this process are RefIdxSymL0 and RefIdxSymL1 specifying the list 0 and list 1 reference picture indices for symmetric motion vector differences, i.e., when sym_mvd_flag is equal to 1 for a coding unit.

The variable RefIdxSymLX with X being 0 and 1 is derived as follows:

- The variable currPic specifies the current picture.

- RefIdxSymL0 is set equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 0 ] − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL0 is set equal to i:

- RefPicList[ 0 ][ i ] is a short-term-reference picture,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > 0,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1.

- RefIdxSymL1 is set equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 1 ]  − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL1 is set equal to i:

- RefPicList[ 1 ][ i ] is a short-term-reference picture,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < 0,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1.

- When RefIdxSymL0 is equal to −1 or RefIdxSymL1 is equal to −1, the following applies:

- RefIdxSymL0 is set equal to −1 and RefIdxSymL1 is set equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 0 ]  − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL0 is set equal to i:

- RefPicList[ 0 ][ i ] is a short-term-reference picture,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < 0,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 1 ]  − 1, the following applies:

- When all of the following conditions are true, RefIdxSymL1 is set equal to i:

- RefPicList[ 1 ][ i ] is a short-term-reference picture,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > 0,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).