| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0195 |
| --- | --- |

| Title: | AHG9: On GCI cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yao-Jen Chang, Vadim Seregin, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following GCI cleanups:

- Remove GCI flags on SEI messages, general_non_packed_constraint_flag and general_non_projected_constraint_flag

- Rename general_frame_only_constraint_flag to no_field_seq_constraint_flag and update semantic to constrain sps_field_seq_flag

- Propose 3 additional GCI flags

  - no_subpic_info_constraint_flag to constrain sps_subpic_info_present_flag

  - no_idr_rpl_constraint_flag to constrain sps_idr_rpl_present_flag

  - no_hrd_constraint_flag to constrain vps_general_hrd_params_present_flag and sps_general_hrd_params_present_flag

# Proposals

## Remove GCI flags

It was recommended in last AHG meeting not to specify SEI message relevant GCI flags. It is proposed to remove the following two SEI message GCI flags specified in VVC draft.

general_non_packed_constraint_flag equal to 1 specifies that there shall not be any frame packing arrangement SEI messages present in the bitstream of the OlsInScope. general_non_packed_constraint_flag equal to 0 does not impose such a constraint.

general_non_projected_constraint_flag equal to 1 specifies that there shall not be any equirectangular projection SEI messages or generalized cubemap projection SEI messages present in the bitstream of the OlsInScope. general_non_projected_constraint_flag equal to 0 does not impose such a constraint.

## general_frame_only_constraint_flag

VVC specifies general_frame_only_constraint_flag as follows:

general_frame_only_constraint_flag equal to 1 specifies that OlsInScope conveys pictures that represent frames. general_frame_only_constraint_flag equal to 0 specifies that OlsInScope conveys pictures that may or may not represent frames.

There is corresponding SPS flag, sps_field_seq_flag, indicating the frames or fields conveyed in CLVS. It is proposed to rename general_frame_only_constraint_flag to no_field_seq_constraint flag and modify the semantic for the consistence as follows:

no_field_seq_constraint_flag equal to 1 specifies that sps_field_seq_flag shall be equal to 0. no_field_seq_constraint_flag equal to 0 does not impose such constraint.

## Additional GCI flags

It is proposed to add the following additional new GCI flags to constrain the relevant SPS syntax elements.

- no_subpic_info_constraint_flag equal to 1 specifies that sps_subpic_info_present_flag shall be equal to 0. no_subpic_info_constraint_flag equal to 0 does not impose such constraint.

- no_idr_rpl_constraint_flag equal to 1 specifies that sps_idr_rpl_present_flag shall be equal to 0, no_idr_rpl_constraint_flag equal to 0 does not impose such constraint.

- no_hrd_constraint_flag equal to 1 specifies that both vps_general_hrd_present_flag and sps_general_hrd_params_present_flag shall be equal to 0. no_hrd_constraint_flag equal to 0 does not impose such constraint.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).