| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0203 |
| --- | --- |

| Title: | AHG9: On signalling of sub-layer level idc |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry | Email: | dr.hendry@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

In the current VVC specification text, the presence of sublayer_level_idc[ i ] is gated by the value of ptl_sublayer_level_present_flag[ i ] and when it is not present, its value is inferred to be equal to the value of sub-layer level idc of the higher sub-layer (i.e., sublayer_level_idc[ i + 1 ]. While the inference rule for sublayer_level_idc[ i ] when not present is done in decreasing order (i.e., from higher to lower sub-layer), the signalling of sublayer_level_idc[ i ] is currently in ascending order (i.e., from lower to higher sub-layer). It is asserted there is no benefit of having signalling a syntax element and its reference rule when it is not present in this way. Moreover, such design causes implementation of the inference rule cannot be done in the same loop as parsing process.

This contribution proposes the following changes:

- Change the signalling of sublayer_level_idc[ i ] to be in descending order, i.e., from the maxNumSubLayersMinus1 – 1 to 0, inclusive.

- Change the signalling of ptl_sublayer_level_present_flag[ i ] to be in descending order as well.

With the proposed change, it is asserted that the signalling and the inference rule for sublayer_level_idc[ i ] would be more natural and easier for implementation of parsing and inference processes since they both are in the same order.

# Introduction

In the current VVC specification text, the presence of sublayer_level_idc[ i ] is gated by the value of ptl_sublayer_level_present_flag[ i ] and when it is not present, its value is inferred to be equal to the value of sub-layer level idc of the higher sub-layer (i.e., sublayer_level_idc[ i + 1 ]. While the inference rule for sublayer_level_idc[ i ] when not present is done in decreasing order (i.e., from higher to lower sub-layer), the signalling of sublayer_level_idc[ i ] is currently in ascending order (i.e., from lower to higher sub-layer) shown in the snippet of syntax and semantics below.

-----------------------------------------------start syntax and semantics-----------------------------------------------

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| ptl_sublayer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( ptl_sublayer_level_present_flag[ i ] ) |  |
| sublayer_level_idc[ i ] | u(8) |
| } |  |

...

ptl_sublayer_level_present_flag[ i ] equal to 1 specifies that level information is present in the profile_tier_level( ) syntax structure for the sublayer representation with TemporalId equal to i. ptl_sublayer_level_present_flag[ i ] equal to 0 specifies that level information is not present in the profile_tier_level( ) syntax structure for the sublayer representation with TemporalId equal to i.

ptl_alignment_zero_bits shall be equal to 0.

The semantics of the syntax element sublayer_level_idc[ i ] is, apart from the specification of the inference of not present values, the same as the syntax element general_level_idc, but apply to the sublayer representation with TemporalId equal to i.

When not present, the value of sublayer_level_idc[ i ] is inferred as follows:

–	sublayer_level_idc[ maxNumSubLayersMinus1 ] is inferred to be equal to general_level_idc of the same profile_tier_level( ) structure,

–	For i from maxNumSubLayersMinus1 − 1 to 0 (in decreasing order of values of i), inclusive, sublayer_level_idc[ i ] is inferred to be equal to sublayer_level_idc[ i + 1 ].

------------------------------------------------end syntax and semantics------------------------------------------------

It is asserted that there is no benefit of having signalling a syntax element in ascending order while the inference rule when it is not present is in descending order. Furthermore, it causes the implementation of the inference rule cannot be done in the same loop as parsing process.

# Proposed changes and spec text

We propose to the following changes to the current VVC specification:

- Change the signalling of sublayer_level_idc[ i ] to be in descending order, i.e., from the maxNumSubLayersMinus1 – 1 to 0, inclusive.

- Change the signalling of ptl_sublayer_level_present_flag[ i ] to be in descending order as well.

The proposed text for this option is as follows:

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| ... |  |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) for( i = maxNumSubLayersMinus1 − 1; i >= 0; i− − ) |  |
| ptl_sublayer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) for( i = maxNumSubLayersMinus1 − 1; i >= 0; i− − ) |  |
| if( ptl_sublayer_level_present_flag[ i ] ) |  |
| sublayer_level_idc[ i ] | u(8) |
| } |  |

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).