| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0204 |
| --- | --- |

| Title: | AHG9: On signalling of the maximum transform skip size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

In this contribution, it is proposed that the maximum transform skip size (sps_log2_transform_skip_max_size_minus2) is signalled by u(2) instead of ue(v). In the current spec, possible values of sps_log2_transform_skip_max_size_minus2 are only in the range of 0 to 3.

Introduction

In the current spec [1], the maximum TU size for transform skip is signalled as sps_log2_transform_skip_max_size_minus2 by ue(v), as follows. However, possible values of sps_log2_transform_skip_max_size_minus2 are only in the range of 0 to 3 (corresponding TU sizes are 4, 8, 16, and 32). As shown in Table 1, bin string lengths by ue(v) are longer than by u(2) except in the case that codeNum is equal to 0. The case that codeNum is equal to 0 does not seem to be the most usual case. For example, codeNum is set as 3 (TU size 32) in the CTC [2]. One of the advantages by using ue(v) is to keep extensibility for the future. But, the extension of the maximum transform skip size needs a lot of changes to the spec, because transform skip is closely related to some other tools such as BDPCM, ACT, LFNST, and so on.

| if( CtbSizeY > 32 ) |  |
| --- | --- |
| sps_max_luma_transform_size_64_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| sps_log2_transform_skip_max_size_minus2 | ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |

sps_log2_transform_skip_max_size_minus2 specifies the maximum block size used for transform skip, and shall be in the range of 0 to 3, inclusive.

The variable MaxTsSize is set equal to 1  <<  ( sps_log2_transform_skip_max_size_minus2 + 2 ).

Table 1 Comparison of ue(v) and u(2).

| codeNum (TU size) | Bit string by ue(v) | Bit string by u(2) |
| --- | --- | --- |
| 0 (4) | 1 | 0 0 |
| 1 (8) | 0 1 0 | 0 1 |
| 2 (16) | 0 1 1 | 1 0 |
| 3 (32) | 0 0 1 0 0 | 1 1 |

Proposed Method

It is proposed that sps_log2_transform_skip_max_size_minus2 is signalled by u(2) instead of ue(v). The suggested text change on the top of S0152-v5 [3] is only one part as follows.

| if( CtbSizeY > 32 ) |  |
| --- | --- |
| sps_max_luma_transform_size_64_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| sps_log2_transform_skip_max_size_minus2 | u(2)ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |

Conclusion

In this contribution, it was proposed that the maximum transform skip size (sps_log2_transform_skip_max_size_minus2) was signalled by u(2) instead of ue(v). It is recommended that the proposed method is adopted to next VVC Draft and VTM.

This work was conducted under the project "Research and Development of Next generation video transmission and communication technology for ultra-high definition video application under versatile purpose and environment" of the Ministry of Internal Affairs and Communications (MIC), Japan.

References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-R2001, teleconference, Apr. 2020.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

- Y.-K. Wang, “AHG2: Editorial input of a text integration for the May 2020 HLS AHG meeting outcome”, JVET-S0152, teleconference, June. 2020.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).