| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0205 |
| --- | --- |

| Title: | AHG9: Inference value of DMVR/BDOF disabled flag in PH |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

In this contribution, it is proposed that values of DMVR/BDOF disabled flag in PH (ph_bdof/dmvr_disabled_flag) are inferred as 1 (DMVR/BDOF disable) when the picture is not qualified to use bi-prediction. In the previous meeting,

Introduction

In the previous meeting held on Apr. 2020 as a teleconference, conditional signalling of BDOF and DMVR disabled flags in PH were adopted. In the proposal [1], BDOF and DMVR disabled flags (ph_bdof/dmvr_disabled_flag) are signalled only in a picture that is qualified to use bi-prediction, because BDOF and DMVR can be used only with bi-prediction. However, in the current spec [2], the values of ph_bdof/dmvr_disabled_flag are inferred as “BDOF/DMVR enabled” for a picture that is not qualified to use bi-prediction, when sps_bdof/dmvr_enabled_flag is equal to 1 and sps_bdof/dmvr_control_present_in_ph_flag is equal to 0, as shown in Table 1.

Table 1 Inferrence values of ph_bdof/dmvr_disabled_flag.

|  | sps_bdof/dmvr_control_present_in_ph_flag |  |
| --- | --- | --- |
|  | == 1 | == 0 |
| bi-pred can be used | n/a (explicitly signalled) | 1 − sps_bdof/dmvr_enabled_flag |
| bi-pred cannot be used | 1 (disabled) | 1 − sps_bdof/dmvr_enabled_flag |

Proposed Method

It is proposed that values of ph_bdof/dmvr_disabled_flag are inferred as “BDOF/DMVR enabled” for a picture that is not qualified to use bi-prediction, as shown in Table 2.

Table 2 Proposed inferrence values of ph_bdof/dmvr_disabled_flag.

|  | sps_bdof/dmvr_control_present_in_ph_flag |  |
| --- | --- | --- |
|  | == 1 | == 0 |
| bi-pred can be used | n/a (explicitly signalled) | 1 − sps_bdof/dmvr_enabled_flag |
| bi-pred cannot be used | 1 (disabled) | 1 (disabled) |

The suggested text changes on the top of S0152-v5 [3] are as follows.

7.3.2.7	Picture header structure syntax

| presenceFlag = 0 |  |
| --- | --- |
| if( ph_inter_slice_allowed_flag ) { |  |
| ... |  |
| if( sps_mmvd_fullpel_only_flag ) |  |
| ph_mmvd_fullpel_only_flag | u(1) |
| presenceFlag = 0 |  |
| if( !pps_rpl_info_in_ph_flag ) /* This condition is intentionally not merged into the next, to avoid possible intepretation of RplsIdx[ i ] being unspecified. */ |  |
| presenceFlag = 1 |  |
| else if( num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 0 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 ) |  |
| presenceFlag = 1 |  |
| if( presenceFlag ) { |  |
| ph_mvd_l1_zero_flag | u(1) |
| if( sps_bdof_control_present_in_ph_flag ) |  |
| ph_bdof_disabled_flag | u(1) |
| if( sps_dmvr_control_present_in_ph_flag ) |  |
| ph_dmvr_disabled_flag | u(1) |
| } |  |

7.4.3.7	Picture header structure semantics

ph_bdof_disabled_flag equal to 1 specifies that bi-directional optical flow inter prediction based inter bi-prediction is disabled and not used in the slices associated with the PH. ph_bdof_disabled_flag equal to 0 specifies that bi-directional optical flow inter prediction based inter bi-prediction is enabled and may be used in the slices associated with the PH.

When not present, the value of ph_bdof_disabled_flag is inferred as follows:

- If presenceFlag is equal to 1sps_bdof_control_present_in_ph_flag is equal to 0, the value of ph_bdof_disabled_flag is inferred to be equal to 1 − sps_bdof_enabled_flag.

- Otherwise (presenceFlag is equal to 0sps_bdof_control_present_in_ph_flag is equal to 1), the value of ph_bdof_disabled_flag is inferred to be equal to 1.

ph_dmvr_disabled_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is disabled and not used in the slices associated with the PH. ph_dmvr_disabled_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction is enabled and may be used in the slices associated with the PH.

When not present, the value of ph_dmvr_disabled_flag is inferred as follows:

- If presenceFlag is equal to 1sps_dmvr_control_present_in_ph_flag is equal to 0, the value of ph_dmvr_disabled_flag is inferred to be equal to 1 − sps_dmvr_enabled_flag.

- Otherwise (presenceFlag is equal to 0sps_dmvr_control_present_in_ph_flag is equal to 1), the value of ph_dmvr_disabled_flag is inferred to be equal to 1.

Conclusion

In this contribution, it was proposed that values of DMVR/BDOF disabled flag in PH were inferred as 1 (DMVR/BDOF disable) when the picture was not qualified to use bi-prediction. It is recommended that the proposed method is adopted to next VVC Draft and VTM.

This work was conducted under the project "Research and Development of Next generation video transmission and communication technology for ultra-high definition video application under versatile purpose and environment" of the Ministry of Internal Affairs and Communications (MIC), Japan.

References

- Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, W. Chen, and X. Wang, “AHG9: On syntax signaling conditions in picture header”, JVET-R0324, teleconference, Apr. 2020.

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-R2001, teleconference, Apr. 2020.

- Y.-K. Wang, “AHG2: Editorial input of a text integration for the May 2020 HLS AHG meeting outcome”, JVET-S0152, teleconference, June. 2020.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).