| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0206-v3 |
| --- | --- |

| Title: | AHG12: Raster scan order subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mitra Damghanian Du Liu Rickard Sjöberg Martin Pettersson Jack Enhorn Jacob Ström Ruoyang Yu | Email: | mitra.damghanian@ericsson.com du.liu@ericsson.com rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com jack.enhorn@ericsson.com jacob.strom@ericsson.com ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following:

- To add a flag, sps_raster_scan_order_subpics_flag, to the SPS, where sps_raster_scan_order_subpics_flag equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS.

- To derive and not signal the top-left CTU position of the subpictures when sps_raster_scan_order_subpics_flag is equal to 1.

For use cases such as OMAF or 360 video, the proposed changes are claimed to reduce 50% of the bit cost for subpicture layout signalling.

Specification text on top of JVET-S0152-v5 and the software implementation on top of VTM-9.0 are provided with the contribution.

In version 2 of the contribution some bit count examples are added and an example subpicture config file for the software implementation is attached with the contribution.

In version 3 of the contribution, a bit count error in example 1 was corrected.

# Introduction

Subpictures in the current VVC specification are signalled in the SPS using the following syntax:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_subpic_info_present_flag | u(1) |
| if( sps_subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 > 0 ) |  |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && sps_pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| sps_subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && sps_pic_height_max_in_luma_samples > CtbSizeY ) |  |
| sps_subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| sps_subpic_treated_as_pic_flag[ i ] | u(1) |
| sps_loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| sps_subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( sps_subpic_id_mapping_explicitly_signalled_flag ) { |  |
| sps_subpic_id_mapping_present_flag | u(1) |
| if( sps_subpic_id_mapping_present_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

In the current VVC design, four syntax elements sps_subpic_ctu_top_left_x[ i ], sps_subpic_ctu_top_left_y[ i ], sps_subpic_width_minus1[ i ] and sps_subpic_height_minus1[ i ] are signalled for each subpicture except the top-left position for the first subpicture and the width and height for the last subpicture. These four syntax elements are u(v) coded and their bit length is specified by the width and height of the picture as shown in their semantics below:

sps_subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_width_max_in_luma_samples + CtbSizeY  − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

sps_subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

sps_subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_width_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_width_minus1[ i ] is inferred to be equal to ( ( sps_pic_width_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) − sps_subpic_ctu_top_left_x[ i ] − 1.

sps_subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_height_minus1[ i ] is inferred to be equal to ( ( sps_pic_height_max_in_luma_samples + CtbSizeY− 1 )  >>  CtbLog2SizeY ) − sps_subpic_ctu_top_left_y[ i ] − 1.

However, if subpictures are in the raster scan order, signalling the top-left position of the subpictures as well as their width and height is an overkill for signalling the subpicture layout. While it is beneficial to have the flexibility for signalling arbitrary order subpictures, the signalling can be simplified for use cases such as a grid-shaped subpicture layout, OMAF subpicture layout and 360-video subpicture layout.

# Proposal

The following is proposed:

- To add an sps_raster_scan_order_subpics_flag to the SPS, where sps_raster_scan_order_subpics_flag equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS.

- To derive and not signal the top-left CTU position of the subpictures when sps_raster_scan_order_subpics_flag is equal to 1.

Proposed syntax and semantics on top of JVET-S0152-v5 are shown with removed text in red strike through and added text in yellow. A software implementation on top of VTM-9.0 is provided with the contribution. To run the software, VTM-9.0 should first be checked out. Then the files in the attached JVET-S0206-source.zip file should replace the source directory of VTM-9.0. A config file, subpicture_OMAF_24_subpics_24_slices_18_tiles.cfg, is also attached with the contribution.

6.5.1.CTB raster scanning, tile scanning, and subpicture scanning processes

…

When sps_raster_scan_order_subpics_flag is equal to 1, the lists SubpicCtuTopLeftX[ i ] and SubpicCtuTopLeftY[ i ], for i ranging from 0 to sps_num_subpics_minus1, inclusive, specifying the horizontal and vertical position of top left CTU of i-th subpicture in unit of CtbSizeY are derived as follows:

for( i = 0; i < PicSizeInCtbsY; i++ ) AssignedCtu[ i ] = 0
for( i = 0; i <= sps_num_subpics_minus1; i++ ) {
	j = 0
	while( AssignedCtu[ j ] ) j++
	SubpicCtuTopLeftX[ i ] = j % PicWidthInCtbsY
	SubpicCtuTopLeftY[ i ] = j / PicWidthInCtbsY
	for( s = SubpicCtuTopLeftX[ i ]; s <= SubpicCtuTopLeftX[ i ] + sps_subpic_width_minus1[ i ]; s++ ) 
		for( t = SubpicCtuTopLeftY[ i ]; t <= SubpicCtuTopLeftY[ i ] + sps_subpic_height_minus1[ i ]; t++ ) 
			AssignedCtu[ s + t * PicWidthInCtbsY ] = 1
}

…

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 > 0 ) { |  |
| sps_raster_scan_order_subpics_flag | u(1) |
| sps_independent_subpics_flag | u(1) |
| } |  |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && !sps_raster_scan_order_subpics_flag && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && !sps_raster_scan_order_subpics_flag && pic_height_max_in_luma_samples > CtbSizeY ) { |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

sps_raster_scan_order_subpics_flag equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS such that for any two integer values i and j in the range of 0 to sps_num_subpics_minus1, inclusive, with j less than i, the position of the top left CTU of the i-th subpicture follows the position of the top left CTU of the j-th subpicture in raster scan order. sps_raster_scan_order_subpics_flag equal to 0 does not impose such a constraint. When not present, the value of sps_raster_scan_order_subpics_flag is inferred to be equal to 0.

sps_subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_width_max_in_luma_samples + CtbSizeY  − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.  (sps_raster_scan_order_subpics_flag ? SubpicCtuTopLeftX[ i ] : 0).

sps_subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( ( sps_pic_height_max_in_luma_samples + CtbSizeY − 1 )  >>  CtbLog2SizeY ) ) bits. When not present, the value of sps_subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.  (sps_raster_scan_order_subpics_flag ? SubpicCtuTopLeftY[ i ] : 0).

# Bit-count Examples

Here are several examples comparing the bit-counts for signalling the subpicture layout with the current VVC signalling and with the proposed signalling. The bit-cost reduction using the proposed method is about 50% for all subpicture layout examples that can have subpictures in raster scan order. The reason is that the proposed method skips signalling the top-left CTU position for each subpicture (two out of the four signalled syntax elements, with the bit-lengths exactly equal to the other two signalled syntax elements) which comprises half of the bit-cost for subpicture layout signalling. At the same time, we add one bit to the signalling by adding the proposed flag.

## Example 1

Figure 1 is illustrating a subpicture layout in one OMAF example with picture size 1920 × 4608 and 24 subpictures. In this layout, the size of the larger subpictures is 768 × 768 and the size of the smaller subpictures is 384 × 384 and the CTU size is 128. Table 1 is comparing the bit-counts for signalling the subpicture layout with the current VVC signalling and with the proposed signalling. About 50% of the bit-cost is reduced using the proposed method.

An config file, subpicture_OMAF_24_subpics_24_slices_18_tiles.cfg, for this example is attached with the contribution.

Figure 1 OMAF example subpicture layout with 24 subpictures (from Figure D.10 in OMAF 2nd edition)

Table 1 Example bit-counts for signalling the subpicture layout of Figure 1, current VVC vs the proposed method

| Syntax element | Codeword bit length | Current VVC |  | Proposed method |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Times signalled | Bit-count | Times signalled | Bit-count |
| sps_subpic_ctu_top_left_x[ i ] | 4 | 23 | 92 | 0 | 0 |
| sps_subpic_ctu_top_left_y[ i ] | 6 | 23 | 138 | 0 | 0 |
| sps_subpic_width_minus1[ i ] | 4 | 23 | 92 | 23 | 92 |
| sps_subpic_height_minus1[ i ] | 6 | 23 | 138 | 23 | 138 |
| Total bit-count |  |  | 460 |  | 230 |

## Example 2

Figure 2 is illustrating a cubemap projection subpicture layout with picture size 3072 × 2048 and 6 subpictures each having the size 1024 × 1024 and CTU size of 128. Table 2 is comparing the bit-counts for signalling the subpicture layout with the current VVC signalling and with the proposed signalling. As in the other example, 50% of the bit-cost is reduced using the proposed method.

Figure 2 Cubemap projection subpicture layout with 6 subpictures.

Table 2 Example bit-counts for signalling the subpicture layout of Figure 2, current VVC vs the proposed method

| Syntax element | Codeword bit length | Current VVC |  | Proposed method |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Times signalled | Bit-count | Times signalled | Bit-count |
| sps_subpic_ctu_top_left_x[ i ] | 5 | 5 | 25 | 0 | 0 |
| sps_subpic_ctu_top_left_y[ i ] | 4 | 5 | 20 | 0 | 0 |
| sps_subpic_width_minus1[ i ] | 5 | 5 | 25 | 5 | 25 |
| sps_subpic_height_minus1[ i ] | 4 | 5 | 20 | 5 | 20 |
| Total bit-count |  |  | 90 |  | 45 |

## Example 3

Figure 3 is illustrating a subpicture layout in an OMAF example with picture size 2560 × 3200 and 6 subpictures. In this layout, the size of larger subpictures is 1280 × 1280, the size of the smaller subpictures is 1280 × 640 and the CTU size is 128. Table 3 is comparing the bit-counts for signalling the subpicture layout with the current VVC signalling and with the proposed signalling. About 50% of the bit-cost is reduced using the proposed method.

Figure 3 One OMAF example subpicture layout with 6 subpictures (from Figure D.7 in OMAF 2nd edition)

Table 3 Example bit-counts for signalling the subpicture layout of Figure 3, current VVC vs the proposed method

| Syntax element | Codeword bit length | Current VVC |  | Proposed method |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Times signalled | Bit-count | Times signalled | Bit-count |
| sps_subpic_ctu_top_left_x[ i ] | 5 | 5 | 25 | 0 | 0 |
| sps_subpic_ctu_top_left_y[ i ] | 5 | 5 | 25 | 0 | 0 |
| sps_subpic_width_minus1[ i ] | 5 | 5 | 25 | 5 | 25 |
| sps_subpic_height_minus1[ i ] | 5 | 5 | 25 | 5 | 25 |
| Total bit-count |  |  | 100 |  | 50 |

## Example 4

Figure 4 is illustrating a subpicture layout in an OMAF example with picture size 6144 × 3072 and 12 × 6 subpictures each having the size of 512 × 512, and the CTU size 128. Table 4 is comparing the bit-counts for signalling the subpicture layout with the current VVC signalling and with the proposed signalling. About 50% of the bit-cost is reduced using the proposed method.

Figure 4 One OMAF example subpicture layout with 72 subpictures (from Figure D.12 in OMAF 2nd edition)

Table 4 Example bit-counts for signalling the subpicture layout of Figure 4, current VVC vs the proposed method

| Syntax element | Codeword bit length | Current VVC |  | Proposed method |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Times signalled | Bit-count | Times signalled | Bit-count |
| sps_subpic_ctu_top_left_x[ i ] | 6 | 71 | 426 | 0 | 0 |
| sps_subpic_ctu_top_left_y[ i ] | 5 | 71 | 355 | 0 | 0 |
| sps_subpic_width_minus1[ i ] | 6 | 71 | 426 | 71 | 426 |
| sps_subpic_height_minus1[ i ] | 5 | 71 | 355 | 71 | 355 |
| Total bit-count |  |  | 1562 |  | 781 |

## Example 5

Figure 5 is illustrating a subpicture layout in an OMAF example with picture size 3840 × 2304 and 16 subpictures with sizes 768 × 2048, 384 × 1024, 768 × 256 and 384 × 128 according to the layout and the CTU size equal to 128. Table 5 is comparing the bit-counts for signalling the subpicture layout with the current VVC signalling and with the proposed signalling. About 50% of the bit-cost is reduced using the proposed method.

Figure 5 One OMAF example subpicture layout with 16 subpictures (from Figure D.9 in OMAF 2nd edition)

Table 5 Example bit-counts for signalling the subpicture layout of Figure 5, current VVC vs the proposed method

| Syntax element | Codeword bit length | Current VVC |  | Proposed method |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Times signalled | Bit-count | Times signalled | Bit-count |
| sps_subpic_ctu_top_left_x[ i ] | 5 | 15 | 75 | 0 | 0 |
| sps_subpic_ctu_top_left_y[ i ] | 5 | 15 | 75 | 0 | 0 |
| sps_subpic_width_minus1[ i ] | 5 | 15 | 75 | 15 | 75 |
| sps_subpic_height_minus1[ i ] | 5 | 15 | 75 | 15 | 75 |
| Total bit-count |  |  | 300 |  | 150 |

## Bit-counts for Different CTU Sizes

Table 6 is summarizing bit-counts for signalling various subpicture layouts as in Figure 1 to Figure 5 for CTU sizes equal to 128, 64 and 32. The bit-count in each case is 50% less using the proposed method compared to the current VVC.

Table 6 Example bit-counts for signalling some subpicture layouts for some CTU sizes, current VVC vs the proposed method

| Subpicture Layout | Total bit-count |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CTU size 128 |  | CTU size 64 |  | CTU size 32 |  |
|  | Current VVC | Proposed method | Current VVC | Proposed method | Current VVC | Proposed method |
| Figure 1 | 506 | 253 | 552 | 251 | 644 | 322 |
| Figure 2 | 90 | 45 | 120 | 60 | 140 | 70 |
| Figure 3 | 100 | 50 | 120 | 60 | 140 | 70 |
| Figure 4 | 1562 | 781 | 1846 | 923 | 2130 | 1065 |
| Figure 5 | 300 | 150 | 360 | 180 | 420 | 210 |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).