| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0212-v1 |
| --- | --- |

| Title: | AHG9: Modifications on VPS OLS DPB related parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu Martin Pettersson Rickard Sjöberg Mitra Damghanian Jack Enhorn Jacob Ström Du Liu | Email: | ruoyang.yu@ericsson.com martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com jack.enhorn@ericsson.com jacob.strom@ericsson.com du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes the following modifications to the VPS OLS DPB related parameters in the VVC draft JVET-S0152-v5:

- Change to code the vps_ols_dpb_pic_width[ i ] and vps_ols_dpb_pic_height[ i ] in units of 8 luma samples. The decoded picture width and height in a CLVS are noted to be constrained to be an integer multiple of Max( 8, MinCbSizeY ), where the MinCbSizeY is the minimum coding block size in luma samples. The modification is asserted to be straightforward and help reduce the size of the VPS in VVC.

- Mandate the vps_ols_dpb_bitdepth_minus8[ i ] to be in the range of 0 to 8, inclusive. The vps_ols_dpb_bitdepth_minus8[ i ] is reported to specify the greatest allowed value of sps_bit_depth_minus8 for all SPSs that are referred to by CLVSs in the CVS for the i-th multi-layer OLS. It is asserted that decoders may allocate memory based on the values of vps_ols_dpb_bitdepth_minus8[ i ] for decoding the i-th multi-layer OLS. It is asserted to be helpful to clearly define the range of vps_ols_dpb_bitdepth_minus8[ i ].

# Proposal

## Proposal 1: Code the syntax elements vps_ols_dpb_pic_width[ i ] and vps_ols_dpb_pic_height[ i ] in units of 8 luma samples.

In the VVC draft JVET-S0152-v5, the decoded picture width or height in a CLVS are noted to be constrained to be an integer multiple of Max( 8, MinCbSizeY ), where the MinCbSizeY is the minimum coding block size in luma samples. Such a constraint can be found under syntax elements sps_pic_width_max_in_luma_samples, sps_pic_height_max_in_luma_samples, pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples.

When there is one or more multi-layer OLS, vps_ols_dpb_pic_width[ i ] and vps_ols_dpb_pic_height[ i ] are present. Those syntax elements specify the width and height in units of luma samples of each picture storage buffer for the i-th multi-layer OLS.

Considering the picture width or height constraint, it is proposed to code the vps_ols_dpb_pic_width[ i ] and vps_ols_dpb_pic_height[ i ] in units of 8 luma samples instead. The modification is asserted to help reducing the size of the VPS in the VVC. In terms of the spec text changes, the modification is asserted be straighforward.

The proposed modifications on top of the VVC draft JVET-S0152-v5 are shown below with added text marked in yellow.

7.4.3.2	Video parameter set RBSP semantics

vps_ols_dpb_pic_width[ i ] specifies the width, in units of luma samples divided by 8, of each picture storage buffer for the i-th multi-layer OLS.

vps_ols_dpb_pic_height[ i ] specifies the height, in units of luma samples divided by 8, of each picture storage buffer for the i-th multi-layer OLS.

…

7.4.3.3	Sequence parameter set RBSP semantics

sps_pic_width_max_in_luma_samples specifies the maximum width, in units of luma samples, of each decoded picture referring to the SPS. sps_pic_width_max_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ).

When sps_video_parameter_set_id is greater than 0 and the SPS is referenced by a layer that is included in the i-th multi-layer OLS specified by the VPS for any i in the range of 0 to NumMultiLayerOlss − 1, inclusive, it is a requirement of bitstream conformance that the value of sps_pic_width_max_in_luma_samples shall be less than or equal to the value of ( vps_ols_dpb_pic_width[ i ] * 8 ).

sps_pic_height_max_in_luma_samples specifies the maximum height, in units of luma samples, of each decoded picture referring to the SPS. sps_pic_height_max_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ).

When sps_video_parameter_set_id is greater than 0 and the SPS is referenced by a layer that is included in the i-th multi-layer OLS specified by the VPS for any i in the range of 0 to NumMultiLayerOlss − 1, inclusive, it is a requirement of bitstream conformance that the value of sps_pic_height_max_in_luma_samples shall be less than or equal to the value of ( vps_ols_dpb_pic_height[ i ] * 8 ).

…

A.4.1	General tier and level limits

The following is specified for expressing the constraints in this annex:

…

  - Otherwise (NumLayersInOls[ TargetOlsIdx ] is greater than 1), PicWidthMaxInSamplesY is set equal to vps_ols_dpb_pic_width[ MultiLayerOlsIdx[ TargetOlsIdx ] ] * 8, PicHeightMaxInSamplesY is set equal to vps_ols_dpb_pic_height[ MultiLayerOlsIdx[ TargetOlsIdx ] ] * 8, PicSizeMaxInSamplesY is set equal to PicWidthMaxInSamplesY * PicHeightMaxInSamplesY, and the applicable dpb_parameters( ) syntax structure is identified by vps_ols_dpb_params_idx[ MultiLayerOlsIdx[ TargetOlsIdx ] ] found in the VPS.

…

C.1	General

This annex specifies the hypothetical reference decoder (HRD) and its use to check bitstream and decoder conformance.

…

–	Otherwise (NumLayersInOls[ TargetOlsIdx ] is greater than 1), the dpb_parameters( ) is identified by vps_ols_dpb_params_idx[ MultiLayerOlsIdx[ TargetOlsIdx ] ] found in the VPS, and the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, and MaxBitDepthMinus8 are set equal to vps_ols_dpb_pic_width[ MultiLayerOlsIdx[ TargetOlsIdx ] ] * 8, vps_ols_dpb_pic_height[ MultiLayerOlsIdx[ TargetOlsIdx ] ] * 8, vps_ols_dpb_chroma_format[ MultiLayerOlsIdx[ TargetOlsIdx ] ], and vps_ols_dpb_bitdepth_minus8[ MultiLayerOlsIdx[ TargetOlsIdx ] ], respectively, found in the VPS.

## Proposal 2: Mandate vps_ols_dpb_bitdepth_minus8[ i ] to be in the range of 0 to 8, inclusive.

In the VVC draft JVET-S0152-v5, the vps_ols_dpb_bitdepth_minus8[ i ] is reported to specify the greatest allowed value of sps_bit_depth_minus8 for all SPSs that are referred to by CLVSs in the CVS for the i-th multi-layer OLS. The vps_ols_dpb_bitdepth_minus8[ i ] is noted to be coded using ue(v).

It is asserted that decoders may allocate memory based on the values of vps_ols_dpb_bitdepth_minus8[ i ] for decoding the i-th multi-layer OLS. It it also noted that this is captured in a NOTE as follows:

“

NOTE 2 – For decoding the i-th multi-layer OLS, the deoder can safely allocate memory for the DPB according to the values of the syntax elements vps_ols_dpb_pic_width[ i ], vps_ols_dpb_pic_height[ i ], vps_ols_dpb_chroma_format[ i ], and vps_ols_dpb_bitdepth_minus8[ i ].

”

It is asserted to be helpful to clearly define the range of vps_ols_dpb_bitdepth_minus8[ i ] in the spec text.

The proposed modifications on top of the VVC draft JVET-S0152-v5 are shown below with added text marked in yellow.

7.4.3.2	Video parameter set RBSP semantics

vps_ols_dpb_bitdepth_minus8[ i ] specifies the greatest allowed value of sps_bit_depth_minus8 for all SPSs that are referred to by CLVSs in the CVS for the i-th multi-layer OLS. When present, the value of vps_ols_dpb_bitdepth_minus8[ i ] shall be in the range of 0 to 8, inclusive.

…

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).