| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0214 |
| --- | --- |

| Title: | AHG8/AHG9: Updates on the implementation of multi-layer decoding and output independent layer arrangement in VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Karim El Assal Alexandre Gabriel Anna van Buerenplein 1 2595 DA Den Haag The Netherlands | Tel: | emmanuel.thomas@tno.nl karim.elassal@tno.nl alexandre.gabriel@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution provides on update the software implementation in the VTM project realising the merging and the decoding of multiple independent layers into a single output decoded picture. The spatial relationship between these independent layers is described by expressing the top, right, bottom and left neighbour layer relative to each layer. This information is signalled in the bitstream (see JVET-S0213 for normative aspect of the signalling). The contribution further explains the different steps implemented in the VTM decoder to construct the final output decoded picture.

# Introduction

This contribution provides the details on the software implementation in VTM enabling:

- The multi-layer decoding support

- The generation of a single output picture based on the independent output layers of the same output layer set

The contribution JVET-S0214 provides the motivation and the normative signalling along with the specification changes.

The source code is available here: https://vcgit.hhi.fraunhofer.de/edrthomas/VVCSoftware_VTM_multi-layer .

For information, this implementation was presented in the recent ACM MMSys 2020 conference whose presentation video can be found online [1].

The revision r1 of this contribution adds the bitstream example section 4.3.1.

The revision r2 adds an analysis in terms of changes needed to VTM 9.1 to support this functionality.

# Single output layer set with independent layers

Independent layers can be created by encoding video sequences separately using the VVC encoder. Then a merge operation, creates a single bitstream out of the input bitstreams, using the stream merger application in VTM. The output bitstream contains one layer per input bitstream. In addition, the SEI message signalling the spatial relationship between the layer is inserted in the output bitstream. The decoder parses the input bitstream, the different layers and the SEI message for the spatial relationship and outputs a single output picture out of the decoding of the different layers as shown on Figure 1.

Figure 1 - Encoding, merging and decoding of multiple independent layers

# Processing steps

In the current implementation, all the relations between the output layers must be explicitly signalled. That is, adjacent output layers must always have matching identifiers. As well as this it is assumed that all output layers have the same resolution – this made the reconstruction logic easier to implement in a first phase but will be improved in a further iteration to allow more flexible signalling.

When the values of two different layers’ boundary_identifier_north  boundary_identifier_south or boundary_identifier_east  boundary_identifier_west are equal then the decoded picture of the first and second layer are adjacent in the output picture and they share a common boundary respectively at the north/south or east/west boundary. The image below has an example of how layer alignment would look.

Figure 3 - How layers relate to each other spatially based with examples of their identifiers

Layers vertically adjacent (north/south) such as Layer 1 and 2 are aligned on their west border – the SW pixel of Layer 2 is adjacent to the NW pixel of Layer 1. Layers horizontally adjacent (east/west) such as Layer 3 and 1 are aligned on their north border – the NE pixel of Layer 3 is adjacent to the NW pixel of Layer 1.

# Modifications to VTM code base

To make the current implementation VTM 6.1 was used. There were two applications altered to be able to enable the functionality: the StreamMergeApp and the DecoderApp.

## StreamMergeApp

The StreamMergeApp, which merges pre-encoded bitstreams, was modified to write the boundary identifiers in the SEI message with the syntax described in the previous section. These SEI message will then be used to know where to position the layers during the composition. As of now, all the relations should be placed explicitly and 0 indicates that there is no connection with another layer on that boundary. N, E, S and W indicate the north, east, south and west boundaries.

Figure 4 – Relation of NESW markers and layers

The usage is as follows for 2 layers:

StreamMergeApp.exe layer1 layer2 output.vvc --jigsaw N E S W N E S W

To join layer1, layer2, layer3 and layer4 into output the command would be the one indicated below:

Figure 5 - Layers with identifiers

StreamMergeApp.exe layer1 layer2 layer3 layer4 output.vvc --jigsaw 0 2 5 0 0 0 9 2 5 7 0 0 9 0 0 7

The figure below shows how two bitstreams have been merged by taking a NAL units from each input and interleaving them in the output bitstream. The SEI message is inserted as well. The output file is a bitstream that can then be decoded by the DecodeApp.

Figure 6 – NAL units are interleaved in the output bitstream

## DecodeApp

In the decoding process, the DecodeApp now supports the creation of multiple decoding contexts as new layers arrive. The read() function reads each of the headers of the NAL unit as the units appear in the bitstream. When a NAL unit with a new layer_id appears a new decoding context is instantiated. When the SEI message appears the layout configuration is read and stored in the respective decoding context's memory. Since every NAL unit has a layer_id the decoder can know which decoding context must receive each NAL unit and sends it accordingly to the correct one.

Figure 7 - NAL units are sent to their respective decoding contexts based on layer_id

It is possible that the layers will not necessarily be output at the same moment. To solve this, after a frame from a layer is decoded it is placed in its decoding context’s decoded picture list. After decoding a frame, the decoding contexts (i.e. layers) are checked to see if they all have at least one frame to output. If so, the POC numbers are used for verification and if they match, their picture buffers are concatenated according to the positioning information provided by the SEI message and the result is sent to an overarching output buffer.

A revision of the Hypothetical Reference Decoder is necessary where multiple decoding contexts run in parallel and an overarching decoded picture buffer is filled every time all the other decoded picture buffers have a picture with the same POC ready to be output.

Figure 8 - New HRD scheme

To decode the content the -p flag is used to indicate which layer_id needs to be decoded.

## Decoding output example

Without explicit use of the -p argument as input, all the independent layers implicitly selected and decoded.

The following is an example command decoding all the independent layers:

DecoderAppStatic -b test/out.vvc -o out.yuv

### Example bitstreams

The example provided with the contribution are listed in Table 2. The contribution JVET-S0213 [2] provides the bitstreams corresponding to each example.

Table 2 - List of examples

| # | Composed picture | GOP structure | FPS | Bit depth | Number of frames |
| --- | --- | --- | --- | --- | --- |
| 1 |  | intra | same in each layer | same in each layer | same in each layer |
| 2 |  | lowdelay_P | same in each layer | same in each layer | same in each layer |
| 3 |  | randomaccess | same in each layer | same in each layer | same in each layer |
| 4 |  | intra | same in each layer | same in each layer | same in each layer |
| 5 |  | intra | same in each layer | same in each layer | same in each layer |
| 6 |  | intra | same in each layer | same in each layer | same in each layer |
| 7 |  | lowdelay_P | different | same in each layer | different |

#### Example 1

File name: BasketballPass_BlowingBubbles_left_right_intra.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_intra_vtm.cfg | 0 1 0 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_intra_vtm.cfg | 0 0 0 1 |

Signaled connected map:

| Layer 0 1 | Layer 1 1 |
| --- | --- |

Decoding output:

Resolution

832 x 240

Screenshot

#### Example 2

File name: BasketballPass_BlowingBubbles_left_right_lowdelay_P.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_lowdelay_P_vtm.cfg | 0 1 0 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_lowdelay_P_vtm.cfg | 0 0 0 1 |

Signaled connected map:

| Layer 0 1 | Layer 1 1 |
| --- | --- |

Decoding output:

Resolution

832 x 240

Screenshot

#### Example 3

File name: BasketballPass_BlowingBubbles_left_right_randomaccess.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_randomaccess_vtm.cfg | 0 1 0 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_randomaccess_vtm.cfg | 0 0 0 1 |

Signaled connected map:

| Layer 0 1 | Layer 1 1 |
| --- | --- |

Decoding output:

Resolution

832 x 240

Screenshot

#### Example 4

File name: BasketballPass_BlowingBubbles_BasketballDrill_left_right_bottom_intra.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_intra_vtm.cfg | 0 1 2 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_intra_vtm.cfg | 0 0 0 1 |
| 2 | BasketballDrill | 832x480 | encoder_intra_vtm.cfg | 2 0 0 0 |

Signaled connected map:

| Layer 0 1 2 | Layer 1 1 |
| --- | --- |
| 2 Layer 2 |  |

Decoding output:

Resolution

832 x 720

Screenshot

#### Example 5

File name: BasketballPass_BlowingBubbles_BasketballDrill_left_right_bottom_intra.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_intra_vtm.cfg | 0 1 0 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_intra_vtm.cfg | 0 0 2 1 |
| 2 | BasketballPass | 416 x 240 | encoder_intra_vtm.cfg | 2 0 0 0 |

Signaled connected map:

| Layer 0 1 | Layer 1 1 2 |
| --- | --- |
|  | 2 Layer 2 |

Decoding output:

Resolution

832 x 480

Screenshot

#### Example 6

File name: BasketballPass_BlowingBubbles_BasketballDrill_left_right_bottom_intra.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- |
| 0 | BasketballPass | 416 x 240 | encoder_intra_vtm.cfg | 0 1 2 0 |
| 1 | BlowingBubbles | 416 x 240 | encoder_intra_vtm.cfg | 2 0 0 0 |
| 2 | BasketballDrill | 832 x 480 | encoder_intra_vtm.cfg | 0 0 0 1 |

Signaled connected map:

| Layer 0 1 2 | Layer 2 1 |
| --- | --- |
| 2 Layer 1 |  |

Decoding output:

Resolution

1248 x 480

Screenshot

#### Example 7

File name: BQSquare_RaceHorses_top_bottom_lowdelay_P.vvc

Encoded layers:

| Layer | Sequence name | Resolution | Nb frames | Encoding configuration | Boundary identifiers (N, E, S, W) |
| --- | --- | --- | --- | --- | --- |
| 0 | BQSquare | 416 x 240 | 600 | encoder_lowdelay_P_vtm.cfg | 0 0 1 0 |
| 1 | RaceHorses | 416 x 240 | 300 | encoder_lowdelay_P_vtm.cfg | 1 0 0 0 |

Signaled connected map:

Layer 0 1

1 Layer 1

Decoding output:

Resolution

416 x 480

Screenshot

## Summary of relevant edited files based on VTM 6.1

Below is a short summary of the main files edited:

| To support merging | Reason |
| --- | --- |
| source/App/StreamMergeApp/StreamMergeMain.cpp | Adds documentation about the --jigsaw command line option and how to use it. |
| source/App/StreamMergeApp/StreamMergeAppCfg.cpp source/App/StreamMergeApp/StreamMergeAppCfg.h | Reads the layout configuration from the command line options and stores the configuration per layer. |
| source/App/StreamMergeApp/StreamMergeApp.cpp | Sets the layout configuration in the SEI message during merging. |

| To support decoding | Reason |
| --- | --- |
| source/App/DecoderApp/DecApp.h | Creation of new decoding instances when necessary as well as other related operations. |
| source/Lib/DecoderLib/VLCReader.cpp source/Lib/EncoderLib/VLCWriter.cpp | Writing/reading SEI message |
| source/Lib/DecoderLib/DecLib.cpp source/Lib/DecoderLib/DecLib.h | Allows reading the layout configuration in the NAL unit header |
| source/Lib/CommonLib/Slice.h | Added support for layout parameter support |

Source code is available here: https://vcgit.hhi.fraunhofer.de/edrthomas/VVCSoftware_VTM_multi-layer

Proponents are willing to provide support for best integrating the contributed software into the project in coordination with VTM software coordinators.

## Expected changes for implementation in VTM 9.1

In VTM 9.1, it is our understanding that the following functionalities are available:

- VTM 9.1 has a configuration file for multi layer which defines the present layers and the ols to populate the VPS parameter values.

- VTM 9.1 has SEI configuration files to set the SEI parameters values in the bitstream.

- VTM 9.1 decoder has an input option to specify the target output layer index to decode or the layer ids to be decoded.

- VTM 9.1 decoder It seems each layer is decoded into a separate yuv file.

Therefore, merging the submitted contribution into VTM 9.1 would mean a limited impact as follows:

- Checking that StreamMergeApp still works in VTM 9.1 with the added SEI message.

- Modify the VTM decoder to parse the SEI message and output the combined picture instead of the per layer picture output when the SEI message is present in bitstream.

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Thomas, Emmanuel, Alexandre Gabriel, and Karim El Assal. "VVC bitstream extraction and merging operations for multi-stream media applications." Proceedings of the 11th ACM Multimedia Systems Conference. 2020. https://www.youtube.com/watch?v=16llRw2cSTI

- JVET-S0213, AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers with example bitstreams, Emmanuel Thomas, 22 June – 1 July 2020