| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0215-v1 |
| --- | --- |

| Title: | AHG2: On residual coding syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1-408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution proposes three aspects.

The first aspect is related to coeff_sign_flag. It is asserted that, in VVC draft 9, the conditions to check if the coeff_sign_flag is context coded or bypass coded do not reflect the intention of the design. A bug report (#855) was submitted regarding this issue. In the proposed method, the necessary conditions to specify if coeff_sign_flag is context coded or by-pass coded is derived based on the variable lastScanPosPass1 instead of RemCcbs. It is asserted that the proposed solution is aligned with the design intention and with the VTM software.

The second and third aspects of this contribution propose semantic bug fix of sig_coeff_flag and sb_coded_flag, respectively to align with the VTM software.

# Aspect 1: On the context/bypass coding coeff_sign_flag

## Problem statement

In transform-skip residual coding (TSRC), the transform coefficients of each coefficient group (CG) is coded in three coding passes. The first two passes are context coded and the third coding pass is by-pass coded. Following is the pseudo code of TSRC process.

lastScanPosPass1 = −1

for( n = 0; n  <=  numSbCoeff − 1  &&  RemCcbs  >=  4; n++ )

decode sig_coeff_flag; RemCcbs --;

decode coeff_sign_flag; RemCcbs --;

decode abs_level_gtx_flag[0] ; RemCcbs --;

decode par_level_flag; RemCcbs --;

lastScanPosPass1  = n;

lastScanPosPass2 = −1

for( n = 0; n  <=  numSbCoeff − 1  &&  RemCcbs  >=  4; n++ )

decode abs_level_gtx_flag[1] ; RemCcbs --;

decode abs_level_gtx_flag[2] ; RemCcbs --;

decode abs_level_gtx_flag[3] ; RemCcbs --;

decode abs_level_gtx_flag[4] ; RemCcbs --;

lastScanPosPass2 = n

for( n = 0; n  <=  numSbCoeff − 1; n++ )

decode abs_remainder

decode coeff_sign_flag

In TSRC, the coeff_sign_flag can be either context coded or by-pass coded based on the value of the remaining number of available context coded bins (RemCcbs). In VVC specification, the conditions to check if the coeff_sign_flag is context coded or bypass coded are defined in the Table 131 of the specification document as follows.

- If the following condition is true, coeff_sign_flag is bypass coded.

transform_skip_flag[ x0 ][ y0 ][ cIdx ] = = 0  | |  RemCcbs = = 0  | |  sh_ts_residual_coding_disabled_flag

- If the following condition is true, coeff_sign_flag is context coded.

transform_skip_flag[ x0 ][ y0 ][ cIdx ] = = 1 && RemCcbs >= 0  &&  !sh_ts_residual_coding_disabled_flag

It is observed that the above conditions are not aligned with the TSRC syntax table and the VTM software. For instance, assume following scenario:

- sh_ts_residual_coding_disabled_flag  == 0, and

- transform_skip_flag == 1, and

- the value of the variable RemCcbs at the beginning of a CG is equal to 3.

In this example, according to the of the VVC specification, the second condition is true, therefore, the coeff_sign_flag is context coded, however, in the VTM software (also it is intended), coeff_sign_flag is by-pass coded.

A bug report (#855) was submitted regarding this issue. In the bug report (#855), three methods were discussed (including the fix described in this contribution). As per proponent’s opinion, the two other methods still do not align with VTM software (specifically in case of above example) and the method proposed in this contribution aligned with VTM software for all corner cases including the example given above.

## Proposed fix

In the proposed method, the necessary conditions to specify if a coeff_sign_flag is whether context coded or by-pass coded is derived based on the variable lastScanPosPass1 instead of RemCcbs. In residual coding syntax table, the lastScanPosPass1 defines the position of the last scanning coefficient after the first pass. The proposed method aligns with the design intention and VTM software. In the proposed method, the conditions are defined as follows.

If the following condition is true, coeff_sign_flag is bypass coded.

transform_skip_flag[ x0 ][ y0 ][ cIdx ] = = 0  | |  n > lastScanPosPass1 RemCcbs = = 0  | |  sh_ts_residual_coding_disabled_flag

If the following condition is true, coeff_sign_flag is context coded.

transform_skip_flag[ x0 ][ y0 ][ cIdx ] = = 1 && n <= lastScanPosPass1 RemCcbs >= 0  &&  !sh_ts_residual_coding_disabled_flag

In addition to the modify the conditions, it is also required to define the variable “ lastScanPosPass1 = n” at the beginning of the first pass loop instead of end of the loop, as shown in Table 1.

Table 1: Changes of the residual coding syntax table of the proposed method

| /* First scan pass */ |  |
| --- | --- |
| inferSbSigCoeffFlag = 1 |  |
| lastScanPosPass1 = −1 |  |
| for( n = 0; n <= numSbCoeff − 1 && RemCcbs >= 4; n++ ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| lastScanPosPass1 = n |  |
| if( sb_coded_flag[ xS ][ yS ] && ( n != numSbCoeff − 1 ¦ ¦ !inferSbSigCoeffFlag ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| RemCcbs− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbSigCoeffFlag = 0 |  |
| } |  |
| CoeffSignLevel[ xC ][ yC ] = 0 |  |
| if( sig_coeff_flag[ xC ][ yC ] { |  |
| coeff_sign_flag[ n ] | ae(v) |
| RemCcbs− − |  |
| CoeffSignLevel[ xC ][ yC ] = ( coeff_sign_flag[ n ] > 0 ? −1 : 1 ) |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| RemCcbs− − |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| RemCcbs− − |  |
| } |  |
| } |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] |  |
| lastScanPosPass1 = n |  |
| } |  |

# Aspect 2: On the semantic of the sig_coeff_flag

## Problem statement

In VVC draft 9, when sig_coeff_flag[ xC ][ yC ] is not present, it is inferred as follows:

- If ( xC, yC ) is the last significant location ( LastSignificantCoeffX, LastSignificantCoeffY ) in scan order or all of the following conditions are true, sig_coeff_flag[ xC ][ yC ] is inferred to be equal to 1:

- ( xC & ( (1 << log2SbW ) − 1 ), yC & ( (1 << log2SbH ) − 1 ) ) is equal to ( 0, 0 ).

- inferSbDcSigCoeffFlag is equal to 1.

- sb_coded_flag[ xS ][ yS ] is equal to 1.

- Otherwise, sig_coeff_flag[ xC ][ yC ] is inferred to be equal to 0.

The above inference rule is valid in case of regular residual coding, however, in case of TSRC, the above inference rule does not reflect the design intention and does not match with VTM software.

## Proposed fix

In the proposed method, the inference value of sig_coeff_flag[ xC ][ yC ] is aligned with the VTM software and derived based on the selected residual coding method. Following is the proposed specification changes.

sig_coeff_flag[ xC ][ yC ] specifies for the transform coefficient location ( xC, yC ) within the current transform block whether the corresponding transform coefficient level at the location ( xC, yC ) is non-zero as follows:

- If sig_coeff_flag[ xC ][ yC ] is equal to 0, the transform coefficient level at the location ( xC, yC ) is set equal to 0.

- Otherwise (sig_coeff_flag[ xC ][ yC ] is equal to 1), the transform coefficient level at the location ( xC, yC ) has a non-zero value.

When sig_coeff_flag[ xC ][ yC ] is not present, it is inferred as follows:

- If transform_skip_flag[ x0 ][ y0 ][ cIdx ]  is equal to 0 or sh_ts_residual_coding_disabled_flag is equal to 1, following applies.

  - If ( xC, yC ) is the last significant location ( LastSignificantCoeffX, LastSignificantCoeffY ) in scan order or all of the following conditions are true, sig_coeff_flag[ xC ][ yC ] is inferred to be equal to 1:

    - ( xC & ( (1 << log2SbW ) − 1 ), yC & ( (1 << log2SbH ) − 1 ) ) is equal to ( 0, 0 ).

    - inferSbDcSigCoeffFlag is equal to 1.

    - sb_coded_flag[ xS ][ yS ] is equal to 1.

  - Otherwise, sig_coeff_flag[ xC ][ yC ] is inferred to be equal to 0.

- Otherwise, following applies.

  - If all of the following conditions are true, sig_coeff_flag[ xC ][ yC ] is inferred to be equal to 1:

    - ( xC & ( (1 << log2SbW ) − 1 ), yC & ( (1 << log2SbH ) − 1 ) ) is equal to ( (1 << log2SbW ) − 1 , (1 << log2SbH ) − 1 ) ).

    - inferSbSigCoeffFlag is equal to 1.

    - sb_coded_flag[ xS ][ yS ] is equal to 1.

  - Otherwise, sig_coeff_flag[ xC ][ yC ] is inferred to be equal to 0.

# Aspect 3: On the semantic of the sb_coded_flag

In VVC draft 9, if sb_coded_flag[ xS ][ yS ] is equal to 1, the following applies:

- If ( xS, yS ) is equal to ( 0, 0 ) and ( LastSignificantCoeffX, LastSignificantCoeffY ) is not equal to ( 0, 0 ), at least one of the 16 sig_coeff_flag syntax elements is present for the subblock at location ( xS, yS ).

- Otherwise, at least one of the 16 transform coefficient levels of the subblock at location ( xS, yS ) has a non-zero value.

The above definition is only valid for regular residual coding and if the number of remaining context coded bins is greater than 0 (i.e. context coded bins are still available) to code the CG.

It is proposed to align the semantic of the sb_coded_flag with VTM software as follows:

sb_coded_flag[ xS ][ yS ] specifies the following for the subblock at location ( xS, yS ) within the current transform block, where a subblock is an array of transform coefficient levels:

- If sb_coded_flag[ xS ][ yS ] is equal to 0, the 16 transform coefficient levels of the subblock at location ( xS, yS ) are inferred to be equal to 0.

- Otherwise (sb_coded_flag[ xS ][ yS ] is equal to 1), the following applies:

  - If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 0 or sh_ts_residual_coding_disabled_flag is equal to 1, following applies:

    - If ( xS, yS ) is equal to ( 0, 0 ) and ( LastSignificantCoeffX, LastSignificantCoeffY ) is not equal to ( 0, 0 ), at least one of the 16 sig_coeff_flag syntax elements is may be present for the subblock at location ( xS, yS ).

    - Otherwise, at least one of the 16 transform coefficient levels of the subblock at location ( xS, yS ) has a non-zero value.

  - Otherwise, at least one of the 16 transform coefficient levels of the subblock at location ( xS, yS ) has a non-zero value.

# Conclusion

This contribution proposes three aspects. The first aspect proposes a bug-fix to fix the conditions used to determine whether a coeff_sign_flag is bypass coded or context coded. The second and third aspects proposes semantic fix of the sig_coeff_flag and sb_coded_flag.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).