| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0219 |
| --- | --- |

| Title: | AHG8/AHG9: On APS NAL unit clean-up |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Jaehyun Lim Seethal Paluri Hendry Seung Hwan Kim | Email: | hm.jang@lge.com junghak.nam@lge.com jaehyun.lim@lge.com seethal.paluri@lge.com dr.hendry@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes the following changes:

Proposal 1: Enable the removal of parameter sets (i.e., PPS and APS) in non-output layer that is not used as reference for decoding of pictures in output layers during extraction process.

Proposal 2: Move the signalling of aps_params_type to earlier position and change the signalling of aps_adaptation_parameter_set_id from u(5) to u(v). The length of the syntax element depends on the type of the APS.

# Proposal 1

It is asserted that for extraction process, VCL NAL units in a non-output layer with TemporalId equal to or lower than the target highest TemporalId may be removed if they are not needed decoding of any output layers. However, APS NAL units in such layer and temporal sub-layer are not removed as well. Typically, in such situation, the higher output layer not only not using VCL from that temporal sub-layer but also not using the parameter sets such as APS and / or PPS from there.

We propose to enable the removal of parameter sets (i.e., PPS and APS) in non-output layer that is not used as reference for decoding of pictures in output layers during extraction process. The changes needed to do this are as follows:

- Modify the semantics of vps_max_tid_il_ref_pics_plus1[ i ][ j ] such that when it is greater than 0 it means for decoding pictures of the i-th layer, no parameter set and picture from the j-th layer with TemporalId greater than vps_max_tid_il_ref_pics_plus1[ i ][ j ] − 1 is used as reference.

- In extraction process, in addition to removing VCL NAL unit, also remove PPS and APS with TemporalId that is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

The text for the proposed changes are shown below:

vps_max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 specifies that the pictures of the j-th layer that are neither IRAP pictures nor GDR pictures with ph_recovery_poc_cnt equal to 0 are not used as ILRPs for decoding of pictures of the i-th layer. vps_max_tid_il_ref_pics_plus1[ i ][ j ] greater than 0 specifies that, for decoding pictures of the i-th layer, no parameter set and picture from the j-th layer with TemporalId greater than vps_max_tid_il_ref_pics_plus1[ i ][ j ] − 1 is used as referenceILRP. When not present, the value of vps_max_tid_il_ref_pics_plus1[ i ][ j ] is inferred to be equal to vps_max_sublayers_minus1 + 1.

...

–	Remove from outBitstream all PPS, APS, and VCL NAL units for which all of the following conditions are true, and their associated non-VCL NAL units with nal_unit_type equal to PH_NUT, FD_NUT, SUFFIX_SEI_NUT, and PREFIX_SEI_NUT with PayloadType not equal to 0, 1, or 130:

–	When the nal_unit_type of the VCL NAL unit is equal to TRAIL_NUT, STSA_NUT, RADL_NUT, or RASL_NUT, or nal_unit_type is equal to GDR_NUT and the associated ph_recovery_poc_cnt is not equal to 0.

–	nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

...

# Proposal 2

According to the current VVC specification text, the range of APS Id varies depending on the APS type, i.e., 0 to 7 for ALF APS and Scaling APS; 0 to 3 for LMCS APS. Accordingly, rather than signalling the APS id with fixed length coding, we may consider signalling it with just necessary number of bits depending on the type.

We propose the following:

- Move the signalling of aps_params_type earlier

- Change the signalling of aps_adaptation_parameter_set_id from u(5) to u(v). The length of the syntax element depends on the aps_params_type.

The text for the proposed changes are shown below:

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| aps_adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| aps_adaptation_parameter_set_id | u(v) |
| ... |  |
| } |  |

aps_adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements. When aps_params_type is equal to ALF_APS or SCALING_APS, the length of aps_adaptation_parameter_set_id is 3 bits. Otherwise (aps_params_type is equal to LMCS_APS), the length of aps_adaptation_parameter_set_id is 2 bits.

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

...

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).