| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0227 |
| --- | --- |

| Title: | AHG9: On MMVD flag in SPS and PH |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kyohei Unno Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes a bug-fix to the semantics of sps_mmvd_fullpel_only_flag. In the latest VVC draft 9, sps_mmvd_fullpel_only_flag and ph_mmvd_fullpel_only_flag are used to determine whether the sample precision of merge mode with motion vector difference (MMVD) is only integer at a sequence or a picture level. Even though sps_mmvd_fullpel_only_flag is defined in the semantics of the flag that MMVD uses integer sample precision when the flag equals to 1, there is a possibility that MMVD may actually use fractional sample precision when the flag equals to 1 due to the signalling of ph_mmvd_only_flag which is parsed only if sps_mmvd_fullpel_only_flag equals to 1. In this contribution, two different options are proposed to solve the bug derived from inconsistency between the semantics definition of sps_mmvd_fullpel_only_flag and the related syntax parsing process.

Introduction

In the latest VVC draft 9 [1], sps_mmvd_fullpel_only_flag and ph_mmvd_fullpel_only_flag are used to determine whether the sample precision of merge mode with motion vector difference (MMVD) is only integer at a sequence or a picture level.

In the semantics of sps_mmvd_fullpel_only_flag, it is defined that MMVD uses only integer sample precision when the flag equals to 1 while that MMVD may use fractional sample precision when the flag equals to 0 as follows:

sps_mmvd_fullpel_only_flag equal to 1 specifies that merge mode with motion vector difference uses integer sample precision. sps_mmvd_fullpel_only_flag equal to 0 specifies that merge mode with motion vector difference may use fractional sample precision. When not present, the value of sps_mmvd_fullpel_only_flag is inferred to be equal to 0.

Similarly, the semantics of ph_mmvd_fullpel_only_flag, which is parsed only if sps_mmvd_fullpel_only_flag equals to 1, is defined as follows:

ph_mmvd_fullpel_only_flag equal to 1 specifies that merge mode with motion vector difference uses integer sample precision in the slices associated with the PH. ph_mmvd_fullpel_only_flag equal to 0 specifies that merge mode with motion vector difference may use fractional sample precision in the slices associated with the PH. When not present, the value of ph_mmvd_fullpel_only_flag is inferred to be 0.

The parsing condition of ph_mmvd_fullpel_only_flag is defined as follows:

Table 7.3.2.7 Picuture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| ph_pic_parameter_set_id | ue(v) |
| … |  |
| if( sps_mmvd_fullpel_only_flag ) |  |
| ph_mmvd_fullpel_only_flag | u(1) |
| … |  |
| } |  |

Shown as the above semantics and syntax parsing condition, there is a possibility that MMVD may actually use fractional sample precision when sps_mmvd_fullpel_only_flag equals to 1, even though the semantics of the flag defines that MMVD uses integer sample precision when the flag equals to 1. The semantics bug is derived from inconsistency between the semantics definition of sps_mmvd_fullpel_only_flag and ph_mmvd_fullpel_only_flag parsing process. In detail, MMVD sample precision may be specified as fractional with ph_mmvd_fullpel_only_flag  even if MMVD sample precision is specified as integer with sps_mmvd_fullpel_only_flag (i.e., in the case of ph_mmvd_fullpel_only_flag == 0 && sps_mmvd_fullpel_only_flag == 1).

Proposed methods

The following two options are proposed to solve the inconsistency.

- Option 1: Change syntax parsing condition

- Option 2: Change semantics

Option 1

In option 1, the parsing condition of ph_mmvd_fullpel_only_flag in syntax table is modified without any semantics changes.

Suggested text changes are shown as follows:

7.3.2.7 Picuture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| ph_pic_parameter_set_id | ue(v) |
| … |  |
| if( !sps_mmvd_fullpel_only_flag ) |  |
| ph_mmvd_fullpel_only_flag | u(1) |
| … |  |
| } |  |

Option 2

In option 2, the syntax name and the semantics of sps_mmvd_fullpel_only_flag are modified without the parsing condition of ph_mmvd_fullpel_only_flag.

Suggested text changes are shown as follows:

sps_mmvd_fullpel_only_enabled_flagsps_mmvd_fullpel_only_flag equal to 1 specifies that merge mode with motion vector difference may useuses integer sample precision. sps_mmvd_fullpel_only_enabled_flag sps_mmvd_fullpel_only_flag equal to 0 specifies that merge mode with motion vector difference usesmay use fractional sample precision. When not present, the value of sps_mmvd_fullpel_only_enabled_flag sps_mmvd_fullpel_only_flag is inferred to be equal to 0.

7.3.2.7 Picuture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| ph_pic_parameter_set_id | ue(v) |
| … |  |
| if(sps_mmvd_fullpel_only_enabled_flagsps_mmvd_fullpel_only_flag ) |  |
| ph_mmvd_fullpel_only_flag | u(1) |
| … |  |
| } |  |

Conclusion

In this contribution, the two different options were proposed to solve the inconsistency between the semantics of sps_mmvd_fullpel_only_flag and parsing condition of ph_mmvd_fullpel_only_flag. Especially, the option 1 is very minor change from the latest VVC draft that ph_mmvd_fullpel_only_flag is parsed only if MMVD sample precision is not specified as only integer (i.e., in the case of sps_mmvd_fullpel_only_flag == 0). On the other hand, the option 2 solves the inconsistency without changing syntaxy parsing process. It is, therefore, recommended that the option 1 or the option 2 are adopted to VVC Draft and VTM.

This work was conducted under the project "Research and Development of Next generation video transmission and communication technology for ultra-high definition video application under versatile purpose and environment" of the Ministry of Internal Affairs and Communications (MIC), Japan.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-R2001, by teleconference, 15-24 April 2020.

Patent rights declaration

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).