| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0229-v2 |
| --- | --- |

| Title: | Bug fix of BDOF for high bit-depth |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Eiich Sasaki Tomohiro Ikai | Tel: Email: | +81-50-5213-6334 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

At the previous April meeting, JVET-R0351 indicated that several coding tools do not work well in case of a high bit-depth coding. BDOF is one of them. In this contribution, a bug fix of specification and software for BDOF when the bit-depth is greater than 12-bit is proposed. As experimental results show, the performance has been improved when InternalBitDepth is equal to 14. This proposal does not change the specification when the bit-depth is from 8-bit to 12-bit and keep the internal bit precision of BDOF even if the bit-depth is greater than 12-bit.

Problem Statement

JVET-R0351 [1] pointed out that BDOF does not work properly in case of higher bit-depth coding. There is a problem with both the specification and the VTM software. This problem has been also reported by the authors in JVET-O0207 [2].

Proposal

The working draft [3] should be changed by the following.

- 8.5.6.5	Bi-directional optical flow prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW + 2)x(nCbH + 2) luma prediction sample arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags predFlagL0 and predFlagL1,

- the reference indices refIdxL0 and refIdxL1,

Output of this process is the (nCbW)x(nCbH) array pbSamples of luma prediction sample values.

The variables shift1, shift2, shift3, shift4, offset4, and mvRefineThres are derived as follows:

- The variable shift1 is set to be equal to 6.

- The variable shift2 is set to be equal to 4Max( 4, bitDepth − 8 ).

- The variable shift3 is set to be equal to 1Max( 1, bitDepth − 11 ).

- The variable shift4 is set equal to Max( 3, 15 − BitDepth ) and the variable offset4 is set equal to 1  <<  ( shift4 − 1 ).

- The variable mvRefineThres is set equal to 1  <<  4.

For xIdx = 0..( nCbW  >>  2 ) − 1 and yIdx = 0..( nCbH  >>  2 ) − 1, the following applies:

- The variable xSb is set equal to ( xIdx  <<  2) + 1 and ySb is set equal to ( yIdx  <<  2 ) + 1.

- The prediction sample values of the current subblock are derived as follows:

  - For x =xSb − 1..xSb + 4, y = ySb − 1..ySb + 4, the following ordered steps apply:

- The locations ( hx, vy ) for each of the corresponding sample locations ( x, y ) inside the prediction sample arrays are derived as follows:

hx = Clip3( 1, nCbW, x )		(991)

vy = Clip3( 1, nCbH, y )		(992)

- The variables gradientHL0[ x ][ y ], gradientVL0[ x ][ y ], gradientHL1[ x ][ y ] and gradientVL1[ x ][ y ] are derived as follows:

gradientHL0[ x ][ y ]  =  ( predSamplesL0[ hx + 1 ][vy]  >>  shift1 ) −	(993)
	( predSampleL0[ hx − 1 ][ vy ] )  >>  shift1 )

gradientVL0[ x ][ y ]  =  ( predSampleL0[ hx ][ vy + 1 ]  >>  shift1 ) −	(994)
	( predSampleL0[ hx ][vy − 1 ] )  >>  shift1 )

gradientHL1[ x ][ y ]  =  ( predSamplesL1[ hx + 1 ][vy]  >>  shift1 ) −	(995)
	( predSampleL1[ hx − 1 ][ vy ] )  >>  shift1 )

gradientVL1[ x ][ y ]  =   ( predSampleL1[ hx ][ vy + 1 ]  >>  shift1 ) −	(996)
	( predSampleL1[ hx ][vy − 1 ] )  >>  shift1 )

- The variables diff[ x ][ y ], tempH[ x ][ y ] and tempV[ x ][ y ] are derived as follows:

diff[ x ][ y ] = (predSamplesL0[ hx ][ vy ]  >>  shift2 ) − ( predSamplesL1[ hx ][ vy ]  >>  shift2 )	(997)

tempH[ x ][ y ] = (gradientHL0[ x ][ y ] + gradientHL1[ x ][ y ] )  >>  shift3	(998)

tempV[ x ][ y ] = (gradientVL0[ x ][ y ] + gradientVL1[ x ][ y ] )  >>  shift3	(999)

  - The variables sGx2, sGy2, sGxGy, sGxdI and sGydI are derived as follows:

sGx2 = ij Abs( tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(1000)

sGy2 = ij Abs( tempV[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(1001)

sGxGy = ij( Sign( tempV[ xSb + i ][ ySb + j ] ) * tempH[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(1002)

sGxdI = ij( −Sign( tempH[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(1003)

sGydI = ij( −Sign( tempV[ xSb + i ][ ySb + j ] ) * diff[ xSb + i ][ ySb + j ] ) with i, j = −1..4	(1004)

  - The horizontal and vertical motion offset of the current subblock are derived as:

vx = sGx2 > 0 ? Clip3( −mvRefineThres + 1, mvRefineThres − 1,	(1005)
	( sGxdI  <<  2 )  >>  Floor( Log2( sGx2 ) ) ) : 0

vy = sGy2 > 0 ? Clip3( −mvRefineThres + 1, mvRefineThres − 1, ( ( sGydI  <<  2 ) −	(1006)
	( ( vx * sGxGy )  >>  1 ) )  >>  Floor( Log2( sGy2 ) ) ) : 0

  - For x =xSb − 1..xSb + 2, y = ySb − 1..ySb + 2, the prediction sample values of the current sub-block are derived as follows:

bdofOffset = vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] )	(1007)
	+ vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] )

pbSamples[ x ][ y ] = Clip3( 0, ( 2BitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ] + offset4 +	(1008)
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )

In the current specification, when the bit depth is greater than 12-bit, the precision of the variables diff, tempH and tempV is increased as shown in Table 1. In the proposed specification change, when the bit depth is greater than 12-bit, the values of shift2 and shift3 are increased; however, the precision of the variables diff, tempH and tempV is constant as shown in Table 2.

Table 1  Internal bit precision of BDOF (Current specification)

|  |  | bitDepth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Value | shift1 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
|  | shift2 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
|  | shift3 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | shift4 | 7 | 6 | 5 | 4 | 3 | 3 | 3 | 3 | 3 |
|  | mvRefineThres | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Precision (dynamic range) | predSamplesL0 predSamplesL1 | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 15 (17) | 16 (18) | 17 (19) | 18 (20) |
|  | gradientHL0 gradientVL0 gradientHL1 gradientVL1 | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 10 (12) | 11 (13) | 12 (14) | 13 (15) |
|  | diff | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 12 (14) | 13 (15) | 14 (16) | 15 (17) |
|  | tempH tempV | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 10 (12) | 11 (13) | 12 (14) | 13 (15) |
|  | bdofOffset | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 16 (18) | 17 (19) | 18 (20) | 19 (21) |
|  | pbSamples | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

Table 2  Internal bit precision of BDOF (Proposal)

|  |  | bitDepth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| Value | shift1 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
|  | shift2 | 4 | 4 | 4 | 4 | 4 | 5 | 6 | 7 | 8 |
|  | shift3 | 1 | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 5 |
|  | shift4 | 7 | 6 | 5 | 4 | 3 | 3 | 3 | 3 | 3 |
|  | mvRefineThres | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |
| Precision (dynamic range) | predSamplesL0 predSamplesL1 | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 14 (16) | 15 (17) | 16 (18) | 17 (19) | 18 (20) |
|  | gradientHL0 gradientVL0 gradientHL1 gradientVL1 | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 9 (11) | 10 (12) | 11 (13) | 12 (14) | 13 (15) |
|  | diff | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) | 11 (13) |
|  |  | tempH tempV |  | 9 (11) |  | 9 (11) |  | 9 (11) |  | 9 (11) |  | 9 (11) |  | 9 (11) |  | 9 (11) |  | 9 (11) |  | 9 (11) |  |
|  | bdofOffset | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 15 (17) | 16 (18) | 17 (19) | 18 (20) | 19 (21) |
|  | pbSamples | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |

Except for changes due to RExt__HIGH_BIT_DEPTH_SUPPORT set to 1, VTM software should be changed by the following.

Note: according to cross-checker’s comment, the change 1 and 3 have been already merged to the latest main branch of VTM software by JVET-R0351.

- Change 1

In /souce/lib/CommonLib/Buffer.cpp, the type castings are not correct.

void addBIOAvgCore(const Pel* src0, int src0Stride, const Pel* src1, int src1Stride, Pel *dst, int dstStride, const Pel *gradX0, const Pel *gradX1, const Pel *gradY0, const Pel*gradY1, int gradStride, int width, int height, int tmpx, int tmpy, int shift, int offset, const ClpRng& clpRng)

{

int b = 0;

for (int y = 0; y < height; y++)

{

for (int x = 0; x < width; x += 4)

{

#if BDOF_HIGH_BITDEPTH_FIX

b = tmpx * (gradX0[x] - gradX1[x]) + tmpy * (gradY0[x] - gradY1[x]);

dst[x] = ClipPel((Pel)rightShift((src0[x] + src1[x] + b + offset), shift), clpRng);

b = tmpx * (gradX0[x + 1] - gradX1[x + 1]) + tmpy * (gradY0[x + 1] - gradY1[x + 1]);

dst[x + 1] = ClipPel((Pel)rightShift((src0[x + 1] + src1[x + 1] + b + offset), shift), clpRng);

b = tmpx * (gradX0[x + 2] - gradX1[x + 2]) + tmpy * (gradY0[x + 2] - gradY1[x + 2]);

dst[x + 2] = ClipPel((Pel)rightShift((src0[x + 2] + src1[x + 2] + b + offset), shift), clpRng);

b = tmpx * (gradX0[x + 3] - gradX1[x + 3]) + tmpy * (gradY0[x + 3] - gradY1[x + 3]);

dst[x + 3] = ClipPel((Pel)rightShift((src0[x + 3] + src1[x + 3] + b + offset), shift), clpRng);

#else

b = tmpx * (gradX0[x] - gradX1[x]) + tmpy * (gradY0[x] - gradY1[x]);

dst[x] = ClipPel((int16_t)rightShift((src0[x] + src1[x] + b + offset), shift), clpRng);

b = tmpx * (gradX0[x + 1] - gradX1[x + 1]) + tmpy * (gradY0[x + 1] - gradY1[x + 1]);

dst[x + 1] = ClipPel((int16_t)rightShift((src0[x + 1] + src1[x + 1] + b + offset), shift), clpRng);

b = tmpx * (gradX0[x + 2] - gradX1[x + 2]) + tmpy * (gradY0[x + 2] - gradY1[x + 2]);

dst[x + 2] = ClipPel((int16_t)rightShift((src0[x + 2] + src1[x + 2] + b + offset), shift), clpRng);

b = tmpx * (gradX0[x + 3] - gradX1[x + 3]) + tmpy * (gradY0[x + 3] - gradY1[x + 3]);

dst[x + 3] = ClipPel((int16_t)rightShift((src0[x + 3] + src1[x + 3] + b + offset), shift), clpRng);

#endif

}

dst += dstStride;       src0 += src0Stride;     src1 += src1Stride;

gradX0 += gradStride; gradX1 += gradStride; gradY0 += gradStride; gradY1 += gradStride;

}

}

- Change 2

In /souce/lib/CommonLib/Buffer.cpp, the shift values should be fixed.

void calcBIOSumsCore(const Pel* srcY0Tmp, const Pel* srcY1Tmp, Pel* gradX0, Pel* gradX1, Pel* gradY0, Pel* gradY1, int xu, int yu, const int src0Stride, const int src1Stride, const int widthG, const int bitDepth, int* sumAbsGX, int* sumAbsGY, int* sumDIX, int* sumDIY, int* sumSignGY_GX)

{

#if BDOF_HIGH_BITDEPTH_FIX

const int shift4 = std::max<int>(4, bitDepth-8);

const int shift5 = std::max<int>(1, bitDepth-11);

#else

int shift4 = 4;

int shift5 = 1;

#endif

for (int y = 0; y < 6; y++)

{

for (int x = 0; x < 6; x++)

{

int tmpGX = (gradX0[x] + gradX1[x]) >> shift5;

int tmpGY = (gradY0[x] + gradY1[x]) >> shift5;

int tmpDI = (int)((srcY1Tmp[x] >> shift4) - (srcY0Tmp[x] >> shift4));

*sumAbsGX += (tmpGX < 0 ? -tmpGX : tmpGX);

*sumAbsGY += (tmpGY < 0 ? -tmpGY : tmpGY);

*sumDIX += (tmpGX < 0 ? -tmpDI : (tmpGX == 0 ? 0 : tmpDI));

*sumDIY += (tmpGY < 0 ? -tmpDI : (tmpGY == 0 ? 0 : tmpDI));

*sumSignGY_GX += (tmpGY < 0 ? -tmpGX : (tmpGY == 0 ? 0 : tmpGX));

}

srcY1Tmp += src1Stride;

srcY0Tmp += src0Stride;

gradX0 += widthG;

gradX1 += widthG;

gradY0 += widthG;

gradY1 += widthG;

}

}

- Change 3

In /souce/lib/CommonLib/InterPrediction.cpp, the source code is not identical to the equation in the document.

void InterPrediction::applyBiOptFlow(const PredictionUnit &pu, const CPelUnitBuf &yuvSrc0, const CPelUnitBuf &yuvSrc1, const int &refIdx0, const int &refIdx1, PelUnitBuf &yuvDst, const BitDepths &clipBitDepths)

{

:

:

#if BDOF_HIGH_BITDEPTH_FIX

const int   shiftNum = std::max<int>(3, IF_INTERNAL_PREC + 1 - bitDepth);

#else

const int   shiftNum = IF_INTERNAL_PREC + 1 - bitDepth;

#endif

const int   offset = (1 << (shiftNum - 1)) + 2 * IF_INTERNAL_OFFS;

const int   limit = ( 1 << 4 ) - 1;

:

:

}

Experimental results

The current VTM-9.0 software can not be compiled due to variable type problems when the define statement RExt__HIGH_BIT_DEPTH_SUPPORT is set to 1. However, changing type casting and fixing a bug for coefficient coding in the use of mixed signed and unsigned data types, the software can be built. By using the bug fixed software, the experiments have been conducted on CTC [4] with InternalBitDepth equal to 14.

The anchor is the result of CTC in 10-bit encoding. As shown in Table 1, the results of VTM-9.0 with RExt__HIGH_BIT_DEPTH_SUPPORT set to 1 and InternalBitDepth equal to 14 are worse than that of CTC (InternalBitDepth equal to 10). That is due to the reported BDOF bug. After the proposed bug fix, the result of InternalBitDepth equal to 14 becomes better than that of CTC as expected. Since all SIMD codes are off when RExt__HIGH_BIT_DEPTH_SUPPORT is set to 1, the encoding and decoding times were increased. It is noted the encoding result of InternalBitDepth equal to 10 was identical to that of CTC.

Table 3	Summary of experimental results (Luma BD-rate) on CTC RA configuration
(RExt__HIGH_BIT_DEPTH_SUPPORT=1 and InternalBitDepth=14)

|  | Class A1 | Class A2 | Class B | Class C | Overall | Class D | Class F |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VTM-9.0 | 2.25% | 3.89% | 2.67% | 2.69% | 2.84% | 3.76% | 0.85% |
| Proposal | -0.87% | -0.74% | -0.42% | -0.40% | -0.57% | -0.19% | -0.62% |

Figure 1 Summary of experimental results when InternalBitDepth is increased.

As several additional investigations, it was found that the reason for the loss is mainly a bug of shift4 which is shown in the change 3 of the software. Therefore, additionally, it is shown that the experimental result of comparison whether the values of shift2 and shift3 should be changed when InternalBitDepth is equal to 14. Both results are identical as shown in Table 4. This result means that it is not necessary to increase the internal bit precision of BDOF and the current specification of shift2 and shift3 should be changed.

Table 4  Experimental result of changing the values of shift2 and shift3

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-9.0(InternalBitDepth=14) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Reference

[1]	A. Browne, S. Keating and K. Sharman, “High bit depth coding”, Joint Video Experts Team Document, JVET-R0351, teleconference, April 2020.

[2]	T. Chujoh, E. Sasaki and T. Ikai, “Non-CE9: A bug fix of BDOF specification”, Joint Video Experts Team Document, JVET-O0207, Gothenburg, July 2020.

[3]	B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 9)”, Joint Video Experts Team Document, JVET-R2001, teleconference, April 2020.

[4]	F. Bossen, J. Boyce, K. Sühring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, Joint Video Experts Team Document, JVET-N1010, Geneva, March 2019.

[5]	T. Chujoh and R. Noda, “Internal bit depth increase for coding efficiency”, ITU-T Q.6/SG16 Document, VCEG-AE13, Marrakech, Jan. 2007

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).