| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0230 |
| --- | --- |

| Title: | AHG9: new GCI flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai | Tel: Email: | ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a new GCI flags as follows:

- no_entropy_coding_sync_constraint_flag

Introduction

There’s a set of GCI syntax in the current draft or proposed in JVET-S AHG. These also include the following categories.

- HEVC era tools:

  - e,g, no_sign_data_hiding_constraint_flag

- Partitioning related tools:

  - e.g. no_pic_partition_constraint_flag, no_rectangular_slice_constraint_flag, no_raster_scan_slice_constraint_flag, one_slice_per_subpicture_constraint_flag in JVET-S0113

For better consistency, it is proposed to add no_entropy_coding_sync_constraint_flag.

Proposals

Table 1 Proposed GCI syntax elements

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| no_entropy_coding_sync_constraint_flag | u(1) |

no_entropy_coding_sync_constraint_flag equal to 1 specifies that sps_entropy_coding_sync_enabled_flag shall be equal to 0. no_entropy_coding_sync_constraint_flag equal to 0 does not impose such a constraint.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).