| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0236-v1 |
| --- | --- |

| Title: | AHG9/AHG12: High-level syntax cleanups on subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Lulin Chen, Olena Chubach, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In this contribution, following modifications related to subpictures are proposed:

1   When sps_num_subpics_minus1 is greater than 0 in the reference layer, the inter-layer reference picture (ILRP) shall have the same value of sps_log2_ctu_size_minus5 as that of the SPS referred to by the current picture.

2	The value of sps_num_subpics_minus1 shall not be greater than MaxSlicesPerPicture – 1.

3	The presence of pps_rect_slice_flag is further conditioned on pps_subpic_id_mapping_present_flag

Proposals

Proposal 1: Same value of sps_log2_ctu_size_minus5 for ILRP

When sps_num_subpics_minus1 is greater than 0 for the SPS referred to by an inter-layer reference picture, it is a requirement of bitstream conformance that the reference picture and the current picture shall have the same subpicture layout for inter-layer coding. Since the value of sps_log2_ctu_size_minus5 is related to the scaling factor of subpicture layout, it is proposed that the SPS referred to by the current picture and the SPS referred to by the reference picture shall have the same value of sps_log2_ctu_size_minus5.

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

7.4.3.3 Sequence parameter set RBSP semantics

sps_subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. sps_subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of sps_subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1.

When sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples.

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_log2_ctu_size_minus5 and the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

### 8.3.2 Decoding process for reference picture lists construction

…

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and sps_num_subpics_minus1 is greater than 0, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] has the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_log2_ctu_size_minus5 and the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

Proposal 2: sps_num_subpics_minus1 upper limited by level constraint

Each subpicture contains at least one slice, and the number of the coded slices is equal to or greater than the number of subpictures in the coded picture. Therefore, the number of the subpictures in the picture shall not be greater than the allowed maximum number of slices of the picture under the specified profile and level constraint. It is proposed that the allowed maximum value of sps_num_subpics_minus1 is equal to Min( MaxSlicesPerPicture, Ceil( sps_pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( sps_pic_height_max_in_luma_samples ÷ CtbSizeY ) ) – 1.

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

7.4.3.3 Sequence parameter set RBSP semantics

sps_num_subpics_minus1 plus 1 specifies the number of subpictures in each picture in the CLVS. The value of sps_num_subpics_minus1 shall be in the range of 0 to Min( MaxSlicesPerPicture, Ceil( sps_pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( sps_pic_height_max_in_luma_samples ÷ CtbSizeY ) ) − 1, inclusive. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0.

Proposal 3: Presence of pps_rect_slice_flag conditioned on pps_subpic_id_mapping_present_flag

When pps_subpic_id_mapping_present_flag is equal to 1, it indicates that rectangular slices shall be used for the coded picture and the value of pps_rect_slice_flag shall be equal to 1. In this contribution,  it is proposed that the presence of pps_rect_slice_flag is further conditioned on pps_subpic_id_mapping_present_flag. When pps_subpic_id_mapping_present_flag is equal to 1, pps_rect_slice_flag is inferred to be equal to 1.

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | u(6) |
| pps_seq_parameter_set_id | u(4) |
| … |  |
| pps_no_pic_partition_flag | u(1) |
| pps_subpic_id_mapping_present_flag | u(1) |
| if( pps_subpic_id_mapping_present_flag ) { |  |
| if( !pps_no_pic_partition_flag ) |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| if( !pps_no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| pps_num_exp_tile_columns_minus1 | ue(v) |
| pps_num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= pps_num_exp_tile_columns_minus1; i++ ) |  |
| pps_tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= pps_num_exp_tile_rows_minus1; i++ ) |  |
| pps_tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| pps_loop_filter_across_tiles_enabled_flag | u(1) |
| if( !pps_subpic_id_mapping_present_flag ) |  |
| pps_rect_slice_flag | u(1) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

References

- Y.-K. Wang, “AHG2: Editorial input of a text integration for the May 2020 HLS AHG meeting outcome,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-S0152, 19th Meeting: by teleconference, 22 June – 1 July 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).