| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0236-v2 |
| --- | --- |

| Title: | AHG9/AHG12: High-level syntax cleanups on subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Lulin Chen, Olena Chubach, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In this contribution, following modifications related to subpictures are proposed:

1  	The value of sps_num_subpics_minus1 shall not be greater than MaxSlicesPerPicture – 1.

2	The presence of pps_rect_slice_flag is further conditioned on pps_subpic_id_mapping_present_flag

Proposals

Proposal 1: sps_num_subpics_minus1 upper limited by level constraint

Each subpicture contains at least one slice, and the number of the coded slices is equal to or greater than the number of subpictures in the coded picture. Therefore, the number of the subpictures in the picture shall not be greater than the allowed maximum number of slices of the picture under the specified profile and level constraint. It is proposed that the allowed maximum value of sps_num_subpics_minus1 is equal to Min( MaxSlicesPerPicture, Ceil( sps_pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( sps_pic_height_max_in_luma_samples ÷ CtbSizeY ) ) – 1.

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

7.4.3.3 Sequence parameter set RBSP semantics

sps_num_subpics_minus1 plus 1 specifies the number of subpictures in each picture in the CLVS. The value of sps_num_subpics_minus1 shall be in the range of 0 to Min( MaxSlicesPerPicture, Ceil( sps_pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( sps_pic_height_max_in_luma_samples ÷ CtbSizeY ) ) − 1, inclusive. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0.

Proposal 2: Presence of pps_rect_slice_flag conditioned on pps_subpic_id_mapping_present_flag

When pps_subpic_id_mapping_present_flag is equal to 1, it indicates that rectangular slices shall be used for the coded picture and the value of pps_rect_slice_flag shall be equal to 1. In this contribution,  it is proposed that the presence of pps_rect_slice_flag is further conditioned on pps_subpic_id_mapping_present_flag. When pps_subpic_id_mapping_present_flag is equal to 1, pps_rect_slice_flag is inferred to be equal to 1.

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | u(6) |
| pps_seq_parameter_set_id | u(4) |
| … |  |
| pps_no_pic_partition_flag | u(1) |
| pps_subpic_id_mapping_present_flag | u(1) |
| if( pps_subpic_id_mapping_present_flag ) { |  |
| if( !pps_no_pic_partition_flag ) |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| if( !pps_no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| pps_num_exp_tile_columns_minus1 | ue(v) |
| pps_num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= pps_num_exp_tile_columns_minus1; i++ ) |  |
| pps_tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= pps_num_exp_tile_rows_minus1; i++ ) |  |
| pps_tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| pps_loop_filter_across_tiles_enabled_flag | u(1) |
| if( !pps_subpic_id_mapping_present_flag ) |  |
| pps_rect_slice_flag | u(1) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

References

- Y.-K. Wang, “AHG2: Editorial input of a text integration for the May 2020 HLS AHG meeting outcome,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-S0152, 19th Meeting: by teleconference, 22 June – 1 July 2020.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).