| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0237-v3 |
| --- | --- |

| Title: | Agenda and [Draft] report of the 19-21 June 2020 HLS AHG pre-meeting |  |  |
| --- | --- | --- | --- |
| Status: | Agenda and report document from the coordinators of the AHG meeting |  |  |
| Purpose: | Agenda and report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Ye-Kui Wang Bytedance Inc. 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +1 908 803 3888 yekui.wang@bytedance.com |
| Source: | Coordinators of the AHG pre-meeting |  |  |

_____________________________

# Summary

The high-level syntax (HLS) AHG pre-meeting, mostly in the scopes of AHGs 8, 9, and 12, held three days of teleconference meeting sessions on June 19-21, 2020 (in UTC time).

The sessions on each of the three days were held during the following time slots (in UTC time):

- 1300 (2 hours)

- [break – 20 minutes]

- 1520 (2 hours)

- [break – 3 hours 40 minutes]

- 1900 (2 hours)

- [break – 20 minutes]

- 2120 (2 hours)

- [break – 11 hours 40 minutes to the next session (when available)]

In addition to the breaks between sessions as noted above, five-to-ten-minute breaks were held at approximately the one-hour midpoint within each meeting session.

Altogether, 12 sessions were held (3 days with 4 sessions per day). The sessions are numbered as Sessions 9 to 20 in this report.

Approximately XXX people attended the AHG pre-meeting, and approximately 120+77 input documents were considered to be within the scope of the AHG, although 81 were (initially) discussed earlier at the May 27-28 HLS AHG meeting, and not all of the remaining ones were discussed at the AHG pre-meeting.

Before the pre-meeting started, approximately 81 (41%) of the 120+77 input documents had been reviewed, resulted in 37 recommendations for normative action, 13 editor action item, and 16 revisits/deferred:

- 3.1.1 High level tool control (8/(16+8)): 1 recommendation, 2 revisits

- 3.1.2 General and misc. HLS topics (16/(16+15)): 7 recommendations, 3 deferred

- 3.1.3 General constraints information (GCI) (15/(25+7)): 7 recommendations (3 with offline studies), 4 editor action items

- 3.1.4 SPS, PPS, and APS cleanups (9/(9+8)): 5 recommendations, 1 deferred

- 3.1.5 PH and SH cleanups (6/(10+4)): 1 recommendation, 3 deferred

- 3.1.6 Reference picture lists cleanups (8/(8+2)): 5 recommendations, 2 editor action items

- 3.1.7 Signalling of virtual boundaries (2/(2+5)): 2 recommendations, 1 editor action item

- 3.1.8 Hypothetical reference decoder (HRD) (6/(12+14)): 2 recommendations, 1 editor action item, 2 deferred

- 3.1.9 DCI, VUI, and SEI (0/(3+3))

- 3.1.10 HLS editorial inputs (0/(4+3))

- 3.2 Subpictures, slices, and tiles (3/(7+5)): 2 recommendations, 2 editor action items

- 3.3 Scalability and RPR cleanups (8/(8+3)): 5 recommendations, 3 editor action items, 3 deferred

# Attendance

Lists of the participants who attended the AHG meeting can be found in spreadsheet records attached with this report. The spreadsheets have not been fully pruned to identify unique participants, so the estimate of about XXX total participants is only a rather rough one. Each meeting session had an average of roughly AAA active connections on the call at any particular moment. This included a few participants who were using multiple connections in order to have dual screen display capability (e.g. for viewing or sharing meeting notes and a presentation, the participant list and/or chat session messages).

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

In order to manage the teleconference resources effectively and reduce potential problems with security and attendance recording, a password was used for the meeting sessions. The password was distributed only to those who had registered for the JVET meeting.

# High-level syntax (HLS) proposals (120+77)

## AHG9: General high-level syntax (105+69)

### High level tool control (16+8)

JVET-S0144 AHG9: A summary of proposals on high level tool control [L. Zhang, Y.-K. Wang (Bytedance)]

This contribution was discussed in the HLS AHG meeting at 0835 UTC on 27 May and in the HLS AHG pre-meeting at ....

This contribution provides a summary of the 23 proposals on High level tool control (the agenda item 3.1.1 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following actions are proposed:

- Removal of SPS BDOF/DMVR/PROF control present flag

Remove the syntax elements sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag and instead condition the presence of ph_bdof_disabled_flag, ph_dmvr_disabled_flag and ph_prof_disabled_flag on the existing syntax elements sps_bdof_enabled_flag, sps_dmvr_enabled_flag and sps_affine_prof_enabled_flag, respectively. (S0044)

Each of these flags was said to save at most one bit per picture. This would increase overhead in some common uses. No clear need for action was evident, so no action was recommended.

- On TMVP derivation considering inter-layer reference picture

  - Disable TMVP when either RefPic or ColRefPic is inter-layer reference picture (ILRP), but not both. (S0089 #1)

  - Enable TMVP with combination of STRP and ILRP on RefPic or ColRefPic, and disable TMVP with combination of LTRP and ILRP. (S0089 #2)

  - Enable TMVP when either RefPic or ColRefPic is ILRP. (S0089 #3)

| RefPic (target reference picture) | ColRefPic (reference picture of collocated block) | Current VVC | Solution #1 | Solution #2 | Solution #3 |
| --- | --- | --- | --- | --- | --- |
| STRP | ILRP | X | X | O | O |
| ILRP | STRP | X | X | O | O |
| LTRP | ILRP | O | X | X | O |
| ILRP | LTRP | O | X | X | O |

Much of the behaviour was originally from HEVC.

It was commented that this has a low-level impact and we should avoid any potentially unnecessary changes in this area. No clear need for action was evident, so no action was recommended.

- On maximum number of subblock merge candidates

Change the range of the value of sps_five_minus_max_num_subblock_merge_cand from 0..5 − sps_sbtmvp_enabled_flag to 0..5 and change the derivation of maximum number of subblock merge candidates in PH as follows: (S0088)

if( sps_affine_enabled_flag && sps_five_minus_max_num_subblock_merge_cand < 5 )

MaxNumSubblockMergeCand = 5 − sps_five_minus_max_num_subblock_merge_cand (88)

else

MaxNumSubblockMergeCand = sps_sbtmvp_enabled_flag  &&  ph_temporal_mvp_enabled_flag

Some participants said that it was intentional not to want to consider ph_temporal_mvp_enabled_flag.

There was no agreement that the proposal was needed, so no action was recommended.

- On PDPC control

  - Add an SPS flag to control PDPC on/off and check it in intra prediction process. (S0072, S0136)

    - For all color formats, add an SPS PDPC disabled flag (named sps_pdpc_disabled_flag) (S0072 method 1), or an SPS PDPC enabled flag (named sps_pdpc_enabled_flag). (S0136)

    - Only for 4:4:4 and 4:2:2 cases, add an SPS PDPC disabled flag (named sps_pdpc_disabled_flag). (S0072 method 3)

  - Add a PDPC constraint flag in the general constraint information (GCI) syntax structure. (S0136, S0072 v2)

There is currently no high-level control syntax for PDPC, although there is some form of on-off switching for it. The proponent indicated that being able to disable it could provide a benefit in some cases (e.g., for SCC and 4:4:4 – as much as 2% for an SCC sequence).

It was commented that adding this control would involve a low-level change.

It was noted that approach a.ii would not affect the Main 10 (4:2:0) profile.

It was commented that HMVP also doesn’t have a high-level flag.

See also the next item.

Revisit.

- On intra reference sample filtering control

  - Add an SPS flag to control intra reference sample filtering on/off and check it in intra prediction process. (S0072, S0150)

    - For all color formats, add an SPS disabled flag (named sps_intra_reference_filter_disabled_flag). (S0072 method 2, S0150)

      - Furthermore, add sps_intra_smoothing_disabled_present_in_sh_flag when sps_intra_reference_filter_disabled_flag is equal to 0, and add sh_intra_smoothing_disabled_flag when sps_intra_smoothing_disabled_present_in_sh_flag is equal to 1. (S0150)

    - Only for 4:4:4 and 4:2:2 cases, add an SPS disabled flag (named sps_intra_reference_filter_disabled_flag). (S0072 method 3)

  - Add an intra reference filtering constrain flag in the GCI syntax structure to constrain the added SPS flag. (S0072 v2)

It was commented that adding this control would involve a low-level change.

Revisit with the previous item (note that S0150 is a submission submitted after the AHG meeting in May).

AHG discussion stopped here at 0920 UTC on 27 May.

This topic was discussed in the HLS AHG at 0030 on 28 May.

- On TSRC

  - Remove no_tsrc_constraint_flag. (S0073, option 3 in section 3.3)

  - Change the semantics of no_tsrc_constraint_flag (removal: italic; addition: bold):

    - Add "When no_transform_skip_constraint_flag is 1, the value of no_tsrc_constraint_flag shall be one." (S0062, method 3)

    - Change the semantics to be as follows (S0069, option 1):

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 or sps_transform_skip_enabled_flag shall be equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

    - Change the semantics to be as follows (S0069, option 2):

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 when sps_transform_skip_enabled_flag is equal to 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

    - Change the semantics to be as follows (S0105):

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

    - Add a semantic constraint to no_tsrc_constraint_flag to disable DQ and SDH in SPS level (S0073, option 1 in section 3.1)

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 and sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag are both equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

    - Add a semantic constraint to no_tsrc_constraint_flag to disable DQ and SDH in all slices (S0073, option 2 in section 3.2)

  - Infer sh_ts_residual_coding_disable_flag to be equal to !sps_transform_skip_enabled_flag instead of 0. (S0062, method 1)

  - Add an SPS flag for TSRC, use it to control the presence of sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag, and change the semantics of no_tsrc_constraint_flag to constrain the SPS flag instead of the SH flag. (S0062, method 2)

  - Change both semantics of no_tsrc_constraint_flag and sh_ts_residual_coding_disabled_flag: (S0069, option 3) as follows (removal: italic; addition: bold)

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

sh_ts_residual_coding_disabled_flag equal to 1 specifies that the syntax structure residual_coding( ) is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the syntax structure residual_ts_coding( ) is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.the following applies:

- If sps_transform_skip_enabled_flag is equal to 0, the value of sh_ts_residual_coding_disabled_flag is inferred to be equal to 1.

- Otherwise, the value of sh_ts_residual_coding_disabled_flag is inferred to be equal to 0.

A suggested principle was to not have GCI constraint flags that constrain combinations.

The no_tsrc_constraint_flag was suggested to be problem in this regard (as it disables three things: TSRC and SDH and DQ). Disabling SDH and DQ has a significant coding efficiency impact. It was thus suggested to remove this constraint flag (item “a” above).

The other aspects (some of which relate to bugs in the specification of the implications of that flag) would also be resolved by that action.

AHG Recommendation (cleanup/bug fix): Remove the no_tsrc_constraint_flag.

TBP here onwards.

- On residual coding

Add the following SPS and PH flags, and revise SH gating conditions and inferences: (S0093)

  - Addition of the SPS flag sps_residual_control_present_in_sh_flag.

  - Addition of picture header flags for DQ and SDH.

  - Modification of gating conditions for sh_dep_quant_enabled_flag, sh_sign_data_hiding_enabled_flag, and sh_ts_residual_coding_disabled_flag to ensure that at most one of these flags are signalled at the slice level.

  - Modifications of inference rules for sh_dep_quant_enabled_flag, sh_sign_data_hiding_enabled_flag, and sh_ts_residual_coding_disabled_flag to match the desired behaviour.

- On presence of explicit scaling list

Allow the aps_chroma_present_flag of the scaling list APS NAL unit referenced by the PH to be equal to 0 when ChromaArrayType is greater than 0. (S0049, aspect 9)

- On chroma QP table signalling

- Change the range of sps_num_points_in_qp_table_minus1[ i ] from 0..63 + QpBdOffset to 0..62 + QpBdOffset. (S0130 aspect 1)

- Add an SPS flag (named sps_chroma_qp_identical_to_luma_flag) to skip signalling of chroma QP information when chroma QPs are set equal to luma QPs. (S0130 aspect 2)

- On ALF APS constraints

Add an SPS flag (named sps_one_aps_per_slice_one_filter_per_aps_flag) to support the following case and change the signalling of aps_adaptation_parameter_set_id, sh_alf_aps_id_luma, sh_alf_aps_id_chroma from u(5)/u(3) to u(v). (S0134):

- Each ALF APS contains up to 1 luma filter, up to 1 chroma filter, and no cross-component filters, and each slice in the CLVS uses 0 or 1 ALF APSs for luma adaptive loop filtering, 0 or 1 ALF APSs for chroma adaptive loop filtering, and applies no cross-component adaptive loop filtering.

- Allow maximum of 115 ALF APSs.

- On deblocking filter parameters

- Inference of chroma DB offsets

    - If pps_chroma_tool_offsets_present_flag is equal to 1, infer the values of the chroma DB offsets in the PH and SH, when not present, to be equal to the chroma DB offsets in the PPS and PH, respectively, instead of inferring the values from the luma DB offsets in the same header (as in the current VVC text). Otherwise, infer the values from the luma DB offsets in the same haeder (as in the current VVC text) (S0053, aspect 1)

    - Infer the chroma DB offsets in the PH and SH to be equal to the values of the chroma DB offsets in the PPS and PH, respectively, when not present, regardless of the value of pps_chroma_tool_offsets_present_flag. But this solution is not recommended. (S0053, aspect 1, alternative solution)

- Removal the condition check of pps_deblocking_filter_override_enabled_flag for signalling the ph_deblocking_filter_override_flag. (S0121, aspect 2)

- Semantics of pps_chroma_tool_offsets_present_flag

Change the semantics such that the chroma deblocking tc and β offset syntax elements may be (instead of "are") present in the PHs or the SHs of pictures referring to the PPS when the flag is equal to 1. (S0053, aspect 2)

- Semantics of pps_chroma_tool_offsets_present_flag

Change the inference rules of non-present syntax elements PH/SH SAO, ALF, CCALF flags. (S0121, aspect 1)

- Infer ph_sao_luma_enabled_flag to be equal to sps_sao_enable_flag instead of 0.

- Infer ph_alf_enabled_flag to be equal to sps_alf_enable_flag instead of 0.

- Infer ph_sao_chroma_enabled_flag to be as follows, instead of 0:

- If sps_sao_enable_flag is equal to 1 and ChromaArrayType is not equal to 0, ph_sao_chroma_enabled_flag is inferred to be equal to 1.

- Otherwise, ph_sao_chroma_enabled_flag is inferred to be equal to 0.

- Infer ph_alf_cb_flag/ph_alf_cr_flag to be 0 if one of the following two conditions are true, and to be 1 otherwise:

- ChromaArrayType is equal to 0.

- ph_alf_enabled_flag is equal to 0.

- Infer ph_cc_alf_cb_enabled_flag/ph_cc_alf_cr_enabled_flag to be 0 if one of the following two conditions are true, and to be 1 otherwise:

- sps_ccalf_enabled_flag is equal to 0.

- ph_alf_enabled_flag is equal to 0.

- Infer sh_alf_cb(r)_flag as follows:

- If sh_alf_enabled_flag or ph_alf_cb(r)_flag is equal to 0, sh_alf_cb(r)_flag is inferred to be equal to 0.

- Otherwise (both sh_alf_enabled_flag and ph_alf_cb(r)_flag are equal to 1), sh_alf_cb(r)_flag is inferred to be equal to 1.

- Infer sh_cc_alf_cb(r)_enabled_flag as follows:

- If sh_alf_enabled_flag or ph_cc_alf_cb(r)_enabled_flag is equal to 0, sh_cc_alf_cb(r)_enabled_flag is inferred to be equal to 0.

- Otherwise (both sh_alf_enabled_flag and ph_cc_alf_cb(r)_enabled_flag are equal to 1), sh_cc_alf_cb(r)_enabled_flag is inferred to be equal to 1.

- Invocation of the decoding process for symmetric motion vector difference reference indices specified in clause 8.3.5

In clause 8.1.2, add the following text as a step after the invocation of the decoding process for generating unavailable reference pictures specified in clause 8.3.4: "At the beginning of the decoding process for each B slice, the decoding process for symmetric motion vector difference reference indices specified in clause 8.3.5 is invoked for derivation of the variables RefIdxSymL0 and RefIdxSymL1.", and at the beginning of clause, at the following: "This process is invoked at the beginning of the decoding process for each B slice, after decoding of the slice header as well as the invocation of the decoding process for reference picture list construction for the slice as specified in clause 8.3.2, but prior to the parsing and decoding of any coding unit." (S0191)

- Make either of the following changes regarding the inferences of sps_scaling_matrix_for_lfnst_disabled_fla and sps_scaling_matrix_designated_colour_space_flag: (S0196)

  - Remove the inferences of both flags, as they are asserted not used when not present.

  - Change the inferred value from 1 to 0 for both flags.

- Make one of the following changes to disallow the ACT and LFNST combination: (S0197)

  - When sps_lfnst_enabled_flag is equal to 1, sps_act_enabled_flag is not signalled and inferred to be 0.

  - When sps_act_enabled_flag is equal to 1, sps_lfnst_enabled_flag is not signalled and inferred to be 0.

  - Add a constraint: The values of sps_act_enabled_flag and sps_lfnst_enabled_flag shall not both be equal to 1.

- Signal an SPS flag to specify whether ph_joint_cbcr_sign_flag is present in PHs. When not present, the value of ph_joint_cbcr_sign_flag is inferred to be equal to 1. (S0201)

- Make the following syntax and semantics changes to infer the values of ph_dmvr_disabled_flag and ph_bdof_disabled_flag to be equal to 1 (DMVR/BDOF disabled) when the picture is not qualified to use bi-prediction, to fix the asserted issue that currently they are inferred to be equal to 0 (DMVR/BDOF enabled) when the picture is not qualified to use bi-prediction: (S0205)

Change the picture header structure syntax and semantics as follows (additions in bold and bigger fonts, removals in italic and smaller fonts):

| presenceFlag = 0 |  |
| --- | --- |
| if( ph_inter_slice_allowed_flag ) { |  |
| ... |  |
| if( sps_mmvd_fullpel_only_flag ) |  |
| ph_mmvd_fullpel_only_flag | u(1) |
| presenceFlag = 0 |  |
| if( !pps_rpl_info_in_ph_flag ) /* This condition is intentionally not merged into the next, to avoid possible interpretation of RplsIdx[ i ] being unspecified. */ |  |
| presenceFlag = 1 |  |
| else if( num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 0 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 ) |  |
| presenceFlag = 1 |  |
| if( presenceFlag ) { |  |
| ph_mvd_l1_zero_flag | u(1) |
| if( sps_bdof_control_present_in_ph_flag ) |  |
| ph_bdof_disabled_flag | u(1) |
| if( sps_dmvr_control_present_in_ph_flag ) |  |
| ph_dmvr_disabled_flag | u(1) |
| } |  |

Change the picture header structure syntax as follows:

ph_bdof_disabled_flag equal to 1 specifies that bi-directional optical flow inter prediction based inter bi-prediction is disabled and not used in the slices associated with the PH. ph_bdof_disabled_flag equal to 0 specifies that bi-directional optical flow inter prediction based inter bi-prediction is enabled and may be used in the slices associated with the PH.

When not present, the value of ph_bdof_disabled_flag is inferred as follows:

- If presenceFlag is equal to 1sps_bdof_control_present_in_ph_flag is equal to 0, the value of ph_bdof_disabled_flag is inferred to be equal to 1 − sps_bdof_enabled_flag.

- Otherwise (presenceFlag is equal to 0sps_bdof_control_present_in_ph_flag is equal to 1), the value of ph_bdof_disabled_flag is inferred to be equal to 1.

ph_dmvr_disabled_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is disabled and not used in the slices associated with the PH. ph_dmvr_disabled_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction is enabled and may be used in the slices associated with the PH.

When not present, the value of ph_dmvr_disabled_flag is inferred as follows:

- If presenceFlag is equal to 1sps_dmvr_control_present_in_ph_flag is equal to 0, the value of ph_dmvr_disabled_flag is inferred to be equal to 1 − sps_dmvr_enabled_flag.

- Otherwise (presenceFlag is equal to 0sps_dmvr_control_present_in_ph_flag is equal to 1), the value of ph_dmvr_disabled_flag is inferred to be equal to 1.

- Add sps_ltdf_enabled_flag for controlling the on/off of long deblocking filters, and add a GCI flag (no_ltdf_constraint_flag) to constrain the new sps_ltdf_enabled_flag. (S0226)

- Make either of the following changes to resolve an asserted inconsistency between the semantics of sps_mmvd_fullpel_only_flag and the related syntax parsing process: (S0227)

  - Change the syntax condition for ph_mmvd_fullpel_only_flag from "if( sps_mmvd_fullpel_only_flag )" to " if( !sps_mmvd_fullpel_only_flag )".

  - Rename sps_mmvd_fullpel_only_flag to sps_mmvd_fullpel_only_enabled_flag, and switch the phrases "uses" and "may use" for the value 1 and 0 in the semantics of this SPS flag.

JVET-S0044 AHG9: Cleanup of high-level signalling of BDOF, DMVR and PROF [M. Pettersson, R. Sjöberg, M. Damghanian, J. Enhorn, Z. Zhang, J. Ström, R. Yu, D. Liu (Ericsson)]

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 9 of this contribution belongs to this category.

JVET-S0053 AHG9: Inference of chroma deblocking offset values in the PH and SH [Z. Zhang, M. Pettersson, M. Damghanian, J. Enhorn, K. Andersson, J. Ström, R. Sjöberg, R. Yu, D. Liu (Ericsson)]

JVET-S0062 AHG9: On no_tsrc_constraint_flag [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-S0072 AHG9: On PDPC and reference sample filtering of non-420 sequences [M. G. Sarwer, Y. Ye (Alibaba)]

JVET-S0069 AHG9: General constraint information on features related to transform skip mode [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Aspect 2 of this contribution belongs to this category.

JVET-S0073 AHG9: Bug-fix of the constraint flag of VVC [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 2 of this contribution belongs to this category.

JVET-S0088 AHG9: Clean-up on derivation of the MaxNumSubblockCand [N. Park, J. Nam, H. Jang, J. Lim, S. Kim(LGE)]

JVET-S0089 AHG8: On TMVP derivation using inter-layer reference picture [N. Park, J. Nam, H. Jang, J. Lim, S. Kim(LGE)]

JVET-S0093 On slice header flags related to residual coding [J. Samuelsson, S. Deshpande, F. Bossen, A. Segall (Sharp)]

JVET-S0148 Crosscheck of JVET-S0093 (On slice header flags related to residual coding) [J. Gan (Canon)]

JVET-S0105 AHG9: Modification of general constraint information [S. McCarthy, P. Yin, T. Lu, F. Pu, W. Husak, T. Chen (Dolby)]

The aspect on changing the semantics of no_tsrc_constraint_flag belongs to this category.

JVET-S0121 AHG9: On deblocking filter, ALF, and SAO enabling flags in PH and SH [N. Hu, V. Seregin, Y. He, M. Karczewicz (Qualcomm)]

JVET-S0130 On chroma QP mapping [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0134 AHG9/AHG12: Relaxing an ALF APS constraint [A. Aminlou, M. M. Hannuksela (Nokia)]

JVET-S0136 AHG9: On signaling of PDPC enabling/disabling flag in SPS [X. Xiu, Y.-W. Chen, H.-J. Jhu, T.-C. Ma, X. Wang (Kwai)]

JVET-S0150 AHG9: On high-level syntax for smoothing intra prediction tools [A. Filippov, V. Rufitskiy, E. Alshina (Huawei)]

JVET-S0191 AHG9: On decoding process for symmetric motion vector difference reference indices [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0196 AHG9: On inference rules of the SPS scaling matrix related flags [M. G. Sarwer, Y. Ye (Alibaba)]

JVET-S0197 AHG9: On ACT and LFNST [M. G. Sarwer, Y. Ye (Alibaba)]

JVET-S0201 AHG9: On signaling the JCCR sign information in PH [C.-W. Kuo, X. Xiu, H.-J. Jhu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-S0205 AHG9: Inference value of DMVR/BDOF disabled flag in PH [K. Unno, K. Kawamura (KDDI)]

JVET-S0226 AHG9: On high level syntax for longer tap deblocking filter [A. M. Kotra, N. Hu, M. Coban, V. Seregin, J. Chen, M. Karczewicz (Qualcomm)]

JVET-S0227 AHG9: On MMVD flag in SPS and PH [Y. Kidani, K. Unno, K. Kawamura (KDDI)]

### General and misc. HLS topics (16+15)

JVET-S0139 AHG9: A summary of proposals on general and misc. HLS topics [Y.-K. Wang (Bytedance)]

This contribution was discussed in the HLS AHG meeting at 0515 UTC on 27 May and in the HLS AHG pre-meeting at ....

This contribution provides a summary of the 30 proposals on general and misc. HLS topics (the agenda item 3.1.2 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following actions are proposed:

- Remove the support of the separate colour plane coding from VVCv1, by removing the syntax elements sps_separate_colour_plane_flag and sh_colour_plane_id and the related text. If such functionality is desired in the future, the SPS and SH extension mechanisms could be used. Another option is to rename these syntax elements to be reserved bits. (S0052)

AHG Recommendation (cleanup): Remove these syntax elements and the associated decoding process as proposed.

- Add a constraint: The value of sps_field_seq_flag shall be the same in all SPSs that are referred to by CLVSs in a CVS. (S0056 aspect 1)

No action seemed necessary on this, as the decoding process is not affected.

- Add a constraint: When sps_field_seq_flag is equal to 1, all fields that compose the same frame shall refer to the same PPS. (S0056 aspect 2)

The concept of fields belonging to the same frame did not seem adequately defined, and no action was recommended on this.

- Add a constraint: When sps_video_parameter_set_id is equal to 0, parameter sets referred to by all VCL NAL units shall have the same value of nuh_layer_id as the VCL NAL units. (S0075 aspect 1 – also covered in another summary document S0141)

This was deferred for consideration in relation to S0141 regarding inference of an OLS when there is no VPS – adding a NOTE may be sufficient.

- Do the following for setting of PictureOutputFlag after receiving the first slice of a picture and before receiving other slices of the picture: (S0077)

- Add a constraint that when a picture contains a mix of RASL_NUT with NAL unit type(s) other than RADL_NUT or a mixed of RADL_NUT with other NAL unit type(s) other than RASL_NUT, then the following applies:

    - If interlace coding is not used (i.e., sps_field_seq_flag is equal to 0), there shall be at least one non-leading picture between the picture and its associated IRAP picture.

    - Otherwise, if interlace coding is used (i.e., sps_field_seq_flag is equal to 1), there shall be at least two non-leading pictures between the picture and its associated IRAP picture.

- Add a NOTE to clarify the derivation of PictureOutputFlag that when decoder received the first slice of the current picture and the NAL unit type of the slice is either RASL_NUT or RADL_NUT and the value of pps_mixed_nalu_types_in_pic_flag is equal to 1, if decoder has not received any non-leading picture from the last IRAP picture, then decoder can determine that the current picture is a RASL picture.

It was noted that PictureOutputFlag is not really needed until all slices of the picture have been decoded, and thus suggested to move the derivation of this flag to the end of the decoding process of the picture.

AHG Recommendation (cleanup): Move the derivation of this flag to the end of the decoding process of the picture.

- Invoke the derivation process for NoOutputOfPriorPicsFlag only once per AU, that is, before the decoding process of the first picture in the CVSS AU but after parsing the slice header of the first slice of the current picture. When NoOutputOfPriorPicsFlag is equal to 1, the process of removing pictures stored in the DPB without outputting them is invoked only once per access unit. (S0078)

AHG Recommendation (BF/expression of existing intent): Adopt.

- Fix an asserted bug (the derivation of prevTid0Pic that is used in the POC decoding process does not consider the value of ph_non_ref_pic_flag), by adding the following constraint: (S0081)

Let picture picA and picB be two pictures with TemporalId equal to 0, it is a constraint of conformance bitstream that DiffPicOrderCnt( picB, picA ) shall not be greater than MaxPicOrderCntLsb / 2 when all of the following conditions are satisfied:

- The values of nuh_layer_id of picA and picB are the same.

- picA and picB are not RADL picture or RASL picture.

- The value of ph_non_reference_picture_flag of picA and picB are both equal to 0.

- picB is the first picture with TemporalId equal to 0 that follows picA in output order.

An alternative approach was suggested, to modify the derivation of prevTid0Pic to consider only pictures with ph_non_ref_pic_flag equal to 0. It was commented that this approach was taken in HEVC.

It was also commented that we should do the same with prevNonDiscardablePic and in C.4 and the semantics of delta_poc_msb_cycle_present_flag.

AHG Recommendation (BF/expression of existing intent): Make the alternative suggested changes.

- When the current picture is a RADL picture, allow RASL pictures with pps_mixed_nalu_types_in_pic_flag is equal to 1 in active entries in RefPicList[ 0 ] or RefPicList[ 1 ]. (S0084, spec text ticket #1035)

AHG Recommendation (BF/expression of existing intent): Adopt (the editor may also add a NOTE to explain that such RADL pictures would still be correctly decodable when random accessing from the CRA picture, due to the existence of subpicture-level constraints).

- Do one of the following to the NOTE in the definition of RASL picture: (S0110 aspect 1)

- Remove the sentence "RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures."

- Change the sentence to be: "RASL pictures with mixed_nalu_types_in_pic_flag equal to 0 are not used as reference pictures for the decoding process of non-RASL pictures."

AHG Recommendation (BF/expression of existing intent): Adopt “b” in principle; editor to produce the precise wording.

- When aud_irap_or_gdr_au_flag is equal to 1, in the AUD syntax, optionally signal the OLS index of the OLS represented by the CVS (aud_cvs_ols_idx) and the highest temporal sublayer present in the CVS (aud_cvs_htid_plus1), each conditioned by its own presence flag, with the following additional details: (S0087)

- aud_cvs_htid_plus1 equal to 0 specifies that all the pictures in the CVS starting with the AUD NAL unit are IRAP pictures or GDR pictures with ph_recovery_poc_cnt equal to 0. aud_cvs_htid_plus1 greater than 0 specifies that all the pictures in the CVS starting with the AUD NAL unit have TemporalId less than aud_cvs_htid_plus1.

- aud_cvs_ols_idx specifies that the CVS starting with the AUD NAL unit does not contain any other layers than those included in the OLS with OLS index equal to aud_cvs_ols_idx.

- When aud_cvs_htid_plus1 is present, it is used to derive the value of Htid. Otherwise, Htid is set by external means (like in VVC Draft 9).

- When aud_cvs_ols_idx is present, it is used to derive the value of TargetOlsIdx. Otherwise, TargetOlsIdx is set by external means (like in VVC Draft 9).

- The sub-bitstream extraction process is modified so that the highest temporal sublayer and the target OLS given as inputs to the process are used in modifying the AUD NAL units in the output sub-bitstream.

It was noted that this requires AUD syntax rewriting for extraction.

A participant suggested to consider using the AUD content only if such information is not provided by external means. Others said that any external means and the AUD content (if present) should be required to match each other. Another participant said this could have a similar concept as for parameter sets – it needs to be available – either in the bitstream or by external means.

Another participant commented that dropping of some NAL units may cause mismatch, although we would not need to specify a decoder response to accidental dropping that produces non-conforming bitstreams.

The spirit is to allow the information to be signalled “in band”. If not present, the current scheme would apply such that external means is needed.

Something similar had been proposed at the January 2020 (Brussels) meeting in JVET-Q0256, using a different NAL unit rather than the AUD. This proposal would make the AUD have variable length.

It was commented that having a different NAL unit might help deal with the rewriting issue.

This was deferred for further study until the main meeting to propose an approach with a distinct NAL unit and text such that the information needs to be available – either in the bitstream or by external means.

- Add a rule to select among OLSs matching BitstreamToDecode: When TargetOlsIdx is not provided via external means, it is set equal to the OLS with the lowest index that contains all layers of AU 0 of BitstreamToDecode and the highest number of output layers. (S0103 aspect 1)

Currently, there is no default, and TargetOlsIdx needs to be provided.

This was deferred for further study until the main meeting in conjunction with the previous item above.

- Do one of the following to enable setting of PictureOutputFlag equal to 0 for pictures in CVSS AUs (for correct picture output behavior when random accessing from a DRAP AU): (S0104)

- Add sps_pic_in_cvss_au_no_output_flag, which equal to 1 specifies that a picture in a CVSS AU referring to the SPS is not output (regardless of the ph_pic_output_flag value in the PH).

- Add an external-means-determined variable NoOutputBeforeDrapFlag for CVSS AUs. PictureOutputFlag is set equal to 0 for pictures in CVSS AUs with NoOutputBeforeDrapFlag equal to 1.

It was asked whether this proposed behaviour is consistent with DRAP being only an SEI message, since this is proposing a change in the main body. DRAP was also an SEI message in HEVC.

It was commented that the distinction between “output” and “display” might allow a system to not display an IRAP picture although the decoder would conceptually “output” it.

It was commented that DRAP could be used for various purposes – not just recovery but also for fast-forward.

Option “b” might be considered similar in spirit with some of these suggestions.

It was commented that it had been intended for DRAP to not affect the main spec, and it seemed too late to change that intent.

No action was recommended for this (within the scope of v1).

- In order to enable signalling the picture rate information without requiring signalling a complete HRD model, allow general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag to be both equal to 0, and when that condition is true, only signal the following HRD parameters syntax elements: num_units_in_tick, time_scale, general_same_pic_timing_in_all_ols_flag, fixed_pic_rate_general_flag[ i ], fixed_pic_rate_within_cvs_flag[ i ], and elemental_duration_in_tc_minus1[ i ]. (S0109)

It was asked whether these can be sent in AVC and HEVC without HRD parameters, and the proponent said this was possible. It was also asked what these would mean if fixed_pic_rate_general_flag[ i ] and fixed_pic_rate_within_cvs_flag[ i ] are equal to 0.

HEVC has a specification of a correspondence between POC and timing in a special case.

It was commented that the semantics of some of these parameters depend on the HRD specification, so the parameters would not make sense without the HRD model (as currently specified).

This was deferred for further study to determine whether it is feasible and desirable to signal picture rate information in the bitstream without signalling HRD parameters in the bitstream.

- Add a prediction constraint on the pictures following a recovery point picture in both decoding and output order, as follows: When the current picture follows a recovery point picture having the same value of nuh_layer_id in both decoding order and output order, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes that recovery point picture in output order or decoding order of the associated GDR picture in the layer with the nuh_layer_id equal to the nuh_layer_id of the reference picture. (S0114)

It was commented that this would prohibit using “clean” areas of “partially dirty” pictures prior to the recovery point. This seems against the spirit of the intent of the GDR design, so no action was recommended on this.

- Make the following changes: (S0124)

- Add derivation of TemporalId, ph_non_ref_pic_flag, and parameter sets to the decoding process for generating unavailable reference pictures (in order to enable checking of some constraints for them).

- Remove the constraint that each inter-layer reference picture entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

- Invoke the decoding process for generating unavailable reference pictures for ILRPs or constrain that ILRPs shall not be equal to "no reference picture".

It was commented that aspects b and c are not necessary. Part of the motivation for item b is to enable more use of signalling the RPL in the PH or SPS rather than SH – e.g., for bit savings.

As proposed “parameter set information” was vague, and specifying inference of ph_pic_parameter_set_id instead was suggested.

AHG Recommendation (BF/expression of existing intent): Adopt aspect a, modified as described.

(Note: This may also need to be an errata item for HEVC.)

- Move the signalling of WPP enabling flag and the entry point offsets present flag from SPS to PPS, and enable the signalling of only the tile entry points when both tiles and WPP are enabled. (S0128)

Part of the motivation of the first proposed change is to enable the second proposed change.

No strong need for action was identified on this, so no action was recommended.

TBP here onwards.

- Make the following changes on end of sequence (EOS) NAL units (S0155):

  - In clause 3 (Definitions), the NOTE as part of the definition of CLVS picture, the phrase "the first PU in the layer of the bitstream that follows an EOS NAL unit in decoding order", change "EOS NAL unit" to "EOS NAL unit in the layer".

  - In clause 7.4.2.2 (NAL unit header semantics), instead of requiring the nuh_layer_id of an EOS NAL unit to be equal to the nuh_layer_id of the associated VCL NAL unit, it is specified that the nuh_layer_id of an EOS NAL unit shall be equal to one of the nuh_layer_id values of the layers present in the CVS.

  - In clause 7.4.2.4.3 (Order of PUs and their association to AUs), it is specified that, when present, the next PU of a particular layer after an EOS NAL unit that belongs to the same layer shall be an IRAP or GDR PU.

  - In clause 7.4.2.4.4 (Order of NAL units and coded pictures and their association to PUs), it is specified that, when an EOS NAL unit is present in a PU, it shall be the last NAL unit among all NAL units within the PU other than other EOS NAL units (when present) or an EOB NAL unit (when present).

  - In clause 7.4.3.10 (End of sequence RBSP semantics), it is specified that, when present, the EOS RBSP specifies that the next subsequent PU that belongs to the same layer as the EOS NAL unit in the bitstream in decoding order (if any) is an IRAP or GDR PU.

- Add either of the following constraints related to EOS NAL units in multi-layer bitstreams (S0174 aspect 2):

- The picture referred to by each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture shall not be a RASL picture when the associated CRA has NoOutputBeforeRecoveryFlag equal to 1 and the current picture is not a RASL picture.

- It is a requirement of bitstream conformance that when a layer k that depends on a layer m and the layer m contains an EOS NAL unit in AU n, layer k shall also contain an EOS NAL in AU n or the next AU n + 1 must contain a CLVSS picture for layer k.

- In clause A.4.2 (Profile-specific level limits), change the definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of slices i.e., items c and d, from being based on MaxSlicesPerPicture and the maximum picture size to be based on MaxSlicesPerPicture * ( the number of pictures in the AU ) and the maximum AU size. (S0156 aspect 1)

- In clause A.4.2 (Profile-specific level limits), change the definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of tiles i.e., items i and j, from being based on MaxTileCols * MaxTileRows and the maximum AU size to be based on MaxTileCols * MaxTileRows * ( the number of pictures in the AU ) and the maximum AU size. (S0156 aspect 2)

- In clause C.4 (Bitstream conformance), the 6th constraint, change "CpbRemovalTime[ n ] less than CpbRemovalTime[ currPic ]" to "DpbOutputTime[ n ] greater than DpbOutputTime[ currPic ]". This is asserted to be a bug, because all decoded pictures in the DPB are always decoded earlier than decoding of the current picture, and thus CpbRemovalTime[ n ] in the context is always less than CpbRemovalTime[ currPic ]. This is also asserted to be a bug in the latest HEVC spec. (S0156 aspect 3)

- Change "less than" in the following condition (that's part of the specification of the first picture of an AU) to "less than or equal to": The value of nuh_layer_id of the VCL NAL unit is less than the nuh_layer_id of the previous picture in decoding order. (S0160 aspect 5)

- Remove the following constraint from the definition of the two still picture profiles: The referenced SPS shall have max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0. (S0160 aspect 6)

- Make sure that each VUI syntax element that describes a property of the bitstream has an inferred/default value when sps_vui_parameters_present_flag is equal to 0. (S0160 aspect 8)

- Enable signalling of TargetOlsIdx and Htid in the bitstream as follows: (S0163)

- Add operation point information (OPI) NAL unit for carrying syntax elements indicating TargetOlsIdx and Htid.

- Including an extension mechanism in OPI NAL unit similar to that in DCI and all parameter sets.

- Allowing OPI NAL unit in all AUs with IRAP or GDR slices (and thus may become IRAP or GDR AUs as a result of layer and/or subpicture extraction), and requiring the content of the OPI NAL unit to be the same within a CVS.

- TargetOlsIdx and Htid remain valid until for one or more CVSs, until they are updated the next time by an OPI NAL unit or through external means.

- When both AUD and OPI NAL units are present in an AU, the OPI NAL unit shall be the next NAL unit after the AUD NAL unit. When no AUD NAL unit is present in an AU and the OPI NAL unit is present in the AU, the OPI NAL unit shall be the first NAL unit in the AU. No more than one instance of the OPI NAL unit shall be present in an AU.

- Allowing to have neither external means nor the OPI NAL unit present for the first AU of the bitstream.

- When no target OLS index is provided in the OPI NAL unit or through external means for the first AU of the bitstream, TargetOlsIdx is inferred equal to the lowest OLS index that contains the highest number of layers and the highest number of output layers among all OLSs.

- When no highest temporal ID is provided in the OPI NAL unit or through external means for the first AU of the bitstream, Htid is inferred equal to vps_ptl_max_tid[ vps_ols_ptl_idx[ TargetOlsIdx ] ].

- Assigning nal_unit_type equal to 12 (which was earlier used for RSV_IRAP_12) for the OPI NAL unit.

- TemporalId of OPI NAL unit shall be equal to 0 and TemporalId of the containing AU shall also be 0.

- Allowing any nuh_layer_id value for the OPI NAL unit. Consequently, in the sub-bitstream extraction process, the OPI NAL unit is added among the NAL units that are not removed based on nuh_layer_id values.

If item h above is adopted, the following items are also proposed:

- In the current VVC draft, if no VPS is present, then vps_ptl_max_tid[ vps_ols_ptl_idx[ TargetOlsIdx ] ] is not inferred. This document proposes to infer it equal to sps_max_sublayers_minus1 when no VPS is present.

- It is proposed to consider changing the syntax element name of vps_ptl_max_tid[ i ] and change its semantics to be more generic.

The proposed semantics are (the bolded italics text at the end of the sentence added): vps_ptl_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the level information is present in the i-th profile_tier_level( ) syntax structure in the VPS and present in the OLSs with index olsIdx such that vps_ols_ptl_idx[ olsIdx ] is equal to i.

- Make the following changes to the constraints on RPLs for RADL pictures and subpictures, considering multi-layer coding as well as its combination with subpictures (additions in bold, removals in italic):

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A RASL picture with pps_mixed_nalu_types_in_pic_flag is equal to 0

NOTE 4 – This means that an active entry of the RPLs of a RADL picture can refer to a RASL picture with pps_mixed_nalu_types_in_pic_flag equal to 1 and the referenced RASL picture may either belong to the same layer or a different layer than the layer containing the current RADL picture. However, when decoding starts from the associated CRA picture, such a RADL picture can still be correctly decoded, because the RADL subpicture(s) in that referenced RASL picture with sps_subpic_treated_as_pic_flag equal to 1 would be correctly decoded, as the RADL picture would only refer to the RADL subpictures in the referenced RASL picture, as imposed by the next constraint that disallows RADL subpictures referring to a RASL subpicture.

  - A picture that precedes the associated IRAP picture in decoding order

- When the current subpicture, with nuh_layer_id equal to a particular value layerId and subpicture index equal to a particular value subpicIdx and sps_subpic_treated_as_pic_flag[ subPicIdx ] equal to 1, is a RADL subpicture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A picture with nuh_layer_id equal to layerId for which the value of nuh_layer_id may or may not be equal to layerId containing a RASL subpicture with subpicture index equal to subpicIdx

  - A RASL picture for which the value of nuh_layer_id is not equal to layerId and the value of sps_num_subpics_minus1 is equal to 0

  - A picture that precedes the picture containing the associated IRAP subpicture in decoding order

- Make the following changes, considering the combination of GDR, subpictures, and virtual boundaries (S0168):

- (Option 1) Do nothing or rewrite virtual boundary information in the SPS in the subpicture sub-bitstream extraction process (without actual proposed changes).

- (Option 2) Add a requirement of bitstream conformance that the extracted first subpicture for the sub-bitstream shall contain an intra-coded area and the following processing are required in sub-bitstream extraction:

    - Locate the first subpicture with an intra-coded area in the GDR picture or the following recovering pictures. This located subpicture becomes the new GDR subpicture.

    - Change the value of nal_unit_type of the new GDR subpicture to GDR_NUT when the new GDR subpicture is not in the current GDR picture.

    - Rewrite the virtual boundary information in the SPS in terms of the new GDR subpicture.

    - Adjust the value of ph_recovery_poc_cnt in the PH when the new GDR subpicture is not the current GDR picture.

- (Option 3, which is on top of option 2) When sps_gdr_enabled_flag is equal to 1, add sps_gdr_subpic_intra_starting_present_flag in the SPS. In the PH, when ph_recovery_poc_cnt is greater than 0 and sps_gdr_subpic_intra_starting_present_flag is equal to 1, add ph_subpic_intra_starting_poc_cnt[ i ] for each subpicture, which specifies "the intra-coded starting point of the i-th decoded subpicture in output order".

- Add a constraint: The recovering pictures of a GDR picture shall precede the recovery point picture in decoding order. (S0188 aspect 1)

- Add a constraint: When pps_mixed_nalu_types_in_pic_flag is equal to 1 for a picture, the picture shall not be a recovering picture or the recovery point picture associated with a GDR picture. (S0188 aspect 2)

- Add a constraint: The value of recoveryPointPocVal of a layer shall be equal to or greater than the recoveryPointPocVal of a reference layer of the layer. (S0188 aspect 3)

- Move the syntax element ph_recovery_poc_cnt to be signalled after ph_poc_msb_cycle_val. (S0188 aspect 4)

- Modify the decoding process for generating unavailable reference pictures to set the values of nuh_layer_id and PicOrderCntVal of unavailable inter-layer reference pictures and mark those pictures as “used for long-term reference”. (S0192 aspect 1)

- Change the definition of still picture profiles to contain one AU only instead of one picture only. (S0091 aspect 1, S0202 aspect 2)

- If yes, add the following two constraints: (S0202 aspect 2)

    - When sps_video_parameter_set_id is not equal to 0, there shall be one OLS specified in the VPS that contains all the layer present in the bitstream and the OLS has only one output layer.

    - The value of max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] in the DPB parameters shall not be less than the maximum of direct reference layers of any layer in the bitstream.

- Add either of the following constraint for a bitstream conforming to the Main 10 Still Picture profile or Main 4:4:4 10 Still Picture profile (S0202 aspect 1):

- When sps_video_parameter_set_id is greater than 0, there shall be an OLS specified in the VPS that contains only the layer that is present in the bitstream.

- The value of sps_video_parameter_set_id shall be equal to 0.

- In the PTL syntax structure, change the signalling of sublayer_level_idc[ i ] to be in descending order of i, i.e., from the maxNumSubLayersMinus1 − 1 to 0, inclusive. (S0203 aspect 1, S0207)

- In the PTL syntax structure, change the signalling of ptl_sublayer_level_present_flag[ i ] to be in descending order of i. (S0203 aspect 2)

- In the PTL syntax structure, move the sub-profile fields to be immediately before general_level_idc, which saves two lines of syntax, "if( profileTierPresentFlag ) {" and "}". (S0216)

- Add an inference rule: When not present, ph_poc_msb_cycle_present_flag is inferred to be equal to 0. (S0239 late, bug ticket #1118)

JVET-S0052 AHG9: Removing separate colour plane coding from VVCv1 [Y.-K. Wang, Z. Deng (Bytedance), J. Boyce (Intel)]

JVET-S0056 AHG9: On field coding [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0075 AHG9: On constraints and inference values when VPS is not present [Hendry, S. Paluri, S. Kim (LGE)]

Aspect 1 of this contribution belongs to this category.

JVET-S0077 AHG9: On derivation of PictureOutputFlag for RASL [Hendry, H. Jang, J. Nam, S. Paluri, S. Kim, J. Lim (LGE)]

JVET-S0078 AHG9: On derivation of NoOutputOfPriorPicsFlag [Hendry, S. Paluri, S. Kim (LGE)]

JVET-S0081 AHG9: On non-referenced picture and POC derivation [Hendry, H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

JVET-S0084 AHG9: On RPL constraint of RADL picture [Hendry (LGE)]

JVET-S0110 AHG9: On HLS Editorial cleanup [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 1 of this contribution belongs to this category.

JVET-S0087 AHG9: On target OLS and sublayers for decoding [M. M. Hannuksela (Nokia)]

JVET-S0103 AHG9: On OLS identification [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspect 1 of this contribution belongs to this category.

JVET-S0104 AHG9: On DRAP output [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

JVET-S0109 AHG9: Signalling of picture rate [B. Heng, W. Wan (Broadcom)]

JVET-S0114 AHG9: On GDR RPL constraint [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0124 AHG9: On unavailable reference pictures [V. Seregin, Y. He, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-S0128 AHG12: On entry point signalling [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

JVET-S0155 AHG9: On EOS NAL units [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0174 AHG9: Miscellaneous cleanups [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspect 2 of this contribution belongs to this category.

JVET-S0156 AHG9: On level definitions and bitstream conformance [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspects 5, 6, and 8 of this contribution belong to this category.

JVET-S0163 AHG9: On target OLS and sublayers for decoding [M. M. Hannuksela (Nokia), R. Skupin (HHI), Hendry (LG Electronics), B. Choi (Tencent)]

JVET-S0167 AHG9: Constraints on reference picture lists of a RADL picture [L. Chen, S.-T. Hsiang, C.-W. Hsu, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0168 AHG9: On GDR subpictures for sub-bitstream extraction [L. Chen, C.-Y. Chen, C.-W. Hsu, O. Chubach, S.-T. Hsiang, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0188 AHG9: On gradual decoding refresh [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0192 AHG9: On reference picture list with generating unavailable reference picture [B. Choi, S. Wenger, S. Liu (Tencent)]

Aspect 1 of this contribution belongs to this category.

JVET-S0091 AHG8/AHG9: On constraints of still picture [K. Abe, T. Nishi, T. Toma, V. Drugeon (Panasonic)]

Aspect 1 of this contribution belongs to this category.

JVET-S0202 AHG9: On Still Picture Profile and multi layers aspect [H. Jang, Hendry, J. Nam, N. Park, S. Paluri, S. Kim, J. Lim (LGE)]

JVET-S0203 AHG9: On signalling of sublayer level idc [Hendry (LGE)]

JVET-S0207 AHG9: Order of the sublayer_level_idc[ i ] syntax elements [Z. Zhang, R. Sjöberg, M. Pettersson, M. Damghanian, J. Ström (Ericsson)]

JVET-S0216 AHG9: Parsing order of profile_tier_level( ) [K. Naser, F. Le Leannec, T. Poirier, M. Kerdranvat (InterDigital)]

JVET-S0239 AHG9: Clean-up on derivation of POC [N. Park, Hendry, J. Nam, H. Jang, J. Lim, S. Kim(LGE)] [late]

### General constraints information (GCI) (25+7)

JVET-S0138 AHG9: A summary of proposals on general constraints information [Y. He (Qualcomm), Y.-K. Wang, Z. Deng (Bytedance)]

This contribution was discussed in the HLS AHG meeting at 2110 UTC on 27 May and in the HLS AHG pre-meeting at ....

This contribution provides a summary of the 31 proposals on general constraints information (GCI) (the agenda item 3.1.3 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following actions are proposed:

On the GCI syntax structure

- Approaches to enable skipping all GCI syntax elements

  - Add a presence flag for general_constraint_info( ) syntax structure. (S0050 #1, S0127)

    - Infer the values of the GCI flags to 0. (S0050 #1a, S0127)

    - Infer the values of max_bitdepth_constraint_idc to 8 and the value of max_chroma_format_constraint_idc value to 3. (S0050 #1). Or infer the value of max_bitdepth_constraint_idc to 2 and max_chroma_format_constraint_idc to 1. (S0127)

Editor action item: Consider the name of max_bitdepth_constraint_idc to clarify its minus8 semantics.

    - Move the alignment bits outside the GCI syntax structure and change the GCI reserved bytes to bits (S0050 #1b).

  - Move the extension length indicator (gci_num_reserved_bytes) from last to first (gci_num_constraint_bytes) in the GCI syntax structure to enable skip all GCI SE signaling when all GCI SEs values are 0. The value of gci_num_reserved_bytes shall be equal to either 0 or 9. (S0092)

It was commented, and agreed, that inference rules should not really be needed; the flags would just have semantics when they are present (just an editorial matter).

It was questioned whether people would really use the flags if they can be skipped.

It was noted that the number of flags is substantial, so having some ability to skip them is desirable.

AHG Recommendation: Agree in spirit to allow some form of skipping the GCI syntax. The exact method was agreed to be for further study to be determined later.

A side activity had been conducted for the above. The result from that side activity was a design proposed in S0179: Signal the GCI syntax structure between the general PTL fields and sub-profile fields in the PTL syntax structure. Signal the gci_present_flag at the beginning of the GCI syntax structure and use it to condition the GCI fields, move the byte alignment to the end of the GCI syntax structure, and change the reserved bytes to reserved bits.

Revisit.

- Categorize GCI SEs into substructures

- Into 11 sub-structures (format, functionality, NALU, partition, pred, intra, inter, transform/quantization/residual, loop filter, layer, SEI) and add inference value in the semantics (S0105), each substructure is gated by a present flag and, when not present, the value of the corresponding constraint flag is inferred to be 0 (S0105 alt)

- Into 3 sub-structures (capability, coding tool, NALU) (S0111)

- Rearrange GCI SEs to put intra-related GCI flags together, and inter-related GCI flags together (S0129 #3)

Approach c was said to be essentially a subset of approach a.

AHG Recommendation: Adopt approach “a”.

It was later recommended that the order of GCI syntax elements should be aligned with the SPS syntax elements. This was included into the side activity for items 4 to 22, and the report of that side activity is in S0161 (see notes under item 4) below).

It was commented that we should not have category-wise skipping. Although some interest was expressed by a proponent, no action was recommended on that aspect.

New GCI SEs

- Weighted prediction

- Two GCI flag, one for each of sps_weighted_pred_flag and sps_weighted_bipred_flag (S0050 #2, S0105, S0131)

- One GCI flag for both sps_weighted_pred_flag and sps_weighted_bipred_flag (S0058)

There are currently no weighted prediction constraint flags.

AHG Recommendation: Adopt approach “b”.

- no_virtual_boundaries_constraint_flag to constrain sps_virtual_boundaries_enabled_flag (S0050 #2, S0105, S0131)

AHG Recommendation: J. Boyce was asked to coordinate further study offline and make a recommendation for items 4-22.

The side activity also included an effort trying to align the order of GCI syntax elements with the SPS syntax elements, related to item 2) above.

The outcome of the side activity was included in S0161.

Revisit.

- no_explicit_scaling_list_constraint_flag/no_scaling_list_constraint_flag to constrain

- sps_explicit_scaling_list_enabled_flag shall be equal to 0 (S0050 #2, S0058 S0066, S0067, S0105)

- and aps_params_type shall not be equal to 2 (S0066)

- no_mtt_constraint_flag to constrain SPS MTT SEs sps_max_mtt_hierarchy_depth_intra_slice_luma, sps_max_mtt_hierarchy_depth_inter_slice and sps_max_mtt_hierarchy_depth_intra_slice_chroma (S0058)

- max_luma_transform_size_32_constraint_flag or no_luma_transform_size_64_constraint_flag to constrain sps_max_luma_transform_size_64_flag (S0058, S0066)

- no_long_term_ref_pic_constraint_flag to constrain sps_long_term_ref_pics_flag (S0058)

- max_log2_ctu_size_constraint_idc (u(2)) to constrain sps_log2_ctu_size_minus5 (S0066)

- max_layers_constraint_idc to constrain vps_max_layers_minus1 (S0113)

- max_sublayers_constraint_idc to constrain vps_max_sublayers_minus1 (S0113)

- max_subpics_constraint_idc to constrain sps_num_subpics_minus1 (S0113)

- no_pic_partition_constraint_flag to constrain pps_no_pic_partition_flag (S0113)

- no_rectangular_slice_constraint_flag to constrain pps_rect_slice_flag (S0113)

- one_slice_per_subpicture_constraint_flag to constrain pps_single_slice_per_subpic_flag (S0113)

- no_conformance_window_constraint_flag to contrain sps_conformance_window_flag (S0131)

- general_single_ols_per_layer_set_constraint_flag to constraint no two OLSs contain the same layer set (S0103#2)

- no_scalability_constraint_flag to constraint scalable and layered coding (S0105)

- no_360_video_constraint_flag to constrain sps_ref_wraparound_enabled_flag, and equirectangular projection SEI messages or generalized cubemap projection SEI messages (S0105)

- lossless_coding_constraint_flag to constrain all picture are losslessly coded (S0054 #8)

- no_scc_constraint_flag to constrain sps_ibc_enabled_flag, sps_bdpcm_enabled_flag and sps_palette_enabled_flag (S0105)

- no_VUI_constraint_flag to constrain sps_vui_parameters_present_flag (S0113)

- SEI related new GCI flags

- no_scalable_nesting_SEI_constraint_flag to constrain scalable nesting SEI (S0105)

- no_subpic_level_SEI_constraint_flag to constrain subpicture level SEI (S0105)

- no_filler_payload_SEI_constraint_flag to constrain filler payload SEI (S0105)

- no_user_data_reg_SEI_constraint_flag to constrain user data registered by Recommendation ITU T.35 SEI (S0105)

- no_user_data_unreg_SEI_constraint_flag to constrain data unregistered SEI (S0105)

- no_film_grain_SEI_constraint_flag to constrain film grain characteristics SEI (S0105)

- no_parameter_set_incl_SEI_constraint_flag to constrain parameter sets inclusion indication SEI (S0105)

- no_decoded_picture_hash_SEI_constraint_flag to constrain decoded picture hash SEI (S0105)

- no_mcdv_SEI_constraint_flag to constrain mastering display colour volume SEI (S0105)

- no_cll_SEI_constraint_flag to constrain content light level SEI (S0105)

- no_drap_sei_constraint_flag to constrain DRAP indication SEI (S0105, S0113)

- no_alt_transfer_char_SEI_constraint_flag to constrain alternative transfer characteristics SEI (S0105)

- no_ambient_view_envir_SEI_constraint_flag to constrain ambient viewing environment SEI (S0105)

- no_ccv_SEI_constraint_flag to constrain content colour volume SEI (S0105)

- no_omni_video_SEI_constraint_flag to constrain ERP, GCMP, sphere rotation, region-wise packing and omnidirectional viewport SEI (S0105, S0113)

- no_field_frame_SEI_constraint_flag to constrain frame-field information SEI (S0105)

- no_sar_SEI_constraint_flag to constrain sample aspect ratio SEI (S0105)

- general_non_HRD_constraint_flag to constrain BP, PT and DU SEI (S0113)

No action was recommended on these SEI-message-related constraints.

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Subpicture, slices, and tiles related

- When one_tile_per_pic_constraint_flag is equal to 1, NumTilesInPic shall be equal to 1. (S0050 #3b)

This proposal seemed editorial.

Editor action item: Check/clarify as appropriate.

- When one_subpic_per_pic_constraint_flag is equal to 1,

- sps_num_subpics_minus1 shall be equal to 0. (S0050 #3c, S0112)

Aspect “a” seemed editorial.

Editor action item: Check/clarify aspect “a” as appropriate.

- sps_subpic_info_present_flag shall be equal to 0. (S0105)

It was commented that aspect “b” would interfere with some BEAM usage involving the sh_subpic_id, so no action was recommended on that.

- no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1 (S0054 #3, S0106 #5, S0112, S0131)

Some reluctance was expressed for aspect “c” in regard to BEAM. We often allow tools to be not used without requiring associated constraint flags to be equal to 1, so no action was recommended on that.

It was noted that the software has some configuration issues for the constraint flags, such that it tends to set the constraint flags whenever some feature is not being used. This could cause splicing/concatenation problems. The software coordinator suggested to have configuration support for multiple modes for these flags in the software – e.g. a) set the flag if the feature is disabled, b) explicitly control the value of the flag. Volunteers for this software work were solicited.

- When one_slice_per_pic_constraint_flag is equal to 1, pps_rect_slice_flag shall be equal to 1. (S0050#5c, S0106 #6)

Such a change did not seem necessary, so no action was taken on this.

- When one_slice_per_pic_constraint_flag is equal to 1, pps_num_slices_in_pic_minus1 shall be equal to 0. (S0050#5c)

Such a change did not seem necessary, so no action was taken on this.

Editor action item: Consider clarifying semantics – e.g., when pps_rect_slice_flag is 1, then pps_num_slices_in_pic_minus1 needs to be equal to 0 (which is already required).

- When one_tile_per_pic_constraint_flag and one_slice_per_pic_constraint flag are both equal to 1, the value of pps_no_pic_partition_flag shall be equal to 1. (S0050 #5, S0112)

Such a change did not seem necessary, so no action was taken on this.

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Single-layer, single-picture related

- Change the semantics of single_layer_constraint_flag equal to 1 from "single_layer_constraint_flag equal to 1 specifies that sps_video_parameter_set_id shall be equal to 0" to one of the following: (S0090)

- Option 1: single_layer_constraint_flag equal to 1 specifies that one of the following conditions shall be fulfilled (1) sps_video_parameter_set_id is equal to 0; (2) The value of vps_max_layers_minus1 in the referred VPS is equal to 0 and all VCL NAL units in the CVS have the same value of nuh_layer_id.

- Option 2: single_layer_constraint_flag equal to 1 specifies that all VCL NAL units in the CVS shall have the same value of nuh_layer_id.

Currently this flag can only be set to 1 if there is no VPS. These proposals allow it to be set to 1 under additional circumstances. Option 2 fits the name of the flag, but there was the question of what is the intent for the use of the flag. Is it for not using the VPS or is it for just indicating having one layer? Some participants said the intent for adoption of the flag was consistent with option 2.

AHG Recommendation: Adopt option 2.

- When single_layer_constraint_flag is equal to 1, all_layers_independent_constraint_flag shall be equal to 1. (S0050 #4, S0054 #6)

Such a change did not seem necessary, so no action was taken on this.

- When general_one_picture_only_constraint_flag is equal to 1,

- no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1 (S0050 #4)

- no_trail_constraint_flag shall be equal to 1 (S0050 #4, S0054 #5)

- no_stsa_constraint_flag shall be equal to 1 (S0050 #4, S0054 #5)

- no_rasl_constraint_flag shall be equal to 1 (S0050 #4, S0054#5)

- no_radl_constraint_flag shall be equal to 1 (S0050 #4, S0054 #5)

The first picture in the bitstream needs to be IRAP or GDR.

In the current draft, we cannot indicate conformance to the Main 10 Still Picture profile without sending the constraint flag. There is also a max dec pic buffering issue.

In HEVC, the Main Still Picture profile has its own value of profile_idc, and there were profile compatibility flags, which are not present in VVC.

AHG Recommendation: Use a different profile_idc for Main 10 Still Picture (we already require Main 10 profile decoders to decode such bitstreams). Do not require the constraint flag to be 1 as part of the profile definition.

It was asked whether we need the general_one_picture_only_constraint_flag.

Further study was recommended to study the final signalling relationship for the still picture profile(s).

It was noted that there is also a proposal S0129 to have a one-picture-only indication in the SPS.

It was commented that all expressed constraints on the values of GCI flags should be removed, as these should just be specified with one-way meaning.

AHG Recommendation: Remove all constraints that require GCI flags to be equal to a value that imposes a constraint.

AHG discussion stopped here 2315 UTC Wednesday 27 May.

TBP here onwards.

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Intra only related

- When intra_only_constraint_flag is equal to 1,

- all_layers_independent_constraint_flag shall be equal to 1 (S0050 #4, S0106 #4. S0112)

- no_ref_pic_resampling_constraint_flag shall be equal to 1 (S0050 #4, S0112, S0131)

- no_res_change_in_clvs_constraint_flag shall be equal to 1 (S0050 #4)

- no_sbt_constraint_flag shall be equal to 1 (S0050 #4, S0060, S0106 #1, S0112, S0129 #3, S0131)

- no_trail_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_stsa_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_rasl_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_radl_constraint_flag shall be equal to 1 (S0106 #2, S0112)

- no_cra_constraint_flag shall be equal to 1 (S0106 #3)

- no_gdr_constraint_flag shall be equal to 1 (S0106 #3)

- no_idr_constraint_flag shall be equal to 0 (S0106 #3)

- no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1 (S0112)

- sps_weighted_pred_flag shall be equal to 0. (S0061)

- sps_weighted_bipred_flag shall be equal to 0. (S0061)

- sps_long_term_ref_pics_flag shall be equal to 0. (S0061)

- sps_inter_layer_ref_pics_present_flag shall be equal to 0. (S0061)

- sps_idr_rpl_present_flag shall be equal to 0. (S0061)

- sps_rpl1_same_as_rpl0_flag shall be equal to 0. (S0061)

- sps_num_ref_pic_lists[ i ] shall be equal to 0. (S0061)

- sps_sbtmvp_enabled_flag shall be equal to 0. (S0061)

- sps_six_minus_max_num_merge_cand shall be equal to 0. (S0061)

- sps_log2_parallel_merge_level_minus2 shall be equal to 0. (S0061)

- sps_explicit_mts_inter_enabled_flag shall be equal to 0. (S0061)

- pps_rpl1_idx_present_flag shall be equal to 0 (S0061)

- pps_rpl_info_in_ph_flag shall be equal to 0 (S0061)

- pps_ref_wraparound_enabled_flag shall be equal to 0 (S0061)

- pps_output_flag_present_flag shall be equal to 0 (S0112)

- ph_pic_output_flag shall be equal to 1 (S0112)

- ph_inter_slice_allowed_flag shall be equal to 0 (S0061, S0112)

- ph_non_ref_pic_flag shall be equal to 1 (S0112)

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Random access related

- When no_cra_constraint_flag is equal to 1, no_rasl_constraint_flag shall be equal to 1. (S0054 #4, S0112)

- When no_idr_constraint_flag is equal to 1 and no_cra_constraint_flag is equal to 1, no_radl_constraint_flag shall be equal to 1. (S0054 #4)

- At least one of the no_gdr_constraint_flag, no_idr_constraint_flag, and no_cra_constraint_flag shall be equal to 0. (S0050 #5, S0112)

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: APS related

- When no_alf_constraint_flag is equal to 1, aps_params_type shall not be equal to 0. (S0066)

- When no_lmcs_constraint_flag equal to 1, aps_params_type shall not be equal to 1. (S0066)

- When no_aps_constraint_flag is equal to 1,

- no_lmcs_constraint_flag shall be equal to 1. (S0050 #4, S0067#1, S0105)

- no_ccalf_constraint_flag shall be equal to 1. (S0050 #4)

- no_explicit_scaling_list_constraint_flag shall be equal to 1. (S0050 #4, S0067 #2, S0105)

- sps_ccalf_enabled_flag, ph_num_alf_aps_ids_luma, ph_alf_cb_flag, ph_alf_cr_flag, sh_num_alf_aps_ids_luma, sh_alf_cb_flag, and sh_alf_cr_flag all be equal to 0. (S0050 #5, S0073 #1.1)

- (alternative to 15.d and 15.f) sps_chroma_alf_enabled_flag (newly added as part of this item), sps_chroma_alf_enabled_flag and sh_num_alf_aps_ids_luma shall be equal to 0 (S0073 #1.2)

- (alternative to 15.d and 15.e) sps_alf_enabled_flag and sps_ccalf_enabled_flag shall both be equal to 0 (S0073 #1.3)

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Video format related

- The value of the u(4)-coded max_bitdepth_constraint_idc shall be in the range of 0 to 8, inclusive. (S0046 #1, S0050 #3a)

- When max_chroma_format_constraint_idc is equal to 0, no_chroma_qp_offset_constraint_flag shall be equal to 1. (S0046 #3, S0060, S0112, S0131)

- When max_chroma_format_constraint_idc is equal to 0, no_act_constraint_flag shall be equal to 1. (S0131)

GCI SEs syntax/semantics changes and constraining GCI and non-GCI SE values based on values of GCI SEs: Others

- When no_ref_pic_resampling_constraint_flag to 1, no_res_change_in_clvs_constraint_flag shall be equal to 1. (S0054 #7, S0105, S0112, S0131)

- When no_transform_skip_constraint_flag is equal to 1, no_bdpcm_constraint_flag shall be equal to 1. (S0050 #4, S0059, S0069#1, S0105, S0112, S0131)

- When no_affine_motion_constraint_flag is equal to 1, no_prof_constraint_flag shall be equal to 1. (S0046 #2, S0112, S0131)

- When all_layers_independent_constraint_flag is equal to 1, the value of sps_inter_layer_ref_pics_present_flag shall be equal to 0. (S0050 #5)

GCI and profiles

- Either change general_one_picture_only_constraint_flag to general_one_au_only_constraint_flag. (S0091 #1) or remove general_one_picture_only_constraint_flag (S0160 #7)

- Constrain the values of max_chroma_format_constraint_idc and max_bitdepth_constraint_idc in main 10 profile. (S0094)

GCI editorial changes

- Rename "max_bitdepth_constraint_idc" to "max_bitdepth_minus8_constraint_idc". (S0105)

- Rename "general_non_packed_constraint_flag" to "general_non_packed_SEI_constraint_flag", and "general_non_projected_constraint_flag" to "general_non_projected_SEI_constraint_flag". (S0105)

- Move the constraint "When general_frame_only_constraint_flag is equal to 1, the value of sps_field_seq_flag shall be equal to 0" from SPS semantics to GCI semantics (S0054 #1, S0105, S0209 #2) and/or rename general_frame_only_constraint_flag" to "no_field_seq_constraint_flag" and change the semantics to constrain sps_field_seq_flag (S0195 #2).

- Move the constraint "When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of pps_mixed_nalu_types_in_pic_flag shall be equal to 0" from PPS semantics to GCI semantics. (S0054 #2, S0209 #2)

- Replace the while-loop for GCI byte alignment with 6 zero-valued bits. (S0209 #3)

- Prefix all syntax elements in the general constraint information with 'gci_'. (S0209 #4)

- Rename "no_tsrc_constraint_flag" to "no_ts_rrc_constraint_flag" (S0238 late)

New submissions (for the June 10th submission deadline) on adding new GCI fields, or removing or changing existing GCI fields

- Remove general_non_packed_constraint_flag and general_non_projected_constraint_flag (S0195 #1)

- Add new GCI flags

  - no_subpic_info_constraint_flag to constrain sps_subpic_info_present_flag (S0195 #3)

  - no_idr_rpl_constraint_flag to constrain sps_idr_rpl_present_flag (S0195 #3)

  - no_hrd_constraint_flag to constrain vps_general_hrd_params_present_flag and sps_general_hrd_params_present_flag (S0195 #3)

  - no_bidirectional_prediction_constraint_flag to constrain pps_rpl1_idx_present_flag and num_ref_entries[ 1 ][ rplsIdx ] (S0209 #1)

  - no_parallel_merge_constraint_flag to constrain sps_log2_parallel_merge_level_minus2 (S0209 #1)

  - no_switchable_cabac_init_constraint_flag to constrain pps_cabac_init_present_flag (S0209 #1)

  - no_entropy_coding_sync_constraint_flag to constrain sps_entropy_coding_sync_enabled_flag (S0230)

JVET-S0046 AHG9: General constraint information semantics constraints [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, J. Enhorn, Z. Zhang, J. Ström, D. Liu (Ericsson)]

JVET-S0050 AHG9: On general constraints information [Z. Deng, Y.-K. Wang, L. Zhang, K. Zhang, K. Fan (Bytedance)]

JVET-S0054 AHG9: Clean-ups on general constraint flags [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0058 AHG9: Additional General Constraint Flags [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-S0059 AHG9: Fixing the Semantics of no_bdpcm_constraint_flag [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-S0060 AHG9: Fixing the Semantics of no_chroma_qp_offset_constraint_flag and no_sbt_constraint_flag [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-S0061 AHG9: HLS Cleanup of All-Intra Profile [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

JVET-S0066 AHG9: On constraint flags [C. Rosewarne, J. Gan (Canon)]

JVET-S0067 AHG9: General constraint information on features using APS [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0069 AHG9: General constraint information on features related to transform skip mode [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Aspect 1 of this contribution belongs to this category.

JVET-S0073 AHG9: Bug-fix of the constraint flag of VVC [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 1 of this contribution belongs to this category.

JVET-S0090 AHG8/AHG9: On single_layer_constraint_flag [K. Abe, T. Nishi, T. Toma, V. Drugeon (Panasonic)]

JVET-S0091 AHG8/AHG9: On constraints of still picture [K. Abe, T. Nishi, T. Toma, V. Drugeon (Panasonic)]

Aspect 2 of this contribution belongs to this category.

JVET-S0092 On constraint info signalling [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

JVET-S0094 AHG9: Profile-level constraints on chroma format and bit depth constraint flags [K. Sharman, S. Keating, A. Browne (Sony)]

JVET-S0103 AHG9: On OLS identification [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspect 2 of this contribution belongs to this category.

JVET-S0105 AHG9: Modification of general constraint information [S. McCarthy, P. Yin, T. Lu, F. Pu, W. Husak, T. Chen (Dolby)]

All aspects other than the aspect on changing the semantics of no_tsrc_constraint_flag belong to this category.

JVET-S0106 AHG9: Some cleanups on general constraint flags [G. Laroche, N. Ouedraogo, P. Onno (Canon)]

JVET-S0111 AHG9: On GCI syntax structure [Y. He, Y.-J. Chang, N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0112 AHG9: On GCI semantic constraints [Y. He, N. Hu, Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0113 AHG9: On new GCI flags [Y. He, Y.-J. Chang, N. Hu, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0127 AHG9: On general constraint info signalling [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

JVET-S0129 AHG9: cleanup on parameter sets and GCI [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

Aspect 3 of this contribution belongs to this category.

JVET-S0131 AHG9: On general constraint information syntax [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspect 7 of this contribution belongs to this category.

JVET-S0161 AHG9: Report of side activity on GCI syntax elements [J. Boyce]

JVET-S0179 AHG9: On conditional signalling of GCI fields [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang, K. Fan (Bytedance), J. Samuelsson, S. Deshpande, A. Segall (Sharp), M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm), Hendry, J. Nam (LGE), S. Wenger, B. Choi (Tencent), M. Zhou (Broadcom)]

JVET-S0195 AHG9: On GCI cleanups [Y. He, Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0209 AHG9: On General Constraint Information [M. Pettersson, R. Yu, R. Sjöberg, M. Damghanian, J. Enhorn, J. Ström, D. Liu (Ericsson)]

JVET-S0230 AHG9: New GCI flag [T. Ikai (Sharp)]

JVET-S0238 AHG9: GCI for RRC [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)] [late]

### SPS, PPS, and APS cleanups (9+8)

JVET-S0142 AHG9: A summary of proposals on SPS, PPS, and APS cleanups [Hendry (LGE)]

This contribution was discussed in the HLS AHG meeting at 2330 UTC on Wednesday 27 May and in the HLS AHG pre-meeting at ....

This contribution provides a summary of the 16 proposals on SPS, PPS, and APS cleanups (the agenda item 3.1.4 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following changes are proposed:

- It is asserted that when extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1, it is allowed that no extension_data_flag syntax elements be present. The following options for change are proposed (JVET-S0042)

  - Option 1: When an extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1, enforce the presence of at least one extension_data_flag syntax element in the syntax.

From the following design in the current spec:

| dci_extension_flag | u(1) |
| --- | --- |
| if( dci_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dci_extension_data_flag | u(1) |

To the following:

| dci_extension_flag | u(1) |
| --- | --- |
| if( dci_extension_flag ) { |  |
| do |  |
| dci_extension_data_flag | u(1) |
| while( more_rbsp_data( ) ) |  |
| } |  |

  - Option 2: Relax the semantics so that an extension_flag in DCI, VPS, SPS, PPS, or APS equal to 1 specifies that specifies extension_data_flag syntax elements may be present. (Currently, it says these flags are present.)

  - Option 3: Do nothing

It was commented that option 2 seems like just a better editorial expression of the what the current syntax already specifies.

AHG Recommendation (editorial clarification): Option 2.

Note: This should be also considered as an errata report for HEVC semantics clarification.

- Rearrange some syntax elements in SPS so that the syntax elements of similar coding tools are grouped together.

  - All of the transform related SPS syntax are placed together (JVET-S0074 aspect 1)

  - sps_lmcs_enabled_flag is signalled right after signalling of sps_ccalf_enabled_flag (JVET-S0074 aspect 1)

  - Group the dual tree related syntax elements together in the SPS (JVET-S0132 aspect 1)

  - Rearrange the partition constraint related syntax elements in the SPS to be consistent with the ordering of the corresponding syntax elements in the picture header (PH). More specifically, group such syntax in the SPS based on whether they are intra or inter related (JVET-S0132 aspect 2)

AHG Recommendation (minor cleanup of logical syntax order): Adopt. It is also desirable to have the constraint flag order match SPS order (to be considered by J. Boyce in further study).

- Not signalling sps_max_sublayers_minus1 when sps_ptl_dpb_hrd_params_present_flag is equal to 0 (JVET-S0079)

  - Move the signalling of sps_max_sublayers_minus1 and sps_reserved_zero_4bits immediately before the signalling of profile_tier_level( 1, sps_max_sublayers_minus1 )

  - Condition the presence of the two syntax elements such that they are present only when sps_ptl_dpb_hrd_params_present_flag is equal to 1.

The proponent reported that the sps_max_sublayers_minus1 value is not actually used when sps_ptl_dpb_hrd_params_present_flag is equal to 0.

It was commented that there may be some interaction with other proposals or the extraction process.

It was commented that sps_max_sublayers_minus1 could be useful to know even if not necessary for parsing and the decoding process.

No clear need for action was evident, so no action was recommended.

- Add a constraint such that when sps_video_parameter_set_id is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of sps_max_sublayers_minus1 shall be equal to the value of vps_max_sublayers_minus1 (JVET-S0115 aspect 2).

It was commented that this had been proposed in JVET-R0125 item 2. At that time a concern had been expressed about what would happen if some sublayers are removed and the SPS is rewritten.

It has been intended that it should be allowed to removed sublayers without rewriting the VPS.

AHG Recommendation (bug fix/expression of existing intent): Change the name and semantics of vps_all_layers_same_num_sublayers_flag to only control VPS syntax without requiring the number of sublayers to be the same for all layers.

No action on the proposed change, due to the agreement recorded above.

- Add new flag sps_one_picture_only_constraint_flag and use the flag to skip signalling of some syntax elements in SPS. The skipped syntax elements include inter prediction, RPL, and POC related syntax elements in both SPS and PH (JVET-S0129 aspect 2).

It was commented that this would require PH rewriting when extracting a single picture from the bitstream (e.g., to remove RPL and POC).

Concern was expressed about POC, as we allow sending POC for IDR pictures.

A similar proposal was considered at the previous meeting, for intra-only rather than for single-picture-only. The basic issues seemed similar.

It was commented that it is an encoder choice how to set the values of non-relevant syntax elements if the bitstream will contain only intra pictures or only one picture.

No clear need for action was identified, so no action was recommended.

- Constrain the value of pps_alf_info_in_ph_flag to be equal to 0 when the PH is in the SH (JVET-S0049 aspect 4)

It was asked why we hadn’t already done something like this, since we did something like this for five other similar syntax elements (see the next item for a list). There had been some argument of wanting the ALF info to be early in the header. However, it was commented that the ALF info can be in the SH already.

It was noted that another proposal has this as one of two options for action (see item 9a in the S0143 summary for S0120).

AHG Recommendation (consistency/cleanup): Adopt.

- When pps_rect_slice_flag is equal to 1 and pps_num_slices_in_pic_minus1 is equal to 0, constrain the values of the 6 PPS flags (i.e., pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag) to be all equal to 0 (JVET-S0049 aspect 5).

The motivation / justification of the proposed changes is as follows:

  - To be better aligned with the inference of the value to 0 when pps_no_pic_partition_flag is equal to 1.

  - For more optimal syntax in the case of pps_rpl_info_in_ph_flag or pps_wp_info_in_ph_flag.

  - For the case of pps_alf_info_in_ph_flag, to enable the use of ALF for the same use case that applies independent parallel encoding of different subpictures as above.

It was commented that this could require SH rewriting if there is an original bitstream and one subpicture is extracted as one picture with one slice. These flags could be in the PPS and would need to be moved to the SH.

It was commented that there is a similar issue for when pps_no_pic_partition_flag is equal to 1. However, it was commented that this flag could be set to 0.

This was deferred for further study.

- Rearrange the syntax elements related to inter slices in the picture parameter set (PPS) in a similar manner as the grouping of the inter-slice related syntax elements in the PH. More specifically, group syntax elements in PPS based on whether they are intra or inter related (JVET-S0132 aspect 3).

It was commented that this change is pretty minor and logical, and this had been tested with software. See also item 2 above.

AHG Recommendation (consistency/cleanup): Adopt.

- Currently due to existing constraints, only 3 bits out of 5 bits of APS Id are needed in current profiles. The following is proposed: split the u(5)-coded aps_adaptation_parameter_set_id into u(2)-coded aps_reserved_zero_2bits followed by u(3)-coded aps_adaptation_parameter_set_id (JVET-S0049 aspect 6).

Currently we signal the APS ID first, followed by the APS type, but the value spaces for different APS types are distinct.

It was commented that in some future profile we might want a larger range of values.

It was concluded that, as proposed, this would really be only an editorial change.

No clear need for action was evident, so no action was recommended.

- If aps_chroma_present_flag is equal to 0, due to an existing constraint, alf_luma_filter_signal_flag must be equal to 1. Based on that, it is proposed that when aps_chroma_present_flag is equal to 0, alf_luma_filter_signal_flag is skipped and inferred to be equal to 1. Otherwise, when alf_chroma_filter_signal_flag and alf_cc_cb_filter_signal_flag are both equal to 0, alf_cc_cr_filter_signal_flag is skipped and inferred to be equal to 1 (JVET-S0049 aspect 7).

No clear need for action was evident, so no action was recommended.

- It is asserted that the memory to store APSs may exceed the max memory used to store the max number of APSs in a PU due to the fact that in a PU, prefix and suffix APS NAL units with particular APS identifier and type can have different contents. The following two constraints are proposed (JVET-S0122)

  - In a PU, for an APS type, a signalled suffix APS shall not have the same identifier as a referenced APS.

It was commented that this topic had been raised the previous meeting.

It was commented that this depends on whether the decoding process is applied after receiving the VCL NAL units or after also receiving the suffix APS NAL units. If the picture is decoded before parsing the suffix APS NAL units, the memory of the prior APS content can be re-used in the on-chip memory.

No clear need for action was evident, so no action was recommended.

  - Any two suffix (prefix resp.) APS NAL units with particular APS identifier and type signalled in a PU cannot have the same APS identifier

This would disallow repetition. It was commented that such repetition is not really a problem.

No clear need for action was evident, so no action was recommended.

Review of the above items in this category completed ~0120 UTC 28 May.

TBP here onwards.

- Allow inferring some SPS syntax elements from SPS in the reference layer. It is asserted that in HEVC such mechanism is allowed (i.e., for scaling list data structure). If we allow this mechanism, the following syntax elements are proposed to be inferred from SPS in direct reference layer (JVET-S0166):

  - RPLs of the current SPS from the SPS referred to by the direct reference layer.

  - Subpicture information of the current SPS from the SPS referred to by the direct reference layer

Detail mechanism: for each category of syntax elements that can be inferred, add a new flag to specify whether the syntax elements are inferred or not. If inferred, signal additional syntax element to specify the index of the direct reference layer.

- It is asserted that the current semantics of sps_sublayer_dpb_params_flag and vps_sublayer_dpb_params_present_flag are inaccurate. It currently says that it control the presence of max_dec_pic_buffering_minus1[ i ], max_num_reorder_pics[ i ], and max_latency_increase_plus1[ i ] syntax elements. However, it actually controls the presence of those syntax elements for temporal sub-layers that are not the highest. It is proposed to update the semantics to be more accurate (JVET-S0169 aspect 1). (added or changed in bold)

sps_sublayer_dpb_params_flag is used to control the presence of max_dec_pic_buffering_minus1[ i ], max_num_reorder_pics[ i ], and max_latency_increase_plus1[ i ] syntax elements in the dpb_parameters( ) syntax strucure in the SPS for i in range from 0 to sps_max_sublayers_minus1 − 1, inclusive, when sps_max_sublayers_minus1 is larger than 0. When not present, the value of sps_sublayer_dpb_params_flag is inferred to be equal to 0.

vps_sublayer_dpb_params_present_flag is used to control the presence of max_dec_pic_buffering_minus1[ j ], max_num_reorder_pics[ j ], and max_latency_increase_plus1[ j ] syntax elements in the dpb_parameters( ) syntax strucure in the VPS for j in range from 0 to vps_dpb_max_tid[ i ] − 1, inclusive, when vps_dpb_max_tid[ i ] in VPS is larger than 0. When not present, vps_sub_dpb_params_info_present_flag is inferred to be equal to 0.

It is noted that j instead of i is used for specifying the range of syntax element array max_dec_pic_buffering_minus1[ ], max_num_reorder_pics[ ], and max_latency_increase_plus1[ ] in order to avoid confusion of the i in vps_dpb_max_tid[ i ].

- Move the two syntax elements (sps_chroma_format_idc and sps_log2_ctu_size_minus5) to take the place of sps_reserved_zero_4bits. (JVET-S0186 aspect 1)

- Reserve sps_seq_parameter_set_id value equal to 15 for future extensibility. Additionally, do the same for value 63 of pps_pic_parameter_set_id. (JVET-S0186 aspect 2)

- Move the location of decoded picture buffer (DPB) parameters syntax structure and hypothetical reference decoder (HRD) parameters syntax structure to directly follow sps_ptl_dpb_hrd_params_present_flag. (JVET-S0199).

Currently only the signalling of PTL syntax structure directly follows sps_ptl_dpb_hrd_params_present_flag (i.e., early in SPS), the signalling of DPB syntax structure is in the middle of SPS, and HRD syntax structure is near the end of SPS. The proposal would move DPB and HRD syntax structures to early position in the SPS.

- Change the coding of syntax element sps_log2_transform_skip_max_size_minus2 from ue(v) to u(2). (JVET-S0204).

- Add additional conditions for the presence of the six PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag, as follows (S0164) (additions in italic):

| ... |  |
| --- | --- |
| if( !pps_no_pic_partition_flag && ( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) && pps_deblocking_filter_override_enabled_flag ) |  |
| pps_dbf_info_in_ph_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |

| ... |  |
| --- | --- |
| if( !pps_no_pic_partition_flag && ( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) ) { |  |
| pps_rpl_info_in_ph_flag | u(1) |
| pps_sao_info_in_ph_flag | u(1) |
| pps_alf_info_in_ph_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && pps_rpl_info_in_ph_flag ) |  |
| pps_wp_info_in_ph_flag | u(1) |
| pps_qp_delta_info_in_ph_flag | u(1) |
| } |  |
| … |  |

- Enable removal of parameter sets (i.e., PPS and APS) in non-output layer that is not used as reference for decoding of pictures in output layers during extraction process, as follows: (JVET-S0219 aspect 1)

- Modify the semantics of vps_max_tid_il_ref_pics_plus1[ i ][ j ] such that when it is greater than 0 it means for decoding pictures of the i-th layer, no parameter set and picture from the j-th layer with TemporalId greater than vps_max_tid_il_ref_pics_plus1[ i ][ j ] − 1 is used as reference.

- In extraction process, in addition to removing VCL NAL unit, also remove PPS and APS with TemporalId that is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

- Move the signalling of aps_params_type to an earlier position and change the signalling of aps_adaptation_parameter_set_id from u(5) to u(v). The length of the syntax element depends on the type of the APS. (JVET-S0219 aspect 2)

- Move the flag indicating the presence of sh_cabac_init_flag from PPS to SPS. (JVET-S0235)

JVET-S0042 JVET-S0042 AHG9: On the parameter set extension mechanism [M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)]

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspects 4 to 7 of this contribution belong to this category.

JVET-S0074 AHG9: On SPS, PH, SH syntax order [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 1 of this contribution belongs to this category.

JVET-S0079 AHG9: On signalling of sps_max_sublayers_minus1 [Hendry (LGE)]

JVET-S0115 AHG9: On SPS cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 2 of this contribution belongs to this category.

JVET-S0129 AHG9: cleanup on parameter sets and GCI [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

Aspect 2 of this contribution belongs to this category.

JVET-S0132 AHG9: On syntax signaling order in SPS and PPS [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-S0122 AHG9: On APS memory constraint [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

JVET-S0164 AHG9: On signalling six PPS flags for indicating the presence of syntax in PH/SH [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0166 AHG9: Inferring information of the SPS from the reference layer [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0169 AHG9: Miscellaneous Cleanups for HLS [B. Wang, S. Esenlik, H. Gao, E. Alshina (Huawei)]

Aspect 1 of this contribution belongs to this category.

JVET-S0186 AHG9: On SPS Cleanup [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0199 AHG9: On syntax grouping in SPS [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, C.-W. Kuo, X. Wang (Kwai Inc.)]

JVET-S0204 AHG9: On signalling of the maximum transform skip size [K. Unno, K. Kawamura (KDDI)]

JVET-S0219 AHG8/AHG9: On APS NAL unit clean-up [H. Jang, J. Nam, J. Lim, S. Paluri, Hendry, S. Kim (LGE)]

JVET-S0235 AHG9: On pps_cabac_init_flag signalling [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

### PH and SH cleanups (10+4)

JVET-S0143 AHG9: A summary of proposals on picture header and slice header cleanups [Hendry (LGE)]

This topic was discussed in the HLS AHG at 2330 on 28 May and in the HLS AHG pre-meeting at ....

This contribution provides a summary of the 13 proposals on picture header and slice header cleanups (the agenda item 3.1.5 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following changes are proposed:

- Add a constraint such that the value of ph_inter_slice_allowed_flag is required to be equal to 1 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 (JVET-S0049 aspect 8).

This proposal seemed undesirable, as it would enforce undesirable behaviour on the picture header based on layer-level characteristics.

- It is asserted there may be issue with the signalling of ph_no_output_of_prior_pics_flag. When ph_gdr_or_irap_pic_flag is equal to 0, but the associated picture the associated picture may or may not be an IRAP picture but the ph_no_output_of_prior_pics_flag is not present but there is no inference value for it. The following options are proposed to fix this asserted issue (JVET-S0055 proposal 1, JVET-S0070 proposal 1)

  - Option 1: Change the current semantics of ph_gdr_or_irap_pic_flag to be a two-way (i.e., if ph_gdr_or_irap_pic_flag is equal 1 the associated picture is a GDR or an IRAP picture, otherwise, if ph_gdr_or_irap_pic_flag is equal to 0, the associated picture is not a GDR nor an IRAP picture) (JVET-S0055 proposal 1 option 1).

  - Option 2: Apply the following (JVET-S0070 proposal 1):

    - Replace the current flag ph_gdr_or_irap_pic_flag with two new flags: ph_irap_pic_flag and ph_gdr_pic_flag

    - Semantics of ph_irap_pic_flag and ph_gdr_pic_flag are two-way.

    - ph_no_output_of_prior_pics_flag is present when at least ph_irap_pic_flag or ph_gdr_pic_flag is equal to 1

  - Option 3: Add the following inference: when not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 0. (JVET-S0055 proposal 1 option 1)

It was agreed that there is a problem. It was commented that the idea of using one-way semantics was to avoid PH rewriting for BEAM usage. It was commented that a two-flag approach had also been previously discussed.

Moving the no_output_of_prior_pics_flag to the slice header was suggested. It had previously been there.

The AHG deferred this topic for further study.

- Add a constraint such that when aud_irap_or_gdr_au_flag is present and the value of aud_irap_or_gdr_au_flag is equal to 1, the value of ph_gdr_or_irap_pic_flag shall be equal to 1. (JVET-S0055 proposal 2)

It was commented that this interacts with BEAM usage. This is related to item 2 above.

The AHG deferred this topic for further study.

- Regarding the signalling of ph_gdr_or_irap_pic_flag when pps_mixed_nalu_types_in_pic_flag is equal to 1 (JVET-S0135):

  - Option 1: add a constraint such that the value of ph_gdr_or_irap_pic_flag shall be equal to 0 when pps_mixed_nalu_types_in_pic_flag is equal to 1

  - Option 2: condition the presence of ph_gdr_or_irap_pic_flag on pps_mixed_nalu_types_in_pic_flag being equal to 0.

  - Option 3: add a constraint to the definition of GDR picture that pps_mixed_nalu_types_in_pic_flag is equal to 0.

For option 3, the proponent provided the text as a NOTE, instead of a constraint.

This is related to items 2 and 3 above.

The AHG deferred this topic for further study.

- Change coding of ph_pic_parameter_set_id from ue(v) to u(6) (JVET-S0076 item b), JVET-S0110 aspect 3)

This would add 5 bits to every picture when only 1 PPS is being used.

No strong need for action was identified, so no action was recommended for this.

- Rearrange the position of syntax elements in PH and SH.

  - Move syntax elements related to SAO and de-blocking process to earlier position in PH and in SH (i.e., signal them right after ALF and LMCS related syntax) JVET-S0074 aspect 2)

This suggestion had been discussed at the previous meeting. This would move the APS ID for the scaling list APS to later in the syntax, which is undesirable. No strong need for action was identified, so no action was recommended for this.

  - Move ph_non_ref_pic_flag to earlier position, i.e., before any syntax element whose presence is conditioned on the value of other syntax element(s). Immediately after ph_gdr_or_irap_pic_flag (JVET-S0076 item a) )

AHG Recommendation (cleanup): Adopt.

  - Move ph_pic_parameter_set_id to earlier position (i.e., after ph_non_ref_pic_flag) (JVET-S0076 item c) )

This was withdrawn due to lack of action on item 5.

  - Move down the location of syntax elements ph_inter_slice_allowed_flag and ph_intra_slice_allowed_flag and condition the presence of ph_inter_slice_allowed_flag only for picture with no inter-layer prediction and non-irap picture and non-gdr picture when ph_recovery_poc_cnt is equal to 0 (JVET-S0070 proposal 2)

It was commented that this would require some SPS parsing for interpretation. There was discussion over whether this would be of interest to the system level.

It could save one or two bits per picture under some circumstances.

There seemed to be no clear need for action on this, so no action was recommended.

Discussion in the HLS AHG stopped here at 0020 UTC on 29 May.

- If item 5.c) is agreed, condition the presence of ph_gdr_pic_flag such that it may be present only when sps_gdr_enabled_flag is equal to 1 (JVET-S0076 item d) )

- Change the constraint on the value of sh_picture_header_in_slice_header_flag as follows: the value of sh_picture_header_in_slice_header_flag shall be equal to 0 when sps_subpic_info_present_flag is equal to 1 and pps_no_pic_partition_flag is equal to 0 (JVET-S0120 aspect 1)

- Change the constraint on the value of sh_picture_header_in_slice_header_flag as follows: (JVET-S0120 aspect 2 and 3)

  - Option 1: the value of sh_picture_header_in_slice_header_flag shall be equal to 0 when  The value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, pps_alf_info_in_ph_flag, or pps_qp_delta_info_in_ph_flag is equal to 1.

  - Option 2: remove the following constraint: “the value of sh_picture_header_in_slice_header_flag shall be equal to 0 when  The value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, or pps_qp_delta_info_in_ph_flag is equal to 1”

This topic was closed; see notes for S0142.

- Change VTM encoder to signal the override flag (i.e., ph_partition_constraints_override_flag) as zero when the derived partition constraints for a PH are identical to the ones in the SPS (JVET-S0133)

- Change the semantics of the signalled values of partition constraints in PH from values for overriding the corresponding values signalled in SPS to the delta values relative to the corresponding values signalled in the SPS (JVET-S0133)

The results reportedly show that the encoder-only change to remove the redundant signalling introduces -0.12% (RA) and -0.06%(LD) BD-rate reduction for class D. On top of the encoder-only change, the proposed syntax modifications introduce -0.20%(RA), -0.19% (LD) BD-rate reduction for class D. In average, the encoder-only change introduces 0.00 (AI), -0.03% (RA) and -0.14%(LDB) BD-rate changes, and on top of the encoder-only change, the proposed syntax modification introduces 0.00(AI), -0.04% (RA) and -0.xx%(LDB) BD-rate changes under CTC.

It is noted that the results show no impact on AI because the current VTM always disables the override of the partition constraints for intra pictures.

- It is asserted that reference rule for the value of ph_collocated_from_l0_flag when L0 empty and L1 non-empty is missing. Update the current inference rule for ph_collocated_from_l0_flag as follows: (JVET-S0174 aspect 3) (added/changed in bold)

ph_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. ph_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1. When not present and ph_temporal_mvp_enabled_flag and pps_rpl_info_in_ph_flag are both equal to 1, the value of ph_collocated_from_l0_flag is inferred to be equal to ( num_ref_entries[ 1 ][ RplsIdx[ 1 ] ]  = =  0 ? 1 : 0 ).

- Further study result for the item 2 above (i.e., harmonization of JVET-S0055 proposal 1 and JVET-S0070 proposal 1). Change IRAP or GDR flag in PH with the following options (JVET-S0193 aspect 1):

  - Option 1: Change the semantics of ph_gdr_or_irap_pic_flag to be two-way.

  - Option 2: Replace the flag ph_gdr_or_irap_pic_flag with two new flags: ph_irap_pic_flag and ph_gdr_pic_flag with two-way semantics.

  - Option 3: Add the following inference: when not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 1.

  - Option 4: Move no_output_of_prior_pics_flag to slice header.

- If option 3 or option 4 from the above item is agreed, change the name of the syntax element ph_gdr_or_irap_pic_flag to ph_irap_pic_flag. (JVET-S0193 aspect 1)

- Do one of the following (JVET-S0200):

  - Option 1: Add a constraint such that ph_inter_slice_allowed_flag shall be equal to 1 when ph_gdr_pic_flag is equal to 1.

  - Option 2: Condition the presence of ph_inter_slice_allowed_flag on ph_gdr_pic_flag being equal to 0.

- Add a constraint such that when vps_max_layers_minus1 is equal to 0, an AUD is present for the current AU, and aud_irap_or_gdr_au_flag is equal to 0, the value of ph_gdr_or_irap_pic_flag shall be equal to 0 (JVET-S0160 aspect 4).

- Add a constraint such that when an AUD is present for the current AU and aud_pic_type is equal to 0, the value of sh_slice_type shall be equal to 2 (JVET-S0160 aspect 2).

- Add a constraint such that when an AUD is present for the current AU and aud_pic_type is equal to 1, the value of sh_slice_type shall be equal to 1 or 2 (JVET-S0160 aspect 3).

- Add the following constraint for CTU size when using ILRP as collocated reference picture as follows (JVET-S0174 aspect 1) (addition in bold):

sh_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

[…]

Let colPicList be set equal to sh_collocated_from_l0_flag ? 0 : 1. It is a requirement of bitstream conformance that the picture referred to by sh_collocated_ref_idx shall be the same for all non-I slices of a coded picture and the value of RprConstraintsActiveFlag[ colPicList ][ sh_collocated_ref_idx ] shall be equal to 0 and the value of sps_log2_ctu_size_minus5 in the SPS referred to by the picture referred to by sh_collocated_ref_idx shall be equal to or greater than the value of sps_log2_ctu_size_minus5 in the SPS referred to by the current picture.

NOTE – The above constraint requires the collocated picture to have the same spatial resolution. the same scaling window offsets and same or larger CTU size as the current picture.

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 8 of this contribution belongs to this category.

JVET-S0055 AHG9: On signaling IRAP and GDR pictures [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0070 AHG9: On flag ph_gdr_or_irap_pic_flag [L. Chen, C.-W. Hsu, S.-T. Hsiang, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0074 AHG9: On SPS, PH, SH syntax order [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 2 of this contribution belongs to this category.

JVET-S0076 AHG9: On syntax elements in the beginning of picture header [Hendry (LGE)]

JVET-S0110 AHG9: On HLS Editorial cleanup [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 3 of this contribution belongs to this category.

JVET-S0120 AHG9: Constraints on sh_picture_header_in_slice_header_flag [N. Hu, V. Seregin, Y. He, M. Karczewicz (Qualcomm)]

JVET-S0133 AHG9: On partition constraints override in picture header [Y.-W. Chen, X. Xiu, H.-J. Jhu, T.-C. Ma, X. Wang (Kwai Inc.)]

JVET-S0135 AHG9: On IRAP and GDR picture signaling in picture header [X. Xiu, Y.-W. Chen, H.-J. Jhu, T.-C. Ma, X. Wang (Kwai)]

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspects 2 to 4 of this contribution belong to this category.

JVET-S0174 AHG9: Miscellaneous cleanups [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspects 1 and 3 of this contribution belong to this category.

JVET-S0193 AHG9: Bugfix and Cleanup on ph_no_output_of_prior_pics_flag and ph_gdr_or_irap_pic_flag [B. Choi, S. Wenger, S. Liu (Tencent), L. Chen, C.-W. Hsu, S.-T. Hsiang, O. Chubach, Y.-W. Huang, S.-M. Lei(MediaTek)]

JVET-S0200 AHG9: On ph_inter_slice_allowed_flag in GDR picture [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, C.-W. Kuo, X. Wang (Kwai Inc.)]

### Reference picture lists cleanups (8+2)

JVET-S0140 AHG9: A summary of proposals on reference picture lists [Y.-K. Wang (Bytedance)]

This contribution was discussed in the HLS AHG meeting at 0500 UTC on Thursday 28 May and in the HLS AHG pre-meeting at ....

This contribution provides a summary of the 9 proposals on reference picture lists cleanups (the agenda item 3.1.6 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner. The following changes were proposed:

- Modify the semantics of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ], such that it (S0045)

- swaps the meaning of positive and negative (consequently swaps the sign assignment of DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] in Equation 155, and change the subtraction to addition when assigning RefPicPocList[ i ][ j ] using DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ] in Equation 205.

The concept is that in other place in the spec, a sign flag value ‘1’ is associated with a negative value and ‘0’ is associated with a positive value.

- associates strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] with DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] instead of with an "i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )".

Both aspects are only editorial. The “bits on the wire” do not change.

Editor action item: The editor is strongly suggested to make the proposed changes as a more precise and natural expression of the intent.

- When sps_inter_layer_ref_pics_present_flag is equal to 1, sps_idr_rpl_present_flag is skipped and inferred to be equal to 1. (S0049 aspect 2)

The motivation is to eliminate a combination that is asserted to not make sense.

This is closely related to items 4 and 5.

It was suggested to remove the sps_idr_rpl_present_flag, as a simplification of the RPL design for IDR pictures. This would require RPL to be present even for all-intra usage, costing about one or two bits per picture to signal an empty RPL.

Another participant said there is not really a problem in the current syntax.

No clear need for action was evident, so no action was recommended.

- Add a constraint: When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, the value of sps_idr_rpl_present_flag shall be equal to 1. (S0049 aspect 2)

The motivation is to eliminate a combination that is asserted to not make sense.

It was commented that it might be desirable to be able to detect the self-contained nature of such an IDR picture.

No clear need for action was evident, so no action was recommended.

- Remove sps_idr_rpl_present_flag (i.e., always signal RPLs for IDR slices, like for non-IDR I slices), and invoke generating unavailable reference picture for IDR pictures: (S0123 approach 1)

See also items 2 and 5. No clear need for action was evident, so no action was recommended.

- As an alternative to 4), do the following and invoke generating unavailable reference picture for IDR pictures: (S0123 approach 2)

  - Do one of the following:

- Change the SH syntax to signal RPL for each IDR slice in a non-independent layer.

- Require sps_idr_rpl_present_flag or pps_rpl_info_in_ph_flag to be equal to 1 for an IDR slice when the current layer is a non-independent layer.

- Require sh_slice_type to be equal to 2 for IDR slice when sps_idr_rpl_present_flag and pps_rpl_info_in_ph_flag are both equal to 0.

  - Change the process for deriving empty RPLs when sps_idr_rpl_present_flag is equal to 0 and nal_unit_type is equal to IDR_W_RADL or IDR_N_LP to involve pps_rpl_info_in_ph_flag.

See also items 2 and 4. For the aspect “a”, no clear need for action was evident, so no action was recommended.

However, for the basic concept of needing to generate unavailable reference picture for IDR pictures, it was agreed there is an issue.

It was commented that this may only be relevant/necessary for a merge/extract scenario.

It was noted that some validity checks for entries of the RPL apply even if the picture is not an active entry.

AHG Recommendation (bug fix/expression of existing intent): Invoke the generation of unavailable reference picture for an IDR picture that has RPLs, and change the RPL constraints in clause 8.3.2 accordingly.

For aspect “b” it was agreed that there is a spec bug relating to when the RPL is in the PH.

AHG Recommendation (bug fix/expression of existing intent): Adopt aspect “b”. Editor may consider the exact expression.

- Add sps_ref_layer_idx_plus1 under the condition "if( sps_inter_layer_ref_pics_present_flag )" to enable skipping ilrp_idx[ listIdx ][ rplsIdx ][ i ] in the RPL syntax structures when the layer has only one direct reference layer, as follows: (S0082)

  - The value of ps_ref_layer_idx_plus1 shall be equal to

( NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] > 1 ) ? 0:
		DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ 0 ] + 1

  - Consequently, when sps_ref_layer_idx_plus1 is greater than 0, ilrp_idx[ listIdx ][ rplsIdx ][ i ] is skipped and inferred to be equal to sps_ref_layer_idx_plus1 − 1.

It is common for there to be only one direct reference layer. In this case it would be possible to skip the ILRP index signalled in the PH/SH (whichever has the RPL structure, if one of them does).

It was noted that something similar is done in HEVC.

It was commented that this seemed like something we would do if it was proposed a couple of meetings earlier. It would be basically a shortcut for a common use case.

The bit savings for this was not so clear, and software had not been tested.

No clear need for action was evident, so no action was recommended.

- Change ilrp_idx[ listIdx ][ rplsIdx ][ i ] to delta_ilrp_idx_minus1[ i ][ RplsIdx ][ i ] as well as the semantics such that the following part of Equation 205 (S0083):

layerIdx = DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idx[ i ][ RplsIdx ][ j ] ]
refPicLayerId = vps_layer_id[ layerIdx ]

is proposed to be changed to be as follows:

ilrpIdx = GeneralLayerIdx[ nuh_layer_id ] − ( delta_ilrp_idx_minus1[ i ][ RplsIdx ][ j ] + 1 )
layerIdx = DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrpIdx ]
refPicLayerId = vps_layer_id[ layerIdx ]

The motivation is partly to make it more possible to be able to share SPSs.

It was commented that some sharing is already enabled since the index is an index into a list of direct reference layers.

Again it was commented that this seemed like something we would do if it was proposed a couple of meetings earlier.

There was a bug noted in the proposed equations, that they should instead be more like this:

ilrpIdx = GeneralLayerIdx[ nuh_layer_id ] − ( delta_ilrp_idx_minus1[ i ][ RplsIdx ][ j ] + 1 )
refPicLayerId = vps_layer_id[ ilrpIdx ]

No clear need for action was evident, so no action was recommended.

- Fix an asserted bug (when sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ], pps_rpl1_idx_present_flag is equal to 0, and rpl_idx[ 1 ] is less than sps_num_ref_pic_lists[ 0 ], the value of rpl_idx[ 1 ] may be out of range), with either of the following two options: (S0085 aspect 1)

  - Add a constraint: When sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ], pps_rpl1_idx_present_flag shall be equal to 1.

  - Add the following to the semantics of rpl_idx[ i ]: When pps_rpl1_idx_present_flag is equal to 0, the syntax element rpl_idx[ 0 ] shall be in the range of 0 to min(sps_num_ref_pic_lists[ 0 ], sps_num_ref_pic_lists[ 1 ]) − 1 and the syntax element rpl_idx[ 0 ] is represented by Ceil( Log2(min( sps_num_ref_pic_lists[ 0 ], sps_num_ref_pic_lists[ 1 ] ) ) ) bits, otherwise (pps_rpl1_idx_present_flag is equal to 1) the syntax element rpl_idx[ i ] is represented by Ceil( Log2( sps_num_ref_pic_lists[ i ] ) ) bits.

It was commented that there is a constraint, on the value range of rpl_idx[ 1 ], that would be violated by the example for describing of the asserted but. Therefore, the asserted bug is not valid.

It was noted that a value range for a syntax element, unless explicitly said to apply only when the syntax element is present, it applies also to inferred values.

No need for an action was identified. Thus no action was taken.

- Fix an asserted bug in the syntax condition "if( !pps_rpl_info_in_ph_flag  | |  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 )" in the PH syntax (when pps_rpl_info_in_ph_flag is equal to 0 the variable RplsIdx[ 1 ] in the context is used undefined), with the following changes: (S0085 aspect 2)

  - Add ", otherwise rpl_sps_flag[ i ] is inferred to be equal to 0" to the end of inference of rpl_sps_flag[ i ].

  - Add "num_ref_entries[ listIdx ][ sps_num_ref_pic_lists[ listIdx ] ] is infered to be equal to 0." to end of the semantics of num_ref_entries[ listIdx ][ rplsIdx ].

It was commented that " if( !pps_rpl_info_in_ph_flag  | |  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 )" is equivalent to "if( !pps_rpl_info_in_ph_flag  | | ( pps_rpl_info_in_ph_flag  &&  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 ) )". Therefore, the asserted bug is not valid.

Generally, a value of a syntax element or variable, when not present, is not inferred/derived when not needed, unless it is referred in a syntax condition or other condition.

It was commented that the text has some issue.

It was initially agreed to add the derivation of the value of RplsIdx[ 1 ] in the PH semantics when pps_rpl_info_in_ph_flag is equal to 0. In later discussion, adding an “if” condition was suggested as an alternative.

Editor action item: The editor is asked to rephrase the text to avoid the problem. The suggested method is to add an extra “if” condition with a comment to explain why it is there.

- Fix an asserted bug that the inference of RplsIdx[ 0 ] is missing, by adding the following inference rule (in addition to the existing inference of RplsIdx[ 1 ]): The value of rpl_idx[ 0 ] is inferred to be equal to 0 when sps_num_ref_pic_lists[ 0 ] is equal to 1 and rpl_sps_flag[ 0 ] is equal to 1. (S0085 aspect 3, S0096 aspect 3)

AHG Recommendation (bug fix/expression of existing intent): Adopt in spirit; infer a value only when needed. Exact expression to be determined offline by the editor.

- Additionally check num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] for conditional signalling of ph_collocated_from_l0_flag and ph_mvd_l1_zero_flag to reduce the unnecessary signalling, by changing "num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0" in the syntax condition to "num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 0  &&  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0". (S0096 aspect 1)

If list 0 is empty, we should not be able to indicate that we are getting the collocated picture from that list.

List 0 could only be empty for an I slice. Usually, for an I slice, if we have a non-empty list, we would make that list 0, but we do not have a constraint that requires that. It was remarked that we might not want to establish such a constraint.

The existing syntax condition seems to have a built-in assumption that list 0 will always be non-empty. This is not a valid assumption.

AHG Recommendation (bug fix/expression of existing intent): Adopt.

- Consider the number of active entries in the reference picture list when inferring the value of sh_collocated_ref_idx so that encoder can have more flexibility to change the number of active entries in slice headers, by changing "sh_collocated_ref_idx is inferred to be equal to ph_collocated_ref_idx" to "sh_collocated_ref_idx is inferred to be equal to min( ph_collocated_ref_idx, NumRefIdxActive[ !sh_collocated_from_l0_flag ] − 1 )". (S0096 aspect 2)

The issue is not actually a bug, because the problem case is prohibited.

The usefulness of the extra flexibility would be to avoid needing to signal the RPL in the SH by allowing it to be sent in the PH in such a case. However, it was remarked that it is not clear that this would be a common case. No clear need for action was evident, so no action was recommended.

- Change the constraint that the collocated picture "shall be the same for all slices of a picture" to "shall be the same for all non-I slices of a picture". (S0096 aspect 4).

It was remarked that this seems necessary to resolve the lack of a collocated picture definition for an I slice, and that this is the same way it was done in HEVC.

AHG Recommendation (bug fix/expression of existing intent): Adopt.

The discussion of the above items of this section in the HLS AHG meeting was completed at 0830 UTC on 28 May.

TBP here onwards.

- Add a constraint such that when sps_idr_rpl_present_flag is equal to 0, pps_mixed_nalus_in_pic_flag is equal to 1, and pps_rpl_info_in_ph_flag is equal to 0, the value of nal_unit_type shall not be equal to IDR_W_RADL or IDR_N_LP. (S0159 aspect 1)

- Add a constraint such that when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, the value of sps_inter_layer_ref_pics_present_flag shall be equal to 1. (S0159 aspect 2)

- Add GDR picture with ph_recovery_poc_cnt equal to 0 (in addition to IRAP picture) to the constraint in clause 8.3.2 on RPLs based on the value of vps_max_tid_il_ref_pics_plus1[ ][ ]. (S0159 aspect 3)

- Mark each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ], of a CLVSS picture as "used for long-term reference". (S0159 aspect 4)

- When num_ref_entries[ listIdx ][ rplsIdx ] in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is equal to 0, ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] is skipped and inferred to be equal to 0. (S0182)

JVET-S0045 AHG9: On the sign of DeltaPocValSt [D. Liu, R. Sjöberg, Z. Zhang, M. Pettersson, M. Damghanian, J. Enhorn, J. Ström, R. Yu(Ericsson)]

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 2 of this contribution belongs to this category.

JVET-S0082 AHG8/AHG9: On signaling of inter-layer reference picture layer index [H. Jang, Hendry, S. Paluri, J. Nam, S. Kim, J. Lim (LGE)]

JVET-S0083 AHG8/AHG9: On signaling of ILRP layer index using delta value [Hendry, H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

JVET-S0085 AHG9: On semantics related to reference picture lists [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

JVET-S0096 AHG9: On RPL syntax and semantics [J. Chen, Y. Ye, R.-L. Liao (Alibaba)]

Aspects 1 to 4 of this contribution belong to this category.

JVET-S0123 AHG9: On reference picture list constraints [V. Seregin, Y. He, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

JVET-S0159 AHG9: Reference picture list (RPL) cleanups [Y.-K. Wang, L. Zhang, Z. Deng (Bytedance)]

JVET-S0182 AHG9: On Reference Picture List Sgnalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

### Signalling of virtual boundaries (2+5)

JVET-S0047 AHG9: Signalling of Virtual Boundary Positions [M. Damghanian, R. Sjöberg, M. Pettersson, Z. Zhang, J. Enhorn, J. Ström, R. Yu, D. Liu]

This contribution was discussed in the HLS AHG meeting at 0837 UTC on Thursday 28 May.

At the 18th JVET meeting, it was decided (in JVET-R0266 item 6) to code virtual boundary positions using ue(v) for the sps_virtual_boundary_pos_x, sps_virtual_boundary_pos_y, ph_virtual_boundary_pos_x and ph_virtual_boundary_pos_y syntax elements instead of fixed length coding using u(13). It is observed that the value of 0 is not allowed for these four syntax elements so this contribution proposes to align the semantics of these four syntax elements with other syntax elements in the specification and use the minus1 mechanism. The following changes to the syntax and semantics are proposed:

- Add “_minus1” to each of the four syntax elements sps_virtual_boundary_pos_x, sps_virtual_boundary_pos_y, ph_virtual_boundary_pos_x and ph_virtual_boundary_pos_y.

- Update the corresponding ranges and derivations in the semantics of the four syntax elements accordingly.

AHG Recommendation (cleanup): Adopt.

JVET-S0065 AHG9: On Virtual Boundary [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

This contribution was discussed in the HLS AHG meeting at 0840 UTC on Thursday 28 May.

Modifications are proposed related to virtual boundary.

The following changes were proposed:

- Proposal 1: An assertedly missing inference rule is specified for sps_virtual_boundaries_present_flag to be inferred to be equal to 0 when not present.

AHG Recommendation (bug fix/expression of existing intent): Adopt.

- Proposal 2: An asserted simplification of text is proposed for virtual boundary derivation.

This aspect is only editorial.

Editor action item: The editor is requested to consider improving the expression. It was remarked that there may be some issues with the text that was proposed.

TBP here onwards.

JVET-S0165 AHG9: Cleanup on signalling virtual boundaries [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0171 AHG9: Cleanup on virtual boundaries for SAO [S.-Y. Lin, Y.-H. Lee, J.-L. Lin, Y.-J. Chen, C.-C. Ju (MediaTek)]

JVET-S0184 AHG9: On Virtual Boundary Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0211 AHG9: Extensibility for the number of virtual boundaries [M. Damghanian, M. Pettersson, R. Sjöberg, J. Enhorn, J. Ström, R. Yu, D. Liu (Ericsson)]

JVET-S0221 AHG9: On virtual boundaries [Y.-J. Chang, M. Coban, V. Seregin, N. Hu, M. Karczewicz (Qualcomm)]

### Hypothetical reference decoder (HRD) (12+14)

See also JVET-S0098 aspects c and d in section 3.3.

JVET-S0141 AHG9: A Summary of Proposals Related to HRD [S. Deshpande (Sharp)]

This contribution was discussed in the HLS AHG meeting at 0850 UTC on Thursday 28 May and in the HLS AHG pre-meeting at ....

This contribution intends to provide a summary of proposals on core aspects of HRD (including HRD operation, related SEI message signalling and sub-bitstream extraction).

Following proposals listed in JVET-S0137-v2 section 3.1.8 are covered in this summary: JVET-S0064, JVET-S0075 (aspect 2), JVET-S0097, JVET-S0080, JVET-S0086, JVET-S0099, JVET-S0101, JVET-S0102, JVET-S0117, JVET-S0118. Additionally JVET-S0049 Item 1 is covered.

Additionally the 14 new proposals listed below in this Section starting from JVET-S0154 are also covered in this summary.

It is suggested that this summary be used for the reviewing of these proposals, such that the discussions may be done in a more structured and efficient manner. The following changes were proposed:

Related to HRD signalling and operation:

- Conditionally signal bp_sublayer_dpb_output_offsets_present_flag (and bp_dpb_output_tid_offset[ i ]) only when bp_max_sublayers_minus1 is greater than 0 and infer it as 0 otherwise?

  - Alternatively, add following constraint: when bp_max_sublayers_minus1 is equal to 0, bp_sublayer_dpb_output_offsets_present_flag shall be equal to 0. (JVET-S0064)

AHG Recommendation (bug fix/expression of existing intent): Adopt the conditional signalling.

- When sps_video_paramater_set_id is equal to 0:

AHG Recommendation (bug fix/expression of existing intent): Adopt inferences a, b, c, e, f, and g.i.

  - Infer TotalNumOlss to be equal to 1 (JVET-S0075, JVET-S0097)

  - Infer NumLayersInOls[ 0 ] to be equal to 1 (JVET-S0075, JVET-S0097)

  - Infer vps_layer_id[ 0 ] to be equal to the single value of nuh_layer_id of all the VCL NAL units. (JVET-S0097)

  - Require that parameter sets referred to shall have same nuh_layer_id value (JVET-S0075)

It was commented that this aspect is not necessary since the extraction process indirectly requires this.

Also, sharing of PSs is only allowed only if all layers are present in the OLS, which would be violated unless the requirement is true.

Editor action item: The editor is asked to consider adding some explanation to point out that this is the case. Item e below may help to make this clear.

  - Infer LayerIdInOls[ 0 ][ 0 ] to be equal to the single value of nuh_layer_id of all the VCL NAL units. (JVET-S0097)

  - Infer NumSubLayersInLayerInOLS[ 0 ][ 0 ] to be equal to sps_max_sublayers_minus1 + 1. (JVET-S0097)

  - Modify the inference rule for vps_max_sub_layers_minus1 and range of values for sps_max_sublayers_minus1 by adding quoted text and not inferring vps_max_layers_minus1 to be equal to 6? (JVET-S0097)

    - “When sps_video_parameter_set_id is greater than 0,” the value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. “When sps_video_parameter_set_id is equal to 0, the value of sps_max_sublayers_minus1 shall be in the range of 0 to 6, inclusive, and vps_max_sublayers_minus1 is inferred to be equal to sps_max_sublayers_minus1.”

or

    - Modify only the inference for vps_max_sub_layers_minus1 (JVET-S0049 Item 1, JVET-S0097) as: When sps_video_parameter_set_id is equal to 0, the value of vps_max_sublayers_minus1 is inferred to be equal to “sps_max_sublayers_minus1” (instead of 6).

It was commented that option ii has a circularity issue in the semantics of sps_max_sublayers_minus1, so option i is preferred.

- Modify the semantics of bp_max_sublayers_minus1 to change the range of values from 0 to vps_max_sublayers_minus1 to instead require it to be equal to maxSubLayers (given as input to the ols_hrd_parameters() syntax structure)? (JVET-S0097)

It was commented that this has a relationship to S0100.

See the notes for S0100 in section 3.3.

- Modify the additional bumping process for DPB by doing following? (JVET-S0080)

  - Move the process for setting the value of PicLatencyCount from within additional bumping process to within the output and removal of pictures from the DPB (i.e., sub clause C.5.2.2) after the invocation of bumping process.

It was commented that no action is needed on this due to the recommended action for PictureOutputFlag for S0077.

  - Remove the additional bumping process or removal of the output of pictures in the additional bumping process.

Defer this for further study.

Discussion in the HLS AHG meeting stopped here at ~0220 on 28 May.

TBP here onwards.

- Fix an asserted bug in Equation C.10 (also applicable to HEVC)? (JVET-S0101)

- Add a constraint on the sum of InitialCpbRemovalDelay and InitialCpbRemovalOffset as follows? (JVET-S0101)

When bp_concatenation_flag is equal to 0, the sum of InitCpbRemovalDelay[ Htid ][ ScIdx ] and InitCpbRemovalDelayOffset[ Htid ][ ScIdx ] shall be the same as in the previous buffering period.

- Include PH and PH NAL units into the bitstream subject to HRD conformance checking using either of the following alternative options? (JVET-S0118)

  - Include PH NAL unit and AUD in Type I bitstream

  - Include PH NAL unit and AUD in Type II bitstream

Related to general and subpicture sub-bitstream extraction processes:

- Perform the asserted bug-fixes for the general sub-bitstream extraction process for determining which AUD NAL units of the input bitstream are kept in the output sub-bitstream as follows? (JVET-S0086)

  - Add AUD_NUT among the NAL unit types that are kept in the output sub-bitstream regardless of its nuh_layer_id value.

  - Add removal of an AUD NAL unit for which all VCL NAL units of an AU are removed by the sub-bitstream extraction.

- Modify subpicture extraction process regarding handling of scalable-nested SEI messages by performing following? (JVET-S0099)

  - Removal of scalable-nested SEI messages for non-target subpictures

  - Removal before replacing non-scalable-nested SEI messages

- Make the following changes related to the general sub-bitstream extraction process? (JVET-S0102)

  - Add a constraint that the BP/PT/DUI SEI messages for any two OLSs with the same layer set are the same.

  - Modification regarding AUs that become IRAP or GDR AU after extraction by doing one of the two options:

    - Option 1: Add AUD writing to the extraction process for AUs that become IRAP AUs or GDR AUs.

    - Option 2: Mandate AUDs to be placed in all AUs that contain IRAP or GDR pictures and add rewriting of aud_irap_or_gdr_au_flag to extraction process.

  - Place PT SEI messages in individual SEI NAL units when general_same_pic_timing_in_all_ols_flag is equal to 1.

  - Require that the scalable-nested and non-scalable-nested BP SEI messages in a CVS have the same values for the five syntax elements which specify lengths of various u(v) syntax elements.

- Modify the subpicture sub-bitstream extraction process regarding handling of scalable-nested SEI messages by performing following? (JVET-S0099)

  - Removal of scalable-nested SEI messages for non-target subpictures

  - Removal before replacing non-scalable-nested SEI messages

Item 61) is related to item 10.a, and item 67) is related to item 10.b.

- Modify the subpicture sub-bitstream extraction process by performing following? (JVET-S0117)

  - Remove rewriting of PPS conformance window syntax elements since pps_conformance_window_flag is asserted to be equal to 0.

  - Remove subpicture ID mapping in SPS and PPS as follows:

    - Remove the syntax elements sps_subpic_id[ j ] in all the referenced SPS NAL units and for each j that is not equal to subpicIdx.

    - Remove the syntax elements pps_subpic_id[ j ] in all the referenced PPS NAL units and for each j that is not equal to subpicIdx.

  - Rewrite virtual boundary syntax elements in SPS.

New submissions (for the June 10th submission deadline)

(Convention for text changes proposed: “quoted text” is proposed to be removed and text inside delimiters < > is proposed to be added by proposals)

Related to HRD signalling and operation:

- Change HRD operation description from being picture-specific to be AU-specific as follows? (JVET-S0157 item 1)

  - In clauses C.2.3, D.3.2, and D.4.2, rename prevNonDiscardablePic to prevNonDiscardableAu, firstPicInPrevBuffPeriod to firstAuInPrevBuffPeriod, notDiscardablePic to notDiscardableAu, and change their descriptions from being picture-specific to AU-specific.

  - In clause D.3.2, change the description of the semantics of bp_concatenation_flag and bp_cpb_removal_delay_delta_minus1 from being picture-specific to be AU-specific.

  - In clause D.3.2, change the description of the condition for the constraint requiring bp_alt_cpb_params_present_flag to be equal to 0 from being picture-specific to be AU-specific.

  - In clause D.4.2, change the descriptions of BpResetFlag, CpbRemovalDelayMsb[ i ], CpbRemovalDelayVal[ i ] and the constraint requiring pt_cpb_alt_timing_info_present_flag to be equal to 0 from being picture-specific to be AU-specific.

  - In clause D.4.2, change the description of the semantics of pt_dpb_output_delay, pt_dpb_output_du_delay, and pt_display_elemental_periods_minus1 from being picture-specific to be AU-specific.

  - In clause D.5.2 (DU information SEI message semantics), change the description of the semantics of dui_dpb_output_du_delay from being picture-specific to be AU-specific.

- In clause D.3.2, modify the constraint related to bp_alt_cpb_params_present_flag as follows: (JVET-S0157 item 2):

When <the associated AU is not an IRAP or GDR AU> “the associated picture is neither a CRA picture nor an IDR picture”, the value of bp_alt_cpb_params_present_flag shall be equal to 0.

- In clause D.4.2, in the constraint related to requiring pt_cpb_alt_timing_info_present_flag to be equal to 0, add that the RASL pictures in the constraint are RASL pictures that do not have mixed NAL unit types as follows? (JVET-S0157 item 4).

When <all pictures in the associated AU are RASL pictures with pps_mixed_nalu_types_in_pic_flag equal to 0> “the associated picture is a RASL picture”, the value of pt_cpb_alt_timing_info_present_flag shall be equal to 0.

- In clause D.5.2, in the semantics of pt_display_elemental_periods_minus1, change fixed_pic_rate_within_cvs_flag[ TemporalId ] (3 instances) to fixed_pic_rate_within_cvs_flag[ Htid ]? (JVET-S0153 item 5)

- Modify constraints as follows: (JVET-S0175 aspect 3)?

When “fixed_pic_rate_within_cvs_flag[ TemporalId ] is equal to 0 or sps_field_seq_flag is equal to 1, the value of” <any of the following applies> pt_display_elemental_periods_minus1 shall be equal to 0:

–	sps_field_seq_flag is equal to 1 <for the output layers in the OLS>.

–	<sps_field_seq_flag is equal to 0 for all output layers of the bitstream and there is not frame-field information SEI message present or display_fields_from_frame_flag is equal to 0 if a frame-field SEI message is associated with the AU and fixed_pic_rate_within_cvs_flag[ TemporalId ] is equal to 0.>

- Remove fixed_pic_rate_general_flag[ i ] from the ols_hrd_parameters() and derive from parameters signalled for the two CVSs including the dpb_parameters() whether the fixed picture rate is achieved across CVSs? (JVET-S0175 aspect 1, 2)

- Add a gating flag for pt_display_elemental_periods_minus1, so that its presence can be controlled and change the derivation of elementalOutputPeriod as follows and change D.9.6 accordingly? (JVET-S0175 aspects 4 and 5)?

–	If a PT SEI message is present for picture n <and pt_display_elemental_periods_present_flag equal to 1>, elementalOutputPeriods is equal to the value of pt_display_elemental_periods_minus1 + 1.

–<	Otherwise, if external means are provided to set the value of elementalOutputPeriods, elementalOutputPeriods is set equal to the value provided via external means.

–	Otherwise , if frame-field information SEI messages are present for at least one of the output pictures in the AU of picture n, the value of elementalOutputPeriods is set to the lowest value of display_elemental_periods_minus1 + 1 among all present frame-field information SEI messages.>

–	Otherwise, elementalOutputPeriods is equal to 1.

- Add following related to when both general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag are equal to 0? (JVET-S0175 aspect 6)

<It is a bitstream requirement that when both general_nal_hrd_params_present_flag  and  general_vcl_hrd_params_present_flag are equal to 0, the following applies.

- elementalOutputPeriods is 1 in the AUs of the CVS and display_elemental_periods_minus1 is set to 0 in all frame-field information SEI messages.

- No pictures with ph_pic_output_flag equal to 0 are present in AUs of the CVS>

- Conditionally signal bp_sublayer_initial_cpb_removal_delay_present_flag, only when bp_max_sublayers_minus1 is greater than 0 and infer it to 0 otherwise? (JVET-S0181 proposal 1, JVET S0157 item 3)

- Move the signalling of bp_max_sublayers_minus1 to be before the signalling of bp_cpb_removal_delay_deltas_present_flag and conditionally signal bp_cpb_removal_delay_deltas_present_flag and bp_num_cpb_removal_delay_deltas_minus1 and bp_cpb_removal_delay_delta_val[ i ] syntax elements, only when bp_max_sublayers_minus1 is greater than 0 and infer bp_cpb_removal_delay_deltas_present_flag equal to 0 otherwise? (JVET-S0181 proposal 2)?

- Move the syntax element pt_cpb_removal_delay_minus1[ bp_max_sublayers_minus1 ]  to locate it near the other CPB removal delay syntax elements, in particular other pt_cpb_removal_delay_minus1[ i ] syntax elements (and pt_cpb_removal_delay_delta_idx[ i ] syntax elements) for i in the range of TemporalId to bp_max_sub_layers_minus1 − 1, inclusive? (JVET-S0185 proposal 1)

- Move the signalling of syntax element bp_alt_cpb_params_present_flag to locate it near the syntax element bp_use_alt_cpb_params_flag where it is used? (JVET-S0185 proposal 2)

- Modify the output and removal of picture from DPB, additional bumping as follows?

  - From JVET-S0198:

C.5.2.2 Output and removal of pictures from the DPB

…

–	Otherwise (the current AU is not a CVSS AU or the current AU is a CVSS AU that is not AU 0 but the current picture is not the first picture of the current AU), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. “When one or more of the following conditions are true, t”<T>he "bumping" process specified in clause C.5.2.4 is invoked repeatedly “while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied,” until <the number of pictures in the DPB is less than max_dec_pic_buffering_minus1[ Htid ] + 1>. “none of the following conditions are true:

- The number of pictures in the DPB that are marked as "needed for output" is greater than max_num_reorder_pics[ Htid ].

- max_latency_increase_plus1[ Htid ] is not equal to 0 and there is at least one picture in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount is greater than or equal to MaxLatencyPictures[ Htid ].

- The number of pictures in the DPB is greater than or equal to max_dec_pic_buffering_minus1[ Htid ] + 1.”

  - From JVET-S0223:

C.5.2.2 Output and removal of pictures from the DPB

–	Otherwise (the current AU is not a CVSS AU, <the current AU is CVSS AU 0,> or the current AU is a CVSS AU that is not AU 0 but the current picture is not the first picture of the current AU), all picture storage buffers containing a picture which are marked as "not needed for output" and "unused for reference" are emptied (without output). For each picture storage buffer that is emptied, the DPB fullness is decremented by one. When one or more of the following conditions are true, the "bumping" process specified in clause C.5.2.4 is invoked repeatedly while further decrementing the DPB fullness by one for each additional picture storage buffer that is emptied, until none of the following conditions are true:

- The number of pictures in the DPB that are marked as "needed for output" is greater than max_num_reorder_pics[ Htid ].

- max_latency_increase_plus1[ Htid ] is not equal to 0 and there is at least one picture in the DPB that is marked as "needed for output" for which the associated variable PicLatencyCount is greater than or equal to MaxLatencyPictures[ Htid ].

- The number of pictures in the DPB is greater than or equal to max_dec_pic_buffering_minus1[ Htid ] + 1.

C.5.2.3 Additional bumping

–	The processes specified in this clause happen instantaneously <for AU n> when the last DU of “AU n containing” the current picture is removed from the CPB.

Related to sub-bitstream extraction

General SEI semantics and constraints:

- Fix an asserted missing integration of an agreement (from JVET-Q0394) that specifies that non-scalable-nested HRD-related SEI messages apply to the OLSs that include the same set of layers as the entire bitstream (instead of only to the 0th OLS)? (JVET-S0178 aspect 1)

  - Consequently, add a constraint such that when there is no OLS that includes the set of layers same as the entire bitstream, there shall be no non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI)? (JVET-S0178 aspect 1.a)

- Specify that non-scalable-nested non-HRD-related SEI messages apply to all layers in the entire bitstream (to be consistent with non-scalable-nested HRD-related SEI messages)? (JVET-S0178 aspect 2)

  - Consequently, remove the constraints on the value of nuh_layer_id for SEI NAL units containing non-scalable-nested SEI messages, such that the value of nuh_layer_id for SEI NAL units containing non-scalable-nested SEI messages is unconstrained, same as for the nuh_layer_id for DCI, VPS, AUD, and EOS NAL units? (JVET-S0178 aspect 2.a)

- Rename VclAssociatedSeiList to NestingForLayersSeiList, and remove payloadType value 203 (SLI) from the list? (JVET-S0178 aspect 3)

- Add the clarification (that was recently added to the HEVC specification as an corrigendum item) on the value ranges of tone_map_id, frame_packing_arrangement_id, knee_function_id, and colour_remap_id, including potential collisions of the interpretation for values of these syntax elements? (JVET-S0178 aspect 4)

- Add a constraint such that when general_same_pic_timing_in_all_ols_flag is equal to 1, there shall be no SEI NAL units that contain a scalable-nested SEI message with payloadType equal to 1 (PT) ? (JVET-S0178 aspect 5)

- Modify an existing constraint, to add the SLI SEI message, such that when an SEI NAL unit contains a non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, or 203? (JVET-S0178 aspect 6)

- Modify an existing constraint, to add the SLI SEI message, that when an SEI NAL unit contains a scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, 203, or 133 (scalable nesting) ? (JVET-S0178 aspect 7)

- Add a constraint such that when an SEI NAL unit contains an SEI message with payloadType equal to 3 (filler payload), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 3? (JVET-S0178 aspect 8)

- Add a constraint such that when an SLI SEI message and a BP SEI message that apply to a particular OLS are present within an AU, the SLI SEI messages shall precede the BP SEI message in decoding order? (JVET-S0178 aspect 9)

Scalable nesting SEI message:

- Modify semantics of sn_num_subpics_minus1 (based on following options)?

  - Clarify the association of scalable-nested SEI messages for subpictures to either layers or OLS as follows? (JVET-S0173 item 2)

sn_num_subpics_minus1 plus 1 specifies the number of subpictures to which the scalable-nested SEI messages apply. <When sn_ols_flag is equal to 0,> the value of sn_num_subpics_minus1 shall be less than or equal to the value of sps_num_subpics_minus1 in the SPS referred to by the pictures <of the indicated layer. Otherwise, when sn_ols_flag is equal to 1, the value of sn_num_subpics_minus1 shall be less than or equal to the value of sps_num_subpics_minus1 in the SPS referred to by the pictures of the highest layer in the OLS with sps_num_subpics_minus1 greater than 0.>

  - Specify the semantics of sn_num_subpics_minus1 and sn_subpic_idx[ i ] in a way such that the syntax elements are about the subpictures of the layers with multiple subpictures per picture? (JVET-S0177 aspect 7)

<Among the layers in the OLSs (when sn_ols_flag is equal to 1) to which the scalable-nested SEI messages apply, or among the layers (when sn_ols_flag is equal to 0) to which the scalable-nested SEI messages apply, those for which the referenced SPSs have sps_num_subpics_minus1 greater than 0 are referred to as the multiSubpicLayers.>

sn_num_subpics_minus1 plus 1 specifies the number of subpictures <in each picture in the multiSubpicLayers> “to which the scalable-nested SEI messages apply”. The value of sn_num_subpics_minus1 shall be less than or equal to the value of sps_num_subpics_minus1 in the SPS<s> referred to by the pictures in the <multiSubpicLayers> “CLVS”.

- Change sn_subpic_id[ i ] to sn_subpic_idx[ i ] and remove sn_subpic_id_len_minus1 as follows? (JVET-S0177 aspect 1)

sn_subpic_id<x>[ i ] “indicates” <specifies the subpicture index of> the i-th subpicture “ID associated with the scalable-nested SEI messages” <in each picture in the multiSubpicLayers. The value of sn_subpic_idx[ i ] shall be less than or equal to the value of sps_num_subpics_minus1 in the SPSs referred to by the pictures in the multiSubpicLayers.> “The length of the sn_subpic_id[ i ] syntax element is sn_subpic_id_len_minus1 + 1 bits.” <The scalable-nested SEI messages also apply to the single subpicure in each picture in the layers that are not in the multiSubpicLayers but are among the layers in the OLSs (when sn_ols_flag is equal to 1) to which the scalable-nested SEI messages apply or among the layers (when sn_ols_flag is equal to 0) to which the scalable-nested SEI messages apply.>

“sn_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sn_subpic_id[ i ]. The value of sn_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of sn_subpic_id_len_minus1 shall be the same for all scalable nesting SEI messages that are present in a CLVS.”

Or change semantics of sn_subpic_id_len_minus1 by changing CLVS to CVS (JVET-S0173 item 2):

sn_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sn_subpic_id[ i ]. The value of sn_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of sn_subpic_id_len_minus1 shall be the same for all scalable nesting SEI messages that are present in a C“L”VS.

- Disallow containing of filler payload SEI messages in a scalable nesting SEI message? (JVET-S0177 aspect 2)

- Constrain that, when a scalable nesting SEI message contains one or more subpicture level information (SLI) SEI messages, the value of sn_ols_flag shall be equal to 1, and the value of sn_subpic_flag shall be equal to 0? (JVET-S0177 aspect 3)

- Modify an existing constraint, to include the SLI message, such that when a scalable nesting SEI message contains a BP, PT, DUI, or SLI SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI)? (JVET-S0177 aspect 4)

- Modify an existing constraint, to include recently added SEI messages, such that when a scalable nesting SEI message contains an SEI message that has payloadType not equal to 3 (filler payload) or 132 (decoded picture harsh), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to PREFIX_SEI_NUT? (JVET-S0177 aspect 5)

- Add a constraint such that when a scalable nesting SEI message contains an SEI message that has payloadType equal to 132 (decoded picture harsh), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to SUFFIX_SEI_NUT? (JVET-S0177 aspect 6)

General sub-bitstream extraction:

- Handling of AUD in sub-bitstream extraction process (JVET-S0102 item 5, JVET-S0225)

  - Restrict aud_irap_or_gdr_au_flag to a one-way constraint in single-layer bitstreams as follows? (JVET-S0102 item 5).

aud_irap_or_gdr_au_flag equal to 1 specifies that the AU containing the AU delimiter is an IRAP or GDR AU. aud_irap_or_gdr_au_flag equal to 0 specifies that the AU containing the AU delimiter is not an IRAP or GDR AU <when NumLayersInOls[ TargetOlsIdx ] is greater than 1>.

  - And/or modify sub-bitstream extraction process using either one of the following two options: (JVET-S0225)

    - Option 1: Add AUD or rewrite aud_irap_or_gdr_au_flag of the existing AUD when the associated extracted AU is an IRAP AU or GDR AU as follows:

<When an AU is an IRAP AU or GDR AU and no associated AUD, add an AUD NAL unit of type AUD_NUT before the first VCL NAL unit of the AU with aud_irap_or_gdr_au_flag equal to 1. Otherwise for any existing AUD, set aud_irap_or_gdr_au_flag equal to 1 for  an IRAP AU or GDR AU.>

    - Option 2: Add AUD or rewrite aud_irap_or_gdr_au_flag of the existing AUD when the associated AU is an IRAP AU or GDR AU and the target OLS has multi-layers; and either remove the existing AUD or rewrite aud_irap_or_gdr_au_flag of the existing AUD when the associated AU is an IRAP AU or GDR AU and the target OLS contains only single layer as follows:

–	<When the targte OLS contains more than one layer,

- For each AU that is an IRAP AU or GDR AU with no associated AUD, add a NAL unit of type AUD_NUT before the first VCL NAL unit of the AU with aud_irap_or_gdr_au_flag equal to 1.

- Otherwise for any existing AUD, set aud_irap_or_gdr_au_flag equal to 1 for an IRAP AU or GDR AU.

–	Otherwise (the target OLS contains only one layer), remove the associated AUD from outBitstream for each AU, or for any existing AUD, set aud_irap_or_gdr_au_flag equal to 1 for an IRAP AU or GDR AU.>

- In the general sub-bitstream extraction process, specify the conditions under which an output sub-bitstream is required to be a conforming bitstream such that the value of tIdTarget is specified to be in the range of 0 to vps_max_sublayers_minus1, inclusive (instead of 0 to 6 inclusive)? (JVET-S0158 aspect 1)

- Specify the general sub-bitstream extraction process such that it would remove, from the output bitstream outBitstream, SEI NAL units that contain a scalable nesting SEI message with sn_ols_flag equal to 0 while the applicable layers as indicted in the scalable nesting SEI message does not include any layer in the target OLS? (JVET-S0158 aspect 2)

- Make the following changes to the 4th step in the general sub-bitstream extraction process? (JVET-S0158 aspect 3)

  - Remove the redundant condition "nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive". (This is asserted to be editorial.)?

  - When removing from the output bitstream the VCL NAL units, instead of removing the associated SEI NAL units containing SEI messages other than the BP, PT, or DUI SEI messages, remove all SEI NAL units that contain SEI messages that become not applicable to any picture or subpicture remaining in the final output bitstream? (Since this is not easy to do, another option is not to remove SEI NAL units in this step.)

- Make the following changes to the last step in the general sub-bitstream extraction process? (JVET-S0158 aspect 4)

  - Change the condition as follows?

When the list LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all <VCL> NAL units in the bitstream <inBitstream>.

  - When at least one layer is removed by the extraction process, also remove all SEI NAL units that contain a non-scalable-nested SLI SEI message (same as for BP, PT, and DUI SEI messsages)?

  - Insert SEI NAL units to directly contain those SEI messages that were scalable-nested SEI messages (with sn_subpic_flag equal to 0 and regardless of the value of sn_ols_flag) that apply only to the output bitstream, and remove their original container SEI NAL units from the output bitstream?

Subpicture level information (SLI) SEI message:

- Clarify the scope of Subpicture Level Information (SLI) SEI to be CVS instead of CLVS? (JVET-S0173 item 1)

- Include all layers of the OLS in MinCr related constraint for subpictures by making the following changes? (JVET-S0173 item 3):

- The sum of the NumBytesInNalUnit variables for AU 0 corresponding to the j-th subpicture shall be less than or equal to FormatCapabilityFactor * ( Max(“SubpicSizeY[ j ]”<AuSizeMaxInSamplesY[ 0 ]>, fR * MaxLumaSr <) >* OlsRefLevelFraction[ i ][ j ] ÷ 256 “)” + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) * OlsRefLevelFraction[ i ][ j ] ) ÷ ( 256 * MinCr ), where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0, at level sli_ref_level_idc[ i ], and MinCr and AuSizeMaxInSamplesY[ 0 ] are derived as indicated in A.4.2.

- Modify the inference rule for sli_ref_level_fraction_minus1[ i ][ j ]? (JVET-S0173 item 5):

When not present, the value of sli_ref_level_fraction_minus1[ i ][ j ] is inferred to be equal to <Max( 256,> Ceil( 256 * SubpicSizeY[ j ] ÷ PicSizeInSamplesY * MaxLumaPs( general_level_idc ) ÷ MaxLumaPs( sli_ref_level_idc[ i ] ) <)>− 1.

- Resolve editorial notes related to subpictures as follows? (JVET-S0173 item 6):

  - Refer to minCR and fR derivation in A.4.2.

  - Replace PicSizeInSamplesY with PicSizeMaxInSamplesY and clarify that the picture size of the highest layer with subpictures in the OLS is taken.

- Replace sps_num_subpics_minus1 with sli_num_subpics_minus1 in semantics of sli_num_ref_levels_minus1? (JVET-S0173 item 7)

- Modify alignment constraint of intendent subpictures by adding sps_subpic_treated_as_pic_flag[ i ] syntax element to the clause as follows? (JVET-S0173 item 4):

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and <sps_log2_ctu_size_minus5 and> shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

- Add sli_max_sublayers_minus1, sli_sublayer_info_present_flag, and a loop for sublayers for the signalled fractions and reference level indicators, to support sublayers? (JVET-S0176 item 1)

- Allow SLI SEI messages to be available either in the bitstream or provided through an external means not specified in this Specification? (JVET-S0176 item 2)

- Change the persistency scope for SLI SEI from one CVS to one or more CVSs, to be consistent with VPSs and SPSs wherein level information are or may be signalled? (JVET-S0176 item 3)

- Change the definition of subpicture sequence to cover the case when there is one or more layers with single subpicture per picture? (JVET-S0176 item 4)

- Require that, when an SLI SEI message is present for a CVS, the value of sps_num_subpics_minus1 shall be the same for all the SPSs referenced by the pictures in the layers with multiple subpictures per picture. (JVET-S0176 item 5)?

- Specify the semantics of sli_num_subpics_minus1 in a way such that the syntax element is about the subpictures of the layers with multiple subpictures per picture? (JVET-S0176 item 6)

- Make variable derivations subpicture sequence specific as follows: Add an array index, which identifies a subpicture sequence, to both the variables SubpicLevelIdc and SubpicLevelIdx, as well as to the arrays SubpicCpbSizeVcl, SubpicCpbSizeNal, SubpicBitRateVcl, and SubpicBitRateNal in the last set of constraints in the semantics of the SLI SEI message? (JVET-S0176 item 7)

Subpicture sub-bitstream extraction:

- Modify the specification of subpicture sequences, which when extracted, are required to be a conforming bitstream? (JVET-S0154 aspect 1)?

- Remove VCL NAL units and their associated filler data NAL units and associated filler payload SEI messages etc. regardless of whether there is an external means for replacing of the parameter sets? (JVET-S0154 aspect 2)

- Remove SEI NAL units containing scalable-nested SEI messages that do not apply to the output bitstream from the output bitstream? (JVET-S0154 aspect 3)

- Specify the subpicture index for identifying the subpicture sequence as the subpicture index of the to-be-extracted subpictures in the layers with multiple subpictures per picture, not the layers with only one subpicture per picture? (JVET-S0154 aspect 4)

- Rewrite both general_level_idc and sublayer_level_idc[ k ] for k in the range of 0 to tIdTarget − 1, inclusive, in the referenced VPSs, when present, and in the referenced SPSs, when NumLayersInOls[ targetOLsIdx ] is equal to 0? (JVET-S0154 aspect 5)

- Rewrite cpb_size_value_minus1[ k ][ j ] and bit_rate_value_minus1[ k ][ j ] for all values of k in the range of 0 to tIdTarget, inclusive, in the referenced VPSs, when present, and in the referenced SPSs, when NumLayersInOls[ targetOLsIdx ] is equal to 0? (JVET-S0154 aspect 6)

- Specify the calculation and rewriting of the scaling window offset parameters as part of the subpicture sub-bitstream extraction process?

  - As per JVET-S0154 aspect 7?

  - As per JVET-S0189?

- Rewrite cbr_flag[ tIdTarget ][ j ] in the referenced VPSs, when present, and in the referenced SPSs, when NumLayersInOls[ targetOLsIdx ] is equal to 0? (JVET-S0154 aspect 8)

- Insert SEI NAL units to directly contain those SEI messages that were scalable-nested SEI messages that apply only to the output bitstream, and remove their original container SEI NAL units from the output bitstream? (JVET-S0154 aspect 9)

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 1 of this contribution belongs to this category.

JVET-S0064 AHG9: On Buffering Period Message Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0075 AHG9: On constraints and inference values when VPS is not present [Hendry, S. Paluri, S. Kim (LGE)]

Aspect 2 of this contribution belongs to this category.

JVET-S0097 AHG8/AHG9: Clarifications to HRD specification for single-layer and multi-layers bitstreams [V. Drugeon, T. Nishi, K. Abe, T. Toma (Panasonic)]

JVET-S0080 AHG9: On additional bumping process in the DPB [S. Paluri, Hendry, S. Kim (LGE)]

JVET-S0086 AHG9: On AUD NAL units in the sub-bitstream extraction process [M. M. Hannuksela (Nokia)

JVET-S0099 AHG12: SEI message handling in subpicture extraction [R. Skupin, Y. Sanchez, K. Suehring (HHI), V. Drugeon (Panasonic)]

JVET-S0101 AHG9: On HRD [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

JVET-S0102 AHG9: On OLS extraction [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

JVET-S0117 AHG12: On Subpicture sub-bitstream exaction [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0118 AHG9: On HRD bitstream [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0154 AHG9/AHG8/AHG12: On the subpicture sub-bitstream extraction process [Y.-K. Wang, Z. Deng, K. Zhang, L. Zhang (Bytedance)]

JVET-S0157 AHG9: HRD and related cleanups [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0158 AHG9/AHG8: On the general sub-bitstream extraction process [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0173 AHG9/AHG12: Subpicture related cleanups [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

JVET-S0175 AHG9: Cleanup of picture rate info and HRD operation without timing SEI messages [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

JVET-S0176 AHG9: On the subpicture level information SEI message [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0177 AHG9: On the scalable nesting SEI message [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0178 AHG9: General SEI semantics and constraints [Y.-K. Wang, Z. Deng (Bytedance)]

JVET-S0181 AHG9: On Buffering Period [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0185 AHG9: On HRD Cleanups [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0189 AHG9/AHG8/AHG12: On subpicture bitstream extraction process [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0198 AHG9: On removal of picture from DPB process [S. Paluri, Hendry, S. Kim (LGE)]

JVET-S0223 AHG9: On picture bumping process [V. Seregin, Y. He, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0225 AHG 9: On AUD for sub-bitstream extraction [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

S0102-v2 item 5 is related.

### DCI, VUI, and SEI (3+3)

TBP.

JVET-S0051 AHG9: Digital signature SEI message [J. Xu, Y.-K. Wang, L. Zhang, K. Zhang (Bytedance)]

JVET-S0107 AHG9/AHG12: Recommended multi-layer composite picture SEI messages [J. Boyce (Intel)]

JVET-S0108 AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers [E. Thomas (TNO)]

JVET-S0172 AHG9: On generalized cubemap projection SEI message [Y.-H. Lee, J.-L. Lin, Y.-J. Chen, C.-C. Ju (MediaTek)]

JVET-S0213 AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers with example bitstreams [E. Thomas (TNO)]

JVET-S0214 AHG8/AHG9: Updates on the implementation of multi-layer decoding and output independent layer arrangement in VTM [E. Thomas (TNO)]

### HLS editorial inputs (4+3)

TBP.

JVET-S0068 AHG9: Editorial improvements on high-level syntax [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-S0096 AHG9: On RPL syntax and semantics [J. Chen, Y. Ye, R.-L. Liao (Alibaba)]

Aspect 5 of this contribution belongs to this category.

JVET-S0110 AHG9: On HLS Editorial cleanup [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 2 of this contribution belongs to this category.

JVET-S0119 AHG9: Expression of existing intent for VPS/SPS/PPS syntax elements [N. Hu, V. Seregin, Y. He, M. Karczewicz (Qualcomm)]

JVET-S0192 AHG9: On reference picture list with generating unavailable reference picture [B. Choi, S. Wenger, S. Liu (Tencent)]

Aspects 2 and 3 of this contribution belong to this category.

JVET-S0194 AHG9: On editorial cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0208 AHG9: Proposed fixes to HLS [M. Pettersson, R. Yu, R. Sjöberg, M. Damghanian, J. Enhorn, J. Ström, D. Liu (Ericsson)]

## AHG12: High-level parallelism and coded picture regions (7+5)

JVET-S0145 AHG12: A summary of proposals on tile, slices, and subpictures [Y.-K. Wang (Bytedance)]

This contribution was discussed in the HLS AHG meeting at 0050 UTC on Friday 29 May and in the HLS AHG pre-meeting at ....

This contribution provides a summary of the 11 proposals on subpictures, slices, and tiles (the agenda item 3.2 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

On tiles and slices

- Allow the value of pps_no_pic_partition_flag to be different for different PPSs that are referred to by coded pictures within a CLVS. (S0049 aspect 3, S0095 aspect 4)

  - Alternatively, move pps_no_pic_partition_flag from the PPS to the SPS (and rename it to sps_no_pic_partition_flag). (S0095 aspect 4)

It was said that it is not clear why this constraint is in the draft and that the constraint does not seem logical. We have different flags regarding slices and tiles.

AHG Recommendation (cleanup): Remove the constraint.

- Do either of the following to reduce the current two options for signalling of a tile containing exactly one rectangular slice to one option only: (S0095 aspect 1)

  - Replace pps_num_exp_slices_in_tile[ i ] with pps_num_exp_slices_in_tile_minus1[ i ].

  - Change the upper limit of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] from RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 2.

There isn’t really a bug or problem, but there are two ways to signal the same thing.

Approach “a” would have a bit cost penalty.

No clear need for action was evident, so no action was recommended on this.

- For a more precise value range, change the upper limit of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] from RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − pps_num_exp_slices_in_tile[ i ]. (S0095 aspect 2)

This is purely editorial.

Editor action item: The editor may wish to consider whether this would improve clarity.

- Change the syntax condition for pps_rect_slice_flag from "if( NumTilesInPic > 1 )" to "if( NumTilesInPic > 3 )", because when the number of tiles in a picture is less than 4, the shape of each slice has to be rectangular. (S0095 aspect 3)

It was commented that this seems somewhat similar in spirit to item 2. This would force the encoder to decide the slice layout at the PPS level in advance when there are fewer than 4 tiles per picture. It was commented that this imposition would be undesirable, so no action was recommended on this.

- Change the semantics of pps_rect_slice_flag to cover the case that rectangular slice is smaller than the tile. (S0095 aspect 5)

This is a correction of an error in the current semantics expression.

AHG Recommendation (editorial bug fix): Adopt.

- The inference condition of pps_single_slice_per_subpic_flag is changed from "When not present" to "When the value of pps_no_pic_partition_flag is equal to 1" so that pps_single_slice_per_subpic_flag is inferred only when it is needed. (S0095 aspect 6)

This is purely editorial. (It makes no technical difference whether we make this change or not.)

Editor action item: The editor may wish to consider whether this would be an editorial improvement.

Discussion in the HLS AHG meeting stopped here at 0120 UTC on 29 May.

- When pps_pic_width_in_luma_samples is less than or equal to CtbSizeY, pps_num_exp_tile_columns_minus1 and pps_tile_column_width_minus1[ i ] are skipped and inferred. (S0116 aspect 1)

  - Alternatively, when the condition is true, the values of pps_num_exp_tile_columns_minus1 and pps_tile_column_width_minus1[ i ] are both constrained to be equal to 0. (S0116 aspect 1)

- When pps_pic_height_in_luma_samples is less than or equal to CtbSizeY, pps_num_exp_tile_row_minus1 and pps_tile_row_height_minus1[ i ] are skipped and inferred. (S0116 aspect 1)

  - Alternatively, when the condition is true, the values pps_num_exp_tile_row_minus1 and pps_tile_row_height_minus1[ i ] are both constrained to be equal to 0. (S0116 aspect 1)

- When NumTilesInPic is equal to 1 and either pps_pic_width_in_luma_samples or pps_pic_height_in_luma_samples is less than or equal to CtbSizeY, pps_tile_idx_delta_present_flag is skipped and inferred to be equal to 0. (S0116 aspect 2)

On subpictures

- Add a shortcut for signalling of subpictures layout, as follows: (S0071)

  - Add sps_subpic_same_res_flag, equal to 1 specifies that all subpictures have the same width and height, respectively.

  - When sps_subpic_same_res_flag is equal to 1, sps_subpic_width_minus1[ i ] and sps_subpic_height_minus1[ i ] are only signalled when i is equal to 0, and sps_subpic_ctu_top_left_x[ i ] and sps_subpic_ctu_top_left_y[ i ] are skipped for all i values.

- When sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples are both less than or equal to CtbSizeY, sps_num_subpics_minus1 is skipped and inferred to be equal to 0. (S0115 aspect 1)

- Merge the two sets of constraints on the combination of subpictures and scalability, in the semantics of sps_subpic_treated_as_pic_flag[ i ] and in the end of clause 8.3.2, respectively, and the constraint that requires the same subpicture ID for subpicture inter prediction referencing within a CLVS, in the PPS semantics, to be places in the end of clause 8.3.2, as follows: (S0125 aspect 1) (removal: italic; addition: bold)

- When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, and sps_num_subpics_minus1 is greater than 0 and the current subpicture with sub-picture index subPicIdx has sps_subpic_treated_as_pic_flag[ subPicIdx ] equal to 1, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples as the current picture, shall have has the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive), and shall have the same value of sub-picture IDs as the current picture (i.e. SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

- On top of item 3), further require the same scaling window offsets, as follows: (S0125 aspect 2) (removal: italic; addition: bold)

- When sps_num_subpics_minus1 is greater than 0 and the current subpicture with sub-picture index subPicIdx has sps_subpic_treated_as_pic_flag[ subPicIdx ] equal to 1, either of the following two conditions (but not both) shall be true:

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples as the current picture, shall have the same scaling windows offsets as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset), shall have the same subpicture layout as the current picture (i.e., the SPSs referred to by that picture and the current picture have the same value of sps_num_subpics_minus1 and the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], and sps_subpic_height_minus1[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive), and shall have the same value of sub-picture IDs as the current picture (i.e. SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive).

  - The picture referred to by each active entry in RefPicList[ 0 ] or RefPicList[ 1 ] is an ILRP for which the value of sps_num_subpics_minus1 is equal to 0.

New submissions (for the June 10th submission deadline)

- Add the following constraint to the semantics of sps_subpic_treated_as_pic_flag[ ] (S0190 aspect 1):

When sps_num_subpics_minus1 is equal to 0, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

- Add a constraint: When the value of sps_subpic_info_present_flag of the current layer is equal to 1, the value of sps_res_change_in_clvs_allowed_flag of the reference layers of the current layer shall be equal to 0. (S0190 aspect 2):

- Infer the value of pps_loop_filter_across_tiles_enabled_flag to be equal to 0 (instead of 1) when not present, for consistency with the inferences of sps_loop_filter_across_subpic_enabled_flag[ i ] and pps_loop_filter_across_slices_enabled_flag. (S0160 aspect 1)

- Add a subpicture merging software, which includes the following setup, into the VTM source code package: (S0162)

- Encoding several bitstreams, each with one subpicture per picture, using the VTM encoder.

- Merging selected encoded bitstreams into a bitstream with multiple subpictures, using the subpicture merging software.

- Decoding the bitstream having multiple subpictures, using the VTM decoder.

- Add a flag, sps_raster_scan_order_subpics_flag, to the SPS, the value equal to 1 specifies that subpictures are ordered in raster scan order in each picture in the CLVS, and when this flag is equal to 1, derive the top-left CTU positions of the subpictures instead of signalling them. (S0206)

- Add a constraint: When sps_num_subpics_minus1 is greater than 0 in the reference layer, the inter-layer reference picture (ILRP) shall have the same value of sps_log2_ctu_size_minus5 as that of the SPS referred to by the current picture. (S0236 aspect 1)

- Specify that the value of sps_num_subpics_minus1 shall be less than MaxSlicesPerPicture. (S0236 aspect 2)

- When pps_subpic_id_mapping_present_flag is equal to 1, skip pps_rect_slice_flag and infer it to be equal to 1. (S0236 aspect 3)

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 3 of this contribution belongs to this category.

JVET-S0071 AHG12: Cleanup of subpicture layout signalling [M. Katsumata, M. Hirabayashi, T. Suzuki (Sony)]

JVET-S0095 AHG12: On tile and slice partitioning related syntax and semantics [J. Chen, Y. Ye, R.-L. Liao (Alibaba)]

JVET-S0115 AHG9: On SPS cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 1 of this contribution belongs to this category.

JVET-S0116 AHG9: On PPS cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0125 AHG9: On sub-picture constraints [V. Seregin, Y. He, Y.-J. Chang, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0190 AHG8/AHG9/AHG12: On reference picture resampling with scalability [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspect 1 of this contribution belongs to this category.

JVET-S0162 AHG3/AHG12: Subpicture merging software [A. Hallapuro, M. M. Hannuksela (Nokia)]

JVET-S0206 AHG12: Raster scan order subpictures [M. Damghanian, D. Liu, R. Sjöberg, M. Pettersson, J. Enhorn, J. Ström, R. Yu (Ericsson)]

JVET-S0236 AHG9/AHG12: High-level syntax cleanups on subpictures [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

## AHG8: Layered coding and resolution adaptivity (8+3)

### General (1+0)

JVET-S0147 AHG8: A summary of proposals on scalability and RPR [Y.-K. Wang (Bytedance)]

This contribution provides a summary of the 10 proposals on scalability and RPR (the agenda item 3.3 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

The following changes were proposed:

On scalability

- When vps_max_layers_minus1 is equal to 0, vps_num_ptls_minus1 is skipped and inferred to be equal to 0. (S0063)

It was commented that this may not be strictly necessary but seems like a very small nice little cleanup and that since we are skipping nearby syntax elements under this condition.

AHG Recommendation (cleanup/consistency): Adopt.

- Fix an asserted bug, that the specification allows to extract sublayers with TemporalId higher than the vps_ptl_max_temporal_id[ 0 ] + 1 for an OLS, by setting NumSubLayersInLayerInOLS[ i ][ k ] to be equal to vps_ptl_max_temporal_id[ i ] + 1, instead of vps_max_sublayers_minus1 + 1, for output layers in Equation 40. (S0100 aspect 1)

AHG Recommendation (bug fix/expression of existing intent): Adopt.

- Add the following constraint to disallow certain indirect reference layers when the value of vps_max_tid_il_ref_pics_plus1[ i ][ j ] is 0 for all the direct reference layers: (S0100 aspect 2)

For any two layers k and j within an OLS i, with k > j, when both NumSubLayersInLayerInOLS[ i ][ j ] and NumSubLayersInLayerInOLS[ i ][ k ] are equal to 0, j is a (direct or indirect) reference layer of k and there is a picture in layer k that is an IRAP picture or a GDR picture with ph_recovery_poc_cnt equal to 0 in an AU, the picture in layer j shall also be an IRAP picture or a GDR picture with ph_recovery_poc_cnt equal to 0, respectively.

It was commented that a similar constraint is already enforced by the requirement that the extracted bitstream shall be a conforming bitstream. The proposed constraint is a bit tighter than the indirect existing constraint.

Editor action item: The editor is requested to consider adding some NOTE/explanation for this aspect.

- Do one of the following, with option b being preferred: (S0100 aspect 3)

  - The values of vps_ptl_max_temporal_id[ i ], vps_dpb_max_temporal_id[ i ] and vps_hrd_max_tid[ i ] shall be the same.

  - The values of vps_ols_dpb_max_temporal_id[ vps_ols_dpb_params_idx[ i ] ] and vps_hrd_max_tid[ vps_ols_hrd_idx[ i ] ] shall be greater than or equal to vps_ptl_max_temporal_id[ vps_ols_ptl_idx[ i ] ].

  - Infer the DPB and HRD parameters for sublayers from vps_ols_dpb_max_temporal_id[ dpbIdx ] and vps_hrd_max_tid[ hrdIdx ] up to vps_ptl_max_temporal_id[ ptlIdx ].

The idea is to have a relationship of these three values for any particular OLS. (The index variables above may not express this concept quite correctly.)

It was noted that S0097 aspect 3 is related (see section 3.1.8).

A participant commented that approach “a” is too restrictive, and indeed that approach “b” is preferred. Another participant considered approach “a” to be better.

Approach “b” would allow to send less unnecessary syntax for HRD and DPB parameters for the various OLSs than approach “a”.

It was asked whether S0097 aspect 3 would take care of the issue by itself.

The AHG agreed to defer this issue and S0097 aspect 3 for further study.

- When not present, the value of vps_max_tid_ref_present_flag[ i ] is inferred to be equal to 0. (S0129 aspect 1)

The contributor said this is not needed after review of the text and withdrew this aspect.

- When vps_all_independent_layers_flag is equal to 1 and vps_each_layer_is_an_ols_flag is equal to 1, the value of vps_ols_mode_idc is inferred to be equal to 0. (also in S0129 aspect 1)

It was noted that this is purely editorial, as the value that is inferred has no effect; this is just a matter of editorial completeness – wanting to always have some value if something is checked in a condition.

It was commented that inferring the value 0 might be confusing or incorrect, since 0 ordinarily has a semantic interpretation that is not intended here. Inferring the value 3 was suggested. Alternatively, adding an extra “if” with a comment seemed better.

Editor action item: The editor is asked to rephrase the text to avoid the problem. The suggested method is to add an extra “if” condition with a comment to explain why it is there.

On RPR

- Change the value ranges of the scaling window offsets such that the scaling window width and height can be up to 16 times the picture width and height, respectively. (S0048 aspect 1)

It was asked whether this might have a memory bandwidth impact and concluded that it does not.

However, it was commented that there could be a bit width impact on some variables.

This is closely related to item 8 below.

The HLS AHG deferred this topic for further study.

- Change the conformance specification for subpicture sequences for the case where higher layers of an OLS contain multiple subpictures while lower layers of OLS do not use subpicture partitioning, as follows: (JVET-S0098)

  - Support extraction of subpictures that have reference layers with only one subpicture.

This is closely related to item 7 above.

The HLS AHG deferred this topic for further study.

  - Change the derivation of RprConstraintsActive[ i ][ j ] to incorporate the number of subpictures in reference picture and current picture, and set RprConstraintsActive[ i ][ j ] to 1 if the two values differ.

This is to fix a clear problem in the text.

AHG Recommendation (bug fix/expression of existing intent): Adopt this aspect.

  - Signal a new syntax element non_subpic_layers_fraction[ i ] that specifies the fraction of the bitstream level limits associated with layers in the bitstream that have sps_num_subpics_minus1 equal to 0, for each i.

The current derivation is asserted to not be sensible for cases where different layers have different subpicture layouts.

AHG Recommendation (bug fix): Adopt this aspect.

  - Handle accumulated levels for subpicture sets by using one of the following options:

    - Option 1: Remove the accumulation and adjust equation D.11 to use a single subpicture.

    - Option 2: Modify equation D.10 (and D.11).

AHG Recommendation (bug fix): Adopt option 1.

- Change the semantics of the sps_ref_pic_resampling_enabled_flag

- to be aligned with the derivation of the RprConstraintsActive variable, which would be set equal to 1 when one or more of the following 6 parameters differ between the current picture and the reference picture: 1) picture width, 2) picture height, 3) scaling window left offset, 4) scaling window right offset, 5) scaling window top offset, and 6) scaling window bottom offset. (S0048 aspect 2)

- such that when sps_ref_pic_resampling_enabled_flag equal to 1, a slice may refer to a reference picture with a different spatial resolution or a different scaling window. (S0057 aspect 1)

This is editorial – a matter of accurately describing the semantics.

Approach “a” refers to PPS content in SPS semantics, which was suggested to not be appropriate. However, it is more explicit regarding what is involved, and this was suggested to be helpful.

The phrase “spatial resolution” in approach “b” was questioned.

Editor action item: The editor is requested to clarify the semantics as appropriate.

- Add pps_res_change_in_clvs_allowed_flag, and when pps_res_change_in_clvs_allowed_flag is equal to 0, skip pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples and infer their values to be equal to sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples, respectively. (S0057 aspect 2)

It was commented that this could introduce a parsing dependency problem for the tile configuration in the PPS. The proposal had not been tested with an implementation. Thus, no action was recommended for this.

- Add either of the following constraints (S0126):

- When sps_res_change_in_clvs_allowed_flag is equal to 0, all pictures in the CLVS shall have the same values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset, respectively.

- When sps_res_change_in_clvs_allowed_flag is equal to 0, a current picture in the CLVS and a reference picture having the same nuh_layer_id shall have the same values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset, respectively.

It was commented that this constraint would limit some use cases and that a similar constraint had previously been considered and agreed to be undesirable (see JVET-R0058 versions 1-3 aspect 2.c).

No clear problem was agreed to exist, so no action was taken on this.

New submissions (for the June 10th submission deadline)

TBP here onwards.

- Change Equation 37 as follows, considering that a reference layer can only be a lower layer: (S0169 aspect 2) (additions in bold and bigger fonts, removals in italic and smaller fonts)

for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0; j < i  <=  vps_max_layers_minus1; j++ ) {
		dependencyFlag[ i ][ j ] = vps_direct_ref_layer_flag[ i ][ j ]
		for( k = j+1 0 ; k < i; k++ )
			if( vps_direct_ref_layer_flag[ i ][ k ]  &&  dependencyFlag[ k ][ j ] )
				dependencyFlag[ i ][ j ] = 1
	}
	LayerUsedAsRefLayerFlag[ i ] = 0
}
for( i = 0; i  <=  vps_max_layers_minus1; i++ ) {
	for( j = 0, d = 0, r = 0; j < i  <=  vps_max_layers_minus1; j++ ) {	(37)
		if( vps_direct_ref_layer_flag[ i ][ j ] ) {
			DirectRefLayerIdx[ i ][ d++ ] = j
			LayerUsedAsRefLayerFlag[ j ] = 1
		}
		if( dependencyFlag[ i ][ j ] )
			RefLayerIdx[ i ][ r++ ] = j
	}
	NumDirectRefLayers[ i ] = d
	NumRefLayers[ i ] = r
}

- Do either of the following, considering that assertedly vps_num_output_layer_sets_minus1 is signalled when vps_max_layers_minus1 is greater than 0 (and when vps_ols_mode_idc is equal to 2) and in this case there are at least two OLSs: (S0183 aspect 1)

  - Change vps_num_output_layer_sets_minus1 to vps_num_output_layer_sets_minus2.

  - Add a constraint: The value of vps_num_output_layer_sets_minus1 shall be greater than 0.

- Add a flag for each multi-layer OLS (except the first one) to indicate whether the OLS DPB picture width, height, chroma format, and bit depth values are the same as the values for the previous multi-layer OLS, and if the flag is equal to 1, these fields are skipped and inferred. (S0183 aspect 2)

- Change to code the vps_ols_dpb_pic_width[ i ] and vps_ols_dpb_pic_height[ i ] in units of 8 luma samples, considering that assertedly the decoded picture width and height in a CLVS are constrained to be an integer multiple of Max( 8, MinCbSizeY ). (S0212 aspect 1)

- Specify that the ue(v)-coded vps_ols_dpb_bitdepth_minus8[ i ] shall be in the range of 0 to 8, inclusive, considering that the value range is currently missing. (S0212 aspect 2)

### Scalability cleanups (3+3)

JVET-S0063 AHG9: On VPS Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0100 AHG9: On OLS and sublayers [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI), Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

JVET-S0129 AHG9: cleanup on parameter sets and GCI [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

Aspect 1 of this contribution belongs to this category.

JVET-S0169 AHG9: Miscellaneous Cleanups for HLS [B. Wang, S. Esenlik, H. Gao, E. Alshina (Huawei)]

Aspect 2 of this contribution belongs to this category.

JVET-S0183 AHG9: On VPS Information Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

JVET-S0212 AHG9: Modifications on VPS OLS DPB related parameters [R. Yu, M. Pettersson, R. Sjöberg, M. Damghanian, J. Enhorn, J. Ström, D. Liu (Ericsson)]

### Reference picture resampling (RPR) cleanups (4+0)

JVET-S0048 AHG9/AHG8: On reference picture resampling [Y.-K. Wang, Z. Deng, K. Zhang, L. Zhang (Bytedance)]

JVET-S0098 AHG9/12: On subpicture conformance [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

JVET-S0057 AHG8/AHG9: On signaling reference picture resampling [B. Choi, S. Wenger, S. Liu (Tencent)]

JVET-S0126 AHG8: On scaling window constraint [Y.-J. Chang, V. Seregin, Y. He, A. K. Ramasubramonian, M. Coban, M. Karczewicz (Qualcomm)]

– End of AHG report –