| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0241 |
| --- | --- |

| Title: | AHG9: On signalling of POC MSB information in picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Hyeongmun Jang Naeri Park Seung Hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution asserts that the current VVC specification text has a bug that may cause POC derivation process cannot derive POC of IRAP pictures in an independent non-output layer in which pictures other than the IRAP picture have been removed because they are not needed for inter-layer prediction.

To fix the above asserted bug, it is proposed to mandate that POC MSB information shall be present in picture header of a picture when all of the following conditions are true:

- The picture is an IRAP picture or GDR picture with associated ph_recovery_poc_cnt_equal to 0 that is not a CLVSS picture

- The picture is in the layer that is an independent layer, not an output layer, and only IRAP pictures and GDR picture with associated ph_recovery_poc_cnt_equal to 0 from the layer are needed for inter-layer prediction by other layer(s).

- The POC difference between the picture and the previous IRAP picture or GDR picture with associated ph_recovery_poc_cnt_equal to 0 is equal to or more than MaxPicOrderCntLsb / 2.

# Problem statement

It is asserted that the current VVC specification text has a bug that may cause POC derivations fails for a certain use-case that is related to the use of removal of un-referred temporal sub-layers in an independent, non-output layer of an OLS (i.e., related to the use of vps_max_tid_il_ref_pics_plus1[ i ][ j ]). Figure below illustrates the case of two layers in which for one of the OLS the layer 0 is a non-output layer and only IRAP pictures in the layer are needed for inter-layer prediction.

If the POC difference between two IRAP pictures in layer 0 is equal to or more than MaxPicOrderCntLsb / 2 and sps_poc_msb_cycle_flag is equal to 0 (i.e., no POC MSB info is present in picture headers), the current POC derivation does not work because decoder cannot find prevTid0Pic when decoding the 2nd CRA, or the 3rd CRA, or the 4th CRA.

To avoid such situation, POC MSB information should be present in the picture header of those IRAP pictures.

# Proposal

To fix the above asserted bug, it is proposed to mandate that POC MSB information shall be present in picture header of a picture when all of the following conditions are true:

- The picture is an IRAP picture or GDR picture with associated ph_recovery_poc_cnt_equal to 0 that is not a CLVSS picture

- The picture is in the layer that is an independent layer, not an output layer, and only IRAP pictures and GDR picture with associated ph_recovery_poc_cnt_equal to 0 from the layer are needed for inter-layer prediction by other layer(s).

- The POC difference between the picture and the previous IRAP picture or GDR picture with associated ph_recovery_poc_cnt_equal to 0 is equal to or more than MaxPicOrderCntLsb / 2.

The proposed text are as follows:

...

sps_poc_msb_cycle_flag equal to 1 specifies that the ph_poc_msb_cycle_present_flag syntax element is present in PHs referring to the SPS. sps_poc_msb_cycle_flag equal to 0 specifies that the ph_poc_msb_cycle_present_flag syntax element is not present in PHs referring to the SPS.

It is a constraint of bitstream conformance that the value of sps_poc_msb_cycle_flag shall be equal to 1 when both of the following conditions are true:

- The value of sps_video_parameter_set_id is not equal to 0.

- The SPS is referred to by at least a VCL NAL unit with layer id equal to nuh_layer_id and value of NumSubLayersInLayerInOLS[ i ][ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 for any value of i in the range 0 TotalNumOlss − 1, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1.

...

ph_poc_msb_cycle_present_flag equal to 1 specifies that the syntax element ph_poc_msb_cycle_val is present in the PH. ph_poc_msb_cycle_present_flag equal to 0 specifies that the syntax element ph_poc_msb_cycle_val is not present in the PH. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and there is a picture in the current AU in a reference layer of the current layer, the value of ph_poc_msb_cycle_present_flag shall be equal to 0.

It is a constraint of bitstream conformance that the value of ph_poc_msb_cycle_present_flag shall be equal to 1 when all of the following conditions are true:

- The picture associated with the picture header is an IRAP picture or GDR picture with associated ph_recovery_poc_cnt equal to 0 that is not a CLVSS picture.

- The POC difference between the picture associated with the picture header and the IRAP picture or GDR picture with associated ph_recovery_poc_cnt equal to 0 that immediately precedes the picture in decoding order is equal to or greater than MaxPicOrderCntLsb / 2.

- The value of sps_video_parameter_set_id is not equal to 0 and the value of NumSubLayersInLayerInOLS[ i ][ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 for i in the range 0 TotalNumOlss − 1, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0.

...

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).