| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0243 |
| --- | --- |

| Title: | On operation beyond 10-bit |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This submission considers the viability of a future 12-bit profile for VVC and whether VVC can be used unchanged at this operating point. Both the coding efficiency and encoder / decoder complexity are considered.

Above 12-bit an issue with the Golomb-Rice coding is reported as the cause of a significant reduction in performance from VVC. Remedies are suggested and further techniques are described, and it is reported that such modifications might allow VTM to be more efficient than HM. It is also indicated that these changes can improve on the existing performance of VVC at the top end of a future 12 bit profile. The submission also highlights the complexity increase of VVC/VTM over HEVC/HM.

A further issue with bitrates associated with processing RGB 4:4:4 content is described and future work is encouraged.

# Operation in a predicted 12-bit profile

Version 1 of VVC contains only 10-bit 4:2:0 and 4:4:4 profiles. In any future version 2 it is likely that there will be demand for profiles beyond 10-bits, particularly a 12-bit 4:4:4 profile. Assuming that this profile follows the same rules used to derive the 10-bit profiles, it is suggested that the 12-bit profiles would have 20% higher bit rates than the equivalent 10-bit profiles. This gives a 150Mbps target for Main 12 4:4:4 60P (Level 4.1 high tier), and 90Mbps for Main 12 4:4:4 30P (Level 4.0, high tier). Potentially even higher bit rates may need to be considered as bit rates tend to go up more quickly as quality increases.

It should be expected that 12-bit content will need to achieve a higher PSNR than for 10-bit. The table below shows sample PSNRs and mean square errors (MSEs) for a series of different bit depths. Although 60.2dB is technically where 10bit is practically lossless (MSE = 1), it is likely to be enough for a 12-bit profile to be useful as the LSBs will contain more structure than non-predictable information.

| MSE | Bit depth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 8 | 10 | 12 | 14 | 16 |
| 1 | 48.1 | 60.2 | 72.2 | 84.3 | 96.3 |
| 2 | 45.1 | 57.2 | 69.2 | 81.3 | 93.3 |
| 4 | 42.1 | 54.2 | 66.2 | 78.3 | 90.3 |
| 8 | 39.1 | 51.2 | 63.2 | 75.3 | 87.3 |

Table 1. PSNRs for a range of bit depths and MSEs

For the purposes of this submission two sequences are being considered: CrowdRun (16 bit RGB 4:4:4 1920x1080 60P); and Balloon (16 bit YUV 4:4:4 1920x1080 30P PQ-coded). CrowdRun, in common with other RGB 4:4:4 sequences studied[1] can be thought as difficult to compress. Balloon, in common with other classic HDR test sequences, is much more compressible. Although the results within this submission were produced using 16-bit processing with extended precision enabled, they can be considered as equivalent to 12-bit results with extended precision disabled when PSNRs are less than 60 dB, as was shown in JVET-R0351.

Figures 1a and 1b show the results of simulations for both CrowdRun and Balloon in both AI and RA configurations. It can be seen that for both sequences VTM outperforms HM, and Balloon approaches PSNRs which would fall within a 12-bit profile. CrowdRun, however, does not reach such PSNRs indicating that it, in common with other non-HDR 16 bit RGB 4:4:4 sequences, is difficult to compress and may require further investigation. However, it is clear that for at least HDR content, VTM is functional at around 60 dB, and that no changes would be absolutely required, as VTM exceeds the performance of HEVC’s HM over this range.

Figure 1a. Simulation results for CrowdRun in comparison with suggested 12-bit rates.

Figure 1b. Simulation results for CrowdRun and Balloon in comparison with suggested 12-bit bitrates.

Some improvement can be made to the Golomb-Rice coding of coefficients at the higher bit rate end of the 12-bit profile. These are presented in section 3.

Given the increased complexity of VVC over HEVC, and the corresponding increase in encoder and decoder times (see table 2), it could be argued that there is not enough differentiation in performance as bit rates increase, including around the higher end of the 12-bit profile range. However, at this time VVC is believed to be suitable as the graphs for Balloon indicate a consistent lead for VTM over HM up to the top of the profile. One way of mitigating the gain in complexity may be to study the applicability of tools and identify any that become less effective as bit rates increase.

| QP | CrowdRun |  |  |  | Balloon |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM 16.21 |  | VTM 9.0 |  | HM 16.21 |  | VTM 9.0 |  |
|  | Encoder (Hours) | Decoder (Secs) | Encoder (Hours) | Decoder (Secs) | Encoder (Hours) | Decoder (Secs) | Encoder (Hours) | Decoder (Secs) |
| 40 | 0.69 | 5.6 | 9.32 | 20.7 | 0.23 | 2.4 | 0.58 | 5.6 |
| 20 | 2.16 | 19.7 | 61.90 | 52.3 | 0.27 | 2.8 | 2.73 | 8.2 |
| 0 | 2.92 | 37.2 | 63.38 | 76.0 | 1.11 | 10.9 | 28.74 | 28.7 |
| -20 | 3.20 | 55.9 | 76.62 | 82.9 | 1.89 | 23.2 | 36.00 | 47.4 |

Table 2. Encoder and decoder times for VTM and HM for 1 GOP at different QPs

One area of further future study may be lossless 12-bit coding where the bitrate is unlimited.

# Operation beyond 12-bit profile

As bit depth increases, it becomes more likely that the LSBs of coefficients are by nature noisier and less structured. Therefore PSNRs/MSE should be followed more rigidly as a guide for the operating point of the codec. i.e. 96 dB is where the MSE=1 for 16-bit, and therefore the point where the 16th bit is actually starting to be preserved during the compression process. For lossless 16-bit, obviously even higher operating points would need to be considered.

To accommodate this quality increase, HEVC also included a parameter called HbrFactor (high bit rate), which increased the bit rates by a factor of 6 for some profiles, but not for the general 16-bit profiles.

As bit rates increase, VTM currently diverges greatly from HM, typically underperforming below QP = 0, as shown in figure 2. Ways to mitigate this have been investigated and rely on modifications to the Golomb-Rice Coding, as detailed in section 3 of this document. An example of these modifications can be seen in figure 3.

If a profile is considered beyond 12-bit, any coding gain would likely come from coefficient coding changes and many of the tools that were effective at lower bit rates could probably be removed. By excluding tools which can no longer prove their efficacy as PSNRs increase, the complexity of a high bit depth profile can be reduced. Further work will be needed to identify which tools in the current 10-bit profiles should be kept in profiles beyond 12-bits.

It is also suggested that inter tools are less useful at the high PSNRs associated with profiles beyond 12-bit as they may simply be adding noise to the residual. In addition, the figures in table 2 show how encoder times can rise dramatically as QP reduces. This could make inter processing impractical at high PSNRs.

In addition to restricting the use of tools there are potentially other ways of reducing the complexity of VVC at these operating points. Some of these will relate to partitioning as at high PSNRs almost all blocks will be 4x4 and few, if any, will be larger than 8x8. It therefore makes little sense starting the partitioning process with a CTU of 128x128.

# Improvements to Golomb-Rice coding

From figures 2a and 2b it can be seen that VTM appears to outperform HM for high QPs down to a certain level, but once this point is reached the bitrate for VTM increases significantly more than for HM. In common with other RGB 4:4:4 sequences studied, this point is at approximately 63dB for CrowdRun. This point is somewhat higher for Balloon as much of the image does not have full-scale luma or chroma information.

Figure 2a. Bitrate comparisons for VTM 9.0 and HM 16.21 for CrowdRun

Figure 2b. Bitrate comparisons for VTM 9.0 and HM 16.21for Balloon.

All the simulations presented in this section are AI at even-numbered QPs in a range 40 to -48 to cover both the CTC range and the lowest possible QPs for 16-bit processing.

## Implementing Rice counters

By analyzing VTM and HM results beyond the point that VTM outperforms HM it was found that the principal reason for the poor performance of VTM was the coding of coefficients using Golomb-Rice. Whilst HM allows the Rice parameter used to vary with the size of the coefficients, VTM constrains these values to a range of 0 to 3 for RRC and precisely 1 for TSRC.

Experiments were conducted using VTM modified to use a form of the Rice parameter derivation technique in HEVC. HEVC maintains 4 counters (split by luma/chroma and transform/transform-skip). These counters are reset to 0 at the start of each slice.

When a coefficient is coded, the Rice parameter value is derived from selected counter as follows:

riceParameter = counter / 4

Once coded, the counter is updated if this is the first coefficient coded in the sub-TU as follows:

if (coeffValue >= (3 << rice)) counter++

if (((coeffValue << 1) < (1 << riceParameter)) && (counter > 0)) counter--;

Thus, as coefficient coding continues, a better approximation for the most efficient Rice parameter is achieved.

For VTM, two different forms of this technique were tried. The first mirrored the approach taken in HM (single Rice), where 4 counters were used and selected depending on luma/chroma, transform/transform-skip.

The second method used multiple sets of such counters with the use of each set providing counters for different block sizes (4x4 and above) and coefficient positions in the TU (Manhattan distance from the top left coefficient, scaled by block size and capped at 4). In addition, the second method updates the counter for each coefficient coded in the sub-TU – not just the first.

Figure 3. Bitrate comparisons for single and multiple Rice methods

Figure 3 shows the results for both the single and multiple Rice methods. Both methods outperform VTM for high PSNRs and are closer to HM in this region. From these graphs, it is clear that the multiple Rice technique outperforms the single Rice technique for the sequences studied. For this reason, the following additional techniques use the multiple Rice technique as their basis.

## Adding a Rice-based shift to existing VVC

The existing Rice parameter derivation technique for transform blocks in VVC uses the 5 nearest coefficients to the bottom right of the coefficient being coded and a lookup table. The technique tested in this sub-section takes this basic method and adds a shift and an addition to accommodate the larger coefficients found when coding values at higher PSNRs. This can most easily be explained as follows (by reference to the current specification text in section 9.3.3.2[2] / VTM 9.0 software).

Here the initial riceParameter is derived using the multiple Rice technique described above:

shift = max(0, riceParameter – 2)

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs += AbsLevel[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ]
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAbs += AbsLevel[ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, (locSumAbs − baseLevel * 5) >> shift )

riceParameter = g_auiGoRiceParsCoeff[locSumAbs] + shift;

For transform skip, the same approach to transform coefficients was used, but the coefficients used in the calculation were to the top left rather than the bottom right.

Figures 4a and 4b show the results for this technique. At the highest QPs both the multiple Rice and the shift technique described in this section behave similarly. However, the shift technique outperforms the multiple Rice technique in the region in which VTM performance starts to drop off, which is about 60dB for RGB 4:4:4 content. In this region, the shift technique performs better than not only the multiple Rice technique but also both VTM and HM. For CrowdRun this gain is in excess of 1 dB at QPs which may be applicable to a 12-bit profile (especially for lossless).

Figure 4a. Bitrate comparisons for Rice based shift method for CrowdRun.

Figure 4b. Bitrate comparisons for Rice based shift method for Balloon.

## Local Rice adaptation

A further technique investigated as an alternative to the shift technique described above was to vary the Rice parameter used by comparing local coefficients that have already been coded with the value predicted by the Rice parameter. In this experiment the same five coefficients closest to the bottom right (top left for TS) were used in this comparison. The pseudo-code below shows how the summing of these local coefficients has been modified from the VVC specification to add a count of the coefficients.

sum = 0

count = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	sum += AbsLevel[ xC + 1 ][ yC ]

count++
	if( xC < (1 << log2TbWidth) − 2 ) {
		sum += AbsLevel[ xC + 2 ][ yC ]
		count++

}
	if( yC < (1 << log2TbHeight) − 1 ) {
		sum += AbsLevel[ xC + 1 ][ yC + 1 ]
		count++

}
}
if( yC < (1 << log2TbHeight) − 1 ) {
	sum += AbsLevel[ xC ][ yC + 1 ]
	count++
	if( yC < (1 << log2TbHeight) − 2 ) {
		sum += AbsLevel[ xC ][ yC + 2 ]
		count++

}
}

The value predicted by these local values is then calculated, and riceParameter (whose initial value is derived using the multiple Rice technique described earlier) is incremented or decremented as follows:

localValue = count = 0 ? -1 : Max( 0, sum/count − baseLevel )

if (localValue > 1 << riceParameter) {

while (localValue > (3 << riceParameter)) riceParameter++

} else if (riceParameter > 0) {

while (localValue < (3 << (riceParameter-2))) riceParameter—

}

The calculation of localValue shown here uses a division. This might be replaced by a different function of the five local coefficients such as max or min. In addition, the size of the adaption in the second stage can be limited by reducing the number of times the loop can be iterated. The figures below show the results if the Rice parameter may vary by plus or minus two from the Rice parameter derived from the counter.

Figures 5a and 5b show that like the previous technique, this technique outperforms both VTM and HM around the point that VTM performance drops away. However, it also shows gains over HM and the previously described techniques at higher PSNRs up to the point that codec becomes virtually lossless. Performance at some QPs (typically those within the low QP Main 10 test conditions range) is less good – the previously described shift technique offers lower bitrates in this range. Despite this, the improvements offered by this technique are applicable to coding at ranges within a future 12-bit profile, particularly if lossless.

Figure 5a. Comparison of bitrates for local Rice adaptation for CrowdRun.

Figure 5b. Comparison of bitrates for local Rice adaptation for Balloon.

## Reducing the number of flags

Currently VVC uses a series of CABAC coded bins to code flags for coefficients (significance, gt1, gt2, and parity). However, as QP decreases and PSNR increases both the number of coefficients and their values become larger. As this happens all the flags other than parity become very efficiently coded but offer little information, and the parity flag becomes less efficiently coded until it reaches a point at which it costs more than one bit to code a flag.

Figures 6a and 6b show the effect of not coding any of these four flags and using bypass coding for every coefficient when using the multiple Rice technique described above.

It is clear from these graphs that although coding coefficient flags offers gains at normal CTC QPs, these gains reduce as the QP goes down below 0, and for CrowdRun actually causes losses. Although these losses may not be significant a potentially more important aspect is that coding these bins with CABAC at such QPs is an unnecessary complexity

Experiments have also been completed which attempt to switch off some or all of these flags as the size of coefficients increases. This is done by calculating a value, blockParameter, before coding coefficients for the sub block. In the experiments conducted this value was computed using the same method as the Rice parameter for the coefficient at the top left of the sub block. Note – this is not necessarily the Rice parameter that will be used to code this coefficient as further updates to the counter for the Rice parameter may occur during coefficient coding.

Using blockParameter it should be possible to construct a system which only uses the full set of flags when it can be predicted they will be beneficial. For example:

if (blockParameter > 10) {

skip flags

} else if (blockParameter > 7)

code significance

} else if (blockParameter > 4) {

code significance, parity

} else {

code significance, gt1, gt2, parity

}

Figure 6a. Bitrate comparison of not coding coefficient flags for CrowdRun.

Figure 6b. Bitrate comparison of not coding coefficient flags for Balloon.

## Blending / selecting techniques

This section has described a number of techniques to improve the coding of coefficient data at QPs, PSNRs and bitrates that might be within future profiles of VVC. In particular the shift technique and the local adaptation technique may look promising at levels not far outside the currently considered range for version 1. The use of combinations of such techniques might also prove advantageous, potentially selected using an initial Rice parameter derived using the multiple Rice technique described earlier. For example:

if (riceParameter > 5) {

riceParamter = shift(riceParameter)

} else {

riceParamter = local(riceParameter)

}

The ideal combination to use is a subject for further work, but this would also require confirmation on test conditions, and interest by the committee for such operating points.

# Conclusion

As expected, VVC’s VTM is better than HEVC’s HM, in terms of coding efficiency, for the operating points defined within the profiles that are to be standardized in VVC version 1. In addition, it also appears to outperform HM over a range of PSNR which might be expected to be applicable to a 12-bit (lossy) profile.

Issues have been identified with coefficient coding at the top end of the 12-bit operating range and beyond. These issues can be resolved by modifying the Golomb-Rice coding of coefficient data. A number of candidate techniques have been described that offer coding gain.

When examining the need for VVC to support high bit depths it should be noted that VTM is significantly more complicated and slower than HM, and this leads to the question:

- How can VVC/VTM be modified so that its complexity/coding trade off is seen to give an advantage of HEVC/HM?

It is suggested that some of the tools currently implemented in VTM become less useful at high PSNRs and that reducing the maximum block size might also reduce the complexity of VVC without affecting bitrate.

Issues have been identified with RGB processing as the bitrates associated with such processing seem to exceed expected high tier operating points. Further work is encouraged.

Further study by JVET is encouraged if profiles above 10-bit are to be considered in any future version 2.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “High bit depth coding”, JVET-R0351, A. Browne, S. Keating and K. Sharman

[2] “Versatile Video Coding (Draft 9)”, JVET-R2001-vA, B. Bross, J. Chen, S. Liu and Y-K. Wang