| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0244 |
| --- | --- |

| Title: | AHG13: On RGB Common Test Condition Regarding LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Hendry Seung Hwan Kim | Email: | jie.zhao@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to disable LMCS (--LMCSEnable=0) for RGB sequences in the non-420 common test condition. This is configuration setting change only. Results show that by turning off LMCS, VTM gives better performance for both RGB natural and RGB screen content.

For RGB Natural content, VTM9 encoder already decides to disable LMCS for 3 out of the 8 sequences.  Average gain of turning off LMCS to all sequences is (-2.07%, 0.59%,-0.55%) for AI, (-1.77%	-2.31%	-1.59%) for RA, -(xxx%, xx%,xx%) for LDB..

For RGB Screen Content, VTM9 encoder already decides to disable LMCS for 13 out of 14 sequences.. For the one sequence that keeps LMCS enabled, turning off LMCS gives gain of (-0.6%, -0.3%,-1.2%) for AI, (-0.2%, 0.5%, -0.9%) for RA, and (xx%, xx%, xx%) for LDB.

If using VTM9 with the bug fix proposed in JVET-S0234 as reference, turning off LMCS still performs better. For RGB natural content, the average gain is (-1.66%, 0.54%,-0.21%) for AI, (-1.28%, -2.29%, -1.08%) for RA, and (xx%, xx%, xx%) for LDB.

# Proposal

In VVC, luma mapping with chroma scaling (LMCS) is a processing block before the loop filters. LMCS has two main components: 1) in-loop mapping of the luma component based on adaptive piecewise linear models; 2) for the chroma components, luma-dependent chroma residual scaling is applied.

Regardless of the actual colour format (YUV, or RGB) of the source, LMCS tool treats one of the channels as luma, and the other channels are chroma. For source of RGB (or GBR) format, perform mapping on one of the color channel, while scaling the other two color channel based on the 1st color channel, may not give desirable result.

In addition, the non-normative LMCS implementation in VTM9 encoder may not have been specially tuned for RGB sequences. As shown in the next section, result show that for RGB sequences, VTM9 performs better without LMCS. Therefore, we propose to disable LMCS for RGB sequences in non-4:2:0 common test conditions. This is config setting change only. It does not need spec or software change. The configuration parameter to change is:

LMCSEnable:                0

.

# Results

The results are evaluated on VTM9.0 [2]. Reference is VTM9 following non-4:2:0 common test conditions [3]].

## RGB Screen Content

## In VTM9, for RGB screen content test sequences, 13 out of the 14 sequences, LMCS has already been disabled according to encoder decision even when config setting LMCSEnable=1. For the one sequence “ArenaOfValor” that encoder decides to keep LMCS enabled, if disable LMCS, it has gain of (-0.6%, -0.3%, -1.2%) in AI and (-0.2%, 0.5%, -0.9%) in RA.

Table 2.1 Performance of LMCSEnable=0

|  |  | AI |  |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FlyingGraphics | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Desktop | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Console | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| ChineseEditing | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Web_Browsing | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Map | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Programming | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SlideShow | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| ArenaOfValor | -0.6% | -0.3% | -1.2% | -0.2% | 0.5% | -0.9% | #VALUE! | #VALUE! | #VALUE! |
| Robot | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| GlassHalf | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | #VALUE! | #VALUE! | #VALUE! |
| Basketball_Screen | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| MissionControlClip2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| MissionControlClip3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

## RGB Natural Content

## In VTM9, for RGB natural content test sequences, 3 out of the 8 sequences, LMCS has already been disabled according to encoder decision..

Table 2.1 Performance of LMCSEnable=0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.36% | -0.64% | -1.16% | 95% | 94% |
| Kimono | -6.62% | 0.62% | -1.30% | 94% | 96% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 98% | 98% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 103% | 100% |
| VenueVu | 0.00% | 0.00% | 0.00% | 99% | 96% |
| DucksAndLegs | -2.62% | 0.96% | 0.09% | 97% | 98% |
| OldTownCross | -6.31% | 3.65% | -1.51% | 96% | 96% |
| ParkScene | -0.68% | 0.13% | -0.51% | 101% | 100% |
| Overall | -2.07% | 0.59% | -0.55% | 95% | 98% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.45% | -3.33% | -2.13% | 100% | 99% |
| Kimono | -10.37% | -6.86% | -8.20% | 100% | 102% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 100% | 99% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 100% | 101% |
| VenueVu | 0.00% | 0.00% | 0.00% | 100% | 98% |
| DucksAndLegs | -1.57% | -2.40% | 0.56% | 100% | 100% |
| OldTownCross | -1.29% | -5.17% | -2.38% | 98% | 97% |
| ParkScene | -0.48% | -0.73% | -0.56% | 98% | 98% |
| Overall | -1.77% | -2.31% | -1.59% | 97% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.97% | -0.74% | -0.86% | 95% | 100% |
| Kimono | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 99% | 95% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 100% | 93% |
| VenueVu | 0.00% | 0.00% | 0.00% | 101% | 101% |
| DucksAndLegs | -4.05% | 0.60% | -0.65% | 92% | 94% |
| OldTownCross | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| ParkScene | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

In JVET-S0234, a bug fix related to ACT and LMCS is proposed. If using VTM9 with the bug fix as reference, the gain of disable LMCS would be the 3rd row in the table below. Overall, it still performs better without LMCS.

| RGB Natual |  | AI |  |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RGB 4:4:4 LMCS off vs. VTM9 | -2.07% | 0.59% | -0.55% | -1.77% | -2.31% | -1.59% | #VALUE! | #VALUE! | #VALUE! |
| VTM9 with bug fix in JVET-S0234 | -0.41% | 0.05% | -0.34% | -0.49% | -0.02% | -0.51% | #VALUE! | #VALUE! | #VALUE! |
| RGB 4:4:4 LMCS off vs. VTM9 with bug fix | -1.66% | 0.54% | -0.21% | -1.28% | -2.29% | -1.08% |  |  |  |

# Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 9)”, JVET-R2001, April 2020, Teleconference

- VTM-9.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-9.0.

- Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-R2013, April 2020, Teleconference

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).