| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0244 |
| --- | --- |

| Title: | AHG13: On RGB Common Test Condition Regarding LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao Hendry Seung Hwan Kim | Email: | jie.zhao@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution reports the LMCS tool off performance for RGB sequences. Result show that LMCS performance on RGB sequences is questionable; it has quite large loss on many RGB sequences. It is unclear whether this is because LMCS is not suitable for RGB sequences or just VTM encoder was not doing a good job on evaluating LMCS parameters. To avoid the confusion and negative performance impact, it is proposed to turn LMCS off for RGB sequences in the non-420 common test condition. This is configuration setting change only.

For RGB Natural content, VTM9 encoder already decides to disable LMCS for 3 out of the 8 sequences.  Using VTM9 as reference, average gain of turning off LMCS to all sequences is (-2.07%, 0.59%,-0.55%) for AI, (-1.77%	-2.31%	-1.59%) for RA, -(xx%, xx%, xx%) for LDB.

For RGB Screen Content, VTM9 encoder already decides to disable LMCS for 13 out of 14 sequences.. For the one sequence that keeps LMCS enabled, turning off LMCS gives gain of (-0.6%, -0.3%,-1.2%) for AI, (-0.2%, 0.5%, -0.9%) for RA, and (xx%, xx%, xx%) for LDB.

If using VTM9 with the mismatch fix proposed in JVET-S0234 as reference, turning off LMCS still performs better for most of the RGB sequences. For RGB natural content, the average gain is (-1.66%, 0.54%,-0.21%) for AI, (-1.28%, -2.29%, -1.08%) for RA, and (xx%, xx%, xx%) for LDB.

[This r1 added more results and per-sequence comparison with JVET-S0234]

# Proposal

In VVC, luma mapping with chroma scaling (LMCS) is a processing block before the loop filters. LMCS has two main components: 1) in-loop mapping of the luma component based on adaptive piecewise linear models; 2) for the chroma components, luma-dependent chroma residual scaling is applied.

Regardless of the actual colour format (YUV, or RGB) of the source, LMCS tool treats one of the channels as luma, and the other channels are chroma. For source of RGB (or GBR) format, perform mapping on one of the color channel, while scaling the other two color channel based on the 1st color channel, may not give desirable result.

In addition, the non-normative LMCS implementation in VTM9 encoder may not have been specially tuned for RGB sequences. As shown in the next section, result show that for RGB sequences, VTM9 performs better without LMCS on most of the sequences. Therefore, we propose to disable LMCS for RGB sequences in non-4:2:0 common test conditions. This is configuration setting change only. It does not need spec or software change. The configuration parameter to change is:

LMCSEnable:                0

.

# Results

The results are evaluated on VTM9.0 [2]. Reference is VTM9 following non-4:2:0 common test conditions [3]].

## RGB Screen Content

## In VTM9, for RGB screen content test sequences, 13 out of the 14 sequences, LMCS has already been disabled according to encoder decision even when config setting LMCSEnable=1. For the one sequence “ArenaOfValor” that encoder decides to keep LMCS enabled, if disable LMCS, it has gain of (-0.6%, -0.3%, -1.2%) in AI and (-0.2%, 0.5%, -0.9%) in RA.

Table 2.1 Performance of LMCSEnable=0

|  |  | AI |  |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FlyingGraphics | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Desktop | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Console | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| ChineseEditing | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Web_Browsing | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Map | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Programming | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| SlideShow | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| ArenaOfValor | -0.6% | -0.3% | -1.2% | -0.2% | 0.5% | -0.9% | #VALUE! | #VALUE! | #VALUE! |
| Robot | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| GlassHalf | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Basketball_Screen | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| MissionControlClip2 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| MissionControlClip3 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |

## RGB Natural Content

## In VTM9, for RGB natural content test sequences, 3 out of the 8 sequences, LMCS has already been disabled according to encoder decision..

Table 2.1 Performance of LMCSEnable=0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.36% | -0.64% | -1.16% | 95% | 94% |
| Kimono | -6.62% | 0.62% | -1.30% | 94% | 96% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 98% | 98% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 103% | 100% |
| VenueVu | 0.00% | 0.00% | 0.00% | 99% | 96% |
| DucksAndLegs | -2.62% | 0.96% | 0.09% | 97% | 98% |
| OldTownCross | -6.31% | 3.65% | -1.51% | 96% | 96% |
| ParkScene | -0.68% | 0.13% | -0.51% | 101% | 100% |
| Overall | -2.07% | 0.59% | -0.55% | 95% | 98% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | -0.45% | -3.33% | -2.13% | 100% | 99% |
| Kimono | -10.37% | -6.86% | -8.20% | 100% | 102% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 100% | 99% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 100% | 101% |
| VenueVu | 0.00% | 0.00% | 0.00% | 100% | 98% |
| DucksAndLegs | -1.57% | -2.40% | 0.56% | 100% | 100% |
| OldTownCross | -1.29% | -5.17% | -2.38% | 98% | 97% |
| ParkScene | -0.48% | -0.73% | -0.56% | 98% | 98% |
| Overall | -1.77% | -2.31% | -1.59% | 97% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Traffic | 0.97% | -0.74% | -0.86% | 95% | 100% |
| Kimono | -0.61% | -0.96% | -0.80% | 92% | 105% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 99% | 95% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 100% | 93% |
| VenueVu | 0.00% | 0.00% | 0.00% | 101% | 101% |
| DucksAndLegs | -4.05% | 0.60% | -0.65% | 92% | 94% |
| OldTownCross | #VALUE! | #VALUE! | #VALUE! | 0% | 0% |
| ParkScene | -0.50% | -0.88% | -0.84% | 98% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Results comparing to JVET-S0234

In JVET-S0234, a bug fix related to ACT and LMCS is proposed. If using VTM9 with the JVET-S0234 bug fix as reference, the performance of disabling LMCS is shown the 3rd row in the table below. Overall, turning off LMCS still performs better.

RGB Natural

|  |  | AI |  |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RGB LMCS off vs. VTM9 | -2.07% | 0.59% | -0.55% | -1.77% | -2.31% | -1.59% |  |  |  |
| VTM9 with mismatch fix in JVET-S0234 | -0.41% | 0.05% | -0.34% | -0.49% | -0.02% | -0.51% | -0.34% | 0.19% | -0.30% |
| LMCS off vs JVET-S0234 | -1.66% | 0.54% | -0.21% | -1.28% | -2.29% | -1.08% |  |  |  |

The side-by-side per-sequence results of LMCS-off and JVET-S0234 are as below. Anchors are VTM9.

| All Intra | VTM9 LMCS off |  |  | VTM9 with JVET-S0234 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Traffic | -0.36% | -0.64% | -1.16% | -0.89% | 0.01% | -0.90% |
| Kimono | -6.62% | 0.62% | -1.30% | -0.25% | 0.00% | -0.62% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| VenueVu | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| DucksAndLegs | -2.62% | 0.96% | 0.09% | -0.48% | 0.13% | -0.68% |
| OldTownCross | -6.31% | 3.65% | -1.51% | -1.45% | 0.22% | -0.40% |
| ParkScene | -0.68% | 0.13% | -0.51% | -0.20% | 0.04% | -0.14% |
| Overall | -2.07% | 0.59% | -0.55% | -0.41% | 0.05% | -0.34% |

| RA | VTM9 LMCS off |  |  | VTM9 with JVET-S0234 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Traffic | -0.45% | -3.33% | -2.13% | -2.08% | -0.23% | -1.86% |
| Kimono | -10.37% | -6.86% | -8.20% | -0.83% | -0.19% | -0.87% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| VenueVu | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| DucksAndLegs | -1.57% | -2.40% | 0.56% | -1.35% | -0.12% | -0.77% |
| OldTownCross | -1.29% | -5.17% | -2.38% | 0.33% | 0.31% | -0.56% |
| ParkScene | -0.48% | -0.73% | -0.56% | 0.01% | 0.06% | -0.06% |
| Overall | -1.77% | -2.31% | -1.59% | -0.49% | -0.02% | -0.51% |

| LDB | VTM9 LMCS off |  |  | VTM9 with JVET-S0234 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Traffic | 0.97% | -0.74% | -0.86% | -0.45% | 0.09% | -0.61% |
| Kimono | -0.61% | -0.96% | -0.80% | -0.44% | 0.02% | -0.45% |
| EBULupoCandlelight | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| EBURainFruits | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| VenueVu | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| DucksAndLegs | -4.05% | 0.60% | -0.65% | 0.23% | 0.07% | 0.28% |
| OldTownCross | #VALUE! | #VALUE! | #VALUE! | -1.53% | 1.30% | -1.39% |
| ParkScene | -0.50% | -0.88% | -0.84% | -0.58% | 0.02% | -0.19% |
| Overall | #VALUE! | #VALUE! | #VALUE! | -0.34% | 0.19% | -0.30% |

RGB Screen Content :

This table below shows ArenaOfValor only. There is no change on other sequences since LMCS is off by VTM9 encoder decision.

| ArenaOfValor |  | AI |  |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LMCS off vs. VTM9 | -0.6% | -0.3% | -1.2% | -0.2% | 0.5% | -0.9% |  |  |  |
| VTM9 with JVET-S0234 mismatch fix | -0.9% | 0.3% | -0.5% | -0.5% | -0.2% | -0.8% | 0.4% | -0.1% | -0.3% |
| LMCS off vs. VTM9 with JVET-S0234 | 0.3% | -0.6% | -0.7% | 0.3% | 0.7% | -0.1% |  |  |  |

# Conclusion

This contribution shows the VTM9 performance on RGB sequences with LMCS is questionable. It has quite large loss on many sequences. It is unclear whether this is because LMCS is not suitable for RGB sequences or just VTM encoder is not doing a good job on evaluating LMCS parameters. Until LMCS can show gain on RGB sequences, to avoid the confusion and negative impact on evaluating other tools, it is proposed to turn LMCS off for RGB sequences in the non-420 common test condition.

# Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 9)”, JVET-R2001, April 2020, Teleconference

- VTM-9.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-9.0.

- Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-R2013, April 2020, Teleconference

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).