| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0245 |
| --- | --- |

| Title: | AHG9: Editorial clarifications of APS ID for LMCS_APS and SCALING_APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn, Jongseok Lee, Donggyu Sim (Digital Insights Inc.) Seungwook Park (Hyundai Motors) | Tel: Email: | +82-2-941-6470 {yjahn, jslee, dgsim} @digitalinsights.co.kr seungwook.park@hyundai.com |
| Source: | Digital Insights Inc., Hyundai Motors |  |  |

_____________________________

# Abstract

This contribution proposes some editorial clarifications related to adaptation parameter set id for luma mapping with chroma scaling and scaling list. It is asserted that the current specification text has a problem when using multiple LMCS_APSs and SCALING_APSs. In this contribution, it is suggested to extend the LMCS and scaling list parameters with adaptation parameter set id in VVC spec.

# Introduction

To make the spec clearer, this contribution proposes the following aspects:

- Proposal 1: Editorial clarification of adaptation parameter set id and related decoding process for LMCS_APS

- Proposal 2: Editorial clarification of adaptation parameter set id and related decoding process for SCALING_APS

# Specification Text – Proposal 1

#### 7.3.2.20	Luma mapping with chroma scaling data syntax

| lmcs_data( ) { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for( i = lmcs_min_bin_idx; i <= 15 − lmcs_delta_max_bin_idx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if( lmcs_delta_abs_cw[ i ] > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| if( aps_chroma_present_flag ) { |  |
| lmcs_delta_abs_crs | u(3) |
| if( lmcs_delta_abs_crs > 0 ) |  |
| lmcs_delta_sign_crs_flag | u(1) |
| } |  |
| } |  |

#### 7.4.3.20		Luma mapping with chroma scaling data semantics

lmcs_min_bin_idx specifies the minimum bin index, LmcsMinBinIdx[ aps_adaptation_parameter_set_id ], used in the luma mapping with chroma scaling construction process. The value of lmcs_min_bin_idx shall be in the range of 0 to 15, inclusive. The value of LmcsMinBinIdx[ aps_adaptation_parameter_set_id ] is set equal to lmcs_min_bin_idx.

lmcs_delta_max_bin_idx specifies the delta value between 15 and the maximum bin index LmcsMaxBinIdx[ aps_adaptation_parameter_set_id ] used in the luma mapping with chroma scaling construction process. The value of lmcs_delta_max_bin_idx shall be in the range of 0 to 15, inclusive. The value of LmcsMaxBinIdx[ aps_adaptation_parameter_set_id ] is set equal to 15 − lmcs_delta_max_bin_idx. The value of LmcsMaxBinIdx[ aps_adaptation_parameter_set_id ] shall be greater than or equal to lmcs_min_bin_idx.

lmcs_delta_cw_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax lmcs_delta_abs_cw[ i ]. The value of lmcs_delta_cw_prec_minus1 shall be in the range of 0 to 14, inclusive.

lmcs_delta_abs_cw[ i ] specifies the absolute delta codeword value for the ith bin.

lmcs_delta_sign_cw_flag[ i ] specifies the sign of the variable lmcsDeltaCW[ i ]  as follows:

- If lmcs_delta_sign_cw_flag[ i ] is equal to 0, lmcsDeltaCW[ i ] is a positive value.

- Otherwise ( lmcs_delta_sign_cw_flag[ i ] is not equal to 0 ), lmcsDeltaCW[ i ] is a negative value.

When lmcs_delta_sign_cw_flag[ i ] is not present, it is inferred to be equal to 0.

The variable OrgCW is derived as follows:

OrgCW = ( 1  <<  BitDepth ) / 16	(98)

The variable lmcsDeltaCW[ i ], with i = lmcs_min_bin_idx..15 − lmcs_delta_max_bin_idx, is derived as follows:

lmcsDeltaCW[ i ] = ( 1 − 2 * lmcs_delta_sign_cw_flag[ i ] ) * lmcs_delta_abs_cw[ i ]	(99)

The variable lmcsCW[ i ] is derived as follows:

- For i = 0.. lmcs_min_bin_idx − 1, lmcsCW[ i ] is set equal 0.

- For i = lmcs_min_bin_idx..15 − lmcs_delta_max_bin_idx, the following applies:

lmcsCW[ i ] = OrgCW + lmcsDeltaCW[ i ]	(100)

The value of lmcsCW[ i ] shall be in the range of OrgCW  >>  3 to ( OrgCW  <<  3 ) − 1, inclusive.

- For i = 16 − lmcs_delta_max_bin_idx..15, lmcsCW[ i ] is set equal 0.

It is a requirement of bitstream conformance that the following condition is true:

i=015lmcsCW[ i ]  <=  ( 1  <<  BitDepth ) − 1	(101)

The variable InputPivot[ aps_adaptation_parameter_set_id ][ i ], with i = 0..15, is derived as follows:

InputPivot[ aps_adaptation_parameter_set_id ][ i ] = i * OrgCW	(102)

The variable LmcsPivot[ aps_adaptation_parameter_set_id ][ i ] with i = 0..16, the variables ScaleCoeff[ aps_adaptation_parameter_set_id ][ i ] and InvScaleCoeff[ aps_adaptation_parameter_set_id ][ i ] with i = 0..15, are derived as follows:

LmcsPivot[ aps_adaptation_parameter_set_id ][ 0 ] = 0
for( i = 0; i  <=  15; i++ ) {
	LmcsPivot[ aps_adaptation_parameter_set_id ][ i + 1 ] = LmcsPivot[ aps_adaptation_parameter_set_id ][ i ] + lmcsCW[ i ]
	ScaleCoeff[ aps_adaptation_parameter_set_id ][ i ] = ( lmcsCW[ i ] * (1  <<  11 ) + ( 1  <<  ( Log2( OrgCW ) − 1 ) ) )  >>  ( Log2( OrgCW ) )
	if( lmcsCW[ i ]  = =  0 )	(103)
		InvScaleCoeff[ aps_adaptation_parameter_set_id ][ i ] = 0
	else
		InvScaleCoeff[ aps_adaptation_parameter_set_id ][ i ] = OrgCW * ( 1  <<  11 ) / lmcsCW[ i ]
}

It is a requirement of bitstream conformance that, for i = lmcs_min_bin_idx..15 − lmcs_delta_max_bin_idx, when the value of LmcsPivot[ aps_adaptation_parameter_set_id ][ i ] is not a multiple of 1  <<  ( BitDepth − 5 ), the value of ( LmcsPivot[ aps_adaptation_parameter_set_id ][ i ]  >>  ( BitDepth − 5 ) ) shall not be equal to the value of ( LmcsPivot[ aps_adaptation_parameter_set_id ][ i + 1 ]  >>  ( BitDepth − 5 ) ).

lmcs_delta_abs_crs specifies the absolute codeword value of the variable lmcsDeltaCrs. When not present, lmcs_delta_abs_crs is inferred to be equal to 0.

lmcs_delta_sign_crs_flag specifies the sign of the variable lmcsDeltaCrs. When not present, lmcs_delta_sign_crs_flag is inferred to be equal to 0.

The variable lmcsDeltaCrs is derived as follows:

lmcsDeltaCrs = ( 1 − 2 * lmcs_delta_sign_crs_flag ) * lmcs_delta_abs_crs	(104)

It is a requirement of bitstream conformance that, when lmcsCW[ i ] is not equal to 0, ( lmcsCW[ i ] + lmcsDeltaCrs ) shall be in the range of ( OrgCW >> 3 ) to ( ( OrgCW << 3 ) − 1 ), inclusive.

The variable ChromaScaleCoeff[ aps_adaptation_parameter_set_id ][ i ], with i = 0…15, is derived as follows:

if( lmcsCW[ i ]  = =  0 )
	ChromaScaleCoeff[ aps_adaptation_parameter_set_id ][ i ] = ( 1  <<  11 )
else					(105)
	ChromaScaleCoeff[ aps_adaptation_parameter_set_id ][ i ] = OrgCW * ( 1  <<  11 ) / ( lmcsCW[ i ] + lmcsDeltaCrs )

#### 8.5.6.7	Weighted sample prediction process for combined merge and intra prediction

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

- the width of the current coding block cbWidth,

- the height of the current coding block cbHeight,

- two (cbWidth)x(cbHeight) arrays predSamplesInter and predSamplesIntra,

- a variable cIdx specifying the colour component index.

Output of this process is the (cbWidth)x(cbHeight) array predSamplesComb of prediction sample values.

The variable scallFact is derived as follows:

scallFactX = ( cIdx  = =  0  | |  SubWidthC  = =  1 ) ? 0 : 1					(1026)

scallFactY = ( cIdx  = =  0  | |  SubHeightC  = =  1 ) ? 0 : 1					(1027)

The neighbouring luma locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to 
( xCb − 1, yCb − 1 + ( cbHeight  <<  scallFactY ) ) and ( xCb − 1 + (cbWidth  <<  scallFactX ), yCb − 1 ), respectively.

For X being replaced by either A or B, the variables availableX and isIntraCodedNeighbourX are derived as follows:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availableX.

- The variable isIntraCodedNeighbourX is derived as follows:

- If availableX is equal to TRUE and CuPredMode[ 0 ][ xNbX ][ yNbX ] is equal to MODE_INTRA, isIntraCodedNeighbourX is set equal to TRUE.

- Otherwise, isIntraCodedNeighbourX is set equal to FALSE.

The weight w is derived as follows:

- If isIntraCodedNeighbourA and isIntraCodedNeighbourB are both equal to TRUE, w is set equal to 3.

- Otherwise, if isIntraCodedNeighbourA and isIntraCodedNeighbourB are both equal to to FALSE, w is set equal to 1.

- Otherwise, w is set equal to 2.

When cIdx is equal to 0 and sh_lmcs_used_flag is equal to 1, predSamplesInter[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are modified as follows:

i = ph_lmcs_aps_id
idxY = predSamplesInter[ x ][ y ]  >>  Log2( OrgCW )
predSamplesInter [ x ][ y ] = Clip1( LmcsPivot[ i ][ idxY ] +	(1028)
	( ScaleCoeff[ i ][ idxY ] * ( predSamplesInter[ x ][ y ] − InputPivot[ i ][ idxY ] ) +
	( 1  <<  10 ) )  >>  11 )

The prediction samples predSamplesComb[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are derived as follows:

predSamplesComb[ x ][ y ] = ( w * predSamplesIntra[ x ][ y ] +	(1029)
	( 4 − w ) * predSamplesInter[ x ][ y ] + 2)  >>  2

#### 8.7.5.2	Picture reconstruction with mapping process for luma samples

Inputs to this process are:

- a location ( xCurr, yCurr ) of the top-left sample of the current block relative to the top-left sample of the current picture,

- a variable nCurrSw specifying the block width,

- a variable nCurrSh specifying the block height,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the luma predicted samples of the current block,

- an (nCurrSw)x(nCurrSh) array resSamples specifying the luma residual samples of the current block.

Outputs of this process is a reconstructed luma picture sample array recSamples.

The (nCurrSw)x(nCurrSh) array of mapped predicted luma samples predMapSamples is derived as follows:

- If one of the following conditions is true, predMapSamples[ i ][ j ] is set equal to predSamples[ i ][ j ] for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

- CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_INTRA.

- CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_IBC.

- CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_PLT.

- CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_INTER and ciip_flag[ xCurr ][ yCurr ] is equal to 1.

- Otherwise (CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_INTER and ciip_flag[ xCurr ][ yCurr ] is equal to 0), the following applies:

idxY =  predSamples[ i ][ j ]  >>  Log2( OrgCW )
predMapSamples[ i ][ j ] = LmcsPivot[ ph_lmcs_aps_id ][ idxY ] +
		( ScaleCoeff[ ph_lmcs_aps_id ][ idxY ] * ( predSamples[ i ][ j ] − InputPivot[ ph_lmcs_aps_id ][ idxY ] ) + ( 1  <<  10 ) )  >>  11	(1232)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh − 1

The reconstructed luma picture sample recSamples is derived as follows:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1( predMapSamples[ i ][ j ]+ resSamples[ i ][ j ] ] )	(1233)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh − 1

#### 8.7.5.3	Picture reconstruction with luma dependent chroma residual scaling process for chroma samples

Inputs to this process are:

- a chroma location ( xCurr, yCurr ) of the top-left chroma sample of the current chroma transform block relative to the top-left chroma sample of the current picture,

- a variable nCurrSw specifying the chroma transform block width,

- a variable nCurrSh specifying the chroma transform block height,

- a variable tuCbfChroma specifying the coded block flag of the current chroma transform block,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the chroma prediction samples of the current block,

- an (nCurrSw)x(nCurrSh) array resSamples specifying the chroma residual samples of the current block,

Output of this process is a reconstructed chroma picture sample array recSamples.

The variable sizeY is set equal to Min( CtbSizeY, 64 ).

The reconstructed chroma picture sample recSamples is derived as follows for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

- If one or more of the following conditions are true, recSamples[ xCurr + i ][ yCurr + j ] is set equal to Clip1( predSamples[ i ][ j ] + resSamples[ i ][ j ] ):

- ph_chroma_residual_scale_flag is equal to 0.

- sh_lmcs_used_flag is equal to 0.

- nCurrSw * nCurrSh is less than or euqal to 4.

- tu_cb_coded_flag [ xCurr ][ yCurr ] is equal to 0 and tu_cr_coded_flag [ xCurr ][ yCurr ] is equal to 0.

- Otherwise, the following applies:

- The current luma location ( xCurrY, yCurrY ) is derived as follows:

( xCurrY, yCurrY ) = ( xCurr * SubWidthC, yCurr * SubHeightC )	(1234)

- The luma location ( xCuCb, yCuCb ) is specified as the top-left luma sample location of the coding unit that contains the luma sample at ( xCurrY / sizeY * sizeY, yCurrY / sizeY * sizeY ).

- The variables availL and availT are derived as follows:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( xCuCb, yCuCb ), the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCuCb − 1, yCuCb ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availL.

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the location ( xCurr, yCurr ) set equal to ( xCuCb, yCuCb ), the neighbouring luma location ( xNbY, yNbY ) set equal to ( xCuCb, yCuCb − 1 ), checkPredModeY set equal to FALSE, and cIdx set equal to 0 as inputs, and the output is assigned to availT.

- The variable currPic specifies the array of reconstructed luma samples in the current picture.

- For the derivation of the variable varScale the following ordered steps apply:

- The variable invAvgLuma is derived as follows:

- The array recLuma[ i ] with i=0..( 2 * sizeY − 1 ) and the variable cnt are derived as follows:

- The variable cnt is set equal to 0.

- When availL is equal to TRUE, the array recLuma[ i ] with i = 0..sizeY − 1 is set equal to currPic[ xCuCb − 1 ][ Min( yCuCb + i, pps_pic_height_in_luma_samples − 1 ) ] with i = 0..sizeY − 1, and cnt is set equal to sizeY.

- When availT is equal to TRUE, the array recLuma[ cnt + i ] with i = 0..sizeY − 1 is set equal to currPic[ Min( xCuCb + i, pps_pic_width_in_luma_samples − 1 ) ][ yCuCb − 1 ] with i = 0..sizeY − 1, and cnt is set equal to ( cnt + sizeY ).

- The variable invAvgLuma is derived as follows:

- If cnt is greater than 0, the following applies:

invAvgLuma = ( k=0cnt-1recLuma k  + ( cnt  >>  1 ) )  >>  Log2( cnt )	(1235)

- Otherwise (cnt is equal to 0), the following applies:

invAvgLuma = 1  <<  ( BitDepth − 1 )	(1236)

- The variable idxYInv is derived by invoking the identification of piece-wise function index process for a luma sample as specified in clause 8.8.2.3 with the variable lumaSample set equal to invAvgLuma as the input and idxYInv as the output.

- The variable varScale is derived as follows:

varScale = ChromaScaleCoeff[ ph_lmcs_aps_id ][ idxYInv ]	(1237)

- The reconstructed chroma picture sample array recSamples is derived as follows:

- If tuCbfChroma is equal to 1, the following applies:

resSamples[ i ][ j ] = Clip3( −( 1 << BitDepth ), ( 1 << BitDepth ) − 1, resSamples[ i ][ j ] )	(1238)

recSamples[ xCurr + i ][ yCurr + j ] = Clip1( predSamples[ i ][ j ] + 	(1239)
	Sign( resSamples[ i ][ j ] ) * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << 10 ) ) >> 11 ) )

- Otherwise (tuCbfChroma is equal to 0), the following applies:

recSamples[ xCurr + i ][ yCurr + j ] = predSamples[ i ][ j ]	(1240)

#### 8.8.2.2	Inverse mapping process for a luma sample

Input to this process is a luma sample lumaSample.

Output of this process is a modified luma sample invLumaSample .

The value of invLumaSample is derived as follows:

- If sh_lmcs_used_flag of the slice that contains the luma sample lumaSample is equal to 1, the following ordered steps apply:

- The variable i specifying the aps_adaptation_parameter_set_id of the LMCS APS is set equal to ph_lmcs_aps_id.

- The variable idxYInv is derived by invoking the identification of piece-wise function index process for a luma sample as specified in clause 8.8.2.3 with lumaSample as the input and idxYInv as the output.

- The variable invSample is derived as follows:

invSample = InputPivot[ i ][ idxYInv ] + ( InvScaleCoeff[ i ][ idxYInv ] * 	(1241)
	( lumaSample − LmcsPivot[ i ][ idxYInv ] ) + ( 1  <<  10 ) )  >>  11

- The inverse mapped luma sample invLumaSample is derived as follows:

invLumaSample = Clip1( invSample )	(1242)

- Otherwise, invLumaSample is set equal to lumaSample.

#### 8.8.2.3	Identification of piecewise function index process for a luma sample

Input to this process is a luma sample lumaSample.

Output of this process is an index idxYInv identifing the piece to which the luma sample lumaSample belongs.

The variable idxYInv is derived as follows:

i = ph_lmcs_aps_id
for( idxYInv = LmcsMinBinIdx[ i ]; idxYInv  <=  LmcsMaxBinIdx[ i ]; idxYInv++ ) {
	if( lumaSample < LmcsPivot[ i ][ idxYInv + 1 ] )	(1243)
		break
}
idxYInv = Min( idxYInv, 15 )

# Specification Text – Proposal 2

#### 7.4.3.21		Scaling list data semantics

scaling_list_copy_mode_flag[ id ] equal to 1 specifies that the values of the scaling list are the same as the values of a reference scaling list. The reference scaling list is specified by scaling_list_pred_id_delta[ id ]. scaling_list_copy_mode_flag[ id ] equal to 0 specifies that scaling_list_pred_mode_flag is present. When not present, the value of scaling_list_copy_mode_flag[ id ] is inferred to be equal to 1.

scaling_list_pred_mode_flag[ id ] equal to 1 specifies that the values of the scaling list can be predicted from a reference scaling list. The reference scaling list is specified by scaling_list_pred_id_delta[ id ]. scaling_list_pred_mode_flag[ id ] equal to 0 specifies that the values of the scaling list are explicitly signalled. When not present, the value of scaling_list_pred_mode_flag[id] is inferred to be equal to 0.

scaling_list_pred_id_delta[ id ] specifies the reference scaling list used to derive the predicted scaling matrix ScalingMatrixPred[ id ]. When not present, the value of scaling_list_pred_id_delta[ id ] is inferred to be equal to 0. The value of scaling_list_pred_id_delta[ id ] shall be in the range of 0 to maxIdDelta with maxIdDelta derived depending on id as follows:

maxIdDelta = ( id < 2 ) ? id : ( ( id < 8 ) ? ( id − 2 ) : ( id − 8 ) )	(106)

The variables refId and matrixSize are derived as follows:

refId = id − scaling_list_pred_id_delta[ id ]		(107)

matrixSize = ( id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 )	(108)

The (matrixSize)x(matrixSize) array ScalingMatrixPred[ x ][ y ] with x = 0..matrixSize − 1, y = 0..matrixSize − 1 and the variable ScalingMatrixDCPred are derived as follows:

- When both scaling_list_copy_mode_flag[ id ] and scaling_list_pred_mode_flag[ id ] are equal to 0, all elements of ScalingMatrixPred are set equal to 8, and the value of ScalingMatrixDCPred is set equal to 8.

- Otherwise, when scaling_list_pred_id_delta[ id ] is equal to 0, all elements of ScalingMatrixPred are set equal to 16, and ScalingMatrixDCPred is set equal to 16.

- Otherwise (either scaling_list_copy_mode_flag[ id ] or scaling_list_pred_mode_flag[ id ] is equal to 1 and scaling_list_pred_id_delta[ id ] is greater than 0), ScalingMatrixPred is set equal to ScalingMatrixRec[ aps_adaptation_parameter_set_id ][ refId ], and the following applies for ScalingMatrixDCPred:

- If refId is greater than 13, ScalingMatrixDCPred is set equal to ScalingMatrixDCRec[ aps_adaptation_parameter_set_id ][ refId − 14 ].

- Otherwise (refId is less than or equal to 13), ScalingMatrixDCPred is set equal to ScalingMatrixPred[ 0 ][ 0 ].

scaling_list_dc_coef[ id − 14 ] is used to derive the value of the variable ScalingMatrixDC[ id − 14 ] when id is greater than 13 as follows:

ScalingMatrixDCRec[ aps_adaptation_parameter_set_id ][ id − 14 ] 
		= ( ScalingMatrixDCPred + scaling_list_dc_coef[ id − 14 ] ) & 255	(109)

When not present, the value of scaling_list_dc_coef[ id − 14 ] is inferred to be equal to 0. The value of scaling_list_dc_coef[ id – 14 ] shall be in the range of −128 to 127, inclusive. The value of ScalingMatrixDCRec[ aps_adaptation_parameter_set_id ][ id − 14 ] shall be greater than 0.

scaling_list_delta_coef[ id ][ i ] specifies the difference between the current matrix coefficient ScalingList[ id ][ i ] and the previous matrix coefficient ScalingList[ id ][ i − 1 ], when scaling_list_copy_mode_flag[ id ] is equal to 0. The value of scaling_list_delta_coef[ id ][ i ] shall be in the range of −128 to 127, inclusive. When  scaling_list_copy_mode_flag[ id ] is equal to 1, all elements of ScalingList[ id ] are set equal to 0.

The (matrixSize)x(matrixSize) array ScalingMatrixRec[ aps_adaptation_parameter_set_id ][ id ] is derived as follows:

ScalingMatrixRec[ aps_adaptation_parameter_set_id ][ id ][ x ][ y ] 
		= ( ScalingMatrixPred[ x ][ y ] + ScalingList[ id ][ k ] ) & 255	(110)
	with k = 0..( matrixSize * matrixSize − 1 ),
		x = DiagScanOrder[ Log2( matrixSize ) ][ Log2( matrixSize ) ][ k ][ 0 ], and
		y = DiagScanOrder[ Log2( matrixSize ) ][ Log2( matrixSize ) ][ k ][ 1 ]

The value of ScalingMatrixRec[ aps_adaptation_parameter_set_id ][ id ][ x ][ y ] shall be greater than 0.

### 8.7.3	Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable predMode specifying the prediction mode of the coding unit,

- a variable cIdx specifying the colour component of the current block.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP and the variable QpActOffset are derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(1154)

QpActOffset = cu_act_enabled_flag[ xTbY ][ yTbY ] ? −5 : 0	(1155)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

qP = Qp′CbCr		(1156)

QpActOffset = cu_act_enabled_flag[ xTbY ][ yTbY ] ? 1 : 0	(1157)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(1158)

QpActOffset = cu_act_enabled_flag[ xTbY ][ yTbY ] ? 1 : 0	(1159)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(1160)

QpActOffset = cu_act_enabled_flag[ xTbY ][ yTbY ] ? 3 : 0	(1161)

The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

qP = Clip3( 0, 63 + QpBdOffset, qP + QpActOffset )	(1162)

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 ) ? 1 : 0	(1163)

bdShift = BitDepth + rectNonTsFlag +		(1164)
		( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + sh_dep_quant_used_flag

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

qP = Clip3( QpPrimeTsMin, 63 + QpBdOffset, qP + QpActOffset )	(1165)

rectNonTsFlag = 0		(1166)

bdShift = 10			(1167)

The variable bdOffset is derived as follows:

bdOffset = ( 1  <<  bdShift )  >>  1		(1168)

The list levelScale[ ][ ] is specified as levelScale[ j ][ k ] = { { 40, 45, 51, 57, 64, 72 }, { 57, 64, 72, 80, 90, 102 } } with j = 0..1, k = 0..5.

The (nTbW)x(nTbH) array dz is set equal to the (nTbW)x(nTbH) array TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- sh_explicit_scaling_list_used_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1.

- sps_scaling_matrix_for_lfnst_disabled_flag is equal to 1 and ApplyLfnstFlag is equal to 1.

- sps_scaling_matrix_for_alternative_colour_space_disabled_flag is equal to 1 and sps_scaling_matrix_designated_colour_space_flag is equal to cu_act_enabled_flag[ xTbY ][ yTbY ].

- Otherwise, the following applies:

- The variable id is derived based on predMode, cIdx, nTbW, and nTbH as specified in Table 38 and the variable log2MatrixSize is derived as follows:

log2MatrixSize = ( id < 2 ) ? 1 : ( id < 8 ) ? 2 : 3	(1169)

- The scaling factor m[ x ][ y ] is derived as follows:

m[ x ][ y ] = ScalingMatrixRec[ ph_scaling_list_aps_id ][ id ][ i ][ j ]
	with i = ( x << log2MatrixSize ) >> Log2( nTbW ),
		 j = ( y << log2MatrixSize ) >> Log2( nTbH )	(1170)

- If id is greater than 13 and both x and y are equal to 0, m[ 0 ][ 0 ] is further modified as follows:

m[ 0 ][ 0 ] = ScalingMatrixDCRec[ ph_scaling_list_aps_id ][ id − 14 ]	(1171)

NOTE – A quantization matrix element m[ x ][ y ] can be zeroed out when any of the following conditions is true

- x is greater than 31

- y is greater than 31

- The decoded tu is not coded by default transform mode (i.e. transform type is not equal to 0 ) and x is greater than 15

- The decoded tu is not coded by default transform mode (i.e. transform type is not equal to 0 ) and y is greater than 15

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If sh_dep_quant_used_flag is equal to 1 and transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(1172)

–	Otherwise (sh_dep_quant_used_flag is equal to 0 or transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << ( qP / 6 )	(1173)

- When BdpcmFlag[ xTbY ][ yYbY ][ cIdx ] is equal to 1, dz[ x ][ y ] is modified as follows:

–	If BdpcmDir[ xTbY ][ yYbY ][ cIdx ] is equal to 0 and x is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x − 1 ][ y ] + dz[ x ][ y ])	(1174)

–	Otherwise, if BdpcmDir[ xTbY ][ yTbY ][ cIdx ] is equal to 1 and y is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x ][ y − 1 ] + dz[ x ][ y ])	(1175)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = ( dz[ x ][ y ] * ls[ x ][ y ] +bdOffset )  >>  bdShift	(1176)

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(1177)

Table 38 – Specification of the scaling matrix identifier variable id according to predMode, cIdx, nTbW, and nTbH

| max( nTbW, nTbH ) |  | 2 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| predMode = MODE_INTRA | cIdx = 0 (Y) |  | 2 | 8 | 14 | 20 | 26 |
|  | cIdx = 1 (Cb) |  | 3 | 9 | 15 | 21 | 21 |
|  | cIdx = 2 (Cr) |  | 4 | 10 | 16 | 22 | 22 |
| predMode = MODE_INTER or MODE_IBC | cIdx = 0 (Y) |  | 5 | 11 | 17 | 23 | 27 |
|  | cIdx = 1 (Cb) | 0 | 6 | 12 | 18 | 24 | 24 |
|  | cIdx = 2 (Cr) | 1 | 7 | 13 | 19 | 25 | 25 |

# Patent rights declaration(s)

Digital Insights Inc. and Hyundai Motors may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).