| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0248-v1 |
| --- | --- |

| Title: | AHG9: HRD text cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang (Bytedance) Yago Sanchez, Robert Skupin (HHI) Sachin Deshpande (Sharp) Vadim Seregin (Qualcomm) Hendry (LGE) Jie Chen (Alibaba) | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc., Fraunhofer HHI, Sharp Labs of America, Inc., Qualcomm Incorporated, LG Electronics Inc., Alibaba Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes, along with some asserted editorial changes, mostly related to the HRD:

- In Equation 41, replace vps_ptl_max_tid[ 0 ] with vps_ptl_max_tid[ vps_ols_ptl_idx[ 0 ] ] and vps_ptl_max_tid[ i ] with vps_ptl_max_tid[ vps_ols_ptl_idx[ i ] ] (for all the instances).

- In clause 8.1.1, add the derivation of the variables DuHrdPreferredFlag and DecodingUnitHrdFlag, simialrly as in HEVC.

- In clauses C.2.2 and C.2.3, change the local variable decodingUnitParamsFlag to be a global variable DecodingUnitParamsFlag, to address the issue that decodingUnitParamsFlag was used in clause C.2.2 without being defied/initialized.

- In clause C.3.2, change the condition on when to remove decoded pictures neither needed for referecne nor needed for output such that it is applied to all pictures except for the first picture of AU 0.

- In clause C.5.2.2, change "Otherwise (the current AU is not a CVSS AU or the current AU is a CVSS AU that is not AU 0 but the current picture is not the first picture of the current AU)" to "Otherwise" as the condition in the parentheses is asserted incorrect.

- In clause C.5.2.3, remove "AU n containing " from the first sentence, i.e., "The processes specified in this clause happen instantaneously when the last DU of AU n containing the current picture is removed from the CPB."

- In clause D.3.2, add the following constraint to disallow the combination of alternative timing for the CRA/DRAP case and the decoding unit HRD mode: When bp_alt_cpb_params_present_flag is equal to 1, the value of bp_du_hrd_params_present_flag shall be equal to 0.

The proposed spec text changes, relative to JVET-S0152-v6, are in the attachment to this contribution.

# Patent rights declaration(s)

Bytedance Inc., Fraunhofer HHI, Sharp Labs of America, Inc., Qualcomm Incorporated, LG Electronics Inc., and Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).