| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0251 |
| --- | --- |

| Title: | Crosscheck of JVET-S0229 (Bug fix of BDOF for high bit-depth) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Adrian Browne Karl Sharman Steve Keating | Email: | adrian.browne@sony.com karl.sharman@sony.com steve.keating@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results for JVET-S0229v1[1].  It is found that the software implements the specification changes in JVET-S0229 but also contains a number of other fixes.  These fixes are compared to similar fixes contained in JVET-R0351.  Results for JVET-S0229, JVET-R0351 and JVET-R0351 with the specification changes for JVET-S0229 are given.  These results show that the gain attributed to JVET-S0229 are due to the fixes common with JVET-R0351 and not the specification changes.

# Introduction

JVET-S0229 contends that there is a bug within BDOF which exhibits losses at higher bit depths when compared with 10-bit CTC results.  The modification to BDOF they propose involves a specification change which increases the magnitude of two shifts for bit depths above 12.  The change is implemented on top of a version of VTM 9.0 with fixes for compiler errors and decoding issues when RExt__HIGH_BIT_DEPTH_SUPPORT is set to 1. These fixes have been done independently of those previously reported in JVET-R0351[2] which have been included in the main branch of VTM since just after the release of VTM 9.1 (Merge request 1678).

JVET-S0229 uses 10-bit CTC results as an anchor, and reports results for 14-bit before and after the proposed BDOF bug fix as follows:

|  | Class A1 | Class A2 | Class B | Class C | Overall | Class D | Class F |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Without BDOF Change | 2.25% | 3.89% | 2.67% | 2.69% | 2.84% | 3.76% | 0.85% |
| With BDOF Change | -0.87% | -0.74% | -0.42% | -0.40% | -0.57% | -0.19% | -0.62% |

Results from JVET-S0229 (14-bit). Anchor VTM9.0 (10-bit)

Only RA results for CTC conditions are provided in the spreadsheets associated with the submission.

The submission does include the update to the draft specification and the proponent supplied source code which implements their proposed change.

# Analysis of Specification / Code Changes

The code supplied by the proponents was compared with the specification change.  All the specification changes were identified in the code, and appear to have been correctly applied.  However, the code changes also includes a number of other fixes which were are equivalent to some of the changes for JVET-R0351.

The common changes are of two types.  Firstly, one of the shifts needs to be aligned with the specification (shift3 in the specification of BDOF, shiftNum in InterPrediction::applyBiOptFlow), and secondly a number of type conversions need correcting to use Pel rather a fixed type (int16_t).

In order to correctly determine if the proposed specification change was the source of the reported gain, VTM9.0+JVET-R0351 was patched with code to implement the proposed specification change.

# Analysis of Results

The following table contains results for VTM9.0 patched with code from JVET-R0351 and JVET-S0229.  For timeliness, results for classes A1 and A2 were not generated.  All results are for 14-bit but compared with VTM9.0 at 10-bit. To ensure that results with extended precision enabled were consistent with results with extended precision disabled these were also generated and found to be similar.

|  | Class B | Class C | Class D | Class F |
| --- | --- | --- | --- | --- |
| S0229 | -0.42%/ 1.18%/0.78% | -0.40%/ 2.35%/2.29% | -0.19%/ 1.82%/1.59% | -0.62%/ 0.66%/0.57% |
| R0351 | -0.41%/ 1.25%/0.82% | -0.40%/ 2.42%/2.41% | -0.26%/ 1.88%/1.57% | -0.57%/ 0.65%/0.73% |
| R0351 + S0229 | -0.42%/ 1.26%/0.86% | -0.42%/ 2.47%/2.34% | -0.28%/ 1.89%/1.65% | -0.59%/ 0.63%/0.70% |
| R0351 extended precision | -0.38%/ 1.16%/0.69% | -0.25%/ 2.75%/2.61% | -0.14%/ 2.25%/2.12% | -0.47%/ 0.79%/1.01% |
| R0351 + S0229 extended precision | -0.37%/ 1.14%/0.66% | -0.26%/ 2.74%/2.64% | -0.14%/ 2.26%/2.23% | -0.47%/ 0.82%/1.04% |

Comparison of results for JVET-R0351 and JVET-S0229 (14-bit) against VTM9.0 (10-bit)

It is notable that all these results give a luma gain with a chroma loss.  The reason for this is not known, and is not mentioned in JVET-S0229.

When the 14-bit (without extended precision) results for JVET-R0351 and JVET-R0351+JVET-S0229 are compared, the actual effect of the specification change can be determined.  The results of this comparison are shown in the following table:

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y | U | V | YUV |
| Class B | -0.01% | 0.01% | 0.04% | -0.01% |
| Class C | -0.02% | 0.06% | -0.07% | -0.02% |
| Class D | -0.02% | 0.01% | 0.08% | -0.02% |
| Class F | -0.02% | -0.03% | -0.03% | -0.02% |

Result of JVET-R0351+JVET-S0229 over JVET-R0351

These results show that the specification change has very little effect on the results, and thus the gain seen in the submission for JVET-S0229 must predominantly be due to the other software fixes which are common with JVET-R0351.

Although the gain achieved here may not be significant, it is important to note that the sequences used are all 8-bit or 10-bit, and the QP range is chosen to be suitable for 10-bit processing.  More indicative results of the effect of the specification change could be produced by using higher bit depth content at lower (perhaps negative) QPs.

# Conclusions

This crosscheck has shown that the software implements the specification changes in JVET-S0229 but that the gains reported are predominantly due to other fixes which have already been fixed in the main branch of VTM.

Due to the source material and operating point chosen, it is difficult to assess if the specification change will have a more significant effect with higher bit depth content and at lower QPs.

The luma gain and associated chroma loss for 14-bit processing shown in the results given here is at present unexplained and further investigation maybe be of interest.

# References

[1] “Bug fix of BDOF for high bit-depth”, JVET-S0229, T. Chujoh, E. Sasaki, T. Ikai

[2] “High bit depth coding”, JVET-R0351, A. Browne, S. Keating and K. Sharman

[3] “Versatile Video Coding (Draft 9)”, JVET-R2001-vA, B. Bross, J. Chen, S. Liu and Y-K. Wang