| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0253 |
| --- | --- |

| Title: | Update on Verification Test Preparation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini Andrew Segall Yan Ye | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com asegall@sharplabs.com yan.ye@alibaba-inc.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

The status of the verification test preparation is summarized. In the SDR category, a close-to-final definition of test sequences and selection of rate points for VTM and HM in the UHD Random Access test case are presented. For the HD Low Delay test case, the identification of test sequences is in progress. For the HDR category, a candidate set of five QP and five HLG sequences with corresponding QP settings are proposed for visual inspection. A tone mapping procedure has also been determined to enable the viewing of the PQ content on available UHD displays. In the 360° video category, viewpaths have been designed. The applicable projection formats for the sequences have to be determined in order to define VTM and HM QPs for the rate points to be assessed visually.

# Introduction

This paper summarizes the status of the VVC verification test preparation. For each category, a brief overview is provided in the subsequence sections. Dedicated AHG meeting were held on 2020-05-15 for SDR [JVET-S0041, JVET-S0043], on 2020-05-27 for 360° video [JVET-S0146, JVET-S0149], and on 2020-05-29 for HDR [JVET-S0151, JVET-S0153].

# Standard Dynamic Range

## UHD Random Access Configuration

A preliminary set of four test sequences has been established. The sequences were assessed in a dry-run subjective assessment experiment with naïve viewers described in [JVET-S0246]. The sequences were selected based on the results of an initial remote experts viewing session

Table 1 – Selected SDR UHD test sequences and rate points

| No. | Test sequence | HM QPs | VTM QPs |
| --- | --- | --- | --- |
| 01 | Marathon_3840x2160_30fps_8bit_420pf | 28 32 35 39 43 | 30 34 38 42 46 |
| 02 | MountainBay2_3840x2160_30fps_420_10bit | 28 30 33 36 39 | 30 33 36 39 42 |
| 03 | Netflix_DrivingPOV_4096x2160_60fps_10bit_420 | 27 31 35 39 43 | 30 34 38 42 46 |
| 09 | NeptuneFountain3_3840x2160_60fps_420_10bit | 28 39 33 36 39 | 30 33 36 39 42 |

The test showed that the selected rate points span a wide quality range as intended. The QP selection for some rate points may be adjusted. Due to feedback of the naïve viewers, one sequence is suggested to be replaced.

## HD Low Delay Configuration

The identification of test sequences is in progress. Trial encoding and the selection of VTM and HM QPs are to be started after this.

# High Dynamic Range

Approximately 44 HDR test sequences were identified for initial evaluation.  Following initial study of these sequences, ten test sequences were identified as potential candidates for further visual inspection. These candidates included five PQ sequences and five HLG sequences.  Calibration and viewing procedures were established for these sequences, which included a study of how to perform appropriate tone mapping for the candidate sequences when using consumer monitors.  The inspection and rate point selection have not been completed due to very limited access to appropriate viewing equipment under the COVID-19 restrictions.

- Some possible candidates for PQ include:

  - Chimera3

    - HM QPs (24, 28, 32, 36 ,40)

    - VTM QPs (26, 30, 34, 38, 42)

  - Chimera5

    - HM QPs (23-24, 27, 31, 35, 39)

    - VTM QPs (26, 30 ,34, 38 ,42)

  - Chimera7

    - HM QPs (28, 32, 36, 40 44)

    - VTM QPs (30, 34, 38, 42, 46)

  - Meridian2

    - HM QPs (23, 28, 31, 35, 39)

    - VTM QPs (26, 30, 34, 38, 42)

  - Meridian5

    - HM QPs (24, 28, 32, 36, 40)

    - VTM QPs (26, 30, 34, 38, 42)

- Some possible candidates for HLG include

  - H3-AMS1

  - H3-AMS3

  - H3-AMS9

  - HLG3

  - HLG4

  - For these sequences, starting point QPs were suggested to be

    - HM QPs (24, 30, 34, 40, 44)

    - VTM QPs (27, 32, 37, 42, 46)

### Suggested procedure for HDR remote viewing

To facilitate remove viewing, a number of procedures were developed to enable remote viewing of the sequences.  The end result is that the group now has a process for performing testing of both HLG and PQ sequences using both consumer and professional UHD monitors with 1,000-nit of peak luminance.  It is noted that the Call for Proposal and Call for Evidence tests required the use of a professional HD monitor with 4,000-nit of peak luminance, which is asserted to not be representative of what is available in the market.

The rest of this sub-section summarizes different aspects of the remote viewing procedure:

Tone Mapping

The PQ content available to JVET is mainly available with UHD resolution and mastered using a 4,000 nit monitor.  To evaluate the content on a UHD monitor with a peak luminance of 1,000 nits, it is necessary to tone map the content.  During the interim period, two methods for tone mapping were studied – the first was to compress the dynamic range linearly to fit within the 1,000 nit container; the second was to allow content above the 1,000 nit capability of the display to saturate.  It was identified that using the second method was preferable.  It is noted that additional refinements to this approach may be considered, though they are not expected to significantly change the test procedure.

Compressed Playback Files

The playback of HDR content can be challenging for some viewers, especially those using consumer monitor, as it is necessary for the monitor to detect the presence of HDR content.  Furthermore, due to the limitations of COVID-19, it is noted that some participants do not have access to laboratory equipment that can be used to control an HDR monitor and/or play uncompressed sequences back in real-time.

To address these issues, a procedure to create compressed versions of the test sequences was formalized.  This allowed for playback on a Windows PC platform as well as from the USB stick (and internal decoder) of a consumer monitor.  The commands are:

PQ content: 
ffmpeg -pix_fmt yuv420p10le -r 60 -s 3840x2160 -i <filename.yuv> -c:v libx265 -crf <crf_value> -x265-params colorprim=bt2020:transfer=smpte2084:colormatrix=bt2020nc <filename.mkv>

HLG content: 
ffmpeg -pix_fmt yuv420p10le -r 60 -s 3840x2160 -i <filename.yuv> -c:v libx265 -crf <crf_value> -x265-params colorprim=bt2020:transfer=arib-std-b67:colormatrix=bt2020nc <filename.mkv>

PC Platform

During the interim period, it was recommended to use a a consumer LG OLED display when possible to evaluate the HDR content.  To drive the LG display in HDR mode, a set of procedures was developed to facilitate reproduceable results.  They were:

- Enable the 4:2:2 12-bit mode in the Nvidia driver

- Enable HDR support in Windows 10  (For example, using the steps at: https://support.microsoft.com/en-us/help/4040263/windows-10-hdr-advanced-color-settings)

- Download a recent version of Video LAN (3.x)

- Select the “HDR Standard” mode (or similar) on the LG display and disable automatic tone mapping

After performing Step 1 and 2, the monitor should provide visual feedback that it is operating in HDR mode.

Internal Decoder Playback

For the internal decoder, it was recommended to copy the MKVs (generated by using the commands above) onto a USB stick and plug into the monitor.  Using the GUI, the user should then navigate to the USB stick and select the file to play.  When playback begins, it was expressed that a viewer should receive visual feedback that the playback is in HDR.  Playback should be paused and the “HDR Standard” mode selected and automatic tone mapping disabled.

In preparing sequences for the internal playback, the resulting bit-rate was usually not visually transparent.  However, it was noted that one benefit of using the internal playback is that it allows a user to check if the choices of modes match the choices you saw in the PC playback.  It not, then it can be concluded that the PC playback is not configured correctly.

Calibration

As noted above, one method for confirming that a user is operating in HDR mode is to compare the output of the internal decoder and your playback system, especially in terms of configuration options (such described above).

In addition, two versions of the Market3 sequences were created and made available.  These sequences allow for further calibration and confirmation for remote viewing.  The first sequence is the original sequence in the “Market3_Original” directory.  This is a very bright sequence with significant content above 1,000 nits.  When played back, the viewer should expect to see a sequence with a visible saturation and/or clipping.  If the image instead appears dark, then the remote monitor is likely still in an SDR mode.

Assuming that the first version of the file results in a saturated image, a second version of the sequence was provided.  This sequence is remapped to 1,000 nits.  And, when played back, the result should be a colorful image – and one similar to what JVET has been looking at for many years now.

### Remote Viewing Status

As a result of the procedure above, two different locations were able to reproduce results using similar monitors (both were modern LG OLED displays).  Additionally, the calibration sequences were reported to be viewed successfully on a third display system.

# 360° Video

For 360° video, a set of nine test sequences was evaluated as reported in JVET-S0146. Bitstreams for these sequences have been made available using the PERP, CMP, and PCMP projection formats for HM and the PERP, HEC, and GCMP projection formats for VTM. For seven of these sequences, view paths were developed for extraction of 1080p viewports, these sequences are listed below.

Table 1 – 360 video test sequences

| No. | Sequence | Frame rate | Resolution | Provider | BD | Camera |
| --- | --- | --- | --- | --- | --- | --- |
| 02 | GT_Sheriff | 30 | 4320x2160 | Nokia | 8 | static |
| 03 | basketball | 30 | 8192x4096 | GoPro | 10 | static |
| 04 | jam_session | 30 | 8192x4096 | GoPro | 8 | static |
| 05 | SkateboardTrick | 60 | 8192x4096 | GoPro | 8 | static |
| 07 | SkateBoardAtBridge | 30 | 6144x3072 | GoPro | 8 | moving |
| 08 | HarborBiking | 30 | 8192x4096 | InterDigital | 8 | moving |
| 09 | KiteFliteWalking | 30 | 8192x4096 | InterDigital | 8 | moving |

Visible rendering issues were observed in the sequences HarborBiking and KiteFliteWalking. These include chroma misalignment in the first frame and filtering artifacts spread over larger areas of the stitched 360° scene.

The QP selection is in progress, using the viewports proposed in JVET-S0146. Since multiple projection formats are available, the assignment of projection formats and sequences for the verification tests has to be decided as well.

In the AHG meeting of 2020-05-27, further aspects were raised [JVET-S0149]. The observed performance of GCMP and PCMP was called into question and was further investigated after the AHG meeting. The behavior of GCMP/PCMP was verified to be normal.

For the CMP-related formats, the usage of subpictures in 360 coding was suggested. This action item is pending. If software and config become available in time, this could be included in the next encoding iteration.

It was recommended to incorporate information on the projection format of the bitstreams in corresponding SEI messages. This action item is pending. If software and config become available in time, this could be included in the next encoding iteration.

# Conclusions

It is recommended to work towards completion of the open tasks during the meeting.