| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0255 |
| --- | --- |

| Title: | AHG9: Semantics fix to ALF and SAO enabling flags in PH |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz Olena Chubach, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei | Email: | nanh@qti.qualcomm.com {olena.chubach, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Qualcomm Inc. and MediaTek Inc. |  |  |  |

_____________________________

# Abstract

In JVET-S0121, it is reported that there is inconsistency in the semantics of ALF and SAO enabling flags in PH and SH. This contribution presents a semantic change such that those PH flags are only meaningful when ph_sao/alf_info_in_ph_flag is equal to 1.

# Introduction

In JVET-S0121, inconsistency in the semantics of ALF and SAO enabling flags is reported. When an ALF or SAO enabling flag is not present in PH, it is inferred to be equal to 0. According to the semantics, the related tool is disabled and not used for a current picture. However, in a slice header, the corresponding enabling flag can be present and may be equal to 1. Then, the tool is enabled and may be used for a current slice, which contradicts disabling a tool in PH.

In VVC draft 9, an editor suggests to apply condition to the meaning of ph_deblocking_filter_disable_flag, such that it is only meaningful when pps_dbf_info_in_ph_flag is equal to 1.

This contribution presents the semantics change of ALF and SAO enabling flags in PH such that they are only meaningful when pps_alf/sao_info_in_ph_flag is equal to 1.

# Text change

When pps_alf_info_ph_flag is equal to 1, ph_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled and may be used for the current picture. ph_alf_enabled_flag equal to 0 specifies that adaptive loop filter is disabled and not used for the current picture. When not present, ph_alf_enabled_flag is inferred to be equal to 0.

When ph_alf_enabled_flag is not present, it is inferred to be equal to 0.

When pps_alf_info_ph_flag is equal to 1, ph_alf_cb_flag equal to 0 specifies that the adaptive loop filter is disabled and not applied to the Cb colour component for the current picture. ph_alf_cb_flag equal to 1 specifies that the adaptive loop filter is enabled and may be applied to the Cb colour component for the current picture. When ph_alf_cb_flag is not present, it is inferred to be equal to 0.

When ph_alf_cb_flag is not present, it is inferred to be equal to 0.

When pps_alf_info_ph_flag is equal to 1, ph_alf_cr_flag equal to 0 specifies that the adaptive loop filter is disabled and not applied to the Cr colour component for the current picture. ph_alf_cr_flag equal to 1 specifies that the adaptive loop filter is enabled and may be applied to the Cr colour component for the current picture. When ph_alf_cr_flag is not present, it is inferred to be equal to 0.

When ph_alf_cr_flag is not present, it is inferred to be equal to 0.

When pps_alf_info_ph_flag is equal to 1, ph_cc_alf_cb_enabled_flag equal to 1 specifies that cross-component adaptive loop filter for the Cb colour component is enabled and may be used for the current picture. ph_cc_alf_cb_enabled_flag equal to 0 specifies that cross-component adaptive loop filter for the Cb colour component is disabled and not used for the current picture. When not present, ph_cc_alf_cb_enabled_flag is inferred to be equal to 0.

When ph_cc_alf_cb_enabled_flag is not present, it is inferred to be equal to 0.

When pps_alf_info_ph_flag is equal to 1, ph_cc_alf_cr_enabled_flag equal to 1 specifies that cross-compoent adaptive loop filter for the Cr colour component is enabled and may be used for the current picture. ph_cc_alf_cr_enabled_flag equal to 0 specifies that cross-component adaptive loop filter for the Cr colour component is disabled and not used for the current picture. When not present, ph_cc_alf_cr_enabled_flag is inferred to be equal to 0.

When ph_cc_alf_cr_enabled_flag is not present, it is inferred to be equal to 0.

When pps_sao_info_ph_flag is equal to 1, ph_sao_luma_enabled_flag equal to 1 specifies that SAO is enabled and may be used for the luma component of the current picture. ph_sao_luma_enabled_flag equal to 0 specifies that SAO is disabled and not used for the luma component of the current picture. When ph_sao_luma_enabled_flag is not present, it is inferred to be equal to 0.

When ph_sao_luma_enabled_flag is not present, it is inferred to be equal to 0.

When pps_sao_info_ph_flag is equal to 1, ph_sao_chroma_enabled_flag equal to 1 specifies that SAO is enabled and may be used for the chroma component of the current picture. ph_sao_chroma_enabled_flag equal to 0 specifies that SAO is disabled and not used for the chroma component of the current picture. When ph_sao_chroma_enabled_flag is not present, it is inferred to be equal to 0.

When ph_sao_chroma_enabled_flag is not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).