| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0256-v1 |
| --- | --- |

| Title: | AHG2: An integration of virtual boundaries adoptions of S0171, S0211 and S0221 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson | Email: | martin.m.pettersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution provides a text integration of the virtual boundaries contributions JVET-S0171, JVET-S0211 and JVET-S0221 recommended from the HLS AHG pre-meeting June 19-21 and adopted at the JVET opening plenary.

The text is provided on top of JVET-S0152-v6 in the attachment.

# Incorporated adoptions and editor action items

The list of incorporated adoptions and editor action items:

- JVET-S0171: Editor action item: The editor is requested to consider the proposed editorial changes for disabling SAO filtering operations across virtual boundaries by moving the specification text to the condition of edge offset mode.

- JVET-S0211: Signal the number of virtual boundaries using ue(v) instead of u(2) for the four syntax elements sps_num_ver_virtual_boundaries, sps_num_hor_virtual_boundaries, ph_num_ver_virtual_boundaries and ph_num_hor_virtual_boundaries while retaining the current value range.

- JVET-S0221 approach 2: Add bitstream conformance constraints as follows (or editorial equivalent):

  - It is a requirement of bitstream conformance that when the value of sps_pic_width_max_in_luma_samples is equal to or smaller than 8, the value of sps_num_ver_virtual_boundaries shall be equal to 0.

  - It is a requirement of bitstream conformance that when the value of sps_pic_height_max_in_luma_samples is equal to or smaller than 8, the value of sps_num_hor_virtual_boundaries shall be equal to 0.

  - It is a requirement of bitstream conformance that when the value of pps_pic_width_in_luma_samples is equal to or smaller than 8, the value of ph_num_ver_virtual_boundaries shall be equal to 0.

  - It is a requirement of bitstream conformance that when the value of pps_pic_height_in_luma_samples is equal to or smaller than 8, the value of ph_num_hor_virtual_boundaries shall be equal to 0.

It also seems desirable to make sure the range constraint never specifies a non-valid range even in disallowed cases.