| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0262-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-S0180 (Addition of a GOP hierarchy of 32 for random access configuration for VTM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Christian Helmrich | Email: | christian.helmrich@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results for contribution JVET-S0180 version 2 [1], which proposes the introduction of a random access (RA) configuration utilizing a group of picture (GOP) size of 32 in addition to a size of 16.  As described in S0180, no changes to the VTM reference software are proposed apart from the addition of a “encoder_randomaccess_vtm_gop32.cfg” file.  The crosschecker’s results on JVET’s SDR common test conditions [2] were found to match the proponents’ results as reported in [1].  Informal viewing for comparing GOP size 16 and 32 was conducted, and visible differences between the two were observed.

# Introduction

Contribution JVET-S0180 [1] proposes the addition of a GOP size of 32 in addition to the existing size of 16 for RA coding with VVC (VTM). A corresponding modification of JVET’s common test conditions [2] to use the larger GOP size was also suggested. Both objective (BD-rate gains) and subjective (visual quality) improvements were reported. An analogous modification for HEVC (HM 16.20) was documented as well.

# Objective Analysis of Proposal

For RA, a GOP size of 16 is currently utilized in [2], along with corresponding TLayer based QP offsets of

QPGOP16 = { –3, 1, 1, 4, 5, 6, 6 },

where value –3 is used only for Intra-only (CRA) pictures. Contribution [1] proposes a GOP size of 32 with

QPGOP32 = { –3, –1, 0, 0, 3, 5, 6, 6 }

as QP cascade, i. e., an unchanged range of QP offsets.  Due to the doubled GOP size, video sequences with frame rates around 50 or 20 fps must be reconfigured to use an Intra (CRA) period of 64 or 32, respectively.

A corresponding SDR CTC objective evaluation on VTM 9.0 [3], where both the reference and the proposed VTM RA conditions operated with integer multiple-of-32 Intra periods on all sequences, was conducted by the proponents of JVET-S0180 and by the crosschecker.  The results reportedly match in terms of BD-rate and, approximately, in encoder run-time ratios.  The decoder run-time ratios observed by the crosschecker are around 99% (proposal faster than reference) whereas the proponents reported a ratio around 95% in [1].

Comparing the proposed modified VTM 9 RA encodings with a GOP size of 32 to HEVC (HM 16.20) with a GOP size of 16, with the Intra periods synchronized to integer multiples-of-32 in both codecs, reportedly results in BD-rate gains of Y –38.05%, U –42.94%, V –44.00% of VTM over HM for the SDR CTC overall average.  For reference, it is noted that the VTM over HM overall average BD-rate results reported in the latest AHG3 report [4] are Y –35.93%, U –39.13%, V –40.09% for SDR CTC, i. e., roughly 2 to 3% lower.

Note: Due to a partial power failure on HHI’s computing cluster on June 23, 2020, the Class A results could not yet be obtained.  Therefore, the crosscheck currently has to be restricted to classes B–F.

Table 1. Crosschecker’s results for proposal JVET-S0180, SDR CTC, integer multiple-of-32 Intra periods.

| vs. VTM 9.0 | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 9.0 (Intra period 64 for 50Hz and 32 for 20Hz) |  |  |  |  |
|  | Y | U | V | EncTime | DecTime |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.31% | -7.22% | -7.59% | 98% | 98% |
| Class C | -3.37% | -5.95% | -6.13% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -3.27% | -5.94% | -6.21% | 101% | 101% |
| Class F | -6.45% | -7.37% | -7.24% | 97% | 100% |

Table 2.  Additional comparison of VTM+JVET-S0180 against HM, SDR CTC, multiple-of-32 Intra periods.

| vs. HM 16.20 | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over HM 16.20 (Intra period 64 for 50Hz and 32 for 20Hz) |  |  |  |  |
|  | Y | U | V | EncTime | DecTime |
| Class A1 | -40.07% | -40.19% | -47.50% | 843% | 166% |
| Class A2 | -44.99% | -43.40% | -41.92% | 924% | 182% |
| Class B | -37.13% | -50.91% | -48.95% | 851% | 161% |
| Class C | -32.46% | -34.69% | -36.76% | 1108% | 172% |
| Class E |  |  |  |  |  |
| Overall | -38.05% | -42.94% | -44.00% | 926% | 169% |
| Class D | -30.35% | -30.75% | -30.56% | 1248% | 176% |
| Class F | -45.75% | -49.29% | -50.34% | 612% | 145% |

# Subjective Analysis of Proposal

In addition to the objective verification of section 2, the crosschecker performed a brief informal subjective verification by comparing the visual quality of GOP size 32 versus GOP size 16 on low-rate (base QP 37) VTM 9.0 RA coded HD sequences, when both encodings employ the same Intra period.  On sequences with little or predominantly regular motion, the crosschecker confirms that no visual disadvantage occurs by the replacement of a GOP size of 16 by a GOP size of 32.  On the RedKayak sequence (30 fps, Intra period 32, streams attached) with irregular motion on the water areas in the last few seconds, the crosschecker is under the impression that the increased GOP size slightly reduces temporal pumping artifacts at the cost of slightly increased blurring of details in the waves.  A first colleague independently confirmed this observation, while a second colleague noticed a visual improvement and a third colleague did not notice an obvious difference.

# References

[1]	K. Andersson, J. Enhorn, R. Sjöberg, J. Ström, and L. Litwic, “Addition of a GOP hierarchy of 32 for 	random access configuration for VTM,” JVET-S0180, version 2, June 22, 2020.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, version 1, May 8, 2019.

[3]	JVET and Fraunhofer HHI, “jvet / VVCSoftware_VTM,” Git commit 5d7f8127, May 26, 2020, online: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/releases/VTM-9.0.

[4]	F. Bossen, X. Li, and K. Sühring, “JVET AHG report: Test model software development (AHG3),” JVET-S0003, version 2, June 22, 2020.