| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0267-v3 |
| --- | --- |

| Title: | AhG on neural network based coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shan Liu, Xiang Li, Wei Wang (Tencent) Elena Alshina (Huawei) Kei Kawamura, Kyohei Unno, Yoshitaka Kidani (KDDI) Ping Wu (ZTE) Andrew Segall (Sharp) Mathias Wien (RWTH Aachen) Jonathan Pfaff, Heiko Schwarz, Benjamin Bross (Fraunhofer HHI) Xianglin Wang, Xiaoyu Xiu, Yi-Wen Chen (Kwai) Zhenzhong Chen (Wuhan University) Jill Boyce (Intel) Yu-Wen Huang (MediaTek) Feng Wu, Dong Liu (USTC) M. Karczewicz, J. Chen (Qualcomm) Dan Grois (Comcast) | Tel: Email: | {shanl, xlxiangli, rickweiwang} @tencent.com elena.alshina@huawei.com ki-kawamura@kddi.com pwuzte@gmail.com asegall@sharplabs.com wien@lfb.rwth-aachen.de firstname.lastname@hhi.fraunhofer.de {xianglinwang, xiaoyuxiu, yiwenchen }@kwai.com zzchen@whu.edu.cn jill.boyce@intel.com yuwen.huang@mediatek.com {fengwu, dongeliu}@ustc.edu.cn {martak, cjianle}@qti.qualcomm.com grois@ieee.org |
| Source: | Tencent, Huawei, KDDI, ZTE, Sharp, RWTH Aachen, Fraunhofer HHI, Kwai, Wuhan University, Intel, MediaTek, USTC, Qualcomm, Comcast |  |  |

_____________________________

# Abstract

Neural Network (NN) based coding tools were studied in JVET AHG9: Neural Networks in Video Coding, during VVC v1 development. Coding tools in various directions were proposed and interesting coding performance improvement was demonstrated during the two-year study. It was decided to discontinue the AHG and focus on finalization of VVC v1 after the last meeting in 2019. As VVC v1 is soon completed, it is proposed to resume this AHG and its work on NN based video coding tool exploration and experiments.

# Introduction

NN based video coding tools has demonstrated some encouraging coding performance gain in recent years. Among the total 46 responses to the joint Call for Proposals (CfP) on video compression with capability beyond HEVC, 6 included learning-based tools with another 4 NN based coding tool proposals. They reported compression benefits against or on top of conventional methods [1]. An ad-hoc group (AHG9) was set up for investigating the compression performance and complexity of neural network-based coding tools in early 2018 [2], with initial mandates as follows.

- Investigate the benefit of deep learning technology in video compression.

- Investigate the complexity impact of using deep learning in video compression.

- Investigate deep learning based coding tools such as CNN loop filter.

- Investigate the relationship between CNN filter and ALF, and other loop filters.

- Investigate the performance of CNN filter used as an in-loop filter or a post-processing filter.

- Investigate the impact of QP on CNN filter.

After the establishment of the AHG, many technical contributions were proposed and a CE was established to conduct more thorough investigation, simulation, tool comparison and evaluation. Two output documents JVET-L1006 and JVET-M1006 were issued and used to provide thorough evaluation results as well as evaluation matrix and methodology. Then a Core Experiment was set up, with a dedicated branch for software sharing, cross check and reference. This was considered a good “dry run” for organized testing of NN based video coding tools.

This AHG work has been paused for a few months to allow experts to focus on VVC v1 finalization. However, the investigation did not stop. As VVC v1 is being finalized, it is recommended to resume this AHG and its work on NN based video coding tool exploration and experiments.

# Complexity analysis and reporting template

AHG9 reported complexity analysis of NN based tools and produced evaluation measurement and reporting template for this purpose. Please refer document JVET-M1006 for detailed report. This document summarizes the suggestions and describes a methodology for neural network coding tool testing. Some certain settings are specified. For other unspecified settings, a reporting template is provided for reporting this information of neural network coding tools.

### Training stage

### 2.1 specified settings

For the training stage of sequence-independent (offline) methods, high resolution images in DIV2K are used as the base image data set for offline training. It includes 800 images for training, and 100 images for validation. The original images should be converted to YUV format, and then using QP values in {22, 27, 32, 37} to generate the training image (YUV format). For other video training data set used for inter frame prediction, it should be reported clearly in the “optional” blank space in table 1. In principle, other image data set besides DIV2K is not suggested. If using other QP values to generate the training images, it should also be reported in the “optional” blank space in table 1.

For sequence-dependent methods, the training data may be the previous frames of the current sequence and the training QP may be the current QP. For clarification, this information should also be reported clearly in the “optional” blank in table 1.

### 2.2 reporting template

Currently, there are many settings corresponding to the network structure and implementation details that cannot be well-specified. Here a reporting template is provided for reporting this information. For other information that need to be reported but not listed in the table, it can be filled in the “optional” blank space as other settings.

Table 1. Reporting Template table for Unspecified Settings in Training Stage

| Mandatory | Information in Training Stage |  |  |
| --- | --- | --- | --- |
|  | Basic Settings: |  |  |
|  |  | learning rate: | (e.g. 5e-4) |
|  |  | optimizer: | (e.g. ADAM) |
|  |  | batch size: | (e.g. 16) |
|  |  | epoch: | (e.g. 300) |
|  |  | loss function: | (e.g. L1, L2, etc) |
|  |  | training GPU: | (e.g. GTX 1080ti) |
|  |  | training time: | (e.g. 48h) |
|  |  | framework: | (e.g. caffe, tf, pytorch, etc) |
| Optional | Other Settings: |  |  |
|  |  | (e.g. patch size:) | (e.g. 64x64, randomly cropped) |
|  |  | (e.g. additional dataset (besides DIV2K):) | (e.g. Video sequences, previous frames in last RAS) |
|  |  | (e.g. QP set for generating training dataset) | (e.g. 0-51, current QP) |
|  |  | (etc) |  |

### Inference stage

In inference stage, codec uses the model parameters for prediction. The complexity and memory usage need to be reported clearly for evaluation. The reported details shall include the following information:

- Total Conv. Layers: Total numbers of convolutional layers in the network structure. If there is no convolutional layer, just fill in 0.

- Total FC Layers: Total number of fully-connected layers in the network structure. If there is no fully-connected layer, just fill in 0.

- Framework: The neural network framework in the inference stage. (e.g. Caffe, Tensorflow, Pytorch)

- Param. Num: Total numbers of the parameters in the neural network.

- Param. Precision: Bits for storing one parameter. Besides, using “I” for indicating the integer, and using “F” for indicating the floating number. For example, if the proposed method uses 16 bit integer to represent a parameter, you can report this information as “16 (I)”

- Mem.P(MB): Memory usage of the total parameters. Mem=P∙B1024*1024, where P is the parameters number, and B indicates the byte for storing one parameter.

- Mem.T(MB): Temporary memory. It denotes the memory used to store the output feature map in each intermediate layer (forward pass). Since different size of input may influence the value, it is suggested to use 3840x2160 as the input size for unification, if there is no parallel operation. Or, if block level parallel operation is used in the proposed method, the block size can be used as the input for calculation, while the input size should be reported. For reporting Mem.T(MB) the calculation process is also suggested to be provided for crosschecking.

Table 2 gives the template for reporting all this information. The BD-rate performance and encoding/decoding complexity are also listed in this table. The test should follow the common test condition. Besides, other information in inference stage is also suggested to report as supplementary.

- Test GPU: The GPU card if it is used in inference stage. e.g., GTX 1080ti.

- Test CPU: The CPU used in inference stage.

- OS: operation system. e.g., Scientific Linux, Ubuntu, etc.

- Fix-point operation: e.g., yes or no.

Table 2. Reporting Template Table for the Information in Inference Stage

|  | Network Details |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Total Conv. Layers | Total FC Layers | Frame-work | Param. Num | Param. Precision | Mem.P(MB) | Mem.T(MB) (e.g., 4K input) |
| test1 |  |  |  |  |  |  |  |
| test2 |  |  |  |  |  |  |  |
|  | All Intra Main10 Over VTM-3.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time(GPU) | Decoding Time(GPU) | Encoding Time(CPU) | Decoding Time(CPU) |
| test1 |  |  |  |  |  |  |  |
| test2 |  |  |  |  |  |  |  |
|  | Random Access Main10 Over VTM-3.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time(GPU) | Decoding Time(GPU) | Encoding Time(CPU) | Decoding Time(CPU) |
| test1 |  |  |  |  |  |  |  |
| test2 |  |  |  |  |  |  |  |
|  | Low delay B Main10 Over VTM-3.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time(GPU) | Decoding Time(GPU) | Encoding Time(CPU) | Decoding Time(CPU) |
| test1 |  |  |  |  |  |  |  |
| test2 |  |  |  |  |  |  |  |

Given the fast progress of GPU technology, it was suggested to add the number of GPUs used in the experiment, as well as the total memory per GPU.

# Core Experiments

More than 100 experts participated the AHG activities and proposed 40 contributions through the study. Detailed contribution list may be found in appendix. Core Experiments were conducted to compare the coding performance and complexity among several NN based filtering tools, using the same software base, test conditions, performance and complexity evaluation matrix. Experts from more than 25 institutes participated the Core Experiments.

Core Experiments and the summary of CE results are provided in below Tables. We can see that NN based filters bring over 4.6% gain (AI, Y) over VTM-4.0 and up to nearly 4% gain (AI, Y) over VTM-5.0. Higher coding gains were observed for chroma components in general.

CE13: Summary Report on Neural Network based Filter for Video Coding [JVET-N0033]

| Test # | Description | Document | Tester | Cross checker |
| --- | --- | --- | --- | --- |
| CE13-1.1a | Loop filter chain: DBF + SAO + ALF + NN filter. NN filter follows the description in M0159 | JVET-N0110 | Y.-L. Hsiao (Mediatek) | H. Yin (Intel) |
| CE13-1.2a | Loop filter chain: DBF + SAO + ALF + NN filter. NN filter follows the description in M0566 | JVET-N0480 | H. Yin (Intel) | Y.-L. Hsiao (Mediatek) |
| CE13-2.1a | DBF + NN filter + SAO + ALF, NN filter follows the description in M0351 | JVET-N0169 | C. Lin (Hikvision) | Y. Wang (Wuhan Univ.) |
| CE13-2.1b | NN filter only, NN filter follows the description in M0351 | JVET-N0169 | C. Lin (Hikvision) | Y. Wang (Wuhan Univ.) |
| CE13-2.1c | NN filter + ALF, NN filter follows the description in M0351 | JVET-N0169 | C. Lin (Hikvision) | Y. Wang (Wuhan Univ.) |
| CE13-2.2a | DBF + NN filter + SAO + ALF, NN filter follows the description in M0508 | JVET-N0254 | Y. Wang (Wuhan Univ.) | C. Lin (Hikvision) |
| CE13-2.2b | NN filter only, NN filter follows the description in M0508 | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.2c | NN filter + ALF, NN filter follows the description in M0508 | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.3a | CTC QP for training, CTC QP for testing (the same as CE13-2.2.a) | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.3b | CTC QP for training, CTC QP + 2 for testing | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.3c | CTC QP for training, CTC QP - 2 for testing | JVET-N0254 | Y. Wang (Wuhan Univ.) |  |
| CE13-2.4a | DBF + NN filter + SAO + ALF, NN filter follows the description in M0510 | JVET-N0513 | Y. Dai (USTC) | Y. Wang (Wuhan Univ.) |
| CE13-2.4b | NN filter only, NN filter follows the description in M0510 | JVET-N0513 | Y. Dai (USTC) |  |
| CE13-2.4c | NN filter + ALF, NN filter follows the description in M0510 | JVET-N0513 | Y. Dai (USTC) |  |
| CE13-2.5a | CTC QP for training, CTC QP for testing(the same as CE13-2.4.a) | JVET-N0513 | Y. Dai (USTC) |  |
| CE13-2.5b | CTC QP for training, CTC QP + 2 for testing | JVET-N0513 | Y. Dai (USTC) |  |
| CE13-2.5c | CTC QP for training, CTC QP - 2 for testing | JVET-N0513 | Y. Dai (USTC) |  |
| CE13-2.6a | DBF + NN filter + SAO + ALF, NN filter follows the description in M0872 | JVET- N710 | K. Kawamura (KDDI) |  |
| CE13-2.6b | NN filter only, NN filter follows the description in M0872 | JVET- N710 | K. Kawamura (KDDI) |  |
| CE13-2.6c | NN filter + ALF, NN filter follows the description in M0872 | JVET- N710 | K. Kawamura (KDDI) |  |
| CE13-2.7a | NN filter adaptive on/off in CTU level | JVET- N710 | K. Kawamura (KDDI) |  |
| CE13-2.7b | NN filter always on in CTU level | JVET- N710 | K. Kawamura (KDDI) |  |

CE13: Summary Report on Neural Network based Filter for Video Coding [JVET-N0033]

|  |  | AI Over VTM-4.0 |  |  |  |  | RA Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC Full test | CE13-1.1a |  |  |  |  |  | -1.36% | -14.96% | -14.91% | 100% | 142% |
|  | CE13-1.2a |  |  |  |  |  | -0.58% | -10.91% | -10.69% | 103% | 127% |
|  | CE13-2.1a | -3.48% | -5.18% | -6.77% | 142% | 38414% |  |  |  |  |  |
|  | CE13-2.1b | -4.14% | -5.49% | -6.70% | 140% | 38411% |  |  |  |  |  |
|  | CE13-2.1c | -4.65% | -6.73% | -7.92% | 139% | 37956% |  |  |  |  |  |
|  | CE13-2.2a | -1.52% | -2.12% | -2.73% | 107% | 4667% | -1.45% | -4.37% | -4.27% | 106% | 7156% |
|  | CE13-2.4a | -0.87% | -0.44% | -0.56% | 106% | 1912% | -0.49% | -0.23% | -0.33% | 124% | 468% |
|  | CE13-2.6a |  |  |  |  |  |  |  |  |  |  |
| Test Condition 2 | CE13-2.2a |  |  |  |  |  | -1.62% | -4.51% | -4.42% | 106% | 7559% |
|  | CE13-2.2b |  |  |  |  |  | 1.51% | 1.21% | 1.80% | 105% | 9910% |
|  | CE13-2.2c |  |  |  |  |  | -0.65% | -2.18% | -1.60% | 105% | 9937% |
|  | CE13-2.4a |  |  |  |  |  | -0.53% | -0.23% | -0.43% | 90% | 349% |
|  | CE13-2.4b |  |  |  |  |  | 5.73% | 6.09% | 6.07% | 95% | 610% |
|  | CE13-2.4c |  |  |  |  |  | 1.14% | 2.52% | 2.65% | 90% | 497% |
|  | CE13-2.6a |  |  |  |  |  | -1.70% | -2.84% | -4.09% | 139% | 63166% |
|  | CE13-2.6b |  |  |  |  |  | 2.26% | 3.86% | 2.09% | 138% | 66303% |
|  | CE13-2.6c |  |  |  |  |  | -0.80% | -1.21% | -2.59% | 138% | 66626% |
|  | CE13-2.7a |  |  |  |  |  | -1.70% | -2.84% | -4.09% | 139% | 63166% |
|  | CE13-2.7b |  |  |  |  |  | -1.68% | -2.79% | -4.05% | 138% | 69946% |
| Test Condition 3 | CE13-2.3a |  |  |  |  |  | -1.62% | -4.51% | -4.42% | 106% | 7559% |
|  | CE13-2.3b |  |  |  |  |  | -1.88% | -4.81% | -4.33% | 107% | 8008% |
|  | CE13-2.3c |  |  |  |  |  | -1.37% | -4.57% | -4.04% | 105% | 6794% |
|  | CE13-2.5a |  |  |  |  |  | -0.53% | -0.23% | -0.43% | 90% | 349% |
|  | CE13-2.5b |  |  |  |  |  | -0.52% | -0.31% | -0.48% | 98% | 378% |
|  | CE13-2.5c |  |  |  |  |  | -0.46% | -0.33% | -0.40% | 113% | 422% |

CE13 Complexity analysis [JVET-N0033]

| Information in the Inference Stage Combining with Codec |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Network Details |  |  |  |  |  |  |
|  | Total Conv. Layers | Total FC Layers | Framework | Param. Num | Param. Precision | Mem.P(MB) | Mem.T(MB)(e.g., 4K input) |
| CE13-1.1 | 3 | 0 | NA | 1506 | 6-bit/32-bit (I) | 0.001282 | 0.344 (128x128, CTU based) |
| CE13-1.2 | 2 | 0 | NA | 692x3 (Luma) | 6-bit/32-bit (I) | 0.0028 | 0.0448(128x128, CTU based) |
|  |  |  |  | 402x3 (Chroma) |  |  |  |
| CE13-2.1 | 8 | 0 | Caffe | 224960 | 32-bit (F) | 0.86 | 1846.945268(192x152, block based) |
| CE13-2.2 | 21 | 0 | PyTorch | 22371 | 32-bit (F) | 0.09 | 196.23(pixels num. < 80000, block based) |
| CE13-2.4 | 10 | 0 | Tensorflow | 7521 | 32-bit (F) | 0.029 | 10.06(128*128,CTU based) |
| CE13-2.6 |  |  |  |  |  |  |  |

CE10: Summary Report on Neural Network based Filter for Video Coding [JVET-O0030]

| Test # | Test Part | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- | --- |
| CE10-1.2a | in-loop/ post filter | CNNLF as in-loop filter | JVET-N0110 | Y.-L. Hsiao (Mediatek) | R. Yang (Intel) |
| CE10-1.7a | in-loop/ post filter | NN Filter as in-loop filter | JVET-N0480 | H. Yin (Intel) | Y.-L. Hsiao (Mediatek) |
| CE10-1.7b |  | NN Filter as post filter |  |  | Y.-L. Hsiao (Mediatek) |
| CE10-1.10a | bit-depth | 6-bit(int) weights | JVET-N0712 | K. Kawamura (KDDI) | H. Yin (Intel) |
| CE10-1.10b |  | 14-bit(int) weights |  |  | H. Yin (Intel) |
| CE10-1.11a | in-loop/ post filtering | CNNF as in loop filter |  | K. Kawamura (KDDI) | H. Yin (Intel) |
| CE10-1.11b |  | CNNF as post filter |  |  | H. Yin (Intel) |
| CE10-2.1a | bit-depth | 32-bit(float) weights | JVET-N0133 | Shuai Wan (NPU) | T. Ouyang (Wuhan Univ) |
| CE10-2.1b |  | 8-bit(int) weights |  |  |  |
| CE10-2.2a | in-loop/ post filter | CNNF as in-loop filter |  |  | C.-Y Lin (Hikvision) |
| CE10-2.2b |  | CNNF as post filter |  |  |  |
|  | generalization capability (QP) |  |  |  |  |
| CE10-2.3b |  | CTC QP+5 |  |  | T. Ouyang (Wuhan Univ) |
| CE10-2.5a | bit-depth | 32-bit(float) weights | JVET-N0254 | Y. Wang (Wuhan Univ.) | M. Wang (NPU) |
| CE10-2.5b |  | 8-bit(int) weights |  |  | M. Wang (NPU) |
| CE10-2.7a | generalization capability (QP) | CTC QP-5 |  |  | M. Wang (NPU) |
| CE10-2.7b |  | CTC QP+5 |  |  | M. Wang (NPU) |
| CE10-2.8c | bit-depth | 8-bit(int) weights | JVET-N0513 | Y. Dai (USTC) H. Zhao (USTC) | C.-Y Lin (Hikvision) |
| CE10-2.10a | bit-depth | 8-bit(int) weights | JVET-N0710 | K. Kawamura (KDDI) | H. Yin (Intel) |
| CE10-2.10b |  | 16-bit(int) weights |  |  | H. Yin (Intel) |
| CE10-2.11a | in-loop/ post filtering | CNNF as in loop filter |  | K. Kawamura (KDDI) | H. Yin (Intel) |
| CE10-2.11b |  | CNNF as post filter |  |  | H. Yin (Intel) |

CE10: Summary Report on Neural Network based Filter for Video Coding [JVET-O0030]

|  |  | AI Over VTM-5.0 |  |  |  |  | RA Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CTC Full test | CE10-1.2a |  |  |  |  |  | -1.20% | -14.17% | -14.11% | 100% | 117% |
|  | CE10-1.7a |  |  |  |  |  | -0.80% | -10.56% | -9.93% | 100% | 129% |
|  | CE10-1.7b |  |  |  |  |  | -0.95% | -10.78% | -10.39% | 100% | 158% |
|  | CE10-1.11a |  |  |  |  |  | -0.04% | -15.54% | -15.17% | 99% | 190% |
|  | CE10-1.11b |  |  |  |  |  | 0.25% | -5.04% | -6.01% | 100% | 213% |
|  | CE10-2.2a | -3.93% | -7.89% | -7.29% | 134% | 74517% | -0.26% | -3.86% | -2.91% | 139% | 132513% |
|  | CE10-2.2b | -3.05% | -9.02% | -10.47% | 136% | 74083% | -1.89% | -11.91% | -8.90% | 139% | 135869% |
|  | CE10-2.8c | -0.51% | -0.30% | -0.60% | 92% | 188% |  |  |  |  |  |
|  | CE10-2.11a |  |  |  |  |  | -1.10% | -2.94% | -3.35% | 127% | 56667% |
|  | CE10-2.11b |  |  |  |  |  | -0.32% | -0.66% | -0.81% | 100% | 64536% |
| Test Condition 2 | CE10-1.10a |  |  |  |  |  | 0.04% | -0.97% | -2.30% | 100% | 127% |
|  | CE10-1.10b |  |  |  |  |  | 0.23% | -15.24% | -13.66% | 100% | 190% |
|  | CE10-2.1a |  |  |  |  |  | -0.46% | -4.11% | -2.80% | 142% | 134684% |
|  | CE10-2.1b |  |  |  |  |  | 0.79% | -5.06% | -4.73% | 192% | 306401% |
|  | CE10-2.5a |  |  |  |  |  | -1.01% | -5.01% | -4.24% | 105% | 5652% |
|  | CE10-2.5b |  |  |  |  |  | -0.84% | -4.13% | -2.92% | 107% | 5515% |
|  | CE10-2.8c |  |  |  |  |  |  |  |  |  |  |
|  | CE10-2.10a |  |  |  |  |  | -1.00% | -2.98% | -3.46% | 128% | 57777% |
|  | CE10-2.10b |  |  |  |  |  | -1.16% | -3.21% | -3.59% | 129% | 59072% |
| Test Condition 3 | CE10-2.2a |  |  |  |  |  | -0.46% | -4.11% | -2.80% | 142% | 134684% |
|  | CE10-2.3b |  |  |  |  |  | -1.52% | -6.15% | -4.31% | 166% | 154379% |
|  | CE10-2.5a |  |  |  |  |  | -1.01% | -5.01% | -4.24% | 105% | 5652% |
|  | CE10-2.7a |  |  |  |  |  | -0.63% | -3.83% | -3.34% | 105% | 5026% |
|  | CE10-2.7b |  |  |  |  |  | -1.15% | -5.01% | -4.47% | 111% | 5525% |

CE10 Complexity analysis [JVET-O0030]

| Information in the Inference Stage Combining with Codec |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Network Details |  |  |  |  |  |  |
|  | Total Conv. Layers | Total FC Layers | Framework | Param. Num | Param. Precision | Mem.P(MB) | Mem.T (MB) (e.g., 4K input) |
| CE10-1.2 | 3 | 0 | NA | 1506 | 6(I), 32(I) | 0.001282 | 0.344 (128x128, CTU based) |
| CE10-1.7 | 2 | 0 | NA | 732x3 (Luma), 402x3 (Chroma) | 6(I), 32(I) | 0.0028 | 0.0448 |
| CE10-1.10 |  |  |  |  |  |  |  |
| CE10-2.1 | 39 | 36 | Tensorflow | 519112 | 32(F) | 4.93 | H*W*2.68*10e-3 |
| CE10-2.5 | 21 | 0 | PyTorch | 22371 | CE10-2.5a: 32(F), CE10-2.5b: 8(I) | CE10-2.5a: 0.09, CE10-2.5b: 0.02 | 196.23 |
| CE10-2.8 | 11 | 0 | cuDNN | 7521 | 8(I)32(I) | 0.01 | 3659(4K as input) |
| CE10-2.10 | 4 | 0 | Caffe | 56481 | CE10-2.10a: 8(I), 15(I) CE10-2.10b: 16(I), 15(I) | CE10-1.10a: 0.056762 CE10-1.10b: 0.112922 | 10.063 (128x128, CTU based) |

Figure 1 Subjective evaluations of Campfire (POC=58) in original vs. VTM5 vs.CE10-2.11a vs. CE10-2.11b

CE10: Informal viewing test [JVET-O0030]

Figure 2 Subjective evaluations of RitualDance (POC=414) in original vs. VTM5 vs. CE10-2.11a vs. CE10-2.11b

# Software Repository and Suggested CTC

Based on AHG discussion and for better comparison and alignment with related CE work, it is suggested to follow VVC v1 CTC JVET-N1010 to evaluate the coding performance of NN based tools. For optional tests, short tests such as one intra period may be used.

To ease study and crosscheck, it is suggested to follow earlier practice of CE on NN based tools to commit code to VTM git repository https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM under AHG or CE branch. As an example, following figure shows the branches of previous CE10 on NN based coding tools.

# Recommendations

NN based coding tools show interesting performance improvement and draw more and more attention. Due to the diversity of NN platform, it is important to have the study in a well-organized manner, preferably making the best use of earlier experience. Therefore, it is proposed to resume the AHG on neural networks in video coding based on earlier work during VVC v1 development with the following updated mandates:

- Investigate the benefit of using neural networks in video compression such as NN loop filter, intra prediction, resampling in adaptive resolution coding, and encoder-side partition mode decisions.

- Investigate the complexity impact of using neural networks in video compression.

- Investigate the complexity measurement of neural network coding tools.

- Investigate benefit of universal versus selectable networks, both in terms of compression benefit and complexity.

- Investigate how NN parameters can be established for operation of the decoding process.

- Investigate SEI messages for NN based post processing.

- Investigate the impact of training materials on the performance of neural network coding tools.

- Investigate the impact of the training process on performance and complexity.

- Identify video data sets available publicly or for standardization purposes and try to build common training video data set.

# Reference

- “JVET AHG report 9: Neural Networks in Video Coding (AHG9)”, S. Liu, L. Wang, P. Wu, H. Yang, ISO/IEC JTC1/SC29/WG11 JVET-J0009.

- “Deep Learning-Based Technology in Responses to the Joint Call for Proposals on Video Compression with Capability beyond HEVC”, D. Liu, Z. Chen, S. Liu, F. Wu, IEEE Transactions on Circuits and Systems for Video Technology, 2019.

- “Methodology and reporting template for neural network coding tool testing”, Y. Li, S. Liu, K. Kawamura, ISO/IEC JTC1/SC29/WG11 JVET-M1006.

- “Description of Core Experiment 13 (CE13): Neural Network based Filter for Video Coding”, Y. Li, S. Liu, K. Kawamura, ISO/IEC JTC1/SC29/WG11 JVET-M1033.

- “CE13: Summary Report on Neural Network based Filter for Video Coding”, Y. Li, S. Liu, K. Kawamura, ISO/IEC JTC1/SC29/WG11 JVET-N0033.

- “Description of Core Experiment 10 (CE10): Neural Network based Filter for Video Coding”, Y. Li, S. Liu, K. Kawamura, ISO/IEC JTC1/SC29/WG11 JVET-N1030.

- “CE10: Summary Report on Neural Network based Filter for Video Coding”, Y. Li, S. Liu, K. Kawamura, ISO/IEC JTC1/SC29/WG11 JVET-O0030.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.

Huawei does not have any current or pending patent rights relating to the technology described in this contribution.

KDDI does not have any current or pending patent rights relating to the technology described in this contribution.

ZTE does not have any current or pending patent rights relating to the technology described in this contribution.

Sharp Labs of America does not have any current or pending patent rights relating to the technology described in this contribution.

RWTH Aachen does not have any current or pending patent rights relating to the technology described in this contribution.

Fraunhofer HHI does not have any current or pending patent rights relating to the technology described in this contribution.

Kwai does not have any current or pending patent rights relating to the technology described in this contribution.

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

Intel does not have any current or pending patent rights relating to the technology described in this contribution.

MediaTek does not have any current or pending patent rights relating to the technology described in this contribution.

USTC does not have any current or pending patent rights relating to the technology described in this contribution.

Qualcomm does not have any current or pending patent rights relating to the technology described in this contribution.

Comcast does not have any current or pending patent rights relating to the technology described in this contribution.

# Appendix

- JVET-I0022, “Convolutional Neural Network Filter (CNNF) for intra frame”, L. Zhou, X. Song, J. Yao, L. Wang, F. Chen (Hikvision), Gwangju, KR, 20–26 Jan. 2018

- JVET-J0034, “AHG9: CNN-based driving of block partitioning for intra slices encoding”, F. Galpin, F. Racapé, P. Bordes, F. Le Léannec, E. François (Technicolor), San Diego, US, 10–20 Apr. 2018

- JVET-J0043, “AHG9: Convolutional Neural Network Filter for inter frame”, J. Yao, X. Song, S. Fang, L. Wang (Hikvision), San Diego, US, 10–20 Apr. 2018

- JVET-J0076, “AHG9: Results of CNN filter in JVET-I0022 as in-loop filter and post-processing filter”, L. Zhao, X. Li, S. Liu (Tencent), San Diego, US, 10–20 Apr. 2018

- JVET-J0037, “Intra Prediction Modes based on Neural Networks”, J. Pfaff, P Helle, D. Maniry, S. Kaltenstadler, B. Stallenberger, P. Merkle, M. Siekmann, H. Schwarz, D. Marpe, T. Wiegand (HHI), San Diego, US, 10–20 Apr. 2018

- JVET-J0014, “Description of SDR, HDR and 360° video coding technology proposal by Fraunhofer HHI”, M. Albrecht, C. Bartnik, S. Bosse, J. Brandenburg, B. Bross, J. Erfurt, V. George, P. Haase, P. Helle, C. Helmrich, A. Henkel, T. Hinz, S. de Luxan Hernandez, S. Kaltenstadler, P. Keydel, H. Kirchhoffer, C. Lehmann, W.-Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, B. Stallenberger, J. Stegemann, K. Suehring, G. Tech, G. Venugopal, S. Walter, A. Wieckowski, T. Wiegand, M. Winken (Fraunhofer HHI), San Diego, US, 10–20 Apr. 2018

- JVET-J0018, “Description of SDR video coding technology proposal by MediaTek”, C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei (MediaTek), San Diego, US, 10–20 Apr. 2018

- JVET-J0022, “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – medium complexity version”, P. Bordes, Y. Chen, C. Chevance, E. François, F. Galpin, M. Kerdranvat, F. Hiron, P. de Lagrange, F. Le Léannec, K. Naser, T. Poirier, F. Racapé, G. Rath, A. Robert, F. Urban, T. Viellard (Technicolor), Y. Chen, W.-J. Chien, H.-C. Chuang, M. Coban, J. Dong, H. E. Egilmez, N. Hu, M. Karczewicz, A. Ramasubramonian, D. Rusanovskyy, A. Said, V. Seregin, G. Van Der Auwera, K. Zhang, L. Zhang (Qualcomm), San Diego, US, 10–20 Apr. 2018

- JVET-J0031, “Description of SDR video coding technology proposal by University of Bristol”, D. Bull, F. Zhang, M. Afonso (Univ. of Bristol), San Diego, US, 10–20 Apr. 2018

- JVET-J0032, “Description of SDR video coding technology proposal by University of Science and Technology of China, Peking University, Harbin Institute of Technology, and Wuhan University (IEEE 1857.10 Study Group)”, F. Wu, D. Liu, J. Xu, B. Li, H. Li, Z. Chen, L. Li, F. Chen, Y. Dai, L. Guo, Y. Li, Y. Li, J. Lin, C. Ma, N. Yan (USTC), W. Gao, S. Ma, R. Xiong, Y. Xu, J. Li (Peking Univ.), X. Fan, N. Zhang, Y. Wang, T. Zhang, M. Gao (Harbin Inst. Tech.), Z. Chen, Y. Zhou, X. Pan, Y. Li, F. Liu, Y. Wang (Wuhan Univ.), San Diego, US, 10–20 Apr. 2018

- JVET-K0158, “AHG9: Separable Convolutional Neural Network Filter with Squeeze-and-Excitation block”, T. Hashimoto, E. Sasaki, T. Ikai (Sharp), Ljubljana, SI, 10–18 July 2018

- JVET-K0222, “AHG9: Convolution neural network loop filter”, Y.-L. Hsiao, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), Ljubljana, SI, 10–18 July 2018

- JVET-K0391, “AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao (Tencent), Ljubljana, SI, 10–18 July 2018

- JVET-K0266, “CE3: Non-linear weighted intra prediction (Test 6.1.1)”, P. Merkle, J. Pfaff, P. Helle, R. Rischke, M. Schäfer, B. Stallenberger, H. Schwarz, D. Marpe, T. Wiegand (Fraunhofer HHI), Ljubljana, SI, 10–18 July 2018

- JVET-L0242, “AHG9: Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao (Tencent), Macao, CN, 03–12 Oct. 2018

- JVET-L0383, “AHG9: Convolution Neural Network Filter”, K. Kawamura, Y. Kidani, S. Naito (KDDI), Macao, CN, 03–12 Oct. 2018

- JVET-M0159, “AHG9: Convolutional neural network loop filter”, Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0215, “AHG9-related: CNN-based lambda-domain rate control for intra frames”, Y. Li, D. Liu, Z. Chen (USTC), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0351, “AHG9: Convolutional Neural Network Filter (CNNF) for Intra Frame”, C. Lin, J. Yao, L. Wang (Hikvision), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0508, “AHG9: Test Results of Dense Residual Convolutional Neural Network based In-Loop Filter”, Y. Wang, Z. Chen, Y. Li (Wuhan Univ.), L. Zhao, S. Liu, X. Li (Tencent), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0510, “AHG9: CNN-based in-loop filter proposed by USTC”, Y. Dai, D. Liu, Y. Li, F. Wu (USTC), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0566, “Adaptive convolutional neural network loop filter”, H. Yin, R. Yang, X. Fang, S. Ma, Y. Yu (Intel), Marrakech, MA, 9–18 Jan. 2019

- JVET-M0691, “AHG9: Complexity analysis about neural network video coding tools”, Y. Li, Z. Chen (WHU), S. Liu (Tencent), Marrakech, MA, 9–18 Jan. 2019

- JVET-N0110, “CE13-1.1: Convolutional neural network loop filter”, Y.-L. Hsiao, O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, Geneva CH, 19–27 March 2019

- JVET-N0169, “CE13-2.1: Convolutional Neural Network Filter (CNNF) for Intra Frame”, J. Yao, L.Wang, Geneva CH, 19–27 March 2019

- JVET-N0254, “CE13: Dense Residual Convolutional Neural Network based In-Loop Filter (Test 2.2 and 2.3)”, Y. Wang, Z. Chen, Y. Li, L. Zhao, S. Liu, X. Li, Geneva CH, 19–27 March 2019

- JVET-N0480, “CE13-1.2: Adaptive convolutional neural network loop filter”, H. Yin, R. Yang, X. Fang, S. Ma, Geneva, CH, 19–27 March 2019

- JVET-N0513, “CE13: Experimental results of CNN-based In-Loop Filter (USTC)”, Y. Dai, D. Liu, N. Yan, F. Wu, Geneva, CH, 19–27 March 2019

- JVET-N0133, “CE13-related: In-loop filter with only CNN-based filter”, S. Wan, M.-Z.Wang, H. Gong, C.-Y. Zou, Y.-Z. Ma, J.-Y. Huo, Y.-F. Yu, Y. Liu, Geneva CH, 19–27 March 2019

- JVET-N0065, “AHG17 & AHG9: Comments on carriage of coding tool parameters in Adaptation Parameter Set”, M. Li, P. Wu, Geneva CH, 19–27 March 2019

- JVET-O0056, “CE10-1.2: Convolutional neural network loop filter”, Y.-L. Hsiao, O. Chubach, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), Gothenburg, SE, 3–12 July 2019

- JVET-O0063, “CE10-1.7: Adaptive convolutional neural network loop filter”, H. Yin, R. Yang, X. Fang, S. Ma (Intel), Gothenburg, SE, 3–12 July 2019

- JVET-O0079, “CE10: Integrated in-loop filter based on CNN (Tests 2.1, 2.2 and 2.3) ”, S. Wan, M.-Z.Wang, H. Gong, C.-Y. Zou (NPU), Y.-Z. Ma, J.-Y. Huo (Xidian Univ.), Y.-F. Yu, Y. Liu (OPPO), Gothenburg, SE, 3–12 July 2019

- JVET-O0101, “CE10: Dense Residual Convolutional Neural Network based In-Loop Filter (Tests 2.5 and 2.7) ”, Y. Wang, T. Ouyang, C. Zou, Y. Li, Z. Chen(Wuhan Univ.), L. Zhao, S. Liu, X. Li(Tencent), Gothenburg, SE, 3–12 July 2019

- JVET-O0131, “CE10-1.10/CE10-1.11: Evaluation results of CNN-based filtering with on-line learning model”, Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI), Gothenburg, SE, 3–12 July 2019

- JVET-O0132, “CE10-2.10/CE10-2.11: Evaluation results of CNN-based filtering with off-line learning model”, Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI), Gothenburg, SE, 3–12 July 2019

- JVET-O0347, “CE10:Neural Network based Filter for Video Coding”, H. Zhao, Y. Dai, D. Liu, N. Yan, H. Li (USTC), X. Chen, H. Yang (Huawei), Gothenburg, SE, 3–12 July 2019

- JVET-O0157, “Non-CE10: A CNN based in-loop filter for intra frame”, L. Xu, J. Zhu, K. Kazui (Fujitsu), Gothenburg, SE, 3–12 July 2019

- JVET-O0348, “CE10-Related: Classifier Based Adaptive Convolutional Neural Network Loop Filter”, H. Yin, R. Yang, X. Fang, S. Ma (Intel), Gothenburg, SE, 3–12 July 2019

- JVET-P0489, “AHG9: Multiple Convolution Neural Networks For Sequence-Independent Processing”, H.Yin, R.Yang, X.Fang, Z.Gao, R.Yang, Geneva CH, 1–11 October 2019