| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 October 2020 | Document: JVET-T_Notes_d1 |
| --- | --- |

| Title: | Meeting Report of the 20th Meeting of the Joint Video Experts Team (JVET), by teleconference, 7–16 October 2020 |  |  |
| --- | --- | --- | --- |
| Status: | Report document from the chairs of JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Gary Sullivan Microsoft Corp. 1 Microsoft Way Redmond, WA 98052 USA Jens-Rainer Ohm Institute of Communication Engineering RWTH Aachen Melatener Straße 23 D-52074 Aachen | Tel: Email: Tel: Email: | +1 425 703 5308 garysull@microsoft.com +49 241 80 27671 ohm@ient.rwth-aachen.de |
| Source: | Chairs of JVET |  |  |

_____________________________

# Summary

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29 held its twentieth meeting during 7–16 October 2020 as an online-only meeting. It had previously been planned to be in Rennes, FR, but this plan was changed due to the difficulties resulting from the COVID-19 pandemic. The JVET meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany). For rapid access to particular topics in this report, a subject categorization is found (with hyperlinks) in section 2.13 of this document. It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of a new standard. The name Versatile Video Coding (VVC) was chosen in April 2018 as the informal nickname for the new standard.

The JVET meeting began at approximately 0500 hours UTC on Wednesday 7 October 2020. Meeting sessions were held on all days except the weekend days of Saturday and Sunday 10 and 11 October 2020, until the meeting was closed at approximately XXXX hours UTC on Friday 16 October 2020. Approximately XXX people attended the JVET meeting, and approximately XXX input documents (not counting crosschecks), and 18 AHG reports were discussed. The latter included reports of 5 AHGs which had originally been established by the Joint Collaborative Team on Video Coding (JCT-VC), after the parent bodies had negotiated to continue those work items within JVET as a single joint team. The meeting took place in a collocated fashion with a meeting of various SG16 Working Groups – where WG 5 is representing the Joint Video Coding Team(s) and their activities from the SC 29 parent body. The subject matter of the JVET meeting activities consisted of work on further development and maintenance of the twin-text video coding technology standards Advanced Video Coding (AVC), High Efficiency Video Coding (HEVC), Versatile Video Coding (VVC), Coding-independent Code Points (Video) (CICP), and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI), as well as related technical reports, software and conformance packages. As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the nineteenth JVET meeting in producing a tenth draft of the VVC standard (which was approved by the parent bodies as version 1 of the VVC standard), and the tenth version of the associated VVC test model (VTM). Further important goals were reviewing technical input on novel aspects of video coding technology, and plan next steps for investigation of candidate technology towards further standard development.

The JVET produced X output documents from the meeting (Update):

- JVET-S2001 Versatile Video Coding specification text (Draft 10)

- JVET-S2002 Algorithm description for Versatile Video Coding and Test Model 10 (VTM 10)

- JVET-S2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 11)

- JVET-S2005 Methodology and reporting template for coding tool testing

- JVET-S2007 Supplemental enhancement information messages for coded video bitstreams (Draft 5)

- JVET-S2008 Conformance testing for versatile video coding (Draft 4)

- JVET-S2009 VVC verification test plan (Draft 3)

- JVET-S2011 JVET common test conditions and evaluation procedures for HDR/WCG video

- JVET-S2016 Summary information on BD-rate experiment evaluation practices.

For the organization and planning of its future work, the JVET established X “ad hoc groups” (AHGs) to progress the work on particular subject areas. At this meeting, no Core Experiments (CE) were defined. The next four JVET meetings were planned for 8–15 January 2021 under SC29 auspices in Capetown, ZA, during 21–28 April 2021 under ITU-T SG16 auspices in Geneva, CH, during 9–16 July 2021 under SC29 auspices in Prague, CZ, and during … .

The document distribution site http://phenix.int-evry.fr/jvet/ was used for distribution of all documents.

The reflector to be used for discussions by the JVET and all its AHGs is the JVET reflector:
jvet@lists.rwth-aachen.de hosted at RWTH Aachen University. For subscription to this list, see https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/.

# Administrative topics

## Organization

The ITU-T/ISO/IEC Joint Video Experts Team (JVET) is a group of video coding experts from the ITU-T Study Group 16 Visual Coding Experts Group (VCEG) and the ISO/IEC JTC 1/SC 29/WG 5. The parent bodies of the JVET are ITU-T WP3/16 and ISO/IEC JTC 1/SC 29.

The Joint Video Experts Team (JVET) of ITU-T WP3/16 and ISO/IEC JTC 1/ SC 29 held its twentieth meeting 7 – 16 October 2020 as an online-only meeting, using Zoom teleconferencing tools. The JVET meeting was held under the chairmanship of Dr Gary Sullivan (Microsoft/USA) and Dr Jens-Rainer Ohm (RWTH Aachen/Germany).

It is further noted that the unabbreviated name of JVET was formerly known as “Joint Video Exploration Team”, but the parent bodies modified it when entering the phase of formal development of the Versatile Video Coding (VVC) and Versatile Supplemental Enhancement Information Messages for Coded Video Bitstreams (VSEI) standards. Furthermore, starting from the twentieth meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical report(s),

- Maintenance and minor enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

## Meeting logistics

Information regarding logistics arrangements for the meeting had been provided via the email reflector jvet@lists.rwth-aachen.de and at http://wftp3.itu.int/av-arch/jvet-site/2020_10_T_Virtual/.

## Primary goals

As a primary goal, the JVET meeting reviewed the work that was performed in the interim period since the nineteenth JVET meeting in producing a tenth draft of the VVC standard (which was approved by the parent bodies as version 1 of the VVC standard), and the tenth version of the associated VVC test model (VTM). Further important goals were reviewing technical input on novel aspects of video coding technology, and plan next steps for investigation of candidate technology towards further standard development.

## Documents and document handling considerations

### General

The documents of the JVET meeting are listed in Annex A of this report. The documents can be found at http://phenix.int-evry.fr/jvet/.

Registration timestamps, initial upload timestamps, and final upload timestamps are listed in Annex A of this report.

The document registration and upload times and dates listed in Annex A and in headings for documents in this report are in Paris/Geneva time. Dates mentioned for purposes of describing events at the meeting (other than as contribution registration and upload times) follow the local time at the meeting facility.

Highlighting of recorded decisions in this report is practised as follows:

- Decisions made by the group that might affect the normative content of a future standard are identified in this report by prefixing the description of the decision with the string “Decision:”.

- Decisions that affect the VTM software but have no normative effect are marked by the string “Decision (SW):”.

- Decisions that fix a “bug” in the VTM description (an error, oversight, or messiness) or in the software are marked by the string “Decision (BF):”.

- Decisions that are merely editorial without effect on the technical content of the draft standard are marked by the string "Decision (Ed.):". Such editorial decisions are merely suggestions to the editor, who has the discretion to determine the final action taken if their judgment differs.

This meeting report is based primarily on notes taken by the JVET chairs. The preliminary notes were also circulated publicly by ftp and http during the meeting on a daily basis. It should be understood by the reader that 1) some notes may appear in abbreviated form, 2) summaries of the content of contributions are often based on abstracts provided by contributing proponents without an intent to imply endorsement of the views expressed therein, and 3) the depth of discussion of the content of the various contributions in this report is not uniform. Generally, the report is written to include as much information about the contributions and discussions as is feasible (in the interest of aiding study), although this approach may not result in the most polished output report.

### Late and incomplete document considerations

The formal deadline for registering and uploading non-administrative contributions had been announced as Wednesday, 30 September 2020. Any documents uploaded after 1159 hours Paris/Geneva time on Thursday 1 October 2020 were considered “officially late”, giving a grace period of 12 hours to accommodate those living in different time zones of the world. The deadline does not apply to AHG reports, and other such reports which can only be produced after the availability of other input documents.

All contribution documents with registration numbers higher than JVET-T0093 were registered after the “officially late” deadline (and therefore were also uploaded late). However, some documents in the “late” range might include break-out activity reports that were generated during the meetings, and are therefore better considered as report documents rather than as late contributions. Also, all cross-check reports were uploaded late.

In many cases, contributions were also revised after the initial version was uploaded. The contribution document archive website retains publicly accessible prior versions in such cases. The timing of late document availability for contributions is generally noted in the section discussing each contribution in this report.

One suggestion to assist with the issue of late submissions was to require the submitters of late contributions and late revisions to describe the characteristics of the late or revised (or missing) material at the beginning of discussion of the contribution. This was agreed to be a helpful approach to be followed at the meeting.

The following technical design proposal contributions were registered and/or uploaded late:

- JVET-T0XXX (a proposal on …), uploaded XX-XX.

- …

It may be observed that some of the above-listed contributions were submissions made in response to issues that arose in discussions during the meeting or from the study of other contributions, and thus could not have been submitted by the ordinary deadline. For example, some of them were proposing combinations or simplifications of other proposals.

The following other document not proposing normative technical content, but with some need for consideration, were registered and/or uploaded late:

- JVET-T0XXX (a document on …), uploaded XX-XX.

- …

All cross-verification reports at this meeting were registered late, and even those registered before the deadline were all uploaded late. In the interest of brevity, these are not specifically identified here. Initial upload times for each document are recorded in Annex A of this report.

The following (X) contribution registrations were later cancelled, withdrawn, never provided, were cross-checks of a withdrawn contribution, or were registered in error: JVET-T0083, ...

“Placeholder” contribution documents that were basically empty of content, or lacking any results showing benefit for the proposed technology, and obviously uploaded with an intent to provide a more complete submission as a revision, had been agreed to be considered unacceptable and to be rejected in the document management system until a more complete version was available (which would then typically be counted as a late contribution). At the current meeting, this situation applied to the initial uploads of documents JVET-T0XXX, … .

Contributions that had significant problems with uploaded versions included the following:

- JVET-T0XXX (…)

- …

As a general policy, missing documents were not to be presented, and late documents (and substantial revisions) could only be presented when there was a consensus to consider them and there was sufficient time available for their review. Again, an exception is applied for AHG reports, CE and HLS topic summaries, and other such reports which can only be produced after the availability of other input documents. There were no objections raised by the group regarding presentation of late contributions, although there was some expression of annoyance and remarks on the difficulty of dealing with late contributions and late revisions.

It was remarked that documents that are substantially revised after the initial upload can also be a problem, as this becomes confusing, interferes with study, and puts an extra burden on synchronization of the discussion. This can especially be a problem in cases where the initial upload is clearly incomplete, and in cases where it is difficult to figure out what parts were changed in a revision. For document contributions, revision marking is very helpful to indicate what has been changed. Also, the “comments” field on the web site can be used to indicate what is different in a revision although participants tend to seldom notice what is recorded there.

A few contributions may have had some problems relating to IPR declarations in the initial uploaded versions (missing declarations, declarations saying they were from the wrong companies, etc.). These issues were corrected by later uploaded versions in a reasonably timely fashion in all cases (to the extent of the awareness of the responsible coordinators).

Some other errors were noticed in other initial document uploads (wrong document numbers or meeting dates or meeting locations in headers, etc.) which were generally sorted out in a reasonably timely fashion. The document web site contains an archive of each upload.

### Outputs of the preceding meeting

All output documents of the previous meeting, particularly the meeting report JVET-S2000, the Versatile Video Coding specification text (Draft 10) JVET-S2001, the Algorithm description for Versatile Video Coding and Test Model 10 (VTM 10) JVET-S2002, the Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 11) JVET-S2004, the Methodology and reporting template for coding tool testing JVET-S2005, the Supplemental enhancement information messages for coded video bitstreams (Draft 5) JVET-S2007, the Conformance testing for VVC (Draft 4) JVET-S2008, the VVC verification test plan (Draft 3) JVET-S2009, the JVET common test conditions and nd evaluation procedures for HDR/WCG video JVET-S2011, the Working practices using objective metrics for evaluation of video coding efficiency experiments (Draft 3) JVET-S2016, and the  Technologies under consideration for VSEI JVET-S2017 had been completed and were approved. The software implementations of VTM (versions 10.0 and 10.1) and 360lib (version 11.0) was also approved. Furthermore, the former JCT-VC output documents from the 40th JCT-VC meeting, particularly the meeting report JCTVC-AN1000, the High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 14 JCTVC-AN1002, the Errata report items for HEVC, AVC, Video CICP, and CP usage TR JCTVC-AN1004, the Annotated regions and shutter interval information SEI messages for AVC (Draft 1) JCTVC-AN1006, and the Usage of video signal type code points (Draft 1 for version 3) JCTVC-AN1008, were approved.

The group was initially asked to review the meeting report(s) of the previous meeting for finalization. The meeting report was later approved with a minor modification of including a missing output document in a list.

The available output documents of the previous meeting and the software had been made available in a reasonably timely fashion.

## Attendance

The list of participants in the JVET meeting can be found in Annex B of this report.

The meeting was open to those qualified to participate either in ITU-T WP3/16 or ISO/IEC JTC 1/‌SC 29/‌WG 11 (including experts who had been personally invited as permitted by ITU-T or ISO/IEC policies).

Participants had been reminded of the need to be properly qualified to attend. Those seeking further information regarding qualifications to attend future meetings may contact the responsible coordinators.

It was further announced that it is necessary to register for the meeting on the WG11 host’s website. Access to the teleconference sessions of the main JVET meeting was controlled with a password that is distributed to the registered participants; this should help overloading the teleconferencing tool.

The following rules were initially set up for the Zoom teleconference meeting:

o Use the “hand-raising” function to enter yourself in the queue to speak (unless otherwise instructed by the session chair). If you are dialed in by phone, request your queue position verbally.

o Stay muted unless you have something to say. (people were muted by default when they join and would need to unmute themselves to speak. The chair may mute anyone who is disrupting the proceedings (e.g. by forgetting they have a live microphone while chatting with their family or by causing bad noise or echo).

o Identify who you are and your affiliation when you begin speaking.

o Use your full name and company/organization affiliation in your joining information. We will use the participation list for attendance records.

o Turn on the chat window and watch for chair communication and side commentary there as well as by audio.

o Avoid overloading people’s internet connections, we do not plan to use video for the teleconferencing calls – only voice and screen sharing. Extensive use of screen sharing is encouraged.

## Agenda

The agenda for the meeting was as follows:

Opening remarks and review of meeting logistics and communication practices

ISO code of conduct reminder

IPR policy reminder and declarations

Contribution document allocation

Review of results of the previous meeting

Reports of ad hoc group (AHG) activities

Consideration of contributions on high-level syntax

Consideration of contributions and communications on project guidance

Consideration of video coding technology contributions

Consideration of information contributions

Coordination of visual quality testing

Coordination activities with other working groups

Approval of output documents and associated editing periods

Future planning: Determination of next steps, discussion of working methods, communication practices, establishment of coordinated experiments (if any), establishment of AHGs, meeting planning, other planning issues

Other business as appropriate for consideration

- The plans for the times of meeting sessions were established as follows, in UTC (2 hours behind the time in Geneva, Paris; 7 hours ahead of the time in Los Angeles, etc.). No session should last longer than 2 hrs.

0500-0700 1st “morning” session [break after 2 hours]

0720-0920 2nd “morning” session

[“overday” break – nearly 10 hours]

1900-2100 1st “evening” session [break after 2 hours]

2120-2320 2nd “evening” session

  - ISO Code of Conduct reminder

Participants were reminded of the ISO Code of Conduct, found at

https://www.iso.org/publication/PUB100397.html.

This includes points relating to:

- Respecting others

- Behaving ethically

- Escalating and resolving disputes

- Working for the net benefit of the international community

- Upholding consensus and governance

- Agreeing to a clear purpose and scope

- Participating actively and managing effective representation

## IPR policy reminder

Participants were reminded of the IPR policy established by the parent organizations of the JVET and were referred to the parent body websites for further information. The IPR policy was summarized for the participants.

The ITU-T/ITU-R/ISO/IEC common patent policy shall apply. Participants were particularly reminded that contributions proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

This obligation is supplemental to, and does not replace, any existing obligations of parties to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC.

Participants were also reminded of the need to formally report patent rights to the top-level parent bodies (using the common reporting form found on the database listed below) and to make verbal and/or document IPR reports within the JVET necessary in the event that they are aware of unreported patents that are essential to implementation of a standard or of a draft standard under development.

Some relevant links for organizational and IPR policy information are provided below:

- http://www.itu.int/ITU-T/ipr/index.html (common patent policy for ITU-T, ITU-R, ISO, and IEC, and guidelines and forms for formal reporting to the parent bodies)

- http://ftp3.itu.int/av-arch/jvet-site (JVET contribution templates)

- http://www.itu.int/ITU-T/dbase/patent/index.html (ITU-T IPR database)

- http://www.itscj.ipsj.or.jp/sc29/29w7proc.htm (JTC 1/‌SC 29 Procedures)

It is noted that the ITU TSB director’s AHG on IPR had issued a clarification of the IPR reporting process for ITU-T standards, as follows, per SG 16 TD 327 (GEN/16):

“TSB has reported to the TSB Director’s IPR Ad Hoc Group that they are receiving Patent Statement and Licensing Declaration forms regarding technology submitted in Contributions that may not yet be incorporated in a draft new or revised Recommendation. The IPR Ad Hoc Group observes that, while disclosure of patent information is strongly encouraged as early as possible, the premature submission of Patent Statement and Licensing Declaration forms is not an appropriate tool for such purpose.

In cases where a contributor wishes to disclose patents related to technology in Contributions, this can be done in the Contributions themselves, or informed verbally or otherwise in written form to the technical group (e.g. a Rapporteur’s group), disclosure which should then be duly noted in the meeting report for future reference and record keeping.

It should be noted that the TSB may not be able to meaningfully classify Patent Statement and Licensing Declaration forms for technology in Contributions, since sometimes there are no means to identify the exact work item to which the disclosure applies, or there is no way to ascertain whether the proposal in a Contribution would be adopted into a draft Recommendation.

Therefore, patent holders should submit the Patent Statement and Licensing Declaration form at the time the patent holder believes that the patent is essential to the implementation of a draft or approved Recommendation.”

The responsible coordinators invited participants to make any necessary verbal reports of previously-unreported IPR in technology that might be considered as prospective candidate for inclusion in future standards, and opened the floor for such reports: No such verbal reports were made.

## Software copyright disclaimer header reminder

It was noted that the VTM software implementation package uses the same software copyright license header as the HEVC reference software, where the latter had been agreed at the 5th meeting of the JCT-VC and approved by both parent bodies at their collocated meetings at that time. This license header language is based on the BSD license with a preceding sentence declaring that other contributor or third party rights, including patent rights, are not granted by the license, as recorded in N 10791 of the 89th meeting of ISO/IEC JTC 1/‌SC 29/‌WG 11. Both ITU and ISO/IEC will be identified in the <OWNER> and <ORGANIZATION> tags in the header. This software is used in the process of designing the VTM software, and for evaluating proposals for technology to be potentially included in the design. This software or parts thereof might be published by ITU-T and ISO/IEC as an example implementation of a future video coding standard and for use as the basis of products to promote adoption of such technology.

Different copyright statements shall not be committed to the committee software repository (in the absence of subsequent review and approval of any such actions). As noted previously, it must be further understood that any initially-adopted such copyright header statement language could further change in response to new information and guidance on the subject in the future.

These considerations apply to the 360Lib video conversion software and HDRTools as well.

## Communication practices

The documents for the meeting can be found at http://phenix.int-evry.fr/jvet/.

It was reminded to send a notice to the chairs in cases of changes to document titles, authors etc.

JVET email lists are managed through the site https://lists.rwth-aachen.de/postorius/lists/jvet.lists.rwth-aachen.de/, and to send email to the reflector, the email address is jvet@lists.rwth-aachen.de. Only members of the reflector can send email to the list. However, membership of the reflector is not limited to qualified JVET participants.

It was emphasized that reflector subscriptions and email sent to the reflector must use real names when subscribing and sending messages and subscribers must respond to inquiries regarding the nature of their interest in the work. The current number of subscribers on the JVET email list was 1245.

+JCT-VC (1294). Future discussions should be on the JVET reflector rather than the JCT-VC reflector (or JVT reflector).

For distribution of test sequences, a password-protected ftp site had been set up at RWTH Aachen University, with a mirror site at FhG-HHI. Accredited members of JVET may contact the responsible JVET coordinators to obtain the password information (but the site is not open for use by others).

## Terminology

- ACT: Adaptive colour transform

- AFF: Adaptive frame-field

- AI: All-intra

- AIF: Adaptive interpolation filtering

- ALF: Adaptive loop filter

- AMP: Asymmetric motion partitioning – a motion prediction partitioning for which the sub-regions of a region are not equal in size (in HEVC, being N/2x2N and 3N/2x2N or 2NxN/2 and 2Nx3N/2 with 2N equal to 16 or 32 for the luma component)

- AMVP: Adaptive motion vector prediction

- AMT or MTS: Adaptive multi-core transform, or multiple transform selection

- AMVR: (Locally) adaptive motion vector resolution

- APS: Adaptation parameter set

- ARC: Adaptive resolution conversion (synonymous with DRC, and a form of RPR)

- ARSS: Adaptive reference sample smoothing

- ATMVP or “subblock-based temporal merging candidates”: Alternative temporal motion vector prediction

- AU: Access unit

- AUD: Access unit delimiter.

- AVC: Advanced video coding – the video coding standard formally published as ITU-T Recommendation H.264 and ISO/IEC 14496-10.

- BA: Block adaptive.

- BC: See CPR or IBC.

- BCW: Biprediction with CU based weighting

- BD: Bjøntegaard-delta – a method for measuring percentage bit rate savings at equal PSNR or decibels of PSNR benefit at equal bit rate (e.g., as described in document VCEG-M33 of April 2001).

- BDOF: Bi-directional optical flow (formerly known as BIO).

- BDPCM: Block-wise DPCM.

- BL: Base layer.

- BMS: Benchmark set (no longer used), a former preliminary compilation of coding tools on top of VTM, which provide somewhat better compression performance, but are not deemed mature for standardzation.

- BoG: Break-out group.

- BR: Bit rate.

- BT: Binary tree.

- BV: Block vector (used for intra BC prediction).

- CABAC: Context-adaptive binary arithmetic coding.

- CBF: Coded block flag(s).

- CC: May refer to context-coded, common (test) conditions, or cross-component.

- CCALF: Cross-component ALF.

- CCLM: Cross-component linear model.

- CCP: Cross-component prediction.

- CE: Core Experiment – a coordinated experiment conducted toward assessment of coding technology.

- CG: Coefficient group.

- CGS: Colour gamut scalability (historically, coarse-grained scalability).

- CIIP: Combined inter/intra prediction.

- CL-RAS: Cross-layer random-access skip.

- CPB: Coded picture buffer.

- CPMV: Control-point motion vector.

- CPMVP: Control-point motion vector prediction (used in affine motion model).

- CPR: Current-picture referencing, also known as IBC – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- CST: Chroma separate tree.

- CTC: Common test conditions.

- CVS: Coded video sequence.

- DCT: Discrete cosine transform (sometimes used loosely to refer to other transforms with conceptually similar characteristics).

- DCTIF: DCT-derived interpolation filter.

- DF: Deblocking filter.

- DMVR: Decoder-side motion vector refinement.

- DoCR: Disposition of comments report.

- DPB: Decoded picture buffer.

- DPCM: Differential pulse-code modulation.

- DPS: Decoding parameter sets.

- DRC: Dynamic resolution conversion (synonymous with ARC, and a form of RPR).

- DT: Decoding time.

- DQ: Dependent quantization.

- ECS: Entropy coding synchronization (typically synonymous with WPP).

- EMT: Explicit multiple-core transform.

- EOTF: Electro-optical transfer function – a function that converts a representation value to a quantity of output light (e.g., light emitted by a display.

- EPB: Emulation prevention byte (as in the emulation_prevention_byte syntax element).

- ECV: Extended Colour Volume (up to WCG).

- EL: Enhancement layer.

- EOS: End of (coded video) sequence.

- ET: Encoding time.

- FRUC: Frame rate up conversion (pattern matched motion vector derivation).

- GCI: General constraints information.

- GDR: Gradual decoding refresh.

- GOP: Group of pictures (somewhat ambiguous).

- GPM: Geometry partitioning mode

- GRA: Gradual random access

- HDR: High dynamic range.

- HEVC: High Efficiency Video Coding – the video coding standard developed and extended by the JCT-VC, formalized by ITU-T as Rec. ITU-T H.265 and by ISO/IEC as ISO/IEC 23008-2.

- HLS: High-level syntax.

- HM: HEVC Test Model – a video coding design containing selected coding tools that constitutes our draft standard design – now also used especially in reference to the (non-normative) encoder algorithms (see WD and TM).

- HMVP: History based motion vector prediction.

- HRD: Hypothetical reference decoder.

- HyGT: Hyper-cube Givens transform (a type of NSST).

- IBC (also Intra BC): Intra block copy, also known as CPR – a technique by which sample values are predicted from other samples in the same picture by means of a displacement vector called a block vector, in a manner conceptually similar to motion-compensated prediction.

- IBDI: Internal bit-depth increase – a technique by which lower bit-depth (8 bits per sample) source video is encoded using higher bit-depth signal processing, ordinarily including higher bit-depth reference picture storage (ordinarily 12 bits per sample).

- IBF: Intra boundary filtering.

- ILP: Inter-layer prediction (in scalable coding).

- ILRP: Inter-layer reference picture.

- IPCM: Intra pulse-code modulation (similar in spirit to IPCM in AVC and HEVC).

- IRAP: Intra random access picture.

- ISP: Intra subblock partitioning

- JCCR: Joint coding of chroma residuals

- JEM: Joint exploration model – the software codebase for future video coding exploration.

- JM: Joint model – the primary software codebase that has been developed for the AVC standard.

- JSVM: Joint scalable video model – another software codebase that has been developed for the AVC standard, which includes support for scalable video coding extensions.

- KLT: Karhunen-Loève transform.

- LB or LDB: Low-delay B – the variant of the LD conditions that uses B pictures.

- LD: Low delay – one of two sets of coding conditions designed to enable interactive real-time communication, with less emphasis on ease of random access (contrast with RA). Typically refers to LB, although also applies to LP.

- LFNST: Low-frequency non-separable transform

- LIC: Local illumination compensation.

- LM: Linear model.

- LMCS: Luma mapping with chroma scaling (formerly sometimes called “in-loop reshaping”)

- LP or LDP: Low-delay P – the variant of the LD conditions that uses P frames.

- LUT: Look-up table.

- LTRP: Long-term reference picture.

- MC: Motion compensation.

- MCP: Motion compensated prediction.

- MDNSST: Mode dependent non-separable secondary transform.

- MIP: Matrix-based intra prediction

- MMLM: Multi-model (cross component) linear mode.

- MMVD: Merge with MVD.

- MPEG: Moving picture experts group (WG 11, the parent body working group in ISO/IEC JTC 1/‌SC 29, one of the two parent bodies of the JVET).

- MPM: Most probable mode (in intra prediction).

- MRL: Multiple reference line intra prediction.

- MV: Motion vector.

- MVD: Motion vector difference.

- NAL: Network abstraction layer.

- NSQT: Non-square quadtree.

- NSST: Non-separable secondary transform.

- NUH: NAL unit header.

- NUT: NAL unit type (as in AVC and HEVC).

- OBMC: Overlapped block motion compensation (e.g., as in H.263 Annex F).

- OETF: Opto-electronic transfer function – a function that converts to input light (e.g., light input to a camera) to a representation value.

- OLS: Output layer set.

- OOTF: Optical-to-optical transfer function – a function that converts input light (e.g. l,ight input to a camera) to output light (e.g., light emitted by a display).

- operation point: A temporal subset of an OLS.

- PDPC: Position-dependent (intra) prediction combination.

- PERP: Padded equirectangular projection (a 360° projection format).

- PH: Picture header.

- PHEC: Padded hybrid equiangular cubemap (a 360° projection format).

- PMMVD: Pattern-matched motion vector derivation.

- POC: Picture order count.

- PoR: Plan of record.

- PROF: Prediction refinement with optical flow

- PPS: Picture parameter set (as in AVC and HEVC).

- PTL: Profile/tier/level combination.

- QM: Quantization matrix (as in AVC and HEVC).

- QP: Quantization parameter (as in AVC and HEVC, sometimes confused with quantization step size).

- QT: Quadtree.

- RA: Random access – a set of coding conditions designed to enable relatively-frequent random access points in the coded video data, with less emphasis on minimization of delay (contrast with LD).

- RADL: Random-access decodable leading (type of picture).

- RASL: Random-access skipped leading (type of picture).

- R-D: Rate-distortion.

- RDO: Rate-distortion optimization.

- RDOQ: Rate-distortion optimized quantization.

- RDPCM: Residual DPCM

- ROT: Rotation operation for low-frequency transform coefficients.

- RPL: Reference picture list.

- RPLM: Reference picture list modification.

- RPR: Reference picture resampling (e.g., as in H.263 Annex P), a special case of which is also known as ARC or DRC.

- RPS: Reference picture set.

- RQT: Residual quadtree.

- RRU: Reduced-resolution update (e.g. as in H.263 Annex Q).

- RVM: Rate variation measure.

- SAO: Sample-adaptive offset.

- SBT: Subblock transform.

- SbTMVP: Subblock based temporal motion vector prediction.

- SCIPU: Smallest chroma intra prediction unit.

- SD: Slice data; alternatively, standard-definition.

- SDH: Sign data hiding.

- SDT: Signal-dependent transform.

- SE: Syntax element.

- SEI: Supplemental enhancement information (as in AVC and HEVC).

- SH: Slice header.

- SHM: Scalable HM.

- SHVC: Scalable high efficiency video coding.

- SIF: Switchable (motion) interpolation filter.

- SIMD: Single instruction, multiple data.

- SMVD: Symmetric MVD.

- SPS: Sequence parameter set (as in AVC and HEVC).

- STMVP: Spatial-temporal motion vector prediction.

- STRP: Short-term reference picture.

- STSA: Step-wise temporal sublayer access.

- TBA/TBD/TBP: To be announced/determined/presented.

- TGM: Text and graphics with motion – a category of content that primarily contains rendered text and graphics with motion, mixed with a relatively small amount of camera-captured content.

- TMVP: Temporal motion vector prediction.

- TS: Transform skip.

- TSRC: Transform skip residual coding.

- TT: Ternary tree.

- UCBDS: Unrestricted center-biased diamond search.

- UWP: Unequal weight prediction.

- VCEG: Visual coding experts group (ITU-T Q.6/16, the relevant rapporteur group in ITU-T WP3/16, which is one of the two parent bodies of the JVET).

- VPS: Video parameter set – a parameter set that describes the overall characteristics of a coded video sequence – conceptually sitting above the SPS in the syntax hierarchy.

- VTM: VVC Test Model.

- VUI: Video usability information.

- VVC: Versatile Video Coding, the standardization project developed by JVET.

- WAIP: Wide-angle intra prediction

- WCG: Wide colour gamut.

- WG: Working group, a group of technical experts (usually used to refer to WG 11, a.k.a. MPEG).

- WPP: Wavefront parallel processing (usually synonymous with ECS).

- Block and unit names in HEVC:

  - CTB: Coding tree block (luma or chroma) – unless the format is monochrome, there are three CTBs per CTU.

  - CTU: Coding tree unit (containing both luma and chroma, synonymous with LCU), with a size of 16x16, 32x32, or 64x64 for the luma component.

  - CB: Coding block (luma or chroma), a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma), the level at which the prediction mode, such as intra versus inter, is determined in HEVC, with a size of 2Nx2N for 2N equal to 8, 16, 32, or 64 for luma.

  - PB: Prediction block (luma or chroma), a luma or chroma block of a PU, the level at which the prediction information is conveyed or the level at which the prediction process is performed in HEVC.

  - PU: Prediction unit (containing both luma and chroma), the level of the prediction control syntax within a CU, with eight shape possibilities in HEVC:

    - 2Nx2N: Having the full width and height of the CU.

    - 2NxN (or Nx2N): Having two areas that each have the full width and half the height of the CU (or having two areas that each have half the width and the full height of the CU).

    - NxN: Having four areas that each have half the width and half the height of the CU, with N equal to 4, 8, 16, or 32 for intra-predicted luma and N equal to 8, 16, or 32 for inter-predicted luma – a case only used when 2N×2N is the minimum CU size.

    - N/2x2N paired with 3N/2x2N or 2NxN/2 paired with 2Nx3N/2: Having two areas that are different in size – cases referred to as AMP, with 2N equal to 16 or 32 for the luma component.

  - TB: Transform block (luma or chroma), a luma or chroma block of a TU, with a size of 4x4, 8x8, 16x16, or 32x32.

  - TU: Transform unit (containing both luma and chroma), the level of the residual transform (or transform skip or palette coding) segmentation within a CU (which, when using inter prediction in HEVC, may sometimes span across multiple PU regions).

- Block and unit names in VVC:

  - CTB: Coding tree block (luma or chroma) – there are three CTBs per CTU in a P or B slice or in an I slice that uses a single tree, and one CTB per luma CTU and two CTBs per chroma CTU in an I slice that uses separate trees.

  - CTU: Coding tree unit (synonymous with LCU, containing both luma and chroma in a P or B slice or in an I slice that uses a single tree, containing only luma or only chroma in an I slice that uses separate trees), with a size of 16x16, 32x32, 64x64, or 128x128 for the luma component.

  - CB: Coding block, a luma or chroma block in a CU.

  - CU: Coding unit (containing both luma and chroma in P/B slice, containing only luma or chroma in I slice), a leaf node of a QTBT. It’s the level at which the prediction process and residual transform are performed in JEM. A CU can be square or rectangle shape.

  - PB: Prediction block, a luma or chroma block of a PU.

  - PU: Prediction unit, has the same size as a CU in the VVC context.

  - TB: Transform block, a luma or chroma block of a TU.

  - TU: Transform unit, has the same size as a CU in the VVC context.

## Opening remarks

Remarks during the opening session of the meeting Wednesday 7 October at 0500 UTC (chaired by GJS and JRO) were as follows.

- Merger of JVET

- Timing and organization of online meetings, calendar

- VVC/VSEI approval and publication status: 
"H.266" | ISO/IEC 23090-3 for VVC and H.274 | ISO/IEC 23002-7 for VSEI

  - H.266 and H.274 pre-published 2020-09-09

- WPOM approval and publication status

- AVC, HEVC and CICP status

- The meeting logistics, agenda, working practices, policies, and document allocation were reviewed.

  - The meeting is conducted using Zoom

  - Having text and software available is crucial (and not just arriving at the end of the meeting).

  - There were no objections voiced in the opening plenary to the consideration of late contributions.

- The results of the previous meeting and the meeting report were reviewed.

- There was somewhat less of a problem of late non-cross-check documents and no “placeholders” –  (see section 2.4.2).

- The primary goals of the meeting were

  - Errata

  - Conformance and software for VVC & VSEI

    - Requests for these were already approved in ISO/IEC, targeting CD at this meeting

  - Verification test planning

  - Potential extensions of VVC

    - Neural network tools

    - High bit rate / high bit depth

  - Potential additional SEI messages for VSEI

- Less documents than recently, not necessary to conduct sessions in parallel

- Scheduling was discussed

- Principles of standards development were discussed.

## Scheduling of discussions

- The plans for the times of meeting sessions were established as follows, in UTC (2 hours behind the time in Geneva, Paris; 7 hours ahead of the time in Los Angeles, etc.). No session should last longer than 2 hrs.

0500-0700 1st “morning” session [break after 2 hours]

0720-0920 2nd “afternoon” session

[“overday” break – nearly 10 hours]

1900-2100 1st “evening” session [break after 2 hours]

2120-2320 2nd “evening” session

All sessions were announced via the calendar in the JVET document site at least 22 hrs. in advance. Particular scheduling notes are shown below, although not necessarily 100% accurate or complete:

- Wed. 7 October, 1st day

  - Session 1:

    - 0500–0550 Opening remarks, review of practices, agenda, IPR reminder

    - 0550–0700 Reports of AHGs 1–3

  - Session 2

    - 0720–0920 Reports of AHGs 4–9

  - Session 3

    - 1900–2100 Errata

  - Session 4

    - 2120–2320 SEI messages

- …

## Contribution topic overview

The approximate subject categories and quantity of contributions per category for the meeting were summarized as follows (note that the noted document counts do not include crosschecks, and may not be completely accurate):

- AHG reports (13 JVET + 5 JCT-VC) (section 3)

- Project development (section 4)

  - General (0)

  - Text development and errata reporting (6)

  - Test conditions (1)

  - Verification test (7)

  - Coding studies and tools on specific use cases (0)

  - Test Material (2)

  - Conformance development (1)

  - Software development (3)

  - Implementation studies (3)

  - Complexity analysis (2)

  - Encoder optimization (2)

  - Profile/tier/level specification (1)

- Low-level tool technology proposals (section 5) with subtopics

  - AHG12: High bit depth coding (10) (section 5.1)

  - AHG10: Neural-network-based technology (11) (section 5.2)

  - Other (1) (section 5.3)

- High-level syntax (HLS) proposals (section 6) with subtopics

  - AHG8: Layered coding and resolution adaptation (2) (section 6.1)

  - AHG9: SEI message studies and proposals (14) (section 6.2)

  - Other (1) (section 6.3)

- Withdrawn (1) (section 7)

- Joint meetings, plenary discussions, BoG reports, Summary of actions (section 8)

- Project planning (section 9)

- Establishment of AHGs (section 10)

- Output documents (section 11)

- Future meeting plans and concluding remarks (section 12)

The document counts above do not include cross-checks and summary reports.

# AHG reports (13+5)

These reports were discussed Wednesday 7 October 2020 during 0550-0700 and 0720–0920 UTC (chaired by GJS & JRO), except as otherwise noted.

JVET-T0001 JVET AHG report: Project management (AHG1) [J.-R. Ohm, G. J. Sullivan]

This document reports on the work of the JVET ad hoc group on Project Management, including an overall status report on the VVC standardization project and the progress made during the interim period since the preceding meeting.

The work of the JVET overall had proceeded well in the interim period with a moderate number of input documents submitted to the current meeting. Intense discussion had been carried out on the group email reflector, and all output documents from the preceding meeting had been produced.

Output documents from the preceding meeting had been made available at the "Phenix" site (http://phenix.int-evry.fr/jvet/) or the ITU-based JVET site (http://wftp3.itu.int/av-arch/jvet-site/2020_06_S_Virtual/), particularly including the following:

- The meeting report (JVET-S2000) [Posted 2020-10-07]

- Versatile Video Coding (Draft 10) (JVET-S2001) [Posted 2020-06-23, last update 2020-09-04]

- Algorithm description for Versatile Video Coding and Test Model 10 (VTM 10) (JVET-S2002) [Posted 2020-08-12]

- Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 11) (JVET-S2004) [Posted 2020-10-02]

- Methodology and reporting template for coding tool testing (JVET-S2005) [Posted 2020-07-20]

- Supplemental enhancement information messages for coded video bitstreams (Draft 5) (JVET-S2007) [Posted 2020-06-29, last update 2020-09-04]

- Conformance testing for Versatile Video Coding (Draft 4) (JVET-S2008) [Posted 2020-10-06]

- Verification test plan (Draft 3) (JVET-S2009) [Posted 2020-07-11, last update 2020-07-31]

- JVET common test conditions and evaluation procedures for HDR/WCG video (JVET-S2011) [Posted 2020-07-23]

- Working practices using objective metrics for evaluation of video coding efficiency experiments (Draft 3) (JVET-S2016) [Posted 2020-07-10]

- Technologies under consideration for VSEI (JVET-S2017) [Posted 2020-08-05]

As a major achievement, JVET-S2001 was submitted to the parent bodies for publication as ITU-T Rec. H.266, and ISO/IEC FDIS 23090-3. Likewise, JVET-S2007 was submitted for publication as ITU-T Rec. H.-274, and ISO/IEC FDIS 23002-7.

The thirteen ad hoc groups had made progress, and reports from those activities had been submitted.

Due to issues associated with the COVID-19 pandemic, a conversion of the meeting to be conducted only online was necessitated.

During the interim period, meetings of AHG4 (for preparing the verification tests) and AHG11 (for discussing aspect of neural-network-based video coding) were held.

Software integration of VTM was finalized approximately according to the plan. Significant activities were also conducted on preparation of verification tests, and on development of VVC conformance testing.

Various problem reports relating to asserted bugs in the software, draft specification text, and reference encoder description had been submitted to an informal "bug tracking" system. That system is not intended as a replacement of our ordinary contribution submission process. However, the bug tracking system was considered to have been helpful to the software coordinators and text editors. The bug tracker reports had been automatically forwarded to the group email reflector, where the issues were discussed – and this is reported to have been helpful.

Roughly 60 input contributions to the current meeting (not counting the AHG summary reports and crosschecks) had been registered for consideration at the meeting. No CEs had been conducted.

It is further noted that, starting from the twentieth JVET meeting, work items which had originally been conducted by the Joint Collaborative Team on Video Coding (JCT-VC) were continued to be conducted in JVET as a single joint team, as negotiated by the parent bodies. This particularly consists of work on

- High Efficiency Video Coding (HEVC) and its extensions, the development of associated conformance test sets, reference software, verification testing, and non-normative guidance information,

- Specification of Coding-independent Code Points (Video) (CICP), and associated technical reports,

- Maintenance and minor enhancement work on the Advanced Video Coding (AVC) standard, associated conformance test sets and reference software.

Consequently, reports of the 5 AHGs which had been established at the 40th JCT-VC meeting were also submitted to JVET.

A preliminary basis for the document subject allocation and meeting notes for the 20th meeting had been made publicly available on the ITU-hosted ftp site http://wftp3.itu.int/av-arch/jvet-site/2020_10_T_Virtual/.

JVET-T0002 JVET AHG report: Draft text and test model algorithm description editing (AHG2) [B. Bross, J. Chen, J. Boyce, S. Kim, S. Liu, Y.-K. Wang, Y. Ye]

This document reports the work of the JVET ad hoc group on draft text and test model algorithm description editing (AHG2) between the 19th meeting by teleconference, (22 June – 1 July 2020) and the 20th meeting by teleconference, (7–16 October 2020).

At the 19th JVET meeting, it was decided to simplify, clean up and fix the existing features for intra picture-prediction, inter-picture prediction, transform, CABAC engine, in-loop filter, and high-level syntax functionalities in the tenth draft of Versatile Video Coding (VVC D10) and the VVC Test Model 10 (VTM 10) encoding as well as the high-level syntax in the fifth draft of the VSEI text specification (VSEI D5).

The bitstream format, conformance, and normative decoding process for Versatile Video Coding are specified in the VVC draft 10 text specification document. The VVC Test Model 10 (VTM 10) Algorithm and Encoder Description document provides an algorithm description as well as an encoder-side description of the VVC Test Model 10, which serves as a tutorial for the algorithm and encoding model implemented in the VTM10.x software. Video usability information parameters and some supplemental enhancement information messages for coded video bitstreams are specified in the VSEI draft 5 text.

An issue tracker (https://jvet.hhi.fraunhofer.de/trac/vvc) was used to facilitate the reporting of errata with the VVC documents.

JVET-S2001 VVC text specification (Draft 10)

Seventeen versions of JVET-S2001 were published by the Editing AHG between the 19th meeting by teleconference, (22 June – 1 July 2020) and the 20th meeting by teleconference, (7–16 October 2020).

JVET-S2002 VVC test model 10 (VTM 10) algorithm and encoder description

One version of JVET-S2002 was published by the Editing AHG between the 19th meeting by teleconference, (22 June – 1 July 2020) and the 20th meeting by teleconference, (7–16 October 2020).

JVET-S2002 has been established based on JVET-R2002. It provides the algorithm description for majority of coding tools in VVC. In this editing period, the overall description has been improved.

JVET-S2007 VSEI text specification (Draft 5)

Seven versions of JVET-S2007 were published by the Editing AHG between the 19th meeting by teleconference, (22 June – 1 July 2020) and the 20th meeting by teleconference, (7–16 October 2020).

Related contributions

- Y.-K. Wang (Bytedance), G. J. Sullivan (Microsoft), “On the syntax design for extending SEI messages and VUI for HEVC, VVC and VSEI,” document JVET-T0048, 20th JVET meeting: by teleconference, 7 – 16 October 2020.

- Y.-K. Wang, Z. Deng (Bytedance), “Some errata items for VVC,” document JVET-T0055, 20th JVET meeting: by teleconference, 7–16 October 2020.

The AHG recommended to:

- Approve the edited JVET-S2001, JVET-S2002 and JVET-S2007 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review AHG2 related contributions and act on them if found to be necessary.

It was commented that there are still some open issues in the bug tracker.

The pre-published ITU-T text and the FDIS input and the JVET output were reported to be aligned.

Additional fixes (roughly 15-20 tickets and some obvious pure typos) had been prepared by the editors with the intent to reflect them in the final publications of ITU-T and ISO/IEC.

An input document reflecting these was requested.

It was remarked that Ticket #1372 was particularly important and should be reviewed. It was commented that the software has the correct behaviour; this is just a matter of making sure the text is correct.

JVET-T0003 JVET AHG report: Test model software development (AHG3) [F. Bossen, X. Li, K. Sühring]

This report summarizes the activities of the AhG3 on Test model software development that has taken place between the 19th and 20th JVET meetings.

VTM software development

Development was continued on the GitLab server, which allows participants to register accounts and use a distributed development workflow based on git.

The server is located at:

https://vcgit.hhi.fraunhofer.de

The registration and development workflow is documented at:

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/wikis/VVC-Software-Development-Workflow

The VTM software can be found at

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/

VTM 9.2 was tagged on Jul. 6, 2020.

VTM 9.3 was tagged on Jul.6, 2020. Relative to VTM 9.2 only macros of the previous cycle are removed.

VTM 10.0 was tagged Aug. 13, 2020. Changes include:

VTM 10.1 release candidate 1 was tagged on Oct. 6, 2020. It is expected that version 10.1 will be tagged shortly thereafter.

CTC Performance

The following tables show VTM 10.0 performance over HM 16.22:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over HM 16.22 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -29,04% | -32,17% | -34,07% | 1591% | 166% |
| Class A2 | -29,29% | -23,92% | -21,06% | 2567% | 172% |
| Class B | -21,73% | -26,96% | -30,76% | 2809% | 172% |
| Class C | -22,54% | -18,95% | -22,70% | 3951% | 188% |
| Class E | -25,76% | -25,91% | -24,46% | 2318% | 165% |
| Overall | -25,06% | -25,37% | -26,85% | 2630% | 173% |
| Class D | -18,47% | -13,31% | -13,42% | 4447% | 180% |
| Class F | -39,33% | -39,73% | -42,22% | 5088% | 174% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over HM 16.22 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -38,76% | -37,00% | -44,06% | 739% | 155% |
| Class A2 | -43,20% | -39,82% | -38,41% | 833% | 172% |
| Class B | -35,02% | -46,82% | -44,61% | 803% | 153% |
| Class C | -29,98% | -30,67% | -32,62% | 1070% | 153% |
| Class E |  |  |  |  |  |
| Overall | -36,06% | -39,15% | -40,06% | 859% | 157% |
| Class D | -27,88% | -26,68% | -26,12% | 1175% | 165% |
| Class F | -41,52% | -44,62% | -45,73% | 607% | 133% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over HM 16.22 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -30,81% | -37,35% | -35,42% | 738% | 142% |
| Class C | -29,13% | -22,47% | -22,45% | 890% | 151% |
| Class E | -33,36% | -40,20% | -34,18% | 364% | 120% |
| Overall | -30,89% | -33,11% | -30,79% | 658% | 139% |
| Class D | -26,02% | -16,68% | -15,88% | 917% | 152% |
| Class F | -42,80% | -44,45% | -44,71% | 498% | 125% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over HM 16.22 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -35,15% | -39,91% | -37,83% | 690% | 158% |
| Class C | -30,83% | -22,55% | -22,66% | 818% | 158% |
| Class E | -36,05% | -43,41% | -37,33% | 365% | 128% |
| Overall | -33,93% | -35,00% | -32,65% | 623% | 150% |
| Class D | -27,47% | -15,71% | -14,93% | 856% | 160% |
| Class F | -42,31% | -43,56% | -44,09% | 536% | 133% |

The following tables show VTM 10.0 performance compared to VTM 9.0:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,01% | 0,00% | 102% | 101% |
| Class A2 | -0,01% | 0,03% | -0,01% | 102% | 99% |
| Class B | -0,01% | -0,05% | -0,04% | 101% | 98% |
| Class C | -0,02% | 0,01% | 0,01% | 101% | 98% |
| Class E | -0,03% | 0,05% | -0,01% | 103% | 99% |
| Overall | -0,01% | 0,00% | -0,01% | 101% | 99% |
| Class D | -0,06% | -0,03% | -0,08% | 100% | 102% |
| Class F | -0,03% | -0,04% | -0,02% | 101% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | -0,01% | -0,01% | 102% | 101% |
| Class A2 | -0,01% | -0,01% | -0,01% | 102% | 99% |
| Class B | -0,03% | -0,04% | -0,03% | 100% | 98% |
| Class C | -0,04% | -0,04% | -0,04% | 102% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0,03% | -0,03% | -0,03% | 101% | 98% |
| Class D | -0,12% | -0,12% | -0,12% | 99% | 98% |
| Class F | -0,07% | -0,07% | -0,07% | 102% | 96% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,03% | -0,03% | -0,03% | 100% | 98% |
| Class C | -0,02% | -0,02% | -0,02% | 99% | 96% |
| Class E | -0,46% | -0,47% | -0,42% | 100% | 96% |
| Overall | -0,13% | -0,14% | -0,12% | 99% | 97% |
| Class D | -0,06% | -0,05% | -0,05% | 99% | 94% |
| Class F | -0,20% | -0,20% | -0,20% | 100% | 96% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM 9.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,03% | -0,03% | -0,03% | 98% | 97% |
| Class C | -0,02% | -0,01% | -0,01% | 98% | 92% |
| Class E | -0,44% | -0,44% | -0,40% | 101% | 95% |
| Overall | -0,13% | -0,13% | -0,12% | 99% | 95% |
| Class D | -0,05% | -0,04% | -0,04% | 99% | 94% |
| Class F | -0,20% | -0,20% | -0,19% | 101% | 98% |

Full results were attached to the AHG report as Excel files.

Issues in VTM 10.0 affecting conformance

The following issues in VTM 10.0 affect conformance:

- A bug related to the combination for deblocking, 4:2:2/4:4:4 chroma formats and ISP (ticket #1353)

- Incorrect output order of pictures (ticket #1396). In this context it should be noted that the Output Picture Log produced by VTM 10.0 doesn’t include layer ids making such issues more difficult to identify and debug.

- Missing HLS features

These issues have been partially resolved in VTM 10.1rc1. In particular the OPL format was changed to include layers ids. Some HLS features are still missing (see sections below) and there are several tickets related to multilayer coding that are still open.

Status of implementation of proposals of previous JVET meetings

Actions of the Q, R, and S meetings that had not yet been marked as resolved were listed in the AHG report.

The bug tracker for VTM and specification text is located at:

https://jvet.hhi.fraunhofer.de/trac/vvc

The bug tracker uses the same accounts as the HM software bug tracker. Users may need to log in again due to the different sub-domain. For spam fighting reasons account registration is only possible at the HM software bug tracker at

https://hevc.hhi.fraunhofer.de/trac/hevc

Please file all issues related to the VVC reference software into the bug tracker. Try to provide all the details, which are necessary to reproduce the issue. Patches for solving issues and improving the software are always appreciated.

The AHG recommended to:

- Continue to develop the VTM reference software

- Improve documentation, especially the software manual

- Resolve any normative issues resulting from the large number of integrations in the most recent development cycle

- Encourage people to test VTM software more extensively outside of common test conditions.

- Encourage people to report all (potential) bugs that they are finding.

- Encourage people to submit bitstreams/test cases that trigger bugs in VTM.

- Encourage people to submit non-normative changes that reduce encoder run time without significantly sacrificing compression performance

- Design and add configuration files to the VTM software for testing of HLS features

- Review VTM-related contributions and determine whether features should be added (or removed) from the software

It was commented that we need people to check the issues that are marked as open to confirm whether any action is really needed and to make the software coordinators aware of the result. The software coordinators also emphasized the need for those who have contributed patches to keep an eye out for email and make sure the issues of its integration have been fully resolved.

End of session #1.

JVET-T0004 JVET AHG report: Test material and visual assessment (AHG4) [V. Baroncini, T. Suzuki, M. Wien, A. Norkin, A. Segall, Y. Ye]

This report summarizes the activities of the AhG4 on Test material and visual assessment that has taken place between the 18th and 19th JVET meetings. Two online AHG meetings were held related to the preparation of the VVC verification tests in the SDR HD and 360° video categories. The inputs to these AHG meetings and the AHG meeting reports are provided in the input documents JVET-T0043, JVET-T0044, JVET-T0045, JVET-T0046, respectively. The results of the visual tests in the SDR UHD category of the VVC verification tests are reported in input document JVET-T0097.

  - Verification test

For the SDR UHD Random Access category, formal visual tests have been conducted at GBTech, Rome, IT, and RWTH Aachen University, Aachen, DE. The testing procedure was adapted to the restrictions due to the COVID-19 pandemic and therefore took more time than initially planned. The results are provided in input contribution JVET-T0097.

An AHG meeting for SDR HD was held on 2020-09-03, reviewing the outcome of a dedicated experts viewing session as reported in JVET-T0043. The AHG meeting report is provided in JVET-T0045. As a major outcome of this meeting, it was suggested to include both, SDR HD Random Access (RA) and SDR HD Low Delay (LD) configurations in the next update of the Verification Test Plan. The identification and collection of SDR HD test sequences for both categories is in progress.

An AHG meeting for 360° video was held on 2020-09-04, reviewing the outcome of an experts viewing session for rate point selection as reported in JVET-T0044. The AHG meeting report is provided in JVET-T0046. Potential improvements to the 360Lib software were identified for the projection formats GCMP and PCMP.

For the HDR category, the Zoom sessions were held between the verification test coordinators and A. Segall on display calibration and process planning. The identified display settings are collected in JVET-T0106. Due to the general delays for the pandemic, the effort of rate point selection has not been completed by the beginning of this meeting.

  - Test sequences

The test sequences used for CfP/CTC are available on ftp://jvet@ftp.ient.rwth-aachen.de in directory “/jvet-cfp” (accredited members of JVET may contact the JVET chairs for login information).

Due to copyright restrictions, the JVET database of test sequences is only available to accredited members of JVET (i.e. members of ISO/IEC MPEG and ITU-T VCEG).

Related contributions

The following related contributions were submitted.

- JVET-T0043 “Status Report on SDR HD Low Delay Verification Test Preparation” [M. Wien (RWTH), V. Baroncini (VABTech ltd)]

- JVET-T0044 “Status Report on 360 Video Verification Test Preparation” [M. Wien (RWTH), V. Baroncini (VABTech), T. Suzuki (Sony)]

- JVET-T0045 “AHG4: Agenda and report of the AHG meeting on the SDR HD Verification Tests on 2020-09-03” [M. Wien (RWTH), V. Baroncini (VABTech ltd)]

- JVET-T0046 “AHG4: Agenda and report of the AHG meeting on the 360 Video Verification Tests on 2020-09-04” [M. Wien (RWTH), V. Baroncini (VABTech ltd), Y. Ye (Alibaba)]

- JVET-T0060 “AHG4: new video conference sequences for HD verification testing” [S. Xu, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

- JVET-T0082 “New HLG sequences for high bit-depth video coding” [K. Kondo, M. Ikeda, A. Browne (Sony)]

- JVET-T0097 “Report on VVC compression performance verification testing in the SDR UHD Random Access Category” [M. Wien (RWTH), V. Baroncini (VABTech)]

- JVET-T0103 “Information on and analysis of the VVC encoders in the SDR UHD verification test” [C. Helmrich, B. Bross, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

- JVET-T0106 “AHG4/AHG7: Report on Display Settings for HDR/WCG Verification Tests” [A. Segall, M. Wien, V. Baroncini]

These contributions should be reviewed.

The AHG recommended:

- To review the input contributions related to the verification test preparation at an early stage of the meeting.

- To continue to discuss and to update the non-finalized categories of the verification test plan

- To create a BoG to progress the update of the verification test plan.

- To collect volunteers to conduct the verification test, including volunteers to encode.

- To review the set of available test sequences for the verification tests and potentially collect more test sequences with a variety of content.

- To continue to collect new test sequences available for JVET with licensing statement.

JVET-T0005 JVET AHG report: Conformance testing (AHG5) [J. Boyce, W. Wan, E. Alshina, F. Bossen, I. Moccagatta, K. Kawamura, K. Sühring, X. Xu]

This document summarizes the activity of AHG5: “Conformance testing” between the 19th Meeting (teleconference, 22 June – 1 July 2020) and the 20th Meeting (teleconference, 7–16 Oct. 2020).

Timeline

The progress on the Conformance testing specification is consistent with the preliminary timeline agreed at the 16th JVET meeting, as follows:

- 17th meeting Jan. 2020: Preliminary guidelines for bitstream preparation (e.g., naming conventions),

- improved list of conformance bitstreams

- 18th meeting Apr. 2020: Final guidelines for bitstream preparation and improved list of conformance

- bitstreams with identified responsible experts, initial bitstreams provided

- 19th meeting July 2020: Confirmed list of bitstreams to be included in v1, collection of bitstream

- candidates for CD ballot at next meeting

- 20th meeting Oct. 2020: CD of conformance specification

- 21st meeting Jan. 2021: Final bitstreams provided, DIS ballot in ISO/IEC

- 22nd meeting April 2021: No action pending DIS ballot

- 23rd meeting July 2021: Final conformance specification

Status on bitstream submission

The status at the time of preparation of this report is as follows:

- 102 bitstream categories have been identified

- Volunteers have been identified for all categories

- 206 bitstreams in 77 bitstream categories have been provided for VTM 10.0

- 25 bitstream categories have no VTM 10.0 bitstreams

- 12 bitstream categories have no provided bitstreams (for any VTM version)

Activities and Discussion

The AHG activities are on schedule with the preliminary timeline shown in section 2.  It is expected to output CD of the Conformance specification from this meeting.

Output document JVET-S2008 “Conformance testing for versatile video coding (Draft 4)” was published on 5 October 2020.

Because CD is planned to be issued from this October meeting, all bitstream volunteers had been requested to update their bitstreams using VTM 10.0. At least one bitstream has been provided in 77 of the 102 identified categories for VTM 10.0. Bitstream submitters were also requested to provide text descriptions of the bitstreams for inclusion in the conformance specification on a shared document, but relatively few have been updated.

A modification was made to the VTM master branch to include layer ID for each picture in the .opl files such as to distinguish between pictures within an access unit.  A new version of the VTM is expected to include the opl change, as well as other changes to multilayer coding, missing HLS features, and a bug fix to 4:2:2/4:4:4 deblocking. Bitstreams that have already been provided in those categories may need to be updated, or the submitters are waiting for the availability of the new VTM to provide initial bitstreams.

The change to the .opl format affects bitstream packages that have already been submitted. It is expected that bitstream packages can be updated to the new format using a script and individual packages do not need to be resubmitted.

It was suggested to add bitstreams with 9-bit depth.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de).

The AHG5 chairs and JVET chairs can be reached at jvet-conformance@lists.rwth-aachen.de. Participants should not subscribe to this list but may send emails to it.

There was one related contribution, discussing gaps in the existing conformance bitstream suite.

- JVET-T0100, AHG5: On gaps in conformance bitstreams [F. Bossen (Sharp)]

The procedure to exchange the bitstream (ftp cite, bitstream files, etc.) is specified in Sec 2 “Procedure” of JVET-R2008. The ftp and http sites for downloading bitstreams are

ftp://ftp3.itu.int/jvet-site/bitstream_exchange/VVC

https://www.itu.int/wftp3/av-arch/jvet-site/bitstream_exchange/VVC/

The ftp site for uploading bitstream file is as follows.

ftp://ftp3.itu.int/jvet-site/dropbox/

(user id: avguest, passwd: Avguest201007)

If using FileZilla, using the Site Manager with the following configuration is suggested:

The AHG recommends the following:

- Consider if should separate the draft Conformance specification into two output documents: the Conformance specification and instructions for generating conformance bitstreams, or should move the instructions to an Annex

- Encourage volunteers of missing conformance bitstreams to provide them quickly

- Encourage conformance bitstream providers to provide text descriptions of provided bitstreams for the Conformance specification online at http://mpeg.expert/live/nextcloud/index.php/f/37368

- Encourage conformance bitstream providers to provide updated bitstreams with corrected naming for those bitstreams indicated in the JVET-S2008 document as needing name changes to be consistent with the naming convention

- Discuss and refine the list of conformance bitstreams and the conformance specification, especially consider adding 9-bit depth and solicit volunteers

- Review submitted bitstreams and consider if the flexibility of the tested tool is sufficiently exercised, including consideration of input contribution JVET-T0100

- Proceed with CD issuance as output of this meeting

There was discussion of whether the conformance testing spec really needs to be normative, and whether the dataset really needs to be part of the approved standard, or could be something external to it that is referenced informatively. (The tests are far from complete in any case.)

It was asked whether to issue a TR rather than a standard (removing all “shall” and “should”).

This was initially agreed – with a long editing period – and a request to change to use a TR.

The same approach was discussed for the software.

The fact that we have called prior reference software and conformance test data normative causes a problem with basically normative dependencies on non-standard external packages. On the other hand, some users may like the normative nature as providing assurance of correctness.

It was noted that there was a software maintance problem previously for 3D-AVC, such that the living codebase was removed from the web.

There would be less naming convenience on the ITU side as H.266.1 and H.266.2.

Revisit.

JVET-T0006 JVET AHG report: 360° video coding, software and test conditions (AHG6) [J. Boyce, Y. He, K. Choi, J.-L. Lin, Y. Ye]

The document summarizes activities on 360-degree video content conversion software development between the 19th (22 June – 1 July 2020) and the 20th (7–16 Oct. 2020) JVET meetings.

Activities

The 360Lib-11.0 software package released on Aug. 28, 2020 included following changes:

- Support frame packing specified by VVC GCMP SEI messages (from JVET-S0257)

- Provide configuration files for those frame packings

Additionally, at the verification testing AHG meeting held on Sept 4th, it was identified that 360Lib did not support blending along the edges of cubemap faces for the GCMP project format using the padded samples. A request was made for this AHG to add this functionality to improve subjective quality of extracted viewports containing the face edges. Subsequently the 360Lib-11.0 software had been further enhanced to support blending along cubemap face edges using the padded samples, and preliminary expert viewing indicates some subjective quality improvement. It is requested to consider integrating this blending functionality into the next release of the 360Lib software.

Software repository and versions

The 360Lib software is developed using a Subversion repository located at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/

The released version of 360Lib-11.0 can be found at:

https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-11.0/

360Lib-11.0 testing results can be found at:

ftp.ient.rwth-aachen.de/ahg/testresults/360Lib-11.0

360Lib bug tracker

https://hevc.hhi.fraunhofer.de/trac/jem/newticket?component=360Lib

360Lib-11.0 results

The first table below is for the projection formats comparison using VTM-10.0 according to 360-degree video CTC (JVET-L1012). It compares padded hybrid equi-angular cubemap (PHEC) coding and padded equi-rectangular projection (PERP) coding using VTM-10.0.

VTM-10.0 PHEC vs PERP (PERP as anchor)

|  | PHEC over PERP (VTM-10.0) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -11.78% | -6.81% | -7.30% | -11.71% | -6.73% | -7.24% |
| Class S2 | -5.35% | -1.83% | -1.48% | -5.34% | -1.73% | -1.41% |
| Overall | -9.20% | -4.81% | -4.98% | -9.16% | -4.73% | -4.91% |

The next table is for PERP coding comparison between VTM-10.0 and HM-16.16. Table 3 is to compare PHEC coding with VTM-10.0 with and CMP coding with HM-16.16.

VTM-10.0 PERP vs HM-16.16 PERP (HM-16.16 PERP as anchor)

|  | VTM-10.0 PERP - Over HM-16.16 PERP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -25.62% | -33.61% | -36.36% | -25.60% | -33.63% | -36.33% |
| Class S2 | -35.07% | -32.96% | -35.62% | -35.06% | -32.99% | -35.67% |
| Overall | -29.40% | -33.35% | -36.06% | -29.38% | -33.37% | -36.07% |

VTM-10.0 PHEC vs HM-16.16 CMP (HM-16.16 CMP as anchor)

|  | VTM-10.0 PHEC - Over HM-16.16 CMP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | End-to-end WS-PSNR |  |  | End-to-end S-PSNR-NN |  |  |
|  | Y | U | V | Y | U | V |
| Class S1 | -30.07% | -35.71% | -38.02% | -29.96% | -35.69% | -37.98% |
| Class S2 | -37.78% | -35.87% | -38.11% | -37.78% | -35.86% | -38.14% |
| Overall | -33.15% | -35.77% | -38.06% | -33.09% | -35.75% | -38.05% |

The AHG recommended to continue software development of the 360Lib software package.

It was commented that there is a small improvement for PHEC in VTM 10.0 relative to VTM 9.

A new version of 360Lib was requested with the new blending functionality.

+ an output document revision to describe it

It was also suggested to move the 360Lib software to a GitLab server. This was agreed.

JVET-T0007 JVET AHG report: Coding of HDR/WCG material (AHG7) [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy]

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 19th meeting by teleconference (22 June – 1 July 2020) and the 20th meeting by teleconference (7–16 October 2020).

The primary activity of the AhG was related to the mandates of studying and evaluating available HDR/WCG test content, comparing the performance of the VTM for HDR/WCG content, and coordinating with AHG4 in preparation for verification testing for HDR video content.

The AhG generated CTC anchors for the VTM according to JVET-P2011. A summary of the performance is provided below, and more detailed information may be found in Excel spreadsheets attached with the AHG report.

NOTE: It is noted that the Class H2 sequences were updated at the 19th JVET meeting. This update consisted of changing the chroma sampling location for the sequences. In the tables below, we report the performance of the VTM10 and VTM9 using the configurations provided in the VTM repository to provide a reference for participants tracking the performance. Similarly, the HM16.18 results use the VTM9 sequences. Please note that the difference in Class H2 is largely due to the sequence change.

Insert results from revised report.

The chroma position issue was discussed. An additional version of report was requested to be provided with additional data to clarify this. The test sequences had changed in preparation for the verification tests. (See the notes of the meeting report of the previous meeting.)

The AHG had significant coordination with AHG4, including sharing results on the new HDR content and expert viewing procedures.

Two contributions related to HDR video coding were noted to have been registered for the meeting.

- JVET-T0082 New HLG sequences for high bit-depth video coding [K. Kondo, M. Ikeda, and A. Browne]

- JVET-T0106 AHG4/AHG7: Report on Display Settings for HDR/WCG Verification Tests [A. Segall, M. Wien and V. Baroncini]

The AHG recommended to review the relevant input contributions

JVET-T0008 JVET AHG report: Layered coding and resolution adaptivity (AHG8) [S. Wenger, A. Segall, M. M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun, P. Topiwala, Y.-K. Wang]

This document summarizes the activity of AHG08: Layered coding and resolution adaptivity, between the 19th JVET meeting (22 June – 1 July 2020) and the 20th JVET meeting by teleconference (7–16 October 2020).

No email traffic on the JVET reflector was observed that was labelled as belonging to AHG8.

Four contributions related to the AHG are provided as input to the current meeting.  The contributions are as follows:

- JVET-T0049 Composite Picture Information SEI Message [Hendry, H. Jang, S. Kim, J. Lim (LGE)]

- JVET-T0065 Conformance Points for Multilayer 8K [F. Bossen (Sharp)]

- JVET-T0070 AHG9/AHG8: Scalability dimension SEI message [Y.-K. Wang, L. Zhang, K. Zhang, Z. Deng (Bytedance)]

- JVET-T0080 AHG9: Multilayered OLS decoded picture assembling SEI message [E. Thomas (TNO)]

The AHG recommended reviewing all contributions related to layered coding and resolution adaptivity.

It was commented that, with the amount of activity that was recently devoted to getting these features into VVC, there should be more activity investigating it.

JVET-T0009 JVET AHG report: SEI message studies (AHG9) [S. McCarthy, J. Boyce, P. de Lagrange, A. Luthra, A. Tourapis, Y.-K. Wang, S. Wenger]

This document summarizes the activity of AHG9: SEI message studies between the 19th JVET Meeting (teleconference, 22 June – 1 July 2020) and the 20h JVET Meeting (teleconference, 7–16 Oct. 2020).

The following contributions were identified as related to AHG9. It was noted that some of the contributions were not identified as AHG9 in their titles.

- JVET-T0048 On the syntax design for extending SEI messages and VUI for HEVC, VVC and VSEI [Y.-K. Wang (Bytedance), G. J. Sullivan (Microsoft)]

- JVET-T0049 Composite Picture Information SEI Message [Hendry, H. Jang, S. Kim, J. Lim (LGE)]

- JVET-T0050 AHG9: HEVC Annotated Regions SEI message software update for HM [P. Guruva reddiar, J. Boyce (Intel)]

- JVET-T0051 AHG9: Object tracking information file format for use with Annotated Regions SEI messages [P. Guruva reddiar, J. Boyce (Intel)]

- JVET-T0052 AHG9: AVC Annotated Regions SEI message software for JM [P. Guruva reddiar, J. Boyce (Intel)]

- JVET-T0053 AHG9: VVC and VSEI Annotated Regions SEI message [J. Boyce, P. Guruva reddiar (Intel)]

- JVET-T0054 Some errata items for HEVC and AVC [Y.-K. Wang, L. Zhang (Bytedance), A. M. Tourapis (Apple)] (Aspects #7 and #8 relate to AHG9)

- JVET-T0055 Some errata items for VVC [Y.-K. Wang, Z. Deng (Bytedance)] (Aspects #2, #3, and #4 relate to AHG9)

- JVET-T0056 AHG9: SEI manifest and SEI prefix indication SEI messages [Y.-K. Wang (Bytedance), T. Stockhammer (Qualcomm), A. M. Tourapis, D. Singer (Apple)]

- JVET-T0059 AHG9: object representation SEI message [J. Chen, Y. Ye, J. Hu, K. Li, L. Hu, Y. Long (Alibaba)]

- JVET-T0066 AHG9: IRAP only HRD SEI message [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (Fraunhofer HHI), Y.-K. Wang, L. Zhang (Bytedance)]

- JVET-T0070 HG9/AHG8: Scalability dimension SEI message [Y.-K. Wang, L. Zhang, K. Zhang, Z. Deng (Bytedance)]

- JVET-T0071 AHG9: SEI messages for support of cross RAP referencing based video coding [Y.-K. Wang (Bytedance)]

- JVET-T0075 AHG9: Errata and editorial clarifications for AVC and HEVC film grain characteristics SEI message semantics [S. McCarthy, P. Yin, W. Husak, T. Lu, F. Pu, T. Chen (Dolby)]

- JVET-T0076 AHG9: ALF control SEI message [H.-B. Teo, H.-W. Sun, C.-S. Lim (Panasonic)]

- JVET-T0080 AHG9: Multilayered OLS decoded picture assembling SEI message [E. Thomas (TNO)]

- JVET-T0090 AHG9: Digital signature SEI message [J. Xu, Y.-K. Wang, L. Zhang, K. Zhang (Bytedance)]

- JVET-T0092 AHG9/AHG11: Neural network based super resolution SEI message [T. Chujoh, E. Sasaki, T. Ikai]

There were no relevant emails besides the AHG kickoff message sent to the JVET reflector during the AHG period.

The AHG recommended to review the related contributions and continue SEI messages studies.

There were relevant input contributions on both errata and SEI messages.

There was a discussion about getting SEI showcase software that’s not otherwise part of the reference software package. – For showcase purposes. A Python script was mentioned for this. It was commented that we already have some software tools that are bundled with the reference software, and if such software is maintained appropriately, we could do this with such SEI showcase software.

End of session #2.

JVET-T0010 JVET AHG report: Encoding algorithm optimization (AHG10) [A. Duenas, A. Tourapis, S. Ikonin, A. Norkin, R. Sjöberg, J. Le Tanou, J.-M. Thiesse]

TBP

JVET-T0011 JVET AHG report: Neural-network-based video coding (AHG11) [A. Alshina, S. Liu, J. Pfaff, M. Wien, P. Wu, Y. Ye]

To review in session 6.

JVET-T0012 JVET AHG report: High bit depth, high bit rate, and high frame rate coding (AHG12) [A. Browne, T. Ikai, X. Xiu]

To review in session 5.

JVET-T0013 JVET AHG report: Tool reporting procedure and testing (AHG13) [W.-J. Chien, J. Boyce, Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu]

TBP

JVET-T0021 JCT-VC AHG report: Project management (AHG1) [G. J. Sullivan, J.-R. Ohm]

JVET-T0022 JCT-VC AHG report: Test model editing and errata reporting (AHG2) [B. Bross, C. Rosewarne, J.-R. Ohm, K. Sharman, G. J. Sullivan, A. Tourapis, Y.-K. Wang]

JVET-T0023 JCT-VC AHG report: Software development and software technical evaluation (AHG3) [K. Sühring, B. Li, K. Sharman, V. Seregin, G. Tech, A. Tourapis]

JVET-T0024 JCT-VC AHG report: Supplemental enhancement information (AHG4) [J. Boyce, C. Fogg, S. McCarthy, G. J. Sullivan]

JVET-T0025 JCT-VC AHG report: Test sequence material (AHG5) [T. Suzuki, V. Baroncini, E. François, P. Topiwala, S. Wenger]Formularende

# Project development (XX)

## General (0)

## Text development and errata reporting (6)

JVET-T0048 On the syntax design for extending SEI messages and VUI for HEVC, VVC and VSEI [Y.-K. Wang (Bytedance), G. J. Sullivan (Microsoft)]

This contribution was discussed 1900 Wednesday 7 October 2020 (chaired by JRO).

This contribution discusses the SEI payload syntax in AVC, HEVC, and VVC/VSEI, and the VUI payload syntax in VVC/VSEI. It reports an asserted bug on the HEVC buffering period SEI message syntax extension and proposes a fix for the asserted bug, and discusses how extensions, when needed, should be specified for the syntax of SEI messages in HEVC and VSEI, and to the syntax of VUI in VSEI.

The first method was suggested to be “cleaner”, future proof, and easier to understand, while the second would isolate the changes to the one place where there is an asserted problem in HEVC.

For VVC, it was asserted that the first proposed approach would enable detection in the VVC spec of whether the VSEI spec had been extended.

It was discussed whether a third approach could be used, or a NOTE could be sufficient to clarify the corner case. It was sugested that the problem could exist only if the use_alt_cpb_params_flag is equal to 1.

The existing extension mechanism for SEI messages (which is there from HEVC and was also used in VVC/VSEI) was reported to cause a problem, as an extension of less than one byte with a pattern 10..0 would be interpreted as a byte alignment mechanism. The issue could only occur for very short extension payloads, as otherwise the presence of the extension would be identified by the payload size. Currently, only the buffering period SEI message could have a case where it is affected.

Two possible solutions are suggested: a) Define a longer payload extension for the SEI messages that are potentially affected (“second method”). b) Define for the buffering period SEI message (and for future SEI messages that might have a similar problem) an extra flag that clearly signals the presence of an extension (“first method”).

During the discussion, it is suggested that another solution could be to put a note into the spec. pointing out this case for buffering period. This would not affect existing implementations, but make encoder manufacturers aware that a decoder would ignore an intended extension in this corner case (which a more smart encoder could avoid). This would however not avoid similar cases in newly defined SEI messages (if not carefully designed, or proponents of such SEI not being aware of this problem), for future extensions the “first method” appears to be cleaner and better to understand.

Offline study and revisit was requested.

JVET-T0054 Some errata items for HEVC and AVC [Y.-K. Wang, L. Zhang (Bytedance), A. M. Tourapis (Apple)]

This contribution was discussed 1945 Wednesday 7 October 2020 (chaired by JRO & GJS).

Errata items for both HEVC and AVC

- Change the range notation from the following:

x = y..z	x takes on integer values starting from y to z, inclusive, with x, y, and z being integer numbers and z being greater than y.

to be as follows (same as in VVCv1):

x = y..z	x takes on integer values starting from y to z, inclusive, with x, y, and z being integer numbers and z being greater than or equal to y.

Errata items for HEVC only

- Search and replace "satisify" with "satisfy".

- In clause 9.3.3.6, fix the wrong indention for the paragraph starting with "Otherwise".

- Search and check "CVS", particularly in semantics, and fix to be CLVS as needed.

- Search and replace instances of "clasue" with "clause".

- The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are initially derived in clause C.2.1 and updated in clause C.2.3, and the second variable is not even used in clause C.2.3. However, currently, the wording in clause C.2.3 is that these variables are derived, which can confuse readers as to why the second variable is derived but not used in clause C.2.3 thus why not just remove the mentioning of the second variable from clause C.2.3.

Similar issue was identified during the development of VVCv1, and the final text in there was to use the phrase "updated" instead of "derived" for these two variables in clause C.2.3.

It is therefore proposed to change the following phrase in clause C.2.3:

The variables InitCpbRemovalDelay[ SchedSelIdx ], InitCpbRemovalDelayOffset[ SchedSelIdx ], CpbDelayOffset and DpbDelayOffset are derived as follows:

to be as follows:

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are updated, and the variables CpbDelayOffset and DpbDelayOffset are derived, as follows:

- Replace all the eight instances of "decoder conformance to profiles" with "the decoding process". These instances are as follows:

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annex A. Decoders conforming to a profile specified in Annex A but not supporting the INBLD capability specified in Annex F shall ignore all vps_extension_data_flag syntax elements.

sps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all sps_extension_data_flag syntax elements.

pps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all pps_extension_data_flag syntax elements.

slice_segment_header_extension_data_byte may have any value. Decoders shall ignore the value of slice_segment_header_extension_data_byte. Its value does not affect decoder conformance to profiles specified in Annex A.

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annexes A, G or H. Decoders conforming to a profile specified in Annexes A, G or H shall ignore all vps_extension_data_flag syntax elements.

vps_non_vui_extension_data_byte may have any value. Decoders shall ignore the value of vps_non_vui_extension_data_byte. Its value does not affect decoder conformance to profiles specified in this version of this Specification.

slice_segment_header_extension_data_bit may have any value. Decoders shall ignore the value of slice_segment_header_extension_data_bit. Its value does not affect decoder conformance to profiles specified in this version of this Specification.

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annexes A, G, H, or I. Decoders conforming to a profile specified in Annexes A, G, H, or I shall ignore all vps_extension_data_flag syntax elements.

- In the informative Table D.1, change the persistence scope for the SEI manifest SEI message and the SEI prefix indication SEI message from "The CLVS containing the SEI message" to "The CVS containing the SEI message", to be aligned with the semantics of the SEI messages. Note that this is not needed for AVC as AVC does not have such an informative table.

- Remove a tab from each of the two middle rows below:

| content_light_level_info( payloadSize ) { | Descriptor |
| --- | --- |
| max_content_light_level | u(16) |
| max_pic_average_light_level | u(16) |
| } |  |

Decision (BF): Adopt into next available revisions.

JVET-T0055 Some errata items for VVC [Y.-K. Wang, Z. Deng (Bytedance)]

This contribution was discussed 1955 Wednesday 7 October 2020 (chaired by JRO & GJS).

This contribution reports some asserted bugs in VVC 1st edition and proposes fixes for the asserted bugs. The asserted bugs and proposed fixes are summarized as follows:

- The current constraint on POC values in clause C.4 unnecessarily requires the prevTid0Pic to be within the POC distance of MaxPicOrderCntLsb / 2 even when the current picture has the ph_poc_msb_cycle_val signalled or has an inter-layer reference picture, and under either condition the inference of POC MSB is not needed. Consequently, the constraint disallows the intended use of the syntax element vps_max_tid_il_ref_pics_plus1[ i ][ j ] equal to 0 without having a great value of sps_log2_max_pic_order_cnt_lsb_minus4.

The proposed fix for this asserted bug is to require the prevTid0Pic to be within the POC distance of MaxPicOrderCntLsb / 2 only when the current picture does not have the ph_poc_msb_cycle_val signalled and does not have an inter-layer reference picture.

It was commented that this is primarily just removing a constraint in a case where the constraint is not necessary, and it may not be strictly necessary to relax the constraint. At least the wording of two NOTEs needs changing, as they imply that the constraint is not imposed.

The proponent said that the design intent was to not impose the tight constraint, as that is what is expressed in the two NOTEs.

It was commented that the POC can be used for detecting AU boundaries, and asked if removing the constraint could confuse AU detection.

Relevant use cases would include multilayer scenarios, such as inter-layer prediction from IRAP pictures only.

If we do not relax the constraint, the worst impact would be that the encoder would need to use more bits for POC LSBs than would have been otherwise necessary.

- In the general sub-bitstream extraction process, the step 9, which removes from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_ols_flag equal to 0 and there is no value in the list NestingLayerId equal to a value in the list LayerIdInOls[ targetOlsIdx ] is redundant.

The proposed fix for this asserted bug is to remove the step 9 of the general sub-bitstream extraction process.

- In the subpicture sub-bitstream extraction process, when at least one VCL NAL unit has been removed by the step 2, the step 5 removes from outBitstream all SEI NAL units that contain scalable nesting SEI messages with sn_subpic_flag equal to 0, including nested SLI SEI messages. However, nested SLI SEI messages, when present, may be needed by the step 6 for parameter sets rewritting.

The proposed fix for this asserted bug is to move step 5 after step 6 in the subpicture sub-bitstream extraction process.

- It is allowed that both nested and non-nested HRD-related SEI messages are present and apply to the same OLS. Similarly, it is also allowed that both nested and non-nested non-HRD-related SEI messages are present and apply to the same layer. However, there lacks a constraint to require that the content of such SEI messages of a particular payload type to be the same.

The proposed fix for this asserted bug is to require such nested and non-nested SEI messages to have the same SEI payload content.

Decision (BF): Adopt into next available revisions. For aspect #1, rephrase the NOTEs without removing the normative constraint.

JVET-T0075 AHG9: Errata and editorial clarifications for AVC and HEVC film grain characteristics SEI message semantics [S. McCarthy, P. Yin, W. Husak, T. Lu, F. Pu, T. Chen (Dolby)]

This contribution was discussed 2030 Wednesday 7 October 2020 (chaired by JRO & GJS).

The contribution proposes editorial corrections and clarifications for the semantics of the film grain characteristics (FGC) SEI message in the current versions of AVC and HEVC. The proposed editorial corrections and clarifications are the same as those already made for the semantics of the FGC SEI message in the current version of VSEI.

The contribution proposed a sentence saying “When VUI parameters are not present for the CVS, and equivalent information is not conveyed by external means, separate_colour_description_present_flag shall be equal to 1.”

There was discussion of this sentence, regarding whether this case is possible in the AVC and HEVC contexts and whether it should discuss VUI parameters presence or colour description syntax presence.

There was an error in subtracting 8 from something that was already specified as “_minus8”.

It was commented that the variable names and usage of multiple index variables could be improved.

Terminology should be double-checked in final editing.

Decision (BF): Adopt into next available revisions, with editorial adjustments as discussed above.

JVET-T0110 AHG2: Editorial and errata input on VVC draft text [B. Bross, J. Chen, S. Liu, Y.-K. Wang]

This contribution was discussed 2100 Wednesday 7 October 2020 (chaired by JRO & GJS).

-v3 of the contribution was discussed.

This document provides integrations of a number of bug fixes reported in the bug tracking system as well as some other editorial changes made by the authors, based on the latest JVET output draft VVC text in JVET-S2001-vH.

List of logged changes:

- Replaced Fig. 20 with the correct one (now the same as in the final FDIS text).

- Fixed tickets #1364, #1365, #1366, #1367, #1368, #1369, #1371, #1372, #1379, #1380, #1381, #1386, #1387, #1388

- Fixed block vector used for IBC-coded chroma blocks in equations (1109, 1110) for 4:2:2 and 4:4:4 9 (Thanks to Li Zhang).

- Fixed typos ‘mesage’ and a couple of wrong minus sign characters for the interpolation filter coefficients.

- Added missing “NAL unit” (3 instances) after “AUD ”.

- Added a dependency on y component to completely fix the issue reported in #1372.

- Added a missing condition for the inference of tu_y_coded_flag to completely fix the issue reported in #1366.

See also the notes in AHG2 report. This is a collection of fixes done in response to tickets beyond -vH of S2001.

Decision (BF): Adopt into next available revision.

Additional offline study for double-checking was encouraged.

It was agreed to produce an output text to include all fixes relative to JVET-S2001-vH and for the editors to work with the publication staff in the parent bodies to get as many of these fixes into the publication processes as possible.

End of session 3.

JVET-T0113 Errata for draft report on usage of video signal type code points [Y. Syed (Comcast)] [late]

TBP

## Test conditions (1)

JVET-T0063 Suggestion to upgrading random access configuration in CTC with GOP 32 configuration [K. Andersson, J. Enhorn, R. Sjöberg, J. Ström, L. Litwic, P. Wennersten (Ericsson)]

## Verification test (7)

JVET-T0043 Status Report on SDR HD Low Delay Verification Test Preparation [M. Wien, V. Baroncini]

JVET-T0044 Status Report on 360 Video Verification Test Preparation [M. Wien, V. Baroncini]

JVET-T0045 AHG4: Agenda and report of the AHG meeting on the SDR HD Verification Tests on 2020-09-03 [M. Wien, V. Baroncini]

JVET-T0046 AHG4: Agenda and report of the AHG meeting on the 360 Video Verification Tests on 2020-09-04 [M. Wien, V. Baroncini, Y. Ye]

JVET-T0097 Report on VVC compression performance verification testing in the SDR UHD Random Access Category [M. Wien (RWTH), V. Baroncini (VABTECH)] [miss]

JVET-T0103 Information on and analysis of the VVC encoders in the SDR UHD verification test [C. Helmrich, B. Bross, J. Pfaff, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late] [miss]

JVET-T0106 AHG4/AHG7: Report on Display Settings for HDR/WCG Verification Tests [A. Segall, M. Wien, V. Baroncini] [late] [miss]

## Coding studies and tools on specific use cases (0)

## Test material (2)

JVET-T0060 AHG4: new video conference sequences for HD verification testing [S. Xu, R.-L. Liao, J. Chen, Y. Ye (Alibaba)]

JVET-T0082 New HLG sequences for high bit-depth video coding [K. Kondo, M. Ikeda, A. Browne (Sony)]

## Conformance development (1)

JVET-T0100 AHG5: On gaps in conformance bitstreams [F. Bossen (Sharp)] [late]

## Software development (3)

JVET-T0064 Addition of ALF filter strength control to VTM [Kenneth Andersson, Jacob Ström (Ericsson)]

JVET-T0109 Crosscheck of JVET-T0064 (Addition of ALF filter strength control to VTM) [N. Hu (Qualcomm)] [late] [miss]

JVET-T0078 GDR Software [S. Hong, L. Wang, K. Panusopone (Nokia)]

JVET-T0081 AHG3: On StreamMergeApp design [E. Thomas (TNO)]

## Implementation studies (3)

JVET-T0061 Multi-threaded VTM decoder description and performance analysis [S. Gudumasu, T. Poirier, F. Urban, F. Hiron, R. Jullian, P. de Lagrange (InterDigital)]

JVET-T0095 Performance of a VVC software decoder [B. Zhu, S. Liu, X. Xu, X. Zhang, C. Gu, L. Wang, W. Feng (Tencent)] [late]

JVET-T0099 Open optimized VVC encoder (VVenC) and decoder (VVdeC) implementations [A. Wieckowski, J. Brandenburg, C. Bartnik, V. George, J. Güther, G. Hege, C. Helmrich, A. Henkel, T. Hinz, C. Lehmann, C. Stoffers, I. Zupancic, B. Bross, H. Schwarz, D. Marpe (HHI)] [late]

## Complexity analysis (2)

JVET-T0067 Bit Stream Feature Analyzer (BSFA) for Coding Tool Statistics based on VTM-10.0 [M. Kränzler, C. Herglotz, A. Kaup]

JVET-T0068 Decoding Time and Energy Assessment of VTM-10.0 and VVdeC [M. Kränzler, C. Herglotz, A. Kaup]

## Encoder optimization (2)

JVET-T0062 AHG10: Extension of rate control to support random access configuration with GOP size of 32 [F. Liu, Z. Liu, Y. Li, Z. Chen (Wuhan Univ.)]

JVET-T0074 AHG10: LMCS encoder algorithm for YUV4:4:4 content [F. Pu, T. Lu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby)]

JVET-T0098 Crosscheck of JVET-T0074:AHG10: LMCS encoder algorithm for YUV4:4:4 content [J. Chen (Alibaba)] [late] [miss]

## Profile/tier/level specification (1)

JVET-T0065 Conformance points for multilayer 8K [F. Bossen (Sharp)]

# Low-level tool technology proposals (22)

## AHG12: High bit depth coding for VVC (10)

JVET-T0047 AHG12: Preliminary common test conditions for high bitdepth video coding [T. Ikai, A. Browne, X. Xiu  (AHG coordinators)]

JVET-T0072 AHG12: Rice parameter selection for high bit depths [A. Browne, S. Keating, K. Sharman (Sony)]

JVET-T0108 Crosscheck of JVET-T0072 (AHG12: Rice parameter selection for high bit depths) [T. Hashimoto, T. Ikai (Sharp)] [late] [miss]

JVET-T0084 AHG12: Transform coefficients range extension for high bit-depth coding [T. Zhou, T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

JVET-T0104 AHG12: Cross-check of JVET-T0084: Transform coefficients range extension for high bit-depth coding [L. Pham Van (Qualcomm)] [late]

JVET-T0085 AHG12: Rice parameter derivation for high bit-depth coding [T. Hashimoto, T. Ikai (Sharp)]

JVET-T0101 Crosscheck of JVET-T0085:AHG12: Rice parameter derivation for high bit-depth coding [Adrian Browne (Sony)] [late]

JVET-T0086 AHG12: SIMD implementation of BDOF for high bit-depth coding [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

JVET-T0087 AHG12: VVC coding tool evaluation for high bit-depth coding [T. Ikai, T. Zhou, T. Hashimoto (Sharp)]

JVET-T0089 AHG12: Slice based Rice parameter selection for transform skip residual coding [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, C.-W. Kuo, X. Wang (Kwai Inc.)]

JVET-T0102 Crosscheck of JVET-T0089:AHG12: Slice based Rice parameter selection for transform skip residual coding [Adrian Browne (Sony)] [late]

JVET-T0091 AHG12: Fix on encoder overflow issue of the LMCS at high bit-depth coding [X. Xiu, H.-J. Jhu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai)]

JVET-T0112 Crosscheck of JVET-T0091 (AHG12: Fix on encoder overflow issue of the LMCS at high bit-depth coding) [F. Pu, T. Lu (Dolby)] [late] [miss]

JVET-T0093 AHG12: On signalling for high-bit depth [Y. Kidani, K. Kawamura, K. Unno (KDDI)]

JVET-T0107 Crosscheck of JVET-T0093 (AHG12: On signalling for high-bit depth) [T. Chujoh (Sharp)] [late] [miss]

JVET-T0105 AHG12: On the Rice parameter derivation for high bit-depth coding [L. Pham Van, D. Rusanovskyy, M. Karczewicz (Qualcomm)] [late]

## AHG11: Neural-network-based technology (11)

JVET-T0041 Methodology and reporting template for neural network coding tool testing [S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois]

JVET-T0042 Report of AHG11 Meeting on Neural Network-based Video Coding on 2020-07-30 [S. Liu, E. Alshina, J. Pfaff, M. Wien, P. Wu, Y. Ye]

JVET-T0057 AHG11: A case study to reduce computation of a neural network based in-loop filter by pruning [C. Auyeung, W. Wang, W. Jiang, X. Li, S. Liu (Tencent)]

JVET-T0058 AHG11: Information on inter-prediction coding tool with deep neural network [B. Choi, Z. Li, W. Wang, W. Jiang, X. Xu, S. Liu (Tencent)]

JVET-T0069 AHG11: SSIM based CNN model for in-loop filtering [T. Ouyang, F. Liu, H. Zhu, Z. Chen (Wuhan Unvi.), X. Xu, S. Liu (Tencent)]

JVET-T0073 AHG11: Neural Network-based intra prediction with transform selection in VVC [T. Dumas, F. Galpin, P. Bordes, F. Leleannec (Interdigital)]

JVET-T0079 AHG11: Neural Network-based In-Loop Filter [H. Wang, M. Karczewicz, J. Chen, A.M. Kotra (Qualcomm)]

JVET-T0088 AHG11: Convolutional neural networks-based in-loop filter [Y. Li, L. Zhang, K. Zhang, Y. He, J. Xu (Bytedance)]

JVET-T0092 AHG9/AHG11: Neural network based super resolution SEI [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

JVET-T0094 AHG11: In-loop filtering based on neural network [T.-C. Ma, W. Chen, X. Xiu, Y.-W. Chen, H.-J. Jhu, C.-W. Kuo, X. Wang (Kwai)] [late]

JVET-T0096 AHG11: Deep neural network for super-resolution [J. Y. Lee (Sejong University), W. Lim, G. Bang (ETRI)] [late]

## Other (1)

JVET-T0111 YCgCo-R: Observations and findings [D. Buitenhuis (VideoLAN), A.M. Tourapis (Apple Inc)] [late] [miss]

# High-level syntax (HLS) proposals (17)

## AHG8: Layered coding and resolution adaptivity (2)

## See also section 6.2.

JVET-T0070 AHG9/AHG8: Scalability dimension SEI message [Y.-K. Wang, L. Zhang, K. Zhang, Z. Deng (Bytedance)]

JVET-T0080 AHG9: Multilayered OLS decoded picture assembling SEI message [E. Thomas (TNO)]

## AHG9: SEI message studies and proposals (14)

See section 4.2 for errata considerations.

JVET-T0049 Composite Picture Information SEI Message [Hendry, H. Jang, S. Kim, J. Lim (LGE)]

This contribution was discussed 2130 Wednesday 7 October 2020 (chaired by JRO & GJS).

This contribution proposes a design for a composite picture information (CPI) SEI message. A composite picture is a picture created / generated from rectangular patches from pictures in an access unit. Similar SEI messages were proposed by JVET-S0123 and JVET-S0107. It is asserted that the design proposed in this contribution differs from the previous contributions as follows:

- The proposed design works for composite picture from not only multi layers but also from a single layer bitstream.

- The proposed design provides more details about handling of issues related to gap and overlap of patches in composite picture.

- In the proposed design, the association between CPI SEI message and its associated output layer set (OLS) is made by carrying CPI SEI message in scalable nesting SEI message. It is asserted that this design is more aligned with how other SEI messages that are associated with OLS.

- The proposed design provides ways for reducing bit cost for signalling.

The design principles of the proposed CPI SEI message are provided in section 2 of this document.

A scene could be composited from pictures in different layers or from regions of pictures.

Examples were shown in two figures.

Figure 1 – Use of composite picture in broadcasting / streaming online game program

Figure 2 – Use of composite picture in broadcasting / streaming live baseball game

For supporting composite picture based for multilayer bitstream, the following SEI message design principles were proposed:

- Composite picture information (CPI) SEI specifies information to create / generate a composite picture from one or more pictures in an access unit.

- A composite picture contains one or more rectangular shaped patches.

No particular relationship between the patches and subpictures or tiles was proposed.

In a previous related contribution, using only whole pictures was envisioned.

In this usage, some parts of the decoded pictures would not be used in the composite picture.

- Patches of a composite picture belong to rectangular areas in pictures of the same access unit.

- For each patch in a composite picture, the following information is signalled:

  - The location of the patch in the composite picture. This information can be signalled as the position of the top-left corner of the patch in the composite picture.

  - The size (i.e., width and height) of the patch in the composite picture.

  - The source of the pels of the patch. This includes the following information:

    - The layer id of the picture in the associated access unit.

    - The location of the area in the picture that corresponds to the patch (i.e., top-left corner)

    - The size of the area in the picture that corresponds to the patch.

- The size of a patch in a composite picture shall be greater than 0 and the associated area (i.e., the source of the pels of the patch) in the associated picture shall be greater than 0.

- There can be gap between two neighboring patches in a composite picture. In other words, there may be area in the composite picture that is not covered by any patch.

- There can be overlap of patches in a composite picture. In other words, there may be area in the composite picture that is covered by two or more patches.

- When there is overlap between two patches, the patch with higher index is rendered after the patch with lower index such that in the overlapped area between the two patches, the pels come from the patch with higher index.

- There shall be no two patches with the same size that covered the same area in a composite picture. This is intended to avoid signalling redundant patch(es) as there is only one will be displayed / visible.

- Two patches in a composite picture may have the same corresponding area in the same picture.

- A patch in a composite picture shall correspond an area in a picture from an output layer only.

- It is possible that a patch in a composite picture corresponds to an area in a picture from an output layer but the picture is not outputted by decoder (e.g., the PictureOutputFlag of the picture is derived to be equal to 0 due to certain condition). In such situation, implementer of the SEI message may skip copying the pels from the source area to the patch in the composite picture (e.g., a gray patch is shown in the composite picture).

- A CPI SEI message describes composition for its associated OLS. The association is established by allowing CPI SEI message to be carried in a scalable nesting SEI message.

  - When a scalable nesting SEI message contains CPI SEI message(s), the value of the value of sn_ols_flag shall be equal to 1.

  - When a CPI SEI message is not present in a scalable nesting SEI message, it applies to the targetOLS (i.e., to the whole bitstream being decoded).

- The CPI SEI message applies to AU A that contains the SEI and all AU that follow AU A in output order until one of the following is true:

  - The end of the bitstream.

  - The next AU contains a new CPI SEI message that applies to the same OLS.

- There shall not be two CPI SEI messages associated with the same OLS but contain different content in an AU. This would create problem for implementer to choose which one is the correct one. Thus, it is constrained that when two CPI SEI message are present in an AU and they are associated with the same OLS, they shall have the same content.

- When present, CPI SEI message is present in AU with TemporalId equal to 0 to avoid the SEI being removed during sublayer extraction.

- When present in a CVS, the first of the CPI SEI message to be present in the CVSS AU.

- Possible bit-saving for signalling:

  - For signalling the position and size of the patch and size of the composite picture, instead of using ue(v) coding, signal the bit length for the coding and code those syntax elements using u(v).

  - Signal a flag to specify whether scaling is allowed. When scaling is not allowed, the size of patch in the composite picture does not need to be signalled as it can be inferred to be equal to the size of the associated area in the source picture.

  - Signal a flag to specify whether cropped picture is used for the composite picture. When the flag is equal to 0, the location of the associated area in the source picture does not need to be signalled as it can be inferred to be equal to the top-left corner of the picture.

  - The size of the composite picture can be not signalled. This can be inferred from the position and size of patches that make up the composite picture.

A participant commented that using some externally filled cache image could also be considered.

The envisioned use would have all the pictures within one multilayer bitstream. It was commented that there can be use cases that don’t obey that assumption, and a system could provide such functionality.

Another participant commented that this is basically a scene description language as would be found in some system designs.

It was asked how to deal with extraction – e.g., if some pictures have been removed from the bitstream. There would be nesting in an OLS to prevent that.

There was also discussion of what happens if the composite picture references a non-output picture. The proponent suggested that filling such areas with flat grey could be an approach to handle that.

Multiparty videoconferencing was a described potential use.

Subpicture usage was mentioned as something that can provide a similar functionality. It was noted that subpictures do not support overlap and that gaps are more difficult using subpictures.

A proponent of a previous related proposal said that a layout based on whole pictures (first figure) could use in-place decoding to avoid needing an extra memory buffer.

Picture overlap with a precedence was proposed.

Basic questions were:

- Should such functionality be done in an SEI message or in a system layer

- If in an SEI message, how general should it be?

There was discussion of whether synchronization would be easier if this is done within a video bitstream or by a system.

It was suggested to consider this in a joint discussion with MPEG Systems and Requirements.

See also JVET-T0080 (and JVET-S2017).

Revisit.

JVET-T0080 AHG9: Multilayered OLS decoded picture assembling SEI message [E. Thomas (TNO)]

This contribution was discussed 2215 Wednesday 7 October 2020 (chaired by JRO & GJS).

This contribution presents a motivation for a combination of the decoded pictures from different layers based on PoC value as opposed to time-based combination for composition use cases.

The proposal is based on the section 2 of JVET-S2017 “Technologies under consideration for VSEI” and can be summarized as follows:

- Each layer contains the information determining its position in the assembled output picture.

- Each layer holds the information of its neighbouring layer.

- The position of each layer in the assembled picture is thus calculated based on the signalled information and the target OLS.

- Decoded pictures with the same PoC are assembled into the assembled picture.

- The existing cropping operation is applied to each decoded picture before assembled into the assembled picture.

Figures showing alternative approaches

PoC-based and time-based synchronization in video pipeline

PoC-based layer combination

This does not propose using overlaps or gaps.

It was suggested to consider this in a joint discussion with MPEG Systems and Requirements.

See also JVET-T0049 (and JVET-S2017).

Revisit.

JVET-T0053 AHG9: VVC and VSEI Annotated Regions SEI message [J. Boyce, P. Guruva reddiar (Intel)]

This contribution was discussed 2240 Wednesday 7 October 2020 (chaired by JRO & GJS).

It is proposed to add an annotated regions SEI message to VVC and VSEI, with identical SEI message payload syntax to that used in HEVC. Draft text and VTM software are provided.

VVC does not yet have an annotated regions SEI message.

The proposal contains only adjustment for terminology and the interface between VVC and VSEI.

Persistence was discussed, but the proposal is to keep this the same as previously specified in HEVC (and in an AVC output document).

It was noted that persistence scope is discussed in an errata report. The proponent indicated that a modified persistence would be complicated to specify, and did not propose any modification of that aspect.

It was asked whether there should be some interaction between the annotation and the subpictures concept. The proponent said that since objects do not align with subpictures, the concepts should probably not be coupled.

Decision: Remove the persistence aspect from the errata collection.

Decision: Adopt as WD, targeting v2 (when that happens).

See also T0059, which is a proposed generalization of the AR SEI message in three ways.

JVET-T0059 AHG9: Object representation SEI message [J. Chen, Y. Ye, J. Hu, K. Li, L. Hu, Y. Long (Alibaba)]

This contribution was discussed 2300 Wednesday 7 October 2020 (chaired by JRO & GJS).

This contribution proposes to provide an object representation SEI message in VSEI to represent objects detected or tracked in the pictures. The objects are represented by bounding boxes or bounding polygons with the depth in the proposed SEI message. Primary and secondary attributes of the object may also be signaled in the proposed SEI message.

In the second version, the design of secondary label present flag was simplified and some bugs in the syntax table and semantics were fixed.

HEVC supports the annotated regions SEI message, which is currently not in the VSEI spec. Though the proposed object representation SEI message is similar to the annotated regions SEI message in some aspects, its syntax supports the following capabilities that are not available in the annotated regions SEI message but are asserted to be necessary to some applications:

- Ability to represent a non-rectangular object using bounding polygons: some sophisticated object detection mechanism on the cloud can trace the object contour accurately, and such information could provide useful features (e.g. virtual background);

- Ability to attach up to two labels to a represented object: the proposed SEI can attach a primary label and a secondary label to a represented object, for example, the primary label can be “car” vs. “pedestrian” and the secondary label can be “red,” “blue”, or “yellow” for a car and “walking” or “standing” for a person;

- The depth of an object: the distance to the “camera” allows the rendering device to determine occlusion more accurately, and to select which objects to emphasize.

For the “secondary” label, a participant said that the primary/secondary content could be embedded within a label – e.g., “car, red”. For more than two degrees (primary, secondary, tertiary, etc.), the proponent said the signaling would get more complicated.

It was commented that the polygon case may need “sensibility constraints”. There are no semantics to ensure that the data that is sent is actually some polygon. There is no definition of what region is identified – which samples in the picture are within the “polygon” and which are not.

It was also commented that the number of vertices of the “polygon” is unconstrained as proposed.

The proponent of T0053 said there could be a benefit of not deviating from what is done in HEVC, by having tools and workflows that work the same way for the different contexts.

It was commented that if one scheme is a subset of the other, then some workflows could use the subset only.

It was commented that if we specify a generalization, we could also hypothetically specify a new SEI message for HEVC using the generalized form to support it in the HEVC context as well as in the VVC context.

It was commented that the use case for the existing form seems more clear than with polygons.

Among the three proposed generalizations, it was commented that the first and third seem the most potentially interesting.

Not much non-proponent support was expressed.

Further study of generalizations was encouraged, perhaps as a separate SEI message.

Stopped here in session 4.

JVET-T0050 AHG9: HEVC Annotated Regions SEI message software update for HM [P. Guruva reddiar, J. Boyce (Intel)]

JVET-T0051 AHG9: Object tracking information file format for use with Annotated Regions SEI messages [P. Guruva reddiar, J. Boyce (Intel)]

JVET-T0052 AHG9: AVC Annotated Regions SEI message software for JM [P. Guruva reddiar, J. Boyce (Intel)]

JVET-T0056 AHG9: SEI manifest and SEI prefix indication SEI messages [Y.-K. Wang (Bytedance), T. Stockhammer (Qualcomm), A. M. Tourapis, D. Singer (Apple)]

JVET-T0066 AHG9: IRAP only HRD SEI message [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (Fraunhofer HHI), Y.-K. Wang, L. Zhang (Bytedance), S. Deshpande (Sharp)]

JVET-T0070 AHG9/AHG8: Scalability dimension SEI message [Y.-K. Wang, L. Zhang, K. Zhang, Z. Deng (Bytedance)]

JVET-T0071 AHG9: SEI messages for support of cross RAP referencing based video coding [Y.-K. Wang (Bytedance)]

JVET-T0076 AHG9: ALF control SEI message [H.-B. Teo, H.-W. Sun, C.-S. Lim (Panasonic)]

JVET-T0080 AHG9: Multilayered OLS decoded picture assembling SEI message [E. Thomas (TNO)]

JVET-T0090 AHG9: Digital signature SEI message [J. Xu, Y.-K. Wang, L. Zhang, K. Zhang (Bytedance)]

JVET-T0092 AHG9/AHG11: Neural network based super resolution SEI [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

## Other (1)

JVET-T0077 CICP- Use cases for signalling of still image graphics (chyrons) in video [Y. Syed, W. Husak, C. Seeger]

# Withdrawn (X)

Section kept for future use.

JVET-T0083 Withdrawn

# Plenary meetings, joint meetings, BoG reports, and summary of actions taken

## Joint meeting with MPEG Systems Monday 29 June 1430-1500

## Joint meeting with JCT-VC, VCEG (Q6/16) and MPEG Requirements Tuesday 30 June 1300-1500

## Closing plenary meeting Wednesday XX June xxxx-xxxx

## BoGs (X)

No formal break-out groups were established at this meeting that produced reports, although some offline studies were reported.

## List of actions taken affecting the draft text of VVC, the VTM, and 360Lib

[Remove this section]

The following is a summary, in the form of a brief list, of the actions taken at the meeting that affect the text of the VVC draft text, VTM or 360Lib description. Both technical and editorial issues are included. This list is provided only as a summary – details of specific actions are noted elsewhere in this report and the list provided here may not be complete and correct. The listing of a document number only indicates that the document is related, not that it was adopted in whole or in part. The description given in the “Tool” column is a best effort for the sake of understanding but may not precisely reflect the functionality of the tool. It is also noted that in cases where several contributions proposed the same method, usually only one of the is listed as adoption below; refer to the meeting notes about the adoption to see which other contributions are related.

[Add actions of Friday 24 April.]

[This is just a reflection of what has already been recorded.]

# Project planning

## Core experiment planning

No CEs were planned at this meeting.

## Drafting of specification text, encoder algorithm descriptions, and software

The following agreement has been established: the editorial team has the discretion to not integrate recorded adoptions for which the available text is grossly inadequate (and cannot be fixed with a reasonable degree of effort), if such a situation hypothetically arises. In such an event, the text would record the intent expressed by the committee without including a full integration of the available inadequate text.

## Plans for improved efficiency and contribution consideration

The group considered it important to have the full design of proposals documented to enable proper study.

Adoptions need to be based on properly drafted working draft text (on normative elements) and HM encoder algorithm descriptions – relative to the existing drafts. Proposal contributions should also provide a software implementation (or at least such software should be made available for study and testing by other participants at the meeting, and software must be made available to cross-checkers in EEs).

Suggestions for future meetings included the following generally-supported principles:

- No review of normative contributions without draft specification text

- VTM algorithm description text is strongly encouraged for non-normative contributions

- Early upload deadline to enable substantial study prior to the meeting

- Using a clock timer to ensure efficient proposal presentations (5 min) and discussions

The document upload deadline for the next meeting was planned to be XXday XX Apr 2020.

As general guidance, it was suggested to avoid usage of company names in document titles, software modules etc., and not to describe a technology by using a company name.

## General issues for experiments

It was emphasized during the opening plenary on January 9 that those rules which had been set up or refined during the 12th meeting should be observed. In particular, for some CEs, results were available late, and some changes in the experimental setup (particularly in CE4) were not discussed on the JVET reflector.

Group coordinated experiments have been planned as follows:

- “Core experiments” (CEs) are the coordinated experiments on coding tools which are deemed to be interesting but require more investigation and could potentially become part of the draft standard by the next meeting.

- A CE is a test of a specific fully described technology in a specific agreed way. It is not a forum for thinking of new ideas (like an AHG). The CE coordinators are responsible for making sure that the CE description is complete and correct and has adequate detail. Reflector discussions about CE description clarity and other aspects of CE plans are encouraged.

- A description of each experiment is to be approved at the meeting at which the experiment plan is established. This should include the issues that were raised by other experts when the tool was presented, e.g., interference with other tools, contribution of different elements that are part of a package, etc. The experiment description document should provide the names of individual people, not just company names.

- Software for tools investigated in a CE will be provided in one or more separate branches of the software repository. Each CE will have a “fork” of the software, and within the CE there may be multiple branches established by the CE coordinator. The software coordinator will help coordinate the creation of these forks and branches and their naming. All JVET members will have read access to the CE software branches (using shared read-only credentials; the method for members to obtain the credentials is TBA on the reflector).

- During the experiment, revisions of the experiment plans can be made, but not substantial changes to the proposed technology.

- The CE description must match the CE testing that is done. The CE description needs to be revised if there has been some change of plans.

- The CE summary report must describe any changes that were made in the process of finalizing the CE.

- By the next meeting it is expected that at least one independent cross-checker will report a detailed analysis of each proposed feature that has been tested and confirm that the implementation is correct. Commentary on the potential benefits and disadvantages of the proposed technology in cross-checking reports is highly encouraged. Having multiple cross-checking reports is also highly encouraged (especially if the cross-checking involves more than confirmation of correct test results). The reports of cross-checking activities may (and generally should) be integrated into the CE report rather than submitted as separate documents.

It is possible to define sub-experiments within particular CEs, for example designated as CEX.a, CEX.b, etc., where X is the basic CE number.

As a general rule, it was agreed that each CE should be run under the same testing conditions using one software codebase, which should be based on the group test model software codebase. An experiment is not to be established as a CE unless there is access given to the participants in (any part of) the CE to the software used to perform the experiments.

The general agreed common conditions for single-layer coding efficiency experiments are described in the output document JVET-N1010.

Experiment descriptions should be written in a way such that it is understood as a JVET output document (written from an objective “third party perspective”, not a proponent perspective – e.g. not referring to methods as “improved”, “optimized”, etc.). The experiment descriptions should generally not express opinions or suggest conclusions – rather, they should just describe what technology will be tested, how it will be tested, who will participate, etc. Responsibilities for contributions to CE work should identify individuals in addition to company names.

CE descriptions contain a basic description of the technology under test, but should not contain excessively verbose descriptions of a technology (at least not unless the technology is not adequately documented elsewhere). Instead, the CE descriptions should refer to the relevant proposal contributions for any necessary further detail. However, the complete detail of what technology will be tested must be available – either in the CE description itself or in documents that are referenced in the CE description that are also available in the JVET document archive.

Any technology must have at least one cross-check partner to establish a CE – a single proponent is not enough. It is highly desirable have more than just one proponent and one cross-checker.

[Add info on software access.]

Some agreements relating to CE activities were established as follows:

- Only qualified JVET members can participate in a CE.

- Participation in a CE is possible without a commitment of submitting an input document to the next meeting. Participation is requested by contacting the CE coordinator.

- All software, results, and documents produced in the CE should be announced and made available to JVET in a timely manner.

- A JVET CE reflector will be established and announced on the main JVET reflector. Discussion of logistics arrangements, exchange of data, minor refinement of the test plans, and preparation of documents shall be conducted on the JVET CE reflector, with subject lines prefixed by “[CEx: ]”, where “x” is the number of the CE. All substantial communications about a CE other than such details shall take place on main JVET reflector. In the case that large amounts of data are to be distributed, it is recommended to send a link to the data rather than the data itself, or upload the data as an input contribution to the next meeting.

General timeline for CEs

T1= 3 weeks after the JVET meeting: To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. Initially assigned description numbers shall not be changed later. If a test is skipped, it is to marked as “withdrawn”.

T2 = Test model software release + 2 weeks or X XX, whichever is earlier: Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector.

- Initial study by cross-checkers can begin.

- Proponents may continue to modify the software in this branch until T3

- 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes

T3: 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later: Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time.

T4: Regular document deadline – 1 week: CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting).

The CE summary reports shall be available by the regular deadline. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text change, as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline). Furthermore, any deviations from the timelines above shall be documented. The numbers used in the summary report shall not be changed relative to the description document.

CE reports may contain additional information about tests of straightforward combinations of the identified technologies. Such supplemental testing needs to be clearly identified in the report if it was not part of the CE plan.

New branches may be created which combine two or more tools included in the CE document or the VTM (as applicable).

It is not necessary to formally name cross-checkers in the initial version of the CE description document. To adopt a proposed feature at the next meeting, we would like see comprehensive cross-checking done, with analysis that the description matches the software, and recommendation of value of the tool given tradeoffs.

The establishment of a CE does not indicate that a proposed technology is mature for adoption or that the testing conducted in the CE is fully adequate for assessing the merits of the technology, and a favourable outcome of CE does not indicate a need for adoption of the technology.

Availability of spec text is important to have a detailed understanding of the technology and also to judge what its impact on the complexity of the spec will be. There must also be sufficient time to study it in detail. CE contributions without sufficiently mature draft spec text in the CE input document should not be considered for adoption.

Lists of participants in CE documents should be pruned to include only the active participants. Read access to software will be available to all members.

## Software development and anchor generation (update)

Proponents of adopted aspects were asked to inform the AHG3 chairs to clarify whether software action is needed.

The planned timeline for software releases was established as follows:

- VTM8.0 will be released by 2020-02-17 including all adoptions necessary for CTC. VTM8.1 with non-CTC adoptions will be released 2020-03-16. Further versions of VTM may be released for additional bug fixing, as appropriate.

- Preparation of the VTM software will include immediate removal of macros that were added in the previous meeting cycle. The software coordinator has the discretion to retain some such macros.

- 360lib software is to be revised for the modified generalized cubemap, which was requested by 2019-02-28

- No change of HDRTools software was noted in response to meeting.

# Establishment of ad hoc groups

The ad hoc groups established to progress work on particular subject areas until the next meeting are described in the table below. The discussion list for all of these ad hoc groups was agreed to be the main JVET reflector (jvet@lists.rwth-aachen.de).

+Meeting plans for ad hocs

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| Project Management (AHG1) (jvet@lists.rwth-aachen.de) - Coordinate overall JVET interim efforts. - Supervise AHG studies. - Report on project status to JVET reflector. - Provide a report to the next meeting on project coordination status. | J.-R. Ohm, G. J. Sullivan (co-chairs) | N |
| Draft text and test model algorithm description editing (AHG2) (jvet@lists.rwth-aachen.de) - Produce and finalize JVET-S2001 VVC text specification draft 10 and JVET-S2007 VSEI text specification draft 5. - Produce and finalize JVET-S2002 VVC Test Model 10 (VTM 10) Algorithm and Encoder Description. - Gather and address comments for finalization of these documents. - Coordinate with test model software development AhG to address issues relating to mismatches between software and text. - Collect and consider errata reports on the texts | B. Bross, J. Chen (co-chairs), J. Boyce, S. Kim, S. Liu, Y.-K. Wang, Y. Ye (vice-chairs) | N |
| Test model software development (AHG3) (jvet@lists.rwth-aachen.de) - Coordinate development of test model (VTM) software and associated configuration files. - Produce documentation of software usage for distribution with the software. - Discuss and make recommendations on the software development process. - Propose improvements to the guideline document for developments of the test model software. - Perform tests of VTM behaviour relative to HEVC and the previous VTM using the VTM common test conditions. - Coordinate with AHG on Draft text and test model algorithm description editing (AHG2) to identify any mismatches between software and text, and make further updates and cleanups to the software as appropriate. - Coordinate with AHG6 for integration with 360lib software. | F. Bossen, X. Li, K. Sühring (co-chairs) | N |
| Test material and visual assessment (AHG4) (jvet@lists.rwth-aachen.de) - Produce the draft verification test plan JVET-S2009 and develop proposed improvements for verification testing of VVC capability. - Maintain the video sequence test material database for testing the VVC standard and potential future extensions. - Identify and recommend appropriate test materials for testing the VVC standard and potential future extensions. - Identify missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material. - Evaluate new test sequences. - Maintain and update the directory structure for the test sequence repository as necessary. - Prepare availability of viewing equipment and facilities arrangements for the next meeting, and prepare testing upon consultation with CE coordinators. | V. Baroncini, T. Suzuki, M. Wien (co-chairs), A. Norkin, A. Segall, Y. Ye (vice-chairs) | Tel. 2 weeks notice |
| Conformance testing (AHG5) (jvet@lists.rwth-aachen.de) - Produce the JVET-S2008 draft conformance testing specification and develop proposed improvements. - Study the requirements of VVC conformance testing to ensure interoperability. - Maintain and update the conformance bitstream database - Study additional testing methodologies to fulfil the needs for VVC conformance testing. | J. Boyce and W. Wan (co-chairs), E. Alshina, F. Bossen, I. Moccagatta, K. Kawamura, K. Sühring, X. Xu (vice-chairs) | N |
| 360° video coding, software and test conditions (AHG6) (jvet@lists.rwth-aachen.de) - Study the effect on compression and subjective quality of different projections formats, resolutions, and packing layouts. - Solicit additional test sequences, and evaluate suitability of test sequences on head-mounted displays and normal 2D displays. - Study the effect of viewport resolution, field of view, and viewport speed/direction on visual comfort. - Prepare and deliver the 360Lib-11 software version and common test condition configuration files according to JVET-L1012. - Generate CTC anchors and PERP results for the VTM according to JVET-L1012 within two weeks of availability of SDR CTC anchors. - Coordinate with AHG4 in preparation for verification testing for 360° video content. - Produce documentation of software usage for distribution with the software. | J. Boyce and Y. He (co-chairs), K. Choi, J.-L. Lin, Y. Ye (vice-chairs) | N |
| Coding of HDR/WCG material (AHG7) (jvet@lists.rwth-aachen.de) - Study and evaluate available HDR/WCG test content. - Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results. - Compare the performance of the VTM and HM for HDR/WCG content. - Generate CTC anchors for the VTM according to JVET-S2011 within two weeks of availability of SDR CTC anchors. - Study the luma/chroma bit allocation in the HDR CTC, especially for HLG content. - Coordinate implementation of HDR anchor aspects in the test model software with AHG3. - Coordinate with AHG4 in preparation for verification testing for HDR video content. - Study additional aspects of coding HDR/WCG content. | A. Segall (chair), E. François, W. Husak, S. Iwamura, D. Rusanovskyy (vice-chairs) | N |
| Layered coding and resolution adaptivity (AHG8) (jvet@lists.rwth-aachen.de) - Study approaches for support of layered scalable coding and adaptive-resolution coding, including spatial, temporal, quality, view, subpicture, and region-of-interest aspects; and analyse their coding efficiency and complexity characteristics. - Consider 360° viewport-dependent streaming and real-time communication applications of layered coding and resolution adaptivity. - Coordinate with AHG2 and AHG3 for text drafting and software development for the layered coding and resolution adaptivity aspects of the VVC design. - Study and develop improvements of the JVET-Q2015 functionality testing condition description. - Propose common test conditions for layered coding and resolution adaptivity. | S. Wenger and A. Segall (co-chairs), M. M. Hannuksela, Hendry, S. McCarthy, Y.-C. Sun, P. Topiwala, Y.-K. Wang (vice-chairs) | N |
| SEI message studies (AHG9) (jvet@lists.rwth-aachen.de) - Study the SEI messages in VVC and VSEI. - Collect software and SEI showcase information for SEI messages, including encoder and decoder implementations and bitstreams for demonstration and testing. - Identify potential needs for additional SEI messages, particularly including those in the TuC JVET-S2xxx. - Study SEI messages defined in HEVC and AVC for potential use in the VVC context. | S. McCarthy (chair), J. Boyce, P. de Lagrange, A. Luthra, A. Tourapis, Y.-K. Wang, S. Wenger (vice-chairs) | N |
| Encoding algorithm optimization (AHG10) (jvet@lists.rwth-aachen.de) - Study the impact of using techniques such as GOP structures and perceptually optimized adaptive quantization for encoder optimization. - Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality. - Study the impact of adaptive quantization. - Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding. - Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality. | A. Duenas, A. Tourapis (co-chairs), S. Ikonin, A. Norkin, R. Sjöberg, J. Le Tanou, J.-M. Thiesse (vice-chairs) | N |
| Neural-network-based video coding (AHG11) (jvet@lists.rwth-aachen.de) - Study potential extensions of VVC with NN-based coding tools for video coding, such as intra or inter prediction modes, partitioning, transforms, and in-loop or post filtering. - Study NN-based encoding optimization for VVC. - Study the impact of training on the performance of candidate technology. - Analyse complexity characteristics and perform complexity analysis of candidate technology. - Identify video test materials, training set materials, and testing methods for assessment of the effectiveness and complexity of considered tools. - Develop reporting templates for test results and analysis of candidate technology. - Coordinate with relevant activities of the parent bodies. | E. Alshina, S. Liu, J. Pfaff, M. Wien, P. Wu, Y. Ye (co-chairs) | Tel. 2 weeks notice |
| High bit depth, high bit rate, and high frame rate coding (AHG12) (jvet@lists.rwth-aachen.de) - Study the benefits and characteristics of VVC coding tools for high bit depth, high bit rate, and high frame rate coding. - Identify potential needs for future extension of VVC to support such application usage. - Define testing conditions and test sequences for high bit depth, high bit rate, and high frame rate coding in coordination with AHG 4. | A. Browne, T. Ikai, X. Xiu (co-chairs) | N |
| Tool reporting procedure and testing (AHG13) (jvet@lists.rwth-aachen.de) - Prepare output document JVET-S2005, which describes the methodology of tool-off testing and a list of tools to be tested by identified testers, including non-CTC configurations as appropriate. - Produce, study and develop improvements of the JVET-R2013 testing condition description for non-4:2:0 colour format coding. - Provide configurations files, bitstreams, and results of tool-on/tool-off testing. - Develop and collect test results for additional testing of VVC capabilities. - Maintain VTM software aspects for memory bandwidth analysis in coordination with AHG3. - Use the tool usage counts and memory bandwidth usage to study the decoder complexity of features in on/off testing. - Prepare a report with results of the tests. | W.-J. Chien, J. Boyce (co-chairs), Y.-W. Chen, R. Chernyak, K. Choi, R. Hashimoto, Y.-W. Huang, H. Jang, R.-L. Liao, S. Liu (vice-chairs) | N |

Former JCT-VC AHGs (change reflector, and merge some)

| Title and Email Reflector | Chairs | Mtg |
| --- | --- | --- |
| JCT-VC project management (AHG1) merge (jct-vc@lists.rwth-aachen.de) - Coordinate overall JCT-VC interim efforts. - Report on project status to JCT-VC reflector. - Provide a report to next meeting on project coordination status. | G. J. Sullivan, J.-R. Ohm (co-chairs) | N |
| Test model editing and errata reporting (AHG2) (jct-vc@lists.rwth-aachen.de) - Propose improvements to the JCTVC-AL1002 HEVC Test Model 16 (HM 16) Update 13 of Encoder Description - Collect reports of errata for the HEVC, AVC, CICP, the codepoint usage TR specification and the published HDR-related technical reports. - Gather and address comments for refinement of these documents. - Coordinate with AHG3 on software development and software technical evaluation to address issues relating to mismatches between software and text. | B. Bross, C. Rosewarne (co-chairs), J.-R. Ohm, K. Sharman, G. J. Sullivan, A. Tourapis, Y.-K. Wang (vice-chairs) | N |
| Software development and software technical evaluation (AHG3) (jct-vc@lists.rwth-aachen.de) - Coordinate development of the HM, SCM, SHM, HTM, MFC, MFCD, JM, JSVM, JMVM, 3DV-ATM, and HDRTools software and their distribution. - Enable software support for recently standardized additional SEI messages. - Produce documentation of software usage for distribution with the software. - Prepare and deliver results, reporting templates, and anchor test results according to JCT-VC common conditions. - Suggest configuration files for additional testing of tools. - Investigate how to minimize the number of separate codebases maintained for group reference software. - Coordinate with AHG2 on HEVC and AVC test model editing and errata reporting to identify any mismatches between software and text. | K. Sühring (chair), B. Li, K. Sharman, V. Seregin, G. Tech, A. Tourapis (vice-chairs) | N |
| Supplemental enhancement information (AHG4) merge (jct-vc@lists.rwth-aachen.de) - Study the status of text and potential needs for SEI messages for AVC and HEVC. - Consider proposals for additional SEI message data and associated syntax and semantics specification. - Develop usage scenario descriptions and showcase demonstrations. - Coordinate with AHG3 for software support of SEI messages. | J. Boyce (chair), C. Fogg, S. McCarthy, G. J. Sullivan, Y.-K. Wang (vice-chairs) | N |
| Test sequence material (AHG5) merge (jct-vc@lists.rwth-aachen.de) - Maintain the video sequence test material database for development of HEVC and its RExt, SHVC and SCC extensions. - Identify, collect, and make available a variety of video sequence test material, especially focusing on new needs for HDR/WCG test material and corresponding SDR test material. - Collect information about test sequences that have been made available by other organizations. - Study coding performance and characteristics in relation to video test materials. - Identify and recommend appropriate test materials and corresponding test conditions for use in development of HEVC and its extensions. - Coordinate with the activities in AHG3 and AHG6 regarding HDR/WCG testing. | T. Suzuki, V. Baroncini (co-chairs), E. François, P. Topiwala, S. Wenger (vice-chairs) | N |

# Output documents

The following documents were agreed to be produced or endorsed as outputs of the meeting. Names recorded below indicate the editors responsible for the document production. Where applicable, dates of planned finalization and corresponding parent-body document numbers are also noted.

It was reminded that in cases where the JVET document is also made available as MPEG output document, a separate version under the MPEG document header should be generated. This version should be sent to GJS and JRO for upload.

JVET-S2000 Meeting Report of the 19th JVET Meeting [G. J. Sullivan, J.-R. Ohm] (2020-10-xx, near next meeting)

Initial versions of the meeting notes (d0 … d8) were made available on a daily basis during the meeting.

JVET-S2001 Versatile Video Coding (Draft 10) [B. Bross, J. Chen, S. Liu, Y.-K. Wang] [WG 11 N 19470 (2020-07-29)

(A technically complete draft available by closure of the meeting as -vA, aside from editorial aspects and double-checking of CABAC initialization values.)

DoCR N 19478.

JVET-S2002 Algorithm description for Versatile Video Coding and Test Model 10 (VTM 10) [J. Chen, Y. Ye, S. Kim] [WG 11 N 19471] (2020-10-05, near next meeting)

Software release of the 10.0 version was expected by 2020-07-31, and contributors need to submit their software revisions at least a week in advance of that date (certainly sooner than that is preferred).

Remains valid – not updated: JVET-N1003 Guidelines for VVC reference software development [K. Sühring] (2019-04-01)

JVET-S2004 Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 11) [Y. Ye, J. Boyce] (2020-10-xx, near next meeting)

It was also noted that a viewport extraction configuration file PCMP was missing from the software package, and other software improvements had been made.

JVET-S2005 Methodology and reporting template for coding tool testing [W.-J. Chien and J. Boyce] (2020-08-15)

Remains valid – not updated: JVET-M1006 Methodology and reporting template for neural network coding tool testing [Y. Li, S. Liu, K. Kawamura] (2019-02-01)

This prior output was produced to capture aspects specific to enable study of neural network techniques.

JVET-S2007 Versatile supplemental enhancement information messages for coded video bitstreams (Draft 5) [J. Boyce, V. Drugeon, G. J. Sullivan, Y.-K. Wang] [WG 11 N 19472] (2020-07-29)

(Technically complete draft available by closure of the meeting as -v3, aside from editorial aspects.)

DoCR WG 11 N 19479.

JVET-S2008 Conformance testing for versatile video coding (Draft 4) [J. Boyce, E. Alshina, F. Bossen, K. Kawamura, I. Moccagatta, W. Wan] [WG 11 N 19474] (2020-10-05, near next meeting)

Bitstreams were requested to be provided by two weeks after the release of the VTM 10.0 software.

Test bitstream providers were to provide bitstream descriptions in the document at the Nextcloud link given above.

JVET-S2009 VVC verification test plan (Draft 3) [M. Wien, V. Baroncini, A. Segall, Y. Ye] [WG 11 N 19475] (2020-07-31)

A preliminary draft was reviewed.

An initial draft is to be available by 2020-07-10.

The verification test coordinators are delegated the selection of GOP size (not changing the IRAP spacing), as determined by expert viewing.

Volunteering test sites will be mandated to successfully conduct a calibration experiment.

Given sufficient resources, we would like to have more than one lab perform each test.

Raw data from each test lab will be provided to the test coordinators.

If more labs volunteer than would be needed, the quality for workmanship in conducting the calibration experiment would be used to select sufficient testing resources.

The test coordinators were asked to produce a budget estimate for the test expenses.

Following the test, a summary report of the financial arrangements will be provided.

The test coordinators will seek volunteers to generate the necessary encodings.

See notes in section 4.4.

Remains valid – not updated: JVET-N1010 JVET common test conditions and software reference configurations for SDR video [F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring] (2019-04-12)

JVET-S2011 JVET common test conditions and evaluation procedures for HDR/WCG video [A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy] (2020-07-24)

Waiting on provision of “DayStreet” and “PeopleInShoppingCenter” test sequences.

Remains valid – not updated: JVET-L1012 JVET common test conditions and evaluation procedures for 360° video [P. Hanhart, J. Boyce, K. Choi, J.-L. Lin] (2018-10-26)

Remains valid – not updated: JVET-R2013 JVET common test conditions and software reference configurations for non-4:2:0 colour formats [Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu] (2020-05-15)

Only an update of the configuration files is planned per S0244, which does not affect the document.

Remains valid – not updated: JVET-Q2014 JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding [T.-C. Ma, A. Nalci, T. Nguyen] (2020-03-02)

Remains valid – not updated: JVET-Q2015 JVET functionality confirmation test conditions for reference picture resampling [J. Luo, V. Seregin] (2020-03-02)

JVET-S2016 Summary information on BD-rate experiment evaluation practices [K. Andersson, F. Bossen, J.-R. Ohm, A. Segall, R. Sjöberg, J. Ström, G. J. Sullivan, A. Tourapis] [WG 11 PDTR N19477 and ITU-T Approval] (2020-07-10)

Request in WG 11 N19476.

Minor refinements of the description.

JVET-S2017 Technologies under consideration for VSEI [J. Boyce, Y.-K. Wang] (2020-07-31)

Former JCT-VC Outputs & leftover notes from 40th JCT-VC:

AVC draft annotated regions & shutter interval

Issue output targeted at inclusion in VSEI

Not yet MPEG request for new edition

Issue PDTR Codepoint usage TR v3 23094-4 (check number)

MPEG request

JCTVC-AN1000 Meeting Report of the 40th JCT-VC Meeting [G. J. Sullivan, J.-R. Ohm (chairs)] [2020-xx] (near the next meeting)

Remains valid – not updated: JCTVC-H1001 HEVC software guidelines [K. Sühring, D. Flynn, F. Bossen (software coordinators)]

JCTVC-AN1002 High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 14 [C. Rosewarne (primary editor), K. Sharman, R. Sjöberg, G. J. Sullivan (co-editors)] (WG 11 N 19473) [2020-10-05] (near next meeting)

Update to describe LDB GOP structure. (New RA GOP structure is already in Update 13.)

[Old notes from prior meeting: This will include a description of the GOP16 structure. The description of the PCC motion search hint functionality of JCTVC-AJ0028 should also be improved. These are to be supported in the HM16.22 software release.]

HM16.22 had still not been released, and the PCC motion search had not yet been included in the work.

Is JCTVC-AJ0028 in the document? Yes, it was described in the document output from the previous meeting.

[In the closing plenary it was mentioned that there is a modified Low-delay B referencing structure used in JVET, described in JVET-P0345. If that fits in the HEVC buffering capacity and we have adequate software/configuration files for it, we would want this in our CTC and test model document. The proponents of that contribution had provided test results in the HEVC context (with about 4.7% gain in luma and somewhat more in chroma). From a look at the contribution, it appeared that the only software impact is on the configuration files. However, it may violate the HEVC buffering capacity. Further study on this was encouraged.]

There is no effect on the CTC document – only in the config files.

The latest tagged version of the HM (16.21) has a capacity-violating RA config (only a 0.2% impact on PSNR) and a not-as-good LDB referencing structure. HM 16.22 will have those features and its release was imminent.

The expected PCC motion search hint changes had not been followed up, and will not be included in HM 16.22.

Remains valid – not updated: JCTVC-AM1003 Draft revised coding-independent code points for video signal type identification [G. Sullivan, T. Suzuki, A. Tourapis] [2020-05-22]

Under ballot, not modified.

MPEG document 19208

+CD for CICP (& request for ISO/IEC)

request 19207.

JCTVC-AN1004 Errata report items for HEVC, AVC, Video CICP, and CP usage TR [G. J. Sullivan, Y. Syed, Y.-K. Wang] [2020-XX] (near next meeting)

Remains valid – not updated: JCTVC-AL1005 Shutter interval information SEI message for HEVC (Draft 2) [S. McCarthy, G. J. Sullivan, Y.-K. Wang] (WG 11 CDAM N 19121  DAM N 19198) (2020-02-07)

A DAM ballot was issued for WG 11 as WG 11 N 19198 (without changing the text).

In ISO/IEC the amendment will be renamed and renumbered.

JCTVC-AN1006 Annotated regions and shutter interval information SEI messages for AVC (Draft 1) [J. Boyce, S. McCarthy, Y.-K. Wang] (near next meeting)

Remains valid – not updated: JCTVC-V1007 SHVC Test Model 11 (SHM 11) Introduction and Encoder Description [G. Barroux, J. Boyce, J. Chen, M. M. Hannuksela, Y. Ye (editors)] (WG 11 N 15778)

JCTVC-AN1008 Usage of video signal type code points (Draft 1 for version 3) [W. Husak, G. J. Sullivan, Y. Syed, A. Tourapis (editors)] 2020-08-07

(only editorial improvements and errata corrections)

PDTR in MPEG N19481

Request in MPEG N19480

Remains valid – not updated: JCTVC-X1009 Common Test Conditions for SHVC [V. Seregin, Y. He (editors)]

Remains valid – not updated JCTVC-O1010 Guidelines for Conformance Testing Bitstream Preparation [T. Suzuki, W. Wan (editors)]

No output: JCTVC-AL1011

No output: JCTVC-AL1012

No output: JCTVC-AL1013

Remains valid – not updated JCTVC-V1014 Screen Content Coding Test Model 7 Encoder Description (SCM 7) [R. Joshi, J. Xu, R. Cohen, S. Liu, Y. Ye (editors)] (WG 11 N 16049)

Remains valid – not updated: JCTVC-Z1015 Common Test Conditions for Screen Content Coding [H. Yu, R. Cohen, K. Rapaka, J. Xu (editors)] [2017-02-17]

No output: JCTVC-AL1016 through JCTVC-AL1019

Remains valid – not updated: JCTVC-Z1020 Common Test Conditions for HDR/WCG Video Coding Experiments [E. François, J. Sole, J. Ström, P. Yin (editors)] [2017-02-17] (1 month)

Remains valid – not updated: – JCTVC-AF1100 Common Test Conditions for HM Video Coding Experiments [K. Sharman, K. Sühring (editors)] [2018-09-14]

(Revision only to be issued if needed for coordination; no such need was identified.)

# Future meeting plans, expressions of thanks, and closing of the meeting

Future meeting plans were established according to the following guidelines:

- Meeting under ITU-T SG 16 auspices when it meets (ordinarily starting meetings on the Wednesday of the first week and closing it on the Wednesday of the second week of the SG 16 meeting – a total of 8 meeting days), and

- Otherwise meeting under ISO/IEC JTC 1/SC 29/WG 11 auspices (or its successor) when it meets (ordinarily starting meetings on the Friday prior to such meetings and closing it at lunchtime on the last day of the WG 11 meeting – a total of 7.5 meeting days).

In cases where an exceptionally high workload is expected for a meeting, an earlier starting date may be defined.

Some specific future meeting plans (to be confirmed) were established as follows:

- Wed. 7 – Fri. 9 and Mon 12 – Friday 16 October 2020, 20th meeting under WG 11 auspices as a teleconference (7.5 meeting days).

- Fri. 8 – Fri. 15 January 2021, 21st meeting under WG 11 auspices in Capetown, ZA.

- Wed. 21 – Wed. 28 April 2021, 22nd meeting under ITU-T auspices in Geneva, CH.

- Fri. 9 – Fri. 16 July 2021, 23rd meeting under WG 11 auspices in Prague, CZ.

The agreed document deadline for the 20th JVET meeting was planned to be Wednesday 30 Sep. 2020.

Vittorio Baroncini, Andrew Segall, Mathias Wien, and Yan Ye were thanked for their efforts in further developing the VVC verification test plan and procedure. GBTech was thanked for kindly providing resources and conducting the dry run for SDR UHD test cases with non-expert viewers. Individuals who participated in the remote experts viewing, and experts who prepared bitstreams were also thanked.

NHK was thanked providing additional 4K HLG test sequences.

Thanks were also expressed to the editors, software coordinators, and all contributors to the development of the VVC and VSEI standards.

The 19h JVET meeting was closed at approximately 2100 hours UTC on Wednesday 1 July 2020.

# Annex A to JVET report:
List of documents

# Annex B to JVET report:
List of meeting participants

The participants of the nineteenth meeting of the JVET, according to an attendance sheet circulated during the meeting sessions (approximately XXX people in total), were as follows: