| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0002-v1 |
| --- | --- |

| Title: | JVET AHG report: Draft text and test model algorithm description editing (AHG2) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Benjamin Bross Jianle Chen Jill Boyce Seung-Hwan Kim Shan Liu Ye-Kui Wang Yan Ye | Email: | benjamin.bross@hhi.fraunhofer.de cjianle@qti.qualcomm.com jill.boyce@intel.com seunghwan3.kim@lge.com shanl@tencent.com yekui.wang@bytedance.com yan.ye@alibaba-inc.com |
| Source: | AHG2 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on draft text and test model algorithm description editing (AHG2) between the 19th meeting by teleconference, (22 June – 1 July 2020) and the 20th meeting by teleconference, (7 – 16 October 2020).

# Mandates

At the 19th JVET meeting, AHG2 on draft text and test model algorithm description editing was established with the following mandates:

- Produce and finalize JVET-S2001 VVC text specification draft 10 and JVET-S2007 VSEI text specification draft 5.

- Produce and finalize JVET-S2002 VVC Test Model 10 (VTM 10) Algorithm and Encoder Description.

- Gather and address comments for finalization of these documents.

- Coordinate with test model software development AhG to address issues relating to mismatches between software and text.

- Collect and consider errata reports on the texts

# Background

At the 19th JVET meeting, it was decided to simplify, clean up and fix the existing features for intra picture-prediction, inter-picture prediction, transform, CABAC engine, in-loop filter, and high-level syntax functionalities in the tenth draft of Versatile Video Coding (VVC D10) and the VVC Test Model 10 (VTM 10) encoding as well as the high-level syntax in the fifth draft of the VSEI text specification (VSEI D5).

The bitstream format, conformance, and normative decoding process for Versatile Video Coding are specified in the VVC draft 10 text specification document [1]. The VVC Test Model 10 (VTM 10) Algorithm and Encoder Description document provides an algorithm description as well as an encoder-side description of the VVC Test Model 10, which serves as a tutorial for the algorithm and encoding model implemented in the VTM10.x software [2]. Video usability information parameters and some supplemental enhancement information messages for coded video bitstreams are specified in the VSEI draft 5 text [3].

An issue tracker (https://jvet.hhi.fraunhofer.de/trac/vvc) was used to facilitate the reporting of errata with the VVC documents.

# JVET-S2001 VVC text specification (Draft 10)

Seventeen versions of JVET-S2001 were published by the Editing AHG between the 19th meeting by teleconference, (22 June – 1 July 2020) and the 20th meeting by teleconference, (7 – 16 October 2020).

JVET-S2001 has been established based on JVET-R2001 and now contains the following:

- Fixed tickets #931, #1268, #1269, #1270, #1271, #1272, #1273, #1274, #1275, #1276, #1277, #1278, #1279, #1280, #1281, #1282, #1284, #1287, #1288, #1290, #1293, #1295, #1300, #1301, #1302, #1303, #1304, #1305, #1306, #1307, #1308, #1309, #1310, #1311, #1313, #1314, #1315, #1316, #1317, #1318, #1321, #1322, #1323, #1324, #1325, #1326, #1327, #1328, #1329, #1330, #1331, #1332, #1333, #1334, #1335, #1336, #1337, #1338, #1339, #1340, #1341, #1342, #1343, #1344, #1345, #1346, #1347, #1348, #1349, #1350, #1351, #1352, #1354, #1355, #1356, #1357, #1358, #1360, #1361, #1362, #1363.

- Changed Tables 136 and 138-141 such that all level values have one fractal digit, like in Table 135.

- Clarified the constraint on mixing of IRAP NAL units with non-IRAP NAL units within a picture. (Thanks to Vadim Seregin and Hendry for their inputs)

- Changed the constraint that sli_max_sublayers_minus1 shall be in the range of 0..vps_max_sublayers_minus1 to that sli_max_sublayers_minus1 shall be equal to vps_max_sublayers_minus1, to fix a bug that when sli_max_sublayers_minus1 is less than vps_max_sublayers_minus1 there would be sublayers for which no level information is signalled or derived based on the SLI SEI message. (Thanks to Zhipin Deng for her inputs)

- Changed step 7 in subclause C.7 to remove non-nested SEI messages that are no longer applicable. (Thanks to Robert Skupin, Yago Sanchez, and Karsten Sühring for their inputs).

- Added OPI_NUT to the list of NAL unit types for which the zero_byte syntax element shall be present.

- Addressed items by checking the DoCR and the ISO/CS editor edited version (but only those that are common for ITU-T and ISO texts), including move of the definitions of "shall", "may", "should", "informative", "reserved", "unspecified", etc. to subclause 5.1, the beginning of the conventions clause.

- Fixed the constraints that the SLI/BP/PT/DUI SEI messages that apply to any OLSs that correspond to the same bitstream shall be the same.

- Fixed some other issues, including the x-refs to Figure C.1, which would add the figure itself when updating the x-ref fields.

- Various typo/bug fixes and other editorial improvements. (thanks to Virginie Drugeon, Miska Hannuksela, Yong He, Gary Sullivan, Vadim Seregin, Remy Foray, Hendry, Jizheng Xu, Zhipin Deng, Frank Bossen, Yago Sanchez, and Huanbang Chen for their inputs)

- Resolved most of the editors notes, including removal of unnecessary abbreviations and syntax functions, rephrasing of NOTEs to avoid "should", "may", etc.

- Changed "The value of nal_unit_type shall be the same for all pictures of a CVSS AU" to "The value of nal_unit_type shall be the same for all pictures in an IRAP or GDR AU", to make the intent clearer. The design intent has always been that each IRAP or GDR AU may become a CVSS AU. For example, when random accessing from a CRA or GDR AU it becomes the first AU in “the bitstream” and consequently a CVSS AU.

- Added definitions of palette and palette prediction, and changed the definitions of block vector and IBC prediction.

- Added Table 140 and Table 141 in the informative subclause A.4.3, listing the maximum picture rates (pictures per second) at the High tier for some example picture sizes when MinCbSizeY is equal to 64.

- Misc. editorial changes (thanks to Peng Yin and Li Zhang for their inputs to many of these)

High level tool control adoptions:

- JVET-S0073 (JVET-S0144 item 6.a): Remove no_tsrc_constraint_flag.

- JVET-S0130 aspect 1 (JVET-S0144 item 9.a): Change the range of sps_num_points_in_qp_table_minus1[ i ] from 0..63 + QpBdOffset to 0..36 − sps_qp_table_start_minus26[ i ].

- JVET-S0053 aspect 1 (JVET-S0144 item 11.a.i): If pps_chroma_tool_offsets_present_flag is equal to 1, infer the values of the chroma DB offsets in the PH and SH, when not present, to be equal to the chroma DB offsets in the PPS and PH, respectively, instead of inferring the values from the luma DB offsets in the same header (as in the current VVC text). Otherwise, infer the values from the luma DB offsets in the same header (as in the current VVC text).

- JVET-S0121 aspect 2 (JVET-S0144 item 11.b): Remove the condition check of pps_deblocking_filter_override_‌enabled_flag for the signalling of ph_deblocking_params_present_flag.

- JVET-S0053 aspect 2 (JVET-S0144 item 12): Change the semantics of pps_chroma_tool_offsets_present_flag to describe that the chroma deblocking tc and β offset syntax elements could be (instead of "are") present in the PH syntax structures or the SHs of pictures referring to the PPS when the flag is equal to 1.

- JVET-S0255 (JVET-S0144 item 15.b): Change the semantics of PH syntax elements related to ALF and SAO to better describe the design intent.

- Changed the semantics of almost all SPS/PPS/PH/SH enabled/disabled flags following the similar spirit of the above item.

- JVET-S0191 (JVET-S0144 item 16): Add the invocations of clauses 8.3.5 and 8.3.6.

- JVET-S0196 (JVET-S0144 item 17): Remove the inferences of sps_scaling_matrix_for_lfnst_disabled_flag and sps_scaling_matrix_designated_colour_space_flag.

- JVET-S0277 (JVET-S0144 item 22): Rename sps_mmvd_fullpel_only_flag to sps_mmvd_fullpel_only_enabled_flag, and switch the phrases "uses" and "may use" for the value 1 and 0 in the semantics of this SPS flag.

General and misc. HLS topics:

- JVET-S0052 (JVET-S0139 item 1): Remove the support of the separate colour plane coding from VVCv1, by removing the syntax elements sps_separate_colour_plane_flag and sh_colour_plane_id and the related text.

- JVET-S0077 (JVET-S0139 item 5): Move the derivation of PictureOutputFlag to the end of the decoding process of the picture.

- JVET-S0078 (JVET-S0139 item 6): Invoke the derivation process for NoOutputOfPriorPicsFlag only once per AU, that is, before the decoding process of the first picture in the CVSS AU but after parsing the slice header of the first slice of the current picture. When NoOutputOfPriorPicsFlag is equal to 1, the process of removing pictures stored in the DPB without outputting them is invoked only once per CVSS access unit.

- JVET-S0081 (JVET-S0139 item 7): Modify the derivation of prevTid0Pic (in the semantics of delta_poc_msb_cycle_present_flag, the POC decoding process) and revNonDiscardablePic as well as the "previous picture in decoding order" in C.4 to consider only pictures with ph_non_ref_pic_flag equal to 0.

- JVET-S0084 (JVET-S0139 item 8): When the current picture is a RADL picture, allow RASL pictures with pps_mixed_nalu_types_in_pic_flag is equal to 1 in active entries in RefPicList[ 0 ] or RefPicList[ 1 ]. (Spec text ticket #1035). The editor may also add a NOTE to explain that such RADL pictures would still be correctly decodable when random accessing from the CRA picture, due to the existence of subpicture-level constraints.

- JVET-S0110 aspect 1 (JVET-S0139 item 9): Change the following sentence in the NOTE in the RASL picture definition to consider that RADL pictures can refer to RASL pictures with mixed_nalu_types_in_pic_flag equal to 1: "RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures."

- JVET-S0124 aspect 1 (JVET-S0139 item 15): Add derivation of TemporalId, ph_non_ref_pic_flag, and ph_pic_parameter_set_id to the decoding process for generating unavailable reference pictures (in order to enable checking of some constraints for them).

- JVET-S0139 item 17: Make the following changes on end of sequence (EOS) NAL units (S0155):

  - In Clause 3 (Definitions), the NOTE as part of the definition of CLVS picture, the phrase "the first PU in the layer of the bitstream that follows an EOS NAL unit in decoding order", change "EOS NAL unit" to "EOS NAL unit in the layer".

  - In subclause 7.4.2.2 (NAL unit header semantics), instead of requiring the nuh_layer_id of an EOS NAL unit to be equal to the nuh_layer_id of the associated VCL NAL unit, it is specified that the nuh_layer_id of an EOS NAL unit shall be equal to one of the nuh_layer_id values of the layers present in the CVS.

  - In subclause 7.4.2.4.3 (Order of PUs and their association to AUs), it is specified that, when present, the next PU of a particular layer after an EOS NAL unit that belongs to the same layer shall be an IRAP or GDR PU.

  - In subclause 7.4.2.4.4 (Order of NAL units and coded pictures and their association to PUs), it is specified that, when an EOS NAL unit is present in a PU, it shall be the last NAL unit among all NAL units within the PU other than other EOS NAL units (when present) or an EOB NAL unit (when present).

  - In subclause 7.4.3.10 (End of sequence RBSP semantics), it is specified that, when present, the EOS RBSP specifies that the next subsequent PU that belongs to the same layer as the EOS NAL unit in the bitstream in decoding order (if any) is an IRAP or GDR PU.

- JVET-S0139 item 18.b: Add the following constraints related to EOS NAL units in multi-layer bitstreams: When an AU auA contains an EOS NAL unit in a layer layerA, for each layer layerB that is present in the CVS and has layerA as a reference layer, the first picture in layerB in decoding order in an AU following auA in decoding order shall be a CLVSS picture. (S0174 aspect 2)

- JVET-S0139 item 19.a: In subclause A.4.2 (Profile-specific level limits), change the definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of slices i.e., items c and d, from being based on MaxSlicesPerPicture and the maximum picture size to be based on MaxSlicesPerAu, which is rename of the existing MaxSlicesPerPicture without changing its value, and change "the number of slices in each picture in AU" to "the number of slices in AU". (S0156 aspect 1 alt)

- JVET-S0139 item 20.a: In subclause A.4.2 (Profile-specific level limits), change the definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of tiles i.e., items i and j, from being based on MaxTileCols * MaxTileRows and the maximum AU size to be based on MaxTilesPerAu, which replaces the existing MaxTileRows, where the value of MaxTilesPerAu is set equal to MaxTileRows * MaxTileCols, and change "the number of tiles in each picture in AU" to "the number of tiles in AU". (S0156 aspect 2 alt)

- JVET-S0139 item 21: In subclause C.4 (Bitstream conformance), the 6th constraint, change "DpbOutputTime[ n ] greater than DpbOutputTime[ currPic ]" to "DpbOutputTime[ n ] greater than CpbRemovalTime[ currPic ]". This is asserted to be a bug, because all decoded pictures in the DPB are always decoded earlier than decoding of the current picture, and thus CpbRemovalTime[ n ] in the context is always less than CpbRemovalTime[ currPic ]. This is also asserted to be a bug in the latest HEVC spec. Also clarify that the number of decoded pictures in the DPB is the number that is computed in the sentence. (S0156 aspect 3)

- JVET-S0139 item 22: Change "less than" in the following condition (that's part of the specification of the first picture of an AU) to "less than or equal to": The value of nuh_layer_id of the VCL NAL unit is less than the nuh_layer_id of the previous picture in decoding order. (S0160 aspect 5)

- JVET-S0139 item 23: Remove the following constraint from the definition of the two still picture profiles: The referenced SPS shall have dpb_max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0. (S0160 aspect 6)

- JVET-S0139 item 24: When sps_vui_parameters_present_flag is equal to 0, the VUI syntax elements are inferred to indicate unspecified and/or unconstrained indications. (S0160 aspect 8, S0266)

- JVET-S0139 item 25: Change an existing constraint such that the value of ph_poc_msb_cycle_present_flag is required to be equal to 0 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and there is an ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture (instead of "and there is a picture in the current AU in a reference layer of the current layer"). (S0160 aspect 9)

- JVET-S0139 item 26: Add a NOTE to explain that in the cases when the prevTid0Pic in POC derivation is the previous IRAP or GDR picture with ph_recovery_poc_cnt equal to 0, e.g., when vps_max_tid_il_ref_pics_plus1[ i ][ j ] is equal to 0, or when the prevTid0Pic in POC derivation is the previous intra picture in decoding, e.g., in single-layer intra-picture-only trick play, encoders could either include ph_poc_msb_cycle_val for each IRAP or GDR picture with ph_recovery_poc_cnt equal to 0, or for each intra picture, or set the value of sps_log2_max_pic_order_cnt_lsb_minus4 to be large enough such that the POC difference between the current picture and the prevTid0Pic is less than MaxPicOrderCntLsb / 2. (S0241)

- JVET-S0139 item 27: Enable signalling of TargetOlsIdx and Htid in the bitstream, modified as suggested to specify the ability to override by external means. (S0163)

- JVET-S0139 item 28: Change the two constraints on RPLs for RADL pictures and subpictures, considering multi-layer coding as well as its combination with subpictures. (S0167)

- JVET-S0139 item 33: Fix the descriptions of the semantics of ph_recovery_poc_cnt and the NOTE in subclause 8.3.4.2 to consider the case with a recovery POC distance of 0. (S0188 aspect 4)

- JVET-S0139 item 36.a: Require that the bitstream shall contain at least one picture with pic_output_flag equal to 1 that is in an output layer. (S0202 aspect 1)

- JVET-S0139 items 37 and 38: In the PTL syntax structure, change the signalling of sublayer_level_idc[ i ] to be in descending order of i, i.e., from the MaxNumSubLayersMinus1 − 1 to 0, inclusive, and change the signalling of ptl_sublayer_level_present_flag[ i ] to be in descending order of i. (S0203 aspect 1 / S0207, S0203 aspect 2)

- JVET-S0139 item 40: Either infer ph_poc_msb_cycle_present_flag to be equal to 0 when it is not present or avoid referring to the flag when it is not present. (S0239, bug ticket #1118)

- JVET-S0210: Specify that a Main 10 (4:4:4 or 4:2:0) Still Picture profile decoder shall be capable of decoding the first picture of a Main 10 (4:4:4 or 4:2:0) profile bitstream if that picture is an IRAP picture or is a GDR picture with ph_recovery_poc_cnt equal to 0 and is in an output layer and has ph_pic_output_flag equal to 1.

- JVET-S0266: Use a syntax handling for the VUI that is similar to that for SEI messages, such that it has optional presence and starts in a byte aligned position and has an indicated length in bytes that is coded as ue(v) and has the same ability to add more data at the end of the VUI.

General constraints information (GCI):

- JVET-S0179 (JVET-S0138 item 1): Signal the GCI syntax structure between the general PTL fields and sub-profile fields in the PTL syntax structure. Signal the gci_present_flag at the beginning of the GCI syntax structure and use it to condition the GCI fields, move the byte alignment to the end of the GCI syntax structure, and change the reserved bytes to reserved bits.

- JVET-S0051/S0127 (JVET-S0138 item 1.a.ii): Consider the name of max_bitdepth_constraint_idc to clarify its minus8 semantics.

- JVET-S0105 / S0161-v3 (JVET-S0138 item 2.a): Signal GCI fields in the order of different categories, including format, functionality, NALU, partition, prediction, intra, inter, transform/quantization/residual, loop filter, layer, others, and within each category, the order of the fields are the same as the corresponding syntax elements in the SPS or other syntax structures, with the following modifications: 1) gci_intra_only_constraint_flag, gci_all_layers_independent_constraint_flag, and gci_one_au_only_constraint_flag placed at the beginning of the GCI syntax structure, named as the "general category"; 2) gci_no_idr_rpl_constraint_flag placed in the "NAL unit type related" category instead of intra coding tool mechanism; 3) gci_no_palette_constraint_flag and gci_no_ibc_constraint_flag merged (in the order as in S0161-v1) into the "intra" category with other intra tools related GCI flags, instead of having their own "prediction mode" category ; 4) gci_no_virtual_boundaries_constraint_flag placed in the "loop filter" category instead of the "partitioning" category.

- JVET-S0058 (JVET-S0138 item 3.b): Add one GCI flag for both sps_weighted_pred_flag and sps_weighted_bipred_flag.

- JVET-S0161 (JVET-S0138 items 4-22): Add the following new GCI fields:

  - no_virtual_boundaries_constraint_flag to constrain sps_virtual_boundaries_enabled_flag. (S0050 #2, S0105, S0131)

  - no_explicit_scaling_list_constraint_flag to constrain sps_explicit_scaling_list_enabled_flag. (S0050 #2, S0058 S0066, S0067, S0105)

  - no_mtt_constraint_flag to constrain SPS MTT SEs sps_max_mtt_hierarchy_depth_intra_slice_luma, sps_max_mtt_hierarchy_depth_inter_slice and sps_max_mtt_hierarchy_depth_intra_slice_chroma. (S0058)

  - no_luma_transform_size_64_constraint_flag to constrain sps_max_luma_transform_size_64_flag. (S0058, S0066)

  - max_log2_ctu_size_constraint_idc, u(2)-coded, to constrain sps_log2_ctu_size_minus5, with the following modification: Change to gci_three_minus_max_log2_ctu_size_constraint_idc such that the value 0 does not impose a constraint. (S0066)

  - no_rectangular_slice_constraint_flag to constrain pps_rect_slice_flag. (S0113)

  - one_slice_per_subpicture_constraint_flag to constrain pps_single_slice_per_subpic_flag (S0113)

- JVET-S0073-v2 aspect 1 option 1 and JVET-S0050-v3 aspect 5 (JVET-S0138 items 4 and 38.d): When no_aps_constraint_flag is equal to 1, sps_ccalf_enabled_flag, ph_num_alf_aps_ids_luma, ph_alf_cb_enabled_flag, ph_alf_cr_enabled_flag, sh_num_alf_aps_ids_luma, sh_alf_cb_enabled_flag, and sh_alf_cr_enabled_flag all be equal to 0.

- JVET-S0138 item 4:

  - Move general_non_packed_constraint_flag and general_non_projected_constraint_flag to the VUI syntax structure, immediately after the general_interlaced_source_flag (and thus remove from the VVC spec).

  - Move frame_only_constraint_flag and single_layer_constraint_flag outside the GCI syntax structure into the PTL syntax structure, rename single_layer_constraint_flag to ptl_multilayer_enabled_flag, and change ptl_alignment_zero_bit to ptl_reserved_zero_bit.

  - Move the GCI syntax structure to be immedaitely after ptl_multilayer_enabled_flag.

- JVET-S0050 aspect 3.b (JVET-S0138 item 24): When one_tile_per_pic_constraint_flag is equal to 1, NumTilesInPic shall be equal to 1.

- JVET-S0050 aspect 3.c / S0112 (JVET-S0138 item 25.a): When one_subpic_per_pic_constraint_flag is equal to 1, sps_num_subpics_minus1 shall be equal to 0.

- JVET-S0050 aspect 5.c (JVET-S0138 item 27): Consider clarifying semantics of one_slice_per_pic_constraint_flag – e.g., when pps_rect_slice_flag is 1, then pps_num_slices_in_pic_minus1 needs to be equal to 0 (which is already required).

- JVET-S0090 option 2 (JVET-S0138 item 29): Constarint that all VCL NAL units shall have the same value of nuh_layer_id when there is only one layer.

- JVET-S0050 aspect 4 / S0054 aspect 4 (JVET-S0138 item 31): Use a different general_profile_idc for Main 10 Still Picture (we already require Main 10 profile decoders to decode such bitstreams). Do not require the constraint flag to be 1 as part of the profile definition.

- JVET-S0050 aspect 4 / S0054 aspect 4 (JVET-S0138 item 31): Remove all constraints that require GCI fields to be equal to a value that imposes a constraint.

- JVET-S0046 aspect 1 / JVET-S0050 aspect 3.a (JVET-S0138 item 39): The value of the u(4)-coded max_bitdepth_minus8_constraint_idc shall be in the range of 0 to 8, inclusive.

- JVET-S0091 aspect 1 (JVET-S0138 item 46): Use profile_idc values that effectively move this bit to the MSB but do not identify it as a flag (Main 10 = 1, Main 10 Still = 64+1, Main 10 4:4:4 = 32+1=33, Main 10 4:4:4 Still = 64+32+1=97, and replace general_one_picture_only_constraint_flag with gci_one_au_only_constraint_flag.

- JVET-S0094 (JVET-S0138 item 47): Make the maximum bit depth to be 16 − max_bitdepth_constraint_idc and the maximum chroma format be 3 − max_chroma_format_constraint_idc.

- JVET-S0054 aspect 1 / JVET-S0105 / JVET-S0209 aspect 2 (JVET-S0138 item 50): Move the constraint "When gci_frame_only_constraint_flag is equal to 1, the value of sps_field_seq_flag shall be equal to 0" from SPS semantics to GCI semantics.

- JVET-S0054 aspect 2 / JVET-S0209 aspect 2 (JVET-S0138 item 51): Move the constraint "When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of pps_mixed_nalu_types_in_pic_flag shall be equal to 0" from PPS semantics to GCI semantics.

- JVET-S0209 aspect 4 (JVET-S0138 item 53): Prefix all syntax elements in the general constraint information with "gci_".

- JVET-S0195 / JVET-S0161-v3 (JVET-S0138 item 56): Replace one_subpic_per_pic_constraint_flag with proposed no_subpic_info_constraint_flag and its semantics and add no_idr_rpl_constraint_flag.

SPS, PPS, APS:

- JVET-S0042 (JVET-S0142 item 1.b): Relax the semantics so that an extension_flag in DCI, VPS, SPS, PPS, or APS equal to 1 specifies that extension_data_flag syntax elements may be present. (Previously, it said these flags are present.)

- JVET-S0074 aspect 1 / S0132 aspects 1 & 2 (JVET-S0142 item 2): Rearrange some syntax elements in the SPS so that the syntax elements of similar coding tools are grouped together.

- JVET-S0115 aspect 2 (JVET-S0142 item 4): Change the name and semantics of vps_all_layers_same_num_sublayers_flag (to be vps_default_ptl_dpb_hrd_max_tid_flag) to only control VPS syntax without requiring the number of sublayers to be the same for all layers.

- JVET-S0049 aspect 4 / S0120 (JVET-S0142 item 6): Constrain the value of pps_alf_info_in_ph_flag to be equal to 0 when the PH is in the SH.

- JVET-S0132 aspect 3 (JVET-S0142 item 8): Rearrange PPS syntax elements based on whether they are intra or inter related.

- JVET-S0169 aspect 1 (JVET-S0142 item 13): Change the semantics of vps_sublayer_dpb_params_present_flag and sps_sublayer_dpb_params_flag, to accurately describe that they control the presence of the dpb_max_dec_pic_buffering_minus1[ i ], dpb_max_num_reorder_pics[ i ], and dpb_max_latency_increase_plus1[ i ] syntax elements for temporal sub-layers that are not the highest.

- JVET-S0186 aspect 1 (JVET-S0142 item 14): Move the syntax elements sps_chroma_format_idc and sps_log2_ctu_size_minus5 to take the place of sps_reserved_zero_4bits.

- JVET-S0219 aspect 1 (JVET-S0142 item 19): Enable removal of APSs in non-output layer that is not used as reference for decoding of pictures in output layers during the general sub-bitstream extraction process, by modifying the semantics of vps_max_tid_il_ref_pics_plus1[ i ][ j ] such that when it is greater than 0 it means that for decoding pictures of the i-th layer, no APS and no picture from the j-th layer with TemporalId greater than vps_max_tid_il_ref_pics_plus1[ i ][ j ] − 1 are used as reference, and in the extraction process, in addition to removing VCL NAL unit, also remove APS NAL units with TemporalId greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

- JVET-S0219 aspect 2 (JVET-S0142 item 20): Move aps_params_type to be before aps_adaptation_parameter_set_id.

PH and SH:

- JVET-S0135 (JVET-S0143 item 4): Add a NOTE to point out that a GDR picture cannot have pps_mixed_nalu_types_in_pic_flag equal to 1.

- JVET-S0076 aspect 1 (JVET-S0143 item 6.b): Move ph_non_ref_pic_flag to earlier position, i.e., before any syntax element whose presence is conditioned on the value of other syntax element(s). Immediately after ph_gdr_or_irap_pic_flag.

- JVET-S0193 aspect 1 (JVET-S0143 item 13): Move no_output_of_prior_pics_flag to the slice header of IRAP and GDR slices (but require it to have the same value in all IRAP and GDR slices of the AU).

- JVET-S0174 aspect 1 (JVET-S0143 item 19): Change the constraint on the collocated picture to additionaly require the same CTU size as the current picture.

RPL:

- JVET-S0045 (JVET-S0140 item 1): Modify the semantics of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] such that the value 1 of the flag means a negative value.

- JVET-S0123 (JVET-S0140 item 5): Invoke the generation of unavailable reference picture for an IDR picture that has RPLs, and change the RPL constraints in subclause 8.3.2 accordingly.

- JVET-S0123 (JVET-S0140 item 5.b): Change the process for deriving empty RPLs when sps_idr_rpl_present_flag is equal to 0 and nal_unit_type is equal to IDR_W_RADL or IDR_N_LP to involve pps_rpl_info_in_ph_flag.

- JVET-S0085 aspect 2 (JVET-S0140 item 9): Fix an asserted bug in the syntax condition "if( !pps_rpl_info_in_ph_flag  | |  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 )" in the PH syntax (when pps_rpl_info_in_ph_flag is equal to 0 the variable RplsIdx[ 1 ] in the context is used undefined), with the following changes: Editor action item: The editor is asked to rephrase the text to avoid the problem. The suggested method is to add an extra “if” condition with a comment to explain why it is there. The change made was to specify a local variable presenceFlag and use local variable in the syntax condition wherein it is possible to interpret RplsIdx[ i ] as unspecified.

- JVET-S0096 aspect 3 (JVET-S0140 item 10): Fix an asserted bug that the inference of rpl_idx[ 0 ] is missing, by adding the following inference rule (in addition to the existing inference of rpl_idx[ 1 ]): The value of rpl_idx[ 0 ] is inferred to be equal to 0 when sps_num_ref_pic_lists[ 0 ] is equal to 1 and rpl_sps_flag[ 0 ] is equal to 1. Adopt in spirit; infer a value only when needed. Exact expression to be determined offline by the editor.

- JVET-S0096 aspect 1 (JVET-S0140 item 11): Require ph_inter_slice_allowed_flag be 0 when the RPL is sent in the PH and list 0 is empty.

- JVET-S0096 aspect 4 (JVET-S0140 item 13): Change the constraint that the collocated picture "shall be the same for all slices of a coded picture" to "shall be the same for all non-I slices of a coded picture".

- JVET-S0159 aspect 3 (JVET-S0140 item 16): Add GDR picture with ph_recovery_poc_cnt equal to 0 (in addition to IRAP picture) to the constraint in subclause 8.3.2 on RPLs based on the value of vps_max_tid_il_ref_pics_plus1[ ][ ].

- JVET-S0159 aspect 4 (JVET-S0140 item 17): Mark each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ], of a CLVSS picture as "used for long-term reference".

- JVET-S0182 (JVET-S0140 item 18): When num_ref_entries[ listIdx ][ rplsIdx ] in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is equal to 0, ltrp_in_header_flag[ listIdx ][ rplsIdx ] is skipped.

Virtual boundaries:

- JVET-S0047: Add “_minus1” to each of the four syntax elements sps_virtual_boundary_pos_x, sps_virtual_boundary_pos_y, ph_virtual_boundary_pos_x and ph_virtual_boundary_pos_y, made other changes accordingly.

- JVET-S0065 aspect 1: Infer sps_virtual_boundaries_present_flag to be equal to 0 when not present.

- JVET-S0171 (JVET-S0256): Editorial changes for disabling SAO filtering operations across virtual boundaries.

- JVET-S0184: Modify the constraint that disallows the combination of subpictures and the signalling of virtual boundaries in the PH  to be directly on related SPS syntax elements.

- JVET-S0211 (JVET-S0256): Signal the number of virtual boundaries using ue(v) instead of the current signalling using u(2) for the four syntax elements sps_num_ver_virtual_boundaries, sps_num_hor_virtual_boundaries, ph_num_ver_virtual_boundaries, and ph_num_hor_virtual_boundaries while retaining the current value ranges.

- JVET-S0221 (JVET-S0256): Constrain the values of sps_num_ver_virtual_boundaries, sps_num_hor_virtual_boundaries, ph_num_ver_virtual_boundaries, and ph_num_hor_virtual_boundaries to equal to 0 when the picture width or height is less than or equal to 8.

HRD:

- JVET-S0064 (JVET-S0141 item 1): Conditionally signal bp_sublayer_dpb_output_offsets_present_flag (and bp_dpb_output_tid_offset[ i ]) only when bp_max_sublayers_minus1 is greater than 0 and infer it as 0 otherwise.

- JVET-S0141 items 2.a, 2.b, 2.c, 2.e, 2.f, and 2.g.i: When sps_video_paramater_set_id is equal to 0:

  - Infer TotalNumOlss to be equal to 1. (JVET-S0075, JVET-S0097)

  - Infer NumLayersInOls[ 0 ] to be equal to 1. (JVET-S0075, JVET-S0097)

  - Infer vps_layer_id[ 0 ] to be equal to the single value of nuh_layer_id of all the VCL NAL units. (JVET-S0097)

  - Infer LayerIdInOls[ 0 ][ 0 ] to be equal to the single value of nuh_layer_id of all the VCL NAL units. (JVET-S0097)

  - Infer NumSubLayersInLayerInOLS[ 0 ][ 0 ] to be equal to sps_max_sublayers_minus1 + 1. (JVET-S0097)

  - Modify the inference rule for vps_max_sub_layers_minus1 (JVET-S0097/S0049 aspect 1) and the value range of sps_max_sublayers_minus1 (JVET-S0097).

- JVET-S0141 item 2.d: Add some explanation to point out that the nuh_layer_id of referenced SPSs, PPSs, and APSs in a single-layer bitstream is equal to the nuh_layer_id of the VCL NAL units. (JVET-S0075)

- JVET-S0141 item 5: Fix an asserted bug in Equation C.10. (JVET-S0101)

- JVET-S0141 item 7: Include PH NAL unit in Type I (but keep AUD in Type II as it was). (JVET-S0118)

- JVET-S0141 item 8: In the general sub-bitstream extraction process, Add AUD_NUT among the NAL unit types that are kept in the output sub-bitstream regardless of its nuh_layer_id value, and add removal of an AUD NAL unit for which all VCL NAL units of an AU are removed by the sub-bitstream extraction. (JVET-S0086)

- JVET-S0141 item 9.a: Add a constraint that the SEI payload of the BP/PT/DUI/SLI SEI messages for any two OLSs with the same layer set are the same. (JVET-S0102)

- JVET-S0141 item 9.b (JVET-S0102):

  - Add a NOTE (or editorial equivalent) to point out that the presence of AUDs is indirectly required for such AUs that contain only IRAP or GDR pictures for the layers of an OLS that contains more than one layer.

  - Specify that when an AUD is present and the AU of the output bitstream of the general sub-bitstream extraction becomes an IRAP or GDR AU, set aud_irap_or_gdr_flag equal to 1.

  - Removing “au_” from the name of aud_irap_or_gdr_au_flag, since it seems redundant.

- JVET-S0141 item 9.c: Place PT SEI messages in individual SEI NAL units when general_same_pic_timing_in_all_ols_flag is equal to 1. (JVET-S0102 aspect 3)

- JVET-S0141 item 9.d: Require that the scalable-nested and non-scalable-nested BP SEI messages in a CVS have the same values for the five syntax elements which specify lengths of various u(v) syntax elements. (JVET-S0102)

- JVET-S0141 item 11: Modify the subpicture sub-bitstream extraction process as follows (JVET-S0117):

  - Remove the rewriting of PPS conformance window syntax elements.

  - Rewrite the subpicture ID mapping in PPS by removing the syntax elements pps_subpic_id[ j ] in all the referenced PPS NAL units, for each j that is not equal to subpicIdx. If the extracted subpicture is not the first subpicture and the original encoder did not explicitly provide the mapping information, then the mapping needs to be provided in the rewritten SPS.

  - Rewrite virtual boundary syntax elements in the SPS (when they are present).

- JVET-S0141 item 12: Change HRD related descriptions in various places from being picture-specific to be AU-specific. (JVET-S0157 item 1)

- JVET-S0141 item 13: In subclause D.3.2, modify the constraint related to bp_alt_cpb_params_present_flag to treat GDR picture with ph_recovery_poc_cnt equal to 0 consistently the same as an IRAP picture. (JVET-S0157 item 2)

- JVET-S0141 item 14: In subclause D.4.2, in the constraint related to requiring pt_cpb_alt_timing_info_present_flag to be equal to 0, add that the RASL pictures in the constraint are RASL pictures that do not have mixed NAL unit types as follows. (JVET-S0157 item 4)

- JVET-S0141 item 15: In subclause D.5.2, in the semantics of pt_display_elemental_periods_minus1, change fixed_pic_rate_within_cvs_flag[ TemporalId ] (3 instances) to fixed_pic_rate_within_cvs_flag[ Htid ]. (JVET-S0157 item 5)

- JVET-S0141 item 16: Replace the constraint "When fixed_pic_rate_within_cvs_flag[ Htid ] is equal to 0 or sps_field_seq_flag is equal to 1, the value of pt_display_elemental_periods_minus1 shall be equal to 0" with "When sps_field_seq_flag is equal to 1, the value of pt_display_elemental_periods_minus1 shall be equal to 0". (JVET-S0175 aspect 3)

- JVET-S0141 item 17: Replace the first two instance of fixed_pic_rate_general_flag[ i ] in the semantics of elemental_duration_in_tc_minus1[ i ] with fixed_pic_rate_within_cvs_flag[ i ]. (JVET-S0175 aspect 1)

- JVET-S0141 item 18 (JVET-S0175 aspects 4 and 5):

  - Replace the paragraph immediately after NOTE 4 of subclause C.1 with the following: "When HRD conformance testing of a bitstream is performed, all DCI NAL units, when available, all VPSs, SPSs, PPSs, and APSs referred to in the VCL NAL units, and appropriate BP, PT, and DUI SEI messages shall be conveyed to the HRD, in a timely manner, either in the bitstream (by non-VCL NAL units), or by other means not specified in this Specification (such as system configuration and timing information)." (JVET-S0175 aspects 4 and 5)

  - Establish the modified constraint (that displayElementalPeriods shall be an integer multiple of elementalOutputPeriods, and for each AU, the smallest value of displayElementalPeriods among the frame field information SEI messages of the output layers shall be equal to elementalOutputPeriods) and use u(8) instead of u(4) for (ffi_)display_elemental_periods_minus1 and pt_display_elemental_periods_minus1, but not otherwise modify the syntax of the PT SEI message.

- JVET-S0141 item 19: Enable signalling of fixed picture rate information without HRD parameters signalled in the bitstream. (JVET-S0175 aspect 6)

- JVET-S0141 item 20: Conditionally signal bp_sublayer_initial_cpb_removal_delay_present_flag, only when bp_max_sublayers_minus1 is greater than 0 and infer it to 0 otherwise. (JVET-S0181 proposal 1, JVET S0157 item 3)

- JVET-S0141 item 21: Move bp_max_sublayers_minus1 and use it to condition bp_cpb_removal_delay_deltas_‌present_flag, and infer bp_cpb_removal_delay_deltas_present_flag equal to 0 when not present. (JVET-S0181 proposal 2)

- JVET-S0141 item 22: Put pt_cpb_removal_delay_minus1[ bp_max_sublayers_minus1 ] first, followed by similar information for sub-layers, followed by pt_dpb_output_delay, followed by the rest of the syntax in the current order. (JVET-S0185 proposal 1)

- JVET-S0141 item 23: Move the signalling of syntax element bp_alt_cpb_params_present_flag to locate it near the syntax element bp_use_alt_cpb_params_flag where it is used. (JVET-S0185 proposal 2)

- JVET-S0141 item 24: Clarify that decrementing of the DPB fullness by one occurs only once for one removal, and remove redundant conditions that trigger the invocation of the "bumping" process. (JVET-S0198)

- JVET-S0141 item 25: In subclause 8.1.1, add the derivation of the variables DuHrdPreferredFlag and DecodingUnitHrdFlag, similarly as in HEVC. (S0248 aspect 2)

- JVET-S0141 item 26: In subclauses C.2.2 and C.2.3, change the local variable decodingUnitParamsFlag to be a global variable DecodingUnitParamsFlag, to address the issue that decodingUnitParamsFlag was used in subclause C.2.2 without being defied/initialized. (S0248 aspect 3)

- JVET-S0141 item 27: In subclause C.3.2, change the condition on when to remove decoded pictures neither needed for referecne nor needed for output such that it is applied to all pictures except for the first picture of AU 0. (S0248 aspect 4)

- JVET-S0141 item 28: In subclause C.5.2.2, change "Otherwise (the current AU is not a CVSS AU or the current AU is a CVSS AU that is not AU 0 but the current picture is not the first picture of the current AU)" to be "Otherwise". (S0248 aspect 5)

- JVET-S0141 item 29: In subclause C.5.2.3, remove "AU n containing " from the first sentence, i.e., "The processes specified in this subclause happen instantaneously when the last DU of AU n containing the current picture is removed from the CPB." (S0223, S0248 aspect 6)

- JVET-S0141 item 30: In subclause D.3.2, add the following constraint to disallow the combination of alternative timing for the CRA/DRAP case and the decoding unit HRD mode: When bp_alt_cpb_params_present_flag is equal to 1, the value of bp_du_hrd_params_present_flag shall be equal to 0. (S0248 aspect 7)

- JVET-S0141 items 31, 31.a, 31.b: Fix an asserted missing integration of an agreement (from JVET-Q0394) that specifies that non-scalable-nested HRD-related SEI messages apply to the OLSs that include the same set of layers as the entire bitstream (instead of only to the 0th OLS). (JVET-S0178 aspect 1)

  - Consequently, add a constraint such that when there is no OLS that includes the set of layers that is the same as the entire bitstream, there shall be no non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI). (JVET-S0178 aspect 1.a)

  - And remove the constraint requiring the value of nuh_layer_id for an SEI NAL unit containing non-scalable-nested HRD-related SEI messages to be equal to vps_layer_id[ 0 ], such that the value of nuh_layer_id for such SEI NAL units is unconstrained, same as for the nuh_layer_id for DCI, VPS, AUD, and EOS NAL units. (JVET-S0178 aspect 1.b)

- JVET-S0141 item 32: Remove the payloadType value 203 (SLI, subpicture level information) from the VclAssociatedSeiList. (JVET-S0178-v3 aspect 2)

- JVET-S0141 item 34: Add a constraint such that when general_same_pic_timing_in_all_ols_flag is equal to 1, there shall be no SEI NAL units that contain a scalable-nested SEI message with payloadType equal to 1 (PT). (JVET-S0178-v3 aspect 4)

- JVET-S0141 item 35: Modify an existing constraint, to add the SLI SEI message, such that when an SEI NAL unit contains a non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, or 203. (JVET-S0178-v3 aspect 5)

- JVET-S0141 item 36: Modify an existing constraint, to add the SLI SEI message, that when an SEI NAL unit contains a scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, 203, or 133 (scalable nesting). (JVET-S0178-v3 aspect 6)

- JVET-S0141 item 37: Add a constraint such that when an SEI NAL unit contains an SEI message with payloadType equal to 3 (filler payload), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 3. (JVET-S0178-v3 aspect 7 / S0102 aspect 3)

- JVET-S0141 item 38: Add a constraint such that when an SLI SEI message and a BP SEI message that apply to a particular OLS are present within an AU, the SLI SEI messages shall precede the BP SEI message in decoding order. (JVET-S0178-v3 aspect 8)

- JVET-S0141 item 39: Modify semantics of sn_num_subpics_minus1 to clarify the association of scalable-nested SEI messages for subpictures to either layers or OLSs and that the syntax element is about the subpictures of the layers with multiple subpictures per picture. (JVET-S0173 aspect 2 / JVET-S0177 aspect 7)

- JVET-S0141 item 40.b: Change the semantics of sn_subpic_id_len_minus1 by changing CLVS to CVS. (JVET-S0173 aspect 2)

- JVET-S0141 item 41: Disallow containing of filler payload SEI messages in a scalable nesting SEI message. (JVET-S0177 aspect 2)

- JVET-S0141 item 42: Constrain that, when a scalable nesting SEI message contains one or more subpicture level information (SLI) SEI messages, the value of sn_ols_flag shall be equal to 1, and the value of sn_subpic_flag shall be equal to 0. (JVET-S0177 aspect 3)

- JVET-S0141 item 43: Modify an existing constraint, to include the SLI message, such that when a scalable nesting SEI message contains a BP, PT, DUI, or SLI SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI). (JVET-S0177 aspect 4)

- JVET-S0141 item 44: Modify an existing constraint, to include recently added SEI messages, such that when a scalable nesting SEI message contains an SEI message that has payloadType not equal to 132 (decoded picture hash), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to PREFIX_SEI_NUT. (JVET-S0177 aspect 5)

- JVET-S0141 item 45: Add a constraint such that when a scalable nesting SEI message contains an SEI message that has payloadType equal to 132 (decoded picture hash), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to SUFFIX_SEI_NUT. (JVET-S0177 aspect 6)

- JVET-S0141 item 47: In the general sub-bitstream extraction process, specify the conditions under which an output sub-bitstream is required to be a conforming bitstream such that the value of tIdTarget is specified to be in the range of 0 to vps_ptl_max_tid[ vps_ols_ptl_idx[ targetOlsIdx ] ], inclusive (instead of 0 to 6 inclusive). (JVET-S0158 aspect 1)

- JVET-S0141 item 48: Specify the general sub-bitstream extraction process such that it would remove, from the output bitstream outBitstream, SEI NAL units that contain a scalable nesting SEI message with sn_ols_flag equal to 0 while the applicable layers as indicted in the scalable nesting SEI message does not include any layer in the target OLS. (JVET-S0158 aspect 2)

- JVET-S0141 item 49.a and 49.b: Make the following changes to the 4th step in the general sub-bitstream extraction process (JVET-S0158 aspect 3):

  - Remove the redundant condition "nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive".

  - Classify SLI SEI messages along with BP, PT, and DUI in this step of the extraction process.

- JVET-S0141 item 50: Make the following changes to the last step in the general sub-bitstream extraction process (JVET-S0158 aspect 4):

  - Change the condition as follows: When the list LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all VCL NAL units in the bitstream inBitstream.

  - When at least one layer is removed by the extraction process, also remove all SEI NAL units that contain a non-scalable-nested SLI SEI message (same as for BP, PT, and DUI SEI messsages).

  - Insert SEI NAL units to directly contain those SEI messages that were scalable-nested HRD-related SEI messages that apply to the output bitstream, and remove their original container SEI NAL units from the output bitstream. When the target OLS includes only one layer, apply the same for scalable-nested non-HRD-related SEI messages.

- JVET-S0141 item 51: Clarify the scope of Subpicture Level Information (SLI) SEI to be CVS instead of CLVS. (JVET-S0173 item 1)

- JVET-S0141 item 52: Include all layers of the OLS in MinCr related constraint for subpictures. (JVET-S0173 item 3)

- JVET-S0141 item 53: Modify the inference rule for sli_ref_level_fraction_minus1[ i ][ j ]. (JVET-S0173 item 5)

- JVET-S0141 item 54: Resolve editorial notes related to subpictures as follows (JVET-S0173 item 6):

  - Refer to minCR and fR derivation in A.4.2.

  - Replace PicSizeInSamplesY with PicSizeMaxInSamplesY and clarify that the picture size of the highest layer with subpictures in the OLS is taken.

  - Start the minCR and fR variable names with capital letters since they are referred to in multiple clauses.

- JVET-S0141 item 55: Replace sps_num_subpics_minus1 with sli_num_subpics_minus1 in semantics of sli_num_ref_levels_minus. (JVET-S0173 item 7, S0176)

- JVET-S0141 item 56: Add sps_log2_ctu_size_minus5 to the constraint on subpicture layout alignment. (JVET-S0173 item 4)

- JVET-S0141 item 57: Add sli_max_sublayers_minus1, sli_sublayer_info_present_flag, and a loop for sublayers for the signalled fractions and reference level indicators, to support levels for sublayers. (JVET-S0176 item 1)

- JVET-S0141 item 58: Allow SLI SEI messages to be available either in the bitstream or provided through an external means not specified in this Specification. (JVET-S0176 item 2)

- JVET-S0141 item 59: Change the persistency scope for SLI SEI from one CVS to one or more CVSs, to be consistent with VPSs and SPSs wherein level information are or may be signalled. (JVET-S0176 item 3)

- JVET-S0141 item 60: Change the definition of subpicture sequence to cover the case when there is one or more layers with single subpicture per picture. (JVET-S0176 item 4)

- JVET-S0141 item 61: Require that, when an SLI SEI message is present for a CVS, the value of sps_num_subpics_minus1 shall be the same for all the SPSs referenced by the pictures in the layers with multiple subpictures per picture. (JVET-S0176 item 5)

- JVET-S0141 item 62: Specify the semantics of sli_num_subpics_minus1 in a way such that the syntax element is about the subpictures of the layers with multiple subpictures per picture. (JVET-S0176 item 6)

- JVET-S0141 item 63: Make variable derivations subpicture sequence specific as follows: Add an array index, which identifies a subpicture sequence, to both the variables SubpicLevelIdc and SubpicLevelIdx, as well as to the arrays SubpicCpbSizeVcl, SubpicCpbSizeNal, SubpicBitRateVcl, and SubpicBitRateNal in the last set of constraints in the semantics of the SLI SEI message. (JVET-S0176 item 7)

- JVET-S0141 item 64: In the subpicture sub-bitstream extraction process, across all layers, for the extraction of subpictures, it should select the subpictures in the same position for the layers that are split into multiple subpictures, and it should select the entire pictures for the layers that are not split into subpictures. (JVET-S0154 aspect 1)

- JVET-S0141 item 65: In the subpicture sub-bitstream extraction process, remove VCL NAL units and their associated filler data NAL units and associated filler payload SEI messages, etc., regardless of whether there is an external means for replacing of the parameter sets. (JVET-S0154 aspect 2)

- JVET-S0141 item 66: In the subpicture sub-bitstream extraction process, remove SEI NAL units containing scalable-nested SEI messages that do not apply to the output bitstream from the output bitstream. (JVET-S0154 aspect 3, S0099)

- JVET-S0141 item 67: In the subpicture sub-bitstream extraction process, specify the subpicture index for identifying the subpicture sequence as the subpicture index of the to-be-extracted subpictures in the layers with multiple subpictures per picture, not the layers with only one subpicture per picture. (JVET-S0154 aspect 4)

- JVET-S0141 item 68: In the subpicture sub-bitstream extraction process, rewrite both general_level_idc and sublayer_level_idc[ k ] for k in the range of 0 to tIdTarget − 1, inclusive, in the referenced VPSs, when present, and in the referenced SPSs, when NumLayersInOls[ targetOlsIdx ] is equal to 1. (JVET-S0154 aspect 5)

- JVET-S0141 item 69: In the subpicture sub-bitstream extraction process, rewrite cpb_size_value_minus1[ k ][ j ] and bit_rate_value_minus1[ k ][ j ] for all values of k in the range of 0 to tIdTarget, inclusive, in the referenced VPSs, when present, and in the referenced SPSs, when NumLayersInOls[ targetOlsIdx ] is equal to 1. (JVET-S0154 aspect 6)

- JVET-S0141 item 70: In the subpicture sub-bitstream extraction process, specify the calculation and rewriting of the scaling window offset parameters as part of the subpicture sub-bitstream extraction process. (JVET-S0154 aspect 7)

- JVET-S0141 item 71: In the subpicture sub-bitstream extraction process, rewrite cbr_flag[ tIdTarget ][ j ] in the referenced VPSs, when present, and in the referenced SPSs, when NumLayersInOls[ targetOlsIdx ] is equal to 1. (JVET-S0154 aspect 8)

- JVET-S0141 item 72: In the subpicture sub-bitstream extraction process, insert SEI NAL units to directly contain those SEI messages that were scalable-nested HRD-related SEI messages that apply to the output bitstream, and remove their original container SEI NAL units from the output bitstream. When the target OLS includes only one layer, apply the same for scalable-nested non-HRD-related SEI messages. (JVET-S0154 aspect 9, S0099)

Tiles, slices, and subpictures:

- JVET-S0145 item 1: Allow the value of pps_no_pic_partition_flag to be different for different PPSs that are referred to by coded pictures within a CLVS. (S0049 aspect 3, S0095 aspect 4)

- JVET-S0145 item 5: Change the semantics of pps_rect_slice_flag to cover the case when rectangular slice is smaller than the tile. (S0095 aspect 5)

- JVET-S0145 item 6: The inference condition of pps_single_slice_per_subpic_flag is changed from "When not present" to "When the value of pps_no_pic_partition_flag is equal to 1" so that pps_single_slice_per_subpic_flag is inferred only when it is needed. (S0095 aspect 6)

- JVET-S0145 item 10: Add a shortcut for signalling of subpictures layout for the case when all the subpictures in each picture of the CVLS have the same size. (S0071)

- JVET-S0145 item 12: Merge the three sets of constraints on the combination of subpictures and scalability, in the semantics of sps_subpic_treated_as_pic_flag[ i ], the semantics of pps_subpic_id[ i ], and in the end of subclause 8.3.2, respectively, to be placed in the end of subclause 8.3.2. (S0258)

- JVET-S0145 item 15: Infer the value of pps_loop_filter_across_tiles_enabled_flag to be equal to 0 (instead of 1) when not present. (S0160 aspect 1)

- JVET-S0145 item 18: Specify that the value of sps_num_subpics_minus1 shall be less than MaxSlicesPerAu. (S0236 aspect 1)

Scalability and RPR:

- JVET-S0147 item 1: When vps_max_layers_minus1 is equal to 0, vps_num_ptls_minus1 is skipped and inferred to be equal to 0. (S0063)

- JVET-S0147 item 2: Set NumSubLayersInLayerInOLS[ i ][ k ] to be equal to vps_ptl_max_temporal_id[ i ] + 1, instead of vps_max_sublayers_minus1 + 1, for output layers in Equation 40. (S0100 aspect 1)

- JVET-S0147 item 4: The values of vps_dpb_max_tid[ vps_ols_dpb_params_idx[ j ] ] and vps_hrd_max_tid[ vps_ols_timing_hrd_idx[ j ] ] shall be greater than or equal to vps_ptl_max_tid[ vps_ols_ptl_idx[ i ] ] for the j-th multi-layer OLS with j in the range from 0 to NumMultiLayerOlss − 1 and i as the index of the j-th multi-layer OLS among all OLSs. (S0100 aspect 3)

- JVET-S0147 item 6: When vps_all_independent_layers_flag is equal to 1 and vps_each_layer_is_an_ols_flag is equal to 1, the value of vps_ols_mode_idc is inferred to be equal to 0. Editor action item: The editor is asked to rephrase the text to avoid the problem. The suggested method is to add an extra “if” condition with a comment to explain why it is there. The change made was to specify a local variable olsModeIdc, which is set equal to vps_ols_mode_idc if vps_each_layer_is_an_ols_flag is equal to 0 and to 4 otherwise, and use olsModeIdc in the syntax condition wherein it is possible to interpret vps_ols_mode_idc as unspecified. (S0129 aspect 1)

- JVET-S0147 item 7: Change the value ranges of the scaling window offsets such that the scaling window width and height can be up to 16 times the picture width and height, respectively. (S0048 aspect 1)

- JVET-S0147 items 8.b, 8.c, 8.d.i: Change the conformance specification for subpicture sequences for the case where higher layers of an OLS contain multiple subpictures while lower layers of OLS do not use subpicture partitioning, as follows: (JVET-S0098)

  - Change the derivation of RprConstraintsActiveFlag[ i ][ j ] to incorporate the number of subpictures in reference picture and current picture, and set RprConstraintsActiveFlag[ i ][ j ] to 1 if the two values differ.

  - Signal a new syntax element non_subpic_layers_fraction[ i ] that specifies the fraction of the bitstream level limits associated with layers in the bitstream that have sps_num_subpics_minus1 equal to 0, for each i.

  - Handle accumulated levels for subpicture sets by removing the accumulation and adjust equation D.11 to use a single subpicture.

- Made some bug fixes, related to JVET-S0098 (SubpicSetLevelIdx −> SubpicLevelIdx), and a missing piece of JVET-R0295 integration in the SLI SEI semantics (index of OlsRefLevelFraction[ i ][ j ] −> [ 0 ][ j ]). (thanks to Robert Skupin for his inputs)

- JVET-S0147 item 9: Change the semantics of the sps_ref_pic_resampling_enabled_flag. (S0048 aspect 2, S0057 aspect 1)

- JVET-S0147 item 13: Change vps_num_output_layer_sets_minus1 to vps_num_output_layer_sets_minus2. (S0183 aspect 1)

- JVET-S0147 item 17: Specify a value range of 0..2 for vps_ols_dpb_bitdepth_minus8[ i ]. (S0212 aspect 2)

- JVET-S0147 item 18: In Equation 41, replace vps_ptl_max_tid[ 0 ] with vps_ptl_max_tid[ vps_ols_ptl_idx[ 0 ] ] and vps_ptl_max_tid[ i ] with vps_ptl_max_tid[ vps_ols_ptl_idx[ i ] ] (for all the instances). (S0248 aspect 1)

Other changes:

- Made some typo/bug fixes and other editorial improvements. (thanks to Du Liu, Peng Yin, and Hendry for their inputs)

- Made some typo/bug fixes and other editorial improvements. (thanks to Zhipin Deng and Miska Hannuksela for their inputs)

Incorporated bug fixes:

- Tickets #1075, #997, #1079, #1066, #1098, #994, #985, #976, #969, #963, #962, #858, #865, #1099, #1125, #1076, #938, #936, #854, #849, #766, #1127, #1128, #1129, #1130, #1131, #887, #895, #920, #933, #939, #951, #946, #956, #974, #975, #989, #990, #1040, #1057, #1056, #1069, #1070, #1081, #1105, #1106, #1118, #1119, #1132, #1149, #1151, #1110, #1133, #1150, #1146, #1087, #1140, #1090, #1107, #1111, #1126, #1135, #1139, #1152, #1153, #1154, #1101, #955, #1156, #1157, #1159, #1160, #1161, #1162, #1165, #1167, #1169, #1170, #1173.

- Tickets #1158, #1163, #1166, #1171, #1172.

- Normative fixes as agreed und discussed during the 19th meeting under 4.2: #986, #1067, #1116, #1124, #1073, #1114, #1059, #845, #623, #1030.

- Tickets #1179, #1181, #1182, #1183, #1184, #1188, #1189, #1193.

- Tickets #1194, #1195, #1196, #1197, #1198, #1199, #1201, #1207, #1212, #1219.

- Tickets #1202, #1203, #1204, #1205, #1206, #1208, #1209, #1210, #1211, #1213, #1214, #1215, #1216, #1217, #1218, #1220, #1221, #1222, #1224, #1225, #1226, #1228, #1229, #1230, #1231, #1232, #1233, #1235, #1236, #1237, #1238, #1239, #1240, #1241, #1242, #1243, #1245, #1246, #1247, #1248, #1249, #1250, #1251, #1252, #1253, #1254, #1255, #1259, #1260, #1262, #1261, #1265, #1266, #1139.

- Fixed ticket #1200, array addressing issues of intra syntax elements, sparse position in MIP to match the original intend as adopted and described in JVET-P0803, added conditions on slices in CABAC init process which have been accidentally removed in vA.

Low-level tool technology:

- JVET-S0215: Align transform skip residual coding syntax with software.

- JVET-S0233: Fixes for ACT to align text with software.

- JVET-S0234: Fixes for ACT in combination with chroma scaling to express exisiting intent.

- JVET-S0222: Incorporated CABAC init values from VTM-9.0 and fixed minor CABAC issues.

Profiles and levels:

- JVET-S0187/m54735: Specify 6 profiles in VVC v1: Main 10, Main 10 4:4:4, Main 10 Still Picture, Main 10 4:4:4 Still Picture, Multilayer Main 10, and Multilayer Main 10 4:4:4 profiles.

- FINB ballot comments: Make high tier support up to 960.

HLS editorial inputs:

- JVET-S0068 aspect 2: Remove a redundant check for deriving the variable signHiddenFlag.

- JVET-S0068 aspect 3: Rename the syntax element sps_internal_bitdepth_minus_input_bitdepth as sps_min_qp_prime_ts.

- JVET-S0096 aspect 5: Correction of the semantics of sh_num_ref_idx_active_override_flag.

- JVET-S0110 aspect 2: Remove "non-IDR" from the NOTE in the reference picture list definition, such that the description is that two reference picture lists, reference picture list 0 and reference picture list 1, are generated for each slice of a picture.

- JVET-S0119: Change the semantics of some VPS, SPS and PPS syntax elements to better express the existing intent by using "may be" instead of "is/are", etc.

- JVET-S0192 aspect 2: Clarify in the sentence on marking an STRP to be an LTRP that an STRP is a picture marked as "used for short-term reference", as in the definition of an STRP.

- JVET-S0192 aspect 3: Add a NOTE clarify that when the bitstream conformance is checked for some of the constraints specified in subclause 8.3.2, the conformance check is to be performed after invoking the decoding process for generating unavailable reference pictures specified in subclause 8.3.4.

- JVET-S0194 aspect 1: Remove a redundant constraint on PPS conformance window syntax elements.

- JVET-S0208:

  - Remove the extra_sh_bits_struct( ) and extra_ph_bits_struct( ) structures and put their content directly in the SPS.

  - Place the check on whether pps_cu_chroma_qp_offset_list_enabled_flag is 1 within the same if-clause in the PPS syntax where pps_cu_chroma_qp_offset_list_enabled_flag is signalled.

  - Remove the definition of “parameter”. It was also suggested to only use the word when referring to things in the VPS, SPS, PPS, APS (not using it for DCI, PTL, GCI syntax elements), or to QPs, or to affine model parameters, or to Rice code parameters, although the final determination of when to use the word is a matter of editorial discretion.

  - Modify the derivation process for the variable TotalNumOlss by removing unnecessary text.

  - Prefix all syntax elements in dpb_parameters( ) with "dpb_".

# JVET-S2002 VVC test model 10 (VTM 10) algorithm and encoder description

One version of JVET-S2002 was published by the Editing AHG between the 19th meeting by teleconference, (22 June – 1 July 2020) and the 20th meeting by teleconference, (7 – 16 October 2020).

JVET-S2002 has been established based on JVET-R2002. It provides the algorithm description for majority of coding tools in VVC. In this editing period, the overall description has been improved.

# JVET-S2007 VSEI text specification (Draft 5)

Seven versions of JVET-S2007 were published by the Editing AHG between the 19th meeting by teleconference, (22 June – 1 July 2020) and the 20th meeting by teleconference, (7 – 16 October 2020).

JVET-S2007 has been established based on JVET-R2007 and now contains the following:

- Fixed ticket #1359.

- JVET-S0172: Add a condition check of gcmp_guard_band_flag, add constraints on gcmp_face_index[ i ] and gcmp_face_rotation[ i ], Include an offset the remapping of sample locations on the half faces of the hemisphere cubemap projection.

- GCI: Move general_non_packed_constraint_flag and general_non_‌projected_‌constraint_‌flag to the VUI. After the general_interlaced_source_flag.

- JVET-S0266: Signal VUI length, make VUI extensible, and add inference values.

- JVET-S0208 aspect 8: Make sure all syntax elements for the SEI messages have a prefix descriptive to the name of the SEI message.

- JVET-S0175 aspect 3: Replace the constraint on ffi_display_elemental_periods_minus1 with “When sps_field_seq_flag is equal to 1, the value of pt_display_elemental_periods_minus1 shall be equal to 0” and say this (or equivalent) in both specs.

- JVET-S0178 aspect 3: Add the clarification on the value ranges of some syntax elements, including potential collisions of the interpretation for values of these syntax elements.

- Editorial changes:

  - Revised semantic constraints for GCMP.

  - Updated the inference of fg_bit_depth_luma_minus8 and fg_bit_depth_chroma_minus8, and added a constraint that when fg_separate_colour_description_present_flag is equal to 0, fg_comp_model_present_flag[ 1 ] and fg_comp_model_present_flag[ 2 ] shall both be equal to 0 for monochrome video.

  - Removal of unnecessary definitions and abbreviations and syntax functions.

  - Rephrasing of definitions to avoid having specific variable names in the definitions.

  - Revision of figures to remove the use of colour and reduce the need for translation of English text.

  - Removal of spaces in syntax element names.

  - Clarification of external means for identifying the number of payload bits and presence of extension data.

  - Clarification of relationship between sample aspect ratio information in VUI parameters and in SEI messages.

  - Rephrasing of NOTEs to avoid “should” and “may”.

  - Rephrasing to avoid using “may” to express uncertainty rather than permission.

  - Rephrasing to use more consistent language for aspects that are “unspecified”.

  - Adding remarks about nominal chroma sample positions described in VVC and HEVC.

  - Adding consideration of monochrome video with quincunx sampling.

  - Removal of reference to undefined concept of default display window parameters.

  - Correction of copy-paste error in semantics of dph_sei_reserved_zero_7bits.

  - Changed the title of this specification to start with the word "Versatile".

  - Updated the scope.

  - Updated some definitions, added a few new definitions, and added a few abbreviations.

  - Moved the definitions of "shall", "may", "should", "informative", "reserved", "unspecified" to subclause 5.1, the beginning of the conventions.

  - Clarified the relationship between the SAR related syntax elements in VUI and the SARI SEI messages.

  - Updated multiple aspects of the semantics of the film grain characteristics SEI message (thanks a lot to Peng Yin, Sean McCarthy, and Taoran Lu for their inputs on these updates):

    - Clarified that the film grain models specified in the film grain characteristics SEI message are expressed for application to decoded pictures that have 4:4:4 colour format with luma and chroma bit depths corresponding to the luma and chroma bit depths of the film grain model and use the same colour representation domain as the identified film grain model, and when the colour format of the decoded video is not 4:4:4 or the decoded video uses a different luma or chroma bit depth from that of the film grain model or uses a different colour representation domain from that of the identified film grain model, an unspecified conversion process is expected to be applied to convert the decoded pictures to the form that is expressed for application of the film grain model.

    - Clarified the semantics of fg_separate_colour_description_present_flag referring to the 6 syntax elements that are present when the value of the flag is equal to 1.

    - Added a constraint such that when VUI parameters are not present for the CLVS or the value of vui_colour_description_present_flag is equal to 0, and equivalent information to that conveyed when vui_colour_description_present_flag is equal to 1 is not conveyed by external means, fg_separate_colour_description_present_flag shall be equal to 1.

    - Consistently use the array format like I[ c ][ x ][ y ] instead of the format like I[ c, x, y ].

    - Clearly describe the derivation of the 8x8 block average and its use in determination of the intensity intervals that apply to each sample of each component.

    - Updated some of the equations to use only the properly defined mathematical functions.

  - In the semantics of the decoded picture hash SEI message, changed the array of the decoded sample values of a component of a decoded picture to be a two-dimension array (from a one-dimension array).

  - In the semantics of the generalized cubemap SEI message, clarified under which conditions the SEI message would have the same functionality as the cubemap projection SEI message specified in in HEVC and AVC.

  - Updated the semantics of the sphere rotation SEI message such that it is clarified that it can be used together with other types of projections, not just those specified in this specification.

  - Updated the semantics of the region-wise packing SEI message such that it is clarified that it can be used together with other types of projections, not just those specified in this specification.

  - In the semantics of the region-wise packing SEI message, clarified that, only when the generalized cubemap SEI message has the same the same functionality as the cubemap projection SEI message specified in in HEVC and AVC, the generalized cubemap SEI message can be used together with the region-wise packing SEI message.

  - In the semantics of the region-wise packing SEI message, added a NOTE to provide more clarify regarding the relative projected picture sample units and the relative region-wise packed picture sample units.

  - Updated the semantics of the omnidirectional viewport SEI message such that it is clarified that it can be used together with other types of projections, not just those specified in this specification.

# Related contribution

- Y.-K. Wang (Bytedance), G. J. Sullivan (Microsoft), “On the syntax design for extending SEI messages and VUI for HEVC, VVC and VSEI,” document JVET-T0048, 20th JVET meeting: by teleconference, 7 – 16 October 2020.

- Y.-K. Wang, Z. Deng (Bytedance), “Some errata items for VVC,” document JVET-T0055, 20th JVET meeting: by teleconference, 7 – 16 October 2020.

# Recommendations

The AHG recommends to:

- Approve the edited JVET-S2001, JVET-S2002 and JVET-S2007 documents as JVET outputs,

- Compare the VVC documents with the VVC software and resolve any discrepancies that may exist, in collaboration with the software AHG,

- Encourage the use of the issue tracker to report issues with the text of both the VVC specification text and the algorithm and encoder description,

- Continue to improve the editorial consistency of VVC text specification and Test Model documents,

- Ensure that, when considering changes to VVC, properly drafted text for addition to the VVC Test Model and/or the VVC specification text is made available in a timely manner,

- Review AHG2 related contributions and act on them if found to be necessary.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 10),” document JVET-S2001, 19th JVET meeting: by teleconference, 22 June – 1 July 2020.

- J. Chen, Y. Ye, S.-H. Kim, “Algorithm description for Versatile Video Coding and Test Model 10 (VTM 10),” document JVET-S2002, 19th JVET meeting: by teleconference, 22 June – 1 July 2020.

- J. Boyce, V. Drugeon, G. J. Sullivan, Y.-K. Wang, “Supplemental enhancement information messages for coded video bitstreams (Draft 5),” document JVET-S2007, 19th JVET meeting: by teleconference, 22 June – 1 July 2020.