| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0007-v2 |
| --- | --- |

| Title: | JVET AHG report: Coding of HDR/WCG Material (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall E. Francois W. Husak S. Iwamura D. Rusanovskyy | Email: | asegall@sharplbas.com Edouard.Francois@InterDigital.com WJH@dolby.com iwamura.s-gc@nhk.or.jp> dmytror@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 19th meeting by teleconference (22 June – 1 July 2020) and the 20th meeting by teleconference (7 – 16 October 2020).

# Mandates

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results.

- Compare the performance of the VTM and HM for HDR/WCG content.

- Generate CTC anchors for the VTM according to JVET-S2011 within two weeks of availability of SDR CTC anchors.

- Study the luma/chroma bit allocation in the HDR CTC, especially for HLG content.

- Coordinate with AHG4 in preparation for verification testing for HDR video content.

- Study additional aspects of coding HDR/WCG content.

# Activities

The primary activity of the AhG was related to the mandates of (i) study and evaluate available HDR/WCG test content, (ii) comparing the performance of the VTM for HDR/WCG content, and (iii) coordinating with AHG4 in preparation for verification testing for HDR video content. This work is described in the following subsection.

## Anchor Generation

The AhG generated CTC anchors for the VTM according to JVET-P2011.  A summary of the performance is provided below, and more detailed information may be found in the included XLS data.

NOTE: It is noted that the Class H2 sequences were updated at the 19th JVET meeting.  This update consisted of changing the chroma sampling location for the sequences.  In the tables below, we report the performance of the VTM10 and VTM9 using the configurations provided in the VTM repository to provide a reference for participants tracking the performance.  Similarly, the HM16.18 results use the VTM9 sequences.  Please note that the difference in Class H2 is largely due to the sequence change.

### VTM 10.0 versus VTM 9.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-9.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.03% | -0.02% | -0.02% | -0.01% | -0.09% | -0.02% | -0.04% | -0.11% | 100% | 101% |
| Class H2 |  |  |  |  |  | 1.78% | -12.52% | 2.87% | 96% | 99% |
| Overall | -0.03% | -0.02% | -0.02% | -0.01% | -0.09% | 0.64% | -4.58% | 0.98% | 98% | 100% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-9.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.08% | -0.07% | -0.07% | -0.07% | -0.08% | -0.07% | -0.07% | -0.08% | 102% | 100% |
| Class H2 |  |  |  |  |  | 2.58% | -16.82% | 14.54% | 99% | 100% |
| Overall | -0.08% | -0.07% | -0.07% | -0.07% | -0.08% | 0.89% | -6.16% | 5.23% | 101% | 100% |

### VTM 10.0 versus HM 16.18

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM-16.18 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -41.23% | -26.74% | -26.21% | -56.67% | -52.06% | -23.54% | -52.57% | -45.24% |  |  |
| Class H2 |  |  |  |  |  | -19.90% | -53.76% | -48.39% |  |  |
| Overall | -41.23% | -26.74% | -26.21% | -56.67% | -52.06% | -22.22% | -53.00% | -46.39% |  |  |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over HM-16.18 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -31.63% | -31.52% | -31.23% | -46.19% | -38.96% | -28.19% | -41.12% | -31.28% |  |  |
| Class H2 |  |  |  |  |  | -26.75% | -63.02% | -56.00% |  |  |
| Overall | -31.63% | -31.52% | -31.23% | -46.19% | -38.96% | -27.67% | -49.08% | -40.27% |  |  |

## Tool-On / Tool-Off Comparisons

In addition to evaluating the performance of VTM 10.0, the AhG also studied the performance of individual coding tools in the context of HDR content.  This was accomplished by conducting a Tool-On/Tool-Off test according to the methodology established in AhG13.

Results are summarized in the tables below.  Additionally, more detailed results are provided in the included XLS data.

The AhG would like to thank the following companies for contributing to the Tool-On tests: Alibaba, Dolby, InterDigital, LG, MediaTek, NHK, and Sharp.

### Class H1 (PQ)

Simulation Results for AI (Class H1)

|  |  |  |  |  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | DE100 | PSNR-L | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 15.98% | 0.98% | 0.88% | 14.33% | 18.93% | 153% | 103% | 149% | 104% |
| DQ | 0.09% | 1.36% | 1.45% | 0.14% | 0.32% | 98% | 97% | 97% | 107% |
| CCLM | 18.98% | 2.33% | 2.15% | 46.70% | 52.70% | 101% | 101% | 101% | 101% |
| MTS | 1.21% | 1.30% | 1.30% | 0.78% | 1.14% | 96% | 96% | 86% | 102% |
| ALF | 10.11% | 2.75% | 2.18% | 18.38% | 37.22% | 97% | 87% | 97% | 90% |
| MRLP | 0.34% | 0.35% | 0.31% | 0.15% | 0.14% | 99% | 101% | 99% | 102% |
| IBC | -0.23% | -0.33% | -0.32% | -0.13% | 0.12% | 144% | 95% | 185% | 103% |
| ISP | 0.14% | 0.61% | 0.70% | -0.40% | -0.12% | 91% | 98% | 86% | 101% |
| LMCS | 2.24% | 1.18% | 0.67% | 2.38% | 1.23% | 100% | 100% | 99% | 101% |
| BDPCM | -0.03% | 0.01% | 0.00% | -0.12% | 0.01% | 102% | 97% | 107% | 101% |
| MIP | 0.52% | 0.73% | 0.58% | 0.16% | 0.16% | 94% | 98% | 88% | 103% |
| LFNST | 0.96% | 0.91% | 0.82% | 1.26% | 2.31% | 106% | 99% | 105% | 103% |
| JCCR | 0.22% | 0.53% | 0.54% | 0.33% | -1.74% | 97% | 100% | 98% | 103% |
| SAO | 0.93% | 0.07% | 0.00% | 1.12% | 2.36% | 101% | 99% | 99% | 99% |
| CCALF | 7.38% | -0.16% | -0.17% | 13.26% | 33.88% | 99% | 98% | 99% | 99% |

Simulation Results for RA (Class H1)

|  |  |  |  |  | VTM RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | DE100 | PSNR-L | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 13.76% | 0.54% | 0.45% | 10.54% | 15.05% | 104% | 104% | 105% | 102% |
| DQ | -0.34% | 1.51% | 1.44% | -1.08% | -0.48% | 97% | 99% | 101% | 104% |
| CCLM | 15.45% | 1.10% | 0.99% | 36.12% | 39.98% | 100% | 102% | 101% | 101% |
| MTS | 0.93% | 1.07% | 1.03% | 0.86% | 1.53% | 99% | 95% | 95% | 101% |
| ALF | 11.63% | 1.95% | 1.63% | 17.71% | 46.73% | 95% | 85% | 96% | 92% |
| AFF | 0.92% | 1.10% | 1.09% | 0.61% | 0.56% | 78% | 102% | 79% | 98% |
| SbTMVP | 0.32% | 0.34% | 0.38% | 0.23% | 0.37% | 104% | 101% | 101% | 100% |
| AMVR | 0.97% | 0.72% | 0.74% | 1.38% | 1.70% | 89% | 101% | 87% | 102% |
| GPM | 0.55% | 0.39% | 0.44% | 1.01% | 1.04% | 97% | 100% | 98% | 102% |
| BDOF | 0.75% | 0.92% | 0.98% | 0.44% | 0.23% | 96% | 99% | 99% | 99% |
| CIIP | 0.05% | 0.12% | 0.15% | -0.10% | -0.02% | 97% | 99% | 99% | 102% |
| MMVD | 0.29% | 0.26% | 0.29% | 0.24% | 0.29% | 91% | 101% | 90% | 101% |
| BCW | 0.70% | 0.26% | 0.22% | 0.42% | 0.65% | 94% | 101% | 95% | 103% |
| MRLP | 0.17% | 0.20% | 0.14% | -0.25% | 0.56% | 100% | 103% | 102% | 104% |
| IBC | 0.30% | -0.08% | -0.01% | 0.16% | 0.90% | 104% | 100% | 110% | 102% |
| ISP | 0.32% | 0.44% | 0.45% | -0.02% | 0.72% | 97% | 99% | 98% | 103% |
| DMVR | 1.24% | 1.09% | 0.95% | 1.31% | 1.40% | 100% | 99% | 102% | 99% |
| SBT | 0.15% | 0.16% | 0.37% | 0.07% | 0.08% | 96% | 101% | 98% | 101% |
| LMCS | 1.11% | 1.37% | 1.07% | 0.68% | 0.42% | 97% | 98% | 99% | 99% |
| SMVD | 0.16% | 0.13% | 0.15% | 0.14% | 0.24% | 97% | 99% | 97% | 101% |
| BDPCM | 0.15% | 0.01% | 0.00% | 0.07% | 0.65% | 101% | 99% | 102% | 101% |
| MIP | 0.58% | 0.51% | 0.34% | 0.12% | 0.96% | 96% | 100% | 98% | 102% |
| LFNST | 0.68% | 0.57% | 0.44% | 0.85% | 2.40% | 97% | 98% | 97% | 101% |
| JCCR | -0.16% | 0.34% | 0.33% | -0.62% | -0.99% | 100% | 99% | 101% | 102% |
| SAO | 0.98% | -0.06% | -0.12% | 1.22% | 2.30% | 100% | 102% | 99% | 99% |
| PROF | 0.05% | 0.06% | 0.08% | 0.01% | -0.17% | 98% | 99% | 100% | 101% |
| CCALF | 9.24% | -0.54% | -0.55% | 14.76% | 42.48% | 101% | 99% | 99% | 98% |

Figure 1. wPSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (Class H1)

Figure 2. wPSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (Class H1)

Figure 3. wPSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (Class H1)

## Coordinating with AHG4

The AHG had significant coordination with AHG4, including sharing results on the new HDR content and expert viewing procedures.

# Contributions

There are three contribution related to HDR video coding.

| JVET-T0082 | New HLG sequences for high bit-depth video coding | K. Kondo, M. Ikeda, and A. Browne |
| --- | --- | --- |
| JVET-T0106 | AHG4/AHG7: Report on Display Settings for HDR/WCG Verification Tests | A. Segall, M. Wien and V. Baroncini |

# Recommendations

The AHG recommends the following:

- Review all input contributions