| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting: by teleconference, 7 Oct – 16 Oct 2020 |  |  | Document: JVET-T0010 |
| --- | --- | --- | --- |
| Title: | JVET AHG report: Encoding algorithm optimizations (AHG10) |  |  |  |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Alberto Duenas Alexis Michael Tourapis Sergey Ikonin Andrey Norkin Rickard Sjöberg J. Le Tanou J.-M. Thiesse | Email: | albertoduenas@fb.com atourapis@apple.com Sergey.Ikonin@huawei.com anorkin@netflix.com rickard.sjoberg@ericsson.com julien.letanou@mediakind.com jean-marc.thiesse@vitec.com |  |
| Source: | AHG |  |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG10: Encoding algorithm optimizations, between the 19th meeting (teleconference, 22 June - 1 July 2020) and the 20th meeting (teleconference, 7-16 October 2020).

# Introduction

At the 19th JVET meeting, the AHG on Encoding algorithm optimizations was established with the following mandates:

- Study the impact of using techniques such as GOP structures and perceptually optimized adaptive quantization for encoder optimization.

- Study encoding techniques of optimization for objective quality metrics and their relationship to subjective quality.

- Study the impact of adaptive quantization.

- Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding.

- Study methods of rate control and rate-distortion optimization and their impact on performance, subjective and objective quality.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). No e-mail related to AHG10 activity was sent to the JVET reflector during the AHG period.

# Overview of input documents related to the AHG

The following input documents were identified to be related to this AHG:

## JVET-T0062: Extension of rate control to support random access configuration with GOP size of 32

This contribution extends the rate control model in the VTM software, that is based on contribution JVET-M0600, to also support the GOP 32 that was presented in JVET-S0180. Compared to a fixed bit ratio rate control model, the extended rate control model presented appears to provide coding gains, in terms of Y-PSNR BD-Rate improvements of ~16% and ~19% for Class C and Class D content respectively.

.

## JVET-T0063: Suggestion to upgrading random access configuration in CTC with GOP 32 configuration

This contribution provides additional information to what was presented at the last meeting in JVET-S0180. In particular,

at the previous JVET teleconference in June, a GOP 32 configuration was included into the VTM. During the interim period between that meeting and this meeting, the GOP 32 configuration was tested for 4K SDR and was selected to be used as part of the verification testing for 4K SDR. In this contribution it is suggested to upgrade all other random access common test conditions, i.e. conditions applicable to HD SDR as well as HDR sequences, to also use a GOP hierarchy of 32. It is asserted that doing so would result in an improved performance for the VTM compared to its current usage. In particular, the BD-rate gains over the current

SDR RA common test conditions of -3.0%, -6.3%, and -6.4% for luma, Cb, and Cr, respectively, are reported. Similarly, for the

HDR RA common test conditions BD-rate gains of -7.9%, -5.2, -4.4%, and -4.5% for deltaE, psnrL, wpsnrY, and psnrY, respectively, are reported.

It is further asserted that visual quality is also improved.

It is not mentioned in the contribution if similar changes should be performed to the anchor reference, i.e. for encodings performed using the HM reference software of HEVC.

## JVET-T0064: Addition of ALF filter strength control to VTM

This contribution asserts that currently, there is no possibility to control the ALF filter strength in the VTM and that it is only possible to turn it on or off completely. This proposal suggests including a configurable ALF filter strength parameter in the VTM reference software, which allows the user to have some control of the ALF filter strength. If the provided ALF filter strength parameter is set to be less than 1, then the magnitude of the filter coefficients is decreased accordingly. Similarly, if the ALF filter strength parameter is set to be larger than 1, then the magnitude of the filter coefficients is increased. Finally, if the ALF filter strength parameter is set to be equal to 1, the current behavior of ALF remains unchanged.

The BDR performance of the proposed modification using the CTC and with the ALF filter strength parameter set to be equal to 0.75, i.e. reducing the filter strength by 25%, is as follows:

AI:    0,27%, 0,25%, 0,31% (Y,U,V)

RA:   0,33%, 0,10%	, 0,16% (Y,U,V)

LDB: 0,30%, 0,11%, 0,23% (Y,U,V)

Further decreasing the filter strength to 50%, by using an ALF filter strength parameter equal to 0.5, has the following impact on BDR:

RA:   0,90%, 0,74%, 0,89% (Y,U,V)

LDB: 0,67%, 0,90%, 1,50% (Y,U,V)

It is claimed that a reduction of the ALF filter strength can, in some cases and especially for inter predictive coding, improve subjective quality.

## JVET-T0078: GDR Software

This contribution provides source code for enabling GDR functionality in the VTM reference software and proposes its inclusion in an upcoming release. The software offers the following functionality:

- Encoding input video sequences into VVC compliant GDR bitstreams using the VTM encoder with the following supported features:

  - Flexible GDR period configured through user specified parameters,

  - Allocation of a similar number of bits per picture, allowing for delays as low as one frame interval,

  - Progressive intra refresh over a GDR period with even distribution of forced intra areas over pictures within the GDR period using the virtual boundary syntax supported in picture header,

  - Application of the necessary (encoding) constraints on coding tools to prevent "leaks" when performing a random access operation, and

  - Exact match at the recovery point (or leak-free).

- Decoding the leak-free GDR bitstream using the VTM decoder.

The VVC specification supports gradual decoding refresh (GDR). However, it is claimed that the VTM reference software does not fully support this functionality. It is claimed that it is desirable to make the source code of the GDR software available and maintained as a part of the VTM package.

## JVET-T0099: Open optimized VVC encoder (VVenC) and decoder (VVdeC) implementations

In September 2020, two months after the first version of VVC was finalized, Fraunhofer HHI released the source code for its optimized VVC encoder (VVenC) and decoder (VVdeC) on GitHub. For VVenC, the goal was to demonstrate comparable or even subjectively better coding efficiency than the VTM reference software at much lower runtime. In addition, VVenC provides real-world encoder features like rate control and multi-threading.

Without QP adaptation for subjective optimization and 6 threads the following PSNR-based YUV BD-rates and speedup factors compared to the HM and the VTM (Figure 1) are reported for four different presets:

Faster		HD: -11.0%, 20x (HM), 160x (VTM)	UHD: -15.9%, 36x (HM), 270x (VTM)

Fast		HD: -25.5%, 16x (HM), 130x (VTM)	UHD: -28.8%, 26x (HM), 200x (VTM)

Medium	HD: -34.4%	, 8.5x (HM), 69x (VTM)	UHD: -37.4%, 14x (HM), 110x (VTM)

Slow		HD: -37.9%	, 2.7x (HM), 22x (VTM)	UHD: -40.6%, 5x (HM), 38x (VTM)

With QP adaptation for subjective optimization and 6 threads, the following MS-SSIM-based YUV BD-rates and speedup factors compared to the HM and the VTM (Figure 1) are reported for the same presets:

Faster		HD: -18.4%, 20x (HM), 160x (VTM)	UHD: -15.4%, 	36x (HM), 270x (VTM)

Fast		HD: -28.4%	, 16x (HM), 130x (VTM)	UHD: -29.3%, 	26x (HM), 200x (VTM)

Medium	HD: -37.5%, 8.5x (HM), 69x (VTM)	UHD: -39.2%, 14x (HM), 110x (VTM)

Slow		HD: -40.2%, 2.7x (HM), 22x (VTM)	UHD: -42.1%, 	5x (HM), 38x (VTM)

Figure 1 – Quality vs Encoding speed tradeoffs achieved using VVenC and the VTM compared HM-16.22

The Fraunhofer Versatile Video Encoder (VVenC) and Decoder (VVdeC) development was initiated to provide publicly available, fast, and efficient VVC software implementations. The VVenC and VVdeC software are based on the VTM, with optimizations including software redesign to mitigate performance bottlenecks, extensive SIMD optimizations, and basic multi-threading support to exploit parallelization.

It is claimed that the VVenC further contains improved encoder search algorithms and supports real-world encoder features, including frame-level rate control and perceptually optimized encoding, and that the bitstreams generated are fully compliant with the VVC specification and its reference decoder (VTM-10.0). Some of the features that this software encoder currently provides include the following:

- Encoder implementation with four predefined quality/speed presets.

- Optimized VVC encoder providing speedups between 22x and 270x over VTM-10.0 for HD and UHD test sequences depending on the chosen quality/speed preset.

- Perceptual optimization to improve subjective video quality.

- Frame-level rate control supporting VBR encoding.

- Expert mode encoder interface available, allowing fine-grained control of the encoding process.

## JVET-T0074: LMCS encoder algorithm for YUV4:4:4 content

This contribution proposes an automatic non-normative encoder optimization algorithm for LMCS to improve coding performance for SDR YUV4:4:4 video content. In VTM10.0, the default LMCS encoder algorithm is optimized for YUV4:2:0 content. However, YUV4:2:0 and YUV4:4:4 signals have very different characteristics. Similarly, YUV4:4:4 camera (natural) content and screen content have different characteristics. The YUV4:4:4 LMCS encoder algorithm proposed in this contribution provides the following coding performance for SDR YUV4:4:4 CTC (reference: LMCS off; test: LMCS on) under VTM10.0 in terms of BDRate and processing time:

- For camera content (YUV4:4:4):

  - AI:  { -0.32%	0.69%	1.04% }, Enc: 104%, Dec: 101%

  - RA:  { -1.13%	2.98%	0.20% }, Enc: 104%, Dec: 100%

- For screen content (YUV4:4:4):

  - AI:  { -1.22%	2.89%	3.08% }, Enc: 104%, Dec: 101%

  - RA:  { -1.64%	-0.01%	0.16% }, Enc: 101%, Dec: 100%

## JVET-T0103: Information on and analysis of the VVC encoders in the SDR UHD verification test (Late)

Document is not available.

Intends to answer the following questions: • Is there any difference in perceptual quality between VTM 9 and 10 decoder compliant bitstreams? • Which objective metrics can model the outcome of the subjective evaluation best?

# Recommendation from AHG

The AHG recommends that the related input contributions are reviewed and to further continue the study of encoding algorithm optimizations in JVET and to coordinate the review with of some of the AHG 12 related to Rice parameter derivation for high bit-depth coding JVET-T0072, JVET-T0085, JVET-T0089 and SIMD implementation of BDOF for high bit-depth coding JVET-T0086.