| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting: by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0013 |
| --- | --- |

| Title: | JVET AHG report: Tool reporting procedure (AHG13) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Jill Boyce (co-chairs) Wei Chen, Yi-Wen Chen, Roman Chernyak, Kiho Choi, Ryoji Hashimoto, Yu-Wen Huang, Hyeongmun Jang, Ru-Ling Liao, Shan Liu (vice-chairs) | Email: | wchien@qti.qualcomm.com jill.boyce@intel.com wei.chen@interdigital.com yiwenchen@kwai.com chernyak.roman@huawei.com kiho14.choi@samsung.com ryoji.hashimoto.te@renesas.com yuwen.huang@mediatek.com hm.jang@lge.com ruling.lrl@alibaba-inc.com shanl@tencent.com |
| Source: | AHG13 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG13: “Tool reporting procedure” between the 19th meeting by teleconference (22 June – 1 July 2020) and the 20th Meeting by teleconference (7 – 16 Oct. 2020). Tool on/off experimental results vs. VTM anchor are provided for the tools specified in JVET-S2005.

# Mandates

The AHG was established with the following mandates:

- Prepare output document JVET-S2005, which describes the methodology of tool-off testing and a list of tools to be tested by identified testers, including non-CTC configurations as appropriate.

- Produce, study and develop improvements of the JVET-R2013 testing condition description for non-4:2:0 colour format coding.

- Provide configurations files, bitstreams, and results of tool-on/tool-off testing.

- Develop and collect test results for additional testing of VVC capabilities.

- Maintain VTM software aspects for memory bandwidth analysis in coordination with AHG3.

- Use the tool usage counts and memory bandwidth usage to study the decoder complexity of features in on/off testing.

- Prepare a report with results of the tests.

# Activities

The initial version of JVET-S2005 “Methodology and reporting template for tool testing” was provided on July 20th.

All tests described in JVET-S2005 were conducted. VTM tool tests were conducted on VTM-10.0 software with VTM configuration by switching off or on specific tool either in configuration files or macros.

The tested tools, testers, and cross-checkers are listed in the tables below.

## Tools included in VTM (Tool off test vs VTM Anchor)

Table 1 List of adoptions included in VTM (Tool off test (unless specified) vs VTM anchor)

| Tool Name | Acronym | Document reference(s) | AI | RA | LD | Tester | Crosscheck |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chroma separate tree | CST | JVET-P0063, JVET-P0406 | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Dependent quantization* | DQ | JVET-P0170, JVET-Q0433, | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Cross-component linear model | CCLM | JVET-O1124, JVET-Q0500, JVET-Q0194, | X | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; leolzhao@ tencent.com) |
| multiple transform set | MTS | JVET-O0294, JVET-O0474, JVET-O0541, JVET-Q0055, JVET-Q0516, | X | X | X | Kiho Choi (kiho14.choi@samsung.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Adaptive loop filter | ALF | JVET-P0162, JVET-P0164, JVET-P0505, JVET-P0554, JVET-P0665, JVET-P1038, JVET-Q0249, JVET-Q0150, JVET-Q0495, JVET-Q0795, | X | X | X | Wei-Jung Chien (wchien@qti.qualcomm.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Affine motion model | AFFINE | JVET-O0070 |  | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; guichunli@ tencent.com) |
| subblock-based temporal merging candidates | SbTMC | JVET-P0385 |  | X | X | Shan Liu (shanl; guichunli@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Adaptive motion vector resolution | AMVR | JVET-O0057 |  | X | X | Shan Liu (shanl; guichunli@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Geometric partition mode | GPM | JVET-Q0806 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Shan Liu (shanl; leolzhao@ tencent.com) |
| Bi-directional optical flow | BDOF | JVET-P0091, JVET-P0519, JVET-P1023, JVET-Q0128, |  | X |  | Kiho Choi (kiho14.choi@samsung.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Combined intra/inter prediction | CIIP | JVET-O0108, JVET-O0681 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Merge with MVD | MMVD | JVET-P1023 |  | X | X | Kiho Choi (kiho14.choi@samsung.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Bi-predictive with CU weights | BCW | JVET-P0280 |  | X | X | Wei-Jung Chien (wchien@qti.qualcomm.com) | Tzu-Der Chuang (peter.chuang@mediatek.com) |
| Multi-reference line prediction | MRLP | JVET-P0418 | X | X | X | Shan Liu (shanl; leolzhao@ tencent.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Intra block copy mode | IBC | JVET-P0400, JVET-P0457, JVET-P1018 | X | X | X | Shan Liu (shanl; xiaozhongxu@ tencent.com) | Wei-Jung Chien (wchien@qti.qualcomm.com) |
| Intra sub-partitioning | ISP | JVET-O0106, JVET-O0341, JVET-O0502 | X | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Decoder motion vector refinement | DMVR | JVET-O0297, JVET-O0590, JVET-O0634 |  | X |  | Hyeongmun Jang (hm.jang@lge.com) | Roman Chernyak (chernyak.roman@huawei.com) |
| Sub-block transform | SBT | JVET-P1026 |  | X | X | Roman Chernyak (chernyak.roman@huawei.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |
| Luma mapping with chroma scaling | LMCS | JVET-P0254, JVET-P0371 | X | X | X | Taoran Lu (tlu@dolby.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Symmetric motion vector difference | SMVD | JVET-O0284, JVET-O0414, JVET-O0567, JVET-O0572 |  | X |  | Yi-Wen Chen(yiwenchen@kwai.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Quantized residual DPCM | BDPCM | JVET-P0059, JVET-Q0089, JVET-Q0110, JVET-Q0785, | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Matrix based intra prediction | MIP | JVET-P0054, JVET-P0199, JVET-P0803, JVET-Q0446 | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Low frequency non-separable transform | LFNST | JVET-P1026, JVET-P0350, JVET-Q0784, | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Joint coding of chrominance residuals | JCCR | JVET-N0054 | X | X | X | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Sampled-adaptive offset | SAO | JVET-Q0441, | X | X | X | Tzu-Der Chuang (peter.chuang@mediatek.com) | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) |
| Prediction refinement using optical flow | PROF | JVET-P0409, JVET-P0057, JVET-P0154, JVET-P0491, JVET-P0653 |  | X | X | Yi-Wen Chen(yiwenchen@kwai.com) | Ru-Ling Liao (ruling.lrl@alibaba-inc.com) |
| Cross component adaptive loop filter | CCALF | JVET-Q0795 | X | X | X | Shan Liu (shanl; xinzzhao@ tencent.com) | Hyeongmun Jang (hm.jang@lge.com) |
| Palette coding mode** | PALLETE | JVET-P0077, JVET-Q0291, JVET-Q0491, JVET-Q0501, JVET-Q0504, JVET-Q0629, JVET-Q0447, JVET-Q0493, JVET-Q0503, JVET-Q0712, | X | X | X | Yung-Hsuan Chao (yunghsua@qti.qualcomm.com) | Yi-Wen Chen(yiwenchen@kwai.com) |
| Adaptive colour transform** | ACT | JVET-P0517, JVET-Q0820, JVET-Q0353, JVET-Q0512, | X | X | X | Xiaoyu Xiu (xiaoyuxiu@kwai.com) | Shan Liu (shanl; xinzzhao@ tencent.com) |

* Test was conducted by disabling DQ and enabling Sign Data Hiding.

** Test was conducted with test sequences and test condition defined in JVET-R2013[1].

# Results

The results of the tests are summarized in Table 3-8 below. The attached spreadsheet provides additional data. Table 9 shows tool test results across several VTM versions. The method of computing combined BD-Rate_YUV is similar to the suggested method in JVET-Q2016[2]. Instead of computing PSNR_YUV for each frame and then averaging frame PSNR_YUVs for a sequence, PSNR_YUV is directly calculated from average PSNR_Y, PSNR_U, and PSNR_V. The difference of the two methods is due to neglectable rounding error. Scatter plots (Figs. 1-3) are also provided for the tested tools in random access configuration, comparing PSNR-Y based bd-rate on the Y axis vs. each of Enc runtime ratio, Dec runtime ratio, and a weighted average of Enc and Dec runtime ratio, (Enc + a*Dec)/(a+1), with a configurable weight, a. The exemplary weighting is set to 6 and can be adjusted in the spreadsheet attached to this report.

Full experimental results and configuration files can be found at the link below:

https://hevc.hhi.fraunhofer.de/svn/svn_VVCTestConfig/branches/VTM-10.0/

There was no bitrate or PSNR differences between testers and cross-checkers.

Encoder and Decoder runtime ratios provided by both the testers and cross-checkers are included in the reporting template, to identify if there were significant runtime differences.

Table 3 Simulation results in all intra configuration (AI) of VTM tool tests. (VTM anchor)

|  |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.41% | 9.87% | 9.55% | 147% | 99% | 153% | 104% |
| DQ | 1.71% | 1.41% | 1.24% | 94% | 97% | 94% | 102% |
| CCLM | 1.54% | 13.89% | 14.76% | 100% | 101% | 99% | 97% |
| MTS | 1.33% | 0.98% | 1.05% | 88% | 99% | 95% | 98% |
| ALF | 2.20% | 12.81% | 12.43% | 100% | 92% | 86% | 82% |
| MRLP | 0.33% | 0.12% | 0.14% | 99% | 101% | 100% | 103% |
| ISP | 0.48% | 0.28% | 0.23% | 85% | 99% | 86% | 100% |
| LMCS | 0.96% | 0.39% | 0.70% | 98% | 98% | 99% | 99% |
| MIP | 0.63% | 0.17% | 0.18% | 90% | 101% | 88% | 101% |
| LFNST | 0.97% | 2.02% | 2.28% | 111% | 100% | 107% | 98% |
| JCCR | 0.57% | 0.57% | 0.64% | 99% | 101% | 98% | 102% |
| SAO | 0.01% | 0.15% | 0.20% | 100% | 91% | 101% | 97% |
| CCALF | -0.14% | 9.69% | 8.55% | 100% | 97% | 101% | 101% |

Table 4 Simulation results in random access configuration (RA) of VTM tool tests. (VTM anchor)

|  |  |  |  | RA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.12% | 4.12% | 4.57% | 98% | 99% | 100% | 99% |
| DQ | 1.60% | 0.95% | 0.66% | 97% | 99% | 99% | 102% |
| CCLM | 1.02% | 11.52% | 12.59% | 99% | 100% | 99% | 100% |
| MTS | 0.75% | 0.66% | 0.64% | 90% | 99% | 93% | 99% |
| ALF | 4.34% | 19.31% | 19.06% | 97% | 89% | 95% | 84% |
| AFFINE | 3.10% | 2.30% | 2.15% | 82% | 97% | 81% | 97% |
| SbTMC | 0.43% | 0.29% | 0.38% | 101% | 100% | 99% | 98% |
| AMVR | 1.42% | 2.15% | 2.36% | 85% | 101% | 88% | 104% |
| GPM | 0.67% | 1.14% | 1.16% | 91% | 100% | 97% | 101% |
| BDOF | 0.76% | 0.33% | 0.26% | 98% | 97% | 94% | 99% |
| CIIP | 0.26% | 0.00% | -0.02% | 97% | 99% | 94% | 97% |
| MMVD | 0.52% | 0.44% | 0.49% | 92% | 100% | 92% | 101% |
| BCW | 0.40% | 0.46% | 0.46% | 91% | 100% | 95% | 102% |
| MRLP | 0.17% | 0.08% | 0.10% | 100% | 100% | 98% | 102% |
| ISP | 0.28% | 0.29% | 0.33% | 96% | 100% | 95% | 100% |
| DMVR | 0.83% | 1.11% | 1.14% | 99% | 97% | 100% | 96% |
| SBT | 0.41% | -0.03% | -0.02% | 94% | 99% | 94% | 99% |
| LMCS | 1.38% | 1.14% | 0.99% | 95% | 97% | 93% | 98% |
| SMVD | 0.25% | 0.23% | 0.23% | 97% | 102% | 96% | 101% |
| MIP | 0.33% | 0.35% | 0.37% | 96% | 100% | 94% | 98% |
| LFNST | 0.70% | 0.78% | 1.08% | 95% | 100% | 92% | 99% |
| JCCR | 0.53% | 0.40% | 0.14% | 99% | 100% | 97% | 98% |
| SAO | 0.08% | 0.14% | 0.31% | 97% | 96% | 100% | 98% |
| PROF | 0.48% | 0.14% | 0.08% | 99% | 100% | 99% | 99% |
| CCALF | -0.13% | 13.88% | 13.73% | 100% | 99% | 99% | 99% |

Table 5 Simulation results in low delay B configuration (LDB) of VTM tool tests. (VTM anchor)

|  |  |  |  | LDB |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 0.07% | 1.64% | 2.36% | 97% | 94% | 102% | 97% |
| DQ | 1.64% | 0.36% | -0.04% | 96% | 95% | 100% | 100% |
| CCLM | 0.00% | 3.22% | 3.58% | 100% | 100% | 100% | 98% |
| MTS | 0.61% | -0.02% | -0.14% | 88% | 98% | 98% | 99% |
| ALF | 4.12% | 25.06% | 19.58% | 99% | 92% | 92% | 81% |
| AFFINE | 3.06% | 2.27% | 2.54% | 72% | 92% | 73% | 93% |
| SbTMC | 0.70% | 0.69% | 0.63% | 101% | 97% | 108% | 100% |
| AMVR | 0.56% | 0.70% | 0.98% | 87% | 100% | 90% | 99% |
| GPM | 1.54% | 1.79% | 1.99% | 89% | 103% | 95% | 100% |
| CIIP | 0.43% | 0.41% | 0.82% | 94% | 100% | 93% | 93% |
| MMVD | 0.49% | 0.22% | 0.26% | 90% | 101% | 96% | 98% |
| BCW | 0.24% | 0.20% | 0.52% | 92% | 94% | 117% | 106% |
| MRLP | 0.08% | -0.13% | 0.08% | 99% | 99% | 99% | 97% |
| ISP | 0.11% | 0.04% | 0.47% | 99% | 99% | 100% | 100% |
| SBT | 0.63% | -0.47% | -0.28% | 92% | 99% | 92% | 98% |
| LMCS | 0.94% | -0.30% | -0.15% | 95% | 96% | 95% | 96% |
| MIP | 0.14% | 0.27% | 0.50% | 95% | 101% | 96% | 100% |
| LFNST | 0.27% | 1.20% | 0.89% | 92% | 102% | 93% | 102% |
| JCCR | 0.13% | 1.92% | 2.44% | 100% | 98% | 99% | 100% |
| SAO | 0.11% | 0.43% | 1.23% | 97% | 93% | 100% | 96% |
| PROF | 0.32% | 0.01% | 0.20% | 96% | 97% | 97% | 95% |
| CCALF | -0.15% | 18.58% | 13.79% | 100% | 97% | 101% | 100% |

Table 6 Simulation results for screen coding tools for ClassF and ClassTGM (VTM anchor)

|  |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| IBC Class F | 15.26% | 14.97% | 15.02% | 54% | 99% | 49% | 99% |
| IBC Class TGM | 47.74% | 45.15% | 45.20% | 63% | 101% | 61% | 106% |
| BDPCM ClassF | 0.91% | 0.92% | 0.98% | 98% | 101% | 98% | 98% |
| BDPCM ClassTGM | 1.44% | 1.57% | 1.57% | 100% | 101% | 97% | 98% |
|  |  |  |  | RA |  |  |  |
| IBC Class F | 12.49% | 12.53% | 12.68% | 86% | 100% | 84% | 100% |
| IBC Class TGM | 22.48% | 22.10% | 22.49% | 90% | 101% | 86% | 100% |
| BDPCM ClassF | 0.70% | 0.68% | 0.93% | 100% | 101% | 98% | 97% |
| BDPCM ClassTGM | 0.79% | 1.03% | 0.98% | 100% | 100% | 97% | 97% |
|  |  |  |  | LD |  |  |  |
| IBC Class F | 6.17% | 5.80% | 6.99% | 86% | 103% | 81% | 98% |
| IBC Class TGM | 11.38% | 11.97% | 12.23% | 87% | 107% | 83% | 101% |
| BDPCM ClassF | 0.35% | -0.37% | 0.35% | 100% | 99% | 102% | 100% |
| BDPCM ClassTGM | 0.29% | 0.42% | 0.37% | 100% | 99% | 103% | 106% |

Table 7 Simulation results of coding tools for color space 4:4:4 (VTM anchor)

|  |  |  |  | AI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| PALETTE, YUV | 13.58% | 16.71% | 18.20% | 98% | 107% | 97% | 107% |
| PALETTE, RGB | 15.43% | 15.39% | 14.67% | 98% | 110% | 97% | 109% |
| ACT, RGB, Sensor | 7.91% | 2.53% | 3.54% | 98% | 101% | 97% | 101% |
| ACT, RGB, Computer | 15.33% | 0.74% | 6.68% | 76% | 100% |  |  |
|  |  |  |  | RA |  |  |  |
| PALETTE, YUV | 9.48% | 12.93% | 14.51% | 100% | 100% | 97% | 97% |
| PALETTE, RGB | 10.90% | 11.08% | 10.62% | 101% | 101% | 100% | 100% |
| ACT, RGB, Sensor | 12.41% | 4.94% | 6.00% | 95% | 99% | 93% | 96% |
| ACT, RGB, Computer | 26.42% | 4.00% | 11.16% | 87% | 99% |  |  |
|  |  |  |  | LD |  |  |  |
| PALETTE, YUV | 5.16% | 8.47% | 9.80% | 97% | 99% | 93% | 95% |
| PALETTE, RGB | 5.74% | 5.95% | 5.79% | 97% | 99% | 97% | 98% |
| ACT, RGB, Sensor | 17.76% | 7.41% | 8.29% | 93% | 95% | 95% | 95% |
| ACT, RGB, Computer | 38.43% | 6.80% | 17.92% | 91% | 102% |  |  |

Table 8 Luma sample usage and memory bandwidth results of VTM tool “off” test. (VTM anchor)

|  | AI |  | RA |  |  | LDB |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | Sample usage | Sample usage | Ave mem BW | Max mem BW | Sample usage | Ave mem BW | Max mem BW |
| CCLM | 48.48% | 3.73% |  |  | 0.76% |  |  |
| ALF | 99.00% | 70.77% |  |  | 67.66% |  |  |
| AFFINE |  | 17.84% |  |  | 26.99% |  |  |
| SbTMC |  | 10.40% |  |  | 12.92% |  |  |
| AMVR |  | 5.43% |  |  | 2.51% |  |  |
| GPM |  | 2.44% |  |  | 6.00% |  |  |
| BDOF |  | 44.99% |  |  |  |  |  |
| CIIP |  | 0.90% |  |  | 1.55% |  |  |
| MMVD |  | 6.84% |  |  | 8.44% |  |  |
| BCW |  | 9.94% |  |  | 8.28% |  |  |
| MRLP | 6.51% | 0.59% |  |  | 0.23% |  |  |
| DMVR |  | 40.35% |  |  |  |  |  |
| SBT |  | 2.61% |  |  | 4.27% |  |  |
| SMVD |  | 2.89% |  |  |  |  |  |
| MIP | 23.76% | 5.09% |  |  | 2.40% |  |  |
| LFNST | 9.86% | 0.79% |  |  | 0.35% |  |  |
| JCCR | 10.95% | 0.50% |  |  | 0.11% |  |  |
| SAO | 31.39% | 6.92% |  |  | 7.76% |  |  |

Table 9 test results of VTM tool “off” test on various VTM versions

|  |  |  |  | VTM | RA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Abbreviation | VTM3 | VTM4 | VTM5 | VTM6 | VTM7 | VTM8 | VTM9 | VTM10 |
| CST | 0.72% | 1.08% | 1.22% | 0.96% | 0.88% | 0.80% | 0.96% | 0.96% |
| DQ | 1.41% | 1.39% | 1.27% | 1.27% | 1.28% | 1.41% | 1.42% | 1.42% |
| CCLM | 3.94% | 4.01% | 3.84% | 3.57% | 3.60% | 3.26% | 3.43% | 3.43% |
| MTS | 1.25% | 0.82% | 0.37% | 0.68% | 0.70% | 0.71% | 0.73% | 0.73% |
| ALF | 3.61% | 3.71% | 4.78% | 4.65% | 4.63% | 7.06% | 7.18% | 7.18% |
| AFFINE | 2.43% | 2.47% | 2.39% | 2.82% | 2.80% | 2.80% | 2.93% | 2.93% |
| SbTMC | 0.52% | 0.43% | 0.40% | 0.48% | 0.43% | 0.43% | 0.41% | 0.41% |
| AMVR | 0.97% | 1.11% | 1.13% | 1.60% | 1.59% | 1.56% | 1.60% | 1.60% |
| GPM | 0.43% | 0.43% | 0.40% | 0.39% | 0.44% | 0.74% | 0.77% | 0.77% |
| BDOF | 1.02% | 0.63% | 0.67% | 0.67% | 0.66% | 0.66% | 0.66% | 0.66% |
| CIIP | 0.43% | 0.51% | 0.32% | 0.24% | 0.23% | 0.22% | 0.20% | 0.20% |
| MMVD | 0.81% | 0.52% | 0.59% | 0.52% | 0.51% | 0.51% | 0.50% | 0.50% |
| BCW | 0.48% | 0.45% | 0.46% | 0.43% | 0.41% | 0.40% | 0.41% | 0.41% |
| MRLP | 0.24% | 0.18% | 0.17% | 0.18% | 0.14% | 0.12% | 0.15% | 0.15% |
| ISP |  | 0.24% | 0.12% | 0.20% | 0.30% | 0.28% | 0.28% | 0.28% |
| DMVR |  | 0.80% | 0.87% | 0.87% | 0.89% | 0.88% | 0.90% | 0.90% |
| SBT |  | 0.33% | 0.34% | 0.31% | 0.31% | 0.31% | 0.32% | 0.32% |
| LMCS |  | 0.64% | 0.61% | 0.97% | 1.36% | 1.32% | 1.32% | 1.32% |
| SMVD |  | 0.26% | 0.24% | 0.27% | 0.26% | 0.26% | 0.24% | 0.24% |
| MIP |  |  | 0.28% | 0.32% | 0.37% | 0.34% | 0.34% | 0.34% |
| LFNST |  |  | 0.75% | 0.60% | 0.74% | 0.75% | 0.75% | 0.75% |
| JCCR |  |  | 0.28% | 0.35% | 0.32% | 0.41% | 0.41% | 0.40% |
| SAO | 0.80% | 0.63% | 0.16% | 0.13% | 0.12% | 0.10% | 0.11% | 0.11% |
| PROF |  |  |  | 0.41% | 0.39% | 0.38% | 0.40% | 0.40% |
| CCALF |  |  |  |  |  | 2.30% | 2.53% | 2.53% |

Figure 1. PSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (VTM anchor)

Figure 2. PSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (VTM anchor)

Figure 3. PSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (VTM anchor)

# Recommendations

The AHG recommends the following:

- Consider the reported tool test results during tool adoption decision making

- Refine list of tested tools and test methodology for the next meeting cycle

# Reference

[1] Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-R2013, by teleconference, 15–24 April 2020

[2] K. Andersson, F. Bossen, J.-R. Ohm, A. Segall, R. Sjöberg, J. Ström, G. J. Sullivan, “Summary information on BD-rate experiment evaluation practices”, JVET-Q2016, Brussels, BE, 7–17 January 2020