| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 7 – 16 October 2020 | Document: JVET-T0043 |
| --- | --- |

| Title: | Status Report on SDR HD Verification Test Preparation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com |
| Source: | Verification Test Coordinators |  |  |

_____________________________

# Abstract

For preparation of the formal verification tests of VVC, rate points for the SDR HD test sequences under consideration in the VVC verification test plan (draft 3) are proposed. Results of a remote experts viewing session on these test sequences are reported. Based on the evolvement of the indicative MOS plots it is suggested that the selection of sequences should be revised. Further, the results indicate that the QPs for the sequences need to be re-adjusted.

# Introduction

The draft plan for VVC verification testing (Draft 3) JVET-S2009 [1] includes a set of SDR HD test sequences under consideration for use in the formal verification tests of VVC. According to the timeline, a preselection and preparation of test sequences and corresponding rate points for viewing has been carried out for the six test sequences listed in JVET-S2009. A remote experts viewing session has been conducted on these test sequences. The results are provided in Section 4 of in this document.

# Test sequences

The set of test sequence candidates in HD resolution for the SDR category as investigated at the 19th JVET meeting is listed in the table below. The Meridian sequence was added compared to the JVET-S2009 set.

Table 1 – SDR HD test sequences under consideration

| No. | Test sequence | Resolution | fps | Frames | md5 |
| --- | --- | --- | --- | --- | --- |
| 11 | Rowing | 1920x1080 | 60 | 603 | 77534c15d75ef495b3f2edb809f2e47b |
| 12 | Show | 1920x1080 | 25 | 248 | 4ea64ce60a5642f41a18276050a62aa1 |
| 13 | Stem | 1920x1080 | 25 | 250 | 9e17f565a8c1beb46a5bc02e680c64ff |
| 14 | Waterfall | 1920x1080 | 60 | 603 | a463beab434c49031ad75037d1769d41 |
| 15 | Witcher1 | 1920x1080 | 30 | 300 | f099b22d6bdf479d0c7d32e821255515 |
| 16 | Meridian | 1920x1080 | 30 | 300 | 514bdb1932c11352f024edc63e524d1d |

The sequences were encoded using VTM-9.1 with the Low-Delay B configuration for QPs 26:4:46 and using HM-16.20 with the Low-Delay B configuration for QPs 24:1:46.

The rate points to be used in the visual tests were selected by choosing the QPs such that the quality range from very low quality to very good quality is covered. For each VTM/HM pair at a given rate point the QPs were selected such that they would represent approximately same quality while. At the same time, approximate rate matching of the HM rate point with the next VTM rate point was pursued. The goal is to enable both, assessment of rate-savings at similar quality and quality improvement at similar rate. The set of proposed QPs is listed in the table below.

Table 2 – SDR HD test sequences under consideration

| No. | Test sequence | Resolution | HM-16.22 QP | VTM-9.0 |
| --- | --- | --- | --- | --- |
| 11 | Rowing | 1920x1080 | 28, 32, 36, 40, 44 | 30, 34, 38, 42, 46 |
| 12 | Show | 1920x1080 | 28, 32, 37, 41, 45 | 30, 34, 38, 42, 46 |
| 13 | Stem | 1920x1080 | 24, 28, 32, 37, 41 | 26, 30, 34, 38, 42 |
| 14 | Waterfall | 1920x1080 | (27), 31, 35, 39, 43 | (30), 34, 38, 42, 46 |
| 15 | Witcher1 | 1920x1080 | 28, 32, 36, 40 | 30, 34, 38, 42 |
| 16 | Meridian | 1920x1080 | 28, 32, 36, 40 | 30, 34, 38, 42 |

The table shows that it was only possible to select four rate points for some test sequences in this initial experiment. This was due to the quality range constraint and the availability of bitstreams. Upon determination of the suitability of test sequences and the corresponding overall quality range, additional intermediate QP values need to be determined.

For the test sequence Waterfall, the bitstream for highest rate point was not assessed due to a temporary bitstream availability issue.

## Preparation of remote viewing assessment

### Remote experts viewing setup and protocol

It was decided to assess the impairment induced by HM and VTM encoding using the Experts Viewing Protocol of ITU-R BT.500-14 [2]. Participants were required to have a suitable computer which is capable of smoothly playing mp4-files in HD resolution. The participants further were required to perform the viewing on a monitor (or TV set) of 32” size or larger. UHD 55” OLED TV sets were recommended.

The test design includes the presentation of basic test cells of the following form

BTC:

- 2 sec	Text “Source” on mid-grey background

- 10 sec	Playback of uncompressed source file

- 2 sec	Text “A” on mid-grey background

- 10 sec	Playback of processed video sequence A (HM)

- 2 sec	Text “B” on mid-grey background

- 10 sec	Playback of processed video sequence B (VTM)

- 2 sec	Text “A” on mid-grey background

- 10 sec	Playback of processed video sequence A (HM)

- 2 sec	Text “B” on mid-grey background

- 10 sec	Playback of processed video sequence B (VTM)

- 5 sec	Text “Vote <id>” on mid-grey background

Thereby, each BTC has a duration of 65 sec.

### Transcoding of YUV files

In order to facilitate remote experts viewing remotely in a coordinated session with participants not necessarily attending from their professional environment, playout on conventional state-of-the-art PCs was required. For this purpose, it was decided to transcode all selected bitstreams to HEVC at high bitrate and wrap those in mp4-containers using ffmpeg. For the original test sequences as well as for the decoded YUV files, the following command line was used:

ffmpeg -video_size 1920x1080 -pix_fmt yuv420p10le -r <fps> -i <yuv file> 
-c:v libx265 -crf 10 <output>.mp4

### Preparation of playout

Due to general availability, the VLC media player [3] was selected as the tool for playout. Participants were requested to have the most recent version of this software installed. The recommended configuration of VLC is as follows:

- Go to Preferences, select "Show settings: All" at the bottom right of the window

- Interface

  - Deactivate “Show controls in full screen mode”

- Select Video

  - Activate Fullscreen

  - Deactivate "Show media title on video"

- Select Playlist

  - Deactivate any looping options

It was recommended to turn off other programs and any file synchronizations since they can infer with the playout. Participants should also make sure that they turned off any picture enhancements of the monitor/TV.

It was recommended to have no strong light sources at the viewing spot (e.g. any sunlight through windows), as well as no reflections from light sources on the screen (e.g. windows / desk lights etc.). No light sources should be in the field of view (e.g. windows, lamps, reflections on glass, mirrors or the like). An unstructured background around the screen such as a plain wall is helpful.

# Remote experts viewing sessions

The test data for this session is available on the JVET ftp site using the regular JVET login under the location below:

/ahg/verificationtests/JVET-T_VT/ sdr-hd-remote-experts-viewing-mp4s.zip

The data was made available for download to the participants well before the session. Further, a template for the voting sheets to be filled during the session and an Excel file to enter the voting data was provided to the participants. The total size of the dataset on the ftp site is about 2.4GB. The participants were provided a test playlist before the session to verify the operation of VLC with the downloaded data.

The test design included two sessions consisting of 16 and 15 randomized and numbered BTCs, respectively.

The assessment meeting was held on Tuesday 2020-09-01 starting at 17:00 UTC. The test was remotely conducted using a Zoom meeting for coordination and discussion. Five experts participated in this session. Results of the assessment are provided in Section 4.

## EVP test

Before the start of the session, the test coordinators informed participants on the procedure of the test, the number of test cells, and the voting procedure. The participants were asked to play a training playlist in order to accommodate to the viewing protocol and the expectable quality range. The training consisted of three BTCs representing best, worst, and medium quality. Stable playout by the VLC player was confirmed. It was confirmed that all participants were equipped with the required voting sheets.

After the introduction, the two viewing sessions were conducted. Participants were asked to fill an EVP scoring sheet for each of these sessions using the 11-grade impairment scale of BT.500-14 shown in Fig. 1 below.

Figure 1 – Numerical impairment scale of ITU-T BT.500-14

After completion of each session, the participants were asked to transfer their scores to the provided Excel-sheet. The layout of the voting sheets is shown in Fig. 2 below.

Figure 2 – Layout of voting sheets used in the remote experts viewing sessions

The break between the two sessions was held until all experts agreed to continue the evaluation. The experts confirmed that the total duration of each session was acceptable. For future assessments, it was recommended by the participants not extend beyond 16 test cells in one session with the used protocol.

# Results

The results show the plots of the luma-PSNR over bitrate curves for the selected QPs on the left and the resulting plots with MOS over bitrate curves on the right for the test sequences under consideration. Due to the low number of participants, only the mean values of the MOS are depicted. Confidence intervals would not be sufficiently meaningful at the given number of participants.

It is emphasized that the presented MOS values must be considered indicative. This relates to the fact of small participants numbers, but even more to the fact that participants were in different time zones (US and Europe), using a variety of displays, and were situated in diverse environments. Nonetheless, the results are considered to be very helpful for the task of preparing the final SDR HD verification test setup.

## Rowing

## Show

## Stem

## Waterfall

## Witcher1

## Meridian

Note: For rate point 3 of the HM-bitstream of Meridian, QP34 instead of the intended QP32 was assessed.

# Conclusions

Note that due to the small number of participants and further due to the special conditions due to COVID restrictions and the thereby required remote testing procedure, the results are considered to be preliminary. Nonetheless, the results are considered very helpful in determining the general suitability of the test sequences as well as serving as an indication of the appropriateness of the chosen quality range.

The following observations are noted:

- The results for the test sequence Rowing seem to indicate that it is somewhat difficult to differentiate the quality at mid-range.

- The test sequence Show was noted to have compression artifacts in the source YUV file. It was indicated to be difficult to assess by some expert viewers.

- The results for the test sequence Stem seem to indicate a steep performance drop around the third rate point. Such abrupt quality changes make a reliable quality evaluation very difficult.

- The test sequence Waterfall stays at relatively high MOS values of about 4 even for the highest QP values under test (QP46 for VTM). This may be an indication that compression artifacts are not very obvious on the content of this test sequence.

- The test sequence Witcher1 appears to have a very steep drop in MOS after some critical rate point, saturating at higher rates. The suitable rate range for visual testing of this sequence might be too small for further consideration.

- Meridian has a significantly lower rate range than the other sequences. It is considered useful nonetheless, since gradual changes in quality are well visible over the whole QP range. For this test sequence, one rate point was accidentally misplaced relative to the intended setting in the visual assessment.

Based on the results and the observations listed above, it is suggested to reconsider the suitability of the test sequences Rowing, Stem, and Witcher1 for the subjective evaluation task. For Waterfall an extension of the range towards lower quality should be investigated. For the test sequences Meridian and Show, and adjustment of the rate points is indicated.

It is generally suggested to consistently use 10 bit test sequences. Further, it is suggested to seek harmonization of the duration of the sequences in order to achieve consistent 10sec playout.

# References

- Mathias Wien, Vittorio Baroncini, Andrew Segall, and Yan Ye, “VVC verification test plan (Draft 3),” Doc. JVET-S2009, Joint Video Experts Team of VCEG and MPEG, 19th meeting, July 2020. [link]

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images. [link]

- VideoLan, VLC Media Player, https://www.videolan.org/vlc/index.html

# Annex A – Encoding task

The encoding of the bitstreams used for this activity was kindly performed by Tencent.