| J oint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: by teleconference, 7 Oct – 16 Oct 2020 | Document: JVET-T0047 |
| --- | --- |

| Title: | AHG12: Preliminary common test conditions for high bitdepth video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai Adrian Browne Xiaoyu Xiu | Email: | ikai.tomohiro@sharp.co.jp adrian.browne@sony.com xiaoyuxiu@kwai.com |
| Source: | AHG12 |  |  |

_____________________________

- Abstract

This document proposes common test conditions (CTC), conversion practices and software reference configurations to be used in the context of high bitdepth video coding experiments. The object of this contribution is to provide a comparison point for the JVET-T meeting and the basis for a CTC for future meetings. The co-chairs seek the opinions of experts in a number of areas which will allow the direction of further work to be determined. The co-chairs also call on the JVET community for it’s help in providing and identifying new test sequences

Review of test sequences

AHG12 considers that a useful CTC should contain the following types of content:

- 12 bit HDR (PQ and HLG) sequences

- 16 bit RGB sequences

- 12/16 bit Domain-specific (e.g. medical) sequences

- High frame rate sequences (>= 100Hz)

In addition, the RExt CTC contained a series of tests for synthesized content involving the addition of MSB to the content. The co-chairs would be interested in the opinion of other experts on the relevance of such tests to the new CTC.

AHG 12 has reviewed the HEVC RExt CTC (JCTVC-P1004)[1] and the VVC HDR CTC (JVET-S2011)[2] and would like to suggest:

- Using 12 bit versions of the PQ sequences from the HDR CTC (convert original half float exr to 12bit PQ by HDRTools)

- Keeping the existing SVT 16 bit RGB sequences from the RExt CTC

- Removing all other sequences (until it is confirmed that theyare both available for use and of value to the new CTC)

There are still considerable gaps in the types of sequence in the initial CTC:

- HLG sequences

- Medical or other domain-specific sequences

High frame rate sequencesWe encourage experts to fill these gaps and hope that new sequences will be made available in submissions to the JVET-T meeting.

Test condition for high bit depth coding

Software

The latest VTM 10.0 should be used and the following macro shall be enabled for both anchor and test

#define JVET_R0351_HIGH_BIT_DEPTH_ENABLED                 1

Results are to be reported using the following template: JVET-T0047_AHG.xlsm

The template uses 6 point bdrate calcuration in JVET-H0033[2].

Configuration

The following tests should be conducted.

- Intra only for 12 bit and 16 bit

- Random access for 12 bit

- Low-delay B for 12 bit and 16 bit

Note: In the all intra configuration, TemporalSubsampleRatio shall be set equal to 8.

The following configuration files provided in the cfg/ folder of the VTM, the common software package, shall be used.

- “All Intra” (AI): encoder_intra_vtm.cfg

- “Random access” (RA): encoder_randomaccess_vtm.cfg

- “Low-delay B” (LB): encoder_lowdelay_vtm.cfg

These configuration files specify the QP offsets for intra and inter predicted frames.

Note: For IntraPeriod to reflect the intra refresh period in the random access test cases. The intra refresh period is dependent on the frame rate of the source and the GOP size in use: a value 16 shall be used for sequences with a frame rate equal to 20fps, 32 for 24fps, 32 for 30fps, 48 for 50fps, 64 for 60fps, and 96 for 100fps.

It is recognized that some of these tests, particularly for low delay, may prove to be .  The opinions of experts are sought

In addition, the following config file or tool settings shall be specified per class (VTM only).

| Sequence class | config file |
| --- | --- |
| SVT | cfg/per-class/formatRGB.cfg |
| PQ422 | cfg/444/yuv444.cfg and cfg/per-class/classH1.cfg |
| PQ444 | cfg/444/yuv444.cfg and cfg/per-class/classH1.cfg |

Initial tests on VTM 10.0 have shown that at present LMCS shows significant losses for 12 bit processing of PQ context, and will require a different configuration for 12 bit processing than for 10 bit.  For at least the JVET T meeting LMCS shall be disabled as follows:

--LMCSEnable=0

It is hoped that at subsequent meetings an alternative configuration of LMCS will be made available and allow the tool to be re-enabled. More generally AHG12 believes collaboration with other experts on a series of tool-on / tool-off tests will be beneficial.

High bit depth configuration

When JVET_R0351_HIGH_BIT_DEPTH_ENABLED is enabled, VTM software gains several RExt related configuration flags inherited from HM. In particular, the ExtendedPrecision flag effects the precision of processing from the transforms onwards. For 12 bit tests this flag should always be 0. For 16 bit tests the flag may be either 0 or 1, but care must be taken to compare the results against the correct VTM anchor.

HM anchor

In the HM software, the following macros shall be enabled.

#define RExt__HIGH_BIT_DEPTH_SUPPORT                 1

The RExt tools shall be used in HM anchor in 12bit 444 and 16bit 444. The following configuration files provided in the cfg/ folder of the HM shall be used.

- All intra (AI): encoder_intra_main_rext.cfg

- Random access (RA): encoder_randomaccess_main_rext.cfg

- Low-delay B (LD): encoder_lowdelay_main_rext.cfg

Otherwise (12 bit 422 and 400), The RExt tools shall be disabled as

- All intra (AI): encoder_intra_main10.cfg

- Random access (RA): encoder_randomaccess_main10.cfg

- Low-delay B (LD): encoder_lowdelay_main10.cfg

Note: In addition, Profile shall be additionaly set to avoid encoder cfg error and InternalBitDepth shall be set so as to overwrite 10 bit bit-depth setting in the above encoder_xxx_main10.cfg config files.

Summary for HM anchor configuration (informative)

|  |  | 12bit |  | 16 bit |
| --- | --- | --- | --- | --- |
|  |  | 422 / 400 | 444 | 400 / 444 |
|  | Profile | main-RExt | main-RExt | main-RExt |
| RExt related flags (provided by config file except for ExtendedPrecision) | ExtendedPrecision | 0 | 0 | 1 |
|  | TransformSkipLog2MaxSize | 2 | 2 |  |
|  | ImplicitResidualDPCM | 0 | 1 |  |
|  | ExplicitResidualDPCM | 0 | 1 |  |
|  | ResidualRotation | 0 | 1 |  |
|  | SingleSignificanceMapContext | 0 | 1 |  |
|  | IntraReferenceSmoothing | 0 | 1 |  |
|  | GolombRiceParameterAdaptation | 0 | 1 |  |
|  | HighPrecisionPredictionWeighting | 0 | 1 |  |
|  | CrossComponentPrediction | 0 | 1 |  |
| Base config file |  | encoder_xxx_main10.cfg | encoder_xxx_main_rext.cfg |  |

When comparing HM and VTM results it is essential that the same value of ExtendedPrecision is used for both sets of results.

QP ranges

Table 1 defines the QP ranges to be used for each internal bit-depth. Each QP range includes six QP values.

The column corresponding to each configuration is interpreted as follows:

- “M” indicates that the test sequence is mandatory in the JVET CTC for the given configuration

- “O” indicates that the test sequence is optional (but encouraged) in the JVET CTC for the given configuration

Table 1. QP ranges for high-bit depth testing.

| Testname | Class | Internal bit-depth | QP range | AI | RA | LDB |
| --- | --- | --- | --- | --- | --- | --- |
| 12bit | SVT, PQ422, PQ444 | 12 | 12, 7, 2, -3, -8, -13 | M | O | M |
| 16bit | SVT | 16 | -8, -13, -18, -23, -28, -33 | M |  | M |
| 12bit | PQ422, PQ444 | 12 |  | O | O | O |
| Lossless 16bit | SVT | 16 |  | O |  | O |

For lossless tests, the existing lossless files in the cfg/lossless directory of VTM shall be used. For RGB content this requires the use of both lossless.cfg and losslessRGB.cfg. For other 422 and 444 content this requires the use of lossless.cfg and lossless444.cfg.

Encoding formats

The encoding format is the chroma format of a test sequence and the colour space (only RGB and YCbCr are considered).  The following setting is applied to the encoder for each encoding format:

- InputChromaFormat: Specifies the chroma format of the test sequence. Use “420”, “422”, “444” and “400” for the 4:2:0, the 4:2:2, the 4:4:4, and the 4:0:0 chroma formats, respectively.

For test sequences using the RGB colour space, the following settings are applied to the encoder:

- InputColourSpaceConvert: Must be set to ‘RGBtoGBR’.

- SNRInternalColourSpace: Should be set to 1.

- OutputInternalColourSpace: Should be set to 0.

For test sequences using the RGB colour space, the decoder should be configured with:

- OutputColourSpaceConvert: Should be set to ‘GBRtoRGB’.

Notes:

The settings for SNRInternalColourSpace and OutputInternalColourSpace are optional since they will not affect the coding performance.

Test sequences

For each test sequence, the following setting is applied:

- InternalBitDepth: Set equal to the input bit-depth for a given test sequence.

| Class | Test sequence filename | Encoded frame count |  |  | Input bit-depth | Internal bit-depth |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | AI | RA | LDB |  |  |
| SVT 16 bit from 10 bit source (intermediate processing) | OldTownCross_1920x1080_50_16bit_444.rgb | 100 | 100 | 50 | 16 | 12 and 16 |
|  | CrowdRun_1920x1080_50_16bit_444.rgb | 100 | 100 | 50 | 16 | 12 and 16 |
|  | ParkJoy_1920x1080_50_16bit_444.rgb | 100 |  |  | 16 | 12 and 16 |
|  | DucksTakeOff_1920x1080_50_16bit_444.rgb | 100 |  |  | 16 | 12 and 16 |
|  | InToTree_1920x1080_50_16bit_444.rgb | 100 |  |  | 16 | 12 and 16 |
| PQ422 12 bit from half float exr | BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_422 | 100 | 100 | 50 | 12 | 12 |
|  | FireEater2Clip4000r1_1920x1080p_25_12b_pq_709_ct2020_422 | 100 | 100 | 50 | 12 | 12 |
|  | Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_422 | 100 |  |  | 12 | 12 |
|  | EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_422 | 100 |  |  | 12 | 12 |
|  | EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_422 | 100 |  |  | 12 | 12 |
| PQ444 12 bit from half float exr | BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_444 | 100 | 100 | 50 | 12 | 12 |
|  | FireEater2Clip4000r1_1920x1080p_25_12b_pq_709_ct2020_444 | 100 | 100 | 50 | 12 | 12 |
|  | Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_444 | 100 |  |  | 12 | 12 |
|  | EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_444 | 100 |  |  | 12 | 12 |
|  | EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_444 | 100 |  |  | 12 | 12 |

Table 2. Test sequences

Source for test sequences

- SVT

ftp://filexchange.ebu.ch
/SVT HDTV test set/1080p50_CgrLevels_SINC_FILTER_SVTdec05_/

Note: the conversion from svt to rgb should be conducted following the procedure described in JCTVC-P1006.

- PQ

ftp://ftp.hhi.fraunhofer.de/ahg/testset_hdr

or

ftp://ftp.ient.rwth-aachen.de/ahg/testset_hdr

Note: the PQ sequences must be converted from half float EXR files using HDRTools. Instructions and a sample script for this conversion are available in the 10 bit zip files prepared for the HDR CTC..

Conclusions

This submission proposes a preliminary set of common test conditions for high bit depth, high bit rate and high frame rate coding. The co-chairs seek the opinions of experts in a number of areas which will allow the direction of further work to be determined. The co-chairs call on the JVET community for it’s help in providing and identifying new test sequences.

MD5

- SVT

6004ff26cafcf2b066faa00c935a7621  CrowdRun_1920x1080_50_16bit_444.rgb

2ab943d7b1c6e3b26a2c6da416b69828  ParkJoy_1920x1080_50_16bit_444.rgb

d882efad39ae3d360896ec09d9213424  DucksTakeOff_1920x1080_50_16bit_444.rgb

b5c552e569a907db2ffc1305b52311d5  InToTree_1920x1080_50_16bit_444.rgb

eb85bbec7ccf315bbb44122d10e222bb  OldTownCross_1920x1080_50_16bit_444.rgb

- PQ422

b029dace9f10af78aa6c3363531b1768  BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_422.yuv

dbf81886e8a8b0fe58fb6516cd767b56  FireEater2Clip4000r1_1920x1080p_25_12b_pq_709_ct2020_422.yuv

716c1ec421a72c46011f31754077eec4  Market3Clip4000r2_1920x1080p_50_12b_pq _709_ct2020_422.yuv

b9f22e80fa8eb33e3c00633e6824844f  EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_422.yuv

a480ed6fc9e9f4e6b84b1ab6e7bc4f1b  EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_422.yuv

- PQ444

1664a322cf91934872afd07035819a05  BalloonFestival_1920x1080p_24_12b_pq_709_ct2020_444.yuv

ddfae56b3d9a12a9488f42786c527b7c  FireEater2Clip4000r1_1920x1080p_25_12b_pq_709_ct2020_444.yuv

564904c4001957472483a3d79b55f17d  Market3Clip4000r2_1920x1080p_50_12b_pq_709_ct2020_444.yuv

544b1b8816d0003e69ce6a28bbd01810  EBU_04_Hurdles_1920x1080p_50_12b_pq_709_ct2020_444.yuv

f6761039392e76336493effdc257bf1a  EBU_06_Starting_1920x1080p_50_12b_pq_709_ct2020_444.yuv

References

[1] C. Rosewarne, K. Sharman, D. Flynn, “Common test conditions and software reference configurations for HEVC range extensions”, JCTVC-P1006, JCTVC, 16th Meeting: San José, US, 9–17 Jan. 2014

[2] A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy , “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-S2011, JVET, 19th Meeting: by teleconference, 22 June-1 July 2020

[3] A. M. Tourapis, D. Singer, Y. Su, K. Mammou, “BD-Rate/BD-PSNR Excel extensions”, JVET-H0030, JVET, 8th Meeting: Macao, CN, 18–25 Oct. 2017