| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0048-v2 |
| --- | --- |

| Title: | On the syntax design for extending SEI messages and VUI for HEVC, VVC and VSEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Bytedance Inc. 8910 University Center Lane San Diego, CA 92122, USA Gary Sullivan Microsoft Corp 1 Microsoft Way Redmond, WA 90852 USA | Tel: Email: | +1 (908) 803-3888 yekui.wang@bytedance.com +1 (425) 703-5308 garysull@microsoft.com |
| Source: | Bytedance Inc., Microsoft Corporation |  |  |

_____________________________

# Abstract

This contribution discusses the SEI payload syntax in AVC, HEVC, and VVC/VSEI, and the VUI payload syntax in VVC/VSEI. It reports an asserted bug on the HEVC buffering period SEI message syntax extension and proposes a fix for the asserted bug, and discusses how extensions, when needed, should be specified for the syntax of SEI messages in HEVC and VSEI, and to the syntax of VUI in VSEI.

In v2 of this contribution, a missing tab in the buffering period SEI messag syntax for the first option of the proposed fix was added.

# Introduction

In AVC, the SEI payload syntax is as follows:

| sei_payload( payloadType, payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| ... |  |  |
| else |  |  |
| reserved_sei_message( payloadSize ) | 5 |  |
| if( !byte_aligned( ) ) { |  |  |
| bit_equal_to_one /* equal to 1 */ | 5 | f(1) |
| while( !byte_aligned( ) ) |  |  |
| bit_equal_to_zero /* equal to 0 */ | 5 | f(1) |
| } |  |  |
| } |  |  |

In HEVC (and basically the same in VVC), the SEI payload syntax is different from AVC, in order to enable better extensibility and is as follows:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

With the semantics of reserved_payload_extension_data specified as follows (note particularly the length of this u(v)-coded syntax element):

reserved_payload_extension_data shall not be present in bitstreams conforming to this version of this Specification. However, decoders conforming to this version of this Specification shall ignore the presence and value of reserved_payload_extension_data. When present, the length, in bits, of reserved_payload_extension_data is equal to 8 * payloadSize − nEarlierBits − nPayloadZeroBits − 1, where nEarlierBits is the number of bits in the sei_payload( ) syntax structure that precede the reserved_payload_extension_data syntax element and nPayloadZeroBits is the number of payload_bit_equal_to_zero syntax elements at the end of the sei_payload( ) syntax structure.

Where the functions more_data_in_payload( ) and payload_extension_present( ) are defined as follows:

more_data_in_payload( ) is specified as follows:

–	If byte_aligned( ) is equal to TRUE and the current position in the sei_payload( ) syntax structure is 8 * payloadSize bits from the beginning of the sei_payload( ) syntax structure, the return value of more_data_in_payload( ) is equal to FALSE.

–	Otherwise, the return value of more_data_in_payload( ) is equal to TRUE.

payload_extension_present( ) is specified as follows:

–	If the current position in the sei_payload( ) syntax structure is not the position of the last (least significant, right-most) bit that is equal to 1 that is less than 8 * payloadSize bits from the beginning of the syntax structure (i.e., the position of the payload_bit_equal_to_one syntax element), the return value of payload_extension_present( ) is equal to TRUE.

–	Otherwise, the return value of payload_extension_present( ) is equal to FALSE.

Such a syntax allows extending the syntax of an SEI message by directly adding new syntax elements to the end of the SEI message syntax. For example, such an extension was made to the buffering period (BP) SEI message in the second edition of the HEVC spec, as follows:

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( payload_extension_present( ) ) |  |
| use_alt_cpb_params_flag | u(1) |
| } |  |

During the standardization of VVC and VSEI, similar SEI payload syntax as in HEVC has been inherited not only to the SEI payload syntax but also to the VUI syntax.

The SEI payload syntax in VVC is the same as in HEVC, and the VUI payload syntax, which was newly introduced in VVC (in AVC and HEVC, the vui_parameters( ) syntax structure is directly included into the SPS syntax without a specific VUI extension mechanism), is as follows:

| vui_payload( payloadSize ) { | Descriptor |
| --- | --- |
| vui_parameters( payloadSize ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| if( more_data_in_payload( ) ) { |  |  |
| if( payload_extension_present( ) ) |  |  |
| vui_reserved_payload_extension_data | u(v) |  |
| vui_payload_bit_equal_to_one /* equal to 1 */ | f(1) |  |
| while( !byte_aligned( ) ) |  |  |
| vui_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |  |
| } |  |  |
| } |  |

# Problems of the HEVC BP SEI message extension

One issue related to the HEVC BP SEI message syntax extension relates to the function payload_extension_present( ). Based on the BP SEI message syntax before the addition of the extension, i.e., the two highlighted rows above, when the number of payload bits happens to be a multiple of 8, the payload_bit_equal_to_one bit does not exist, and consequently the function payload_extension_present( ) is not really checkable.

Therefore, the syntax should be changed as follows:

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( more_data_in_payload( ) ) |  |
| if( payload_extension_present( ) ) |  |
| use_alt_cpb_params_flag | u(1) |
| } |  |

There is yet another issue related to the SEI payload syntax after extending the BP SEI message syntax. Before the syntax of an SEI message is extended, the byte alignment trailing bits, i.e., the bits represented by the syntax below, in an SEI payload exist only when the number of bits of the present syntax elements in the SEI message syntax is not a multiple of 8:

| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |

On the other hand, looking at the SEI payload syntax (copied below again for convenience), whenever there is at least one bit of an extension syntax added, the byte alignment trailing bits would be present in the SEI payload, regardless of whether the number of bits of the present syntax elements in the SEI message syntax is a multiple of 8 or not:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

# Proposed fixes

One approach to address both problems discussed above is to make the following changes to the SEI payload syntax and the BP SEI message syntax:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| ExtensionBitsPresentFlag = 0 |  |
| ... |  |
| else |  |
| reserved_sei_message( payloadSize ) |  |
| if( ExtensionBitsPresentFlag ¦ ¦ more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| reserved_payload_extension_data | u(v) |
| payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( more_data_in_payload( ) ) |  |
| if( payload_extension_present( ) ) { |  |
| ExtensionBitsPresentFlag = 1 |  |
| use_alt_cpb_params_flag | u(1) |
| } |  |
| } |  |

Another approach is to make the following changes to the BP SEI message syntax only:

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( more_data_in_payload( ) ) |  |
| if( payload_extension_present( ) ) { |  |
| use_alt_cpb_params_flag | u(1) |
| if( byte_aligned( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| bp_reserved_payload_extension_data | u(v) |
| bp_extension_indication_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| bp_extension_indication_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |
| } |  |

The semantics of newly added syntax elements bp_reserved_payload_extension_data (note particularly the length of this u(v)-coded syntax element), bp_extension_indication_bit_equal_to_one, and bp_extension_indication_bit_equal‌_to_zero are specified as follows:

bp_reserved_payload_extension_data shall not be present in bitstreams conforming to this version of this Specification. However, decoders conforming to this version of this Specification shall ignore the presence and value of bp_reserved_payload_extension_data. When present, the length, in bits, of bp_reserved_payload_extension_data is equal to 8 * payloadSize − nEarlierBits − nPayloadZeroBits − 1, where nEarlierBits is the number of bits in the buffering_period( ) syntax structure that precede the bp_reserved_payload_extension_data syntax element and nPayloadZeroBits is the number of bp_extension_indication_bit_equal_to_zero syntax elements at the end of the buffering_period( ) syntax structure, which is also the end of the sei_payload( ) syntax structure containing the buffering_period( ) syntax structure when bp_extension_indication_bit_equal_to_one is present.

bp_extension_indication_bit_equal_to_one shall be equal to 1.

bp_extension_indication_bit_equal_to_zero shall be equal to 0.

Comparing the two approahces, the advantage of the first approach is that the syntax is simpler and easier to understand, including architecturally clearer by leaving the “SEI trailing bits”, when present, as part of the SEI payload syntax instead of the SEI message syntax. The advantage of the second approach is that no change to the SEI payload syntax is needed.

# Discussion of 2nd and further syntax extensions

Note that all the discussions above, including those in the introduction section, and in this section also apply to extensions to any other SEI messages in HEVC and VSEI, as well as to extensions to the VUIS syntax in VSEI.

For a first extension, similar changes above can be made.

For a second or further extension, it is assumed that the syntax of the first extension will be present, and consequently the payload_bit_equal_to_one bit will always be present, therefore, the new extension syntax can be added directly starting with "if( payload_extension_present( ) )", without the need of adding "if( more_data_in_payload( ) )" again, like the following:

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| use_alt_cpb_params_flag | u(1) |
| if( payload_extension_present( ) ) |  |
| 2nd extension syntax | ... |
| } |  |
| } |  |

# Patent rights declaration(s)

Bytedance Inc. and Microsoft Corporation do not have any current or pending patent rights relating to the technology proposed in this contribution (to the extent of the personal awareness of the authors).