| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0049 |
| --- | --- |

| Title: | Composite Picture Information SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Hyeongmun Jang Seung Hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com hm.jang@lge.com seunghwan3.kim@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes a design for composite picture information (CPI) SEI message. A composite picture is a picture created / generated from rectangular patches from pictures in an access unit. Similar SEI messages were proposed by JVET-S0123 and JVET-S0107. It is asserted that the design proposed in this contribution differs from the previous contributions as follows:

- The proposed design works for composite picture from not only multi layers but also single layer bitstream.

- The proposed design provides more details about handling of issues related to gap and overlap of patches in composite picture.

- In the proposed design, the association between CPI SEI message and its associated output layer set (OLS) is made by carrying CPI SEI message in scalable nesting SEI message. It is asserted that this design is more aligned with how other SEI messages that are associated with OLS.

- The proposed design provides ways for reducing bit cost for signalling.

The design principles of the proposed CPI SEI message are provided in section 2 of this document.

# Introduction

Support for describing composite picture from rectangular patches of reconstructed pictures from a multilayer bitstream using SEI message were proposed by [1][2][3]. In addition to the use case for multi-party video conferencing described in [3], the authors see potential use of composite picture for broadcasting and streaming summarization use-cases.

Figure 1 below illustrates the use of composite picture broadcasting / streaming an online gaming program. For the program, viewer can view each gamer screen, highlighted gamer screen, scoreboard, and also summary of the game in one view using composite display. In this use-case, all part of a picture from each layer is used in the composite picture.

Figure 1 – Use of composite picture in broadcasting / streaming online game program

It is also possible that not all part of a picture is used in a composite picture. Figure 2 below illustrates the use-case where cropped picture from the source picture is used in composite picture. Furthermore, a picture from a particular layer may be the source of multiple area (i.e., patch) in a composite picture as shown in Figure 2 where the whole picture of layer 0 is used for general view and cropped area (i.e., the part the shows the  hitter) is used for hitter view.

Figure 2 – Use of composite picture in broadcasting / streaming live baseball game

# Proposed design principles

For supporting composite picture based for multilayer bitstream, we propose the following SEI message design:

- Composite picture information (CPI) SEI specifies information to create / generate a composite picture from one or more pictures in an access unit.

- A composite picture contains one or more rectangular shaped patches.

- Patches of a composite picture belongs to rectangular areas in pictures of the same access unit.

- For each patch in a composite picture, the following information is signalled:

  - The location of the patch in the composite picture. This information can be signalled as the position of the top-left corner of the patch in the composite picture.

  - The size (i.e., width and height) of the patch in the composite picture.

  - The source of the pels of the patch. This includes the following information:

    - The layer id of the picture in the associated access unit.

    - The location of the area in the picture that corresponds to the patch (i.e., top-left corner)

    - The size of the area in the picture that corresponds to the patch.

- The size of a patch in a composite picture shall be greater than 0 and the associated area (i.e., the source of the pels of the patch) in the associated picture shall be greater than 0.

- There can be gap between two neighboring patches in a composite picture. In other words, there may be area in the composite picture that is not covered by any patch.

- There can be overlap of patches in a composite picture. In other words, there may be area in the composite picture that is covered by two or more patches.

- When there is overlap between two patches, the patch with higher index is rendered after the patch with lower index such that in the overlapped area between the two patches, the pels come from the patch with higher index.

- There shall be no two patches with the same size that covered the same area in a composite picture. This is intended to avoid signalling redundant patch(es) as there is only one will be displayed / visible.

- Two patches in a composite picture may have the same corresponding area in the same picture.

- A patch in a composite picture shall correspond an area in a picture from an output layer only.

- It is possible that a patch in a composite picture corresponds to an area in a picture from an output layer but the picture is not outputted by decoder (e.g., the PictureOutputFlag of the picture is derived to be equal to 0 due to certain condition). In such situation, implementer of the SEI message may skip copying the pels from the source area to the patch in the composite picture (e.g., a gray patch is shown in the composite picture).

- A CPI SEI message describes composition for its associated OLS. The association is established by allowing CPI SEI message to be carried in a scalable nesting SEI message.

  - When a scalable nesting SEI message contains CPI SEI message(s), the value of the value of sn_ols_flag shall be equal to 1.

  - When a CPI SEI message is not present in a scalable nesting SEI message, it applies to the targetOLS (i.e., to the whole bitstream being decoded).

- The CPI SEI message applies to AU A that contains the SEI and all AU that follow AU A in output order until one of the following is true:

  - The end of the bitstream.

  - The next AU contains a new CPI SEI message that applies to the same OLS.

- There shall not be two CPI SEI messages associated with the same OLS but contain different content in an AU. This would create problem for implementer to choose which one is the correct one. Thus, it is constrained that when two CPI SEI message are present in an AU and they are associated with the same OLS, they shall have the same content.

- When present, CPI SEI message is present in AU with TemporalId equal to 0 to avoid the SEI being removed during sublayer extraction.

- When present in a CVS, the first of the CPI SEI message to be present in the CVSS AU.

- Possible bit-saving for signalling:

  - For signalling the position and size of the patch and size of the composite picture, instead of using ue(v) coding, signal the bit length for the coding and code those syntax elements using u(v).

  - Signal a flag to specify whether scaling is allowed. When scaling is not allowed, the size of patch in the composite picture does not need to be signalled as it can be inferred to be equal to the size of the associated area in the source picture.

  - Signal a flag to specify whether cropped picture is used for the composite picture. When the flag is equal to 0, the location of the associated area in the source picture does not need to be signalled as it can be inferred to be equal to the top-left corner of the picture.

  - The size of the composite picture can be not signalled. This can be inferred from the position and size of patches that make up the composite picture.

# Text for composite picture information SEI message

Syntax and Semantics of CPI SEI message

| composite_picture_info( ) { | Descriptor |
| --- | --- |
| cpi_position_and_size_bit_len | ue(v) |
| cpi_num_patches_minus1 | ue(v) |
| cpi_cropped_source_area_allowed_flag | u(1) |
| cpi_scaling_allowed_flag | u(1) |
| cpi_gap_allowed_flag | u(1) |
| for ( i = 0; i <= cpi_num_patches_minus1; i++ ) { |  |
| cpi_patch_layer_id[ i ] | u(6) |
| if( cpi_cropped_source_area_allowed_flag ) { |  |
| cpi_patch_source_top_left_x[ i ] | u(v) |
| cpi_patch_source_top_left_y[ i ] | u(v) |
| } |  |
| cpi_patch_source_luma_width_minus1[ i ] | u(v) |
| cpi_patch_source_luma_height_minus1[ i ] | u(v) |
| cpi_patch_dest_top_left_x[ i ] | u(v) |
| cpi_patch_dest_top_left_y[ i ] | u(v) |
| if( cpi_scaling_flag) { |  |
| cpi_patch_dest_luma_width_minus1[ i ] | u(v) |
| cpi_patch_dest_luma_height_minus1[ i ] | u(v) |
| } |  |
| } |  |
| } |  |

A composite picture info (CPI) SEI message provides information for constructing composite picture from one or more patches originated from pictures of an access unit.

When one or more CPI SEI message is present in a bitstream, the first CPI SEI message shall be present in a CVSS AU. When two or more CPI SEI message are present in an AU, the content of the CPI SEI messages shall be the same.

When a scalable nesting SEI message contains a CPI SEI message, the value of sn_ols_flag shall be equal to 1.

A CPI SEI message applies to the AU A that contains the SEI message and all AUs that follow the AU A in output order until one of the following applies:

- The end of the bitstream.

- The next AU contains a new CPI SEI message that applies to the target OLS.

When present, CPI SEI message or scalable nesting SEI message that contains CPI SEI message, shall be contained in an access unit with temporal Id equal to 0. The temporal Id of the CPI SEI message or the scalable nesting SEI message that contains the CPI SEI message shall be equal to 0.

cpi_position_and_size_bit_len_minus1 plus 1 specifies the number of bits for signalling of syntax elements cpi_width_in_luma_samples, cpi_height_in_luma_samples, cpi_patch_source_top_left_x[ i ], cpi_patch_source_top_left_y[ i ], cpi_patch_source_width_minus1[ i ] , cpi_patch_source_height_minus1[ i ], cpi_patch_dest_top_left_x[ i ], cpi_patch_dest_top_left_y[ i ], cpi_patch_source_dest_minus1[ i ], and cpi_patch_source_dest_minus1[ i ].

cpi_num_patches_minus1 specifies the number of patches of each composite picture created from AUs in which the SEI applies.

cpi_cropped_source_area_allowed_flag equal to 1 specifies that cpi_patch_source_top_left_x[ i ] and cpi_patch_source_top_left_y[ i ] are present. cpi_cropped_source_area_allowed_flag equal to 0 specifies that cpi_patch_source_top_left_x[ i ] and cpi_patch_source_top_left_y[ i ] are present are not present.

cpi_scaling_allowed_flag equal to 1 specifies that cpi_patch_dest_luma_width[ i ] and cpi_patch_dest_luma_width[ i ] are present. cpi_scaling_allowed_flag equal to 0 specifies that cpi_patch_dest_luma_width[ i ] and cpi_patch_dest_luma_width[ i ] are not present.

cpi_gap_allowed_flag equal to 1 specifies that the union of all patches does not cover all area in the composite picture. cpi_gap_allowed_flag equal to 0 specifies that the union of all patches covers all area in the composite picture.

NOTE: When cpi_gap_allowed_flag is equal to 1, pels in the area that is not covered by any patch need to be initialized with a valid pel value.

cpi_patch_layer_id[ i ] specifies the layer Id of the source picture for the i-th patch.

It is constrained that when CPI SEI message is contained in a scalable nesting SEI message, the value of  cpi_patch_layer_id[ i ] for i in the ranges from 0 to cpi_num_patches_minus1, inclusive, shall be equal to one of layer included in the OLS associated with the SEI message.

It is constrained that when CPI SEI message is not contained in a scalable nesting SEI message, the value of  cpi_patch_layer_id[ i ] for i in the ranges from 0 to cpi_num_patches_minus1, inclusive, shall be equal to one of layer that is present in the bitstream.

It is constrained that the layer with layer Id equal to cpi_patch_layer_id[ i ] for i in the ranges from 0 to cpi_num_patches_minus1, inclusive, shall be an output layer in the output layer set associated with the SEI message.

Picture with layer Id equal to cpi_patch_layer_id[ i ] for i in the ranges from 0 to cpi_num_patches_minus1, inclusive, may not be present in the AU in which the SEI applies. When The AU has no picture in cpi_patch_layer_id[ i ], application that generate the composite picture based on the SEI may assign any valid pel value for the i-th patch in the composite picture. Furthermore, in such situation, application may further assign the same pel values for the entire area covered the i-th patch.

cpi_patch_source_top_left_x[ i ] specifies horizontal position of top left pel in the source picture for the i-th patch. The value of cpi_patch_source_top_left_x[ i ] shall be less than the width of the source picture minus 1 (or alternatively, minus 2). The length of the cpi_patch_source_top_left_x[ i ] syntax element is cpi_position_and_size_bit_len_minus1 + 1 bits. When not present, the value of cpi_patch_source_top_left_x[ i ] is inferred to be equal to 0.

cpi_patch_source_top_left_y[ i ] specifies vertical position of top left pel in the source picture for the i-th patch. The value of cpi_patch_source_top_left_y[ i ] shall be less than the height of the source picture minus 1 (or alternatively, minus 2). The length of the cpi_patch_source_top_left_y[ i ] syntax element is cpi_position_and_size_bit_len_minus1 + 1 bits. When not present, the value of cpi_patch_source_top_left_y[ i ] is inferred to be equal to 0.

cpi_patch_source_width_minus1[ i ] plus 1 specifies width of the area in the source picture for the i-th patch. The sum of cpi_patch_source_top_left_x[ i ] and cpi_patch_source_width_minus1[ i ] shall be less than the width, in the unit of luma sample, of the source picture. The length of the cpi_patch_source_width_minus1[ i ] syntax element is cpi_position_and_size_bit_len_minus1 + 1 bits.

When the value of cpi_cropped_source_area_allowed_flag equal to 0, it is constrained that the value of cpi_patch_source_width_minus1[ i ] shall be equal to the pps_pic_width_in_luma_samples − 1 of the associated picture.

cpi_patch_source_height_minus1[ i ] plus 1 specifies height of the area in the source picture for the i-th patch. The sum of cpi_patch_source_top_left_y[ i ] and cpi_patch_source_height_minus1[ i ] shall be less than the height, in the unit of luma sample, of the source picture. The length of the cpi_patch_source_height_minus1[ i ] syntax element is cpi_position_and_size_bit_len_minus1 + 1 bits.

When the value of cpi_cropped_source_area_allowed_flag equal to 0, it is constrained that the value of cpi_patch_source_height_minus1[ i ] shall be equal to the pps_pic_height_in_luma_samples − 1 of the associated picture.

The value of cpi_patch_source_top_left_x[ i ] + cpi_patch_source_width_minus1[ i ] shall be less than the picture width in luma samples minus 1 of any picture in the layer with layer id equal to cpi_patch_layer_id[ i ] which the SEI message applies.

The value of cpi_patch_source_top_left_y[ i ] + cpi_patch_source_height_minus1[ i ] shall be less than the picture height in luma samples minus 1 of any picture in the layer with layer id equal to cpi_patch_layer_id[ i ] which the SEI message applies.

cpi_patch_dest_top_left_x[ i ] specifies horizontal position of top left pel in the composite picture for the i-th patch. The value of cpi_patch_dest_top_left_x[ i ] shall be less than the value of cpi_width_in_luma_samples – 1 (or alternatively, minus 2). The length of the cpi_patch_dest_top_left_x[ i ] syntax element is cpi_position_and_size_bit_len_minus1 + 1 bits.

cpi_patch_dest_top_left_y[ i ] specifies vertical position of top left pel in the composite picture for the i-th patch. The value of cpi_patch_dest_top_left_y[ i ] shall be less than the value of cpi_height_in_luma_samples – 1 (or alternatively, minus 2). The length of the cpi_patch_dest_top_left_y[ i ] syntax element is cpi_position_and_size_bit_len_minus1 + 1 bits.

cpi_patch_dest_width_minus1[ i ] plus 1 specifies width of the area in the composite picture for the i-th patch. The sum of cpi_patch_dest_top_left_x[ i ] and cpi_patch_dest_width_minus1[ i ] shall be less than the value of cpi_width_in_luma_samples – 1. When not present, the value of cpi_patch_dest_width_minus1[ i ] is inferred to be equal to cpi_source_width_minus1[ i ]. The length of the cpi_patch_dest_width_minus1[ i ] syntax element is cpi_position_and_size_bit_len_minus1 + 1 bits.

cpi_patch_dest_height_minus1[ i ] plus 1 specifies height of the area in the composite picture for the i-th patch. The sum of cpi_patch_dest_top_left_y[ i ] and cpi_patch_dest_height_minus1[ i ] shall be less than the value of cpi_height_in_luma_samples – 1. When not present, the value of cpi_patch_dest_height_minus1[ i ] is inferred to be equal to cpi_source_height_minus1[ i ]. The length of the cpi_patch_dest_height_minus1[ i ] syntax element is cpi_position_and_size_bit_len_minus1 + 1 bits.

The variables CompositePictureWidth and CompositePictureHeight, specifying the width and height, respectively, of the composite picture are derived as follows:

CompositePictureWidth = cpi_patch_dest_top_left_x[ 0 ] + cpi_patch_dest_width_minus1[ 0 ] + 1
CompositePictureHeight = cpi_patch_dest_top_left_y[ 0 ] + cpi_patch_dest_height_minus1[ 0 ] + 1
for( i = 0; i  <=  cpi_num_patches_minus1; i++ ) {
	if( (cpi_patch_dest_top_left_x[ i ] + cpi_patch_dest_width_minus1[ i ] + 1 ) > CompositePictureWidth )
		CompositePictureWidth = cpi_patch_dest_top_left_x[ i ] + cpi_patch_dest_width_minus1[ i ] + 1
	if( (cpi_patch_dest_top_left_y[ i ] + cpi_patch_dest_height_minus1[ i ] + 1 ) > CompositePictureHeight )
		CompositePictureHeight = cpi_patch_dest_top_left_y[ i ] + cpi_patch_dest_height_minus1[ i ] + 1
}

For area that is covered by two or more patches, the value of each pel in such area shall be from the patch with the highest i-th index among those patches.

It is constrained that there shall be no two patches in the composite picture, i-th patch and j-th patch, such that the values of cpi_patch_dest_top_left_x[ i ], cpi_patch_dest_top_left_y[ i ], cpi_patch_dest_width_minus1[ i ], and cpi_patch_dest_height_minus1[ i ] are equal to cpi_patch_dest_top_left_x[ j ], cpi_patch_dest_top_left_y[ j ], cpi_patch_dest_width_minus1[ j ], and cpi_patch_dest_height_minus1[ j ], respectively.

It is constrained that when two patches in the composite picture, i-th patch and the j-th patch, have the values of cpi_patch_dest_top_left_x[ i ] and cpi_patch_dest_top_left_y[ i ] equal to cpi_patch_dest_top_left_x[ j ] and cpi_patch_dest_top_left_y[ j ], respectively, and the values of cpi_patch_dest_width[ i ] and cpi_patch_dest_height[ i ] are greater than cpi_patch_dest_width[ j ] and cpi_patch_dest_height[ j ], respectively, then j shall be greater than i.

# References

- J. Boyce, “AHG8/AHG12: Recommended multi-layer composite picture SEI messages”, JVET-S0107, the 19th JVET meeting, by teleconference, 22 June – 1 July 2020.

- E. Thomas, A. van. Buerenplein, “AHG8/AHG9: Refinement of proposed positioning information SEI message of output independent layers with example bitstreams”, JVET-S0123, the 19th JVET meeting, by teleconference, 22 June – 1 July 2020.

- E. Thomas, A. van. Buerenplein, “AHG8/AHG9: Positioning information SEI message of output independent layers”, JVET-R0307, the 18th JVET meeting, by teleconference, 15–24 April 2020.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).