| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0050-v2 |
| --- | --- |

| Title: | AHG9: HEVC Annotated Regions SEI message software update for HM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Palanivel Guruva reddiar Jill Boyce | Email: | palanivel.guruvareddiar@intel.com jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

An update is provided for the HM software encoder implementation of the Annotated Regions SEI message. This version of the software detects static regions in the input configuration file and avoids updating the per picture region parameters (position and size) signalled in the SEI message for static regions.

# Contribution

This contribution provides updated software for supporting the Annotated Regions SEI message in the HM software.

The configuration format used for the SEI message in the HM encoder is not changed. The config file contains the position and size for the regions intended for signaling in the SEI message. The change is that the HM encoder can detect static regions with the same size and position relative to the previously coded picture and avoid signalling updated size and position for the static regions. This simplifies usage, as it moves the burden of identifying static objects to the HM encoder and not to the input, so that it can be used with inputs which contain the current picture size and position, without any understanding of changes relative to previously coded pictures.

It is proposed to update the HM software to includes this update. It was provided to the software coordinators to review on 27 August ‘20.

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).