| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0051 |
| --- | --- |

| Title: | Object tracking information file format for use with Annotated Regions SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Palanivel Guruva reddiar Jill Boyce | Email: | palanivel.guruvareddiar@intel.com jill.boyce@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

This contribution describes an uncompressed file format defined for describing per picture object tracking information. Example files for the JVET Class E test content is provided, based on annotations provided by Simon Frasier University.

Python scripts are provided to create HM or VTM Annotated Regions SEI config files from the object tracking information text files, and to generate the object tracking information files from the decoder’s output SEI message logs.

# File Format

This contribution provides example object tracking information files for the JVET Class E test content, for rectangular regions updated per picture.

The object tracking information file format is defined as follows.

- A separate file for each picture, using a 3-digit picture number in the file name, with extension *.txt (for example FourPeople_1280x720_60_seq_000.txt).  In each per picture file, one line is present for each object, which contains the following (space separated)

  - Object description: string

  - Object ID: uint

  - Object top left luma sample x-position: uint

  - Object top left luma sample y-position: uint

  - Object width in luma samples: uint

  - Object height in luma samples: uint

- A list file containing a summary of annotation labels for each picture.

# Conversion scripts

Python scripts (using Python3) are provided to create an HM or VTM Annotated Regions SEI config files from the object tracking information files, and to generate the object tracking information files from the decoder’s output SEI message logs.

To create the Annotated Regions SEI message config files, the usage is as follows:

Python3 anno_label_to_ar_sei_cfg.py input_folder input_token output_folder output_token label_list  num_frames

For example,

Python3 anno_label_to_ar_sei_cfg.py --input_folder anno_labels/ --input_token FourPeople_1280x720_60_seq --output_folder ar_sei_cfg_files/ --output_token FourPeople_1280x720_60_seq –label_list FourPeople_1280x720_object.list  --num_frames 600

will use 600 object tracking information files to create a HM config files named FourPeople_1280x720_60_seq_<POC>.txt in the folder named ar_sei_cfg_files/

To run the HM/VTM encoder using that config file, do the following:

./TAppEncoderStatic -c ../cfg/encoder_lowdelay_P_main.cfg --SEIAnnotatedRegionsFileRoot=ar_sei_cfg_files/ FourPeople_1280x720_60_seq

To run the HM/VTM decoder using the HEVC bitstream containing the annotated regions SEI message, do the following to create the output SEI message log.

./TAppDecoderStatic --BitstreamFile=../bitstreams/Fourpeople_with_sei.bin --ReconFile=../decoded/ Fourpeople_with_sei.yuv --SEIAnnotatedRegionsInfoFilename=ar_sei_dec_log /Fourpeople_dec_log.txt

To create the object tracking information files, the usage is as follows:

Python3 dec_log_to_anno_label.py decoder_log output_folder output_token

For example,

Python3 dec_log_to_anno_label.py --decoder_log ar_sei_dec_log/Fourpeople_dec_log.txt --output_folder annotation_labels_dec/FourPeople/ --output_token FourPeople_1280x720_60_seq

will create 600 object tracking information files in the folder named FourPeople/

# Example Files

Object tracking information files are attached for the Class E sequences. Annotated data files were provided privately by Simon Fraser University (SFU), related to contribution m54737. Those files provided by SFU were in a different format and were converted to the format described in this document.

The tables below list the average number of objects identified in each of the provided example files, for the JVET Class E sequences.

| Sequence | # of objects |
| --- | --- |
| FourPeople | 29 |
| Johnny | 11 |
| KristenAndSara | 15 |

Figure 1-3 below shows an example frames from the Class E sequences, where the bounding boxes have been overlaid on the video and the object description indicated. The script based on OpenCV and FFMPEG used to generate this video from the object overlay information file is also provided. The usage of this script is as follows,

Python3 ar_sei_display.py video_file input_folder input_token output_folder output_token num_frames

As an example,

Python3 ar_sei_display.py --video_file FourPeople_1280x720_60p_i420_8bit.yuv --input_folder annotation_labels_dec/FourPeople/ --input_token FourPeople_1280x720_60_seq --output_folder frames_with_annotations/FourPeople/ --output_token FourPeople_1280x720_60_seq --num_frames 600

Figure.1: Frame #95 of Four People sequence with bounding boxes overlaid

Figure.2: Frame #372 of KristenAndSara sequence with bounding boxes overlaid

# Figure.3: Frame #372 of Johnny sequence with bounding boxes overlaid

# References

[1] H. Choi, E. Hosseini, S.R. Alvar, R.A. Cohen, I.V. Bajić (SFU), A. Karabutov, Yin Zhao, E. Alshina (Huawei), m54737  [VCM] Object labelled dataset on raw video sequences

[2] OpenCV, https://opencv.org/

[3] FFMPEG, https://www.ffmpeg.org/

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).