| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0053-v2 |
| --- | --- |

| Title: | AHG9: VVC and VSEI Annotated Regions SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce Palanivel Guruva reddiar | Email: | jill.boyce@intel.com palanivel.guruvareddiar@intel.com |
| Source: | Intel |  |  |

_____________________________

# Abstract

It is proposed to add an Annotated Regions SEI message to VVC and VSEI, with identical SEI message payload syntax to that used in HEVC. Draft text and VTM software are provided.

# Introduction

The proposed specification text is in Section 3. The SEI message payload syntax is identical to the HEVC Annotated Regions SEI message. Updated semantics have been provided to accommodate the separation of SEI message support in the VSEI and the VVC specifications.

# Software

The attached software patch for the VTM adds support for the SEI message. The usage and configuration file input format are identical to that in the HM, including the ability to detect static regions provided in JVET-T0050.

# Specification text

## VVC

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| else if( payloadType = = 4 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 19 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 45 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 129 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| parameter_sets_inclusion_indication( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 137 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| mastering_display_colour_volume( payloadSize ) |  |
| else if( payloadType = = 144 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| content_light_level_info( payloadSize ) |  |
| else if( payloadType = = 145 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| dependent_rap_indication( payloadSize ) |  |
| else if( payloadType = = 147 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| alternative_transfer_characteristics( payloadSize ) |  |
| else if( payloadType = = 148 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| ambient_viewing_environment( payloadSize ) |  |
| else if( payloadType = = 149 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| content_colour_volume( payloadSize ) |  |
| else if( payloadType = = 150 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| equirectangular_projection( payloadSize ) |  |
| else if( payloadType = = 153 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| generalized_cubemap_projection( payloadSize ) |  |
| else if( payloadType = = 154 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sphere_rotation( payloadSize ) |  |
| else if( payloadType = = 155 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| regionwise_packing( payloadSize ) |  |
| else if( payloadType = = 156 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| omni_viewport( payloadSize ) |  |
| else if( payloadType = = 168 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_field_info( payloadSize ) |  |
| else if( payloadType = = 202) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| annotated_regions( payloadSize ) |  |
| else if( payloadType = = 203 ) |  |
| subpic_level_info( payloadSize ) |  |
| else if( payloadType = = 204 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sample_aspect_ratio_info( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| filler_payload( payloadSize ) |  |
| if( payloadType = = 132 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| sei_reserved_payload_extension_data | u(v) |
| sei_payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| sei_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

D.9.7. Use of the annotated regions SEI messages

For purposes of interpretation of the annotated regions SEI message, the following variables are specified:

- CroppedWidth is set equal to  pic_width_in_luma_samples −
		SubWidthC  * ( pps_conf_win_right_offset + pps_conf_win_left_offset )

- CroppedHeight is set equal to pic_height_in_luma_samples −
		SubHeightC * ( pps_conf_win_bottom_offset + pps_conf_win_top_offset )

- ConfWinLeftOffset is set equal to pps_conf_win_left_offset

- ConfWinTopOffset is set equal to pps_conf_win_top_offset

## VSEI

2.3 Additional references

–	ISO 11664-1 (in force), Colorimetry – Part 1: CIE standard colorimetric observers.

–	ISO/IEC 11578:1996, Information technology – Open Systems Interconnection – Remote Procedure Call (RPC).

–	Recommendation ITU-T T.35:2000, Procedure for the allocation of ITU-T defined codes for non standard facilities.

–	IETF RFC 1321 (in force), The MD5 Message-Digest Algorithm.

–	IETF RFC 5646 (in force), Tags for Identifying Languages.

8.18 Annotated regions SEI message

8.18.1 Annotated regions SEI message syntax

| annotated_regions( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| ar_cancel_flag | 5 | u(1) |
| if(!ar_cancel_flag) { |  |  |
| ar_not_optimized_for_viewing_flag | 5 | u(1) |
| ar_true_motion_flag | 5 | u(1) |
| ar_occluded_object_flag | 5 | u(1) |
| ar_partial_object_flag_present_flag | 5 | u(1) |
| ar_object_label_present_flag | 5 | u(1) |
| ar_object_confidence_info_present_flag | 5 | u(1) |
| if( ar_object_confidence_info_present_flag ) |  |  |
| ar_object_confidence_length_minus1 | 5 | u(4) |
| if( ar_object_label_present_flag ) { |  |  |
| ar_object_label_language_present_flag | 5 | u(1) |
| if( ar_object_label_language_present_flag ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| ar_bit_equal_to_zero /* equal to 0 */ | 5 | f(1) |
| ar_object_label_language | 5 | st(v) |
| } |  |  |
| ar_num_label_updates | 5 | ue(v) |
| for( i = 0; i < ar_num_ label_updates; i++ ) { |  |  |
| ar_label_idx[ i ] | 5 | ue(v) |
| ar_label_cancel_flag | 5 | u(1) |
| LabelAssigned[ ar_label_idx[ i ] ] = !ar_label_cancel_flag |  |  |
| if( !ar_label_cancel_flag ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| ar_bit_equal_to_zero /* equal to 0 */ | 5 | f(1) |
| ar_label[ ar_label_idx[ i ] ] | 5 | st(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| ar_num_object_updates | 5 | ue(v) |
| for( i = 0; i <= ar_num_object_updates; i++ ) { |  |  |
| ar_object_idx[ i ] | 5 | ue(v) |
| ar_object_cancel_flag | 5 | u(1) |
| ObjectTracked[ ar_object_idx[ i ] ] = !ar_object_cancel_flag |  |  |
| if( !ar_object_cancel_flag ) { |  |  |
| if( ar_object_label_present_flag ) { |  |  |
| ar_object_label_update_flag | 5 | u(1) |
| if( ar_object_label_update_flag ) |  |  |
| ar_object_label_idx[ ar_object_idx[ i ] ] | 5 | ue(v) |
| } |  |  |
| ar_bounding_box_update_flag | 5 | u(1) |
| if( ar_bounding_box_update_flag ) { |  |  |
| ar_bounding_box_cancel_flag | 5 | u(1) |
| ObjectBoundingBoxAvail[ ar_object_idx[ i ] ] = !ar_bounding_box_cancel_flag |  |  |
| if( !ar_bounding_box_cancel_flag ) { |  |  |
| ar_bounding_box_top[ ar_object_idx[ i ] ] | 5 | u(16) |
| ar_bounding_box_left[ ar_object_idx[ i ] ] | 5 | u(16) |
| ar_bounding_box_width[ ar_object_idx[ i ] ] | 5 | u(16) |
| ar_bounding_box_height[ ar_object_idx[ i ] ] | 5 | u(16) |
| if( ar_partial_object_flag_present_flag ) |  |  |
| ar_partial_object_flag[ ar_object_idx[ i ] ] | 5 | u(1) |
| if( ar_object_confidence_info_present_flag ) |  |  |
| ar_object_confidence[ ar_object_idx[ i ] ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

8.18.2 Annotated regions SEI message semantics

The annotated regions SEI message carries parameters that identify annotated regions using bounding boxes representing the size and location of identified objects.

Use of this SEI message requires the definition of the following variables:

- A cropped picture width and picture height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

- A chroma sub-sampling width and height, SubWidthC and SubHeightC, respectively.

- A conformance cropping window left offset, ConfWinLeftOffset

- A conformance cropping window top offset, ConfWinTopOffset

ar_cancel_flag equal to 1 indicates that the annotated regions SEI message cancels the persistence of any previous annotated regions SEI message that is associated with one or more layers to which the annotated regions SEI message applies. ar_cancel_flag equal to 0 indicates that annotated regions information follows.

When ar_cancel_flag equal to 1 or a new CVS of the current layer begins, the variables LabelAssigned[ i ], ObjectTracked[ i ], and ObjectBoundingBoxAvail are set equal to 0 for i in the range of 0 to 255, inclusive.

ar_not_optimized_for_viewing_flag equal to 1 indicates that the decoded pictures that the annotated regions SEI message applies to are not optimized for user viewing, but rather are optimized for some other purpose such as algorithmic object classification performance. ar_not_optimized_for_viewing_flag equal to 0 indicates that the decoded pictures that the annotated regions SEI message applies to may or may not be optimized for user viewing.

ar_true_motion_flag equal to 1 indicates that the motion information in the coded pictures that the annotated regions SEI message applies to was selected with a goal of accurately representing object motion for objects in the annotated regions. ar_true_motion_flag equal to 0 indicates that the motion information in the coded pictures that the annotated regions SEI message applies to may or may not be selected with a goal of accurately representing object motion for objects in the annotated regions.

ar_occluded_object_flag equal to 1 indicates that the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ], and ar_bounding_box_height[ ar_object_idx[ i ] ] syntax elements each represent the size and location of an object or a portion of an object that may not be visible or may be only partially visible within the cropped decoded picture. ar_occluded_object_flag equal to 0 indicates that the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ], and ar_bounding_box_height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that is entirely visible within the cropped decoded picture. It is a requirement of bitstream conformance that the value of ar_occluded_object_flag shall be the same for all annotated_regions( ) syntax structures within a CVS.

ar_partial_object_flag_present_flag equal to 1 indicates that ar_partial_object_flag[ ar_object_idx[ i ] ] syntax elements are present. ar_partial_object_flag_present_flag equal to 0 indicates that ar_partial_object_flag[ ar_object_idx[ i ] ] syntax elements are not present. It is a requirement of bitstream conformance that the value of ar_partial_object_flag_present_flag shall be the same for all annotated_regions( ) syntax structures within a CVS.

ar_object_label_present_flag equal to 1 indicates that label information corresponding to objects in the annotated regions is present. ar_object_label_present_flag equal to 0 indicates that label information corresponding to the objects in the annotated regions is not present.

ar_object_confidence_info_present_flag equal to 1 indicates that ar_object_confidence[ ar_object_idx[ i ] ] syntax elements are present. ar_object_confidence_info_present_flag equal to 0 indicates that ar_object_confidence[ ar_object_idx[ i ] ] syntax elements are not present. It is a requirement of bitstream conformance that the value of ar_object_confidence_present_flag shall be the same for all annotated_regions( ) syntax structures within a CVS.

ar_object_confidence_length_minus1 + 1 specifies the length, in bits, of the ar_object_confidence[ ar_object_idx[ i ] ] syntax elements. It is a requirement of bitstream conformance that the value of ar_object_confidence_length_minus1 shall be the same for all annotated_regions( ) syntax structures within a CVS.

ar_object_label_language_present_flag equal to 1 indicates that the ar_object_label_language syntax element is present. ar_object_label_language_present_flag equal to 0 indicates that the ar_object_label_language syntax element is not present.

ar_bit_equal_to_zero shall be equal to zero.

ar_object_label_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the ar_object_label_language syntax element shall be less than or equal to 255 bytes, not including the null termination byte. When not present, the language of the label is unspecified.

ar_num_label_updates indicates the total number of labels associated with the annotated regions that will be signalled. The value of ar_num_label_updates shall be in the range of 0 to 255, inclusive.

ar_label_idx[ i ] indicates the index of the signalled label . The value of ar_label_idx[ i ] shall be in the range of 0 to 255, inclusive.

ar_label_cancel_flag equal to 1 cancels the persistence scope of the ar_label_idx[ i ]-th label. ar_label_cancel_flag equal to 0 indicates that the ar_label_idx[ i ]-th label will be assigned a signalled value.

ar_label[ ar_label_idx[ i ] ] specifies the contents of the ar_label_idx[ i ] –th label. The length of the ar_label[ ar_label_idx[ i ] ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

ar_num_object_updates indicates the number of object updates to be signalled. ar_num_object_updates shall be in the range of 0 to 255, inclusive.

ar_object_idx[ i ] is the index of the object parameters to be signalled. ar_object_idx[ i ] shall be in the range of 0 to 255, inclusive.

ar_object_cancel_flag equal to 1 cancels the persistence scope of the ar_object_idx[ i ]-th object. ar_object_cancel_flag equal to 0 indicates that parameters associated with the ar_object_idx[ i ]-th object tracked object will be signalled.

ar_object_label_update_flag equal to 1 indicates that an object label will be signalled. ar_object_label_update_flag equal to 0 indicates that an object label will not be signalled.

ar_object_label_idx[ ar_object_idx[ i ] ] indicates the index of the label corresponding to the ar_object_idx[ i ]-th object. When ar_object_label_idx[ ar_object_idx[ i ] ] is not present, its value is inferred from a previous annotated regions SEI messages in output order in the same CVS, if any.

ar_bounding_box_update_flag equal to 1 indicates that object bounding box parameters will be signalled.  ar_bounding_box_update_flag equal to 0 indicates that object bounding box parameters will not be signalled.

ar_bounding_box_cancel_flag equal to 1 cancels the persistence scope of the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ], ar_bounding_box_height[ ar_object_idx[ i ] ]. ar_partial_object_flag[ ar_object_idx[ i ] ], and ar_object_confidence[ ar_object_idx[ i ] ]. ar_bounding_box_cancel_flag equal to 0 indicates that ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] ar_bounding_box_height[ ar_object_idx[ i ] ] ar_partial_object_flag[ ar_object_idx[ i ] ], and ar_object_confidence[ ar_object_idx[ i ] ] syntax elements will be signalled.

ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ], and ar_bounding_box_height[ ar_object_idx[ i ] ] specify the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the ar_object_idx[ i ]-th object in the cropped decoded picture, relative to the conformance cropping window specified by the active SPS.

The value of ar_bounding_box_left[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedWidth / SubWidthC − 1, inclusive.

The value of ar_bounding_box_top[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedHeight / SubHeightC − 1, inclusive.

The value of ar_bounding_box_width[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedWidth / SubWidthC − ar_bounding_box_left[ ar_object_idx[ i ] ], inclusive.

The value of ar_bounding_box_height[ ar_object_idx[ i ] ] shall be in the range of 0 to CroppedHeight / SubHeightC − ar_bounding_box_top[ ar_object_idx[ i ] ], inclusive.

The identified object rectangle contains the luma samples with horizontal picture coordinates from SubWidthC * ( ConfWinLeftOffset  + ar_bounding_box_left[ ar_object_idx[ i ] ] ) to SubWidthC * ( ConfWinLeftOffset  + ar_bounding_box_left[ ar_object_idx[ i ] ] + ar_bounding_box_width[ ar_object_idx[ i ] ] ) − 1, inclusive, and vertical picture coordinates from SubHeightC * ( ConfWinTopOffset + ar_bounding_box_top[ ar_object_idx[ i ] ] ) to SubHeightC * ( ConfWinTopOffset + ar_bounding_box_top[ ar_object_idx[ i ] ] + ar_bounding_box_height[ ar_object_idx[ i ] ] ) − 1, inclusive.

The values of ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] and ar_bounding_box_height[ ar_object_idx[ i ] ] persist in output order within the CVS for each value of ar_object_idx[ i ]. When not present, the values of  ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] or ar_bounding_box_height[ ar_object_idx[ i ] ] are inferred from a previous annotated regions SEI message in output order in the CVS, if any.

ar_partial_object_flag[ ar_object_idx[ i ] ] equal to 1 indicates that the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] and ar_bounding_box_height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that is only partially visible within the cropped decoded picture. ar_partial_object_flag[ ar_object_idx[ i ] ] equal to 0 indicates that the ar_bounding_box_top[ ar_object_idx[ i ] ], ar_bounding_box_left[ ar_object_idx[ i ] ], ar_bounding_box_width[ ar_object_idx[ i ] ] and ar_bounding_box_height[ ar_object_idx[ i ] ] syntax elements represent the size and location of an object that may or may not be only partially visible within the cropped decoded picture. When not present, the value of ar_partial_object_flag[ ar_object_idx[ i ] ] is inferred from a previous annotated regions SEI message in output order in the CVS, if any.

ar_object_confidence[ ar_object_idx[ i ] ] indicates the degree of confidence associated with the ar_object_idx[ i ]-th object, in units of 2−( ar_object_confidence_length_minus1 + 1 ), such that a higher value of ar_object_confidence[ ar_object_idx[ i ] ] indicates a higher degree of confidence. The length of the ar_object_confidence[ ar_object_idx[ i ] ] syntax element is ar_object_confidence_length_minus1 + 1 bits. When not present, the value of_object_confidence[ ar_object_idx[ i ] ] is inferred from a previous annotated regions SEI message in output order in the CVS, if any.

# Patent rights declaration(s)

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).