| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0054-v1 |
| --- | --- |

| Title: | Some errata items for HEVC and AVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang, Li Zhang Bytedance Inc. 8910 University Center Lane San Diego, CA 92122, USA Alexis M. Tourapis Apple 1 Apple Park Way Cupertino, CA 95014 USA | Tel: Email: | yekui.wang@bytedance.com lizhang.idm@bytedance.com atourapis@apple.com |
| Source: | Bytedance Inc., Apple |  |  |

_____________________________

# Abstract

This contribution reports some asserted bugs in HEVC and AVC and proposes fixes for the asserted bugs.

# Errata items for both HEVC and AVC

- Change the range notation from the following:

x = y..z	x takes on integer values starting from y to z, inclusive, with x, y, and z being integer numbers and z being greater than y.

to be as follows (same as in VVCv1):

x = y..z	x takes on integer values starting from y to z, inclusive, with x, y, and z being integer numbers and z being greater than or equal to y.

# Errata items for HEVC only

- Search and replace "satisify" with "satisfy".

- In clause 9.3.3.6, fix the wrong indention for the paragraph starting with "Otherwise".

- Search and check "CVS", particularly in semantics, and fix to be CLVS as needed.

- Search and replace instances of "clasue" with "clause".

- The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are initially derived in clause C.2.1 and updated in clause C.2.3, and the second variable is not even used in clause C.2.3. However, currently, the wording in clause C.2.3 is that these variables are derived, which can confuse readers as to why the second variable is derived but not used in clause C.2.3 thus why not just remove the mentioning of the second variable from clause C.2.3.

Similar issue was identified during the development of VVCv1, and the final text in there was to use the phrase "updated" instead of "derived" for these two variables in clause C.2.3.

It is therefore proposed to change the following phrase in clause C.2.3:

The variables InitCpbRemovalDelay[ SchedSelIdx ], InitCpbRemovalDelayOffset[ SchedSelIdx ], CpbDelayOffset and DpbDelayOffset are derived as follows:

to be as follows:

The variables InitCpbRemovalDelay[ SchedSelIdx ] and InitCpbRemovalDelayOffset[ SchedSelIdx ] are updated, and the variables CpbDelayOffset and DpbDelayOffset are derived, as follows:

- Replace all the eight instances of "decoder conformance to profiles" with "the decoding process". These instances are as follows:

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annex A. Decoders conforming to a profile specified in Annex A but not supporting the INBLD capability specified in Annex F shall ignore all vps_extension_data_flag syntax elements.

sps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all sps_extension_data_flag syntax elements.

pps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all pps_extension_data_flag syntax elements.

slice_segment_header_extension_data_byte may have any value. Decoders shall ignore the value of slice_segment_header_extension_data_byte. Its value does not affect decoder conformance to profiles specified in Annex A.

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annexes A, G or H. Decoders conforming to a profile specified in Annexes A, G or H shall ignore all vps_extension_data_flag syntax elements.

vps_non_vui_extension_data_byte may have any value. Decoders shall ignore the value of vps_non_vui_extension_data_byte. Its value does not affect decoder conformance to profiles specified in this version of this Specification.

slice_segment_header_extension_data_bit may have any value. Decoders shall ignore the value of slice_segment_header_extension_data_bit. Its value does not affect decoder conformance to profiles specified in this version of this Specification.

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annexes A, G, H, or I. Decoders conforming to a profile specified in Annexes A, G, H, or I shall ignore all vps_extension_data_flag syntax elements.

- In the informative Table D.1, change the persistence scope for the SEI manifest SEI message and the SEI prefix indication SEI message from "The CLVS containing the SEI message" to "The CVS containing the SEI message", to be aligned with the semantics of the SEI messages. Note that this is not needed for AVC as AVC does not have such an informative table.

- Remove a tab from each of the two middle rows below:

| content_light_level_info( payloadSize ) { | Descriptor |
| --- | --- |
| max_content_light_level | u(16) |
| max_pic_average_light_level | u(16) |
| } |  |

# Patent rights declaration(s)

Bytedance Inc. and Apply do not have any current or pending patent rights relating to the technology proposed in this contribution (to the extent of the personal awareness of the authors).