| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0056-v1 |
| --- | --- |

| Title: | AHG9: SEI manifest and SEI prefix indication SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang (Bytedance) Thomas Stockhammer (Qualcomm) Alexis Tourapis, David Singer (Apple) | Tel: Email: | yekui.wang@bytedance.com tsto@qti.qualcomm.com atourapis@apple.com singer@apple.com |
| Source: | Bytedance Inc., Qualcomm Incorporated, Apple |  |  |

_____________________________

# Abstract

This contribution proposes to bring the SEI manifest SEI message and the SEI prefix indication SEI messages that are in AVC, HEVC, and V-PCC to VVC/VSEI.

Compared to the HEVC text for these two SEI messages, only two changes are made, as follows:

- In the NOTE in the semantics of the SEI manifest SEI message, the mentioning of the two SEI messages that are not present in ITU-T H.266 | ISO/IEC 23090-3 or ITU-T H.274 | ISO/IEC 23002-7 is removed, as shown below:

NOTE – An example of such a usage of an SEI manifest SEI message is to express the expectation that there are no frame packing arrangement SEI messages, segmented rectangular frame packing arrangement SEI messages, display orientation SEI messages, or omnidirectional projection indication SEI messages in the CVS, and therefore that the rendering of the decoded video pictures for display purposes would not need any of the additional post-processing that is commonly associated with the interpretation of these SEI messages.

- The persistence scope of the two SEI messages in the informative table is changed from "The CLVS containing the SEI message" to "The CVS containing the SEI message", to be aligned with the semantics of the SEI messages. Note that this change is also proposed as an errata item for HEVC in JVET-T0054. This is not needed for AVC as AVC does not have such an informative table.

# Text for the proposed SEI manifest and SEI prefix indication SEI messages

  - SEI manifest SEI message

    - SEI manifest SEI message syntax

| sei_manifest( payloadSize ) { | Descriptor |
| --- | --- |
| manifest_num_sei_msg_types | u(16) |
| for( i = 0; i < manifest_num_sei_msg_types; i++ ) { |  |
| manifest_sei_payload_type[ i ] | u(16) |
| manifest_sei_description[ i ] | u(8) |
| } |  |
| } |  |

    - SEI manifest SEI message semantics

The SEI manifest SEI message conveys information on SEI messages that are indicated as expected (i.e., likely) to be present or not present. Such information may include the following:

–	The indication that certain types of SEI messages are expected (i.e., likely) to be present (although not guaranteed to be present) in the CVS.

–	For each type of SEI message that is indicated as expected (i.e., likely) to be present in the CVS, the degree of expressed necessity of interpretation of the SEI messages of this type, as follows:

- The degree of necessity of interpretation of an SEI message type may be indicated as "necessary", "unnecessary", or "undetermined".

- An SEI message is indicated by the encoder (i.e., the content producer) as being "necessary" when the information conveyed by the SEI message is considered as necessary for interpretation by the decoder or receiving system in order to properly process the content and enable an adequate user experience; it does not mean that the bitstream is required to contain the SEI message in order to be a conforming bitstream. It is at the discretion of the encoder to determine which SEI messages are to be considered as necessary in a particular CVS. However, it is suggested that some SEI messages, such as the frame packing arrangement, segmented rectangular frame packing arrangement, and omnidirectional projection indication SEI messages, should typically be considered as necessary.

–	The indication that certain types of SEI messages are expected (i.e., likely) not to be present (although not guaranteed not to be present) in the CVS.

NOTE – An example of such a usage of an SEI manifest SEI message is to express the expectation that there are no frame packing arrangement SEI messages or omnidirectional projection indication SEI messages in the CVS, and therefore that the rendering of the decoded video pictures for display purposes would not need any of the additional post-processing that is commonly associated with the interpretation of these SEI messages.

The content of an SEI manifest SEI message may, for example, be used by transport-layer or systems-layer processing elements to determine whether the CVS is suitable for delivery to a receiving and decoding system, based on whether the receiving system can properly process the CVS to enable an adequate user experience or whether the CVS satisfies the application needs.

When an SEI manifest SEI message is present in any access unit of a CVS, an SEI manifest SEI message shall be present in the first access unit of the CVS. The SEI manifest SEI message persists in decoding order from the current access unit until the end of the CVS. When there are multiple SEI manifest SEI messages present in a CVS, they shall have the same content.

An SEI NAL unit containing an SEI manifest SEI message shall not contain any other SEI messages other than SEI prefix indication SEI messages. When present in an SEI NAL unit, the SEI manifest SEI message shall be the first SEI message in the SEI NAL unit.

manifest_num_sei_msg_types specifies the number of types of SEI messages for which information is provided in the SEI manifest SEI message.

manifest_sei_payload_type[ i ] indicates the payloadType value of the i-th type of SEI message for which information is provided in the SEI manifest SEI message. The values of manifest_sei_payload_type[ m ] and manifest_sei_payload_type[ n ] shall not be identical when m is not equal to n.

manifest_sei_description[ i ] provides information on SEI messages with payloadType equal to manifest_sei_payload_type[ i ] as specified in Table X.

Table X – Interpretation of manifest_sei_description[ i ]

| - Value | - Description |
| --- | --- |
| 0 | Indicates that there is no SEI message with payloadType equal to manifest_sei_payload_type[ i ] expected to be present in the CVS. |
| 1 | Indicates that there are SEI messages with payloadType equal to manifest_sei_payload_type[ i ] expected to be present in the CVS, and these SEI messages are considered as necessary. |
| 2 | Indicates that there are SEI messages with payloadType equal to manifest_sei_payload_type[ i ] expected to be present in the CVS, and these SEI messages are considered as unnecessary. |
| 3 | Indicates that there are SEI messages with payloadType equal to manifest_sei_payload_type[ i ] expected to be present in the CVS, and the necessity of these SEI messages is undetermined. |
| 4..255 | Reserved |

The value of manifest_sei_description[ i ] shall be in the range of 0 to 3, inclusive, in bitstreams conforming to this version of this Specification. Other values for manifest_sei_description[ i ] are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of manifest_sei_description[ i ] greater than or equal to 4 to appear in the syntax and shall ignore all information for payloadType equal to manifest_sei_payload_type[ i ] signalled in the SEI manifest SEI message and shall ignore all SEI prefix indication SEI messages with prefix_sei_payload_type equal to manifest_sei_payload_type[ i ] when manifest_sei_description[ i ] is greater than or equal to 4.

  - SEI prefix indication SEI message

    - SEI prefix indication SEI message syntax

| sei_prefix_indication( payloadSize ) { | Descriptor |
| --- | --- |
| prefix_sei_payload_type | u(16) |
| num_sei_prefix_indications_minus1 | u(8) |
| for( i = 0; i <= num_sei_prefix_indications_minus1; i++ ) { |  |
| num_bits_in_prefix_indication_minus1[ i ] | u(16) |
| for( j = 0; j <= num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

    - SEI prefix indication SEI message semantics

The SEI prefix indication SEI message carries one or more SEI prefix indications for SEI messages of a particular value of payloadType. Each SEI prefix indication is a bit string that follows the SEI payload syntax of that value of payloadType and contains a number of complete syntax elements starting from the first syntax element in the SEI payload.

Each SEI prefix indication for an SEI message of a particular value of payloadType indicates that one or more SEI messages of this value of payloadType are expected (i.e., likely) to be present in the CVS and to start with the provided bit string. A starting bit string would typically contain only a true subset of an SEI payload of the type of SEI message indicated by the payloadType, may contain a complete SEI payload, and shall not contain more than a complete SEI payload. It is not prohibited for SEI messages of the indicated value of payloadType to be present that do not start with any of the indicated bit strings.

These SEI prefix indications should provide sufficient information for indicating what type of processing is needed or what type of content is included. The former (type of processing) indicates decoder-side processing capability, e.g., whether some type of frame unpacking is needed. The latter (type of content) indicates, for example, whether the bitstream contains subtitle captions in a particular language.

The content of an SEI prefix indication SEI message may, for example, be used by transport-layer or systems-layer processing elements to determine whether the CVS is suitable for delivery to a receiving and decoding system, based on whether the receiving system can properly process the CVS to enable an adequate user experience or whether the CVS satisfies the application needs (as determined in some manner by external means outside the scope of this Specification).

In one example, when the payloadType indicates the frame packing arrangement SEI message, an SEI prefix indication should include up to at least the syntax element frame_packing_arrangement_type; and when the payloadType indicates the omnidirectional projection indication SEI message, an SEI prefix indication should include up to at least the syntax element projection_type.

In another example, for user data registered SEI messages that are used to carry captioning information, an SEI prefix indication should include up to at least the language code; and for user data unregistered SEI messages extended for private use, an SEI prefix indication should include up to at least the UUID.

When an SEI prefix indication SEI message is present in any access unit of a CVS, an SEI prefix indication SEI message shall be present in the first access unit of the CVS. The SEI prefix indication SEI message persists in decoding order from the current access unit until the end of the CVS. When there are multiple SEI prefix indication SEI messages present in a CVS for a particular value of payloadType, they shall have the same content.

An SEI NAL unit containing an SEI prefix indication SEI message for a particular value of payloadType shall not contain any other SEI messages other than an SEI manifest SEI message and SEI prefix indication SEI messages for other values of payloadType.

prefix_sei_payload_type indicates the payloadType value of the SEI messages for which one or more SEI prefix indications are provided in the SEI prefix indication SEI message. When an SEI manifest SEI message is also present for the CVS, the value of prefix_sei_payload_type shall be equal to one of the manifest_sei_payload_type[ m ] values for which manifest_sei_description[ m ] is equal to 1 to 3, inclusive, as indicated by an SEI manifest SEI message that applies to the CVS.

num_sei_prefix_indications_minus1 plus 1 specifies the number of SEI prefix indications.

num_bits_in_prefix_indication_minus1[ i ] plus 1 specifies the number of bits in the i-th SEI prefix indication.

sei_prefix_data_bit[ i ][ j ] specifies the j-th bit of the i-th SEI prefix indication.

The bits sei_prefix_data_bit[ i ][ j ] for j ranging from 0 to num_bits_in_prefix_indication_minus1[ i ], inclusive, follow the syntax of the SEI payload with payloadType equal to prefix_sei_payload_type, and contain a number of complete syntax elements starting from the first syntax element in the SEI payload syntax, and may or may not contain all the syntax elements in the SEI payload syntax. The last bit of these bits (i.e., the bit sei_prefix_data_bit[ i ][ num_bits_in_prefix_indication_minus1[ i ] ]) shall be the last bit of a syntax element in the SEI payload syntax, unless it is a bit within an itu_t_t35_payload_byte or user_data_payload_byte.

NOTE – The exception for itu_t_t35_payload_byte and user_data_payload_byte is provided because these syntax elements may contain externally-specified syntax elements, and the determination of the boundaries of such externally-specified syntax elements is a matter outside the scope of this Specification.

byte_alignment_bit_equal_to_one shall be equal to 1.

  - Addition to Table 4 in the VSEI spec

Table 4 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| SEI manifest | The CVS containing the SEI message |
| SEI prefix indication | The CVS containing the SEI message |

# Patent rights declaration(s)

Bytedance Inc. does not have any current or pending patent rights relating to the technology proposed in this contribution (to the extent of the personal awareness of the authors).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Apple may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).