| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0057_r3 |
| --- | --- |

| Title: | AHG11: A Case Study to Reduce Computation of Neural Network based In-Loop Filter by Pruning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Wei Wang, Wei Jiang, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 {cauyeung, rickweiwang, vwjiang, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In general, the method of zeroing out weights, i.e. pruning, is applicable to reduce the number of multiply-adds in a convolutional neural network. To study the effect of pruning to coding efficiency, this contribution uses the dense residual convolutional neural network based in-loop filter (DRNLF) from JVET-O0101 as a case study. In this case study, the DRNLF was integrated with VTM-10.0 and tested according to testing condition in JVET-T0041, with and without pruning of 25% of the filters in the convolution layers. Without 25% filter pruning, the DRNLF resulted in BD-rate of -0.96% / -0.77% / -0.81% / -1.31% for RA/LDB/LDP/AI configuration. With 25% filter pruning, BD-rate became -0.79% / -0.64% / -0.65 % / -1.07%, respectively. In general, 25% pruning can lead to a reduction of 25% of the number of multiply-add in a convolutional neural network based coding tool and save 25% of neural network parameters.

# Introduction

The purpose of this contribution has two aspects. In one aspect, this contribution reports the coding efficiency of the dense residual convolutional neural network based in-loop filter (DRNLF) from JVET-O0101 [1] when it is integrated in VTM-10.0 and tested according to AHG11 [2] testing conditions. In the other aspect, this contribution reports how the number of multiply-adds in a convolution neural network can be removed to potentially reduce computation and so is the size of neural network parameters. This contribution uses the DRNLF as a case study to report the trade-off of multiply-add reduction, by zeroing out weights in DRNLF, and its impact on coding efficiency.

# Dense residual convolutional neural network based in-loop filter (DRNLF) from JVET-O0101

The DRNLF from JVET-O0101 [1] is integrated into VTM-10.0. It is an additional filter between deblocking filter and SAO. It works with deblocking, SAO, ALF and CCALF to improve coding efficiency. Fig. 1 shows the network structure of the dense residual convolutional neural network based in-loop filter (DRNLF), where N and M denotes the number of dense residual unit (DRU) and convolution kernel, respectively. Specifically, N is set to 4 and M is set to 32 for the tradeoff between computational efficiency and performance. Normalized QP map is concatenated with the reconstructed frame as the input to DRN.

Figure 1: Network structure of DRNLF.

The main body of the DRNLF consists of series of DRU. The structure of DRU is shown in Fig. 2. The DRU directly propagates its input to the subsequent unit through the shortcut. To further reduce the computational cost, 3x3 depth-wise separable convolutional (DSC) layer is applied in DRU.

Figure 2: Dense Residual Unit (DRU)

Finally, the output of the network has 3 channels, which corresponds to Y, Cb, Cr, respectively. The filter is applied to both intra and inter pictures. Additional flags are signaled to indicate the on/off of the DRNLF at picture level and CTU level.

# Reduction of computation of DRNLF by pruning

The computation of a convolutional neural network is related to the size of the 4D weight tensor Wnmhw in each convolution layer of the neural network, where n is the number of output filters (filter), m is the number of input channels (channel), and h×w is the size of the 2D convolution kernel. The computation of a convolution layer could be reduced by zeroing out some of the coefficients in W and, therefore, reduce the number of multiply-add operations. This method of zeroing out the coefficients in W is known as pruning in neural network compression. In this contribution, the DRNLF is presented as a case study to study the impact of coding efficiency and pruning. This contribution reports the results of the DRNNLF pruning out the number of output filters by 25%.

As shown in Figure 3A, the 4D weight tensor W can be unfolded as a two-dimensional array where each element in the 2D array is a 1D array which consists of the serialized filter coefficients of a h×w 2D convolution kernel. In Figure 3A, the column of the 2D array corresponds to the input channels. The row of the 2D array corresponds to the output filters. This contribution considers the pruning of the coefficients in W in Figure 3A by filter pruning which zeros out 25% of the rows depicted in Figure 3B. Consequently, both the number of multiply-add and the size of neural network parameters can be reduced by 25%.

For software implementation of filter pruning on CPU/GPU device, the speed increase is in relation to prune rate. While the software implementation is still undergoing, the memory saving and MAC saving in this proposal are estimated by prune rate.

Figure 3 (A) The 4D array W is unfolded as a 2D array with each element in the 2D array consists of a 1D array of the serialized 2D filter coefficients. (B) The 2D array can be pruned by row, by column, or by the position in the 1D array to reduce the number of multiple-adds.

# Network Information

The network information for training and inference with and without pruning is the same. The network information for training and influence is reported in this section. In particular, the training was performed with GPU and QP = 22, 27, 32, 37; the inference was performed with CPU only and QP = 22, 27, 32, 37, 42.

## Training

Table 1: Network Information for DRNLF in Training Stage.

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Nvidia DGX station (2x Intel Xeon E5-2698 v4 2.2 GHz x 20, 256GB RAM and 4x 32GB Tesla V100) |
|  |  | 2x Tesla V100 are used for training |
|  | SW environment: | Ubuntu 18.04.2 |
|  |  | Cuda 9.0, Cudnn 7.1.0.2 |
|  | Framework: | PyTorch v0.4.1 |
|  | Epoch: | 300 |
|  | Batch size: | 16 |
|  | Training time: | 24h |
|  | Training data information: | DIV2K(compressed images and corresponding uncompressed images) |
|  | Configurations for generating compressed training data (if different to VTM CTC): | QP 22, 27, 32, 37 |
| Optional |  |  |
|  | Patch size | 64x64 |
|  | Learning rate: | 2.00E-05 |
|  | Optimizer: | ADAM |
|  | Loss function: | L1 |
|  | Preprocessing: | randomly cropped |
|  | Other information: | Depth separable convolution |

## Inference

Table 2: Network Information for DRNLF Testing in Inference Stage without Pruning.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) CPU E5-2620 v3 |
|  | SW environment: | CentOS Linux 7 |
|  |  | CPU |
|  | Framework: | Pytorch 1.2.0 |
|  | Total Conv. Layers | 13 |
|  | Total FC Layers | 0 |
|  | Total Parameter Number | 22371 |
|  | Framework: | 32 (F) |
|  | Memory Parameter (MB) | 0.09 |
|  | Memory Temp (MB) | 20344.92188 |
|  | MAC (Giga) per 3840x2160 pixels | 186.64 G |
| Optional | Preprocessing: | No |
|  | Other information: | Depth separable convolution |

Table 3: Network Information for DRNLF Testing in Inference Stage with Pruning.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) CPU E5-2620 v3 |
|  | SW environment: | CentOS Linux 7 |
|  |  | CPU |
|  | Framework: | Pytorch 1.2.0 |
|  | Total Conv. Layers | 13 |
|  | Total FC Layers | 0 |
|  | Total Parameter Number | 16778 |
|  | Framework: | 32 (F) |
|  | Memory Parameter (MB) | 0.067 |
|  | Memory Temp (MB) | 15238.70(estimation) |
|  | MAC (Giga) | 139.98 G (estimation) |
| Optional | Preprocessing: | No |
|  | Other information: | Depth separable convolution |

# Results

## Overall results summary

In the overall test results, the DRNLF with no pruning resulted in BD-rate of -0.96% / -0.77% / -0.81% / -1.31% for RA/LDB/LDP/AI configuration. With 25% filter pruning, BD-rate became -0.79% / -0.64% / -0.65 % / -1.07%, respectively. Although no attempt was made to reduce the actual number of multiply-add in the decoder inference stage, 25% filter pruning resulted in -2.73%/-1.44%/-4.87%/-0.37% in CPU decoding time relatively to without pruning.

|  | Random Access Over VTM-10.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| No Pruning | -0.96% | -3.88% | -3.82% | 105% | 3334% |  |  |
| 25% Filter Pruning | -0.79% | -3.70% | -2.82% | 105% | 3244% |  |  |
|  | Low delay B Over VTM-10.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Overall | -0.77% | -4.88% | -4.64% | 107% | 3904% |  |  |
| 25% Filter Pruning | -0.64% | -4.58% | -4.17% | 106% | 3848% |  |  |
|  | Low delay P Over VTM-10.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Overall | -0.81% | -5.06% | -5.12% | 110% | 4370% |  |  |
| 25% Filter Pruning | -0.65% | -4.95% | -4.49% | 109% | 4157% |  |  |
| (Optional) | All Intra Over VTM-10.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Overall | -1.31% | -2.69% | -3.38% | 108% | 3431% |  |  |
| 25% Filter Pruning | -1.07% | -2.79% | -2.74% | 108% | 3418% |  |  |

## Results without pruning

|  | Random Access Over VTM-10.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 | -1.05% | -3.35% | -5.38% | 107% | 4473% |  |  |
| Class A2 | -1.30% | -4.22% | -2.58% | 106% | 3201% |  |  |
| Class B | -0.74% | -4.13% | -3.56% | 105% | 2772% |  |  |
| Class C | -0.93% | -3.70% | -3.89% | 104% | 3475% |  |  |
| Class E | O | O | O | O | O | O | O |
| Overall | -0.96% | -3.88% | -3.82% | 105% | 3334% |  |  |
| Class D | -1.05% | -2.34% | -3.28% | 104% | 2011% |  |  |
| Class F | -0.31% | -1.70% | -1.74% | 109% | 1476% |  |  |
|  | Low delay B Over VTM-10.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 | O | O | O | O | O | O | O |
| Class A2 | O | O | O | O | O | O | O |
| Class B | -0.51% | -5.33% | -4.53% | 106% | 4346% |  |  |
| Class C | -0.58% | -6.43% | -6.70% | 103% | 4978% |  |  |
| Class E | -1.44% | -2.06% | -2.05% | 114% | 2361% |  |  |
| Overall | -0.77% | -4.88% | -4.64% | 107% | 3904% |  |  |
| Class D | -0.91% | -3.85% | -4.63% | 103% | 2783% |  |  |
| Class F | -0.33% | -3.29% | -2.35% | 111% | 2247% |  |  |
|  | Low delay P Over VTM-10.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 | O | O | O | O | O | O | O |
| Class A2 | O | O | O | O | O | O | O |
| Class B | -0.53% | -5.78% | -5.15% | 108% | 4951% |  |  |
| Class C | -0.59% | -6.58% | -6.53% | 105% | 5279% |  |  |
| Class E | -1.57% | -1.83% | -3.17% | 120% | 2759% |  |  |
| Overall | -0.81% | -5.06% | -5.12% | 110% | 4370% |  |  |
| Class D | -0.98% | -4.43% | -4.12% | 105% | 3552% |  |  |
| Class F | -0.37% | -3.64% | -3.49% | 112% | 2216% |  |  |
| (Optional) | All Intra Over VTM-10.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 | -0.96% | -2.05% | -4.46% | 114% | 3405% |  |  |
| Class A2 | -1.22% | -2.70% | -1.95% | 108% | 2886% |  |  |
| Class B | -0.89% | -2.91% | -3.50% | 106% | 3037% |  |  |
| Class C | -1.58% | -3.63% | -4.24% | 104% | 3353% |  |  |
| Class E | -2.11% | -1.69% | -2.36% | 108% | 5192% |  |  |
| Overall | -1.31% | -2.69% | -3.38% | 108% | 3431% |  |  |
| Class D | -2.05% | -2.09% | -4.53% | 103% | 2982% |  |  |
| Class F | -0.67% | -2.16% | -2.07% | 104% | 2180% |  |  |

## Results with 25% filter pruning

|  | Random Access Over VTM-10.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 | -0.73% | -3.36% | -2.26% | 106% | 3775% |  |  |
| Class A2 | -1.14% | -4.05% | -2.56% | 106% | 3130% |  |  |
| Class B | -0.65% | -3.95% | -3.14% | 105% | 2851% |  |  |
| Class C | -0.74% | -3.37% | -3.02% | 104% | 3492% |  |  |
| Class E | O | O | O | O | O | O | O |
| Overall | -0.79% | -3.70% | -2.82% | 105% | 3244% |  |  |
| Class D | -0.78% | -1.91% | -2.98% | 104% | 2078% |  |  |
| Class F | -0.39% | -1.38% | -1.56% | 110% | 1814% |  |  |
|  | Low delay B Over VTM-10.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 | O | O | O | O | O | O | O |
| Class A2 | O | O | O | O | O | O | O |
| Class B | -0.43% | -5.25% | -3.98% | 105% | 4392% |  |  |
| Class C | -0.47% | -5.82% | -5.41% | 103% | 4858% |  |  |
| Class E | -1.23% | -1.84% | -2.82% | 113% | 2262% |  |  |
| Overall | -0.64% | -4.58% | -4.17% | 106% | 3848% |  |  |
| Class D | -0.64% | -3.17% | -4.85% | 103% | 2862% |  |  |
| Class F | -0.36% | -2.64% | -1.37% | 110% | 2612% |  |  |
|  | Low delay P Over VTM-10.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 | O | O | O | O | O | O | O |
| Class A2 | O | O | O | O | O | O | O |
| Class B | -0.47% | -5.78% | -4.11% | 107% | 4793% |  |  |
| Class C | -0.42% | -6.06% | -5.33% | 105% | 5046% |  |  |
| Class E | -1.26% | -2.08% | -4.00% | 118% | 2533% |  |  |
| Overall | -0.65% | -4.95% | -4.49% | 109% | 4157% |  |  |
| Class D | -0.63% | -3.70% | -3.71% | 106% | 3714% |  |  |
| Class F | -0.43% | -2.83% | -2.80% | 112% | 2644% |  |  |
| (Optional) | All Intra Over VTM-10.0 |  |  |  |  |  |  |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 | -0.68% | -2.30% | -2.20% | 115% | 3488% |  |  |
| Class A2 | -0.99% | -2.74% | -1.88% | 108% | 2847% |  |  |
| Class B | -0.79% | -2.89% | -2.97% | 106% | 2985% |  |  |
| Class C | -1.23% | -3.66% | -3.48% | 104% | 3400% |  |  |
| Class E | -1.80% | -2.03% | -2.79% | 107% | 5078% |  |  |
| Overall | -1.07% | -2.79% | -2.74% | 108% | 3418% |  |  |
| Class D | -1.52% | -1.78% | -3.76% | 104% | 2735% |  |  |
| Class F | -0.74% | -1.94% | -1.91% | 103% | 2715% |  |  |

# References

- Yinbin Wang, Tong Ouyang, Cong Zou, Yiming Li, Zhenzhong Chen, Liang Zhao, Shan Liu, Xiang Li, “CE10: Dense Residual Convolutional Neural Network based In-Loop Filter (Tests 2.5 and 2.7)”, JVET-O0101-v2,15th Meeting: Gothenburg, SE, 3–12 July 2019.

- Shan Liu, Andrew Segall, Elena Alshina, Jill Boyce, “Methodology and reporting template for neural network coding tool testing”, JVET-T0041-v2, 20th Meeting: by teleconference, 7 – 16 October 2020.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).