| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 # 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0058-v3 |
| --- | --- |

| Title: | AHG11: Information on inter-prediction coding tool with deep neural network |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi, Zeqiang Li, Wei Wang, Wei Jiang, Xiaozhong Xu, Shan Liu | Email: | {bdchoi, zeqiangli}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This informative contribution reports some preliminary results of deep neural network (DNN) utilization for inter-prediction. The idea is inserting of a new (virtual) reference picture, which is generated by inference processes of trained networks, into reference picture list (RPL). The entire networks consist of several sub-network models for edge-detection, optical flow estimation/compensation and detail enhancement. Since networks were trained with small size training sequences, classes C&D show outstanding coding gains (2.18%, 3.93% respectively in luma for RA), in comparison to other classes.

# Introduction

Deep neural network (DNN) is a powerful tool of interest in video compression as well as computer vision. Recently, a number of academic papers and literatures [1]-[6] reported the considerable improvements in terms of coding efficiency and visual improvements, while their computational complexities are still high. In JVET, DNN-based in-loop filtering and intra-/inter-prediction tools have been researched in the context of AhG activities [7]-[9]. As an aspiration to develop DNN-based video coding standard in near future, we researched and developed a DNN-based inter-prediction tool based on VTM10. Although only prototype-level tool description and its preliminary results are reported in this informative contribution, its reported coding gains show promising results of the DNN-based coding tool. This contribution document consists of several sections with the introduction of the proposed DNN-based inter-prediction framework, neural network models, simulation results with coding performance and complexity analysis, and finally discussion on future directions.

# Framework of inter-prediction with neural networks

Utilization of frame rate up conversion (FRUC) techniques [10] and decoder-side motion vector derivation has been actively tried and proposed in JVET and its former standard groups. While the new coding tools DMVR and BIO have been adopted into VVC v1, FRUC technologies that utilize full optical flow estimation and blending multiple reference pictures for inter-prediction are still under research. Observing recent academic papers [1]-[6] reported outstanding results on DNN-based FRUC, we tried to apply the state-of-the-art DNN-based FRUC technology to inter-prediction coding in VTM10. Fig. 1 shows a basic inter-prediction framework with neural network models. The framework is inspired by [5].

Figure 1. Proposed inter-prediction framework with neural network process

When multiple decoded pictures are stored in decoded picture buffer (DPB) as reference pictures for motion-compensated prediction, one or more forward and backward reference pictures may be available according to the GOP structure. By default, one nearest forward reference picture and one nearest backward one that have minimum POC distances from the current picture are used for generating the virtual reference picture. For example, as shown in Fig.1, when the POC value of the current picture is equal to 5 with a given GOP structure, the decoded reference pictures with POC values equal to 4 or 6 are used for generating a virtual reference picture of the current picture, as input data of network inference process. If the virtual reference picture is generated through multiple network inference processes including optical flow estimation-compensation and detail enhancement, the virtual reference picture that is synthesized to be similar to the current picture is stored in DPB as another reference picture and used for motion-compensated prediction. The POC value of the virtual reference picture is set equal to the POC value of the current picture (POC 5 in the example in Fig. 1). To avoid any conflict with temporal motion vector prediction or POC-based MV scaling, the values of MVs of the virtual reference picture are set equal to zero by default and the virtual picture is marked as “long-term reference”. Ideally, the virtual reference picture for en-/decoding the current picture can be used for en-/decoding the following picture. However, in our current implementation, the virtual reference picture is removed from DPB, when decoding process of the current picture is completed.

For coding large sequences (e.g. 4K or 8K), network inference process cannot be applied to the entire picture due to resource limits (e.g. memory). In that case, the virtual reference picture is hypothetically divided into multiple virtual regions and each region is independently generated through network inference process. The independently generated virtual reference regions are used for composition of the entire virtual reference or used for region-wise inter-prediction process. The following section have more details on network models for generation of virtual picture or region.

# Neural network models

For spatial alignment between multiple sequential pictures, usually optical flow estimation and compensation with inference process of pre-trained neural network are utilized in motion-compensated frame interpolation. From our research on frame interpolation or rate up-conversion based on neural network, however, we observed that the edges and features of pictures are blurred and lost through the typical optical flow estimation and compensation with picture fusion process. To overcome the limits of the conventional approaches and generate the high-quality reference picture for better coding efficiency, we proposed three step (optical flow estimation, compensation and detail enhancement) network inference processes with several pre-trained models, as shown in Fig. 2.

Figure 2. Proposed neural network process for generation of virtual reference picture

When reference data are fed into the optical flow estimation module, the edges of reference pictures are detected with pre-trained edge detecting neural network. The strong edges or corner points are considered features. When the edges, boundaries or features are obtained, feature-centric optical flow vectors are estimated. The process of optical flow compensation is composed of warping process with offset compensation and blending with weighted average. If two or more intermediate virtual reference pictures are generated from multiple input reference pictures, those intermediate frames are blended into one virtual reference picture, with weighted average that are pre-trained.

Once an intermediate virtual reference is obtained, the image quality of the virtual reference picture may be enhanced with a post-processing, utilizing two input reference picture and their pre-obtained edge and feature data. In this inference process shallow edge or features obtained from each (input) decoded reference picture are spatially compensated or shifted with pre-trained network modules and blended again with the generated virtual reference picture to enhance the details.

# Simulation results

The below tables show the complexity analysis and the coding performance based on [9].

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | CPU: Intel(R) Xeon(R) CPU E5-2698 v4 @ 2.20GHz GPU: Tesla V100-DGXS-32GB |
|  | SW environment: | OS: Ubuntu 18.04.2 LTS GPU driver: Nvidia driver 418.67 |
|  | Framework: | Pytorch 1.14 + CUDA 10.1 + cuDNN 7.6.4 |
|  | Epoch: | 70 |
|  | Batch size: | 8 |
|  | Training time: | 7 days |
|  | Training data information: | Xiph.org[11], BVI-DVC [12] |
|  | Configurations for generating compressed training data (if different to VTM CTC): | Same as VTM CTC |

Table 2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
|  | HW environment: | CPU: Intel Core i9-9900x CPU @ 3.60GHz, 32GB Memory GPU: RTX 2080ti-11GB |
|  | SW environment: | OS: Ubuntu 20.04.3 GPU driver: Nvidia driver 400 |
|  | Framework: | Pytorch 1.14 + CUDA 10.1 + cuDNN 7.6.4 |
|  | Total Parameter Number | 125,993,047 |
|  | MAC (Giga) | 200 GMac |

Table 3. BD-rate and Coding Time for NN-based Video Coding Tool Testing in Inference Stage 
(Tested 33frames with GOP=32 and patch size 512x512)

|  | Random Access Over VTM-10.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate (Y) | BD-rate (U) | BD-rate (V) | Encoding Time (CPU) | Decoding Time (CPU) | Encoding Time (GPU) | Decoding Time (GPU) |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.68% | 1.43% | 1.52% |  |  | 133.82% | 45103.39% |
| Class C | -2.18% | -0.27% | 1.37% |  |  | 116.60% | 34643.88% |
| Class E | O | O | O | O | O | O | O |
| Overall |  |  |  |  |  |  |  |
| Class D | -3.93% | 4.27% | 6.51% |  |  | 128.06% | 59024.62% |
| Class F |  |  |  |  |  |  |  |

# Future directions and conclusion

Although the proposed neural network-based inter-prediction method shows promising results, its computational complexity is still as high as other neural network-based inter-prediction tools, especially on the decoding time. One possible way to reduce the decoding complexity could be generating the virtual reference data through block-wise inference processing, only when the region of the block is used as reference data at decoder. However, reducing the patch size of the inference process may degrade the overall coding gain, because it disallows any optical flow estimation/compensation across patch boundaries. Another possible way to reduce inference complexity could be optimizing the networks for edge/feature-detection, optical flow-estimation/compensation and detail enhancement.

For improvement of coding efficiency, interactions with CU-level inter-prediction tools would be further studied. Most inter-prediction coding tools such as Merge, sub-CU inter-prediction (e.g affine mode), DMVR, and BIO are affected by the insertion of virtual reference picture. Those inter-prediction coding tools can be normatively or non-normatively optimized for better interactions with the proposed neural network based inter prediction. Another important point is model training. In this simulation, network models have been trained with small resolution images for fast evaluation. Therefore, it cannot cope with large motion in large sequences. In the next meetings, we will report simulation results with network models trained with large motions.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] S. Niklaus, L. Mai, and F. Liu. “Video frame interpolation via adaptive separable convolution”, IEEE ICCV, Oct 2017.

[2] W. Bao, W.-S. Lai, X. Zhang, Z. Gao, and M.-H. Yang, “Memc-net: Motion estimation and motion compensation driven neural network for video interpolation and enhancement”, IEEE Trans. on Pattern Analysis and Machine Intelligence, 2018.

[3] Y. Tian, Y. Zhang, Y. Fu, and C. Xu, “TDAN: Temporally deformable alignment network for video super-resolution”, arXiv preprint arXiv:1812.02898, 2018.

[4] S. Gui, C. Wang, Q. Chen and D. Tao, “FeatureFlow: Robust Video Interpolation via Structure-to-texture Generation”, IEEE CVPR, 2020.

[5] H. Choi and I. V. Bajić, "Deep Frame Prediction for Video Coding," in IEEE Transactions on Circuits and Systems for Video Technology, vol. 30, no. 7, pp. 1843-1855, July 2020.

[6] S. Huo, D. Liu, F. Wu and H. Li, "Convolutional neural network-based motion compensation refinement for video coding", Proc. IEEE ISCAS, pp. 1-4, May 2018.

[7] S. Liu, L. Wang, P. Wu, H. Yang, “JVET AHG report 9: Neural Networks in Video Coding (AHG9)”, ISO/IEC JTC1/SC29/WG11 JVET-J0009.

[8] Y. Li, S. Liu, K. Kawamura, "Methodology and reporting template for neural network coding tool testing”, ISO/IEC JTC1/SC29/WG11 JVET-M1006.

[9] S. Liu, A. Segall, E. Alshina, J. Boyce, M. Wien, D. Grois, “Methodology and reporting template for neural network coding tool testing”, ISO/IEC JTC1/SC29 JVET-T0041.

[10] B.-D. Choi, J.-W. Han, C.-S. Kim and S.-J. Ko, "Motion-compensated frame interpolation using bilateral motion estimation and adaptive overlapped block motion compensation", IEEE Trans. Circuits Syst. Video Technol., vol. 17, no. 4, pp. 407-416, Apr. 2007.

[11] https://media.xiph.org/video/derf/

[12] Fan Zhang, Di Ma, David Bull: BVI-DVC. https://doi.org/10.5523/bris.3hj4t64fkbrgn2ghwp9en4vhtn