| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0060 |
| --- | --- |

| Title: | AHG4: new video conference sequences for HD verification testing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shengyang Xu Ru-Ling Liao Jianhua Chen Yan Ye | Tel: Email: | ruling.lrl@alibaba-inc.com yan.ye@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution proposes six video conference sequences for HD verification testing. The brief description and RD performance of the proposed sequences are also included in this contribution.

Introduction

This contribution provides six video conference sequences as candidates for verifying the coding efficiency of the VVC standard.

Sequences

All the sequences are captured using Logitech Webcam C925e. The sequences are recorded in 4:2:2 color format and converted to YUV 4:2:0 color format using ffmpeg:

ffmpeg -f avfoundation -pixel_format uyvy422 -video_size 1920x1080 -framerate 30 -i 1: -r 30 in.yuv

ffmpeg -s 1920x1080 -pix_fmt uyvy422 -i in.yuv -c:v rawvideo -format rawvideo -pix_fmt yuv420p out.yuv

Table 1 and Table 2 show the detailed information of the sequences.

Table 1. Specification of the proposed sequences

| Sequences | Resolution@FPS | Frames | Bit Depth |
| --- | --- | --- | --- |
| BGXG_TwoPeopleTalking_1920x1080_30_8bit_420_600f.yuv | 1920x1080@30 | 600 | 8 |
| BGXG_OnePeopleLeave_1920x1080_30_8bit_420_600f.yuv | 1920x1080@30 | 600 | 8 |
| BGStatic2Moving_OnePeople_1920x1080_30_8bit_420_450f.yuv | 1920x1080@30 | 450 | 8 |
| BGMoving2Sit_Onepeople_1920x1080_30_8bit_420_600f.yuv | 1920x1080@30 | 600 | 8 |
| BGMoving2Static_OnePeople_1920x1080_30_8bit_420_450f.yuv | 1920x1080@30 | 450 | 8 |
| BGXG_OnePeopleMoving_1920x1080_30_8bit_420_450f.yuv | 1920x1080@30 | 450 | 8 |

Table 2. Md5Sum of sequences

| Sequences | MD5 |
| --- | --- |
| BGXG_TwoPeopleTalking_1920x1080_30_8bit_420_600f.yuv | c6204e9d174612b05f3b617ff2e0027b |
| BGXG_OnePeopleLeave_1920x1080_30_8bit_420_600f.yuv | 2ecc1622998dea32889b3a93550eda88 |
| BGStatic2Moving_OnePeople_1920x1080_30_8bit_420_450f.yuv | fe3b72f9f4fd3474a93b66e4db10c4be |
| BGMoving2Sit_Onepeople_1920x1080_30_8bit_420_600f.yuv | 38abc7bbfea5d60eac5f39b63ec6b183 |
| BGMoving2Static_OnePeople_1920x1080_30_8bit_420_450f.yuv | 5dcfe2bc3904dcbf33d58c9164b7ddc9 |
| BGXG_OnePeopleMoving_1920x1080_30_8bit_420_450f.yuv | 4c402ad2df23fa6aeecfb9c9174f9191 |

Sequence Thumbnails

Figure 1. BGXG_TwoPeopleTalking_1920x1080_30_8bit_420_600f.yuv 
(Two people talk to each other. Static background with illumination changes and subtle camera motion)

Figure 2. BGXG_OnePeopleLeave_1920x1080_30_8bit_420_600f.yuv 
(Static background. The person walks away after talking)

Figure 3. BGStatic2Moving_OnePeople_1920x1080_30_8bit_420_450f.yuv 
(People are moving while talking. There are background changes, camera motions, autofocus effect and illumination changes)

Figure 4. BGMoving2Sit_Onepeople_1920x1080_30_8bit_420_600f.yuv 
(Background changes, camera motions and illumination changes)

Figure 5. BGMoving2Static_OnePeople_1920x1080_30_8bit_420_450f.yuv 
(Background changes, camera motions and illumination changes)

Figure 6. BGXG_OnePeopleMoving_1920x1080_30_8bit_420_450f.yuv 
(Background changes, camera motions and illumination changes)

RD performance

The RD performance of the proposed sequences are evaluated using VTM-10.0 and HM-16.22 with low delay B configuration. For each sequence, the first 300 frames are encoded with QP 22, 27, 32, and 37. The BD-rate results are shown in Table 3.

Table 3. Bitrate and PSNR results of sequences

|  | QP | HM-16.22 |  |  |  | VTM-10.0 |  |  |  | Y BD-rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | kbps | Y psnr | U psnr | V psnr | kbps | Y psnr | U psnr | V psnr |  |
| BGXG_TwoPeopleTalking | 22 | 4906.07 | 43.78 | 48.12 | 47.21 | 4448.74 | 44.14 | 48.18 | 47.29 | -29.57% |
|  | 27 | 1052.67 | 40.97 | 46.64 | 45.52 | 909.15 | 41.34 | 46.92 | 45.87 |  |
|  | 32 | 438.49 | 38.65 | 45.27 | 44.07 | 360.76 | 39.24 | 45.48 | 44.34 |  |
|  | 37 | 234.31 | 36.10 | 43.95 | 42.75 | 189.91 | 36.89 | 44.24 | 42.71 |  |
| BGXG_OnePeopleLeave | 22 | 4796.73 | 43.77 | 48.38 | 48.03 | 4424.24 | 44.15 | 48.47 | 48.13 | -31.35% |
|  | 27 | 924.07 | 40.98 | 46.72 | 46.38 | 804.96 | 41.38 | 47.12 | 46.77 |  |
|  | 32 | 364.06 | 38.69 | 45.31 | 44.94 | 295.19 | 39.32 | 45.63 | 45.36 |  |
|  | 37 | 196.79 | 36.11 | 43.94 | 43.55 | 156.46 | 36.98 | 44.31 | 43.66 |  |
| BGStatic2Moving_OnePeople | 22 | 3706.94 | 46.10 | 48.81 | 48.11 | 3094.49 | 46.51 | 48.98 | 48.25 | -33.16% |
|  | 27 | 1368.04 | 43.21 | 47.50 | 46.68 | 1112.21 | 43.81 | 47.72 | 46.86 |  |
|  | 32 | 649.15 | 40.49 | 46.39 | 45.56 | 521.22 | 41.26 | 46.47 | 45.52 |  |
|  | 37 | 344.29 | 37.85 | 45.21 | 44.32 | 276.18 | 38.72 | 45.38 | 44.27 |  |
| BGMoving2Sit_Onepeople | 22 | 3445.84 | 46.52 | 49.29 | 48.82 | 2801.71 | 46.95 | 49.52 | 49.01 | -36.11% |
|  | 27 | 1320.20 | 43.54 | 47.89 | 47.31 | 1041.49 | 44.22 | 48.20 | 47.55 |  |
|  | 32 | 628.82 | 40.69 | 46.78 | 46.16 | 493.75 | 41.60 | 46.91 | 46.16 |  |
|  | 37 | 336.75 | 37.98 | 45.63 | 44.99 | 265.23 | 39.02 | 45.80 | 44.92 |  |
| BGMoving2Static_OnePeople | 22 | 3352.56 | 46.62 | 49.47 | 49.00 | 2693.75 | 47.08 | 49.74 | 49.22 | -35.65% |
|  | 27 | 1319.47 | 43.63 | 47.98 | 47.42 | 1039.39 | 44.33 | 48.29 | 47.63 |  |
|  | 32 | 638.76 | 40.77 | 46.82 | 46.27 | 501.89 | 41.65 | 46.94 | 46.18 |  |
|  | 37 | 345.73 | 38.11 | 45.71 | 45.13 | 271.81 | 39.08 | 45.73 | 44.96 |  |
| BGXG_OnePeopleMoving | 22 | 4402.48 | 46.23 | 48.96 | 48.69 | 3577.31 | 46.71 | 49.19 | 48.91 | -34.08% |
|  | 27 | 1762.51 | 42.98 | 47.39 | 47.01 | 1412.36 | 43.71 | 47.60 | 47.21 |  |
|  | 32 | 854.37 | 39.95 | 46.22 | 45.79 | 680.47 | 40.83 | 46.13 | 45.68 |  |
|  | 37 | 453.48 | 37.16 | 45.02 | 44.60 | 362.35 | 38.10 | 44.88 | 44.38 |  |

Copyright information

The proposed sequences are produced by Alibaba Group and all intellectual property rights remain with Alibaba Group.

The following uses are allowed for the contributed sequences:

- Sequences may be published in technical papers, played at technology research and development events.

- Sequences may be used by Standards committees. (e.g., ITU, MPEG, VQEG).

The following uses are NOT allowed for the contributed sequences:

- Do not publish snapshots in product brochures.

- Do not use video for marketing purposes.

- Do not redistribute video with a commercial product.

- Do not use in television shows, commercials, or movies.

References

[1] M. Wien and V. Baroncini, “AHG4: Agenda and report of the AHG meeting on the SDR HD Verification Tests on 2020-09-03,” JVET-T0045, Sep. 2020.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).