| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0061 |
| --- | --- |

| Title: | Multi-threaded VTM decoder description and performance analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Srinivas Gudumasu Tangi Poirier Fabrice Urban Franck Hiron Remi Jullian Philippe de Lagrange | Tel: Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution describes a VTM-based VVC decoder implementation using parallel processing. The design takes advantage of both task and data parallelization to distribute the decoding modules on a general-purpose multi-core processor. The reported average decoding time ratios compared to the VTM10.0 decoder are 12% and 14% for Class A and Class B sequences respectively, in Random Access test conditions using 16 decoding threads.

# Introduction

This contribution is an update of contribution JVET-R0390 and describes a software VVC decoder using parallel processing on general-purpose multicore processor. The parallel processing features have been added to the native VTM decoder while taking care of limiting the changes in the VTM code. The main update compared to the version presented in JVET-R0390 relates to improving the parallelism implementation. This parallelism is independent from the tiles and slices features.

The software can be made available for review and insertion in a branch of the VTM.

Figure 1 illustrates a general block diagram of VTM10.0 decoder. VVC decoding process can be divided into the following tasks.

- CABAC decoding

- Motion Vector Component Derivation

- Inter CU & CIIP inter reconstruction

- Intra CU & CIIP intra reconstruction

- Inverse reshaping

- In-loop filtering

  - De-blocking Filter

  - Sample Adaptive Offset

  - Adaptive Loop Filter

Figure 1 VVC Decoding architecture

The following 2 parallelization approaches are applied to VVC decoder.

- Load balanced task parallelization

- CTU based data parallelization.

Approach I (load balancing) applies to the parsing stage: the decoder pipeline consists of CABAC decoding and slice decoding tasks. CABAC decoding and slice decoding are running simultaneously on different slices or pictures due to data dependency. In case of low QP or more complicated content, single thread CABAC decoding may be lagging behind the slice decoding. Multiple threads may be allocated to CABAC decoding to balance the workload between CABAC decoding and slice decoding and minimize the effect of CABAC decoding latency.

Approach II is applied on slice decoding tasks, including Inter and CIIP inter reconstruction, Intra and CIIP intra reconstruction, motion vector derivation, re-shaping, de-blocking filter, Sample Adaptive Offset (SAO) and Adaptive Loop Filtering (ALF). Approach II provides fine granularity parallelism on top of existing tile-based parallelization.

# Proposed Method

## Overall Parallelization Architecture

For current VVC decoding pipeline design, threads are added for CABAC decoding and the remaining threads are allocated for slice decoding. The CABAC decoded output is stored in a first in first out (FIFO) queue. These threads continue CABAC decoding of the sub-sequent NAL units. Slice decoding executes in parallel on the CABAC decoded output as shown in Figure 2.

Figure 2 Overview parallelization architecture

## Parallelization of slice decoding tasks

### Thread scheduling

In this section, we illustrate the thread scheduling mechanism in Figure 3 and Figure 4. A thread pool and a FIFO activity queue are created for processing the slice decoding tasks activities. A thread pool is created based on the number of available CPU cores in the multi-core system. Each slice decoding task shown in Figure 2 is further divided into multiple sub-tasks. Each sub-task may consist of a CTU or a group of CTUs or one row of CTUs. A scheduling thread pushes the sub-tasks into activity queue. Any free thread from the thread pool fetches the first available sub-task from the FIFO activity queue and executes the task. As a result, the sub-tasks of a decoding task can be executed in parallel with specific synchronization. Sub-tasks of the next decoding task can be kicked off before all the sub-tasks of the previous decoding task are completed.

Figure 3 Thread scheduling mechanism for slice/picture reconstruction tasks

Figure 4 Thread scheduling mechanism

### Wavefront parallelism

A traditional WPP parallelization scheme is applied both for motion vector component derivation and intra CU prediction, as in Figure 5: each thread handles one CTU row with the data dependency on its left, top and/or top right neighbor CU. In this approach, each CTU row is treated as a sub-task, and each sub-task is processed by a free thread. Multiple CTU rows are executed in parallel with CTU-wise synchronization.

Figure 5 Thread scheduling mechanism for MVCD task

### Inter CU reconstruction

After motion vector component derivation, inter CU reconstruction along with the CIIP CU inter reconstruction is processed. The inter CU reconstruction process is divided into multiple sub-tasks. Each inter reconstruction sub-task may consist of one or a group of CTUs.

The scheduling thread pushes the inter CU reconstruction process of a CTU or a group of CTUs to the activity queue in increasing CTU number order. An available free thread fetches the first available CTU or CTU group from the activity queue and performs the inter CU reconstruction process. Multiple threads can perform the inter CU reconstruction process of the CTU groups simultaneously as shown in Figure 6.

Inter CUs residual reconstruction for chroma samples is separated from inter CU reconstruction process as chroma residual calculation is based on the nearest VPDU reconstructed luma samples when LMCS flag is enabled. The nearest VPDU reconstructed luma samples may not be available as intra CUs reconstruction process is not yet completed for those VPDU samples. So, inter CUs residual reconstruction for chroma samples is postponed and executed with other intra CUs reconstruction process which occurs at a later stage.

The inter prediction of a CIIP mode CU can be combined with other inter CU prediction process. CIIP intra CU reconstruction process is combined with other intra CUs reconstruction process.

Figure 6 Thread scheduling mechanism for Inter CU reconstruction task

### Inverse Re-shaper

After the CU reconstruction task for an entire CTU row, inverse reshaping for this entire CTU row is executed. Each CTU row is treated as a sub-task, and each sub-task is processed by a free thread as in Figure 7.

Note: In this approach, inverse reshaping is executed in parallel of CU reconstruction. Synchronization is done by CTU row.

Figure 7 Thread scheduling mechanism for Inverse Re-shaping task

### De-blocking filter

The de-blocking module in the VVC loop-filtering task is applied immediately after the inverse re-shaper. In VVC, the vertical de-blocking filter is applied on the horizontal de-blocking filter output. Since there is no data dependency between CTUs for either the horizontal or vertical filtering tasks, all CTUs for the whole frame can be processed in parallel. The first available thread fetches the next available CTU or CTU group (e.g. 4 CTUs) from the activity queue for horizontal de-blocking filtering. Vertical de-blocking filtering checks the completion of the horizontal filtering for the current and CTU below as shown in Figure 8.

Figure 8 Thread Scheduling mechanism for De-blocking filter task

### Sample Adaptive Offset & Adaptive Loop-Filter

All CTUs of the entire frame are processed in parallel for ALF and SAO. ALF process is divided into multiple sub-task with each task containing one or a group of CTUs. The scheduling thread pushes one or a group of CTUs sub-task to the activity queue. Free thread fetches first available sub-task and executes ALF process on those CTU(s). The same thread scheduling mechanism is applied for SAO processing.

## Video Player

A VVC video player application is developed using OpenGL framework. The player application fetches the decoded output from the decoded picture buffer (DPB) and renders to output device in real-time.  OpenGL based pixel shader is used to convert the 10 bit 4:2:0 YUV format video frame to RGB32 format in real-time for display purpose.

# Experimental Results

The following experiments were conducted using the VTM-10.0 reference software and the JVET common test conditions [3]. An Intel Dual Xeon E5-2690 processor with 24 physical cores running at 2.6 GHz is used to generate experimental results for both the anchor i.e. VTM-10.0 and proposed parallelization.

The test results of the proposed implementation are compared against the VTM-10.0 anchor. Section 3.1 provides experimental results combining the methods proposed in Section 2. Section 3.2 provides results depending on the number of threads used.

The decoding time ratio is calculated as the percentage from the proposed method(s) over VTM-10.0.

| Decoding Time %=TProposedTVTM10.0×100 (1) |  |
| --- | --- |

where, TVTM10.0 is the time taken by VTM-10.0 and  TProposed is the time from the proposed method.

## Performance improvement combining all decoding tasks

The percentage decoding time ratio using 16 decoding threads calculated based on (1) by combining approaches I and II discussed in Section 2 is reported in Table 1, Table 2, Figure 9 and Figure 10. Six Class A and five Class B sequences from CTC sequences are used for our experiments. Bitstreams are generated from Class A: 3840x2160 10 bit 4:2:0 YUV and Class B: 1920x1080 8 bit 4:2:0 YUV format sequences at QPs: 22, 27, 32, 37.

Table 1. FPS improvement due to CABAC and slice decoding parallelization for Class A sequences

| Sequence | QP | Freq (Hz) | FPS |  | Dec Time ratio (%) |
| --- | --- | --- | --- | --- | --- |
| Class A |  |  | VTM 10 | Proposed | Proposed/VTM-10.0 |
| A1_Tango2 | 22 | 60 | 2.0 | 18.0 | 11% |
|  | 27 | 60 | 2.7 | 23.3 | 11% |
|  | 32 | 60 | 2.9 | 25.1 | 11% |
|  | 37 | 60 | 3.3 | 27.4 | 12% |
| FoodMarket4 | 22 | 60 | 2.3 | 19.8 | 11% |
|  | 27 | 60 | 2.5 | 23.0 | 11% |
|  | 32 | 60 | 2.9 | 24.2 | 12% |
|  | 37 | 60 | 3.1 | 26.7 | 11% |
| CampfireParty2 | 22 | 30 | 1.4 | 10.5 | 13% |
|  | 27 | 30 | 2.2 | 16.3 | 13% |
|  | 32 | 30 | 2.7 | 20.4 | 13% |
|  | 37 | 30 | 3.1 | 23.8 | 12% |
| CatRobot1 | 22 | 60 | 2.0 | 16.2 | 11% |
|  | 27 | 60 | 2.6 | 21.5 | 12% |
|  | 32 | 60 | 3.0 | 22.8 | 12% |
|  | 37 | 60 | 3.4 | 25.7 | 13% |
| DaylightRoad2 | 22 | 60 | 1.6 | 12.4 | 12% |
|  | 27 | 60 | 2.4 | 19.5 | 11% |
|  | 32 | 60 | 2.8 | 24.0 | 11% |
|  | 37 | 60 | 3.3 | 25.9 | 12% |
| ParkRunning3 | 22 | 50 | 1.2 | 12.0 | 10% |
|  | 27 | 50 | 1.5 | 14.3 | 10% |
|  | 32 | 50 | 1.9 | 16.6 | 11% |
|  | 37 | 50 | 2.3 | 20.1 | 11% |

Figure 9 Performance improvement combining all decoding tasks for Class A sequences

Table 2. FPS improvement due to CABAC and slice decoding parallelization for Class B sequences

| Sequence | QP | Freq (Hz) | FPS |  | Dec Time ratio (%) |
| --- | --- | --- | --- | --- | --- |
| Class B |  |  | VTM-10.0 | Proposed | Proposed/VTM-10.0 |
| MarketPlace | 22 | 60 | 7.3 | 60.0 | 12% |
|  | 27 | 60 | 9.4 | 73.4 | 12% |
|  | 32 | 60 | 11.4 | 86.2 | 13% |
|  | 37 | 60 | 12.8 | 99.1 | 13% |
| RitualDance | 22 | 60 | 7.5 | 60.7 | 12% |
|  | 27 | 60 | 9.6 | 71.3 | 13% |
|  | 32 | 60 | 11.3 | 80.3 | 14% |
|  | 37 | 60 | 12.9 | 94.7 | 13% |
| Cactus | 22 | 50 | 7.0 | 47.3 | 14% |
|  | 27 | 50 | 10.4 | 68.9 | 14% |
|  | 32 | 50 | 12.1 | 84.8 | 13% |
|  | 37 | 50 | 13.8 | 96.9 | 14% |
| BasketballDrive | 22 | 50 | 6.6 | 44.3 | 14% |
|  | 27 | 50 | 8.6 | 65.8 | 13% |
|  | 32 | 50 | 10.5 | 79.6 | 13% |
|  | 37 | 50 | 12.4 | 87.5 | 14% |
| BQTerrace | 22 | 60 | 4.9 | 29.1 | 16% |
|  | 27 | 60 | 10.1 | 66.6 | 14% |
|  | 32 | 60 | 12.9 | 89.9 | 14% |
|  | 37 | 60 | 15.1 | 108.2 | 13% |

Figure 10 Performance improvement combining all decoding tasks for Class B sequences

## Performance improvement w.r.t. number of threads

The decoding time reduction depends on the total number of threads. An Intel Dual Xeon E5-2690 processor with 24 physical cores (hyper-threading disabled) was used for our experimentation. Table 3 shows the decoded frames per second (FPS) vs. number of threads details for six Class A and five Class B sequences at QPs: 22, 27, 32, 37. Figure 11 provides the decoding speed in FPS with respect to number of threads for Class A sequences. Figure 12 provides the decoding speed in FPS with respect to number of threads for Class B sequences.

Table 3. Performance improvement w.r.t. number of threads

| Sequences | QP | FPS w.r.t. number of threads |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | VTM-10.0 | 4 | 8 | 12 | 16 |
| CatRobot1 | 22 | 2.0 | 5.7 | 10.7 | 14.5 | 16.2 |
|  | 27 | 2.6 | 7.6 | 14.2 | 18.3 | 21.5 |
|  | 32 | 3.0 | 8.7 | 16.1 | 21.3 | 22.8 |
|  | 37 | 3.4 | 9.4 | 17.3 | 23.2 | 25.7 |
| DaylightRoad2 | 22 | 1.6 | 4.9 | 9.3 | 11.3 | 12.4 |
|  | 27 | 2.4 | 6.8 | 13.2 | 17.5 | 19.5 |
|  | 32 | 2.8 | 7.9 | 15.2 | 20.5 | 24 |
|  | 37 | 3.3 | 9.1 | 17.2 | 23 | 25.9 |
| CampFireParty | 22 | 1.4 | 4 | 6.2 | 8 | 10.5 |
|  | 27 | 2.2 | 6.8 | 11.8 | 14.2 | 16.3 |
|  | 32 | 2.7 | 8.8 | 14.1 | 17.8 | 20.4 |
|  | 37 | 3.1 | 10 | 16.8 | 20.7 | 23.8 |
| Tango2 | 22 | 2.0 | 6.2 | 11.6 | 15 | 18 |
|  | 27 | 2.7 | 7.8 | 14.4 | 19.3 | 23.3 |
|  | 32 | 2.9 | 8.2 | 16.1 | 21 | 25.1 |
|  | 37 | 3.3 | 9.4 | 17.6 | 23.4 | 27.4 |
| FoodMarket4 | 22 | 2.3 | 6.5 | 12.7 | 17 | 19.8 |
|  | 27 | 2.5 | 7.6 | 14 | 19.5 | 23 |
|  | 32 | 2.9 | 8 | 15.5 | 20.9 | 24.2 |
|  | 37 | 3.1 | 9.1 | 17.1 | 23 | 26.7 |
| ParkRunning3 | 22 | 1.2 | 3.5 | 7.5 | 9.7 | 12 |
|  | 27 | 1.5 | 4.6 | 9 | 12 | 14.3 |
|  | 32 | 1.9 | 5.5 | 10.6 | 14.2 | 16.6 |
|  | 37 | 2.3 | 6.8 | 13.1 | 17.5 | 20.1 |
| BQTerrace | 22 | 4.71 | 10.3 | 20.2 | 21.8 | 29.1 |
|  | 27 | 10.63 | 30.1 | 53.2 | 64.2 | 66.6 |
|  | 32 | 13.78 | 37.7 | 65.9 | 84.4 | 89.9 |
|  | 37 | 15.58 | 41.2 | 76.1 | 97.1 | 108.2 |
| RitualDance | 22 | 7.64 | 24.4 | 42.2 | 52.2 | 60.7 |
|  | 27 | 9.58 | 29.1 | 50.3 | 64.5 | 71.3 |
|  | 32 | 11.38 | 34 | 59.5 | 76.3 | 80.3 |
|  | 37 | 13.16 | 37.2 | 67 | 84.6 | 94.7 |
| MarketPlace | 22 | 7.25 | 21.2 | 39.2 | 51.4 | 60 |
|  | 27 | 9.45 | 28 | 50.4 | 66.4 | 73.4 |
|  | 32 | 11.51 | 31.9 | 58.8 | 79.2 | 86.2 |
|  | 37 | 13.39 | 36 | 67.9 | 89.1 | 99.1 |
| BasketBallDrive | 22 | 6.51 | 18.8 | 35.6 | 41.3 | 44.3 |
|  | 27 | 8.95 | 27 | 47 | 60 | 65.8 |
|  | 32 | 10.73 | 31.6 | 55.6 | 72.7 | 79.6 |
|  | 37 | 12.28 | 35 | 63.2 | 80.1 | 87.5 |
| Cactus | 22 | 6.85 | 20.1 | 36.4 | 42.4 | 47.3 |
|  | 27 | 10.21 | 30.4 | 53.5 | 64 | 68.9 |
|  | 32 | 12.24 | 35.6 | 62.1 | 77.6 | 84.8 |
|  | 37 | 13.89 | 39.9 | 72 | 86.2 | 96.9 |

Figure 11 Performance improvement with respect to threads for Class A sequences

Figure 12 Performance improvement with respect to threads for Class B sequences

# Conclusion

In this contribution a modified VTM10.0 VVC decoder has been described. It uses parallel processing on multi-core computing platform. The decoder has been evaluated on JVET Class A and Class B test sequences under RA configuration, and achieves decoding speedup (vs VTM10) from 7.7x to 10x for six Class A sequences and from 6.3x to 8.3x for five Class B sequences.

The software can be made available for review and insertion in a branch of the VTM.

# Reference

F. Bossen, AHG16: Performance of a reasonably fast VVC software decoder, JVET-S0224, Teleconference, July 2020.

Fraunhofer Versatile Video Decoder (VVdeC), https://github.com/fraunhoferhhi/vvdec

F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, CN, October 2018.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).