| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-0063 |
| --- | --- |

| Title: | Suggestion to upgrading random access configuration in CTC with GOP 32 configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Jack Enhorn, Rickard Sjöberg, Jacob Ström, Lukasz Litwic, Per Wennersten | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

At the JVET teleconference in June a GOP 32 configuration was included into VTM. During the interim period between that meeting and this meeting the GOP 32 configuration was tested for 4K SDR and was selected to be used as part of verification testing for 4K SDR. In this contribution we suggest upgrading the common conditions to also use a GOP hierarchy of 32 to have VTM to perform as well as possible.

The BD rate effect for VTM-10.0:

CTC SDR RA -3.0%, -6.3%, -6.4% (luma,Cb/Cr)

CTC HDR RA -7.9%, -5.2, -4.4%, -4.5% (deltaE, psnrL, wpsnrY, psnrY)

It is also asserted that the visual quality is improved.

# Proposal

Make the configuration file encoder_randomaccess_vtm_gop32.cfg (currently present among the configuration files in VTM) to be used by default for the CTC for VVC.

# Results

## Subjective performance

The proposed GOP hierarchy of 32 pictures makes predictions more stable and also give less intra pulsing.

## Objective performance

The objective performance (BDR) for the proposal is shown below in comparison to VTM-10.0 [3] using SDR CTC [4] and HDR CTC [5] with the modification of setting the intra period to be a multiple of 32. The objective results show a nice gain in coding efficiency. Encoding time is measured on simulations run on a cluster with machines of similar capability. Decoding time is measured with anchor and test on same machine.

Table 1: BDR for a GOP hierarchy of 32 for SDR CTC VTM-10.0 random access configuration

|  | Over VTM-10.0 (Intra period 64 for 50Hz and 32 for 20Hz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,94% | -4,82% | -5,04% | 101% | 100% |
| Class A2 | -3,10% | -6,84% | -6,07% | 98% | 101% |
| Class B | -3,31% | -7,23% | -7,59% | 100% | 99% |
| Class C | -3,37% | -5,95% | -6,14% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -3,01% | -6,33% | -6,39% | 100% | 100% |
| Class D | -3,27% | -5,94% | -6,22% | 99% | 102% |
| Class F | -6,46% | -7,39% | -7,25% | 94% | 98% |

Table 2: BDR for a GOP hierarchy of 32 for HDR CTC VTM-10.0 random access configuration

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-10.0 (50Hz with intra period 64) |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -7,85% | -5,19% | -4,41% | -8,73% | -8,92% | -4,24% | -8,05% | -8,38% | 99% | 100% |
| Class H2 |  |  |  |  |  | -4,83% | -7,02% | -7,33% | 106% | 100% |
| Overall | -7,85% | -5,19% | -4,41% | -8,73% | -8,92% | -4,46% | -7,67% | -8,00% | 101% | 100% |

Table 3: BDR for a GOP hierarchy of 32 for some additional HDR sequences VTM-10.0 random access configuration (QP 30, 34, 38, 42)

| Sequence | HDR | DE100 | PSNR-L100 | wPSNR Y | wPSNR U | wPSNR V | wPSNR Y | PSNR U | PSNR V |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ChimeraHDR3 | PQ | -6,34% | -5,53% | -5,24% | -13,26% | -10,88% | -4,94% | -12,94% | -10,41% |
| ChimeraHDR5 | PQ | -6,85% | -5,07% | -4,46% | -12,84% | -6,69% | -4,64% | -13,06% | -6,35% |
| ChimeraHDR7 | PQ | -6,32% | -3,76% | -3,79% | -9,29% | -6,41% | -3,97% | -8,82% | -5,86% |
| MeridianHDR2 | PQ | -5,75% | -5,41% | -5,31% | -8,91% | -7,18% | -5,31% | -8,57% | -7,40% |
| MeridianHDR5 | PQ | -5,48% | -4,01% | -3,94% | -9,56% | -6,76% | -4,21% | -9,26% | -6,89% |
| H3_AMS01 | HLG |  |  |  |  |  | -5,02% | -6,99% | -7,66% |
| H3_AMS03 | HLG |  |  |  |  |  | -4,67% | -4,58% | -4,72% |
| H3_AMS09 | HLG |  |  |  |  |  | -3,78% | -6,98% | -7,12% |
| RiverByBoat | HLG |  |  |  |  |  | -3,69% | -8,26% | -3,74% |
| WaterFallWide | HLG |  |  |  |  |  | -3,15% | -3,60% | 1,09% |

# Conclusion

This contribution shows that there is a clear objective benefit to use GOP 32 instead of GOP 16. It is also asserted that the visual quality is improved by increasing the GOP. To make VVC to perform at its best for random access it is therefore suggested to enable the GOP 32 configuration in CTC.

# References

[1] K. Andersson,  P. Wennersten, R. Sjöberg, J. Samuelsson, J. Ström, P. Hermansson, M. Pettersson, ”Non-normative JEM encoder improvements”, JVET-B0039, San Diego, USA, February 2016.

[2] K. Andersson,  J. Enhorn, R. Sjöberg, J. Ström, L. Litwic, “Addition of a GOP hierarchy of 32 for random access configuration for VTM”, JVET-S0180, Teleconference, June 2020.

[3] VTM-10.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-10.0

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, JVET-N1010, Geneva, Switzerland, March 2019.

[5] A. Segall, E. François, W. Husak, S. Iwamura and D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011, Geneva, Switzerland, October 2019.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).