| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0064 |
| --- | --- |

| Title: | Addition of ALF filter strength control to VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Jacob Ström | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution asserts that currently, there is no possibility to control the ALF filter strength in VTM, it is only possible to turn it on or off completely. This proposal suggests including a configurable ALF filter strength parameter to VTM to enable control of the ALF filter strength. The ALF filter strength parameter scales the magnitudes of the filter coefficients according to the parameter such that a value less than 1 decreases the magnitudes of the filter coefficients and a value larger than 1 increases the magnitudes of the filter coefficients. If the strength parameter is equal to 1 the current behavior is unchanged.

The BDR performance on CTC with the ALF filter strength parameter equal to 0.75, e.g. reducing the filter strength by 25%, is as follows:

AI:    0,27%, 0,25%, 0,31% (Y,U,V)

RA:   0,33%, 0,10%	, 0,16% (Y,U,V)

LDB: 0,30%, 0,11%, 0,23% (Y,U,V)

Further decreasing the filter strength to 50% by using the ALF filter strength parameter equal to 0.5 has the following impact on BDR:

RA:   0,90%, 0,74%, 0,89% (Y,U,V)

LDB: 0,67%, 0,90%, 1,50% (Y,U,V)

It is claimed that a reduction of the ALF filter strength in some cases can improve the subjective quality of VTM for inter predictive coding.

# Introduction

ALF is a great tool to remove coding noise and contributes to a significant part of the objective and subjective coding efficiency of VTM. To add further flexibility for ALF in VTM it would be great to enable control of the amount of filtering of ALF.

# Proposal

The filter strength by ALF is to a large degree determined by the magnitude of the ALF filter coefficients. To reduce the filter strength, the magnitude of the filter coefficients can be made smaller.

We therefore propose to include a configuration parameter ALFStrength into VTM that scales the magnitudes of the ALF filter coefficients when it is not set to 1.

To make sure that all filter coefficients gets a smaller magnitude, the use of fixed filter coefficients is omitted when ALFStrength is not set to 1. The reason is that the fixed filter coefficients cannot be altered and will therefore always be at full strength. Furthermore, the refinement of filter coefficients after quantization is also omitted when ALFStrength not is set to 1 to assure that the reduced magnitudes are kept. Otherwise the refinement process may reinflate the magnitudes, resulting in a full-strength filter.

# Results

## Subjective example

Its hard to visualize video quality with a frame, but on sequences where ALF is used a lot like on BasketballDrill, a reduction of the filter strength can reduce the amount of blurring, as seen in Figure 1 and Figure 2 for the low-delay B configuration and in Figure 3 and Figure 4 for the random access configuration.

Figure 1: BasketballDrill LDB at QP 37, POC 599, Left: VTM-10.0, Middle: ALFStrength equal to 0.75, Right: ALFStrength equal to 0.5

Figure 2: BasketballDrill LDB at QP 32, POC 599, Left: VTM-10.0, Middle: ALFStrength equal to 0.75, Right: ALFStrength equal to 0.5

Figure 3: BasketballDrill RA at QP 37, POC 599, Left: VTM-10.0, Middle: ALFStrength equal to 0.75, Right: ALFStrength equal to 0.5

Figure 4: BasketballDrill RA at QP 32, POC 599, Left: VTM-10.0, Middle: ALFStrength equal to 0.75, Right: ALFStrength equal to 0.5

## Objective performance

The objective performance (BDR) for the proposal is shown below in comparison to VTM-10.0 [1] using SDR CTC [2]. The objective results show a slight loss in BDR compared to the reference. Encoding time is measured on simulations run on a cluster with machines with similar capability is not reliable. Decoding time is measured on same machine.

### ALFStrength equal to 0.75

|  | All Intra VTM-10.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,18% | 0,49% | 0,55% | 101% | 102% |
| Class A2 | 0,26% | 0,23% | 0,13% | 110% | 99% |
| Class B | 0,24% | 0,17% | 0,29% | 103% | 101% |
| Class C | 0,26% | 0,18% | 0,30% | 106% | 100% |
| Class E | 0,45% | 0,24% | 0,29% | 106% | 99% |
| Overall | 0,27% | 0,25% | 0,31% | 105% | 100% |
| Class D | 0,21% | 0,30% | 0,41% | 100% | 104% |
| Class F | 0,33% | 0,23% | 0,24% | 102% | 98% |

|  | Random access VTM-10.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,31% | 0,20% | 0,34% | 103% | 100% |
| Class A2 | 0,44% | 0,18% | 0,06% | 106% | 100% |
| Class B | 0,38% | 0,19% | 0,24% | 102% | 100% |
| Class C | 0,19% | -0,12% | -0,02% | 106% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,33% | 0,10% | 0,16% | 104% | 100% |
| Class D | 0,22% | -0,38% | -0,20% | 105% | 102% |
| Class F | 0,22% | 0,09% | 0,18% | 105% | 102% |

|  | Over VTM-10.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,30% | 0,14% | 0,63% | 104% | 100% |
| Class C | 0,20% | -0,06% | -0,16% | 98% | 101% |
| Class E | 0,42% | 0,30% | 0,06% | 103% | 99% |
| Overall | 0,30% | 0,11% | 0,23% | 102% | 100% |
| Class D | 0,21% | 0,23% | -0,41% | 104% | 98% |
| Class F | 0,11% | -0,67% | 0,35% | 103% | 102% |

### ALFStrength equal to 0.5

|  | Random access VTM-10.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,86% | 0,82% | 1,16% | 103% | 101% |
| Class A2 | 1,22% | 0,80% | 1,00% | 104% | 101% |
| Class B | 0,97% | 0,93% | 0,98% | 105% | 102% |
| Class C | 0,62% | 0,38% | 0,48% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0,90% | 0,74% | 0,89% | 103% | 101% |
| Class D | 0,75% | -0,07% | 0,02% | 103% | 102% |
| Class F | 0,61% | 0,59% | 0,84% | 103% | 102% |

|  | Over VTM-10.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,77% | 1,33% | 2,68% | 106% | 101% |
| Class C | 0,46% | 0,15% | 0,23% | 102% | 100% |
| Class E | 0,78% | 1,17% | 1,24% | 104% | 100% |
| Overall | 0,67% | 0,90% | 1,50% | 104% | 100% |
| Class D | 0,45% | 0,59% | -0,38% | 102% | 100% |
| Class F | 0,52% | 0,58% | 0,77% | 106% | 101% |

# Conclusions

It has been showed that the filter strength of ALF can be controlled by scaling the filter coefficients. With an ALF filter strength parameter of 0.75, most of the gain of ALF can be kept but with weaker filtering. Reduction of the filter strength can reduce the amount of blurring while still removing coding artifacts and can also in some cases give better visual quality. To enable a possibility to control the ALF filter strength it is suggested to include the proposed ALF filter strength parameter to the VTM software. The default setting is suggested to be 1, e.g., no change of filter strength.

# References

[1] VTM-10.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-10.0

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, JVET-N1010, Geneva, Switzerland, March 2019.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).