| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0065-v1 |
| --- | --- |

| Title: | Conformance points for multilayer 8K |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd |  |  |

_____________________________

# Abstract

It is asserted that the profile and level definitions provided in the first version of VVC are inadequate to support multilayer 8K applications. For example, no level definition can support a two-layer bitstream, where the highest layer is 8K and both layers use a typical random access coding structure such as defined in common test conditions. Changes to profile and level definitions are proposed to provide support for such use case.

# Problem Statement

A two-layer stream, where the higher layer is 8K and both layers use a typical random access coding structure, requires a large decoded picture buffer (DPB). For example, the configuration used for common test conditions requires a DPB of size 6 (dpb_max_dec_pic_buffering_minus1 is set to 5). With two layers, the required DPB size is 12. No profile and level combination defined in the first version of VVC supports this use case: the highest level (6.2) cannot accommodate a DPB larger than 8 pictures at 8K resolution.

# Proposal

Several proposals are described below to address the issue.

## Modify existing multilayer profiles

Define maxDpfPicBuf to be equal to 16 for the existing multilayer profiles. In section A.4.2, replace

where MaxLumaPs is specified in Table 135, maxDpbPicBuf is equal to 8, and dpb_max_dec_pic_buffering_minus1[ Htid ] is found in or derived from the applicable dpb_parameters( ) syntax structure.

with

where MaxLumaPs is specified in Table 135, maxDpbPicBuf is equal to 16 if the bitstream is indicated to conform to the Multilayer Main 10 or Multilayer Main 10 4:4:4 profile and equal to 8 otherwise, and dpb_max_dec_pic_buffering_minus1[ Htid ] is found in or derived from the applicable dpb_parameters( ) syntax structure.

Alternatively, maxDpbPicBuf can be defined to be equal to 32 for the multilayer profiles, such as to support more than two layers at the maximum resolution defined by each level.

The increase of maxDpbPicBuf can result in larger values of maxDpbSize, meaning that a decoder would have to manage a larger number of pictures in the decoded picture buffer. If so desired, this can be mitigated by introducing a cap on the value of maxDpbSize. For example, the derivation of maxDpbSize

if( 2 * PicSizeMaxInSamplesY  <=  MaxLumaPs )
	MaxDpbSize = 2 * maxDpbPicBuf
else if( 3 * PicSizeMaxInSamplesY  <=  2 * MaxLumaPs )
	MaxDpbSize = 3 * maxDpbPicBuf / 2		(1576)
else
	MaxDpbSize = maxDpbPicBuf

can be modified to:

if( 2 * PicSizeMaxInSamplesY  <=  MaxLumaPs )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, maxDpbSizeLimit )
else if( 3 * PicSizeMaxInSamplesY  <=  2 * MaxLumaPs )
	MaxDpbSize = Min ( 3 * maxDpbPicBuf / 2, maxDpbSizeLimit )	(1576)
else
	MaxDpbSize = maxDpbPicBuf

where maxDpbSizeLimit can be set equal to 16 (or another value that is larger than or equal to maxDpbPicBuf).

## Add a level 7

Add a level 7 that supports 16K resolution at 30 fps. Given the pixel per second rate, such a level would also support 8K resolution at 120fps. The value of MaxDpbSize would be 16 for 8K content.

In table 135, add the highlighted row:

| Level | general_level_idc value* | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | High tier |  |  |  |
| 1.0 | 16 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2.0 | 32 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 35 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3.0 | 48 | 552 960 | 6 000 | - | 30 | 4 | 2 |
| 3.1 | 51 | 983 040 | 10 000 | - | 40 | 9 | 3 |
| 4.0 | 64 | 2 228 224 | 12 000 | 30 000 | 75 | 25 | 5 |
| 4.1 | 67 | 2 228 224 | 20 000 | 50 000 | 75 | 25 | 5 |
| 5.0 | 80 | 8 912 896 | 25 000 | 100 000 | 200 | 110 | 10 |
| 5.1 | 83 | 8 912 896 | 40 000 | 160 000 | 200 | 110 | 10 |
| 5.2 | 86 | 8 912 896 | 60 000 | 240 000 | 200 | 110 | 10 |
| 6.0 | 96 | 35 651 584 | 80 000 | 240 000 | 600 | 440 | 20 |
| 6.1 | 99 | 35 651 584 | 120 000 | 480 000 | 600 | 440 | 20 |
| 6.2 | 102 | 35 651 584 | 180 000 | 800 000 | 600 | 440 | 20 |
| 7.0 | 112 | 142 606 336 | 240 000 | 800 000 | 1800 | 1760 | 40 |
| * The level numbers in this table are in the form of "majorNum.minorNum", and the value of general_level_idc for each of the levels is equal to majorNum * 16 + minorNum * 3. |  |  |  |  |  |  |  |

In table 136, add the highlighted row:

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1.0 | 552 960 | 128 | - | 2 | 2 |
| 2.0 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3.0 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4.0 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5.0 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.0 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |
| 7.0 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |

## Add a level 6.3

Add level 6.3 that supports 12K resolution (e.g., 11520 x 6480) at 60 fps. Given the pixel per second rate, such a level would also support 8K resolution at 120fps. The value of MaxDpbSize would be 16 for 8K content.

In table 135, add the highlighted row:

| Level | general_level_idc value* | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per AU MaxSlicesPerAu | Max # of tiles MaxTilesPerAu | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main tier | High tier |  |  |  |
| 1.0 | 16 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2.0 | 32 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 35 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3.0 | 48 | 552 960 | 6 000 | - | 30 | 4 | 2 |
| 3.1 | 51 | 983 040 | 10 000 | - | 40 | 9 | 3 |
| 4.0 | 64 | 2 228 224 | 12 000 | 30 000 | 75 | 25 | 5 |
| 4.1 | 67 | 2 228 224 | 20 000 | 50 000 | 75 | 25 | 5 |
| 5.0 | 80 | 8 912 896 | 25 000 | 100 000 | 200 | 110 | 10 |
| 5.1 | 83 | 8 912 896 | 40 000 | 160 000 | 200 | 110 | 10 |
| 5.2 | 86 | 8 912 896 | 60 000 | 240 000 | 200 | 110 | 10 |
| 6.0 | 96 | 35 651 584 | 80 000 | 240 000 | 600 | 440 | 20 |
| 6.1 | 99 | 35 651 584 | 120 000 | 480 000 | 600 | 440 | 20 |
| 6.2 | 102 | 35 651 584 | 180 000 | 800 000 | 600 | 440 | 20 |
| 6.3 | 105 | 80 216 064 | 240 000 | 800 000 | 1000 | 990 | 30 |
| * The level numbers in this table are in the form of "majorNum.minorNum", and the value of general_level_idc for each of the levels is equal to majorNum * 16 + minorNum * 3. |  |  |  |  |  |  |  |

In table 136, add the highlighted row:

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1.0 | 552 960 | 128 | - | 2 | 2 |
| 2.0 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3.0 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4.0 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5.0 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.0 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 800 000 | 8 | 4 |

## Add multilayer profiles

Add multilayer profiles Multilayer Extended Memory 10 and Multilayer Extended Memory 10 4:4:4, and assign general_profile_idc values 18 and 50 to them. Alternatively values 25 and 57 can be used. In such case the bit of weight 8 in general_profile_idc can be interpreted to signal extended reference memory.

Define maxDpfPicBuf to be equal to 16 for the added multilayer profiles. In section A.4.2, replace

where MaxLumaPs is specified in Table 135, maxDpbPicBuf is equal to 8, and dpb_max_dec_pic_buffering_minus1[ Htid ] is found in or derived from the applicable dpb_parameters( ) syntax structure.

with

where MaxLumaPs is specified in Table 135, maxDpbPicBuf is equal to 16 if the bitstream is indicated to conform to the Multilayer Extended Memory 10 or Multilayer Extended Memory 10 4:4:4 profile and equal to 8 otherwise, and dpb_max_dec_pic_buffering_minus1[ Htid ] is found in or derived from the applicable dpb_parameters( ) syntax structure.

Alternatively, maxDpbPicBuf could be defined to be equal to 32 for the multilayer profiles, such as to support more than two layers at the maximum resolution defined by each level.

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).