| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 20th Meeting, by teleconference, 7 – 16 Oct. 2020 | Document: JVET-T0066 |
| --- | --- |

| Title: | AHG9: IRAP only HRD SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin, Yago Sanchez, Karsten Suehring, Thomas Schierl Fraunhofer HHI Ye-Kui Wang, Li Zhang Bytedance Inc. 8910 University Center Lane San Diego, CA 92122, USA Sachin Deshpande Sharp Labs of America, Inc | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de yekui.wang@bytedance.com lizhang.idm@bytedance.com sdeshpande@sharplabs.com |
| Source: | Fraunhofer HHI, Bytedance Inc., Sharp Labs of America, Inc |  |  |

_____________________________

# Abstract

This document proposes to add a new IRAP only HRD (IOH) SEI message to VVC that carries HRD information for an IRAP-only sub-bitstream that can be used in decoder trick play operation. The proposed IOH SEI message indicates irap_only_level_idc, a maximum speedup as well as bitrate and CPB size of the IRAP-only sub-bitstream. Related changes to the scalable nesting SEI message in Annex D and extraction process and HRD timing derivation in Annex C are proposed.

Revision 1 of this document adds initial CPB removal delay and offset signalling to the proposed IOH SEI message.

# Introduction

At the JVET-R meeting, JVET-R0068 (item 2) proposed to indicate level information for an IRAP-only sub-bitstream to be carried in the PTL structure in parameter sets that hosts level information of all sub-bitstreams that consist of layer or sub-layer subset. The proposal asserted that such information would be useful for applications that perform trick-play operations based on IRAP AUs only, e.g. fast-forwarding.

During presentation of JVET-R0068, the following review comments were made (JVET-R_Notes_dH). This document discusses additional thoughts in italics as follows:

- It was commented that the encoder could use TemporalId 0 for IRAP pictures. However, it was commented that this will make one less sublayer to be used.

In addition, VVC does not have means to indicate such a sublayer assignment scheme to player applications for them to make use of IRAP only trick-play.

- It was commented that in intra based trick play, some IRAP pictures would be output/displayed multiple times.

It is asserted that frame repetition is typically handled by the player application itself and not by the decoder.

- It was commented that trick play can also use non-intra pictures, and scene cuts may appear in arbitrary positions and can be coded as IRAP pictures.

The proposed additional signaling does not prevent other trick play modes. Trick play with non-intra pictures can still be used, e.g. TemporalId 0 AUs which have a well-defined HRD. Furthermore, arbitrary IRAP picture position due to scene cuts with proper timing do not prohibit trick play as frame rate does not have to be constant in trick-play.

- It was commented that DASH already makes use of this information, as a bitstream property.

Although, there are different flavors of trick plays, it is asserted that the only IRAP mode is commonly used. In fact, the DASH standard covers this mode as a special mode and allows signaling to determine the conformance point of the sub-bitstream result of downloading only the IRAP AUs of a bitstream. Therefore, specifying HRD conformance for such a decoder operation would be desirable from Systems perspective.

- The proposal (JVET-R0068) did not include text of an extraction process to extract this extra subset, or an HRD model for such a subset.

The issue is addressed in this proposal.

- It was commented that this is not necessarily how a decoding system would use intra-only trick play.

As aforementioned, there might be other ways of doing trick play, but there are systems that are based on this basic functionality and care about the conformance point aspect of the sub-bitstream containing only IRAP AUs. What is done beyond this conformance point at the receiver side for more advanced trick modes is be out of the scope of the specification.

Furthermore, there are two aspects for such trick-play sub-bitstreams that are untouched by the proposal in JVET-R0068 as follows.

- Maximum playback rates within level

A decoder cannot derive the HRD timing for such trick-play sub-bitstreams from the level indication itself but requires guidance on how to interpret the HRD timings indicated in the respective BP and PT SEI messages. Specifically, a limit to the maximum speedup that still conforms to the indicated level limits for trick play is required. When tools such as RPR are used, a decoder needs to analyze the complete bitstream before making correct judgements about original and trick-play sample rates.

- Unequal IRAP periods in layered streams

Unequal IRAP periods are common in layered bitstream, e.g. providing fast random access for low quality video in a lower layer while limiting intra coding overhead for high-quality video in a higher layer. When extracting all IRAP NAL units from such a bitstream, the resulting sub-bitstream contains AUs that only have pictures in the non-output layers, where IRAPs were not aligned originally.

The current proposal addresses this by providing a level_idc indication for the sub-bitstream that contains only IRAP AUs, i.e. for single layer OLSs, all NAL units of type IDR_W_RADL and IDR_NLP with the associated non-VCL NAL units and pps_mixed_nalu_types_in_pic_flag equal to 1 in the respective PPSs, or, for multi-layer OLSs, all AUs with ph_gdr_or_irap_pic_flag equal to 1 in the associated AUD.

This document proposes an SEI message that provides information about the HRD conformance of IRAP AUs only in the following. Necessary further additions to Annex C and D of VVC are also proposed. In summary, the following is proposed:

- Adding new IRAP only HRD (IOH) SEI message allowing AU-based HRD operation with an IRAP AUs only sub-bitstream to Annex D.

  - Indicates a level_idc for IRAP AUs only sub-bitstreams of the OLS.

  - Allows varying speedup of up to an indicated maximum speedup.

  - Derivation of Bitrate and CpbSize for IRAP AUs only sub-bitstreams.

- Integration of IOH SEI message in SN SEI message in Annex D.

- Derivation of proper timing for IRAP AUs only sub-bitstreams in Annex C.

- Extraction process of IRAP AUs only sub-bitstreams in Annex C.

# Proposed additions to Annex D

D.X.1 IRAP only HRD information SEI message syntax

| irap_only_hrd_information( payloadSize ) { | Descriptor |
| --- | --- |
| irap_only_level_idc | u(5) |
| irap_only_reserved_zero_3bits | u(3) |
| irap_only_max_speedup_minus100 | u(16) |
| irap_only_general_nal_hrd_params_present_flag | u(1) |
| irap_only_general_vcl_hrd_params_present_flag | u(1) |
| irap_only_cpb_cnt_minus1 | ue(v) |
| for( i = 0; i < irap_only_cpb_cnt_minus1; i++) { |  |
| if( irap_only_general_nal_hrd_params_present_flag ){ |  |
| irap_only_nal_bit_rate_value_minus1[ i ] | ue(v) |
| irap_only_nal_cpb_size_value_minus1[ i ] | ue(v) |
| irap_only_nal_initial_cpb_removal_delay[ i ] | u(v) |
| irap_only_nal_initial_cpb_removal_offset[ i ] | u(v) |
| } |  |
| if( irap_only_general_vcl_hrd_params_present_flag ){ |  |
| irap_only_vcl_bit_rate_value_minus1[ i ] | ue(v) |
| irap_only_vcl_cpb_size_value_minus1[ i ] | ue(v) |
| irap_only_vcl_initial_cpb_removal_delay[ i ] | u(v) |
| irap_only_vcl_initial_cpb_removal_offset[i ] | u(v) |
| } |  |
| } |  |
| } |  |

D.X.2 IRAP only HRD information SEI message semantics

The IRAP only HRD information (IOH) SEI message contains information about the level that the sub-bitstream consisting of only the IRAP AU sequence in the set of CVSs of the OLSs to which the SEI message applies, denoted as targetCvss, conform to when testing the conformance of the extracted bitstreams containing the IRAP AU sequence according to Annex A. The OLSs to which the IOH message applies are also referred to as the applicable OLSs or the associated OLSs. A CVS in the remainder of this subclause refers to a CVS of the applicable OLSs. An IRAP AU sequence consists of all IRAP AUs within targetCvss.

When an IOH SEI message is present (either being in the bitstream or provided through an external means not specified in this Specification) for any AU of a CVS, an IOH SEI message shall be present for the first AU of the CVS. The IOH SEI message persists in decoding order from the current AU until the next AU containing an IOH SEI message for which the content differs from the current IOH SEI message or the end of the bitstream. All IOH SEI messages that apply to the same CVS shall have the same content.

irap_only_level_idc indicates a level, as specified in Annex A, to which the sub-bitstream corresponding to the IRAP AU sequence of the targetCvss conforms. An IOH SEI message shall not contain values of irap_only_level_idc other than those specified in Annex A. Other values of irap_only_level_idc are reserved for future use by ITU-T | ISO/IEC.

irap_only_max_speedup_minus100 plus 100 divided by 100 specifies the maximum value of speedup to be applied to the HRD timings of the sub-bitstream corresponding to the IRAP AU sequence of the targetCvss that still conforms to the indicated irap_only_level_idc. The value of irap_only_max_speedup_minus100 shall be in the range of 0 to 216 − 101, inclusive.

The variable MaxSpeedupVal is derived to be equal to ( irap_only_max_speedup_minus100 + 100 ) ÷ 100.

irap_only_general_nal_hrd_params_present_flag equal to 1 specifies that NAL HRD parameters (pertaining to Type II bitstream conformance point) are present in the IOH SEI message. irap_only_general_nal_hrd_params_present_flag equal to 0 specifies that NAL HRD parameters are not present in the IOH SEI message.

irap_only_general_vcl_hrd_params_present_flag equal to 1 specifies that VCL HRD parameters (pertaining to Type I bitstream conformance point) are present in the IOH SEI message. irap_only_general_vcl_hrd_params_present_flag equal to 0 specifies that VCL HRD parameters are not present in the IOH SEI message.

irap_only_cpb_cnt_minus1 plus 1 specifies the number of alternative CPB delivery schedules. The value of irap_only_cpb_cnt_minus1 shall be in the range of 0 to 31, inclusive.

It is a requirement of bistream conformance that irap_only_general_nal_hrd_params_present_flag, irap_only_general_vcl_hrd_params_present_flag and irap_only_cpb_cnt_minus1 are equal to general_nal_hrd_params_present_flag, general_vcl_hrd_params_present_flag and hrd_cpb_cnt_minus1 respectively.

irap_only_nal_bit_rate_value_minus1[ i ] (together with bit_rate_scale) specifies the input bit rate for the i-th CPB for the sub-bitstream corresponding to the IRAP AU sequence of the targetCvss when the CPB operates at the AU level. irap_only_nal_bit_rate_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. The bit rate in bits per second is given by:

BitRate[ MaxSubLayersMinus1 + 1 ][ i ] = ( irap_only_nal_bit_rate_value_minus1[ i ] + 1 ) * 2( 6 + bit_rate_scale ) * SpeedupFactor

where SpeedupFactor is as specified in subclause C.1 and the value is equal to MaxSpeedupVal.

NOTE – Playback of the IRAP AU sequence of the targetCvss with any value of SpeedupFactor in the range of 1 to MaxSpeedupVal, inclusive, will conform to the level indicated by irap_only_level_idc when timings are derived according to Annex C.2 and C.3.

irap_only_nal_cpb_size_value_minus1[ i ] (together with cpb_size_scale) specifies the CPB size for the i-th CPB for the sub-bitstream corresponding to the IRAP AU sequence of the targetCvss when the CPB operates at the AU level. irap_only_nal_cpb_size_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. The CPB size in bits is given by:

CpbSize[ MaxSubLayersMinus1 + 1 ][ i ] = ( irap_only_nal_cpb_size_value_minus1[ i ] + 1 ) * 2( 4 + cpb_size_scale ).

irap_only_nal_initial_cpb_removal_delay[ i ] specifies the i-th initial CPB removal delay for the NAL HRD in units of a 90 kHz clock of the IRAP AU sequence of the targetCvss when the CPB operates at the AU level. The length of irap_only_nal_initial_cpb_removal_delay[ i ] is bp_cpb_initial_removal_delay_length_minus1 + 1 bits. The value of irap_only_nal_initial_cpb_removal_delay[ i ] shall not be equal to 0 and shall be less than or equal to 90000 * ( CpbSize[ MaxSubLayersMinus1 + 1 ][ i ]  BitRate[ MaxSubLayersMinus1 + 1 ][ i ] ), the time-equivalent of the CPB size in 90 kHz clock units.

irap_only_nal_initial_cpb_removal_offset[ i ] specifies the i-th initial CPB removal offset of the IRAP AU sequence of the targetCvss for the NAL HRD in units of a 90 kHz clock when the CPB operates at the AU level. The length of irap_only_nal_initial_cpb_removal_offset[ i ] is bp_cpb_initial_removal_delay_length_minus1 + 1 bits.

Over the entire CVS, for each value of i,the sum of irap_only_nal_initial_cpb_removal_delay[ i ] and irap_only_nal_initial_cpb_removal_offset[ i ] shall be constant.

irap_only_vcl_bit_rate_value_minus1[ i ] (together with bit_rate_scale) specifies the input bit rate for the i-th CPB for the sub-bitstream corresponding to the IRAP AU sequence of the targetCvss when the CPB operates at the AU level. irap_only_vcl_bit_rate_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. The bit rate in bits per second is given by:

BitRate[ MaxSubLayersMinus1 + 1 ][ i ] = ( irap_only_vcl_bit_rate_value_minus1[ i ] + 1 ) * 2( 6 + bit_rate_scale ) * SpeedupFactor

where SpeedupFactor is as specified in subclause C.1 and the value is equal to MaxSpeedupVal.

irap_only_vcl_cpb_size_value_minus1[ i ] (together with cpb_size_scale) specifies the CPB size for the i-th CPB for the sub-bitstream corresponding to the IRAP AU sequence of the targetCvss when the CPB operates at the AU level. irap_only_vcl_cpb_size_value_minus1[ i ] shall be in the range of 0 to 232 − 2, inclusive. The CPB size in bits is given by:

CpbSize[ MaxSubLayersMinus1 + 1 ][ i ] = ( irap_only_vcl_cpb_size_value_minus1[ i ] + 1 ) * 2( 4 + cpb_size_scale ).

irap_only_vcl_initial_cpb_removal_delay[ i ] specifies the i-th initial CPB removal delay for the VCL HRD in units of a 90 kHz clock of the IRAP AU sequence of the targetCvss when the CPB operates at the AU level. The length of irap_only_vcl_initial_cpb_removal_delay[ i ] is bp_cpb_initial_removal_delay_length_minus1 + 1 bits. The value of irap_only_vcl_initial_cpb_removal_delay[ i ] shall not be equal to 0 and shall be less than or equal to 90000 * ( CpbSize[ MaxSubLayersMinus1 + 1 ][ i ]  BitRate[ MaxSubLayersMinus1 + 1 ][ i ] ), the time-equivalent of the CPB size in 90 kHz clock units.

irap_only_vcl_initial_cpb_removal_offset[ i ] specifies the i-th initial CPB removal offset of the IRAP AU sequence of the targetCvss for the VCL HRD in units of a 90 kHz clock when the CPB operates at the AU level. The length of irap_only_vcl_initial_cpb_removal_offset[ i ] is bp_cpb_initial_removal_delay_length_minus1 + 1 bits.

Over the entire CVS, for each value of i, the sum of irap_only_vcl_initial_cpb_removal_delay[ i ] and irap_only_vcl_initial_cpb_removal_offset[ i ] shall be constant.

# Proposed (highlighted) changes to Annex D:

D.2.2 General SEI payload semantics

[…]

The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 129, 137, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 168, and 204.

The list PicUnitRepConSeiList is set to consist of the payloadType values 0, 1, 19, 45, 129, 133, 137, 147 to 150, inclusive, 153 to 156, inclusive, 168, 203, and 204 and 205.

NOTE 4 – VclAssociatedSeiList consists of the payloadType values of the SEI messages that, when non-scalable-nested, infer constraints on the NAL unit header of the SEI NAL unit on the basis of the NAL unit header of the associated VCL NAL unit. PicUnitRepConSeiList consists of the payloadType values of the SEI messages that are subject to the restriction on 4 repetitions per PU.

It is a requirement of bitstream conformance that the following restrictions apply on containing of SEI messages in SEI NAL units:

–	When general_same_pic_timing_in_all_ols_flag is equal to 1, there shall be no SEI NAL unit that contains a scalable-nested SEI message with payloadType equal to 1 (PT), and when an SEI NAL unit contains a non-scalable-nested SEI message with payloadType equal to 1 (PT), the SEI NAL unit shall not contain any other SEI message with payloadType not equal 1.

–	When an SEI NAL unit contains a non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI) or 205 (IOH), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, or 203 or 205 (IOH).

–	When an SEI NAL unit contains a scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 0, 1, 130, 203, or 133 (scalable nesting) or 205 (IOH).

–	When an SEI NAL unit contains an SEI message with payloadType equal to 3 (filler payload), the SEI NAL unit shall not contain any other SEI message with payloadType not equal to 3.

The following applies on the applicable OLSs or layers of non-scalable-nested SEI messages:

–	For a non-scalable-nested SEI message, when payloadType is equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), or 205 (IOH) the non-scalable-nested SEI message applies to all the OLSs, when present, that consist of all layers in the current CVS in the entire bitstream. When there is no OLS that consists of all layers in the current CVS the entire bitstream, there shall be no non-scalable-nested SEI message with payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI) or 205 (IOH).

–	For a non-scalable-nested SEI message, when payloadType is equal to any value among VclAssociatedSeiList, the non-scalable-nested SEI message applies only to the layer for which the VCL NAL units have nuh_layer_id equal to the nuh_layer_id of the SEI NAL unit containing the SEI message.

It is a requirement of bitstream conformance that the following restrictions apply on the value of nuh_layer_id of SEI NAL units:

–	When a non-scalable-nested SEI message has payloadType equal to any value among VclAssociatedSeiList, the SEI NAL unit containing the non-scalable-nested SEI message shall have nuh_layer_id equal to the value of nuh_layer_id of the VCL NAL unit associated with the SEI NAL unit.

–	An SEI NAL unit containing a scalable nesting SEI message shall have nuh_layer_id equal to the lowest value of nuh_layer_id of all layers to which the scalable-nested SEI messages apply (when sn_ols_flag of the scalable nesting SEI message is equal to 0) or the lowest value of nuh_layer_id of all layers in the OLSs to which the scalable-nested SEI message apply (when sn_ols_flag of the scalable nesting SEI message is equal to 1).

NOTE 5 – Same as for DCI, OPI, VPS, AUD, and EOB NAL units, the value of nuh_layer_id for SEI NAL units that contain non-scalable-nested SEI messages with payloadType equal to 0 (BP), 1 (PT), or 130 (DUI), or 203 (SLI) or 205 (IOH) is not constrained.

D.6.2 Scalable nesting SEI message semantics

The scalable nesting SEI message provides a mechanism to associate SEI messages with specific OLSs, specific layers, or specific sets of subpictures.

A scalable nesting SEI message contains one or more SEI messages. The SEI messages contained in the scalable nesting SEI message are also referred to as the scalable-nested SEI messages.

It is a requirement of bitstream conformance that the following restrictions apply on containing of SEI messages in a scalable nesting SEI message:

–	An SEI message that has payloadType equal to 3 (filler payload) or 133 (scalable nesting) shall not be contained in a scalable nesting SEI message.

–	When a scalable nesting SEI message contains a BP, PT, DUI, or SLI, or 205 (IOH) SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI) or 205 (IOH).

It is a requirement of bitstream conformance that the following restriction applies on the value of the nal_unit_type of the SEI NAL unit containing a scalable nesting SEI message:

–	When a scalable nesting SEI message contains an SEI message that has payloadType not equal to 132 (decoded picture hash), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to PREFIX_SEI_NUT.

–	When a scalable nesting SEI message contains an SEI message that has payloadType equal to 132 (decoded picture hash), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to SUFFIX_SEI_NUT.

sn_ols_flag equal to 1 specifies that the scalable-nested SEI messages apply to specific OLSs. sn_ols_flag equal to 0 specifies that the scalable-nested SEI messages apply to specific layers.

It is a requirement of bitstream conformance that the following restrictions apply on the value of sn_ols_flag:

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI), or 205 (IOH) the value of sn_ols_flag shall be equal to 1.

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to a value in VclAssociatedSeiList, the value of sn_ols_flag shall be equal to 0.

# Proposed (highlighted) changes to Annex C:

C.1 General

[…]

For each test, the following ordered steps apply in the order listed, followed by the processes described after these steps in this subclause:

- An operation point under test, denoted as targetOp, is selected by selecting a target OLS with OLS index opOlsIdx, a highest TemporalId value opTid, OnlyIrapAusFlag and optionally, a list of target subpicture index values opSubpicIdxList[ j ] for j from 0 to NumLayersInOls[ opOlsIdx ] − 1, inclusive and optionally a playback speedup factor SpeedupFactor. The value of opOlsIdx is in the range of 0 to TotalNumOlss − 1, inclusive. The value of opTid is in the range of 0 to vps_max_sublayers_minus1, inclusive. The value of SpeedupFactor is set equal to MaxSpeedupVal. When not present, the value of SpeedupFactor is inferred to be equal to 1.

If opSubpicIdxList[ ] is not present, targetOp consists of pictures, and each pair of the selected values of opOlsIdx and opTid shall be such that the sub-bitstream BitstreamToDecode that is the output by invoking the sub-bitstream extraction process as specified in subclause C.6 with entireBitstream, opOlsIdx, and opTid, and OnlyIrapAusFlag as inputs satisifies the following condition:

–	There is at least one VCL NAL unit with TemporalId equal to opTid in BitstreamToDecode.

Otherwise (opSubpicIdxList[ ] is present), targetOp consists of subpictures, and each set of the selected values of opOlsIdx, opTid, and opSubpicIdxList[ j ] for j from 0 to NumLayersInOls[ opOlsIdx ] − 1, inclusive, shall be such that the sub-bitstream BitstreamToDecode that is the output by invoking the subpicture sub-bitstream extraction process as specified in subclause C.7 with entireBitstream, opOlsIdx, opTid, and opSubpicIdxList[ j ] for j from 0 to NumLayersInOls[ opOlsIdx ] − 1, inclusive, and OnlyIrapAusFlag, as inputs satisifies the following conditions:

–	There is at least one VCL NAL unit with TemporalId equal to opTid in BitstreamToDecode.

–	There is at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ opOlsIdx ][ j ] and sh_subpic_id equal to SubpicIdVal[ opSubpicIdxList[ j ] ] for each j in the range of 0 to NumLayersInOls[ opOlsIdx ] − 1, inclusive.

NOTE 1 – Regardless of whether opSubpicIdxList[ ] is present, due to the bitstream conformance requirement of each IRAP or GDR AU to be complete, there is at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ opOlsIdx ][ j ] for each j from 0 to NumLayersInOls[ opOlsIdx ] − 1, inclusive.

- If opSubpicIdxList[ ] is not present the following applies:

–	If the layers in targetOp include all layers in entireBitstream and opTid is equal to the highest TemporalId value among all NAL units in entireBitstream, BitstreamToDecode is set to be identical to entireBitstream.

–	Otherwise, BitstreamToDecode is set to be the output by invoking the sub-bitstream extraction process as specified in subclause C.6 with entireBitstream, opOlsIdx, and opTid as inputs.

Otherwise (opSubpicIdxList[ ] is  present), BitstreamToDecode is set to be the output by invoking the subpicture sub-bitstream extraction process as specified in subclause C.7 with entireBitstream, opOlsIdx, opTid and opSubpicIdxList[ j ] for j from 0 to NumLayersInOls[ opOlsIdx ] − 1, inclusive, as inputs.

- The values of TargetOlsIdx and Htid are set equal to opOlsIdx and opTid, respectively, of targetOp.

- The general_timing_hrd_parameters( ) syntax structure, the ols_timing_hrd_parameters( ) syntax structure, and the sublayer_hrd_parameters( ) syntax structure or the IOH SEI message applicable to BitstreamToDecode are selected as follows:

–	If NumLayersInOls[ TargetOlsIdx ] is equal to 1, the general_timing_hrd_parameters( ) syntax structure and the ols_timing_hrd_parameters( ) syntax structure in the SPS (or provided through an external means not specified in this Specification) are selected. Otherwise, the general_timing_hrd_parameters( ) syntax structure and the vps_ols_timing_hrd_idx[ MultiLayerOlsIdx[ TargetOlsIdx ] ]-th ols_timing_hrd_parameters( ) syntax structure in the VPS (or provided through an external means not specified in this Specification) are selected.

–	If OnlyIrapAusFlag is equal to 0, within the selected ols_timing_hrd_parameters( ) syntax structure, for testing of the Type I bitstream conformance point, the sublayer_hrd_parameters( Htid ) syntax structure that immediately follows the condition "if( general_vcl_hrd_params_present_flag )" is selected and the variable NalHrdModeFlag is set equal to 0, and for testing of the Type II bitstream conformance point, the sublayer_hrd_parameters( Htid ) syntax structure that immediately follows the condition "if( general_nal_hrd_params_present_flag )" is selected and the variable NalHrdModeFlag is set equal to 1. When BitstreamToDecode is a Type II bitstream and NalHrdModeFlag is equal to 0, all non-VCL NAL units except the PH and filler data NAL units, and all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex B), when present, are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

–	Otherwise (OnlyIrapAusFlag is equal to 1), within the selected IOH SEI message, for testing of the Type I bitstream conformance point, the syntax elements within the condition "if( irap_only_general_vcl_hrd_params_present_flag )" are selected and the variable NalHrdModeFlag is set equal to 0, and for testing of the Type II bitstream conformance point, the syntax elements within the condition "if( irap_only_general_nal_hrd_params_present_flag )" are selected and the variable NalHrdModeFlag is set equal to 1. When BitstreamToDecode is a Type II bitstream and NalHrdModeFlag is equal to 0, all non-VCL NAL units except the PH and filler data NAL units, and all leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex B), when present, are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

- An AU associated with a BP SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) applicable to TargetOp is selected as the HRD initialization point and referred to as AU 0.

- When general_du_hrd_params_present_flag in the selected general_timing_hrd_parameters( ) syntax structure is equal to 1, the CPB is scheduled to operate either at the AU level (in which case the variable DecodingUnitHrdFlag is set equal to 0) or at the DU level (in which case the variable DecodingUnitHrdFlag is set equal to 1). Otherwise, DecodingUnitHrdFlag is set equal to 0 and the CPB is scheduled to operate at the AU level.

- For each AU in BitstreamToDecode starting from AU 0, the BP SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the AU and applies to TargetOlsIdx is selected, and the PT SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the AU and applies to TargetOlsIdx is selected, and when DecodingUnitHrdFlag is equal to 1 and bp_du_cpb_params_in_pic_timing_sei_flag is equal to 0, the DUI SEI messages (present in BitstreamToDecode or available through external means not specified in this Specification) that are associated with DUs in the AU and apply to TargetOlsIdx are selected.

- A value of ScIdx is selected. The selected ScIdx shall be in the range of 0 to hrd_cpb_cnt_minus1, inclusive.

- When the BP SEI message associated with AU 0 has bp_alt_cpb_params_present_flag equal to 0 or OnlyIrapAusFlag is equal to 1, the variable DefaultInitCpbParamsFlag is set equal to 1. Otherwise, when the BP SEI message associated with AU 0 has bp_alt_cpb_params_present_flag equal to 1, either of the following applies for selection of the initial CPB removal delay and delay offset:

–	If OnlyIrapAusFlag is equal to 1, the following applies:

–	If NalHrdModeFlag is equal to 1, the initial CPB removal delay and delay offset represented by irap_only_nal_initial_cpb_removal_delay[ ScIdx ] and irap_only_nal_initial_cpb_removal_offset[ ScIdx ], respectively, in the selected IOH SEI message are selected. Otherwise, (i.e. if OnlyIrapAusFlag is equal to 1 and NalHrdModeFlag is equal to 0), the initial CPB removal delay and delay offset represented by irap_only_vcl_initial_cpb_removal_delay[ ScIdx ] and irap_only_vcl_initial_cpb_removal_offset[ ScIdx ], respectively, in the selected IOH SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 1.

–	Otherwise (OnlyIrapAusFlag is equal to 0), the following applies:

–	If NalHrdModeFlag is equal to 1, the default initial CPB removal delay and delay offset represented by bp_nal_initial_cpb_removal_delay[ Htid ][ ScIdx ] and bp_nal_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message are selected. Otherwise, the default initial CPB removal delay and delay offset represented by bp_vcl_initial_cpb_removal_delay[ Htid ][ ScIdx ] and bp_vcl_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 1.

–	If NalHrdModeFlag is equal to 1, the alternative initial CPB removal delay and delay offset represented by bp_nal_initial_cpb_removal_delay[ Htid ][ ScIdx ] and bp_nal_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message and pt_nal_cpb_alt_initial_removal_delay_delta[ Htid ][ ScIdx ] and pt_nal_cpb_alt_initial_removal_offset_delta[ Htid ][ ScIdx ], respectively, in the PT SEI message associated with the AU following AU 0 in decoding order are selected. Otherwise, the alternative initial CPB removal delay and delay offset represented by bp_vcl_initial_cpb_removal_delay[ Htid ][ ScIdx ] and bp_vcl_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message and pt_vcl_cpb_alt_initial_removal_delay_delta[ Htid ][ ScIdx ] and pt_vcl_cpb_alt_initial_removal_offset_delta[ Htid ][ ScIdx ], respectively, in the PT SEI message associated with the AU following AU 0 in decoding order are selected. The variable DefaultInitCpbParamsFlag is set equal to 0, and one of the following applies:

–	The RASL AUs that contain RASL pictures with pps_mixed_nalu_types_in_pic_flag equal to 0 and are associated with CRA pictures contained in AU 0 are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

–	All AUs following AU 0 in decoding order up to an AU associated with a DRAP indication SEI message are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

Each conformance test consists of a combination of one option selected in each of these steps. When there is more than one option for a step, for any particular conformance test only one option is chosen. All possible combinations of all the steps form the entire set of conformance tests. For each operation point under test, the number of bitstream conformance tests to be performed is equal to n0 * n1 * ( ( 2 − n6 ) * n2 + n3 + n4 *( 1 − n6 ) + n5 *( 1 − n6 ) ), where the values of n0, n1, n2, n3, n4, and n5 are specified as follows:

–	n0 is set equal to 2.

–	n1 is equal to hrd_cpb_cnt_minus1 + 1.

–	n2 is the number of AUs in BitstreamToDecode that each is associated with a BP SEI message applicable to TargetOlsIdx and for which all of the following conditions are true:

–	nal_unit_type is equal to CRA_NUT.

–	The associated BP SEI message has bp_alt_cpb_params_present_flag equal to 1.

–	There is at least one RASL picture with pps_mixed_nalu_types_in_pic_flag equal to 0 associated with the AU.

–	n3 is the number of IRAP or GDR AUs in BitstreamToDecode that each is associated with a BP SEI message applicable to TargetOlsIdx and for which at least one the following conditions is false:

–	nal_unit_type is equal to CRA_NUT.

–	The associated BP SEI message has bp_alt_cpb_params_present_flag equal to 1.

–	There is at least one RASL picture with pps_mixed_nalu_types_in_pic_flag equal to 0 associated with the AU.

–	n4 is the number of AUs in BitstreamToDecode that each is associated with a DRAP indication SEI message applicable to TargetOlsIdx and for each of which the associated PT SEI message has pt_cpb_alt_timing_info_present_flag equal to 1.

–	n5 is derived as follows:

–	If general_du_hrd_params_present_flag in the selected general_timing_hrd_parameters( ) syntax structure is equal to 0, n5 is equal to 1.

–	Otherwise, n5 is equal to 2.

–	n6 is derived as follows:

–	If an IOH SEI message is associated with the bitstream, n6 is equal to 1.

–	Otherwise, n6 is equal to 0.

NOTE 2 – n0 corresponds to conformance tests for Type I bitstream conformance and Type II bitstream conformance. n1 corresponds to conformance tests for each CPB delivery schedule. n2 corresponds to conformance tests for bitstreams starting at each CRA pictures with associated RASL pictures and alternative initial CPB removal delay and delay offset present. These tests are performed twice: once for bitstream keeping and once for bitstreams removing RASL pictures associated with the CRA. n3 corresponds to conformance tests for bitstreams starting at each IRAP or GDR AU that is not a CRA with associated RASL pictures and alternative initial CPB removal delay and delay offset present. n4 corresponds to conformance tests for bitstreams starting with each IRAP with associated DRAP pictures with alternative timing information present and result of removing all AUs between a DRAP picture with alternative timing information present and the preceding IRAP. n5 corresponds to conformance tests for AU based conformance and DU-based when general_du_hrd_params_present_flag is equal to 1.

When BitstreamToDecode is a Type II bitstream, the following applies:

–	If OnlyIrapAusFlag is equal to 0 and the sublayer_hrd_parameters( Htid ) syntax structure that immediately follows the condition "if( general_vcl_hrd_params_present_flag )" is selected, the test is conducted at the Type I conformance point shown in Figure 20, and only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

–	Otherwise, if OnlyIrapAusFlag is equal to 0 and the sublayer_hrd_parameters( Htid ) syntax structure that immediately follows the condition "if( general_nal_hrd_params_present_flag )" is selected, the test is conducted at the Type II conformance point shown in Figure 20, and all bytes of the Type II bitstream, which could be a NAL unit stream or a byte stream, are counted for the input bit rate and CPB storage.

–	Otherwise, if OnlyIrapAusFlag is equal to 1 and the syntax elements within the condition "if( irap_only_general_vcl_hrd_params_present_flag )" in the IOH SEI are selected, the test is conducted at the Type I conformance point shown in Figure 20, and only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

–	Otherwise (OnlyIrapAusFlag is equal to 1 and the syntax elements within the condition "if( irap_only_general_nal_hrd_params_present_flag )" in the IOH SEI are selected), the test is conducted at the Type II conformance point shown in Figure 20, and all bytes of the Type II bitstream, which could be a NAL unit stream or a byte stream, are counted for the input bit rate and CPB storage.

NOTE 3 – NAL HRD parameters established by a value of ScIdx for the Type II conformance point shown in Figure 20 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure 20 for the same values of InitCpbRemovalDelay[ ScIdx ], BitRate[ Htid ][ ScIdx ] and CpbSize[ Htid ][ ScIdx ] for the variable bit rate (VBR) case (cbr_flag[ Htid ][ ScIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive.

[…]

For each bitstream conformance test, the CPB size (number of bits) is CpbSize[ Htid ][ ScIdx ] as specified in subclause 7.4.6.3 or subclause D.X.2, for OnlyIrapAusFlag equal to 0 and 1, respectively, where ScIdx and the HRD parameters are specified above in this subclause, and the DPB parameters dpb_max_dec_pic_buffering_minus1[ Htid ], dpb_max_num_reorder_pics[ Htid ], and MaxLatencyPictures[ Htid ] are found in or derived from the dpb_parameters( ) syntax structure that applies to the target OLS as follows:

–	If NumLayersInOls[ TargetOlsIdx ] is equal to 1, the dpb_parameters( ) syntax structure is found in the SPS, and the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, and MaxBitDepthMinus8 are set equal to sps_pic_width_max_in_luma_samples, sps_pic_height_max_in_luma_samples, sps_chroma_format_idc, and sps_bitdepth_minus8, respectively, found in the SPS.

–	Otherwise (NumLayersInOls[ TargetOlsIdx ] is greater than 1), the dpb_parameters( ) syntax structure is identified by vps_ols_dpb_params_idx[ MultiLayerOlsIdx[ TargetOlsIdx ] ] found in the VPS, and the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, and MaxBitDepthMinus8 are set equal to vps_ols_dpb_pic_width[ MultiLayerOlsIdx[ TargetOlsIdx ] ], vps_ols_dpb_pic_height[ MultiLayerOlsIdx[ TargetOlsIdx ] ], vps_ols_dpb_chroma_format[ MultiLayerOlsIdx[ TargetOlsIdx ] ], and vps_ols_dpb_bitdepth_minus8[ MultiLayerOlsIdx[ TargetOlsIdx ] ], respectively, found in the VPS.

If DecodingUnitHrdFlag is equal to 0, the HRD operates at the AU level and each DU is an AU. Otherwise the HRD operates at the DU level and each DU is a subset of an AU.

NOTE 6 – If the HRD operates at the AU level, each time when some bits are removed from the CPB, a DU that is an entire AU is removed from the CPB. Otherwise (the HRD operates at the DU level), each time when some bits are removed from the CPB, a DU that is a subset of an AU is removed from the CPB. Regardless of whether the HRD operates at access unt level or DU level, each time when some picture is output from the DPB, an entire decoded picture is output from the DPB, though the picture output time is derived based on the differently derived CPB removal times and the differently signalled DPB output delays.

The following is specified for expressing the constraints in this annex:

–	Each AU is referred to as AU n, where the number n identifies the particular AU. AU 0 is selected per step 5 above. The value of n is incremented by 1 for each subsequent AU in decoding order.

–	Each DU is referred to as DU m, where the number m identifies the particular DU. The first DU in decoding order in AU 0 is referred to as DU 0. The value of m is incremented by 1 for each subsequent DU in decoding order.

NOTE 7 – The numbering of DUs is relative to the first DU in AU 0.

–	Picture n refers to the coded picture or the decoded picture of AU n.

The HRD operates as follows:

–	The HRD is initialized at DU 0, with both the CPB and the DPB being set to be empty (the DPB fullness is set equal to 0).

NOTE 8 – After initialization, the HRD is not initialized again by subsequent BP SEI messages.

–	Data associated with DUs that flow into the CPB according to a specified arrival schedule are delivered by the hypothetical stream scheduler (HSS).

–	The data associated with each DU are removed and decoded instantaneously by the instantaneous decoding process at the CPB removal time of the DU.

–	Each decoded picture is placed in the DPB.

–	A decoded picture is removed from the DPB when it becomes no longer needed for inter prediction reference and no longer needed for output.

For each bitstream conformance test, the operation of the CPB is specified in subclause C.2, the instantaneous decoder operation is specified in Clauses 2 through 9, the operation of the DPB is specified in subclause C.3 and the output cropping is specified in subclauses C.3.3 and C.5.2.2.

HSS and HRD information concerning the number of enumerated delivery schedules and their associated bit rates and buffer sizes is specified in subclauses 7.3.5.1 and 7.4.6.1 or subclause D.X.2, for OnlyIrapAusFlag equal to 0 and 1, respectively. The HRD is initialized as specified by the BP SEI message (specified in subclause D.3). The removal timing of DUs from the CPB and output timing of decoded pictures from the DPB is specified using information in PT SEI messages (specified in subclause D.4) or in DUI SEI messages (specified in subclause D.5). All timing information relating to a specific DU shall arrive prior to the CPB removal time of the DU.

[…]

C.2.2 Timing of DU arrival

[…]

The final arrival time for DU m is derived as follows:

if( !decodingUnitParamsFlag )
		AuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ] ÷ 
		BitRate[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ]	(1584) 
else
		DuFinalArrivalTime[ m ] = initArrivalTime[ m ] + sizeInbits[ m ] ÷
		BitRate[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ]

where sizeInbits[ m ] is the size in bits of DU m, counting the bits of the VCL NAL units, the PH NAL units, and the filler data NAL units for the Type I conformance point or all bits of the Type II bitstream for the Type II conformance point, where the Type I and Type II conformance points are as shown in Figure 20.

The values of ScIdx, BitRate[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ] and CpbSize[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ] are constrained as follows:

–	If the content of the selected general_timing_hrd_parameters( ) syntax structures for the AU containing AU m and the previous AU differ, the HSS selects a value ScIdx1 of ScIdx from among the values of ScIdx provided in the selected general_timing_hrd_parameters( ) syntax structures for the AU containing AU m that results in a BitRate[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx1 ] or CpbSize[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx1 ] for the AU containing AU m. The value of BitRate[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx1 ] or CpbSize[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx1 ] may differ from the value of BitRate[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx0 ] or CpbSize[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx0 ] for the value ScIdx0 of ScIdx that was in use for the previous AU.

–	Otherwise, the HSS continues to operate with the previous values of ScIdx, BitRate[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ] and CpbSize[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ].

When the HSS selects values of BitRate[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ] or CpbSize[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ] that differ from those of the previous AU, the following applies:

–	The variable BitRate[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ] comes into effect at the initial CPB arrival time of the current AU.

–	The variable CpbSize[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ] comes into effect as follows:

–	If the new value of CpbSize[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ] is greater than the old CPB size, it comes into effect at the initial CPB arrival time of the current AU.

–	Otherwise, the new value of CpbSize[ OnlyIrapAusFlag ? (MaxSubLayersMinus1 + 1) : Htid ][ ScIdx ] comes into effect at the CPB removal time of the current AU.

C.2.3 Timing of DU removal and decoding of DU

The values of the variables InitCpbRemovalDelay[ ScIdx ] and InitCpbRemovalDelayOffset[ ScIdx ] are updated as follows:

–	If OnlyIrapAusFlag is equal to 1, the following applies:

–	InitCpbRemovalDelay[ ScIdx ] and InitCpbRemovalDelayOffset[ ScIdx ] are set equal to the values of irap_only_nal_initial_cpb_removal_delay[ ScIdx ] and irap_only_nal_initial_cpb_removal_offset[ ScIdx ], respectively, when NalHrdModeFlag is equal to 1, or irap_only_vcl_initial_cpb_removal_‌delay[ ScIdx ] and irap_only_vcl_initial_cpb_removal_offset[ ScIdx ], respectively, when NalHrdModeFlag is equal to 0, where the BP SEI message is selected as specified in subclause C.1.

–	Otherwise (OnlyIrapAusFlag is equal to 0), the following applies:

–	If one or more of the following conditions are true, InitCpbRemovalDelay[ ScIdx ] and InitCpbRemovalDelayOffset[ ScIdx ] are set equal to the values of bp_nal_initial_cpb_removal_‌delay[ Htid ][ ScIdx ] and bp_nal_initial_cpb_removal_offset[ Htid ][ ScIdx ] minus the values of pt_nal_cpb_alt_initial_removal_delay_delta[ Htid ][ ScIdx ] and pt_nal_cpb_alt_initial_removal_offset_‌delta[ Htid ][ ScIdx ] of AU n + 1, respectively, when NalHrdModeFlag is equal to 1, or the values of bp_vcl_initial_cpb_removal_delay[ Htid ][ ScIdx ] and pt_vcl_bp_vcl_initial_cpb_removal_offset[ Htid ][ ScIdx ] minus the values of pt_vcl_cpb_alt_initial_removal_delay_delta[ Htid ][ ScIdx ] and cpb_alt_initial_removal_‌offset_delta[ Htid ][ ScIdx ] of AU n + 1, respectively, when NalHrdModeFlag is equal to 0, where the BP SEI message and the PT SEI message are selected as specified in subclause C.1:

–	UseAltCpbParamsFlag for AU n is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Otherwise, if the value of DecodingUnitParamsFlag is equal to 1, InitCpbRemovalDelay[ ScIdx ] and InitCpbRemovalDelayOffset[ ScIdx ] are set equal to the values of the BP SEI message syntax elements bp_nal_initial_alt_cpb_removal_delay[ Htid ][ ScIdx ] and bp_nal_initial_alt_cpb_removal_offset[ Htid ][ ScIdx ], respectively, when NalHrdModeFlag is equal to 1, or bp_vcl_initial_alt_cpb_removal_delay[ Htid ][ ScIdx ] and bp_vcl_initial_alt_cpb_removal_offset[ Htid ][ ScIdx ], respectively, when NalHrdModeFlag is equal to 0, where the BP SEI message containing the syntax elements is selected as specified in subclause C.1.

–	Otherwise, InitCpbRemovalDelay[ ScIdx ] and InitCpbRemovalDelayOffset[ ScIdx ] are set equal to the values of bp_nal_initial_cpb_removal_delay[ Htid ][ ScIdx ] and bp_nal_initial_cpb_removal_‌offset[ Htid ][ ScIdx ], respectively, when NalHrdModeFlag is equal to 1, or bp_vcl_initial_cpb_removal_‌delay[ Htid ][ ScIdx ] and bp_vcl_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, when NalHrdModeFlag is equal to 0, where the BP SEI message is selected as specified in subclause C.1.

The nominal removal time of the AU n from the CPB is specified as follows:

–	If AU n is the AU with n equal to 0 (the AU that initializes the HRD), the nominal removal time of the AU from the CPB is specified by:

AuNominalRemovalTime[ 0 ] = InitCpbRemovalDelay[ ScIdx ] ÷ SpeedupFactor ÷ 90000	(1585)

–	Otherwise, the following applies:

–	When AU n is the first AU of a BP that does not initialize the HRD, the following applies:

The nominal removal time of the AU n from the CPB is specified by:

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstAuInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
	tmpCpbDelayOffset = CpbDelayOffset
} else {
	baseTime1 = AuNominalRemovalTime[ prevNonDiscardableAu ]
	tmpCpbRemovalDelay1 = ( auCpbRemovalDelayDeltaMinus1 + 1 )
	baseTime2 = AuNominalRemovalTime[ n − 1 ]
	tmpCpbRemovalDelay2 = Ceil( ( InitCpbRemovalDelay[ ScIdx ] ÷ 90000 +	(1586)
			AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] ) ÷ ClockTick )
	if( baseTime1 + ClockTick * tmpCpbRemovalDelay1 <
			baseTime2 + ClockTick * tmpCpbRemovalDelay2 ) {
		baseTime = baseTime2
		tmpCpbRemovalDelay = tmpCpbRemovalDelay2
	} else {
		baseTime = baseTime1
		tmpCpbRemovalDelay = tmpCpbRemovalDelay1
	}
	tmpCpbDelayOffset = 0
}
AuNominalRemovalTime[ n ] = baseTime + ( ClockTick * tmpCpbRemovalDelay ÷ SpeedupFactor − tmpCpbDelayOffset )

where AuNominalRemovalTime[ firstAuInPrevBuffPeriod ] is the nominal removal time of the first AU of the previous BP, AuNominalRemovalTime[ prevNonDiscardableAu ] is the nominal removal time of the previous AU in decoding order with TemporalId equal to 0 that has at least one picture that has ph_non_ref_pic_flag equal to 0 that is not a RASL or RADL picture, AuCpbRemovalDelayVal is the value of CpbRemovalDelayVal[ Htid ] derived according to pt_cpb_removal_delay_minus1[ Htid ] and pt_cpb_removal_delay_delta_idx[ Htid ] in the PT SEI message, and bp_cpb_removal_delay_delta_val[ pt_cpb_removal_delay_delta_idx[ Htid ] ] in the BP SEI message, selected as specified in subclause C.1, associated with AU n and concatenationFlag and auCpbRemovalDelayDeltaMinus1 are the values of the syntax elements bp_concatenation_flag and bp_cpb_removal_delay_delta_minus1, respectively, in the BP SEI message, selected as specified in subclause C.1, associated with AU n.

After the derivation of the nominal CPB removal time and before the derivation of the DPB output time of access unit n, the variables DpbDelayOffset and CpbDelayOffset are derived as:

–	If one or more of the following conditions are true, DpbDelayOffset is set equal to the value of the PT SEI message syntax element pt_nal_dpb_delay_offset[ Htid ] (when NalHrdModeFlag is equal to 1) or pt_vcl_dpb_delay_offset[ Htid ] (when NalHrdModeFlag is equal to 0) of AU n + 1, and CpbDelayOffset is set equal to the value of the PT SEI message syntax element pt_nal_cpb_delay_offset[ Htid ] (when NalHrdModeFlag is equal to 1) or pt_vcl_cpb_delay_offset[ Htid ] (when NalHrdModeFlag is equal to 0) of AU n + 1, where the PT SEI message containing the syntax elements is selected as specified in subclause C.1:

–	UseAltCpbParamsFlag for AU n is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Otherwise, DpbDelayOffset and CpbDelayOffset are both set equal to 0.

–	When AU n is not the first AU of a BP, the nominal removal time of the AU n from the CPB is specified by:

AuNominalRemovalTime[ n ] = AuNominalRemovalTime[ firstAuInCurrBuffPeriod ] +
		ClockTick * ( AuCpbRemovalDelayVal − CpbDelayOffset ) ÷ SpeedupFactor	(1587)

where AuNominalRemovalTime[ firstAuInCurrBuffPeriod ] is the nominal removal time of the first AU of the current BP and AuCpbRemovalDelayVal is the value of CpbRemovalDelayVal[ OpTid ] derived according to pt_cpb_removal_delay_minus1[ OpTid ] and pt_cpb_removal_delay_delta_idx[ OpTid ] in the PT SEI message, and bp_cpb_removal_delay_delta_val[ pt_cpb_removal_delay_delta_idx[ OpTid ] ] in the BP SEI message, selected as specified in subclause C.1, associated with AU n.

[…]

C.6 IRAP AU sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, and a target highest TemporalId value tIdTarget and a value OnlyIrapAusFlag.

Output of this process is a sub-bitstream outBitstream.

The OLS with OLS index targetOlsIdx is referred to as the target OLS.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that satisfies all of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream is the output of the process specified in this subclause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and tIdTarget equal to any value in the range of 0 to vps_ptl_max_tid[ vps_ols_ptl_idx[ targetOlsIdx ] ], inclusive, as inputs.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

The output sub-bitstream OutBitstream is derived by applying the following ordered steps:

- The bitstream outBitstream is set to be identical to the bitstream inBitstream.

- Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

- Remove from outBitstream all NAL units that have nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ], and are not DCI, OPI, VPS, AUD, or EOB NAL units, and are not SEI NAL units containing non-scalable-nested SEI messages with payload PayloadType equal to 0, 1, 130, or 203.

- Remove from outBitstream all APS and VCL NAL units for which all of the following conditions are true, and the associated non-VCL NAL units of these VCL NAL units with nal_unit_type equal to PH_NUT or FD_NUT, or with nal_unit_type equal to SUFFIX_SEI_NUT or PREFIX_SEI_NUT and containing SEI messages with PayloadType not equal to any of 0 (BP), 1 (PT), 130 (DUI), and 203 (SLI):

–	nal_unit_type is equal to APS_NUT, TRAIL_NUT, STSA_NUT, RADL_NUT, or RASL_NUT, or nal_unit_type is equal to GDR_NUT and the associated ph_recovery_poc_cnt is greater than 0.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

- When OnlyIrapAusFlag is equal to 1, remove from outBitstream all NAL units of an AU for which the following applies:

–	NumLayersInOls[ targetOlsIdx ] is equal to 1 and there is at least one VCL NAL unit in the AU having nal_unit_type not equal to IDR_W_RADL, IDR_NLP or CRA_NUT.

–	NumLayersInOls[ targetOlsIdx ] is greater than 1 and no AUD NAL unit with aud_irap_or_gdr_flag equal to 1 is present in the AU.

- When all VCL NAL units of an AU are removed by steps 2, 3, or 4 above and an AUD or OPI NAL unit is present in the AU, remove the AUD or OPI NAL unit from outBitstream.

- For each OPI NAL unit in outBitstream, set opi_htid_info_present_flag equal to 1, set opi_ols_info_present_flag equal to 1, set opi_htid_plus1 equal to tIdTarget + 1, and set opi_ols_idx equal to targetOlsIdx.

- When an AUD is present in an AU in outBitstream and the AU becomes an IRAP or GDR AU, set aud_irap_or_gdr_flag of the AUD equal to 1.

- Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_ols_flag equal to 1 and there is no value of i in the range of 0 to sn_num_olss_minus1, inclusive, such that NestingOlsIdx[ i ] is equal to targetOlsIdx.

- Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_ols_flag equal to 0 and there is no value in the list NestingLayerId equal to a value in the list LayerIdInOls[ targetOlsIdx ].

- When LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all VCL NAL units in the bitstream inBitstream, the following applies in the order listed:

- Remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 0 (BP), 130 (DUI), or 203 (SLI) or 205(IOH).

- When general_same_pic_timing_in_all_ols_flag is equal to 0, remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 1 (PT).

- When outBitstream contains an SEI NAL unit seiNalUnitA that contains a scalable nesting SEI message with sn_ols_flag equal to 1 and sn_subpic_flag equal to 0 that applies to the target OLS, or when NumLayersInOls[ targetOlsIdx ] is equal to 1 and outBitstream contains an SEI NAL unit seiNalUnitA that contains a scalable nesting SEI message with sn_ols_flag is equal to 0 and sn_subpic_flag equal to 0 that applies to the layer in outBitstream, generate a new SEI NAL unit seiNalUnitB, include it in the PU containing seiNalUnitA immediately after seiNalUnitA, extract the scalable-nested SEI messages from the scalable nesting SEI message and include them directly in seiNalUnitB (as non-scalable-nested SEI messages), and remove seiNalUnitA from outBitstream.

C.7 Subpicture sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, a target highest TemporalId value tIdTarget, and a list  of target subpicture index values subpicIdxTarget[ i ] for i from 0 to NumLayersInOls[ targetOlsIdx ] − 1, inclusive, and a value OnlyIrapAusFlag.

Output of this process is a sub-bitstream outBitstream.

The OLS with OLS index targetOlsIdx is referred to as the target OLS. Among the layers in the target OLS, those for which the referenced SPSs have sps_num_subpics_minus1 greater than 0 are referred to as the multiSubpicLayers.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that satisfies all of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream is the output of the process specified in this subclause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, tIdTarget equal to any value in the range of 0 to vps_max_sublayers_minus1, inclusive, and the list subpicIdxTarget[ i ] for i from 0 to NumLayersInOls[ targetOlsIdx ] − 1, inclusive, satisfying the following conditions, as inputs:

–	The value of subpicIdxTarget[ i ] is equal to a value in the range of 0 to sps_num_subpics_minus1, inclusive, such that sps_subpic_treated_as_pic_flag[ subpicIdxTarget[ i ] ] is equal to 1, where sps_num_subpics_minus1 and sps_subpic_treated_as_pic_flag[ subpicIdxTarget[ i ] ] are found in or inferred based on the SPS referred to by the layer with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ i ].

NOTE 1 – When the sps_num_subpics_minus1 for the layer with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ i ] is equal to 0, the value of subpicIdxTarget[ i ] is equal to 0.

–	For any two different integer values of m and n, when sps_num_subpics_minus1 is greater than 0 for both layers with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ m ] and LayerIdInOls[ targetOlsIdx ][ n ], respectively, subpicIdxTarget[ m ] is equal to subpicIdxTarget[ n ].

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in the list LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE 2 – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ i ] and with sh_subpic_id equal to SubpicIdVal[ subpicIdxTarget[ i ] ] for each i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1, inclusive.

The output sub-bitstream outBitstream is derived by the following order steps:

- The sub-bitstream extraction process, specified in Annex C.6, is invoked with inBitstream, targetOlsIdx, and tIdTarget and OnlyIrapAusFlag as inputs and the output of the process is assigned to outBitstream.

[…]

# Patent rights declaration(s)

Fraunhofer HHI, Bytedance Inc. and Sharp Labs of America, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)